| Robertson's Remedial and Painting Pty Ltd | Document No: RPD-PR-DEM-001 |
| --- | --- |
| DEMOLITION WORK PROCEDURE | Version: 1.0 ¦ Date: Feb 2025 ¦ Review: Feb 2027 |
| Approved: Leslie Robertson (Director) | Ed.6 Ref: Checklist 5, Appendix B ¦ WHS Reg 2017 Ch.8 Pt.6 |

1. Purpose

This procedure sets out how RPD manages demolition work to comply with WHS Regulation 2017 Chapter 8 Part 6 and SafeWork NSW Code of Practice: Demolition Work. It applies to all RPD workers, subcontractors and visitors on sites where demolition is performed.

2. Scope

Covers partial demolition, strip-out and removal of building elements during remedial, cladding replacement and concrete repair works. Full structural demolition requiring a licensed demolisher (Class 1) is excluded — RPD engages a licensed demolition contractor for that scope.

3. Procedure — Before, During, After

| Phase | Step | Action | Responsible | Record |
| --- | --- | --- | --- | --- |
| BEFORE | 1 | Confirm scope: identify elements to be demolished, adjacent structures, services (electrical, gas, water, comms). Obtain relevant drawings. | Project Manager | Project file |
|  | 2 | Commission hazardous materials survey (asbestos, lead paint, silica) by competent person. Do NOT commence demolition until clearance received. | Project Manager | Survey report |
|  | 3 | Prepare SWMS for demolition task. Ensure it addresses: falling objects, structural collapse, dust/noise, services isolation, manual handling, working at heights. Consult workers during preparation. | Site Supervisor | SWMS (Breadcrumb) |
|  | 4 | Establish exclusion zone with barriers and signage. Confirm safe distances from adjacent occupied areas. Notify neighbours / strata if required. | Site Supervisor | Site photos |
|  | 5 | Isolate and confirm de-energisation of all services (electrical, gas, hydraulic). Tag and lock out. Obtain written confirmation from service provider. | Site Supervisor | Isolation cert |
| DURING | 6 | Toolbox talk: brief all workers on SWMS, exclusion zones, emergency procedure and stop-work triggers. All workers sign on. | Site Supervisor | Toolbox (Breadcrumb) |
|  | 7 | Demolish top-down, one level at a time. Do not undercut or remove structural supports prematurely. Use mechanical methods in preference to manual where practicable. | Demolition crew | — |
|  | 8 | Continuous monitoring: dust suppression (wet methods), noise controls, exclusion zone integrity. Stop work if unexpected hazard found (e.g. ACM, unstable structure). | Site Supervisor | Hazard report (Breadcrumb) |
|  | 9 | Minimum PPE: hard hat, safety glasses, P2 respirator (dust), steel-cap boots, high-vis, hearing protection, gloves. Additional RPE if lead/silica identified. | All workers | — |
| AFTER | 10 | Waste segregation: separate general, hazardous (ACM, lead) and recyclable waste. Dispose via licensed waste contractor. Retain waste dockets. | Site Supervisor | Waste dockets |
|  | 11 | Make safe: temporary propping, edge protection on exposed openings, hoarding. Confirm structure is stable before leaving site. | Site Supervisor | Inspection (Breadcrumb) |
|  | 12 | Close out: update site drawings if required. File all records in Breadcrumb project folder. Debrief crew on lessons learned. | Project Manager | Project file |

4. Stop Work Triggers

Any worker must stop work immediately and notify the Site Supervisor if: suspected asbestos or hazardous material found  •  unexpected structural movement or cracking  •  live/unconfirmed services encountered  •  exclusion zone breached by public or other trades  •  weather conditions create unsafe exposure (high wind, lightning).

5. References:  WHS Act 2011  •  WHS Regulation 2017 Ch.8 Pt.6  •  SafeWork NSW COP: Demolition Work  •  RPD WHSMS V7 s.4.24  •  HY Standard: Demolition rev8

Generated by Gatekeeper WHS Advisor  •  Benchmarked against Hansen Yuncken Demolition Standard rev8 and SafeWork NSW COP