Clarence St Strip-Out

| PCBU: | [Insert PCBU here] | Workplace location: |  |
| --- | --- | --- | --- |
| Works Manager: |  | Date SWMS provided to PC: | 31/03/2026 |
| Work activity: | Clarence St Strip-Out | Principal Contractor (PC): | [Insert PCBU here] |
| High Risk Construction Work (HRCW): ☑ Risk of a person falling more than 2 metres ☑ Likely to involve disturbing asbestos ☐ Work in or near a shaft or trench deeper than 1.5 m or a tunnel ☐ Work on or near chemical, fuel or refrigerant lines ☐ Tilt-up or precast concrete elements ☐ Work on a telecommunication tower ☐ Temporary load-bearing support for structural alterations or repairs ☐ Use of explosives ☑ Work on or near energised electrical installations or services ☐ Work on, in or adjacent to a road, railway, shipping lane or other traffic corridor in use by traffic other than pedestrians ☑ Demolition of load-bearing structure ☐ Work in or near a confined space ☐ Work on or near pressurised gas mains or piping ☐ Work in an area that may have a contaminated or flammable atmosphere ☐ Work in an area with movement of powered mobile plant ☐ Work in areas with artificial extremes of temperature ☐ Work in or near water or other liquid that involves a risk of drowning ☐ Diving work |  |  |  |
| Person responsible for ensuring compliance with SWMS: | [Insert Supervisor name here] | Date SWMS received: | 31/03/2026 |
| What measures are in place to ensure compliance with the SWMS? | Toolbox meetings, SWMS sign off, job observations and supervision review. If issues with the SWMS or new hazards are identified, the supervisor must be notified. When changes are made to SWMS, it will be communicated to all workers. |  |  |
| Person responsible for reviewing SWMS control measures: |  | Date SWMS received by reviewer: | 31/03/2026 |
| How will the SWMS control measures be reviewed? | The control measures implemented will be reviewed and if necessary, revised annually or if work methods change, the control measures are not effective in controlling the risk, a new hazard/risk is identified or following an incident. The SWMS will be reviewed in consultation with workers and/or others who may be affected by the SWMS. Any changes to the SWMS will be communicated with workers at induction, daily pre-starts and toolbox talks. |  |  |
| Reviewer’s signature: |  | Review date: | 31/03/2026 |
| This SWMS must be kept and be available for inspection until the high-risk construction work to which this SWMS relates is completed. If the SWMS is revised, all versions should be kept. If a notifiable incident occurs in relation to the high-risk construction work in this SWMS, the SWMS must be kept for at least 2 years from the date of the notifiable incident. |  |  |  |

PRE-REQUISITES TABLE

| Must be in place before starting work |  |  |  |
| --- | --- | --- | --- |
| PPE — All Persons on Site | • steel-capped footwear • hi-viz shirt or vest • eye protection • hearing protection (>85dB) • chemical-resistant gloves (nitrile or task-appropriate) • Hard hat where overhead risk present | Additional PPE | • Not applicable for this scope |
| Licences / Qualifications | • Licensed supervisor on site at all times during removal • Asbestos Assessor — clearance inspection before re-occupation • Licensed demolition supervisor — on site at all times • Structural engineer sign-off — pre-demolition engineering assessment • Structural monitoring — where adjacent structures at risk, engineer-specified monitoring regime in place before demolition • Falling object exclusion zone — established and maintained for full height of demolition face plus 5m | Permits & Approvals | • Asbestos removal permit — required before any licensed removal • Council development consent — where required • SafeWork NSW demolition notification — structures over 6m • Pre-demolition hazardous materials survey — by licensed assessor, completed and reviewed before any work • Utility isolation certificates — electricity, gas, water, and telecommunications confirmed disconnected before demolition • Electrical isolation permit — lock-out tag-out before work |
| Plant & Equipment | • As per task requirements — see controls column | Hazardous Substances | • No hazardous substances identified — confirm with site supervisor |
| Consultation | • Workers consulted during SWMS development • Site-specific induction completed before work commences • Toolbox talk conducted at start of each shift | Legislative Basis | • WHS Act 2011 (NSW) • WHS Regulation 2017 (NSW) • SafeWork NSW Codes of Practice • WHS Reg 2017 Part 8.3 — asbestos removal duties |

Brief description: Clarence St Strip-Out

STEP-BY-STEP SAFE WORK METHOD TABLE

| Step | Task | Hazard | Risk (Pre) | Controls | Risk (Post) | Responsibility | CCVS Code |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1.1 | Confirm site readiness and establish site control | • Services still energised — electric shock or arc flash during work setup • Asbestos-containing materials not identified or isolated — inhalation exposure before removal phase | High (9) | Engineering Controls: • Verify all electrical switchboards are isolated, locked off, and tagged with 'DO NOT OPERATE' notices at point of isolation. • Confirm building is unoccupied — conduct walk-through with site supervisor and record occupancy status in site diary. • Confirm skip-bin access via loading dock is clear — measure and photograph access route before skip placement. • Brief all site personnel on exclusion zone boundaries, services status, and asbestos locations before work starts. Administrative Controls: • Record building occupancy confirmation in site diary with date, time, and supervisor name. • Verify all workers hold White Card (CPCCWHS1001) and asbestos-related licences (CPCCDE3002 or CPCCDE3014 as applicable) — check before induction. | Medium (4) | • SUP — Verify exclusion zone complete, barriers intact, access controlled before work starts • WKR — Set up and maintain barriers, stop work if zone breached, report access issues | 🛑 STOP-WORK TRIGGER • Any energised electrical circuit is discovered not listed on isolation checklist or tag-lock is disturbed. • Building occupant is found on site or unauthorised person enters exclusion zone. |
| 1.2 | Remove asbestos-containing materials under Class B licence | • Inhalation of asbestos fibres — exposure during removal, handling, or transport • Contamination of non-removal areas — fibres spread outside containment during work • Inadequate waste containment — asbestos waste exposure during bagging or bin loading | High (9) | Engineering Controls: • Establish full containment enclosure around removal area — seal doors, windows, penetrations with heavy-duty plastic sheeting and duct tape before any disturbance. • Operate negative pressure unit with HEPA filtration during all removal work — maintain continuous extraction and monitor pressure gauge remains in negative range. • Apply water mist to asbestos materials 15 minutes before removal and keep damp throughout disturbance — use low-pressure spray only, never dry cut or break out. • Place all removed asbestos materials immediately into labelled heavy-duty asbestos waste bags — seal and label each bag with asbestos warning label and licence holder name before any movement. • Decontaminate all waste bags and external containment surfaces with damp cloths before transport from removal area — remove visible dust. • Brief licensed removalist and all workers on removal method, contamination risk zones, and emergency procedures before any work starts. • Monitor air quality during removal using real-time particle counter — record readings every 30 minutes and stop work if airborne fibre level cannot be controlled. • Establish exclusion zone 2 metres beyond containment boundary — prohibit all unauthorised entry and restrict to licensed removalist and safety supervisor only. Administrative Controls: • tell SafeWork NSW of asbestos removal work according to WHS Reg 2017 Sch 3 cl.18 — obtain written acknowledgement. • Complete and retain asbestos removal work method statement including containment design, removal sequence, waste transport route, and disposal contractor details. • Record air monitoring readings on site — retain for 5 years post-removal. • Arrange Class B waste transporter and licensed asbestos waste disposal facility before removal commencement. • Decontaminate site after removal — wipe all non-removal areas and surfaces with damp cloth, dispose of contamination cloths as asbestos waste. | Medium (3) | • SUP — Supervise Remove asbestos-containing materials under Class • WKR — Perform Remove asbestos-containing materials under Class per SWMS | ⚠️ HOLD POINT 1.2 1. Do not start removal until: licensed asbestos removalist and site safety supervisor on site, containment enclosure complete and pressure-tested to negative, water supply confirmed operational, and waste disposal contractor confirmed to receive labelled asbestos waste. 🛑 STOP-WORK TRIGGER • Negative pressure unit fails or containment is breached — seal and do not re-enter until containment is restored and re-tested. • Water suppression fails or materials become dry during disturbance — re-dampen before continuing. • Air monitoring shows fibre level cannot be controlled or respirator fails — evacuate containment, decontaminate, and reassess before re-entry. RISK CODE:SIL-H6 |
| 1.3 | Remove partition walls and structural supports | • Unintended collapse of primary structure — inadequate support or incorrect member identification • Struck by swinging or shifting wall sections — manual or tool-assisted removal | High (9) | Engineering Controls: • Install debris containment netting or hoarding to full height plus 2m to capture falling objects and dust within exclusion zone. • Cut and lower wall sections manually or with hand tools; do not drop or swing unsupported sections. • Disconnect or isolate all services within partition walls (electrical, water, gas, HVAC) before removal; tag and verify isolation. • check primary structure for damage, lateral movement, or unintended settlement after each major wall removal; stop work if movement detected. Administrative Controls: • Obtain asbestos removal permit and Class A or B licence before work starts. • Appoint structural engineer to oversee member identification and sequence approval. • Conduct air monitoring during any friable asbestos removal (Class A); record results daily. | Medium (4) | • SUP — Supervise Remove partition walls and structural • WKR — Perform Remove partition walls and structural per SWMS | ⚠️ HOLD POINT 1.3 1. Do not start partition wall removal until: structural engineer has identified and approved all load-bearing members in writing and confirmed sequence of removal. 🛑 STOP-WORK TRIGGER • Primary structure shows lateral movement, vertical settlement, cracking, or any sign of instability. • Exclusion zone is breached or debris escapes containment netting. • Unidentified load-bearing member or unexpected structural condition is encountered during removal. RISK CODE:SIL-H6 |
| 1.4 | Remove floor coverings and adhesive backing | • Dust inhalation — silica or vinyl dust released during stripping and adhesive removal • Chemical exposure — adhesive fumes or solvents during wet removal or grinding • Cut or puncture wound — sharp tools or concealed fasteners during stripping work | Medium (6) | Engineering Controls: • Establish wet suppression protocol: spray floor with water mist before and during scraping to prevent dust release; maintain damp surface throughout stripping work. • Install local exhaust ventilation or portable dust extractor with HEPA filter at point of removal; maintain continuous extraction during scraping and grinding operations. • Wear organic vapour respirator (per SDS Section 8) when adhesive fumes present; switch to P2 particulate mask if only dust generation observed and wet suppression fails. • Barricade work area with plastic sheeting and tape seams; restrict access to licensed floor stripping personnel only; maintain negative air pressure if available. • Check floor substrate before stripping for concealed fasteners, wire mesh, or unknown materials; stop work immediately if asbestos-containing material is suspected. • Use hand tools (floor scraper, putty knife) in preference to power grinders where practicable; if grinding required, operate within dust extraction capture zone only. • Contain and segregate all stripped material in labelled waste bins; do not allow offcuts or dust to accumulate on adjacent surfaces or escape containment area. • check scraper and tool blades before use for cracks or damage; replace blunt or broken tools immediately to prevent flying fragments or excessive pressure. Administrative Controls: • Secure asbestos removal permit (Class A or B) from SafeWork NSW if ACM identified; file permit on site. • Maintain SDS for all adhesive removal agents on point of use; brief workers on Section 8 (exposure controls) and Section 16 (disposal) before application. • Keep waste register recording all floor coverings, adhesive residue, and ACM removed; classify and dispose according to licensed waste contractor requirements. • Document air monitoring results (if Class A asbestos removal) or dust level checks during stripping; retain records for 5 years. • Conduct toolbox talk before work begins: cover dust hazards, SDS fume warnings, wet suppression requirement, and stop-work triggers. | Low (2) | • SUP — Supervise Remove floor coverings and adhesive • WKR — Perform Remove floor coverings and adhesive per SWMS | ⚠️ HOLD POINT 1.4 1. Do not start removal until: wet suppression system is operational, dust extractor is running, and respiratory protection is fitted and checked 🛑 STOP-WORK TRIGGER • Dust suppression system fails, wet mist is interrupted, or visible dust cloud escapes containment area • Unknown material or suspected asbestos is encountered; do not disturb further until licensed assessor confirms material type • Respiratory protection is removed, damaged, or wearer reports odour breakthrough or breathing difficulty RISK CODE:SIL-H6 |
| 1.5 | check and clean stripped floor space | • Residual asbestos fibres or dust inhalation — incomplete removal or re-suspension during final clean • Unidentified hazardous material exposure — concealed contaminants not removed in earlier phases • Trip or fall hazard — debris, protrusions, or uneven floor during final inspection | Medium (6) | Engineering Controls: • Stop all work immediately if any suspect asbestos material is visible or suspected during inspection. • Visually check floor surface for residual dust, debris, or material fragments before cleaning starts. • Use wet-sweep method only — spray floor lightly with water before sweeping to prevent dust re-suspension. • Collect all swept debris into sealed plastic bags labelled 'Asbestos Waste' — do not shake out or dry-sweep. • Wipe down all exposed surfaces (walls, beams, ducts) with damp cloths — do not dry-brush or use compressed air. • Photograph final cleaned floor space and collect photographic record before handing over area. • Wear Type 5 protective suit, hood, and organic vapour respirator (per SDS) during entire inspection and clean. Administrative Controls: • Confirm air-monitoring consultant (NATA-accredited or equivalent) engaged and available for clearance testing. • Record final inspection date, time, weather conditions, and inspector name in site diary. • Retain photographic record and clearance certificate in project WHS file for minimum 2 years. | Low (2) | • SUP — Verify access equipment safe, approve defect list, sign off completion • WKR — Inspect per specification, photograph defects, report issues | ⚠️ HOLD POINT 1.5 1. Do not start any subsequent fit-out works until: project WHS coordinator confirms all asbestos waste has been removed from site to approved facility and disposal docket received 🛑 STOP-WORK TRIGGER • Any suspect asbestos material, friable dust, or non-friable fragments are visible on floor or surfaces after cleaning • Wet-sweep method cannot be maintained due to floor drainage or water containment failure • Licensed air-monitoring clearance cannot be obtained within project programme window RISK CODE:SIL-H6 |
| 1.6 | Disconnect electrical, data, and mechanical services | • Electric shock or arc flash — contact with live circuits during isolation work • Uncontrolled pressurised mechanical lines — sudden release of HVAC or plumbing pressure • Incomplete or incorrect isolation — residual energy release after lock-out | High (6) | Engineering Controls: • Obtain single-line diagram or electrical schedule from building owner or facilities manager before isolation work starts. • Licensed electrician to verify all power circuits de-energised at switchboard using multimeter or voltage tester on each circuit before disconnection. • Lock-out each isolated switch or circuit breaker and attach isolation tag with date, time, and licensed electrician name. • Mechanical tradesperson to isolate HVAC pressure lines using ball valves; bleed residual pressure into approved container before disconnection. • Mechanical tradesperson to isolate plumbing water supply at mains valve and drain all lines into approved container before disconnection. • Photograph or video-record each lock-out and isolation point; store record on site for verification. • Prohibit any reconnection or removal of isolation tags except by licensed electrician or mechanical tradesperson who applied them. Administrative Controls: • Brief all site personnel on isolation points and lock-out locations before work starts. • tell building owner or facilities manager of isolation commencement and estimated completion time. | Medium (3) | • SUP — Supervise Disconnect electrical, data, and mechanical • WKR — Perform Disconnect electrical, data, and mechanical per SWMS | ⚠️ HOLD POINT 1.6 1. Do not start disconnection until: single-line diagram or electrical schedule is obtained and verified on site by licensed electrician. 🛑 STOP-WORK TRIGGER • Multimeter or voltage tester indicates voltage on any circuit after switch isolation or lock-out. • Any isolation tag is removed, damaged, or missing from a switch or valve. • Residual pressure cannot be fully released from HVAC or plumbing lines before disconnection. RISK CODE:ELE-H6 |
| 1.7 | Remove suspended ceiling system | • Falling ceiling tiles, grid, or brackets — struck on head or body during dismantling • Collapse of unsupported ceiling grid — sudden drop or instability during removal • Entanglement in suspended brackets or wiring — limb or torso trapped during work | High (6) | Engineering Controls: • Establish exclusion zone below ceiling work area — minimum 3 m radius, barricade all entry points with shade cloth or hard barriers • Install temporary props or temporary ceiling support at 2 m centres before removing ceiling grid — check prop stability and bearing on floor before work starts • Remove ceiling tiles first by hand into labelled collection bins held at shoulder height — do not throw or drop tiles from height • Dismantle ceiling grid in methodical sections — work from centre outward, support each grid section with temporary props or jack until fully released • Check for and tag asbestos-containing materials found during removal — cease work in that section, isolate area, tell licensed asbestos removal contractor before continuing • Collect all removed material into labelled waste streams — segregate asbestos-suspect material into separate skip for disposal to licensed facility • Wear hard hat, hi-viz shirt or vest, and cut-resistant gloves during dismantling — wear organic vapour respirator if dust or disturbed adhesive dust is visible Administrative Controls: • Brief all workers before start: location of exclusion zone, location of temporary props, asbestos identification criteria, stop-work triggers, and location of waste segregation bins • Record all removed material type, location, and destination — document any asbestos-suspect material separately with photos and weight • tell SafeWork NSW if Class A asbestos removal is required — arrange for air monitoring contractor and licensed Class A removal contractor before work recommences | Medium (2) | • SUP — Supervise Remove suspended ceiling system • WKR — Perform Remove suspended ceiling system per SWMS | ⚠️ HOLD POINT 1.7 1. Do not dismantle grid sections until: temporary props are installed at 2 m centres, verified stable on floor, and no evidence of grid sag or instability is visible 🛑 STOP-WORK TRIGGER • Asbestos-containing material is identified or suspected — isolate section, barricade entry, tell SafeWork NSW and licensed asbestos removal contractor before resuming • Ceiling grid shows signs of sag, cracking, or sudden drop during dismantling — move personnel clear, reinforce with additional props, reassess stability before continuing • Any exclusion zone barrier is breached or removed — reestablish barricade and brief all site personnel before work resumes RISK CODE:SIL-H6 |
| 1.8 | Collect and stage demolition waste for skip removal | • Asbestos waste cross-contamination — mixing of licensed asbestos bins with general demolition waste • Manual handling injury — strain or back injury during waste consolidation and bin loading • Struck by shifting waste load — unstable pile during stacking or loading via dock | High (6) | Engineering Controls: • Establish separate asbestos waste staging area minimum 3 m from general demolition waste — marked with warning signs and barricade tape. • Load asbestos waste into licensed disposal bins only — verify bin labels match asbestos waste classification (friable or non-friable) before loading. • Prohibit mixing of asbestos and general waste — if cross-contamination occurs, isolate contaminated general waste and treat as asbestos waste per CoP. • Bag all loose asbestos waste in double-thickness plastic bags before placing in licensed bin — seal bags and label with asbestos warning label. • Conduct visual check of each asbestos waste bin contents before sealing — confirm no general demolition material has entered the bin. • Complete waste manifest for asbestos waste showing bin ID, weight, classification, and licensed transporter details before skip removal. • Use mechanical handling (pallet jack or forklift) for loading skip bins via dock — prohibit manual lifting of loads exceeding 20 kg. • Stack waste piles no higher than 1.5 m and make sure stable base — check pile stability before and after loading each additional load. Administrative Controls: • Confirm SafeWork NSW notification has been lodged and demolition permit issued (if structure >6 m). • Obtain licensed waste transporter details and confirm EPA licence number matches waste manifest. • Record asbestos waste bin ID, weight, classification, and transporter on waste manifest — retain copy for project file. • Provide induction on asbestos waste segregation to all workers before task starts — include no-mix rule and emergency response if spill occurs. | Medium (3) | • SUP — Supervise Collect and stage demolition waste • WKR — Perform Collect and stage demolition waste per SWMS | ⚠️ HOLD POINT 1.8 1. Do not load asbestos waste into skip bins until: licensed asbestos bins are on-site, labelled, and segregation area is marked and barricaded. 2. Do not remove skip bins until: waste manifests are completed, signed, and all asbestos bins sealed and verified as containing only asbestos waste. 🛑 STOP-WORK TRIGGER • Asbestos waste is found mixed with general demolition waste — isolate contaminated loads and treat all as asbestos waste. • Licensed asbestos bins are not available or labelling is incomplete — do not stage asbestos waste without compliant bins. • Waste pile becomes unstable or shifts during loading — cease loading and restack to stable configuration. RISK CODE:SIL-H6 |
| 1.9 | Remove fixed joinery and built-in fittings | • Struck by falling joinery or cabinetry during removal — collapse or unplanned drop • Cut or crush injury — fingers or hands in saws, pry bars, or pinch points • Back or strain injury — manual handling of heavy cupboards or long shelving units | Medium (6) | Engineering Controls: • Secure all cupboards and shelving to wall frame or temporary props before cutting or prying — prevent collapse during partial removal. • Remove contents and strip internal shelves first — reduce weight and mass before structural removal. • Use two workers minimum for units over 30 kg — manual handling control and spotting for dropped items. • Pry from top corner or central stud only — distribute load and prevent sudden collapse or tip. • Keep hands clear of pinch points and saw path — use pry bar or wedge to open gaps, do not use fingers. • Wear cut-resistant gloves, eye protection, and steel-capped boots — protect against splinters, saw blades, and dropped items. • Establish exclusion zone at least 2 m from removal work — prevent struck-by injury to other workers. Administrative Controls: • If non-friable asbestos identified, tell SafeWork NSW and obtain Class B Removal Licence holder approval before removal. • If friable asbestos suspected, stop work and engage Class A licensed remover — air monitoring and containment required. • tell project manager of asbestos findings and removal method confirmation before work resumes. • make sure all workers hold White Card (CPCCWHS1001) and appropriate asbestos removal competency where applicable. | Low (2) | • SUP — Supervise Remove fixed joinery and built-in • WKR — Perform Remove fixed joinery and built-in per SWMS | ⚠️ HOLD POINT 1.9 1. Do not remove fixed joinery until: all contents removed, internal shelves stripped, and joinery stability assessed by removing worker. 🛑 STOP-WORK TRIGGER • Suspected asbestos material is encountered (backing board, adhesive, gasket, or pipe wrap) — do not disturb; contact licensed assessor immediately. • Joinery or cupboard shows signs of collapse, tilting, or cracking during cutting or prying — secure immediately and reassess support. • Concealed utilities (electrical conduit, water pipe, gas line) are exposed during removal — isolate and confirm safe before proceeding. RISK CODE:SIL-H6 |
| 1.10 | Remove site establishment and demobilise | • Premature loss of exclusion control — fencing or signage removed before all staff clear site • Public or unauthorised entry — building not fully secured during demobilisation • Trip or manual handling injury — removal of temporary fixtures or equipment | Medium (6) | Engineering Controls: • Confirm all hazardous materials removal complete and air clearance certificates obtained before removing any exclusion fencing or signage. • Barricade building entry points with plywood or temporary barriers until final lock-up. Do not remove until building handover confirmed. • Remove temporary fencing in reverse order — perimeter first, then exclusion zones — only after all workers and equipment cleared from site. • Remove signage only after fencing removed and exclusion zone fully delineated. Photograph final site state before removal. • Segregate removed equipment and tools into designated removal zones. Use trolleys or mechanical handling for items over 20 kg. • Brief all site personnel on demob sequence before any exclusion control is removed. Confirm final headcount before gate closure. • Conduct final walkthrough with site supervisor to confirm no tools, debris, or hazardous materials left on site. • Secure all building entry doors and windows with locks verified working. Provide owner with key receipt and sign-off documentation. Administrative Controls: • Obtain written confirmation from site supervisor that all workers have cleared site before removing any temporary fencing. • Obtain air clearance certificates and hazmat removal sign-offs and file with site demob record. • Complete site demobilisation checklist with photographs of secured building and cleared site. • Provide owner with keys, lock verification, and photographic handover documentation. • Record final site condition and sign-off in project close-out documentation. | Low (2) | • SUP — Verify exclusion zone complete, barriers intact, access controlled before work starts • WKR — Set up and maintain barriers, stop work if zone breached, report access issues | ⚠️ HOLD POINT 1.10 1. Do not remove any exclusion fencing or signage until: all hazardous materials removal is complete, air clearance certificates are obtained, and final site walkthrough confirms no active work remains 2. Do not hand building back to owner until: all entry points are secured and locked, keys are handed over with documented receipt, and a final photographic record of secure condition is completed 🛑 STOP-WORK TRIGGER • Any unauthorised person enters the exclusion zone during demobilisation or any entry point is found unlocked or unsecured during final checks • Hazardous materials survey reports outstanding findings or air clearance cannot be confirmed before exclusion control removal |

CCVS TABLE

| Task | Critical Control | Who Checks | How Often | What They Look For |
| --- | --- | --- | --- | --- |
| Remove asbestos-containing materials under Class B licence | Dust extraction running and P2 respirator fitted before each grinding cycle | Licensed removalist and safety supervisor | continuous during removal, recorded every 30 minutes | Real-time particle counter readings recorded on site log; negative pressure unit display visible and documented |
| Remove partition walls and structural supports | Dust extraction running and P2 respirator fitted before each grinding cycle | Structural engineer (approval); Site supervisor (daily verification); Workers (pre-removal briefing). | Structural engineer: before removal sequence approval and after major removal milestones. Supervisor: before each shift and after each large wall removal. Air monitoring: continuous during friable asbestos removal. | Engineer sign-off on member identification drawing; supervisor daily log of exclusion zone integrity and structure condition checks; air monitoring certificates (if Class A); photographic record of containment integrity. |
| Remove floor coverings and adhesive backing | Verify wet suppression is active and dust extractor running before each stripping cycle; check visible dust escape at containment boundary. | Supervisor (site lead or asbestos removal licence holder if ACM present) | Continuous during removal work; check every 30 minutes during extended operations | Visual confirmation of damp floor surface, extractor fan running, no dust haze escaping work area; record check-off on daily site log |
| check and clean stripped floor space | Visually check floor for residual dust or material fragments; photograph before and after cleaning. | Supervisor / Licensed asbestos removal operative | Continuous during inspection and cleaning; photographic record at start and completion | Dated photographs, site diary entry, clearance certificate from licensed air-monitoring consultant |
| Disconnect electrical, data, and mechanical services | Licensed electrician uses multimeter to verify zero voltage on each circuit immediately before and after lock-out. | Licensed electrician | Continuous during isolation; each circuit verified once before proceeding to next circuit. | Multimeter reading recorded (zero volts) on site log or isolation record; photograph of meter display at each switch. |
| Remove suspended ceiling system | Dust extraction running and P2 respirator fitted before each grinding cycle | Supervisor | Before work start and continuous during ceiling removal | Barricade photo, entry log, worker sign-off at briefing |
| Collect and stage demolition waste for skip removal | Dust extraction running and P2 respirator fitted before each grinding cycle | Supervisor and Workers | Before sealing each asbestos bin; before each skip removal | Waste manifest signed by supervisor confirming bin contents verified; photograph of sealed bins if required by site protocol |
| Remove fixed joinery and built-in fittings | Dust extraction running and P2 respirator fitted before each grinding cycle | Supervisor and removing workers | Before each cupboard or shelving unit removal | Visual check for friable backing, adhesive tape residue, or gasket; photograph if suspected ACM; licensed assessor sign-off if asbestos confirmed |

RISK RATING MATRIX — Consequence × Likelihood

| Risk Level | Description of Consequence or Impact | Consequence | Unlikely (1) | Possible (2) | Almost Certain (3) |
| --- | --- | --- | --- | --- | --- |
| HIGH | Actual/Potential fatality, disability or irreversible damage. Major structural failure. Off-site environmental discharge not contained, significant long-term environmental harm. | Major (3) | Medium (3) | High (6) | High (9) |
| MEDIUM | Actual/Potential temporary disability, MTI or LTI. Structural failure/damage, >1-day outage. On-site environmental discharge contained, minor remediation, short-term harm. | Moderate (2) | Low (2) | Medium (4) | High (6) |
| LOW | Incident requiring first aid. Environmental discharge immediately contained, minor clean-up, no short-term environmental harm. | Minor (1) | Low (1) | Low (2) | Medium (3) |
| Level | Likelihood / Probability |  |  |  |  |
| Almost Certain (3) | Occurs frequently; >66% chance of occurring |  |  |  |  |
| Possible (2) | Could happen occasionally; >33% but <66% chance of occurring |  |  |  |  |
| Unlikely (1) | May occur only in exceptional circumstances; <33% chance of occurring |  |  |  |  |
| Class / Ranking | Description / Requirements |  |  |  |  |
| High 6, 9 | STOP immediately. Implement controls. Controls recorded on a SWMS. |  |  |  |  |
| Medium 3, 4 | Planned control. Controls recorded on a SWMS. |  |  |  |  |
| Low 1, 2 | Managed via routine procedure. |  |  |  |  |
| Note — WHS Act s.18 “reasonably practicable”: Requires consideration of the likelihood of the risk, degree of harm, knowledge of the hazard, availability and suitability of controls, and cost vs. risk. If you cannot show how that decision was made, the action becomes harder to defend after an incident. |  |  |  |  |  |

| WORKER SWMS INDUCTION SIGN-OFF |  |  |  |
| --- | --- | --- | --- |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |

| WORKER SWMS AMENDMENT SIGN-OFF |  |  |  |
| --- | --- | --- | --- |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |

If work methods, site conditions, or hazards change — stop, complete this table, re-brief all workers, obtain new sign-offs before resuming. Required under WHS Regulation r.300(3).

| SWMS AMENDMENTS |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Under WHS Act s18, “reasonably practicable” requires consideration of likelihood of risk, degree of harm, what the person knows about the hazard, availability and suitability of controls, cost vs risk. If you cannot show how that decision was made, the action becomes harder to defend after an incident. |  |  |  |  |  |  |
| Date | Work Activity / Task | Hazard / Risk | Risk Rating (Pre) | Controls — Hierarchy of Control | Risk Rating (Post) | Responsible |

This SWMS is a living document. It must be reviewed if work methods change, new hazards are identified or following any incident. Supervisor/Project Manager is responsible for ensuring this document remains current. NSW WHS Regulation r.300(3).

| WORKER SWMS AMENDMENT SIGN-OFF |  |  |  |
| --- | --- | --- | --- |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |

If work methods, site conditions, or hazards change — stop, complete this table, re-brief all workers, obtain new sign-offs before resuming. Required under WHS Regulation r.300(3).

| SWMS AMENDMENTS |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Under WHS Act s18, “reasonably practicable” requires consideration of likelihood of risk, degree of harm, what the person knows about the hazard, availability and suitability of controls, cost vs risk. If you cannot show how that decision was made, the action becomes harder to defend after an incident. |  |  |  |  |  |  |
| Date | Work Activity / Task | Hazard / Risk | Risk Rating (Pre) | Controls — Hierarchy of Control | Risk Rating (Post) | Responsible |

This SWMS is a living document. It must be reviewed if work methods change, new hazards are identified or following any incident. Supervisor/Project Manager is responsible for ensuring this document remains current. NSW WHS Regulation r.300(3).