| Robertson's Remedial and Painting Pty Ltd | Document No: RPD-PR-ASB-001 |
| --- | --- |
| ASBESTOS MANAGEMENT PROCEDURE | Version: 1.0 ¦ Date: Feb 2025 ¦ Review: Feb 2027 |
| Approved: Leslie Robertson (Director) | Ed.6 Ref: Checklist 5, 9 ¦ WHS Reg 2017 Ch.8 Pt.8 |

1. Purpose

This procedure sets out how RPD identifies, manages and controls asbestos-containing material (ACM) encountered during remedial, cladding and painting works. It complies with WHS Regulation 2017 Chapter 8 Part 8 and SafeWork NSW Code of Practice: How to Manage and Control Asbestos in the Workplace.

2. Scope

All RPD sites where ACM may be present. Covers Class B (non-friable) asbestos removal by RPD’s licensed workers. Class A (friable) removal is subcontracted to a licensed Class A removalist.

3. Procedure — Before, During, After

| Phase | # | Action | Who | Record |
| --- | --- | --- | --- | --- |
| BEFORE | 1 | Check for existing asbestos register or survey report. If none exists or building is pre-2003, commission a hazardous materials survey by a competent person before work starts. | Project Manager | Survey report |
|  | 2 | Review survey results. Mark ACM locations on site drawings. Determine if ACM will be disturbed by RPD’s scope of works. | Project Manager | Marked drawings |
|  | 3 | If ACM will be disturbed: prepare Asbestos Removal Control Plan. Prepare SWMS specific to asbestos removal task. Notify SafeWork NSW if removing >10m². | Site Supervisor | ARCP, SWMS (Breadcrumb) |
|  | 4 | Establish exclusion zone with barriers, signage and decontamination area. Confirm Class B licence currency for all removal workers. | Site Supervisor | Licence register (Breadcrumb) |
| DURING | 5 | Toolbox talk: brief all workers on ACM locations, no-go zones, SWMS, emergency procedure. All sign on. | Site Supervisor | Toolbox (Breadcrumb) |
|  | 6 | Wet-removal methods. Keep ACM intact where possible. Double-bag waste in 200µm poly, label ‘CAUTION ASBESTOS’. No power tools on ACM unless HEPA-attached. | Removal crew | — |
|  | 7 | Minimum PPE: P2 half-face respirator (P3 for friable), disposable coveralls, safety glasses, gloves, steel-cap boots. Decontaminate on exit from exclusion zone. | All workers | — |
|  | 8 | Air monitoring during removal if required by ARCP. Visual inspection during works — stop if unexpected ACM type found. | Site Supervisor | Air monitoring report |
| AFTER | 9 | Clearance inspection by independent competent person (or licensed assessor for friable). Obtain clearance certificate before other trades re-enter. | Project Manager | Clearance cert |
|  | 10 | Dispose ACM waste via licensed asbestos waste transporter to licensed facility. Retain waste dockets for 40 years. | Site Supervisor | Waste dockets |
|  | 11 | Update site asbestos register. File all records in Breadcrumb project folder. | Project Manager | Updated register |

4. Stop Work Triggers  Suspected friable asbestos found (crumbly, powder)  •  ACM not identified on survey discovered  •  PPE failure or fit issue  •  Exclusion zone breached  •  Air monitoring exceeds 0.01 f/mL.

5. References:  WHS Act 2011  •  WHS Reg 2017 Ch.8 Pt.8  •  SafeWork NSW COP: How to Manage and Control Asbestos  •  SafeWork NSW COP: How to Safely Remove Asbestos  •  RPD WHSMS V7 s.4.23  •  HY Standard: Asbestos rev8

Generated by Gatekeeper WHS Advisor  •  Benchmarked against Hansen Yuncken Asbestos Standard rev8 and SafeWork NSW COPs