| Robertson's Remedial and Painting Pty Ltd | Document No: RPD-PR-GRD-001 |
| --- | --- |
| GROUND WORKS PROCEDURE | Version: 1.0 ¦ Date: Feb 2025 ¦ Review: Feb 2027 |
| Approved: Leslie Robertson (Director) | Ed.6 Ref: Checklist 5, Appendix B ¦ WHS Reg 2017 Ch.8 Pt.4 |

1. Purpose

This procedure sets out how RPD manages ground works including excavation, trenching and ground penetration during remedial construction. It complies with WHS Regulation 2017 Chapter 8 Part 4 and SafeWork NSW Code of Practice: Excavation Work.

2. Scope

All RPD sites where excavation, trenching, ground penetration or footpath/pavement removal is performed. Includes pier holes, service trenches and footing excavations associated with remedial works.

3. Procedure — Before, During, After

| Phase | # | Action | Who | Record |
| --- | --- | --- | --- | --- |
| BEFORE | 1 | Obtain Dial Before You Dig (DBYD) plans for the site. Identify all underground services: electrical, gas, water, sewer, telecommunications, stormwater. | Project Manager | DBYD plans |
|  | 2 | Engage utility locator to physically locate and mark services where excavation depth exceeds 300mm or within 1m of known services. | Project Manager | Locator report |
|  | 3 | Assess soil conditions, groundwater, adjacent structures and traffic loading. Determine shoring/benching requirements for excavations >1.5m deep. | Site Supervisor | Risk assessment (Breadcrumb) |
|  | 4 | Prepare SWMS for excavation task. Include: trench collapse, services strike, falling into excavation, mobile plant near edge, flooding. | Site Supervisor | SWMS (Breadcrumb) |
|  | 5 | Establish barricading around excavation zone. Position spoil at least 1m from excavation edge. | Site Supervisor | Site photos |
| DURING | 6 | Toolbox talk: brief all workers on SWMS, service locations, emergency procedure, no-go zones. All sign on. | Site Supervisor | Toolbox (Breadcrumb) |
|  | 7 | Hand-dig within 500mm of marked services. No mechanical excavation within this zone. Use non-destructive digging (vacuum/hydro) if available. | Excavation crew | — |
|  | 8 | Inspect excavation walls daily and after rain. Install shoring if >1.5m deep or soil conditions unstable. Maintain safe access (ladder within 9m travel). | Site Supervisor | Inspection (Breadcrumb) |
|  | 9 | Minimum PPE: hard hat, steel-cap boots, high-vis, safety glasses, gloves. Hearing protection near plant. | All workers | — |
| AFTER | 10 | Backfill in layers, compact as required. Reinstate surfaces. Remove barricading only when excavation fully backfilled or permanently covered. | Site Supervisor | Inspection (Breadcrumb) |
|  | 11 | File DBYD plans, locator report and all records in Breadcrumb project folder. | Project Manager | Project file |

4. Stop Work Triggers  Unidentified service encountered  •  Gas smell or water ingress  •  Trench wall cracking or movement  •  Excavation exceeds 1.5m without shoring  •  Contaminated soil suspected (odour, discolouration).

5. References:  WHS Act 2011  •  WHS Reg 2017 Ch.8 Pt.4  •  SafeWork NSW COP: Excavation Work  •  RPD WHSMS V7 s.4.18  •  HY Standard: Ground Works rev8

Generated by Gatekeeper WHS Advisor  •  Benchmarked against Hansen Yuncken Ground Works Standard rev8 and SafeWork NSW COP