| Robertson's Remedial and Painting Pty Ltd | Document No: RPD-PR-MPL-001 |
| --- | --- |
| MOBILE PLANT PROCEDURE | Version: 1.0 ¦ Date: Feb 2025 ¦ Review: Feb 2027 |
| Approved: Leslie Robertson (Director) | Ed.6 Ref: Checklist 5, 7 ¦ WHS Reg 2017 Ch.5 Pt.3 |

1. Purpose

This procedure sets out how RPD manages mobile plant and equipment on construction sites to prevent crush, collision and rollover incidents. It complies with WHS Regulation 2017 Chapter 5 Part 3 and SafeWork NSW Code of Practice: Managing the Risks of Plant in the Workplace.

2. Scope

All mobile plant operated on or delivered to RPD sites including: EWPs (boom lifts, scissor lifts), forklifts, excavators, skid steers, truck-mounted cranes, concrete pumps and delivery vehicles. Covers RPD-owned and hired plant.

3. Procedure — Before, During, After

| Phase | # | Action | Who | Record |
| --- | --- | --- | --- | --- |
| BEFORE | 1 | Confirm plant is suitable for site conditions: ground bearing capacity, overhead clearances, proximity to edges/excavations, power lines. | Project Manager | Plant selection record |
|  | 2 | Verify operator holds correct High-Risk Work Licence (e.g. WP for boom, LF for forklift) or competency for non-HRWL plant. Check licence expiry. | Site Supervisor | Licence register (Breadcrumb) |
|  | 3 | Pre-start inspection by operator: fluid levels, tyres/tracks, guards, emergency stops, SWL markings, lights, horn, seatbelt, fire extinguisher. Log defects. | Operator | Pre-start checklist (Breadcrumb) |
|  | 4 | Establish exclusion zones and traffic management: separate pedestrians from plant operating areas. Spotter required for reversing in congested areas. | Site Supervisor | Site plan |
|  | 5 | Prepare SWMS addressing: crush/collision, rollover, falling loads, contact with overhead services, and pedestrian interaction. | Site Supervisor | SWMS (Breadcrumb) |
| DURING | 6 | Toolbox talk: brief all site workers on plant movements, exclusion zones, communication signals, emergency stops. All sign on. | Site Supervisor | Toolbox (Breadcrumb) |
|  | 7 | Operate within manufacturer’s rated capacity. No modification of plant or safety devices. Maintain safe distance from edges (1.5x depth of excavation). | Operator | — |
|  | 8 | Maintain 4m clearance from overhead powerlines (or approach distances per Table of No Go Zones). If closer work needed, isolate or use insulated plant. | Operator / Supervisor | Permit (if req’d) |
|  | 9 | Shut down plant when unattended: lower attachments, engage park brake, remove key. No passengers unless plant designed for them. | Operator | — |
| AFTER | 10 | End-of-day inspection: log any faults. Tag out defective plant — do not operate until repaired. Record in plant register. | Operator | Plant register (Breadcrumb) |
|  | 11 | Scheduled servicing per manufacturer intervals. Maintain service records. Annual or periodic inspection for registered plant. | Project Manager | Service records |

4. Stop Work Triggers  Plant defect affecting safety (brakes, steering, SIS)  •  Operator not licensed/competent  •  Ground conditions unstable  •  Overhead powerlines within approach distance  •  Exclusion zones not maintained  •  Visibility inadequate (fog, dust, lighting).

5. References:  WHS Act 2011  •  WHS Reg 2017 Ch.5 Pt.3  •  SafeWork NSW COP: Managing the Risks of Plant  •  RPD WHSMS V7 s.4.20  •  HY Standard: Mobile Plant rev8

Generated by Gatekeeper WHS Advisor  •  Benchmarked against Hansen Yuncken Mobile Plant Standard rev8 and SafeWork NSW COP