| Robertson's Remedial and Painting Pty Ltd | Document No: RPD-PR-WAH-001 |
| --- | --- |
| WORK AT HEIGHT PROCEDURE | Version: 1.0 ¦ Date: Feb 2025 ¦ Review: Feb 2027 |
| Approved: Leslie Robertson (Director) | Ed.6 Ref: Checklist 5, Appendix B ¦ WHS Reg 2017 Ch.6 Pt.4 |

1. Purpose

This procedure sets out how RPD prevents falls from height — the leading cause of death in Australian construction. It complies with WHS Regulation 2017 Chapter 6 Part 4 and SafeWork NSW Code of Practice: Managing the Risk of Falls at Workplaces.

2. Scope

All work where a person could fall 2m or more, or where a fall from any height could result in injury (e.g. fall into water, onto sharp objects). Covers: facade/balcony remedial work, roof access, scaffold use, EWP use, ladder use, penetration/void protection.

3. Procedure — Before, During, After

| Phase | # | Action | Who | Record |
| --- | --- | --- | --- | --- |
| BEFORE | 1 | Can the task be done from ground level? Hierarchy: 1° Eliminate work at height. 2° Use passive fall prevention (scaffold, guardrails, EWP). 3° Work positioning. 4° Fall arrest. 5° Admin controls. | Project Manager | Risk assessment (Breadcrumb) |
|  | 2 | Select access method per hierarchy. For RPD remedial work: scaffold or EWP preferred over rope access. If rope access required, engage licensed rope access contractor. | Project Manager | Project file |
|  | 3 | Prepare SWMS for the specific height task. Address: fall from edge, fall through surface, falling objects, rescue plan, weather limits (wind, rain, lightning). | Site Supervisor | SWMS (Breadcrumb) |
|  | 4 | Verify competency: EWP operators (HRWL WP/boom, competency/scissor), scaffolders (HRWL SB/SI/SA), harness users (trained in inspection, fit, rescue). | Site Supervisor | Licence register (Breadcrumb) |
|  | 5 | Inspect all fall prevention/arrest equipment: harnesses (in-date, no damage), lanyards, anchor points (rated 15kN or 12kN static), guardrails, netting. | Workers | Pre-use inspection |
| DURING | 6 | Toolbox talk: brief on fall hazards, SWMS, rescue plan, exclusion zones below, weather limits. All workers sign on. | Site Supervisor | Toolbox (Breadcrumb) |
|  | 7 | 100% tie-off when using fall arrest systems — maintain connection at all times. Two points of attachment during transfer. Ensure adequate fall clearance below. | All workers | — |
|  | 8 | Penetrations and voids: cover with secured load-rated covers marked ‘HOLE — DO NOT REMOVE’ or install guardrails around opening. | Site Supervisor | Site photos |
|  | 9 | Exclusion zone below all overhead work: barricade + signage. Secure tools and materials against falling (tethered tools, toe boards, debris netting). | All workers | — |
| AFTER | 10 | Inspect edge protection remains intact on completion. Do not remove guardrails until all height work on that level is complete and area made safe. | Site Supervisor | Inspection (Breadcrumb) |
|  | 11 | Return and inspect fall arrest equipment. Quarantine damaged items. Record inspections. File all records in Breadcrumb. | Site Supervisor | Equipment register |

4. Stop Work Triggers  Guardrails missing or incomplete  •  Harness/lanyard damaged or out-of-date  •  Anchor point not rated or not verified  •  No rescue plan in place  •  Wind >40 km/h (or lower per SWMS)  •  Lightning within 10 km  •  Unprotected penetration/void found.

5. References:  WHS Act 2011  •  WHS Reg 2017 Ch.6 Pt.4  •  SafeWork NSW COP: Managing the Risk of Falls  •  AS/NZS 1891 Harnesses  •  RPD WHSMS V7 s.4.21–4.22  •  HY Standard: Work at Height rev9

Generated by Gatekeeper WHS Advisor  •  Benchmarked against Hansen Yuncken Work at Height Standard rev9 and SafeWork NSW COP