| Robertson's Remedial and Painting Pty Ltd | Document No: RPD-PR-ASB-001 |
| --- | --- |
| ASBESTOS MANAGEMENT PROCEDURE | Version: 1.0 ¦ Date: Feb 2025 ¦ Review: Feb 2027 |
| Approved: Leslie Robertson (Director) | Ed.6 Ref: Checklist 5, 9 ¦ WHS Reg 2017 Ch.8 Pt.8 |

1. Purpose

This procedure sets out how RPD identifies, manages and controls asbestos-containing material (ACM) encountered during remedial, cladding and painting works. It complies with WHS Regulation 2017 Chapter 8 Part 8 and SafeWork NSW Code of Practice: How to Manage and Control Asbestos in the Workplace.

2. Scope

All RPD sites where ACM may be present. Covers Class B (non-friable) asbestos removal by RPD’s licensed workers. Class A (friable) removal is subcontracted to a licensed Class A removalist.

3. Procedure — Before, During, After

| Phase | # | Action | Who | Record |
| --- | --- | --- | --- | --- |
| BEFORE | 1 | Check for existing asbestos register or survey report. If none exists or building is pre-2003, commission a hazardous materials survey by a competent person before work starts. | Project Manager | Survey report |
|  | 2 | Review survey results. Mark ACM locations on site drawings. Determine if ACM will be disturbed by RPD’s scope of works. | Project Manager | Marked drawings |
|  | 3 | If ACM will be disturbed: prepare Asbestos Removal Control Plan. Prepare SWMS specific to asbestos removal task. Notify SafeWork NSW if removing >10m². | Site Supervisor | ARCP, SWMS (Breadcrumb) |
|  | 4 | Establish exclusion zone with barriers, signage and decontamination area. Confirm Class B licence currency for all removal workers. | Site Supervisor | Licence register (Breadcrumb) |
| DURING | 5 | Toolbox talk: brief all workers on ACM locations, no-go zones, SWMS, emergency procedure. All sign on. | Site Supervisor | Toolbox (Breadcrumb) |
|  | 6 | Wet-removal methods. Keep ACM intact where possible. Double-bag waste in 200µm poly, label ‘CAUTION ASBESTOS’. No power tools on ACM unless HEPA-attached. | Removal crew | — |
|  | 7 | Minimum PPE: P2 half-face respirator (P3 for friable), disposable coveralls, safety glasses, gloves, steel-cap boots. Decontaminate on exit from exclusion zone. | All workers | — |
|  | 8 | Air monitoring during removal if required by ARCP. Visual inspection during works — stop if unexpected ACM type found. | Site Supervisor | Air monitoring report |
| AFTER | 9 | Clearance inspection by independent competent person (or licensed assessor for friable). Obtain clearance certificate before other trades re-enter. | Project Manager | Clearance cert |
|  | 10 | Dispose ACM waste via licensed asbestos waste transporter to licensed facility. Retain waste dockets for 40 years. | Site Supervisor | Waste dockets |
|  | 11 | Update site asbestos register. File all records in Breadcrumb project folder. | Project Manager | Updated register |

4. Stop Work Triggers  Suspected friable asbestos found (crumbly, powder)  •  ACM not identified on survey discovered  •  PPE failure or fit issue  •  Exclusion zone breached  •  Air monitoring exceeds 0.01 f/mL.

5. References:  WHS Act 2011  •  WHS Reg 2017 Ch.8 Pt.8  •  SafeWork NSW COP: How to Manage and Control Asbestos  •  SafeWork NSW COP: How to Safely Remove Asbestos  •  RPD WHSMS V7 s.4.23  •  HY Standard: Asbestos rev8

Generated by Gatekeeper WHS Advisor  •  Benchmarked against Hansen Yuncken Asbestos Standard rev8 and SafeWork NSW COPs

| Robertson's Remedial and Painting Pty Ltd | Document No: RPD-PR-GRD-001 |
| --- | --- |
| GROUND WORKS PROCEDURE | Version: 1.0 ¦ Date: Feb 2025 ¦ Review: Feb 2027 |
| Approved: Leslie Robertson (Director) | Ed.6 Ref: Checklist 5, Appendix B ¦ WHS Reg 2017 Ch.8 Pt.4 |

1. Purpose

This procedure sets out how RPD manages ground works including excavation, trenching and ground penetration during remedial construction. It complies with WHS Regulation 2017 Chapter 8 Part 4 and SafeWork NSW Code of Practice: Excavation Work.

2. Scope

All RPD sites where excavation, trenching, ground penetration or footpath/pavement removal is performed. Includes pier holes, service trenches and footing excavations associated with remedial works.

3. Procedure — Before, During, After

| Phase | # | Action | Who | Record |
| --- | --- | --- | --- | --- |
| BEFORE | 1 | Obtain Dial Before You Dig (DBYD) plans for the site. Identify all underground services: electrical, gas, water, sewer, telecommunications, stormwater. | Project Manager | DBYD plans |
|  | 2 | Engage utility locator to physically locate and mark services where excavation depth exceeds 300mm or within 1m of known services. | Project Manager | Locator report |
|  | 3 | Assess soil conditions, groundwater, adjacent structures and traffic loading. Determine shoring/benching requirements for excavations >1.5m deep. | Site Supervisor | Risk assessment (Breadcrumb) |
|  | 4 | Prepare SWMS for excavation task. Include: trench collapse, services strike, falling into excavation, mobile plant near edge, flooding. | Site Supervisor | SWMS (Breadcrumb) |
|  | 5 | Establish barricading around excavation zone. Position spoil at least 1m from excavation edge. | Site Supervisor | Site photos |
| DURING | 6 | Toolbox talk: brief all workers on SWMS, service locations, emergency procedure, no-go zones. All sign on. | Site Supervisor | Toolbox (Breadcrumb) |
|  | 7 | Hand-dig within 500mm of marked services. No mechanical excavation within this zone. Use non-destructive digging (vacuum/hydro) if available. | Excavation crew | — |
|  | 8 | Inspect excavation walls daily and after rain. Install shoring if >1.5m deep or soil conditions unstable. Maintain safe access (ladder within 9m travel). | Site Supervisor | Inspection (Breadcrumb) |
|  | 9 | Minimum PPE: hard hat, steel-cap boots, high-vis, safety glasses, gloves. Hearing protection near plant. | All workers | — |
| AFTER | 10 | Backfill in layers, compact as required. Reinstate surfaces. Remove barricading only when excavation fully backfilled or permanently covered. | Site Supervisor | Inspection (Breadcrumb) |
|  | 11 | File DBYD plans, locator report and all records in Breadcrumb project folder. | Project Manager | Project file |

4. Stop Work Triggers  Unidentified service encountered  •  Gas smell or water ingress  •  Trench wall cracking or movement  •  Excavation exceeds 1.5m without shoring  •  Contaminated soil suspected (odour, discolouration).

5. References:  WHS Act 2011  •  WHS Reg 2017 Ch.8 Pt.4  •  SafeWork NSW COP: Excavation Work  •  RPD WHSMS V7 s.4.18  •  HY Standard: Ground Works rev8

Generated by Gatekeeper WHS Advisor  •  Benchmarked against Hansen Yuncken Ground Works Standard rev8 and SafeWork NSW COP

| Robertson's Remedial and Painting Pty Ltd | Document No: RPD-PR-MPL-001 |
| --- | --- |
| MOBILE PLANT PROCEDURE | Version: 1.0 ¦ Date: Feb 2025 ¦ Review: Feb 2027 |
| Approved: Leslie Robertson (Director) | Ed.6 Ref: Checklist 5, 7 ¦ WHS Reg 2017 Ch.5 Pt.3 |

1. Purpose

This procedure sets out how RPD manages mobile plant and equipment on construction sites to prevent crush, collision and rollover incidents. It complies with WHS Regulation 2017 Chapter 5 Part 3 and SafeWork NSW Code of Practice: Managing the Risks of Plant in the Workplace.

2. Scope

All mobile plant operated on or delivered to RPD sites including: EWPs (boom lifts, scissor lifts), forklifts, excavators, skid steers, truck-mounted cranes, concrete pumps and delivery vehicles. Covers RPD-owned and hired plant.

3. Procedure — Before, During, After

| Phase | # | Action | Who | Record |
| --- | --- | --- | --- | --- |
| BEFORE | 1 | Confirm plant is suitable for site conditions: ground bearing capacity, overhead clearances, proximity to edges/excavations, power lines. | Project Manager | Plant selection record |
|  | 2 | Verify operator holds correct High-Risk Work Licence (e.g. WP for boom, LF for forklift) or competency for non-HRWL plant. Check licence expiry. | Site Supervisor | Licence register (Breadcrumb) |
|  | 3 | Pre-start inspection by operator: fluid levels, tyres/tracks, guards, emergency stops, SWL markings, lights, horn, seatbelt, fire extinguisher. Log defects. | Operator | Pre-start checklist (Breadcrumb) |
|  | 4 | Establish exclusion zones and traffic management: separate pedestrians from plant operating areas. Spotter required for reversing in congested areas. | Site Supervisor | Site plan |
|  | 5 | Prepare SWMS addressing: crush/collision, rollover, falling loads, contact with overhead services, and pedestrian interaction. | Site Supervisor | SWMS (Breadcrumb) |
| DURING | 6 | Toolbox talk: brief all site workers on plant movements, exclusion zones, communication signals, emergency stops. All sign on. | Site Supervisor | Toolbox (Breadcrumb) |
|  | 7 | Operate within manufacturer’s rated capacity. No modification of plant or safety devices. Maintain safe distance from edges (1.5x depth of excavation). | Operator | — |
|  | 8 | Maintain 4m clearance from overhead powerlines (or approach distances per Table of No Go Zones). If closer work needed, isolate or use insulated plant. | Operator / Supervisor | Permit (if req’d) |
|  | 9 | Shut down plant when unattended: lower attachments, engage park brake, remove key. No passengers unless plant designed for them. | Operator | — |
| AFTER | 10 | End-of-day inspection: log any faults. Tag out defective plant — do not operate until repaired. Record in plant register. | Operator | Plant register (Breadcrumb) |
|  | 11 | Scheduled servicing per manufacturer intervals. Maintain service records. Annual or periodic inspection for registered plant. | Project Manager | Service records |

4. Stop Work Triggers  Plant defect affecting safety (brakes, steering, SIS)  •  Operator not licensed/competent  •  Ground conditions unstable  •  Overhead powerlines within approach distance  •  Exclusion zones not maintained  •  Visibility inadequate (fog, dust, lighting).

5. References:  WHS Act 2011  •  WHS Reg 2017 Ch.5 Pt.3  •  SafeWork NSW COP: Managing the Risks of Plant  •  RPD WHSMS V7 s.4.20  •  HY Standard: Mobile Plant rev8

Generated by Gatekeeper WHS Advisor  •  Benchmarked against Hansen Yuncken Mobile Plant Standard rev8 and SafeWork NSW COP

| Robertson's Remedial and Painting Pty Ltd | Document No: RPD-PR-TMP-001 |
| --- | --- |
| TEMPORARY WORKS PROCEDURE | Version: 1.0 ¦ Date: Feb 2025 ¦ Review: Feb 2027 |
| Approved: Leslie Robertson (Director) | Ed.6 Ref: Checklist 5, 7 ¦ WHS Reg 2017 Ch.6 Pt.4 |

1. Purpose

This procedure sets out how RPD designs, installs, maintains and removes temporary works including scaffolding, propping, shoring, formwork, temporary edge protection and temporary structures. It complies with WHS Regulation 2017 and SafeWork NSW Code of Practice: Construction Work.

2. Scope

All temporary structures on RPD sites that provide access, support loads, or provide protection during remedial works. Includes scaffolding (tube & coupler, modular, mobile), propping systems, temporary hoarding and edge protection.

3. Procedure — Before, During, After

| Phase | # | Action | Who | Record |
| --- | --- | --- | --- | --- |
| BEFORE | 1 | Determine temporary works requirement from scope drawings. Identify loads, duration, access requirements and site constraints. | Project Manager | Project file |
|  | 2 | Engage competent designer where required: scaffold >4m needs scaffold design or manufacturer’s specifications. Propping needs engineer certification. | Project Manager | Design / spec |
|  | 3 | Prepare SWMS for erection, use and dismantling of temporary works. Address: collapse, falling objects, manual handling, working at heights during erection. | Site Supervisor | SWMS (Breadcrumb) |
|  | 4 | Verify competency: scaffolders hold correct HRWL (SB — basic, SI — intermediate, SA — advanced). Riggers for structural temp works. | Site Supervisor | Licence register (Breadcrumb) |
| DURING | 5 | Toolbox talk before erection: brief crew on design, sequence, exclusion zone below work area, wind limits, emergency plan. All sign on. | Site Supervisor | Toolbox (Breadcrumb) |
|  | 6 | Erect to design/specification. Scaffold: ensure sole plates on firm ground, all standards plumb, ledgers and bracing complete before platform boards. | Scaffolder | — |
|  | 7 | Handover inspection before use: scaffold tag system (green = safe, red = do not use). Complete scaffold register. Confirm guardrails, toeboards, access ladders. | Site Supervisor | Scaffold register / tag |
|  | 8 | Ongoing inspections: after erection, weekly, after adverse weather, after any modification. Record on Breadcrumb inspection module. Re-tag after each inspection. | Site Supervisor | Inspection (Breadcrumb) |
|  | 9 | No modification by unqualified persons. No loading beyond design capacity. No removal of guardrails, planks or bracing without re-assessment. | All workers | — |
| AFTER | 10 | Dismantle in reverse order of erection. Exclusion zone below. Inspect components for damage — quarantine defective items. | Scaffolder | Scaffold register |
|  | 11 | File design, inspection records and handover certificates in Breadcrumb project folder. | Project Manager | Project file |

4. Stop Work Triggers  Temporary works not erected to design  •  Missing guardrails, toeboards or bracing  •  Red scaffold tag or no tag  •  Wind exceeds design limit  •  Ground settlement or movement at base  •  Unauthorised modification discovered.

5. References:  WHS Act 2011  •  WHS Reg 2017 Ch.6 Pt.4  •  SafeWork NSW COP: Construction Work  •  AS/NZS 1576 Scaffolding  •  AS 3610 Formwork  •  HY Standard: Temporary Works Rev9

Generated by Gatekeeper WHS Advisor  •  Benchmarked against Hansen Yuncken Temporary Works Standard Rev9 and AS/NZS 1576

| Robertson's Remedial and Painting Pty Ltd | Document No: RPD-PR-WAH-001 |
| --- | --- |
| WORK AT HEIGHT PROCEDURE | Version: 1.0 ¦ Date: Feb 2025 ¦ Review: Feb 2027 |
| Approved: Leslie Robertson (Director) | Ed.6 Ref: Checklist 5, Appendix B ¦ WHS Reg 2017 Ch.6 Pt.4 |

1. Purpose

This procedure sets out how RPD prevents falls from height — the leading cause of death in Australian construction. It complies with WHS Regulation 2017 Chapter 6 Part 4 and SafeWork NSW Code of Practice: Managing the Risk of Falls at Workplaces.

2. Scope

All work where a person could fall 2m or more, or where a fall from any height could result in injury (e.g. fall into water, onto sharp objects). Covers: facade/balcony remedial work, roof access, scaffold use, EWP use, ladder use, penetration/void protection.

3. Procedure — Before, During, After

| Phase | # | Action | Who | Record |
| --- | --- | --- | --- | --- |
| BEFORE | 1 | Can the task be done from ground level? Hierarchy: 1° Eliminate work at height. 2° Use passive fall prevention (scaffold, guardrails, EWP). 3° Work positioning. 4° Fall arrest. 5° Admin controls. | Project Manager | Risk assessment (Breadcrumb) |
|  | 2 | Select access method per hierarchy. For RPD remedial work: scaffold or EWP preferred over rope access. If rope access required, engage licensed rope access contractor. | Project Manager | Project file |
|  | 3 | Prepare SWMS for the specific height task. Address: fall from edge, fall through surface, falling objects, rescue plan, weather limits (wind, rain, lightning). | Site Supervisor | SWMS (Breadcrumb) |
|  | 4 | Verify competency: EWP operators (HRWL WP/boom, competency/scissor), scaffolders (HRWL SB/SI/SA), harness users (trained in inspection, fit, rescue). | Site Supervisor | Licence register (Breadcrumb) |
|  | 5 | Inspect all fall prevention/arrest equipment: harnesses (in-date, no damage), lanyards, anchor points (rated 15kN or 12kN static), guardrails, netting. | Workers | Pre-use inspection |
| DURING | 6 | Toolbox talk: brief on fall hazards, SWMS, rescue plan, exclusion zones below, weather limits. All workers sign on. | Site Supervisor | Toolbox (Breadcrumb) |
|  | 7 | 100% tie-off when using fall arrest systems — maintain connection at all times. Two points of attachment during transfer. Ensure adequate fall clearance below. | All workers | — |
|  | 8 | Penetrations and voids: cover with secured load-rated covers marked ‘HOLE — DO NOT REMOVE’ or install guardrails around opening. | Site Supervisor | Site photos |
|  | 9 | Exclusion zone below all overhead work: barricade + signage. Secure tools and materials against falling (tethered tools, toe boards, debris netting). | All workers | — |
| AFTER | 10 | Inspect edge protection remains intact on completion. Do not remove guardrails until all height work on that level is complete and area made safe. | Site Supervisor | Inspection (Breadcrumb) |
|  | 11 | Return and inspect fall arrest equipment. Quarantine damaged items. Record inspections. File all records in Breadcrumb. | Site Supervisor | Equipment register |

4. Stop Work Triggers  Guardrails missing or incomplete  •  Harness/lanyard damaged or out-of-date  •  Anchor point not rated or not verified  •  No rescue plan in place  •  Wind >40 km/h (or lower per SWMS)  •  Lightning within 10 km  •  Unprotected penetration/void found.

5. References:  WHS Act 2011  •  WHS Reg 2017 Ch.6 Pt.4  •  SafeWork NSW COP: Managing the Risk of Falls  •  AS/NZS 1891 Harnesses  •  RPD WHSMS V7 s.4.21–4.22  •  HY Standard: Work at Height rev9

Generated by Gatekeeper WHS Advisor  •  Benchmarked against Hansen Yuncken Work at Height Standard rev9 and SafeWork NSW COP

| Robertson's Remedial and Painting Pty Ltd | Document No: RPD-PR-SVC-001 |
| --- | --- |
| UNDERGROUND AND OVERHEAD SERVICES PROCEDURE | Version: 1.0 ¦ Date: Feb 2025 ¦ Review: Feb 2027 |
| Approved: Leslie Robertson (Director) | Ed.6 Ref: Checklist 5, Appendix B ¦ WHS Reg 2017 Ch.8 Pt.4, Ch.4 Pt.4.7 |

1. Purpose

This procedure sets out how RPD identifies and manages risks from underground and overhead services (electrical, gas, water, sewer, telecommunications) during construction works. It complies with WHS Regulation 2017 and SafeWork NSW Code of Practice: Managing Electrical Risks in the Workplace.

2. Scope

All RPD sites where work is performed near underground services (excavation, penetration, drilling, cutting) or overhead services (powerlines, comms cables). Applies to all workers including subcontractors operating plant.

3. Procedure — Before, During, After

| Phase | # | Action | Who | Record |
| --- | --- | --- | --- | --- |
| BEFORE | 1 | Obtain Dial Before You Dig (DBYD) service plans for the site. Review plans and mark known service routes on site drawings. | Project Manager | DBYD plans |
|  | 2 | Engage licensed utility locator to physically locate, mark and verify services. Use electromagnetic locator + GPR where possible. Mark with paint/flags on ground. | Project Manager | Locator report |
|  | 3 | Identify overhead services: powerlines, comms cables. Determine voltage and No Go Zone distances per Table of No Go Zones (e.g. <1kV = 1m, 1–132kV = 3m, 132–330kV = 6m). | Site Supervisor | Site plan |
|  | 4 | If work within No Go Zone required: request supply authority isolation, install tiger tails/insulated covers, or use spotter + signage. Obtain written approval. | Project Manager | Isolation cert / permit |
|  | 5 | Prepare SWMS addressing: electrocution, gas leak/explosion, service strike flooding, contact with overhead lines by plant/materials. | Site Supervisor | SWMS (Breadcrumb) |
| DURING | 6 | Toolbox talk: brief all workers on service locations, No Go Zones, emergency procedure (gas leak evacuation, electrical contact — stay in cab). All sign on. | Site Supervisor | Toolbox (Breadcrumb) |
|  | 7 | Hand-dig within 500mm of marked underground services. Pot-hole to expose service before mechanical excavation in vicinity. Use non-destructive digging where available. | Excavation crew | — |
|  | 8 | Overhead services: maintain clearances at all times. Height limiters on EWPs/cranes where overhead risk exists. Spotter for plant movements near lines. | Operator / Spotter | — |
|  | 9 | Drilling/coring into walls, floors, ceilings: scan with cable/pipe detector before every penetration. Mark safe drill zones. Assume live until proven otherwise. | All workers | Scan log |
| AFTER | 10 | Backfill around exposed services with appropriate bedding material. Marker tape 300mm above underground services. Reinstate service covers/lids. | Site Supervisor | Inspection (Breadcrumb) |
|  | 11 | Update site drawings with any newly discovered or relocated services. File all records in Breadcrumb project folder. | Project Manager | Updated drawings |

4. Stop Work Triggers  Unidentified service encountered  •  Gas smell detected  •  Water/sewage ingress from unknown source  •  Plant or materials within No Go Zone of powerlines  •  Cable detector indicates live service in drill path  •  Service isolation not confirmed in writing.

5. References:  WHS Act 2011  •  WHS Reg 2017 Ch.8 Pt.4, Ch.4 Pt.4.7  •  SafeWork NSW COP: Managing Electrical Risks  •  SafeWork NSW COP: Excavation Work  •  RPD WHSMS V7 s.4.17  •  HY Standard: Underground and Overhead Services rev8

Generated by Gatekeeper WHS Advisor  •  Benchmarked against Hansen Yuncken Underground and Overhead Services Standard rev8 and SafeWork NSW COPs