George St Facade

| PCBU: | [Insert PCBU here] | Workplace location: |  |
| --- | --- | --- | --- |
| Works Manager: |  | Date SWMS provided to PC: | 31/03/2026 |
| Work activity: | George St Facade | Principal Contractor (PC): | [Insert PCBU here] |
| High Risk Construction Work (HRCW): ☑ Risk of a person falling more than 2 metres ☐ Likely to involve disturbing asbestos ☐ Work in or near a shaft or trench deeper than 1.5 m or a tunnel ☐ Work on or near chemical, fuel or refrigerant lines ☐ Tilt-up or precast concrete elements ☐ Work on a telecommunication tower ☐ Temporary load-bearing support for structural alterations or repairs ☐ Use of explosives ☐ Work on or near energised electrical installations or services ☐ Work on, in or adjacent to a road, railway, shipping lane or other traffic corridor in use by traffic other than pedestrians ☐ Demolition of load-bearing structure ☐ Work in or near a confined space ☐ Work on or near pressurised gas mains or piping ☐ Work in an area that may have a contaminated or flammable atmosphere ☑ Work in an area with movement of powered mobile plant ☐ Work in areas with artificial extremes of temperature ☐ Work in or near water or other liquid that involves a risk of drowning ☐ Diving work |  |  |  |
| Person responsible for ensuring compliance with SWMS: | [Insert Supervisor name here] | Date SWMS received: | 31/03/2026 |
| What measures are in place to ensure compliance with the SWMS? | Toolbox meetings, SWMS sign off, job observations and supervision review. If issues with the SWMS or new hazards are identified, the supervisor must be notified. When changes are made to SWMS, it will be communicated to all workers. |  |  |
| Person responsible for reviewing SWMS control measures: |  | Date SWMS received by reviewer: | 31/03/2026 |
| How will the SWMS control measures be reviewed? | The control measures implemented will be reviewed and if necessary, revised annually or if work methods change, the control measures are not effective in controlling the risk, a new hazard/risk is identified or following an incident. The SWMS will be reviewed in consultation with workers and/or others who may be affected by the SWMS. Any changes to the SWMS will be communicated with workers at induction, daily pre-starts and toolbox talks. |  |  |
| Reviewer’s signature: |  | Review date: | 31/03/2026 |
| This SWMS must be kept and be available for inspection until the high-risk construction work to which this SWMS relates is completed. If the SWMS is revised, all versions should be kept. If a notifiable incident occurs in relation to the high-risk construction work in this SWMS, the SWMS must be kept for at least 2 years from the date of the notifiable incident. |  |  |  |

PRE-REQUISITES TABLE

| Must be in place before starting work |  |  |  |
| --- | --- | --- | --- |
| PPE — All Persons on Site | • steel-capped footwear • hi-viz shirt or vest • eye protection • hearing protection (>85dB) • chemical-resistant gloves (nitrile or task-appropriate) • Hard hat where overhead risk present | Additional PPE | • Fall prevention hierarchy applied: eliminate > isolate > minimise • Guardrails preferred. Fall restraint before fall arrest • Rescue plan documented for all harness work • Working at Heights licence/training verified before elevated work |
| Licences / Qualifications | • Competent person — weekly inspection and after adverse weather • Structural engineer — where scaffold exceeds 15m or non-standard loading • Rescue plan — documented before work above 2m begins • Rescue equipment — on site and ready for immediate deployment before any elevated work starts • Occupant notification — 48 hours minimum before noisy or dusty work • Exclusion zone — clearly marked, prevents public access to work area | Permits & Approvals | • Scaffold erection permit — signed by licenced scaffolder before use • Handover certificate — before any worker uses completed scaffold • Working at heights permit — signed before each elevated task • Noise management plan — required where work affects occupants • Dust containment plan — hoarding or plastic sheeting before work starts • Silica dust management plan — documented before cutting or grinding |
| Plant & Equipment | • Scaffolding • Angle grinder • HEPA vacuum | Hazardous Substances | • Respirable crystalline silica (RCS) — SDS on site — WES 0.05 mg/m³ TWA — P2 minimum • Epoxy primer — SDS on site — flammable liquid |
| Consultation | • Workers consulted during SWMS development • Site-specific induction completed before work commences • Toolbox talk conducted at start of each shift | Legislative Basis | • WHS Act 2011 (NSW) • WHS Regulation 2017 (NSW) • SafeWork NSW Codes of Practice • WHS Reg 2017 Part 4.4 — fall prevention hierarchy applies to all fall risk |

Brief description: George St Facade

STEP-BY-STEP SAFE WORK METHOD TABLE

| Step | Task | Hazard | Risk (Pre) | Controls | Risk (Post) | Responsibility | CCVS Code |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1.1 | Establish site, scaffold, and occupant interface plan | • Fall from height — worker slip or trip during scaffold erection on uneven or wet ground • Struck by scaffold components — dropped or misaligned members during erection • Collapse or instability of incomplete scaffold — load applied before bracing complete • Struck by vehicle — site traffic in proximity to scaffold footprint during erection • Electrical contact — scaffold erected near or touching overhead powerlines | High (9) | Engineering Controls: • Confirm scaffold design and load calculations signed by engineer before erection starts. • Mark and exclude all areas below working edge with 100% compliant barriers — minimum 1.2 m height or full netting. • Barricade vehicle access within 5 m of scaffold footprint — traffic controller present during erection. • Check overhead powerlines clearance minimum 3 m horizontal distance — de-energise or isolate if closer. • check ground bearing surface — compact soil, level concrete, or approved bearing plates. Stop work if soft, sloping, or contaminated. • Erect bracing and cross-bracing in strict design sequence — no work at height until lower sections fully braced. • Secure all scaffold connections using approved couplers — no bolt substitution or temporary lashing. • Install guardrails and mid-rails to all working edges — 1.1 m top rail, 0.5 m mid-rail, maximum 0.5 m gap to edge. • Notify building occupants / body corporate / strata manager of work schedule, access restrictions, and expected noise/dust at least 48 hours before work starts Administrative Controls: • Scaffold erection permit — signed by licenced scaffolder before erection starts. • Handover certificate — signed by licenced scaffolder before first worker access. • Working at heights permit — signed by supervisor before any elevated work. • Noise management plan — delivered to building management and occupants before site start. • Site induction register — completed and signed by all workers before site entry. • Ground bearing assessment — documented by supervising scaffolder before erection. • Occupant notification letter — delivered minimum 5 working days before erection; emergency contact number provided. • Close and barricade balconies directly below active work zone before elevated work starts each day • Notify affected residents before any noise/dust generating work on their level or adjacent levels • Residents responsible for clearing personal items, furniture, potted plants, and non-common fixtures from balcony and work areas before each phase — area left/excluded if not cleared • Temporary access to neighbouring properties required — confirm access arrangements before commencing • Vegetation trimmed away from surfaces designated for painting or remedial works prior to commencement — area excluded if vegetation impedes access • On-site parking or provision of parking permits required for duration of works • Final inspection and agreed scope of works to be confirmed before commencement | Medium (6) | • SUP — Verify exclusion zone complete, barriers intact, access controlled before work starts • WKR — Set up and maintain barriers, stop work if zone breached, report access issues | ⚠️ HOLD POINT 1.1 1. Do not erect scaffold above base height until: ground bearing condition checked and approved by supervising scaffolder. 2. Do not allow worker access to completed scaffold until: handover certificate signed by licenced scaffolder and all bracing visible complete. 🛑 STOP-WORK TRIGGER • Ground bearing subsides, shifts, or differs from design — re-check and re-approve before resuming. • Overhead powerline clearance cannot be maintained — contact network operator and de-energise or isolate before proceeding. • Vehicle or pedestrian breaches exclusion zone during erection — re-establish barriers and traffic control before resuming. • Occupant balcony below work zone is not closed and barricaded RISK CODE:WAH-H6 |
| 1.2 | Remove existing render, failed sealants, and loose concrete spall | • Fall from height — worker loss of balance or slip on wet or dusty scaffold platform • Struck by falling debris — loose render or spall fragments ejected downward toward occupied-building exclusion zone or personnel below • Noise-induced hearing damage — mechanical removal tool use without adequate hearing protection (noise >85 dB) • Eye injury — fine concrete or render particles striking unprotected eyes during high-energy removal | High (9) | Engineering Controls: • Suppress dust at source using wet cutting or grinder attachment with integrated water feed — no dry grinding, angle grinder fitted with wet suppression shroud only. • Provide local dust extraction — handheld grinder connected to HEPA-filter dust extractor with minimum 150 L/min airflow, positioned to capture dust at tool point. • Erect dust containment barrier around work area — clear plastic sheeting sealed to scaffold to contain debris and prevent fallout into occupied zone below. • Verify SDS for render composition on site before work starts — confirm asbestos status and silica content with site supervisor. • Rotate workers on high-exposure tasks (grinding, cutting, spalling removal) — no single worker exceeds 2 hours continuous mechanical removal per shift. • Barricade exclusion zone below work area — sign and erect physical barrier to prevent unauthorised entry; verify zone is clear before each tool start. • Check scaffold platform before each shift — confirm handrails intact, platform clear of debris and pooled water, no trip hazards or loose planking. • Maintain three-point contact during access and egress — no running, jumping, or working on wet or dusty surfaces; use taglines for tool transfer. Administrative Controls: • Obtain signed scaffold erection permit from licenced scaffolder before any worker access. • Obtain signed working-at-heights permit before each elevated task; display on site. • Obtain noise management plan from project manager and distribute to all workers; include measures for nearby occupants. • Verify White Card (CPCCWHS1001) induction current for all workers before site entry. • Verify scaffolding licence (class appropriate to type and height) held by erecting scaffolder; record on site. • Verify working-at-heights competency (RIIOHS204E or equivalent) held by all workers using scaffold; record current currency. • Obtain SDS for render product and any additives on point of use; keep in weather-proof container at site. • Close and barricade balconies directly below active work zone before elevated work starts each day • Notify affected residents before any noise/dust generating work on their level or adjacent levels | Medium (6) | • SUP — Supervise repairs, check scaffold safety, approve debris controls, sign hold points • WKR — Perform repairs per SWMS, wear PPE, use dust control, report defects | ⚠️ HOLD POINT 1.2 1. Do not start mechanical removal until: Site supervisor confirms SDS review complete, asbestos status confirmed clear, and dust extraction system tested and operational. 2. Do not allow first worker onto completed scaffold until: Licenced scaffolder provides signed handover certificate and all harness anchor points are tagged and load-tested. 🛑 STOP-WORK TRIGGER • Dust suppression system fails, water feed stops, or grinder operates dry for >1 minute. • Debris escapes containment barrier, falls outside exclusion zone, or strikes building below. • Worker slips, loses balance, or loses three-point contact on scaffold platform; check cause before resuming. • Occupant balcony below work zone is not closed and barricaded RISK CODE:SIL-H6 |
| 1.3 | Expose concrete substrate and assess damage | • Inhalation of dust — vacuum extraction failure or inadequate dust suppression during loose material removal • Fall from height — worker overreaching or loss of balance during close-proximity assessment of cracks and spalling • Struck by falling tools or instruments — dropped measuring device, camera, or pencil from scaffold platform • Asbestos disturbance — latent risk if sealants, adhesives, or coatings contain asbestos (unconfirmed but common in older render systems) | High (6) | Engineering Controls: • Operate HEPA dust extractor continuously during loose material removal — position extraction hose within 300mm of work zone. • Wet-suppress all loose material before removal — spray with water mist or damp cloth to suppress airborne dust. • Check HEPA filter condition and airflow before each shift — replace if bypass or loss of suction occurs. • Prohibit dry grinding, chipping, or abrasive removal — all dust-generating work must use wet suppression or extraction. • Secure all hand tools and instruments (measuring tape, pencil, camera) to scaffold rail via lanyard — no unsecured items on platform. • Position worker harness attachment at scaffold frame — maintain connection whenever reaching beyond arm's length to assess cracks or spalling. • Barricade working perimeter 2m below scaffold — restrict access to exclusion zone; post 'Falling Objects' signage on scaffold. • Document all asbestos-suspect materials (sealants, adhesives, coatings) with photographs before disturbance — flag for lab analysis if uncertain of age or composition. Administrative Controls: • Verify scaffold erection permit and handover certificate before any worker accesses platform. • Confirm working at heights permit signed and current competency verified (RIIOHS204E or equivalent). • Obtain SDS for any chemical sealants, adhesives, or repair primers before exposure assessment commences. • tell building occupants of dust suppression work — provide warning if extraction noise or vibration affects occupied spaces. • Record vacuum filter change, extraction airflow test, and asbestos-suspect material flags in site diary with date and initials. | Medium (4) | • SUP — Supervise repairs, check scaffold safety, approve debris controls, sign hold points • WKR — Perform repairs per SWMS, wear PPE, use dust control, report defects | ⚠️ HOLD POINT 1.3 1. Do not expose substrate using power tools until: HEPA dust extractor tested, airflow verified, and wet suppression system operational. 2. Do not document or photograph asbestos-suspect sealants or coatings until: material sample collected and flagged for lab analysis confirmation. 🛑 STOP-WORK TRIGGER • HEPA vacuum suction loss or filter bypass detected — do not resume until filter replaced and airflow restored. • Worker loses harness connection or steps beyond barricade — do not resume until connection reattached and perimeter re-secured. • Unknown friable material, discoloured render, or suspected asbestos coating is exposed — do not proceed until lab confirmation received. RISK CODE:SIL-M4 |
| 1.4 | Execute crack stitching and concrete repair | • Fall from height — worker loss of balance or overreach while installing helical stitches at high elevation • Struck by hand tools or materials — dropped stitching anchor, wrench, or repair mortar container during application • Chemical exposure — inhalation of fumes or skin contact with uncured concrete repair mortar or adhesive • Inadequate curing — premature loading or exposure to rain before cure-time compliance, resulting in repair failure | High (9) | Engineering Controls: • Wear full body harness tethered to certified anchor point before starting stitch installation at height. • Install edge protection or guardrail around work perimeter — minimum 1.1 m high — before starting work. • Establish exclusion zone directly below work area — barricade and delineate with caution tape — keep public and workers out. • Secure all hand tools and materials in tool belt or lanyard — no loose items on scaffold or at height. • Review SDS for concrete repair mortar and epoxy resin anchors on point of use — confirm ventilation and skin contact PPE match product requirements. • Ventilate work area continuously — open-side work or mechanical fan — during application and initial cure window. • Apply repair mortar only to dry, clean substrate — remove loose concrete, dust, and contaminants with wire brush and vacuum before application. • Apply mortar in weather window only — no rain forecast for 24 hours post-application — confirm with site supervisor before starting. Administrative Controls: • Obtain and display scaffold erection permit and handover certificate before any work at height starts. • Obtain and display working at heights permit signed by supervisor before each elevated task. • Brief all workers on fall arrest and rescue procedure at start of shift. • Record completion date and cure-time start on repair schedule — attach photographic evidence of completed area. • Conduct final inspection after cure time elapsed — check repair adhesion, surface finish, and cure hardness before sign-off. • tell building occupants of noise and odour during application — provide warning notice at site entry. • Close and barricade balconies directly below active work zone before elevated work starts each day • Notify affected residents before any noise/dust generating work on their level or adjacent levels | Medium (6) | • SUP — Supervise repairs, check scaffold safety, approve debris controls, sign hold points • WKR — Perform repairs per SWMS, wear PPE, use dust control, report defects | ⚠️ HOLD POINT 1.4 1. Do not install helical stitches or apply repair mortar until: scaffold erected, inspected, and handover certificate signed by licenced scaffolder. 2. Do not apply repair mortar until: substrate dry, cleaned, approved by supervisor, and weather window (no rain for 24 hours) confirmed. 🛑 STOP-WORK TRIGGER • Harness anchor point fails, edge protection is damaged or incomplete, or worker loses balance or shows signs of overreach. • Rain begins or forecast changes to rain within 24-hour cure window — halt application and cover wet areas with plastic sheeting. • SDS requirements for ventilation or PPE cannot be met, or chemical exposure symptoms (dizziness, skin irritation) occur. • Occupant balcony below work zone is not closed and barricaded RISK CODE:SIL-H6 |
| 1.5 | check facade repairs and coating finish | • Fall from height — worker loss of balance or slip during walk-over inspection on wet or slippery platform • Struck by falling debris or tools — dropped inspection equipment or scaffold components during close-proximity assessment • Weather exposure — prolonged inspection in high wind, rain, or cold conditions causing loss of grip or concentration • Incomplete or defective repair — undetected coating defects, colour mismatches, or stitching inadequacy requiring rework | High (9) | Engineering Controls: • Confirm coating substrate is dry and grip tested before inspection platform walk-over commences. • Establish 1.5 m exclusion zone below inspection platform. Post signage and barricade zone with safety netting or catch net. • Conduct platform surface condition check before each shift — test foot grip, identify algae, moss, or wet patches, remove slippery debris. • Wear non-slip footwear with verified grip rating (AS/NZS 2210.3) — test grip on wet platform before starting. • Secure inspection torch, magnifying glass, and sample colour swatches in tool belt — never carry loose in hands. • Establish weather stop rule: suspend inspection if wind speed exceeds 40 km/h, rain reduces visibility below 10 m, or platform becomes visibly wet. • Brief all workers on defect identification criteria before inspection — photograph reference samples of acceptable colour match, texture consistency, crack stitch visibility. • check in pairs — one worker on platform, one spotter on adjacent scaffold bay. Spotter maintains visual contact and confirms platform stability. Administrative Controls: • Verify all inspectors hold current White Card (CPCCWHS1001) and Working at Heights competency (RIIOHS204E or equivalent). • Confirm scaffold erection permit issued by licenced scaffolder before site entry. • Obtain handover certificate from scaffolder before any worker accesses platform. • tell building occupants of inspection schedule — coordinate noise, vibration, and access restrictions. • Provide site-specific induction covering platform access, defect identification criteria, and emergency response procedure. • Record inspection findings on site log — defect location, type, remedial action required, sign-off by inspector and approver. • Brief spotter on communication protocol — hand signals or radio contact always during platform work. • Close and barricade balconies directly below active work zone before elevated work starts each day • Notify affected residents before any noise/dust generating work on their level or adjacent levels | Medium (6) | • SUP — Verify scaffold and fall arrest systems, enforce exclusion zone, check SDS • WKR — Use harness and lanyard, wear respiratory protection, report faults | ⚠️ HOLD POINT 1.5 1. Do not start inspection until: scaffold erection permit signed by licenced scaffolder, handover certificate provided, and platform stability confirmed by independent check. 2. Do not proceed with remedial work identification until: all defects photographed, documented, and approved by coating consultant or project manager. 🛑 STOP-WORK TRIGGER • Platform surface grip fails, becomes wet, or worker reports loss of balance or near-slip. • Wind speed exceeds 40 km/h, visibility drops below 10 m, or rain renders platform surface unsafe. • Scaffold shows movement, sway, or structural concern — isolate and tell scaffolder immediately. • Occupant balcony below work zone is not closed and barricaded |
| 1.6 | Confirm scaffold safety, load capacity, and edge protection | • Fall from height — guardrail or toeboard failure not detected before use • Overload failure — platform load capacity breached by personnel, materials, or equipment • Inadequate handholds — worker loss of grip or balance on slippery or absent handholds • Wind-induced collapse — scaffold not adequately braced or stability compromised by wind gusts >20 knots | High (9) | Engineering Controls: • Obtain and review scaffold erection permit and handover certificate signed by licenced scaffolder before any worker accesses platform. • Check all guardrails for damage, loose fixings, or gaps; measure height from platform to top rail — must be 950–1150 mm. • Check toeboard height minimum 100 mm and check for splits, loose fixings, or missing sections along full platform length. • Verify handholds are continuous, secure, and free from grease or contamination; confirm grip diameter 25–40 mm for comfortable grip. • Confirm platform load capacity label is visible and legible; work out total load (personnel + materials + equipment) and verify does not exceed rated capacity. • Conduct wind assessment — stop work if sustained wind exceeds 20 knots or gusts exceed 25 knots; confirm bracing and guy-ropes are taut and undamaged. • Confirm scaffold is tagged 'SAFE TO USE' by licenced scaffolder within last 7 days; check tag date and scaffolder name against permit. • Brief all workers on platform load limit, exclusion zones below scaffold, and fall arrest requirements before work commences. Administrative Controls: • Maintain register of scaffold inspection sign-offs — record date, time, inspector name, and any defects identified. • tell site supervisor and licenced scaffolder immediately if any defect is found; do not permit workaround repairs — only licenced scaffolder may remediate. • Repeat pre-work scaffold check daily and after any weather event, high wind, or vibration-generating work on adjacent floors. • Close and barricade balconies directly below active work zone before elevated work starts each day • Notify affected residents before any noise/dust generating work on their level or adjacent levels | Medium (6) | • SUP — Verify scaffold or EWP certification, supervise erection, sign off load checks • WKR — Follow erection plan, wear harness at height, report defects | ⚠️ HOLD POINT 1.6 1. Do not allow any worker onto scaffold platform until: licenced scaffolder has signed handover certificate and 'SAFE TO USE' tag is current and visible. 2. Do not position materials or equipment on platform until: load calculation is confirmed and total load is verified below rated capacity. 🛑 STOP-WORK TRIGGER • Guardrail height is below 950 mm, toeboard is missing or damaged, or handholds are not continuous. • Wind speed exceeds 20 knots sustained or gusts exceed 25 knots. • Scaffold 'SAFE TO USE' tag is absent, outdated, or platform load is approached or exceeded. • Occupant balcony below work zone is not closed and barricaded RISK CODE:WAH-H6 |
| 1.7 | Prime and prepare repaired substrate | • Inhalation of primer vapours — inadequate ventilation or respiratory protection during primer application • Fall from height — slippery scaffold platform from wet primer drips or splashes • Chemical skin contact — uncured primer contact causing irritation or sensitisation • Fire or explosion hazard — primer vapour accumulation in low-wind conditions or near ignition source | High (6) | Engineering Controls: • Verify SDS on point of use before primer application starts. Check Section 8 (exposure controls) for organic vapour respirator requirement and ventilation threshold. • Establish continuous mechanical ventilation or natural airflow (minimum 0.5 m/s cross-breeze) across work area during primer application and for full cure window stated on SDS. • Wear organic vapour respirator (APF 10, per SDS Section 8) if ventilation cannot be confirmed or wind drops below minimum threshold during application. • Restrict access to scaffold platform during primer application. Post 'Wet Primer — Do Not Enter' sign at access point until surface is touch-dry (time per SDS). • Wipe or sweep wet primer drips from scaffold platform edge and walkways immediately after application using lint-free cloth or damp cloth — no dry sweeping. • Allow full cure time stated on SDS before light sanding or damp-wiping substrate. Record cure start time and sign-off time on job sheet. • Use damp cloth or vacuum-equipped sander (HEPA filter, silica dust suppression) for residual dust removal. Never dry-sand cured primer coat. • Wear chemical-resistant gloves (nitrile minimum; check SDS for upgrade to butyl or neoprene if specified) during primer application and cleanup. Administrative Controls: • tell site induction co-ordinator and site supervisor of primer application schedule 24 hours before work. Include ventilation plan, exclusion zone, fire watch requirement. • Record on daily work log: primer application start time, ventilation method (mechanical / natural), organic vapour respirator use (yes/no), cure window start time, cure completion time. • Obtain current SDS from primer supplier before application. Confirm Section 8 organic vapour threshold, cure time (Section 5), and skin contact first aid (Section 4). • Conduct safety talk with all workers on access restriction, slippery platform hazard, and chemical contact protocol immediately before primer application. • Establish fire watch: stationary spotter positioned upwind of primer application area during application and for period specified on SDS (typically 30–60 min post-application). • Close and barricade balconies directly below active work zone before elevated work starts each day • Notify affected residents before any noise/dust generating work on their level or adjacent levels | Medium (4) | • SUP — Supervise Prime and prepare repaired substrate • WKR — Perform Prime and prepare repaired substrate per SWMS | ⚠️ HOLD POINT 1.7 1. Do not apply primer until: SDS has been reviewed on point of use, ventilation requirement confirmed, and organic vapour respirator fitted and tested if required by SDS Section 8. 2. Do not apply anti-carbonation coating until: substrate cure time confirmed complete (per SDS Section 5), light sanding or damp-wiping finished, and substrate is visually clean and dry. 🛑 STOP-WORK TRIGGER • Wind drops below minimum ventilation threshold stated on SDS, or worker reports dizziness, nausea, or headache during primer application. • Primer vapour is visible as mist or haze across work area, or air quality cannot be confirmed as safe during application. • Scaffold platform becomes slippery from primer splash, or wet primer drips cannot be managed without creating fall hazard. • Occupant balcony below work zone is not closed and barricaded RISK CODE:WAH-H6 |
| 1.8 | Apply anti-carbonation coating system | • Inhalation of coating vapours — inadequate ventilation or respiratory protection during application of multi-coat system • Fall from height — slippery scaffold platform from wet coating drips or overspray • Chemical exposure — skin contact with uncured coating causing irritation or allergic reaction • Poor coating coverage — inadequate application or curing conditions resulting in uneven protection and corrosion risk | High (6) | Engineering Controls: • Obtain SDS for anti-carbonation coating before work starts. Confirm resin type, VOC content, cure time, and respiratory protection requirement at point of use. • Establish continuous forced ventilation (mechanical fan or natural cross-wind) across scaffold platform during application and cure. Verify wind speed or fan output meets SDS requirement — minimum 0.5 m/s air velocity across work face. • Wear organic vapour respirator (cartridge type per SDS Section 8) fitted and checked before each coat application. Do not use P2 dust masks for solvent or resin vapours. • check scaffold platform before each shift for wet drips, overspray, or slippery surface. Clean or dry affected areas with absorbent cloth before resuming work. Prohibit barefoot or smooth-soled footwear on platform. • Apply first coat in thin, even passes. Do not re-wet or overload substrate. Allow full cure time per SDS before second coat — minimum 16 hours or as specified by manufacturer, verified in writing. • Establish wet coating exclusion zone on scaffold platform — barricade wet areas with warning tape and cones. Post 'Wet Paint' signage visible to all workers and building occupants. • Wear chemical-resistant gloves (nitrile or butyl per SDS) and hi-viz shirt or vest • Do not start second coat until substrate is fully cured and inspected for even coverage. Check for bare patches, thin areas, or colour mismatch — mark and fix before final coat. Administrative Controls: • Confirm scaffold erection permit and handover certificate signed by licenced scaffolder before any coating work commences. • Confirm working at heights permit signed before each elevation or shift. • Record SDS receipt, ventilation test, cure times between coats, and coverage inspection sign-offs in site diary or QA register. • tell building management of work schedule, cure times, and ventilation method to manage occupied building disruption. • Obtain manufacturer's application data sheet or technical bulletin — confirm minimum temperature, humidity, and cure time before work starts. • Close and barricade balconies directly below active work zone before elevated work starts each day • Notify affected residents before any noise/dust generating work on their level or adjacent levels | Medium (4) | • SUP — Verify scaffold and fall arrest systems, enforce exclusion zone, check SDS • WKR — Use harness and lanyard, wear respiratory protection, report faults | ⚠️ HOLD POINT 1.8 1. Do not apply coating until: SDS is on site, ventilation method is installed and tested, and organic vapour respirator cartridges are confirmed current and fitted 2. Do not apply second or final coat until: first coat is fully cured per SDS time, substrate coverage is uniform and inspected, and wind/weather conditions remain within cure window 🛑 STOP-WORK TRIGGER • Forced ventilation fails, wind drops below 0.5 m/s, or organic vapour respirator cartridge is damaged, wet, or not fitted • Rain or temperature drops below SDS minimum cure temperature during application or cure window • Scaffold platform becomes slippery with wet coating and cannot be dried, or overspray contaminates adjacent work areas or building occupant spaces • Occupant balcony below work zone is not closed and barricaded RISK CODE:CHM-H6 |
| 1.9 | Apply textured acrylic render finish coat | • Inhalation of render spray or vapour — inadequate ventilation or respiratory protection during application • Fall from height — slippery scaffold platform from wet render splashes or overspray • Chemical exposure — skin or eye contact with uncured acrylic render causing irritation • Inadequate application — poor coverage, colour variation, or texture mismatch due to weather, contamination, or operator error | High (6) | Engineering Controls: • Read SDS on point of use before opening render product — note Section 8 respiratory and Section 11 toxicity limits. • Confirm weather window: dry, temp 15–25°C, humidity <85%, no rain forecast next 48 hours — stop work if conditions cannot be met. • Wet-suppress render spray using fine-mist water spray or air-aspirated application gun — prevent dry aerosol escape from nozzle. • Maintain natural cross-ventilation or position portable fan upwind to dilute vapour plume away from operator and building occupants. • Wear organic vapour respirator (per SDS Section 8) fitted over beard-free face — not P2 dust mask. • Establish exclusion zone (minimum 2 m radius) and tell building occupants of work schedule — prohibit entry during application. • check scaffold platform before each shift — remove wet render splashes immediately using damp cloth to prevent slipping hazard. • Wear chemical-resistant gloves and face shield — change gloves hourly or if render contacts skin. Administrative Controls: • Verify working at heights permit is signed and valid before starting elevated work. • Confirm all operators hold current RIIOHS204E or equivalent working-at-height competency — sight evidence before start of day. • Brief all building occupants on exclusion zone, work schedule, and expected duration — provide contact number for concerns. • Keep SDS for textured acrylic render accessible on site for duration of task — make available to first aid responder if exposure occurs. • Record actual weather, start/end time, product batch number, and colour match confirmation on site log daily. • Close and barricade balconies directly below active work zone before elevated work starts each day • Notify affected residents before any noise/dust generating work on their level or adjacent levels | Medium (4) | • SUP — Verify scaffold and fall arrest systems, enforce exclusion zone, check SDS • WKR — Use harness and lanyard, wear respiratory protection, report faults | ⚠️ HOLD POINT 1.9 1. Do not apply render until: scaffold handover certificate is signed by licenced scaffolder and weather conditions meet product specification. 2. Do not allow building occupants or adjacent workers into exclusion zone until: render is cured per SDS and overspray residue is cleaned from adjacent surfaces. 🛑 STOP-WORK TRIGGER • Wind speed exceeds 15 km/h, rain forecast within 24 hours, or temperature falls below 15°C or exceeds 25°C during application. • Organic vapour respirator becomes damaged, does not fit, or ventilation cannot be maintained. • Scaffold platform becomes wet or slippery and cannot be dried immediately. • Occupant balcony below work zone is not closed and barricaded RISK CODE:CHM-H6 |
| 1.10 | Execute touch-up repairs and quality corrections | • Fall from height — worker overreach or loss of balance during high-elevation touch-up work on scaffold • Inhalation of coating or primer vapours — inadequate ventilation during localised reapplication of coatings • Chemical exposure — skin or respiratory contact with uncured touch-up materials • Colour mismatch or surface defects — inadequate blending or curing resulting in visible repair marks | High (9) | Engineering Controls: • Verify scaffold is tagged 'Safe to Use' and handover certificate is signed by licenced scaffolder before any worker accesses it. • Install edge protection (kick board and mid-rail) on all exposed scaffold edges where touch-up work occurs at height >2m. • check harness, lanyard, and anchor point for damage before each shift; do not use if damaged or anchor point capacity unknown. • Tie off to independent anchor point rated ≥10 kN before starting work; movement between anchor points only with continuous attachment. • Establish exclusion zone 3m radius below all elevated touch-up work; restrict pedestrian and vehicle access with signs and barriers. • Ventilate work area continuously during coating application; open windows and doors or use mechanical extraction fan to manage vapour. • Review SDS for coating product on point of use; identify skin contact, inhalation, and cure time controls before work starts. • Apply touch-up coating only within environmental window specified on SDS (temperature, humidity, wind speed); do not apply outside limits. Administrative Controls: • Confirm Working at Heights Permit is current and signed by Site Supervisor before each elevated task. • Verify all workers hold current White Card (CPCCWHS1001) and Working at Heights competency (RIIOHS204E or current equivalent). • Provide SDS training on coating product to all workers before starting; confirm understanding in toolbox talk. • Brief occupants on active elevated work and exclusion zone; restrict occupied building access to area below touch-up work. • Record completion of touch-up repairs, colour match verification, and curing time in project quality log. • Conduct final inspection of blended coating and sign off on quality acceptance before demobilisation. • Close and barricade balconies directly below active work zone before elevated work starts each day • Notify affected residents before any noise/dust generating work on their level or adjacent levels | Medium (6) | • SUP — Verify access equipment safe, approve defect list, sign off completion • WKR — Inspect per specification, photograph defects, report issues | ⚠️ HOLD POINT 1.10 1. Do not access scaffold for touch-up work until: Scaffold handover certificate is signed by licenced scaffolder and edge protection is installed and tagged. 2. Do not apply coating until: SDS on point of use, environmental window confirmed, and exclusion zone is established and signed off by Site Supervisor. 🛑 STOP-WORK TRIGGER • Harness, lanyard, or anchor point is damaged or anchor point capacity cannot be confirmed. • Environmental window cannot be maintained (temperature, humidity, or wind outside SDS limits). • Worker overreaches beyond scaffold edge, or edge protection is damaged or incomplete. • Occupant balcony below work zone is not closed and barricaded RISK CODE:SIL-H6 |
| 1.11 | Clean site and remove scaffold | • Fall from height — worker slip or trip during platform cleaning, debris removal, or final inspection before demob • Struck by falling tools or debris — dropped equipment or unsecured materials during cleanup from scaffold platform • Inhalation of damp dust — liberation of settled dust during damp-wiping or dry-cleaning of facade • Collapse or instability — incomplete bracing or damaged components during scaffold partial dismantling | High (9) | Engineering Controls: • Damp-wipe all settled dust from facade and scaffold using low-pressure water spray and cloth before any dry cleaning or tool movement. • Establish 5 m exclusion zone below scaffold during final cleanup and debris removal — no occupants or pedestrians permitted. • check scaffold for loose bracing, damaged ties, or missing components before cleanup commences — do not work on defective scaffold. • Secure all tools and small items in closed containers on platform — no loose objects permitted above head height of people below. • Wear harness attached to certified anchor point during all work on platform above 2 m — harness and anchor inspected before use. • Brief all workers on pedestrian traffic routes and occupant interface zones before cleanup starts — mark final walkover path with tape. • Remove debris only via waste chute or closed skip — do not throw materials from height. • Conduct final walkover of all occupied-building interfaces and common areas — confirm no dust, debris, or residual hazard remains. Administrative Controls: • Confirm scaffold remains certified and tagged until final removal — do not permit use of damaged or uncertified scaffold. • Obtain site supervisor approval before final walkover commences. • Record final area clearance and occupant notification in site diary. • Coordinate with building management to confirm access to common areas and interface zones. • tell occupants of final cleanup schedule and temporary access restrictions. | Medium (6) | • SUP — Supervise dismantling sequence, verify exclusion zone below, sign off progressive removal • WKR — Dismantle per reverse-erection plan, wear harness at height, report defects | ⚠️ HOLD POINT 1.11 1. Do not remove scaffold until: Site supervisor confirms all debris is cleared, facade damp-dust is complete, and final walkover sign-off is obtained. 2. Do not permit occupants to re-occupy affected areas until: all barriers are removed, area is clean, and post-work inspection is documented. 🛑 STOP-WORK TRIGGER • Worker loses grip or balance on platform — do not resume until cause is identified and corrected. • Unsecured objects fall or escape exclusion zone — implement additional containment and investigate cause. • Visible bracing movement, cracking, or damage occurs during cleanup — tell scaffolder immediately. RISK CODE:WAH-H6 |
| 1.12 | Demobilise and verify site reinstatement | • Fall from height — worker loss of balance during final scaffold dismantling or inspection of platform before removal • Struck by falling scaffold components — dropped or misaligned members during dismantling sequence • Inadequate site restoration — incomplete removal of barriers, exclusion zones, or debris leaving occupied-building interface unsafe • Incomplete handover — missing documentation, safety checklist, or induction close-out creating liability gap | High (9) | Engineering Controls: • Establish exclusion zone below scaffold perimeter — minimum 2m radius — barricade with tape and signage before dismantling starts • Brief all workers on dismantling sequence and component drop zones — identify landing areas and confirm communication method before first member removal • Remove scaffold from top down — systematically dismantle horizontal beams first, then standards, then base; never force or lever members • Secure each removed member to designated stack area immediately — do not leave components unsecured on walkways or platforms • Use hand signals or two-way radio to communicate between dismantling crew and ground spotter — spotter remains in constant visual contact with all work • check all scaffold attachment points and base condition before final removal — photograph any cracks, deformation, or instability and report to Principal Contractor • Complete final safety checklist: all barriers removed, exclusion zones cleared, debris collected, access routes unobstructed, building services reinstated • Confirm site induction close-out with all workers — record names, dates and sign-off on project induction register Administrative Controls: • Retain scaffold erection permit and handover certificate in project file — required for audit and insurance • Obtain photographic record of final site condition before and after demobilisation — evidence of reinstatement compliance • Complete noise management final checklist confirming all high-noise plant has been removed and building occupants have been notified of site return to normal • Record all workers present during demobilisation on site diary — cross-reference against White Card and WAH competency register | Medium (6) | • SUP — Verify exclusion zone complete, barriers intact, access controlled before work starts • WKR — Set up and maintain barriers, stop work if zone breached, report access issues | ⚠️ HOLD POINT 1.12 1. Do not dismantle scaffold until: licenced scaffolder has verified erection method and base integrity, and exclusion zone is secured and signed off by Site Supervisor 2. Do not hand over site to Principal Contractor until: final safety checklist is complete, all debris is removed, all workers have completed induction close-out, and Principal Contractor has conducted final walk-through 🛑 STOP-WORK TRIGGER • Any scaffold member shows cracks, bent sections, or loose connections during dismantling — do not remove until assessed by licenced scaffolder • Wind exceeds 12 m/s or rain conditions affect grip and balance during elevated dismantling work — resume only when weather improves • Exclusion zone is breached by occupants or unauthorised personnel — re-establish barrier and restart sequence only when zone is clear |

CCVS TABLE

| Task | Critical Control | Who Checks | How Often | What They Look For |
| --- | --- | --- | --- | --- |
| Establish site, scaffold, and occupant interface plan | Confirm bracing installation matches scaffold design before load applied — visual check by supervising scaffolder each shift. | Supervising scaffolder / Site supervisor | Before erection commences, after each major section completion, daily during erection, before handover | Erection plan signed off with dated photographs showing bracing at each level; handover certificate counter-signed |
| Remove existing render, failed sealants, and loose concrete spall | Verify water feed to grinder active before each removal cycle; confirm dust extraction audible and air speed ≥150 L/min. | Site supervisor / Tool operator | Before each tool start and every 30 minutes continuous work | Visual check of water spray and dust collector fill; sound log if noise >85 dB(A) |
| Expose concrete substrate and assess damage | HEPA vacuum airflow test — compare pre-shift and mid-shift suction pressure; filter visual check for dust bypass or visible damage. | Site Supervisor / Appointed Competent Person | Before each shift start; every 2 hours during active dust generation; immediately after filter change. | Vacuum airflow gauge reading recorded in site diary; filter change tag dated; worker sign-off sheet confirming extraction operational confirmation. |
| Execute crack stitching and concrete repair | Dust extraction running and P2 respirator fitted before each grinding cycle | Supervisor and workers | Before each shift start and continuous during elevated work | Visual harness and anchor inspection checklist — dated and signed by supervisor; photographic record of anchor point installation. |
| Confirm scaffold safety, load capacity, and edge protection | Check guardrail height, toeboard integrity, handhold continuity, and 'SAFE TO USE' tag daily before any worker access. | Supervisor / Site induction lead | daily before work starts, after weather events, after high-impact work on adjacent levels | Daily scaffold inspection checklist completed and filed; 'SAFE TO USE' tag visible and current; no defects recorded. |
| Prime and prepare repaired substrate | Confirm continuous mechanical ventilation or minimum 0.5 m/s cross-breeze across work area. Record wind speed or fan setting before primer application and at 30 min intervals. | Site Supervisor / Crew Leader | Before application start, continuous during application, and at 30 min intervals during cure window | Anemometer reading (natural ventilation) or fan status log (mechanical ventilation). Operator sign-off on job sheet confirming ventilation threshold met. SDS Section 8 requirement documented. |
| Apply anti-carbonation coating system | Verify continuous forced ventilation across scaffold platform and organic vapour respirator fitted before each coat application. | Supervisor / Workers | Before each coat application; continuous during work; after each shift before cure period | SDS receipt logged; ventilation fan running or wind speed verified; respirator cartridge dated and fitted; scaffold platform cleaned and dried; cure time logged in site diary with supervisor sign-off; coverage inspection form completed |
| Apply textured acrylic render finish coat | SDS reviewed for product hazards, RPE matched to SDS requirements (organic vapour cartridge for solvents, not P2 dust mask), ventilation confirmed operational | Supervisor | before each work session (daily start and after any break >1 hour) | Verbal fit-check sign-off, SDS Section 8 requirement noted in site log, cartridge expiry date visible on filter. |
| Execute touch-up repairs and quality corrections | Dust extraction running and P2 respirator fitted before each grinding cycle | Supervisor | continuous during elevated work | Visual observation; harness inspection tag; project safety log entry; photographic record of colour match verification. |
| Clean site and remove scaffold | Check harness and anchor point are secure before each worker enters platform. | Site supervisor and each worker before starting | Before each work session and after any fall or near-miss | Visual inspection log and harness tag record |

RISK RATING MATRIX — Consequence × Likelihood

| Risk Level | Description of Consequence or Impact | Consequence | Unlikely (1) | Possible (2) | Almost Certain (3) |
| --- | --- | --- | --- | --- | --- |
| HIGH | Actual/Potential fatality, disability or irreversible damage. Major structural failure. Off-site environmental discharge not contained, significant long-term environmental harm. | Major (3) | Medium (3) | High (6) | High (9) |
| MEDIUM | Actual/Potential temporary disability, MTI or LTI. Structural failure/damage, >1-day outage. On-site environmental discharge contained, minor remediation, short-term harm. | Moderate (2) | Low (2) | Medium (4) | High (6) |
| LOW | Incident requiring first aid. Environmental discharge immediately contained, minor clean-up, no short-term environmental harm. | Minor (1) | Low (1) | Low (2) | Medium (3) |
| Level | Likelihood / Probability |  |  |  |  |
| Almost Certain (3) | Occurs frequently; >66% chance of occurring |  |  |  |  |
| Possible (2) | Could happen occasionally; >33% but <66% chance of occurring |  |  |  |  |
| Unlikely (1) | May occur only in exceptional circumstances; <33% chance of occurring |  |  |  |  |
| Class / Ranking | Description / Requirements |  |  |  |  |
| High 6, 9 | STOP immediately. Implement controls. Controls recorded on a SWMS. |  |  |  |  |
| Medium 3, 4 | Planned control. Controls recorded on a SWMS. |  |  |  |  |
| Low 1, 2 | Managed via routine procedure. |  |  |  |  |
| Note — WHS Act s.18 “reasonably practicable”: Requires consideration of the likelihood of the risk, degree of harm, knowledge of the hazard, availability and suitability of controls, and cost vs. risk. If you cannot show how that decision was made, the action becomes harder to defend after an incident. |  |  |  |  |  |

| WORKER SWMS INDUCTION SIGN-OFF |  |  |  |
| --- | --- | --- | --- |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |

| WORKER SWMS AMENDMENT SIGN-OFF |  |  |  |
| --- | --- | --- | --- |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |

If work methods, site conditions, or hazards change — stop, complete this table, re-brief all workers, obtain new sign-offs before resuming. Required under WHS Regulation r.300(3).

| SWMS AMENDMENTS |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Under WHS Act s18, “reasonably practicable” requires consideration of likelihood of risk, degree of harm, what the person knows about the hazard, availability and suitability of controls, cost vs risk. If you cannot show how that decision was made, the action becomes harder to defend after an incident. |  |  |  |  |  |  |
| Date | Work Activity / Task | Hazard / Risk | Risk Rating (Pre) | Controls — Hierarchy of Control | Risk Rating (Post) | Responsible |

This SWMS is a living document. It must be reviewed if work methods change, new hazards are identified or following any incident. Supervisor/Project Manager is responsible for ensuring this document remains current. NSW WHS Regulation r.300(3).

| WORKER SWMS AMENDMENT SIGN-OFF |  |  |  |
| --- | --- | --- | --- |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |

If work methods, site conditions, or hazards change — stop, complete this table, re-brief all workers, obtain new sign-offs before resuming. Required under WHS Regulation r.300(3).

| SWMS AMENDMENTS |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Under WHS Act s18, “reasonably practicable” requires consideration of likelihood of risk, degree of harm, what the person knows about the hazard, availability and suitability of controls, cost vs risk. If you cannot show how that decision was made, the action becomes harder to defend after an incident. |  |  |  |  |  |  |
| Date | Work Activity / Task | Hazard / Risk | Risk Rating (Pre) | Controls — Hierarchy of Control | Risk Rating (Post) | Responsible |

This SWMS is a living document. It must be reviewed if work methods change, new hazards are identified or following any incident. Supervisor/Project Manager is responsible for ensuring this document remains current. NSW WHS Regulation r.300(3).