| PCBU: | ■ PBCU: Robertson's Remedial and Painting Pty Ltd 10/56 Buffalo Road, Gladesville NSW 2111 Phone: (02) 9181 3519 ¦ ABN: 16 140 746 247 |  |  |  | Workplace location: | ■ Site: 22 Smith Street, Surry Hills NSW 2010 |
| --- | --- | --- | --- | --- | --- | --- |
| Works Manager: | ■ Works Manager: Alan Richardson |  |  |  | Date SWMS provided to PC: | ■ Date: 27/02/2026 |
| Work activity: | ■ Description: Concrete spalling repairs and epoxy crack injection to exterior façade elements. EWP (boom lift) access. Pre-2003 building — lead paint assumed present. Surry Hills urban location — public and pedestrian interface managed. |  |  |  | Principal Contractor (PC): | ■ PC: Mirvac |
| High Risk Construction Work (HRCW): | [✓] Risk of a person falling more than 2 metres |  | [ ] Work on a telecommunication tower |  | [ ] Demolition of load-bearing structure |  |
|  | [ ] Likely to involve disturbing asbestos |  | [ ] Temporary load-bearing support for structural alterations or repairs |  | [ ] Work in or near a confined space |  |
|  | [ ] Work in or near a shaft or trench deeper than 1.5 m or a tunnel |  | [ ] Use of explosives |  | [ ] Work on or near pressurised gas mains or piping |  |
|  | [ ] Work on or near chemical, fuel or refrigerant lines |  | [ ] Work on or near energised electrical installations or services |  | [ ] Work in an area that may have a contaminated or flammable atmosphere |  |
|  | [ ] Tilt-up or precast concrete elements |  | [ ] Work on, in or adjacent to a road, railway, shipping lane or other traffic corridor in use by traffic other than pedestrians |  | [✓] Work in an area with movement of powered mobile plant |  |
|  | [ ] Work in areas with artificial extremes of temperature |  | [ ] Work in or near water or other liquid that involves a risk of drowning |  | [ ] Diving work |  |
| Person responsible for ensuring compliance with SWMS: |  | ■ Supervisor or Team Leader: Alan Richardson |  | Date SWMS received: | ■ Date: 27/02/2026 |  |
| What measures are in place to ensure compliance with the SWMS? |  | Toolbox meetings, SWMS sign off, job observations and supervision review. If issues with the SWMS or new hazards are identified, the supervisor must be notified. When changes are made to SWMS, it will be communicated to all workers. |  |  |  |  |
| Person responsible for reviewing SWMS control measures: |  | ■ Project Manager or Manager: Alan Richardson |  | Date SWMS received by reviewer: | ■ Date: 27/02/2026 |  |
| How will the SWMS control measures be reviewed? |  | The control measures implemented will be reviewed and if necessary, revised annually or if work methods change, the control measures are not effective in controlling the risk, a new hazard/risk is identified or following an incident. The SWMS will be reviewed in consultation with workers and/or others who may be affected by the SWMS. Any changes to the SWMS will be communicated with workers at induction, daily pre-starts and toolbox talks. |  |  |  |  |
| Reviewer’s signature: |  | ■ Project Manager or Manager: Alan Richardson |  | Review date: | ■ Date: 27/02/2026 |  |
| This SWMS must be kept and be available for inspection until the high-risk construction work to which this SWMS relates is completed. If the SWMS is revised, all versions should be kept. If a notifiable incident occurs in relation to the high-risk construction work in this SWMS, the SWMS must be kept for at least 2 years from the date of the notifiable incident. |  |  |  |  |  |  |

SWMS — Concrete Spalling Repairs & Epoxy Injection — 22 Smith St Surry Hills [CCVS VERSION]

| PPE required: | ■ PPE: Hard hat AS/NZS 1801; Hi-vis vest or shirt; Steel-capped footwear AS/NZS 2210.3; Eye protection (safety glasses or goggles); P2/P3 respirator (concrete grinding, lead paint disturbance); Hearing protection >85 dB (grinding, breaking); Cut-resistant gloves (general); Chemical-resistant gloves (epoxy, coatings); Disposable coveralls (lead paint work); Full body harness and lanyard (EWP — clipped to manufacturer anchor). |
| --- | --- |

Consolidated

| Task | Hazard | Risk (Pre) | Control | Risk (Post) | Responsibility | Code |
| --- | --- | --- | --- | --- | --- | --- |
| 1. Site Induction and Daily Sign-In | Workers starting without site awareness or SWMS understanding. | Low (1) | - Induct all workers day one, daily toolbox talk, sign SWMS before starting — no signature, no start. | Low (1) | Supervisor | SYS-L1 |
| 2. Site Setup and Public Protection | Pedestrian or worker struck by vehicle — public entering work zone — dropped objects to footpath. | Medium (4) | - Barricade, signage, overhead protection, TPMP per Mirvac, pedestrian diversion, HOLD POINT if zone compromised. | Low (1) | Supervisor | TRF-M4 |
| 3. EWP Operation — Boom Lift | Fall from basket, tip-over, entrapment against façade — dropped objects to public below. | High (6) | - CCVS HOLD POINTS Pre-start check, firm ground, WP licence, spotter, harness clipped, exclusion zone confirmed. | Low (2) | Supervisor / Operator | WAH-H6 |
| 4. Lead Paint — Testing and Controls | Lead dust inhalation and ingestion — pre-2003 building, lead paint assumed on all surfaces. | High (6) | - CCVS HOLD POINTS Lead test or assume present, wet methods only, HEPA vac, containment sheeting, P2 minimum, decontamination enforced. | Low (2) | Supervisor | LED-H6 |
| 5. Concrete Breakout and Surface Preparation | Silica dust from concrete breakout — lead dust from painted surfaces — flying debris — vibration. | High (6) | - CCVS HOLD POINTS On-tool extraction or wet suppression, P2/P3 RPE, lead-safe controls active, exclusion zone, services scan. | Low (2) | Supervisor | SIL-H6 |
| 6. Concrete Spalling Repair — Structural | Hidden structural deterioration — uncontrolled concrete fall — chemical exposure from repair products. | High (6) | - CCVS HOLD POINTS Engineer assessment and written specification, Mirvac-approved method, materials per spec, STOP WORK if extent exceeded. | Low (2) | Supervisor | STR-H6 |
| 7. Epoxy Crack Injection | Epoxy skin sensitisation — chemical burns from hardener — solvent vapour — injection pressure. | Medium (4) | - SDS on site, chemical-resistant gloves, eye protection, equipment maintained, spill kit, first aid for contact. | Low (1) | Supervisor | ENV-M4 |
| 8. Portable Electrical Tools and Extension Leads | Electric shock from damaged tools — RCD failure — leads in wet conditions. | Medium (4) | - RCD on all 240V, tested and tagged AS/NZS 3760, battery tools where practicable, visual inspection pre-use. | Low (1) | Supervisor | ELE-M4 |
| 9. Emergency Response | Medical emergency, fire, height rescue, chemical spill on site. | High (9) | - Call 000, first aid kit on site, emergency plan briefed daily, muster point confirmed, rescue plan in place. | Low (1) | Supervisor | EMR-H9 |

Detail

| Task | Hazard | Risk (Pre) | Control | Risk (Post) | Responsibility | Code |
| --- | --- | --- | --- | --- | --- | --- |
| 1. Site Induction and Daily Sign-In (ALWAYS TASK 1 — ALL WORKERS) | Workers commencing without site awareness. SWMS controls not understood or communicated. Emergency procedures not briefed. | Low (1) | SYS (Low — C=1): Controls in place. Admin: PC site induction completed by all workers on first day — recorded in site diary or digital system. Admin: Toolbox talk conducted pre-commencement each day — covers tasks, hazards, controls, weather, site changes. Recorded. Admin: All workers sign SWMS before commencing work each day. No signature — no start. Admin: Emergency assembly point and contacts briefed at induction and repeated daily. Admin: Any site changes, new tasks, or changed conditions — SWMS reviewed and re-signed before work recommences. | Low (1) | Supervisor | SYS-L1 |
| 2. Site Setup and Public Protection Establish exclusion zones, barricades, and overhead protection. Surry Hills urban location — pedestrian and public interface. | Worker or pedestrian struck by vehicle. Public entry into work zone. Dropped objects or debris falling to footpath. Slips and trips during setup. | Medium (4) | TRF (Medium — C=2): Controls in place. Engineering: Barricade and signage — controlled work area. Pedestrian diversion around exclusion zone. Overhead protection (catch scaffold or debris netting) where work is above footpath or public area — no unprotected drop zone. Admin: Traffic management plan (TPMP) implemented per Mirvac requirements. Schedule deliveries off-peak. Coordinate with Mirvac site office for pedestrian management. HOLD POINT: If exclusion zone cannot be maintained without encroaching on footpath or roadway — STOP and implement approved TPMP before continuing. PPE: Hi-vis vest or shirt, steel-capped footwear, hard hat. | Low (1) | Supervisor | TRF-M4 |
| 3. EWP Operation — Boom Lift Boom lift EWP for access to exterior façade — concrete spalling repairs and crack injection. All height work via EWP only. | Fall from basket (>2m). Tip-over or instability on uneven or soft ground. Entrapment or crush against building façade or overhead structure. Dropped objects to ground level. Collision with pedestrians or plant. | High (6) | WAH (High — C=3) CCVS HOLD POINTS: Work must not commence until: 1. Daily pre-start inspection completed — defects nil — recorded. 2. Ground confirmed firm and level on hardstand — outriggers fully deployed. 3. Competent operator — WP licence sighted and confirmed. 4. Overhead clearances and façade setbacks measured and confirmed safe. 5. Harness inspected — lanyard clipped to manufacturer anchor point. 6. Exclusion zone established — spotter confirmed on station. Confirmation retained in site diary or digital system. Engineering: Daily pre-start inspection completed and recorded. Ground firm and level — hardstand only. Outriggers and stabilisers deployed per manufacturer. Admin: Competent operator — WP licence sighted. Spotter mandatory when manoeuvring near building façade. Exclusion zone around EWP — barricaded from public access. Tools and materials secured in basket — no loose items on platform edge. PPE: Harness clipped to manufacturer anchor, hard hat, hi-vis vest or shirt, steel-capped footwear. Remove from exposure and rectify if ground unstable or fault detected. | Low (2) | Supervisor / Operator | WAH-H6 |
| 4. Lead Paint — Testing and Controls Pre-2003 building — lead paint assumed present on all painted surfaces. Lead-safe work practices for all surface disturbance activities. | Lead dust inhalation — lead poisoning, neurological damage. Lead ingestion — hand-to-mouth contamination. Lead contamination of work area, clothing, and public areas below. Environmental contamination — lead dust or debris to stormwater. | High (6) | LED (High — C=3) CCVS HOLD POINTS: Work must not commence until: 1. Lead testing completed (XRF or lab) OR lead-safe controls applied as default for all surface disturbance on this pre-2003 building. 2. Containment sheeting installed to prevent lead dust spread to public areas and stormwater. 3. P2 respirators (minimum) confirmed fit-tested and available for all workers. 4. HEPA vacuum confirmed on site and operational. 5. Decontamination procedure briefed — coveralls removed before leaving work area, hands and face washed before eating or drinking. 6. Lead waste disposal method confirmed — labelled containers on site. Confirmation retained in site diary or digital system. Engineering: Wet methods only — no dry sanding, scraping, or grinding of painted surfaces. HEPA vacuum for all dust collection. Containment sheeting below all work areas. Admin: Test for lead before work or assume present (pre-2003 default). Decontamination mandatory — coveralls removed in work zone, wash hands and face before eating or drinking. Lead waste labelled and disposed per EPA guidelines. Air monitoring if extensive disturbance. PPE: P2 respirator (minimum), disposable coveralls, cut-resistant gloves, eye protection. P3 for extensive removal or elevated concentrations. Remove from exposure and rectify if dust controls fail. | Low (2) | Supervisor | LED-H6 |
| 5. Concrete Breakout and Surface Preparation Remove delaminated and spalled concrete back to sound substrate. Grind edges and prepare surface for repair mortar application. | Silica dust inhalation — silicosis, lung cancer. Lead dust from painted surfaces (see LED controls). Flying debris and concrete fragments. Noise and vibration — HAVS from extended grinding. Dropped debris to ground level. Striking concealed reinforcement or services. | High (6) | SIL (High — C=3) CCVS HOLD POINTS: Work must not commence until: 1. On-tool HEPA extraction or wet suppression confirmed fitted and operational on all grinders and breakers. 2. P2/P3 respirators confirmed fit-tested and available. 3. Exclusion zone established below work area — debris catch or overhead protection in place. 4. Lead-safe controls confirmed active (see LED task) — wet methods for all surface disturbance on painted substrates. 5. Services scan completed — confirm no live services in breakout zone. Confirmation retained in site diary or digital system. Engineering: On-tool HEPA extraction or wet suppression on all grinders and breakers. Blade guard fitted and intact. Debris catch below EWP to prevent fallout to public. Admin: P2 or P3 respirator — fit-tested. Eye and face protection mandatory. Hearing protection (>85 dB). Exclusion zone around work area for debris. Inspect grinding wheels before fitting — no cracks or damage. Vibration rotation — limit individual exposure, maintain tooling. PPE: P2/P3 respirator, face shield, hearing protection (>85 dB), cut-resistant gloves, steel-capped footwear. Remove from exposure and rectify if dust controls fail. | Low (2) | Supervisor | SIL-H6 |
| 6. Concrete Spalling Repair — Structural Treat exposed reinforcement, apply bonding agent and repair mortar to restore structural concrete profile. Engineering assessment required. | Structural deterioration beyond assessed extent — hidden corrosion. Further delamination during repair — uncontrolled concrete fall. Chemical exposure from repair mortar, bonding agents, and rust inhibitor. Manual handling of repair materials at height in EWP basket. | High (6) | STR (High — C=3) CCVS HOLD POINTS: Work must not commence until: 1. Structural engineer assessment completed — repair specification and extent of breakout confirmed in writing. 2. Repair method statement reviewed and approved by Mirvac. 3. Repair materials confirmed on site and matching specification (bonding agent, rust inhibitor, repair mortar). 4. Exclusion zone confirmed below all repair areas. Confirmation retained in site diary or digital system. Engineering: Structural engineer to assess and specify repair extent — written confirmation on file before commencing. Repair materials per engineer specification — no substitution. Admin: Clean exposed rebar to bright metal — wire brush or needle gun. Apply rust inhibitor per manufacturer instructions. Apply bonding agent to prepared surface before mortar. Repair mortar applied in lifts per specification — do not overload. SDS reviewed for all repair products. If deterioration exceeds assessed extent — STOP WORK: notify engineer and Mirvac before continuing. PPE: Eye protection, chemical-resistant gloves, P2 respirator (dust from wire brushing), steel-capped footwear. STOP WORK: Any unassessed structural deterioration, excessive rebar corrosion, or cracking beyond repair scope — stop and notify engineer. | Low (2) | Supervisor | STR-H6 |
| 7. Epoxy Crack Injection Identify and mark cracks, install injection ports, seal crack face, inject epoxy resin under low pressure, remove ports and finish. | Skin sensitisation from epoxy resin — allergic dermatitis. Eye contact with epoxy hardener — chemical burns. Solvent vapour inhalation from injection products. Injection equipment under pressure — hose failure. Manual handling of injection equipment at height. | Medium (4) | ENV (Medium — C=2): Controls in place. Engineering: Injection equipment maintained per manufacturer — pressure relief valve functional. Hose connections checked before use. Admin: SDS reviewed for epoxy resin and hardener before use — controls confirmed and SDS on site. Ventilation confirmed adequate — outdoor work via EWP. No ignition sources near solvent-based products. Drill injection ports using dust-controlled method (see SIL task). Spill kit on site — epoxy drips contained by drop sheet below. PPE: Chemical-resistant gloves (nitrile minimum), eye protection (safety glasses or goggles), P2 respirator if in semi-enclosed area, disposable coveralls recommended. Remove from exposure and rectify if ventilation inadequate. First aid: skin contact — wash immediately with soap and water. Eye contact — flush 15 minutes, seek medical attention. | Low (1) | Supervisor | ENV-M4 |
| 8. Portable Electrical Tools and Extension Leads Use of 240V grinders, drills, needle guns, and extension leads on site. | Electric shock from damaged tools or leads. RCD failure — unprotected circuit. Leads in wet conditions from pressure washing or wet grinding. Overloaded circuits — fire risk. | Medium (4) | ELE (Medium — C=2): Controls in place. Engineering: All 240V tools and leads protected by RCD — tested before use. Use battery tools where practicable. Admin: All tools and leads tested and tagged AS/NZS 3760 — construction site minimum 3-monthly. Visual inspection pre-use — no damaged plugs, cords, or housings. No daisy-chaining extension leads. Keep leads off ground where possible. Isolate before clearing jams or changing attachments. Keep electrical equipment away from water — use battery tools in wet areas. PPE: Steel-capped footwear. | Low (1) | Supervisor | ELE-M4 |
| 9. Emergency Response (ALWAYS FINAL TASK — ALL WORKERS) | Medical emergency on site. Fire or chemical spill. Worker requiring rescue from height or confined space. Uncontrolled public access during incident. | High (9) | EMR (High — C=3): Controls in place. GENERAL: Site emergency plan briefed at induction and daily toolbox. First aider and first aid kit on site at all times. Emergency contacts displayed at site entry and in site diary. Call 000 for any serious injury or emergency. FIRE: Sound alarm. Evacuate to muster point. Call 000. Do not re-enter. Headcount all workers. MEDICAL: Call 000. First aider to patient. Do not move injured person unless in immediate danger. HEIGHT RESCUE: Rescue plan briefed before work starts each day. Rescue equipment confirmed on site. Suspended worker recovered within 10 minutes — suspension trauma risk. CHEMICAL SPILL: Spill kit on site. Isolate area. Dispose per SDS Section 13. MUSTER POINT: Nominated at induction — briefed daily. | Low (1) | Supervisor | EMR-H9 |

| SWMS Amendments (more space at the end of this document) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

| Risk Level | Description of consequence or impact | Consequence | Likelihood/Probability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Unlikely (1) | Possible (2) | Almost Certain (3) |
| High Level of harm | Actual/Potential fatality, disability or irreversible damage. Major structural failure/damage. Off-site environmental discharge/release not contained and significant long-term environmental harm. | Major (3) | Medium (3) | High (6) | High (9) |
| Medium Level of harm | Actual/Potential temporary disability, MTI or LTI. Structural failure/damage, >1-day outage. On-site environmental discharge/release contained, minor remediation, short-term environmental harm. | Moderate (2) | Low (2) | Medium (4) | High (6) |
| Low Level of harm | Incident that has the potential to cause persons to require first aid. Environmental discharge/release immediately contained, minor level clean-up with no short-term environmental harm. | Minor (1) | Low (1) | Low (2) | Medium (3) |
| Level | Likelihood/Probability |  |  |  |  |
| Almost Certain | Occurs frequently; >66% chance of occurring |  |  |  |  |
| Possible | Could happen occasionally; >33% but <66% chance of occurring |  |  |  |  |
| Unlikely | May occur only in exceptional circumstances; <33% chance of occurring |  |  |  |  |
| Class/Ranking | Description/Requirements |  |  |  |  |
| High 6, 9 | Stop immediately. Implement controls. Controls recorded on a SWMS. |  |  |  |  |
| Medium 3, 4 | Planned control. Controls recorded on a SWMS. |  |  |  |  |
| Low 1, 2 | Managed via routine procedure. |  |  |  |  |

Under WHS Act s18, “reasonably practicable” requires consideration of likelihood of risk, degree of harm, what the person knows about the hazard, availability and suitability of controls, cost vs risk. If you cannot show how that decision was made, the action becomes harder to defend after an incident.

| Relevant legislation: | WHS Act 2011 (NSW), WHS Regulation 2017 (NSW), applicable NSW Codes of Practice, relevant Australian Standards, and SafeWork NSW guidance material., WHS Regulation 2017 Part 6.3 r299 (SWMS), SafeWork NSW Code of Practice — Managing Risks of Falls in Workplaces, SafeWork NSW Code of Practice — Managing the Risk of Lead in the Workplace, AS/NZS 4836 Safe Working on or Near Low-Voltage Electrical Installations |
| --- | --- |
| Frequency of review and site inspections: | This SWMS will be reviewed: before work commences, when site conditions change, when new hazards are identified, after an incident, annually (minimum). All revisions communicated via toolbox talk. Daily site inspections and job completion inspections to be completed by Project Manager or Supervisor. |

| PPE required: | ■ PPE: Hard hat AS/NZS 1801; Hi-vis vest or shirt; Steel-capped footwear AS/NZS 2210.3; Eye protection (safety glasses or goggles); P2/P3 respirator (concrete grinding, lead paint disturbance); Hearing protection >85 dB (grinding, breaking); Cut-resistant gloves (general); Chemical-resistant gloves (epoxy, coatings); Disposable coveralls (lead paint work); Full body harness and lanyard (EWP — clipped to manufacturer anchor). |
| --- | --- |
| List the permits, certificates, SafeWork NSW Approvals, required to complete the work: | ■ Permits Required: PC site induction (Mirvac) — ALL workers on first day; SWMS sign-on — ALL workers before commencing work each day; Toolbox talk pre-commencement — conducted and recorded daily; EWP permit or logbook — daily pre-start recorded; Lead-safe work notification — SafeWork NSW if required. GOVERNANCE (CCVS v16.0): Critical controls for high-risk work must be verified prior to commencement; verification recorded via documented or digital system. |
| List of the training required by workers to commence the work: | Construction Industry Induction Card and Safe Work Method Statement induction |
| List the qualifications of workers doing the work: | ■ Qualifications: White Card (Construction Industry Induction) — ALL workers; First Aid Certificate HLTAID011 — minimum one worker on site at all times; EWP licence (WP class) — all EWP operators; Lead-safe work training — all workers disturbing painted surfaces. |
| List of plant and equipment that will be used on site: | ■ Plant: Boom lift EWP (WP licence required); Angle grinders (dust extraction fitted); Needle gun / wire brush (rebar preparation); Hammer drill / rotary hammer; Epoxy crack injection pump and fittings; HEPA vacuum; Extension leads (tested and tagged AS/NZS 3760); RCD safety switches. |
| List maintenance checks for plant and equipment: | Daily pre-start checks recorded and maintenance as per manufacturers guidelines. |
| Hazardous substances: | ■ Substances: Epoxy crack injection resin and hardener — SDS on site; Structural repair mortar — SDS on site; Bonding agent / primer — SDS on site; Rust inhibitor (rebar treatment) — SDS on site; Lead paint dust — assume present (pre-2003 building); Silica dust — concrete grinding and breakout. |
| Working at Heights Risk Assessment (if applicable) | A-frame ladders may be used for short-duration painting tasks where a site-specific Working at Heights Risk Assessment (WaH) confirms ladder use is reasonably practicable; ladder must be AS/NZS 1892 compliant, stable, fully opened and used without overreaching. Working at Heights Risk Assessment (if >2m): Height of task: _______, duration of task: _______, ground stable and level, heavy exertion required? Yes / No, overreaching required? Yes / No, scaffold reasonably practicable? Yes / No, EWP reasonably practicable? Yes / No. A-frame ladder appropriate control? Yes / No Assessed by: ________________Date: ________ This creates documentary evidence. |

| WORKER INDUCTION SIGNOFF |  |  |  |
| --- | --- | --- | --- |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |

| SWMS Amendments |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

AUDIT: STD-1-1-SYS | STD-4-2-TRF | CCVS-6-3-WAH | CCVS-6-3-LED | CCVS-6-3-SIL | CCVS-6-3-STR | STD-4-2-ENV | STD-4-2-ELE | STD-9-3-EMR