| PCBU: | ■ PBCU: Robertson's Remedial and Painting Pty Ltd 10/56 Buffalo Road, Gladesville NSW 2111 Phone: (02) 9181 3519 ¦ ABN: 16 140 746 247 |  |  |  | Workplace location: | ■ Site: [Site Address] |
| --- | --- | --- | --- | --- | --- | --- |
| Works Manager: | ■ Works Manager: Jim Georgiadis |  |  |  | Date SWMS provided to PC: | ■ Date: [DD/MM/YYYY] |
| Work activity: | ■ Description: Concrete screed pump operations — sand/cement screed placement for tile beds using line pump (e.g. Putzmeister M740). Non-structural screeds, typically 40mm+ thickness. |  |  |  | Principal Contractor (PC): | ■ PC: [Principal Contractor] |
| High Risk Construction Work (HRCW): | [ ] Risk of a person falling more than 2 metres |  | [ ] Work on a telecommunication tower |  | [ ] Demolition of load-bearing structure |  |
|  | [ ] Likely to involve disturbing asbestos |  | [ ] Temporary load-bearing support for structural alterations or repairs |  | [ ] Work in or near a confined space |  |
|  | [ ] Work in or near a shaft or trench deeper than 1.5 m or a tunnel |  | [ ] Use of explosives |  | [✓] Work on or near pressurised gas mains or piping |  |
|  | [ ] Work on or near chemical, fuel or refrigerant lines |  | [ ] Work on or near energised electrical installations or services |  | [ ] Work in an area that may have a contaminated or flammable atmosphere |  |
|  | [ ] Tilt-up or precast concrete elements |  | [ ] Work on, in or adjacent to a road, railway, shipping lane or other traffic corridor in use by traffic other than pedestrians |  | [ ] Work in an area with movement of powered mobile plant |  |
|  | [ ] Work in areas with artificial extremes of temperature |  | [ ] Work in or near water or other liquid that involves a risk of drowning |  | [ ] Diving work |  |
| Person responsible for ensuring compliance with SWMS: |  | ■ Supervisor or Team Leader: [Supervisor Name] |  | Date SWMS received: | ■ Date: [DD/MM/YYYY] |  |
| What measures are in place to ensure compliance with the SWMS? |  | Toolbox meetings, SWMS sign off, job observations and supervision review. If issues with the SWMS or new hazards are identified, the supervisor must be notified. When changes are made to SWMS, it will be communicated to all workers. |  |  |  |  |
| Person responsible for reviewing SWMS control measures: |  | ■ Project Manager or Manager: Alan Richardson |  | Date SWMS received by reviewer: | ■ Date: [DD/MM/YYYY] |  |
| How will the SWMS control measures be reviewed? |  | The control measures implemented will be reviewed and if necessary, revised annually or if work methods change, the control measures are not effective in controlling the risk, a new hazard/risk is identified or following an incident. The SWMS will be reviewed in consultation with workers and/or others who may be affected by the SWMS. Any changes to the SWMS will be communicated with workers at induction, daily pre-starts and toolbox talks. |  |  |  |  |
| Reviewer’s signature: |  | ■ Project Manager or Manager: Alan Richardson |  | Review date: | ■ Date: [DD/MM/YYYY] |  |
| This SWMS must be kept and be available for inspection until the high-risk construction work to which this SWMS relates is completed. If the SWMS is revised, all versions should be kept. If a notifiable incident occurs in relation to the high-risk construction work in this SWMS, the SWMS must be kept for at least 2 years from the date of the notifiable incident. |  |  |  |  |  |  |

SWMS — Concrete Screed Pump Operations — [Site] [CCVS VERSION]

| PPE required: | ■ PPE: Hard hat AS/NZS 1801; Hi-vis vest or shirt; Steel-capped footwear AS/NZS 2210.3; Waterproof boots (placement and washout); Eye protection; P2 dust mask (dry cement handling); Hearing protection >85 dB (pump operation); Chemical-resistant gloves (nitrile); Long sleeves (alkaline burn protection). |
| --- | --- |

Consolidated

| Task | Hazard | Risk (Pre) | Control | Risk (Post) | Responsibility | Code |
| --- | --- | --- | --- | --- | --- | --- |
| 1. Site Induction and Daily Sign-In | Workers starting without site awareness or SWMS understanding. | Low (1) | Induct all workers day one, daily toolbox talk, sign SWMS before starting — no signature, no start. | Low (1) | Supervisor | SYS-L1 |
| 2. Screed Pump Setup, Hose Connection and Pressure Testing | High-pressure injection injury — hose whip from coupling failure — manual handling of pump and hoses. | High (6) | CCVS HOLD POINTS Operator competency confirmed, couplings inspected and secured, pressure test before pumping, exclusion zone during start-up, emergency shutdown briefed. | Low (2) | Supervisor / Pump Operator | ENE-H6 |
| 3. Material Preparation and Mesh Placement | Manual handling of bags — cement dust inhalation — cut injuries from mesh — slips on materials. | Medium (4) | Mechanical aids where available, bolt cutters for mesh, SDS reviewed, P2 dust mask and nitrile gloves. | Low (1) | Supervisor / Worker | ENV-M4 |
| 4. Screed Pumping, Placement and Levelling | Alkaline burns — slip on wet screed — noise — manual handling strain — hose trip hazard. | Medium (4) | Hose secured, non-slip walkways, pump-nozzle comms, pour sequence planned, waterproof boots and nitrile gloves. | Low (1) | Supervisor / Pump Operator / Worker | ENV-M4 |
| 5. Pump Cleanup, Washout and Demobilisation | Alkaline washout to stormwater — high-pressure flush — manual handling during demobilisation. | Medium (4) | Washout contained in bund, no stormwater discharge, pump depressurised before disconnection, site left clean. | Low (1) | Supervisor / Pump Operator | ENV-M4 |
| 6. Emergency Response | Medical emergency, fire, height rescue, chemical spill on site. | High (9) | Call 000, first aid kit on site, emergency plan briefed daily, muster point confirmed, rescue plan in place. | Low (1) | Supervisor | EMR-H9 |

Detail

| Task | Hazard | Risk (Pre) | Control | Risk (Post) | Responsibility | Code |
| --- | --- | --- | --- | --- | --- | --- |
| 1. Site Induction and Daily Sign-In (ALWAYS TASK 1 — ALL WORKERS) | Workers commencing without site awareness. SWMS controls not understood or communicated. Emergency procedures not briefed. | Low (1) | SYS (Low — C=1): Controls in place. Admin: PC site induction completed by all workers on first day — recorded in site diary or digital system. Admin: Toolbox talk conducted pre-commencement each day — covers tasks, hazards, controls, weather, site changes. Recorded. Admin: All workers sign SWMS before commencing work each day. No signature — no start. Admin: Emergency assembly point and contacts briefed at induction and repeated daily. Admin: Any site changes, new tasks, or changed conditions — SWMS reviewed and re-signed before work recommences. | Low (1) | Supervisor | SYS-L1 |
| 2. Screed Pump Setup, Hose Connection and Pressure Testing Position line pump, connect delivery hoses, inspect all couplings and safety clips, pressure test before pumping. | High-pressure injection injury from pump line — sand/cement injected under skin causes tissue necrosis, compartment syndrome, potential amputation or death. Hose whip from coupling failure. Manual handling of pump components and delivery hoses. Electrical hazard from pump motor. | High (6) | ENE (High — C=3) CCVS HOLD POINTS: Work must not commence until: 1. Pump operator trained and competent in specific pump model — training records sighted. 2. All delivery line couplings inspected, secured, and safety clips/pins confirmed — no worn or damaged fittings. 3. Pressure test completed at rated working pressure before pumping screed — no leaks, no coupling movement. 4. Exclusion zone established around pump and full length of delivery line — no personnel in hose whip zone during start-up and priming. Confirmation retained in site diary or digital system. Engineering: Hose clamps and couplings rated to pump maximum pressure — safety whip checks on all hose joints — delivery line secured and supported to prevent movement — RCD protection on electric pump. Admin: Pump manufacturer operating manual on site — daily pre-start inspection recorded — hose and coupling replacement schedule maintained — emergency shutdown procedure briefed to all workers. PPE: Steel-capped footwear, eye protection, hearing protection (>85 dB), cut-resistant gloves, hi-vis vest or shirt. STOP WORK if: Hose coupling leaking or damaged — pump pressure exceeds rated limit — delivery line unsecured or unsupported — blockage in line (do not attempt to clear under pressure) — electrical fault on pump motor. | Low (2) | Supervisor / Pump Operator | ENE-H6 |
| 3. Material Preparation and Mesh Placement Handle and stage cement and sand bags, cut and lay galvanised steel mesh reinforcement to specification. | Manual handling of cement and sand bags (20-25 kg). Cement dust inhalation and alkaline skin/eye contact. Cut and puncture injuries from mesh handling and cutting. Slip and trip on materials and offcuts. | Medium (4) | ENV (Medium — C=2): Controls in place. Engineering: Mechanical aids for repetitive bag handling where available — mesh cut with bolt cutters (not angle grinder) to reduce sparks and noise — material staged to minimise carry distances. Admin: SDS for cement reviewed — correct mix ratio confirmed (1:3 or 1:4 cement:sand per specification) — mesh specification and lap requirements confirmed before placement — rotate workers on manual handling tasks. PPE: P2 dust mask (dry cement handling), eye protection, chemical-resistant gloves (nitrile), steel-capped footwear, long sleeves. | Low (1) | Supervisor / Worker | ENV-M4 |
| 4. Screed Pumping, Placement and Levelling Pump sand/cement screed via delivery line, place to specified thickness, screed and level to falls. | Alkaline burns from wet cement screed (pH 12-13). Slip hazard on wet screed surface. Noise from pump operation. Manual handling strain from screeding and levelling. Hose movement and trip hazard. | Medium (4) | ENV (Medium — C=2): Controls in place. Engineering: Delivery hose routed and secured to prevent trip hazard — non-slip walkways maintained around pour area — pump operator maintains visual contact with nozzle operator at all times. Admin: Pour sequence planned to avoid workers walking on fresh screed — nozzle operator and pump operator communicate via agreed signals (radio or hand) — skin contact with wet screed washed immediately with clean water — screed thickness confirmed against specification during placement. PPE: Waterproof boots, chemical-resistant gloves (nitrile), eye protection, hearing protection (>85 dB), hi-vis vest or shirt, long sleeves. STOP WORK if: Communication between pump and nozzle operator fails — screed mix consistency incorrect (too wet or too dry to pump) — hose unsecured or moved from supported position. | Low (1) | Supervisor / Pump Operator / Worker | ENV-M4 |
| 5. Pump Cleanup, Washout and Demobilisation Flush delivery lines, contain washout water, disconnect hoses, clean and remove pump from site. | Alkaline washout water — environmental contamination if discharged to stormwater. High-pressure water during line flush. Manual handling during hose disconnection and pump removal. Slip on wet surfaces. | Medium (4) | ENV (Medium — C=2): Controls in place. Engineering: Washout water contained in designated bund or container — no discharge to stormwater drains, gutters, or ground — pump depressurised before disconnecting any coupling. Admin: Washout location agreed before pumping commences — washout water pH tested if discharge to sewer required (council approval) — all hoses and couplings cleaned, inspected, and stored — site left clean and free of screed residue. PPE: Waterproof boots, chemical-resistant gloves (nitrile), eye protection, hi-vis vest or shirt. STOP WORK if: No containment available for washout water — pump not fully depressurised before disconnection. | Low (1) | Supervisor / Pump Operator | ENV-M4 |
| 6. Emergency Response (ALWAYS FINAL TASK — ALL WORKERS) | Medical emergency on site. Fire or chemical spill. Worker requiring rescue from height or confined space. Uncontrolled public access during incident. | High (9) | EMR (High — C=3): Controls in place. GENERAL: Site emergency plan briefed at induction and daily toolbox. First aider and first aid kit on site at all times. Emergency contacts displayed at site entry and in site diary. Call 000 for any serious injury or emergency. FIRE: Sound alarm. Evacuate to muster point. Call 000. Do not re-enter. Headcount all workers. MEDICAL: Call 000. First aider to patient. Do not move injured person unless in immediate danger. HEIGHT RESCUE: Rescue plan briefed before work starts each day. Rescue equipment confirmed on site. Suspended worker recovered within 10 minutes — suspension trauma risk. CHEMICAL SPILL: Spill kit on site. Isolate area. Dispose per SDS Section 13. MUSTER POINT: Nominated at induction — briefed daily. | Low (1) | Supervisor | EMR-H9 |

| SWMS Amendments (more space at the end of this document) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

| Risk Level | Description of consequence or impact | Consequence | Likelihood/Probability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Unlikely (1) | Possible (2) | Almost Certain (3) |
| High Level of harm | Actual/Potential fatality, disability or irreversible damage. Major structural failure/damage. Off-site environmental discharge/release not contained and significant long-term environmental harm. | Major (3) | Medium (3) | High (6) | High (9) |
| Medium Level of harm | Actual/Potential temporary disability, MTI or LTI. Structural failure/damage, >1-day outage. On-site environmental discharge/release contained, minor remediation, short-term environmental harm. | Moderate (2) | Low (2) | Medium (4) | High (6) |
| Low Level of harm | Incident that has the potential to cause persons to require first aid. Environmental discharge/release immediately contained, minor level clean-up with no short-term environmental harm. | Minor (1) | Low (1) | Low (2) | Medium (3) |
| Level | Likelihood/Probability |  |  |  |  |
| Almost Certain | Occurs frequently; >66% chance of occurring |  |  |  |  |
| Possible | Could happen occasionally; >33% but <66% chance of occurring |  |  |  |  |
| Unlikely | May occur only in exceptional circumstances; <33% chance of occurring |  |  |  |  |
| Class/Ranking | Description/Requirements |  |  |  |  |
| High 6, 9 | Stop immediately. Implement controls. Controls recorded on a SWMS. |  |  |  |  |
| Medium 3, 4 | Planned control. Controls recorded on a SWMS. |  |  |  |  |
| Low 1, 2 | Managed via routine procedure. |  |  |  |  |

Under WHS Act s18, “reasonably practicable” requires consideration of likelihood of risk, degree of harm, what the person knows about the hazard, availability and suitability of controls, cost vs risk. If you cannot show how that decision was made, the action becomes harder to defend after an incident.

| Relevant legislation: | WHS Act 2011 (NSW), WHS Regulation 2017 (NSW), applicable NSW Codes of Practice, relevant Australian Standards, and SafeWork NSW guidance material., WHS Regulation 2017 Part 6.3 r299 (SWMS), SafeWork NSW Code of Practice — Managing the Risks of Plant in the Workplace |
| --- | --- |
| Frequency of review and site inspections: | This SWMS will be reviewed: before work commences, when site conditions change, when new hazards are identified, after an incident, annually (minimum). All revisions communicated via toolbox talk. Daily site inspections and job completion inspections to be completed by Project Manager or Supervisor. |

| PPE required: | ■ PPE: Hard hat AS/NZS 1801; Hi-vis vest or shirt; Steel-capped footwear AS/NZS 2210.3; Waterproof boots (placement and washout); Eye protection; P2 dust mask (dry cement handling); Hearing protection >85 dB (pump operation); Chemical-resistant gloves (nitrile); Long sleeves (alkaline burn protection). |
| --- | --- |
| List the permits, certificates, SafeWork NSW Approvals, required to complete the work: | ■ Permits Required: PC site induction — ALL workers on first day; SWMS sign-on — ALL workers before commencing work each day; Toolbox talk pre-commencement — conducted and recorded daily. GOVERNANCE (CCVS v16.0): Critical controls for high-risk work must be verified prior to commencement; verification recorded via documented or digital system. |
| List of the training required by workers to commence the work: | Construction Industry Induction Card and Safe Work Method Statement induction |
| List the qualifications of workers doing the work: | ■ Qualifications: White Card (Construction Industry Induction) — ALL workers; First Aid Certificate HLTAID011 — minimum one worker on site at all times; Screed pump operator competency — manufacturer-specific training or demonstrated equivalent experience. |
| List of plant and equipment that will be used on site: | ■ Plant: Concrete screed pump (line pump, e.g. Putzmeister M740); Delivery hoses and couplings (rated to pump max pressure); Screeding rails and straightedge; Bolt cutters (mesh cutting); Wheelbarrow; Extension leads (tested and tagged AS/NZS 3760); RCD safety switches. |
| List maintenance checks for plant and equipment: | Daily pre-start checks recorded and maintenance as per manufacturers guidelines. |
| Hazardous substances: | ■ Substances: Portland cement — SDS on site; Sharp sand (washed); Galvanised steel mesh reinforcement. |
| Working at Heights Risk Assessment (if applicable) | A-frame ladders may be used for short-duration painting tasks where a site-specific Working at Heights Risk Assessment (WaH) confirms ladder use is reasonably practicable; ladder must be AS/NZS 1892 compliant, stable, fully opened and used without overreaching. Working at Heights Risk Assessment (if >2m): Height of task: _______, duration of task: _______, ground stable and level, heavy exertion required? Yes / No, overreaching required? Yes / No, scaffold reasonably practicable? Yes / No, EWP reasonably practicable? Yes / No. A-frame ladder appropriate control? Yes / No Assessed by: ________________Date: ________ This creates documentary evidence. |

| WORKER INDUCTION SIGNOFF |  |  |  |
| --- | --- | --- | --- |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |

| SWMS Amendments |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

AUDIT: STD-1-1-SYS | CCVS-6-3-ENE | STD-4-2-ENV | STD-4-2-ENV | STD-4-2-ENV | STD-9-3-EMR