| PCBU: | ■ PBCU: RPD Pty Ltd 1 Safety Street Sydney NSW 2000 Phone: 02 9000 0000 ¦ ABN: 12 345 678 901 |  |  |  | Workplace location: | ■ Site: 45 Construction Ave, Parramatta NSW 2150 |
| --- | --- | --- | --- | --- | --- | --- |
| Works Manager: | ■ Works Manager: Alan Richardson |  |  |  | Date SWMS provided to PC: | ■ Date: 24/02/2026 |
| Work activity: | ■ Description: General waterproofing to balcony slabs. Access via ground level and ladders. Surface preparation, primer application, and liquid membrane application. Approximately 3 days duration. |  |  |  | Principal Contractor (PC): | ■ PC: SD Group |
| High Risk Construction Work (HRCW): | [✓] Risk of a person falling more than 2 metres |  | [ ] Work on a telecommunication tower |  | [ ] Demolition of load-bearing structure |  |
|  | [ ] Likely to involve disturbing asbestos |  | [ ] Temporary load-bearing support for structural alterations or repairs |  | [ ] Work in or near a confined space |  |
|  | [ ] Work in or near a shaft or trench deeper than 1.5 m or a tunnel |  | [ ] Use of explosives |  | [ ] Work on or near pressurised gas mains or piping |  |
|  | [ ] Work on or near chemical, fuel or refrigerant lines |  | [ ] Work on or near energised electrical installations or services |  | [ ] Work in an area that may have a contaminated or flammable atmosphere |  |
|  | [ ] Tilt-up or precast concrete elements |  | [ ] Work on, in or adjacent to a road, railway, shipping lane or other traffic corridor in use by traffic other than pedestrians |  | [ ] Work in an area with movement of powered mobile plant |  |
|  | [ ] Work in areas with artificial extremes of temperature |  | [ ] Work in or near water or other liquid that involves a risk of drowning |  | [ ] Diving work |  |
| Person responsible for ensuring compliance with SWMS: |  | ■ Supervisor or Team Leader: Alan Richardson |  | Date SWMS received: | ■ Date: 24/02/2026 |  |
| What measures are in place to ensure compliance with the SWMS? |  | Toolbox meetings, SWMS sign off, job observations and supervision review. If issues with the SWMS or new hazards are identified, the supervisor must be notified. When changes are made to SWMS, it will be communicated to all workers. |  |  |  |  |
| Person responsible for reviewing SWMS control measures: |  | ■ Project Manager or Manager: Alan Richardson |  | Date SWMS received by reviewer: | ■ Date: 24/02/2026 |  |
| How will the SWMS control measures be reviewed? |  | The control measures implemented will be reviewed and if necessary, revised annually or if work methods change, the control measures are not effective in controlling the risk, a new hazard/risk is identified or following an incident. The SWMS will be reviewed in consultation with workers and/or others who may be affected by the SWMS. Any changes to the SWMS will be communicated with workers at induction, daily pre-starts and toolbox talks. |  |  |  |  |
| Reviewer’s signature: |  | ■ Project Manager or Manager: Alan Richardson |  | Review date: | ■ Date: 24/02/2026 |  |
| This SWMS must be kept and be available for inspection until the high-risk construction work to which this SWMS relates is completed. If the SWMS is revised, all versions should be kept. If a notifiable incident occurs in relation to the high-risk construction work in this SWMS, the SWMS must be kept for at least 2 years from the date of the notifiable incident. |  |  |  |  |  |  |

SWMS — Balcony Waterproofing — 45 Construction Ave Parramatta [STANDARD VERSION]

| PPE required: | ■ PPE: Hard hat AS/NZS 1801; Safety glasses or goggles; Chemical-resistant gloves; Steel cap safety boots AS/NZS 2210.3; Hi-vis vest; P2/P3 respirator (grinding and membrane application); Hearing protection (grinding); Coveralls (membrane application); Face shield (grinding); Non-slip footwear (ladder use). |
| --- | --- |

Consolidated

| Task | Hazard | Risk (Pre) | Control | Risk (Post) | Responsibility | Code |
| --- | --- | --- | --- | --- | --- | --- |
| 1. Site Induction and Daily Sign-In | Workers starting without site awareness or SWMS understanding. | Low (1) | - Induct all workers day one, daily toolbox talk, sign SWMS before starting — no signature, no start. | Low (1) | Supervisor | SYS-L1 |
| 2. Ladder Use — Short Duration Only | Fall from ladder — kick-out, overreaching, dropped tools onto persons below. | High (6) | - EWP or scaffold first - WaH RA required - 4:1 angle - three points contact - barricade below. | Low (2) | Alan Richardson | WAH-H6 |
| 3. Surface Preparation — Balcony Substrate | Silica dust inhalation from grinding concrete balcony slabs — silicosis risk. | High (6) | - On-tool extraction or wet suppression - P2/P3 respirator - face shield - exclusion zone. | Low (2) | Alan Richardson | SIL-H6 |
| 4. Waterproofing Membrane Application | Solvent vapour in enclosed balcony, isocyanate sensitisation, ignition risk, slip on uncured membrane. | High (6) | - Forced ventilation - SDS controls - isocyanate fitness check - ignition eliminated - barricade until cured. | Low (2) | Alan Richardson | ENV-H6 |
| 5. Chemical Application — Solvents and Cleaning Agents | Solvent vapour inhalation, skin contact, ignition risk, spill to stormwater. | Medium (4) | - SDS reviewed - ventilation confirmed - ignition sources eliminated - spill kit on site. | Low (1) | Alan Richardson | ENV-M4 |
| 6. Public Interface — Residential Premises | Public or occupant entry into work zone — dropped objects — noise and dust impact. | Medium (4) | - Hoarding or barriers to separate public - overhead catch protection - coordinate hours - secure site each shift. | Low (1) | Alan Richardson | TRF-M4 |
| 7. Emergency Response | Medical emergency, fire, height rescue, chemical spill on site. | High (9) | - Call 000, first aid kit on site, emergency plan briefed daily, muster point confirmed, rescue plan in place. | Low (1) | Supervisor | EMR-H9 |

Detail

| Task | Hazard | Risk (Pre) | Control | Risk (Post) | Responsibility | Code |
| --- | --- | --- | --- | --- | --- | --- |
| 1. Site Induction and Daily Sign-In (ALWAYS TASK 1 — ALL WORKERS) | Workers commencing without site awareness. SWMS controls not understood or communicated. Emergency procedures not briefed. | Low (1) | SYS (Low — C=1): Controls in place. Admin: PC site induction completed by all workers on first day — recorded in site diary or digital system. Admin: Toolbox talk conducted pre-commencement each day — covers tasks, hazards, controls, weather, site changes. Recorded. Admin: All workers sign SWMS before commencing work each day. No signature — no start. Admin: Emergency assembly point and contacts briefed at induction and repeated daily. Admin: Any site changes, new tasks, or changed conditions — SWMS reviewed and re-signed before work recommences. | Low (1) | Supervisor | SYS-L1 |
| 2. Ladder Use — Short Duration Only A-frame or extension ladder for balcony access where EWP or scaffold is not reasonably practicable. | Fall from height. Kick-out or instability. Overreaching. Dropped tools striking persons below. | High (6) | WAH (High — C=3): Controls in place. Eliminate: Use EWP or scaffold first — ladder only where not practicable. Engineering: Industrial-rated ladder, correct angle (4:1 extension), non-slip feet, secured where possible. Admin: Site-specific WaH RA completed before use — confirms ladder is only practicable method. Three points of contact at all times. No top two rungs. No overreaching. Barricade area below. PPE: Non-slip footwear, helmet, tool lanyards. Remove from exposure and rectify if ladder is damaged or conditions unsafe. | Low (2) | Alan Richardson | WAH-H6 |
| 3. Surface Preparation — Balcony Substrate Grinding, sanding, and cleaning concrete balcony slabs prior to waterproofing membrane application. | Silica dust inhalation from grinding concrete — silicosis risk. Flying debris and sparks. Chemical exposure from cleaners. Noise and vibration from angle grinder. | High (6) | SIL (High — C=3): Controls in place. Eliminate: Wet sanding where practicable — no dry grinding. Engineering: On-tool extraction (HEPA vacuum) or wet suppression on grinder. Blade guard fitted and intact. Admin: P2 or P3 respirator — fit-tested where required. Eye and face protection. Hearing protection. Exclusion zone around grinder for debris. Inspect wheel before fitting — no cracks or damage. Test for lead or assume present if pre-2000 building — lead-safe procedures apply. PPE: P2/P3 respirator, face shield, hearing protection, gloves. Remove from exposure and rectify if dust controls fail. | Low (2) | Alan Richardson | SIL-H6 |
| 4. Waterproofing Membrane Application Apply liquid waterproofing primer and membrane to balcony slabs. | Solvent vapour inhalation in semi-enclosed balcony spaces. Skin sensitisation from isocyanates or epoxy components. Ignition of flammable membrane products. Slip on wet or uncured membrane. | High (6) | ENV (High — C=3): Controls in place. Engineering: Forced ventilation in semi-enclosed balcony spaces. Continuous air monitoring if isocyanate-based product used. Admin: SDS reviewed for primer and membrane — confirm if isocyanate, epoxy, or solvent-based. Medical fitness confirmed for isocyanate exposure. Ignition sources eliminated before application. No walking on uncured membrane — barricade until cured. PPE: Supplied-air respirator or half-face with appropriate cartridge (per SDS), chemical-resistant gloves, coveralls, eye protection. | Low (2) | Alan Richardson | ENV-H6 |
| 5. Chemical Application — Solvents and Cleaning Agents Use of mineral turpentine, primers, and cleaning agents during waterproofing preparation and application. | Solvent vapour inhalation. Skin and eye contact. Ignition of flammable vapours. Environmental contamination — spill to stormwater or drain. | Medium (4) | ENV (Medium — C=2): Controls in place. Admin: SDS reviewed before use — controls confirmed. Ventilation confirmed adequate before application. Ignition sources eliminated before opening solvent-based products. Spill kit on site. No disposal to drain or stormwater. PPE: Respirator per SDS, chemical-resistant gloves, eye protection, coveralls. Remove from exposure and rectify if ventilation inadequate. | Low (1) | Alan Richardson | ENV-M4 |
| 6. Public Interface — Residential Premises Works on balconies of occupied residential building with public nearby. | Public or occupant entry into work zone. Dropped objects or debris reaching public areas. Noise, dust, or fume impact on occupants. Slips and trips — tools, leads, debris in public areas. | Medium (4) | TRF (Medium — C=2): Controls in place. Engineering: Hoarding, barriers, or screens to fully separate public from work zone. Overhead catch protection where drop risk to public areas. Admin: Coordinate work hours with building manager. Noise, dust, and fume management — schedule high-impact work for agreed times. Work zone inspected and secured at end of each shift. Clear and maintain emergency egress at all times. Residents notified before commencement. PPE: Hi-vis, safety boots, appropriate dust/fume controls. | Low (1) | Alan Richardson | TRF-M4 |
| 7. Emergency Response (ALWAYS FINAL TASK — ALL WORKERS) | Medical emergency on site. Fire or chemical spill. Worker requiring rescue from height or confined space. Uncontrolled public access during incident. | High (9) | EMR (High — C=3): Controls in place. GENERAL: Site emergency plan briefed at induction and daily toolbox. First aider and first aid kit on site at all times. Emergency contacts displayed at site entry and in site diary. Call 000 for any serious injury or emergency. FIRE: Sound alarm. Evacuate to muster point. Call 000. Do not re-enter. Headcount all workers. MEDICAL: Call 000. First aider to patient. Do not move injured person unless in immediate danger. HEIGHT RESCUE: Rescue plan briefed before work starts each day. Rescue equipment confirmed on site. Suspended worker recovered within 10 minutes — suspension trauma risk. CHEMICAL SPILL: Spill kit on site. Isolate area. Dispose per SDS Section 13. MUSTER POINT: Nominated at induction — briefed daily. | Low (1) | Supervisor | EMR-H9 |

| SWMS Amendments (more space at the end of this document) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

| Risk Level | Description of consequence or impact | Consequence | Likelihood/Probability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Unlikely (1) | Possible (2) | Almost Certain (3) |
| High Level of harm | Actual/Potential fatality, disability or irreversible damage. Major structural failure/damage. Off-site environmental discharge/release not contained and significant long-term environmental harm. | Major (3) | Medium (3) | High (6) | High (9) |
| Medium Level of harm | Actual/Potential temporary disability, MTI or LTI. Structural failure/damage, >1-day outage. On-site environmental discharge/release contained, minor remediation, short-term environmental harm. | Moderate (2) | Low (2) | Medium (4) | High (6) |
| Low Level of harm | Incident that has the potential to cause persons to require first aid. Environmental discharge/release immediately contained, minor level clean-up with no short-term environmental harm. | Minor (1) | Low (1) | Low (2) | Medium (3) |
| Level | Likelihood/Probability |  |  |  |  |
| Almost Certain | Occurs frequently; >66% chance of occurring |  |  |  |  |
| Possible | Could happen occasionally; >33% but <66% chance of occurring |  |  |  |  |
| Unlikely | May occur only in exceptional circumstances; <33% chance of occurring |  |  |  |  |
| Class/Ranking | Description/Requirements |  |  |  |  |
| High 6, 9 | Stop immediately. Implement controls. Controls recorded on a SWMS. |  |  |  |  |
| Medium 3, 4 | Planned control. Controls recorded on a SWMS. |  |  |  |  |
| Low 1, 2 | Managed via routine procedure. |  |  |  |  |

Under WHS Act s18, “reasonably practicable” requires consideration of likelihood of risk, degree of harm, what the person knows about the hazard, availability and suitability of controls, cost vs risk. If you cannot show how that decision was made, the action becomes harder to defend after an incident.

| Relevant legislation: | WHS Act 2011 (NSW), WHS Regulation 2017 (NSW), applicable NSW Codes of Practice, relevant Australian Standards, and SafeWork NSW guidance material., WHS Regulation 2017 Part 6.3 r299 (SWMS), AS/NZS 1891 (industrial fall protection), SafeWork NSW Code of Practice: Managing the Risk of Falls at Workplaces |
| --- | --- |
| Frequency of review and site inspections: | This SWMS will be reviewed: before work commences, when site conditions change, when new hazards are identified, after an incident, annually (minimum). All revisions communicated via toolbox talk. Daily site inspections and job completion inspections to be completed by Project Manager or Supervisor. |

| PPE required: | ■ PPE: Hard hat AS/NZS 1801; Safety glasses or goggles; Chemical-resistant gloves; Steel cap safety boots AS/NZS 2210.3; Hi-vis vest; P2/P3 respirator (grinding and membrane application); Hearing protection (grinding); Coveralls (membrane application); Face shield (grinding); Non-slip footwear (ladder use). |
| --- | --- |
| List the permits, certificates, SafeWork NSW Approvals, required to complete the work: | ■ Permits Required: PC site induction — ALL workers on first day; SWMS sign-on — ALL workers before commencing work each day; Toolbox talk pre-commencement — conducted and recorded daily. GOVERNANCE (CCVS v16.0): Critical controls for high-risk work must be verified prior to commencement; verification recorded via documented or digital system. |
| List of the training required by workers to commence the work: | Construction Industry Induction Card and Safe Work Method Statement induction |
| List the qualifications of workers doing the work: | ■ Qualifications: White Card (Construction Industry Induction) — ALL workers; First Aid Certificate HLTAID011 — minimum one worker on site at all times. |
| List of plant and equipment that will be used on site: | ■ Plant: Angle grinder (with HEPA vacuum extraction or wet suppression); Hand tools — scrapers, trowels, rollers, brushes. |
| List maintenance checks for plant and equipment: | Daily pre-start checks recorded and maintenance as per manufacturers guidelines. |
| Hazardous substances: | ■ Substances: Waterproofing primer (confirm SDS on site before use); Waterproofing membrane — liquid applied (confirm SDS on site before use); Mineral turpentine (confirm SDS on site before use). |
| Working at Heights Risk Assessment (if applicable) | A-frame ladders may be used for short-duration painting tasks where a site-specific Working at Heights Risk Assessment (WaH) confirms ladder use is reasonably practicable; ladder must be AS/NZS 1892 compliant, stable, fully opened and used without overreaching. Working at Heights Risk Assessment (if >2m): Height of task: _______, duration of task: _______, ground stable and level, heavy exertion required? Yes / No, overreaching required? Yes / No, scaffold reasonably practicable? Yes / No, EWP reasonably practicable? Yes / No. A-frame ladder appropriate control? Yes / No Assessed by: ________________Date: ________ This creates documentary evidence. |

| WORKER INDUCTION SIGNOFF |  |  |  |
| --- | --- | --- | --- |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will cease work if controls are not established. |

| SWMS Amendments |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

AUDIT: STD-1-1-SYS | STD-6-3-WAH | STD-6-3-SIL | STD-6-3-ENV | STD-4-2-ENV | STD-4-2-TRF | STD-9-3-EMR