RPD SWMS

ARCHITECTURE GOVERNANCE LAYER

Robertson’s Remedial and Painting Pty Ltd

| Document: | RPD SWMS Architecture Governance Layer | Ref: | RPD-GOV-001 |
| --- | --- | --- | --- |
| Version: | 1.0 | Date: | 24 Feb 2026 |
| Author: | Alan Richardson — WHS Consultant | Status: | DRAFT |
| Approved: | Leslie Robertson — Director, RPD | Next Review: | Feb 2027 |

# 1. Purpose

This document is the single source of truth for how Robertson’s Remedial and Painting Pty Ltd (RPD) builds, codes, and verifies Safe Work Method Statements. Every Master SWMS, Standalone SWMS, and supporting document produced under the RPD WHS Management System V8 must comply with this governance layer.

It defines:

•  The 8-code hazard classification system used across all SWMS

•  The CCVS (Code-Category-Verification-Status) trigger logic for critical hold points

•  The HOLD POINT and STOP WORK verification framework

•  The common controls library — reusable tasks shared across multiple SWMS

•  The risk scoring methodology

•  The audit trail from code → control → verification → record

# 2. The 9-Code Hazard Classification System

Every task in every RPD SWMS is assigned exactly one code. The code identifies the primary hazard category. Eight codes cover RPD’s full scope of work.

| Code | Category | Covers | CCVS | Notes |
| --- | --- | --- | --- | --- |
| SYS | System — Governance & Emergency | Site induction, daily sign-in, toolbox talks, emergency response, muster, first aid | Never | Governance bookend. Opens and closes every SWMS. No pre-commencement gate — EMR must be executable immediately. |
| WAH | Work at Height | Scaffold, EWP, harness (restraint and arrest), edge protection, penetrations, voids, rope access (IRA) | C=3 | Covers ALL height work. No separate codes for restraint, arrest, or rope access — WAH is the single code. |
| SIL | Silica & Dust | Drilling, cutting, grinding, core drilling, demolition breakout (non-structural), jackhammer, concrete saw | C=3 | Any task generating respirable crystalline silica dust. Wet suppression or HEPA extraction mandatory. |
| STR | Structural | Demolition of load bearing structures, temporary propping, shoring, structural alterations | C=3 | Requires engineer’s design or approval. Licensed demolisher for Class 1. Distinct from non-structural strip-out (which sits under SIL). |
| ELE | Electrical | Power tools and leads at C=2. Electrical isolation and LOTO at C=3. | C=3 only | Dual consequence code. Tools/leads = C=2 (no hold point). Isolation work = C=3 (hold point required). |
| HOT | Hot Works | Welding, oxy cutting, grinding with sparks, any ignition source near combustibles | C=3 | Hot work permit required. Fire watch mandatory. Combustibles removed or wetted. |
| HAZ | Hazardous Materials | Asbestos (Class B non-friable), lead paint, hazardous chemicals, solvents, adhesives | C=3 | Single code for all hazmat. Specific protocols for asbestos (6-step) and lead (6-step) embedded in controls. |
| PLT | Plant & Equipment | Mobile plant, crane trucks, forklifts, excavators, delivery vehicles, hoists | C=3 | Operator HRWL verified. Exclusion zones. Spotter where required. Pre-start inspection. |
| PRE | Preparation & General Trades | Sanding (timber, metal), scraping, filling, rendering, patching, crack filling, sealant application, pressure washing, chemical cleaning, general trades preparation. No silica-containing materials. | Never | Non-silica dust, chemical exposure, pressure injection, manual handling. Standard controls only — no CCVS hold points. If cutting, grinding or drilling into concrete, masonry or tile — task moves to SIL. |

Rule: One task = one code. If a task involves multiple hazards (e.g. grinding on scaffold), assign the code with the highest consequence. The other hazard’s controls are still applied but the task is filed under the dominant code.

# 3. CCVS Trigger Logic

CCVS (Code-Category-Verification-Status) determines whether a task requires a pre-commencement HOLD POINT. The trigger is simple:

| CCVS TRIGGERED — HOLD POINT REQUIRED Critical category code (WAH, SIL, STR, ELE, HOT, HAZ, PLT) AND Consequence = 3 | NO CCVS — STANDARD CONTROLS SYS or PRE code (always) OR any code at Consequence = 1 or 2 |
| --- | --- |

Example: ELE at C=2 (power tools and leads) = standard controls, no hold point. ELE at C=3 (electrical isolation) = HOLD POINT required.

Example: WAH at C=3 (scaffold work) = HOLD POINT. SYS at C=1 (site induction) = standard controls.

# 4. HOLD POINT and STOP WORK Definitions

| Type | Definition | Record |
| --- | --- | --- |
| HOLD POINT | Pre-commencement gate. Work must not commence on this task until the stated condition is verified by the responsible person. This is a CCVS verification checkpoint. | Supervisor sign-off in site diary, Breadcrumb digital record, or ITP witness point (where applicable). Record must identify: who verified, what was verified, date and time. |
| STOP WORK | Reactive trigger. If the stated condition occurs during work, all work on this task ceases immediately until the condition is rectified and the task is re-assessed. | Incident/near-miss report in Breadcrumb. Record must identify: what triggered the stop, corrective action taken, who authorised recommencement. |

HOLD POINT appears on: CCVS tasks only (critical category at C=3).

STOP WORK appears on: Any task where conditions can change during work, regardless of CCVS status.

ITP alignment: HOLD POINT codes map directly to principal contractor Inspection and Test Plan hold points and witness points. This alignment will be formalised in Phase 2B.

# 5. Standard Task Control Structure

Every task in every RPD SWMS follows this structure. No exceptions.

| Task Header | [Number]. [Task Name] |
| --- | --- |
| Code + Risk | [CODE] — [Risk Level: Low / Medium / High] |
| Scope | One to two sentences describing what this task involves. |
| Engineering | Physical controls: barriers, extraction, guarding, isolation, anchors, edge protection. |
| Admin | Procedural controls: inspections, permits, competency, signage, communication, records. |
| PPE | Personal protective equipment specific to this task. |
| HOLD POINT | CCVS tasks only. Pre-commencement gate condition. If not met — STOP. |
| STOP WORK | Any task. Reactive trigger condition. If this occurs — cease immediately. |

Engineering controls always listed before Admin controls (hierarchy of controls). PPE is always last — it is the final line of defence, never the primary control.

# 6. Risk Scoring Methodology

Each task carries a risk level and a consequence score. These two values determine whether CCVS hold points apply.

| Risk Level | Consequence | CCVS? | Verification | Example |
| --- | --- | --- | --- | --- |
| Low | C=1 | No | Standard controls | SYS — Induction |
| Medium | C=2 | No | Standard controls | ELE — Power tools |
| High | C=3 | Yes (critical code) | HOLD POINT | WAH — Scaffold |

Note: High risk at C=3 only triggers CCVS if the code is a critical category (WAH, SIL, STR, ELE, HOT, HAZ, PLT). SYS and PRE are never CCVS regardless of any other factor.

# 7. Common Controls Library

Common controls are tasks that appear identically across multiple Master SWMS documents. They are written once in the library and pulled into each SWMS where they apply. This ensures every RPD worker sees the same language, same codes, and same controls regardless of which SWMS they are working under.

Current common controls: The following tasks are shared across two or more Master SWMS:

| # | Common Task | Code | Rem. | Paint | Spray | Gnd. | Clad. | Demo |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Site Induction and Daily Sign-In | SYS | Y | Y | Y | Y | Y | Y |
| 2 | Residents and Public Interface | SYS | Y | Y | Y | Y | Y | Y |
| 3 | Scaffold — Erection, Use, Dismantling | WAH | Y | Y | Y | — | Y | — |
| 4 | Fall Restraint — Harness and Lanyard | WAH | Y | Y | Y | Y | Y | Y |
| 5 | Drilling, Cutting, Grinding — Silica | SIL | Y | Y | — | Y | Y | Y |
| 6 | Surface Preparation (no silica) | PRE | Y | Y | — | — | — | — |
| 7 | Electrical Tools and Extension Leads | ELE | Y | Y | Y | Y | Y | — |
| 8 | Emergency Response | SYS | Y | Y | Y | Y | Y | Y |

Master-specific tasks (e.g. Demolition — Tile Bed Removal in Remedial Works, or Spray Booth Setup in Spray Painting) are written within that Master SWMS only and do not appear in the common library.

Rule: If a control appears in 2 or more Master SWMS, it must be sourced from the common library. No manual duplication.

# 8. Audit Trail

The 9-code system creates a direct audit path from any SWMS task to its verification record. An FSC auditor, SafeWork NSW inspector, or principal contractor superintendent follows this chain:

| Step | Auditor Action | Evidence Location |
| --- | --- | --- |
| 1 | Identify task code on SWMS (e.g. WAH) | SWMS document — task row |
| 2 | Check risk assessment: code + consequence score | SWMS risk assessment table |
| 3 | If C=3 + critical code → confirm HOLD POINT exists | SWMS document — task control column |
| 4 | Check HOLD POINT was signed off before work commenced | Site diary, Breadcrumb, or ITP record |
| 5 | If STOP WORK triggered → check incident/near-miss report | Breadcrumb incident module |
| 6 | Verify worker competency for task code (e.g. HRWL for PLT) | Breadcrumb competency module or licence register |

Key principle: The code is the index. Every query an auditor can ask — “show me your height work controls,” “show me your silica management,” “show me your hold point records” — is answered by filtering on the code.

# 9. Document Hierarchy

This governance layer sits above all SWMS documents. Changes flow downward.

| Layer | Document | Purpose |
| --- | --- | --- |
| 1 | Architecture Governance Layer (this document) | Defines codes, CCVS logic, control structure, audit trail. Single source of truth. |
| 2 | Common Controls Library | Reusable task definitions shared across Master SWMS. Written once, inherited by all. |
| 3 | Master SWMS (5 documents) | Remedial Works, Painting Works, Spray Painting, Groundworks, Cladding Works. Each contains common + specific tasks. |
| 4 | Standalone SWMS (11 documents) | EWP, Scaffold, Swing Stage, Abrasive Blasting, Sandblasting, Roofing & Guttering, Confined Space, Plumbing Works, Electrical Works, Hot Works, Demolition of Load Bearing Structures. |
| 5 | Inspection and Test Plans (future) | HOLD POINT codes from SWMS map to ITP hold points and witness points. Connects RPD verification to PC verification. |

# 10. Version Control

| Version | Date | Author | Change Description |
| --- | --- | --- | --- |
| 1.0 | 24 Feb 2026 | Alan Richardson | Initial release. 9-code system, CCVS trigger logic, HOLD POINT / STOP WORK framework, common controls library, audit trail, document hierarchy. |

— End of Document —