| Job Description: | Grinding | Date: | 01-06-25 | Risk Rating: Medium |
| --- | --- | --- | --- | --- |
| Name, & Contact Details: Robertson’s Remedial and Painting Pty Ltd # Unit 10/56 Buffalo Road Gladesville NSW 2111 Phone: (02) 9181 3519 A.B.N 16 140 746 247 |  | Site Location: 6a Francis road, Bandai Beach |  |  |
| Name & Phone Number of Person Completing SWI: Ben Knox (0408 253 832) Name & Phone Number of the Person Supervising the Work: Dave Oste - 0476 265 066 |  |  |  |  |
| Comments: This Safe Work Instruction (SWI) is to be used in conjunction with the Safe Work Method Statement for the task being performed. Operating instructions from the supplier of plant are to be obtained and followed. |  |  |  |  |

| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| --- | --- | --- | --- | --- | --- |
| Setting up the job | Inappropriate use of grinder | Medium (3) | Only use grinder where other more effective tools could not be used such as drop saws. Ensure work area is clean, tidy and well lit. Do not use in wet weather or rain and mop up any spills in the area. The material being used must be secured so that it cannot move. The area needs to be screened off (when there is a risk to safety) with welding screens so that sparks do not land on others or on flammable materials. All flammable materials are to be removed from the area or appropriately covered. Welding apron to be worn to prevent clothes catching on fire where necessary. Ensure all workers are moved away to a safe distance. Use correct manual handling when lifting and use lifting devices where needed. | Low (1) | Workers |
| Checking the grinder | The grinding disk may break and hit workers at high speed causing serious injury | Medium (3) | Cutting wheels or discs should not be used for grinding jobs, and grinding wheels should not be used for cutting jobs. Wheels designed for a particular revolution speed should not be used on machines of different speeds. Wheels should be used only for the specific material and purpose for which they are designed, and according to the manufacturer's recommendations. Wheels worn small through use should be discarded and NEVER used on smaller machines. Ensure the correct flange and locking nut is in place. Ensure the guard and handles are secure. Ensure there are no defects or damage to the disc. | Low (2) | Workers |
| Connecting power | Possible electrical hazard if the device or power cord is damaged | High (6) | Prior to use all electrical grinders need to be checked for an electrical test tag to ensure that it is current and has been passed. The power lead and tool should be checked for damage prior to use. Ensure that the power outlet is not damaged. Do not run power leads across paths or in areas where they may be damaged (e.g. run over) | Low (2) | Workers |
| Operating grinder | Shavings or dust | Medium (3) | Always wear safety glasses or goggles. | Low (1) | Workers |
|  | Noise | Medium (3) | Hearing protection to be worn at all times | Low (1) | Workers |
| Operating grinder | Grinding disk or grinder may come into contact with workers Kick back | Medium (4) | Stay alert, watch what you are doing and use common sense when grinding Do not use while tired or under the influence of drugs, alcohol, or medication. Two hands should always be used to operate grinder, including small models. Large angle grinders should always have a side handle. Allow the grinder to "run up" to operating speed before applying it to the job. Hold the grinder against the work piece with minimum pressure, so it doesn't "dig in" and cause it to kick back. Never bump the grinder on to the job, or let the disc hit any other object while grinding. Keep the grinding disc at a 15 to 30 degree angle to the work. Adopt a comfortable stance, with feet apart and well balanced, and with a clear view of the job. Never use a grinder between the legs while sitting on the floor. Disconnect the power or air hose and place the grinder on a bench with the disc facing upwards when not in use. Never put a grinder down until the disc stops rotating. Remove the plug from the power point or the air hose before changing discs. Use correct manual handling technique and lifting devices where needed | Low (2) | Workers |
| Packing up job | Housekeeping | Medium (3) | All tools to be checked for damage and then returned to appropriate storage position out of reach of children and untrained personnel. Power leads and air hoses must be rolled up and stored appropriately. All metal or other filings should be cleaned up and disposed of. All PPE must be returned to storage position. | Low (2) | Workers |

| Personal Protective Equipment (PPE) required: | Enclosed leather steel capped boots (As per AS/NZS 2210) Safety Glasses (As per AS/NZS 1336) Hearing protection (As per AS/NZS 1269) |
| --- | --- |
| Training and Qualifications required by workers to complete the work: | General Construction induction certificate (White card) Induction into SWI |
| List the Permits, Certificates, SafeWork NSW Approvals, required to complete the work: |  |
| Relevant legislation: For further details refer to the Legal Requirements Register | Work in accordance with Work Health and Safety Act 2011 and Work Health and Safety Regulation 2025, workplace injury management and workers compensation act 1998, Workers Compensation Regulation, AS/NZS1337.1 Personal eye protection-Eye and face protectors for occupational applications, AS/NZS 1269.3 Occupational Noise management-Hearing protection program, AS/NZS 1801 industrial safety helmets, AS/NZS 2210.1 Safety, protective and occupational footwear-Guide to selection, care and use (2010) & AS/NZS 2210.2 to 2210.9 Occupational protective footwear, AS/NZS 3820 Essential safety requirements for electrical equipment. Safe Work Australia Codes of Practice - Construction Work - Work Health and Safety Consultation, Co-operation and Co-ordination - Managing the Work Environment and Facilities - First Aid in the Workplace - Hazardous Manual Tasks - Managing Noise and Preventing Hearing Loss at Work - How to manage Work Health and Safety Risks |
| Hazardous Substances |  |
| Plant/equipment that will be used on site: Maintenance checks for plant & equipment: | Grinder Power tools inspected and tagged monthly |
| Frequency of site/workplace inspections: | Daily site hazard analysis inspection completed by site supervisor |
| The control measures implemented will be reviewed in consultation with workers and or others who may be affected by the SWI. Any changes to the SWI will be communicated with workers at induction, daily pre-starts and toolbox talks. |  |

# Sign Off and Approvals

| Person responsible for site supervision of the work, inspecting and approving work areas, approval of and compliance with SWI, protective measures, plant, equipment: |  |
| --- | --- |
| Name and position: | Signature: |
| SWI Prepared by: Ben Knox – Compliance OH&S | Signature: |

| Risk Level | Description of consequence or impact | Consequence | Likelihood/Probability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Unlikely (1) | Possible (2) | Almost Certain (3) |
| High Level of harm | Actual/Potential fatality, disability or irreversible damage. Major structural failure/damage. Off-site environmental discharge/release not contained and significant long-term environmental harm. | Major (3) | Medium (3) | High (6) | High (9) |
| Medium Level of harm | Actual/Potential temporary disability, MTI or LTI. Structural failure/damage, >1-day outage. On-site environmental discharge/release contained, minor remediation, short-term environmental harm. | Moderate (2) | Low (2) | Medium (4) | High (6) |
| Low Level of harm | Incident that has the potential to cause persons to require first aid. Environmental discharge/release immediately contained, minor level clean-up with no short term environmental harm. | Minor (1) | Low (1) | Low (2) | Medium (3) |
| Level | Likelihood/Probability |  |  |  |  |
| Almost Certain | Occurs frequently; >66% chance of occurring |  |  |  |  |
| Possible | Could happen occasionally; >33% but <66% chance of occurring |  |  |  |  |
| Unlikely | May occur only in exceptional circumstances; <33% chance of occurring |  |  |  |  |
| Class/Ranking | Description/Requirements |  |  |  |  |
| High 6, 9 | Stop immediately. Implement permanent preventative controls. |  |  |  |  |
| Medium 3, 4 | Will require operational planning. Actions will be recorded on a SWI. |  |  |  |  |
| Low 1, 2 | Will require detailed pre-planning. Actions will be recorded on a SWI. Will require localised control measures. |  |  |  |  |

SWI has been developed using the hierarchy of controls to reduce the risk and likelihood of an incident/injury occurring. The use of Elimination, Substitution,

Engineering/isolation, Administrative and PPE measures will be used in the above order of preference wherever practicable.

Induction confirmation:

This SWI has been developed in consultation with the workers and has been read and signed by all workers involved in this activity. I note that I have read and understood the above safe work instruction and will ensure work is completed accordingly.

| Date: | Name: | Signature: |
| --- | --- | --- |