| Job Description: | Core Drilling | Date: | 06-02-2026 | Risk Rating: Medium |
| --- | --- | --- | --- | --- |
| Name, & Contact Details: Robertson’s Remedial and Painting Pty Ltd # Unit 10/56 Buffalo Road Gladesville NSW 2111 Phone: (02) 9181 3519 A.B.N 16 140 746 247 |  | Site Location: 6a Francis Street, Bondi Beach |  |  |
| Name & Phone Number of Person Completing SWI: Ben Knox (0408 253 832) Name & Phone Number of the Person Supervising the Work: Dave Oste - 0476 265 066 |  |  |  |  |
| Comments: This Safe Work Instruction (SWI) is to be used in conjunction with the Safe Work Method Statement for the task being performed. Operating instructions from the supplier of plant are to be obtained and followed. |  |  |  |  |

| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| --- | --- | --- | --- | --- | --- |
| Complete pre start check on drill | The drill may not be working appropriately and could cause electrical shock or injury | Medium (3) | Structural approval to be obtained prior to drilling. Ensure there are no defects or damage to the drill. Ensure that the appropriate drill bit is used for the material being drilled. Prestart visual inspection of equipment to be completed prior to commencement. Check machine for leaks and damage and remove from service or repair if damaged. Ensure all fittings are securely fixed. Ensure the handle is secure. Ensure chuck and drill bit are appropriately secured. Keep all body parts, clothing and hair clear of moving machinery. Ensure machine has current test tag and is plugged into RCD/ELCB. | Low (1) | Workers & Supervisor |
| Connecting power | Possible electrical hazard if the drill or power cord is damaged | High (6) | Prior to use the drill must be checked for an electrical test tag to ensure that it is current and has been passed. Check the power lead and the drill for damage prior to use. Ensure that the power outlet is not damaged. Do not run power leads or hoses across paths or in areas where they may be damaged (eg run over). RCD must be used at all times. | Low (2) | Workers & Supervisor |
| Operating core drill to drill pilot hole and secure drill to surface | Injury due to equipment failure | Medium (3) | Pre start visual inspection of equipment to be completed prior to commencement. Ensure work area is clean, tidy and well lit. Ensure all individuals are moved away to a safe distance. Ensure water does not run into machine and cause damage. Use wet vac to collect waste | Low (1) | Workers |
|  | Hearing may be damaged due to noise | Medium (3) | Hearing protection to be worn at all times | Low (1) | Workers |
|  | Eyes may be damaged | Medium (3) | Always wear safety glasses or goggles | Low (1) | Workers |
|  | Dust inhalation | Medium (3) | Water to be used to minimise dust Where dust is present operators to wear dust mask Wet vac and slurry bags to be used to contain waste | Low (1) | Workers |
| Drill pilot hole and secure drill to surface | Cuts, bruises or fractures if body parts come into contact with drill | Medium (4) | Stay alert, watch what you are doing and use common sense when drilling Do not use while tired or under the influence of drugs, alcohol, or medication. Two hands always used to operate drill, including small models. Select drill bit appropriate for work ie sharp masonry bit. Allow the drill to "run up" to operating speed before applying it to the job. Hold the drill against the work piece with minimum pressure, so it doesn't "dig in" and cause it to kick back. Adopt a comfortable stance, with feet apart and well balanced, and with a clear view. Never use a drill between the legs while sitting on the floor. Disconnect the power and place the drill on a bench when not in use. Never put a drill down until the bit stops rotating. Remove the power before changing bits. Never touch any parts that are hot. Never wear leather gloves. Ensure that all clothing is close fitting and that any lose clothing, jewellery or hair is secured and out of the way or the drill. Frequently back the drill out of deep cuts to clean and cool the bit. | Low (2) | Workers |
|  | Electrocution or injury due to exposure to services or cables | High (6) | Prior to commencement of work isolate lock out and tag out necessary services. Prior to commencement of work any underground services to be identified and located All workers to allow for inaccuracies in the plans and drill with caution Once identified services should be clearly marked and isolated. Operator to take care while drilling to spot any cables etc | Low (2) | Workers |
| Operating Core Drill | Injury due to equipment failure | Medium (3) | As above for drilling pilot hole as well as the following: Ensure water supply is appropriate to keep bit cool and prevent dust. Check drill bit to ensure it is sharp and not damaged. Ensure suction pad is working appropriately to prevent machine falling. Ensure lock nut is tight to prevent machine moving. Ensure clutch is working appropriately. Stop and allow drill to cool down when drilling for long periods Ensure fire extinguisher is available at all times Ensure water does not run into machine and cause damage. | Low (1) | Workers |
|  | Cuts, bruises or fractures if body parts come into contact with drill | Medium (4) | Stay alert, watch what you are doing and use common sense when drilling Do not use while tired or under the influence of drugs, alcohol, or medication. Two hands always used to operate drill, including small models. Select drill bit appropriate for work ie sharp masonry bit. Allow the drill to "run up" to operating speed before applying it to the job. Hold the drill against the work piece with minimum pressure, so it doesn't "dig in" and cause it to kick back. Adopt a comfortable stance, with feet apart and well balanced, and with a clear view. Never use a drill between the legs while sitting on the floor. Disconnect the power and place the drill on a bench when not in use. Never put a drill down until the bit stops rotating. Remove the power before changing bits. Never touch any parts that are hot. Never wear leather gloves. Ensure that all clothing is close fitting and that any lose clothing, jewellery or hair is secured and out of the way or the drill. Frequently back the drill out of deep cuts to clean and cool the bit. | Low (2) | Workers |
| Operating Core Drill | Eyes may be damaged | Medium (3) | Always wear safety glasses or goggles | Low (1) | Workers |
|  | Hearing damage | Medium (3) | Hearing protection to be worn while drill is in operation. | Low (1) | Workers |
|  | Manual Handling injury while removing cores | Medium (4) | Materials to be delivered directly to site where possible. Correct manual handling procedures to be followed: Use mechanical lifting devices and trolleys where possible Only lift objects within your safe lifting limit (use team lifts where possible) Clear a path, assess the load, get a good grip, ensure a wide stance, bend the knees, keep the spine strait, use the legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely | Low (1) | Workers |
|  | Injury to workers from core falling | Medium (4) | Spotter to be in place to observe area under slab to ensure persons do not enter work area. Determine the location where core will drop. Drill a pilot hole wherever possible. Plug the pilot hole to prevent water flowing below. Spotter to communicate with the driller prior to drilling and then when hole is 50mm from completion. Secure the core catcher to the underside of the slab No persons to stand directly under the core hole drilling area. Secure off work area around spotter and driller. A hard hat must be worn by the spotter at all times. Only trained and competent workers to operate core drill. Once the hole is completed remove the core catcher and remove the core. Never attempt to catch the core if it has not fallen into the core catcher allow it to drop into the barricaded area. | Low (2) | Workers |
| Packing up job | Workers may trip over items not packed away correctly causing injury | Low (2) | All tools to be checked for damage and then returned to appropriate storage position out of reach of children and untrained personnel. Power leads must be rolled up and stored appropriately. All waste should be cleaned up and disposed of. All PPE must be returned to storage position. | Low (1) | Workers |

| Personal Protective Equipment (PPE) required: | Enclosed steel capped boots (As per AS/NZS 2210) Safety Glasses (As per AS/NZS 1337) Hearing protection (As per AS/NZS 1269) |
| --- | --- |
| Training and Qualifications required by workers to complete the work: | General Construction induction certificate (White card) Relevant trade certificate Induction into SWI |
| List the Permits, Certificates, SafeWork NSW Approvals, required to complete the work: |  |
| Relevant legislation: For further details refer to the Legal Requirements Register | Work in accordance with Work Health and Safety Act 2011 and Work Health and Safety Regulation 2017, workplace injury management and workers compensation act 1998, Workers Compensation Regulation 2016, AS/NZS1337.1 Personal eye protection-Eye and face protectors for occupational applications, AS/NZS 1801 industrial safety helmets (1997), AS/NZS 2210.1 Safety, protective and occupational footwear-Guide to selection, care and use (2010). Safe Work Australia Codes of Practice - Construction Work - Work Health and Safety Consultation, Co-operation and Co-ordination - Managing the Work Environment and Facilities - First Aid in the Workplace - Hazardous Manual Tasks - Managing Noise and Preventing Hearing Loss at Work - How to manage Work Health and Safety Risks |
| Hazardous Substances |  |
| Plant/equipment that will be used on site: Maintenance checks for plant & equipment: | Core Drill Prestart inspections, tested and tagged and regular servicing as per manufacturers instructions |
| Frequency of site/workplace inspections: | Daily site inspections to be completed by Site Supervisor |
| The control measures implemented will be reviewed in consultation with workers and or others who may be affected by the SWI. Any changes to the SWI will be communicated with workers at induction, daily pre-starts and toolbox talks. |  |

# Sign Off and Approvals

| Person responsible for site supervision of the work, inspecting and approving work areas, approval of and compliance with SWI, protective measures, plant, equipment: |  |
| --- | --- |
| Name and position: | Signature: |
| SWI Prepared by: Ben Knox – Compliance OH&S | Signature: |

| Risk Level | Description of consequence or impact | Consequence | Likelihood/Probability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Unlikely (1) | Possible (2) | Almost Certain (3) |
| High Level of harm | Actual/Potential fatality, disability or irreversible damage. Major structural failure/damage. Off-site environmental discharge/release not contained and significant long-term environmental harm. | Major (3) | Medium (3) | High (6) | High (9) |
| Medium Level of harm | Actual/Potential temporary disability, MTI or LTI. Structural failure/damage, >1-day outage. On-site environmental discharge/release contained, minor remediation, short-term environmental harm. | Moderate (2) | Low (2) | Medium (4) | High (6) |
| Low Level of harm | Incident that has the potential to cause persons to require first aid. Environmental discharge/release immediately contained, minor level clean-up with no short term environmental harm. | Minor (1) | Low (1) | Low (2) | Medium (3) |
| Level | Likelihood/Probability |  |  |  |  |
| Almost Certain | Occurs frequently; >66% chance of occurring |  |  |  |  |
| Possible | Could happen occasionally; >33% but <66% chance of occurring |  |  |  |  |
| Unlikely | May occur only in exceptional circumstances; <33% chance of occurring |  |  |  |  |
| Class/Ranking | Description/Requirements |  |  |  |  |
| High 6, 9 | Stop immediately. Implement permanent preventative controls. |  |  |  |  |
| Medium 3, 4 | Will require operational planning. Actions will be recorded on a SWI. |  |  |  |  |
| Low 1, 2 | Will require detailed pre-planning. Actions will be recorded on a SWI. Will require localised control measures. |  |  |  |  |

SWI has been developed using the hierarchy of controls to reduce the risk and likelihood of an incident/injury occurring. The use of Elimination, Substitution,

Engineering/isolation, Administrative and PPE measures will be used in the above order of preference wherever practicable.

Induction confirmation:

This SWI has been developed in consultation with the workers and has been read and signed by all workers involved in this activity. I note that I have read and understood the above safe work instruction and will ensure work is completed accordingly.

| Date: | Name: | Signature: |
| --- | --- | --- |