| Job Description: |  | Welding/Oxy |  |  | Date: |  | Risk Rating: Medium |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name, & Contact Details: Robertson’s Remedial and Painting Pty Ltd # Unit 10/56 Buffalo Road Gladesville NSW 2111 Phone: (02) 9181 3519 A.B.N 16 140 746 247 |  |  |  |  | Site Location: |  |  |  |  |
| Name & Phone Number of Person Completing SWI: Ben Knox (0408 253 832) Name & Phone Number of the Person Supervising the Work: |  |  |  |  |  |  |  |  |  |
| Comments: This Safe Work Instruction (SWI) is to be used in conjunction with the Safe Work Method Statement for the task being performed. Operating instructions from the supplier of plant are to be obtained and followed. |  |  |  |  |  |  |  |  |  |
| Procedure (In Steps): | Possible Hazards: |  | Risk Score | Safety Controls: |  |  |  | Residual Risk | Responsible Person |
| Setting up the job | When using the oxy sparks and UV rays may come into contact with another worker or the worker using the oxy |  | Medium (3) | Welding gloves, long sleeve pants and shirts, must be worn at all times. Where necessary welding apron, hood, welding face shield and leather sleeves to be worn. Ensure work area is clean, tidy and well lit. Workers to follow hot work procedure and ensure stand by person is in place and extinguisher is at hand. Ensure that the material is secured. Where necessary the area is to be screened off with welding screens so that sparks do not land on others or on flammable materials. All flammable materials are to be removed from the area or appropriately covered do not operate in explosive atmospheres. Cover or remove all cables, hoses and wires that may be damaged by the work Ensure all individuals are moved away to a safe distance. Work in a well ventilated area or if this is not possible use respirator. |  |  |  | Low (1) | Workers and Supervisor |
| Procedure (In Steps): | Possible Hazards: |  | Risk Score | Safety Controls: |  |  |  | Residual Risk | Responsible Person |
| Complete pre start check on equipment | There may be a gas leak that could cause an explosion causing burns and injury |  | High (6) | Check the system for leaks by pressurizing to 100kpa, use pressure drop method and locate leaks using soapy water. Fittings must be free of grease or oil. Ensure the gas bottles are secure at all times. Ensure there are no defects or damage to the oxy. Check all fittings are tight. Ensure flashback arrester is fitted. |  |  |  | Low (2) | Workers |
| Connecting hoses and tools | Possible fire if the device or hoses are damaged |  | Medium (3) | Prior to use the device needs to be checked for damage and leaks, check the hoses, bottles and equipment. Do not run hoses across paths or in areas where they may be damaged (eg run over). Ensure bottles are chained to the trolley. Take care with transport and place on level surface. |  |  |  | Low (2) | Workers |
| Operating device | “Flash burns” to eyes Burns to skin from “slag” Hazardous Fumes & Manual handling risk |  | Medium (3) | Stay alert, watch what you are doing and use common sense when operating Do not use while tired or under the influence of drugs, alcohol, or medication. Ensure that the metal is secure. Adopt a comfortable stance, with feet apart and well balanced, and with a clear view. Never use between the legs while sitting on the floor. Keep all fumes and gases from the breathing area and ensure adequate ventilation. Never touch any parts that are hot. Use correct manual handling technique and trolley Welding jacket and protective headwear to be worn where necessary. Set flows to appropriate levels. Always light with lint lighter or pilot light. Acetylene first then turn on oxy. Heat up metal prior to cutting/welding. Ensure that off cuts or slag is secured or are falling in safe area. Avoid molten metal landing on body parts or flammable materials. |  |  |  | Low (2) | Workers |
| Procedure (In Steps): | Possible Hazards: |  | Risk Score | Safety Controls: |  |  |  | Residual Risk | Responsible Person |
| Turning of device. Shut down. | Explosion Burns Escaping gas Fire |  | Low (2) | Close acetylene valve first then oxy valve. Release gas from regulators and lines. Close cylinder valves. Back off regulators Check job for burning or smoking material |  |  |  | Low (1) | Workers |
| Packing up job | Workers may trip over items not packed away correctly causing injury |  | Low (2) | All tools to be checked for damage and then returned to appropriate storage position out of reach of untrained personnel. Power leads must be rolled up and stored appropriately. All metal or other filings should be cleaned up and disposed of. All PPE must be returned to storage position. |  |  |  | Low (1) | Workers |

| Personal Protective Equipment (PPE) required: | Enclosed leather steel capped boots (As per AS/NZS 2210) Leather gloves (As per AS/NZS 2161) Welding glasses and apron |
| --- | --- |
| Training and Qualifications required by workers to complete the work: | Induction into SWI |
| List the Permits, Certificates, SafeWork NSW Approvals, required to complete the work: |  |
| Relevant legislation: For further details refer to the Legal Requirements Register | Work in accordance with Work Health and Safety Act 2011 and Work Health and Safety Regulation 2017, Workplace injury management and workers compensation act 1998, Workers Compensation Regulation 2016, AS/NZS1337.1 Personal eye protection-Eye and face protectors for occupational applications (2010), AS/NZS 1801 industrial safety helmets (1997), AS/NZS 2210.1 Safety, protective and occupational footwear-Guide to selection, care and use (2010) & AS/NZS 2210.2 to 2210.9 Occupational protective footwear. Safe Work Australia Codes of Practice - Construction Work - Work Health and Safety Consultation, Co-operation and Co-ordination - Managing the Work Environment and Facilities - First Aid in the Workplace - Hazardous Manual Tasks - Managing Noise and Preventing Hearing Loss at Work - How to manage Work Health and Safety Risks |
| Hazardous Substances | Oxygen, Acetylene |
| Plant/equipment that will be used on site: | Oxy Welder or cutter |
| Maintenance checks for plant & equipment: | Pre-start check and monthly maintenance to be completed on equipment |
| Frequency of site/workplace inspections: | Daily site inspections to be completed by Site Supervisor |
| The control measures implemented will be reviewed in consultation with workers and or others who may be affected by the SWI. Any changes to the SWI will be communicated with workers at induction, daily pre-starts and toolbox talks. |  |

# Sign Off and Approvals

| Person responsible for site supervision of the work, inspecting and approving work areas, approval of and compliance with SWI, protective measures, plant, equipment: |  |
| --- | --- |
| Name and position: | Signature: |
| SWI Prepared by: Ben Knox – Compliance OH&S | Signature: |

| Risk Level | Description of consequence or impact | Consequence | Likelihood/Probability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Unlikely (1) | Possible (2) | Almost Certain (3) |
| High Level of harm | Actual/Potential fatality, disability or irreversible damage. Major structural failure/damage. Off-site environmental discharge/release not contained and significant long-term environmental harm. | Major (3) | Medium (3) | High (6) | High (9) |
| Medium Level of harm | Actual/Potential temporary disability, MTI or LTI. Structural failure/damage, >1-day outage. On-site environmental discharge/release contained, minor remediation, short-term environmental harm. | Moderate (2) | Low (2) | Medium (4) | High (6) |
| Low Level of harm | Incident that has the potential to cause persons to require first aid. Environmental discharge/release immediately contained, minor level clean-up with no short term environmental harm. | Minor (1) | Low (1) | Low (2) | Medium (3) |
| Level | Likelihood/Probability |  |  |  |  |
| Almost Certain | Occurs frequently; >66% chance of occurring |  |  |  |  |
| Possible | Could happen occasionally; >33% but <66% chance of occurring |  |  |  |  |
| Unlikely | May occur only in exceptional circumstances; <33% chance of occurring |  |  |  |  |
| Class/Ranking | Description/Requirements |  |  |  |  |
| High 6, 9 | Stop immediately. Implement permanent preventative controls. |  |  |  |  |
| Medium 3, 4 | Will require operational planning. Actions will be recorded on a SWI. |  |  |  |  |
| Low 1, 2 | Will require detailed pre-planning. Actions will be recorded on a SWI. Will require localised control measures. |  |  |  |  |

SWI has been developed using the hierarchy of controls to reduce the risk and likely hood of an incident/injury occurring. The use of Elimination, Substitution,

Engineering/isolation, Administrative and PPE measures will be used in the above order of preference wherever practicable.

Induction confirmation:

This SWI has been developed in consultation with the workers and has been read and signed by all workers involved in this activity. I note that I have read and understood the above safe work instruction and will ensure work is completed accordingly.

| Date: | Name: | Signature: |
| --- | --- | --- |