| Job Description: |  | High Pressure Water Jetting |  |  | Date: |  | Risk Rating: Medium |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name, & Contact Details: Robertson’s Remedial and Painting Pty Ltd # Unit 10/56 Buffalo Road Gladesville NSW 2111 Phone: (02) 9181 3519 A.B.N 16 140 746 247 |  |  |  |  | Site Location: |  |  |  |  |
| Name & Phone Number of Person Completing SWI: Ben Knox (0408 253 832) Name & Phone Number of the Person Supervising the Work: |  |  |  |  |  |  |  |  |  |
| Comments: This Safe Work Instruction (SWI) is to be used in conjunction with the Safe Work Method Statement for the task being performed. Operating instructions from the supplier of plant are to be obtained and followed. |  |  |  |  |  |  |  |  |  |
| Procedure (In Steps): | Possible Hazards: |  | Risk Score | Safety Controls: |  |  |  | Residual Risk | Responsible Person |
| Pre start checks | Disorderly work area may result in injury |  | Medium (3) | Ensure work area is clean, tidy and well lit. Ensure all individuals are moved away to a safe distance and work area is isolated. Only manoeuvre on a smooth surface, ensure all penetrations are covered or secured. Remove all obstacles in the path of travel. Do not operate tools in explosive atmosphere such as areas with flammable liquids, gases or dust. Ensure all equipment near jetting operations (particularly electrical equipment) should be shielded/protected or isolated from debris and the ingress of water from operating the equipment. Work activities should be planned to provide safe access to the equipment or surface being jetted. Ensure barricading and signage requirements are in place. Ensure hoses are protected at all times from being run over, crushed by vehicles or melted. Ensure there is no apparent structural damage to the hose, e.g. corroded or broken wires, bulging, kinking, twists, cuts or flat spots. Check all end fittings and couplings are in good order and of the correct pressure rating for the unit. Check all jetting nozzles are kept clean and stored safely. Ensure the outlets are not damaged or obstructed. All hose assemblies are to be visually inspected prior to each use to ensure that the correct pressure rating and size is selected. |  |  |  | Low (1) | Workers |
| Procedure (In Steps): | Possible Hazards: |  | Risk Score | Safety Controls: |  |  |  | Residual Risk | Responsible Person |
| Pre start checks | Exposure to hazardous materials/chemicals from jetting |  | Medium (3) | An SDS should be obtained before starting water jetting operations to identify if hazardous materials are present in the substrate or coating being jetted. If jetting is to be conducted on building materials and the building was constructed prior to 2004, the asbestos register must be reviewed. Use of water jets on asbestos-containing materials is prohibited. |  |  |  | Low (2) | Worker |
|  | Injuries to Workers and the public |  | High (6) | Implement barricades and signage around the work area prior to commencing cleaning operations. All persons other than the operator must be removed from work area. Keep work area clear of loose items and debris to prevent tripping and slipping hazards. All injuries to be reported immediately and medical assistance sought immediately. If an incident occurs where pressurised water penetrates or appears to have penetrated the skin, medical attention must be sought immediately. |  |  |  | Low (2) | Workers |
|  |  |  | Medium (3) | All incidents or injuries, whether resulting in lost time or no lost time should be recorded, also near misses should be recorded so that any potential problems can be identified and measures implemented to minimize or eliminate these potential hazards. |  |  |  | Low (2) | Workers |
|  | The equipment may not be working appropriately or may fail causing serious injury |  | Medium (3) | Ensure that equipment is sourced from professional companies that have an excellent safety record. Ensure the manufacturers guidelines/recommendations are followed. Ensure the equipment is inspected in accordance with the manufacturer’s recommendations. Prior to operating, operator to conduct inspection to ensure safe for use including checking mechanical and safety devices. Operators must be trained and competent in the use of equipment. Check the currency of hose tags or certificates. Do not expose tools to wet conditions. Ensure all safety guards and mechanisms are in proper working order with current test tag. |  |  |  | Low (1) | Workers |
| Procedure (In Steps): | Possible Hazards: |  | Risk Score | Safety Controls: |  |  |  | Residual Risk | Responsible Person |
| Pre start checks | The equipment may not be working appropriately or may fail causing serious injury |  | Medium (3) | Ensure that equipment is sourced from professional companies that have an excellent safety record. Ensure the manufacturers guidelines/recommendations are followed. Ensure the equipment is inspected in accordance with the manufacturer’s recommendations. Prior to operating, operator to conduct inspection to ensure safe for use including checking mechanical and safety devices. Operators must be trained and competent in the use of equipment. Check the currency of hose tags or certificates. Do not expose tools to wet conditions. Ensure all safety guards and mechanisms are in proper working order with current test tag. |  |  |  | Low (1) | Workers |
|  | Eye injury |  | High (6) | Approved safety goggles or face shields to be worn at all times. High-pressure hose never to be pointed towards individuals. |  |  |  | Low (2) | Workers |
|  | Cuts/abrasions or injury from high pressure hose |  | High (6) | Appropriate protective footwear to be worn at all times (steel capped boots). Workers to wear long pants or overalls at all times. Workers to wear approved gloves at all times. Workers never to point hose in direction of others. All persons other than the operator must be removed from the work area- minimum of a 3 metre distance. Jetting operations to stop if an unauthorised or inadequately protected person enters the work area. Maintain grip on hose with two hands at all times. Be aware of public or others accessing site. |  |  |  | Low (2) | Workers |
|  | Equipment failure |  | High (6) | Check machine for fuel leaks and damage and remove from service or repair if damaged. Store fuel in secure location. Ensure all hoses/fittings are securely fixed. Check the currency of hose tags or certificates. Keep all body parts. Clothing and hair clear of pump. Operators must be trained and competent in the use of equipment. Jetting operations to stop if a malfunction occurs. |  |  |  | Low (2) | Workers |
| Procedure (In Steps): | Possible Hazards: |  | Risk Score | Safety Controls: |  |  |  | Residual Risk | Responsible Person |
| Pre start checks | Pipe cleaning |  | High (6) | Workers should check pipe cleaning activities are set up so the nozzle cannot physically come out of the pipe when it is under pressure. Hoses should be clearly marked at a suitable distance from the nozzle to indicate the location of the nozzle as it is withdrawn from the pipe or tube. |  |  |  | Low (2) | Workers |
|  | Repetitive strain and muscular injury due to repetitive work and prolonged fixed postures |  | Medium (3) | Workers must warm up and stretch prior to commencing work. Workers to rotate tasks to avoid repetitive tasks and fixed postures. Workers to avoid excessive straining and over reaching. Workers to avoid overhead work where possible, as this may cause unstable worker positioning and increase the risk of musculoskeletal disorders. |  |  |  | Low (2) | Workers |
| Operation of high pressure water jet | Objects close to engine may catch fire |  | High (6) | Keep all combustible items a safe distance from engine. Ensure a clear area is maintained around the engine. |  |  |  | Low (2) | Workers |
|  | Asphyxiation from pump fumes |  | High (6) | Engine emits deadly carbon monoxide gas, use engine outdoors only, away from open windows, vents or doors. Never use inside homes, garages, crawl spaces, or other enclosed areas. Fumes that can be fatal can build up in these areas. The use of fans, and opening doors and windows does not provide adequate fresh air. |  |  |  | Low (2) | Workers |
|  | Hearing damage |  | Medium (3) | Hearing protection to be worn while blaster is in operation. |  |  |  | Low (1) | Workers |
|  | Skin cancer, sunburn and eye damage due to UV radiation exposure |  | Medium (3) | Workers must wear appropriate PPE including: sun cream, sun hats, long sleeve shirts, and sunglasses while working outside where necessary. |  |  |  | Low (2) | Workers |
|  | Heatstroke and dehydration |  | Medium (3) | Workers must remain hydrated through regularly drinking water. Workers to stay out of sun where possible and implement rest breaks or job rotation where necessary. Workers to consider stopping work in extreme heat. |  |  |  | Low (2) | Workers |
| Procedure (In Steps): | Possible Hazards: |  | Risk Score | Safety Controls: |  |  |  | Residual Risk | Responsible Person |
| Operation of high pressure water jet | Dirt, grit and/or dust in eyes / eye injury |  | Medium (3) | Workers must wear appropriate eye protection during pressure cleaning activities: clear safety glasses or a face mask. |  |  |  | Low (1) | Workers |
|  | Respiratory |  | Medium (3) | Workers must wear approved respiratory PPE (i.e. dust mask or respirator) during pressure cleaning when in a confined space and when exposed to drifting mist that may contain bacteria, chemicals or hazardous materials. |  |  |  | Low (1) | Workers |
|  | Noise |  | Medium (3) | Hearing protection to be worn during pressure cleaning (particularly whilst working next to engine or using a hand lance). |  |  |  | Low (1) | Workers |
| Wash down | Waste entering drains |  | Medium (3) | All drains to be covered with geo fabric. Silt sock to be placed around drain. Workers to monitor drains to ensure waste is not entering drains. |  |  |  | Low (2) | Workers |
| Disposal of Hazardous Chemicals (Decontamination of work area) | Contamination |  | Medium (3) | Refer to the SDS for appropriate safety instructions in relation to disposal of and cleaning up of the relevant product in order to avoid contamination. Wear appropriate PPE as listed in the SDS. |  |  |  | Low (2) | Workers |
| End Identification (when used as a drain cleaner) | Injuries to Workers |  | Medium (3) | Workers to ensure hoses are clearly marked at a nominal distance of 60cm (24”) from the nozzle t indicate the location of the nozzle. |  |  |  | Low (2) | Workers |
| Repairs being made to the system or components are being replaced/when not in use and left unattended | Injuries to Workers |  | Medium (3) | Depressurise the system prior to repairs being made and following completion of a job. Repairs to be completed by competent workers. |  |  |  | Low (2) | Workers |
| Packing up job | Workers may trip over items not packed away correctly causing injury |  | Medium (3) | All tools to be checked for damage and then returned to appropriate storage position out of reach of children and untrained personnel. Power leads must be rolled up and stored appropriately. All metal or other filings should be cleaned up and disposed of. All PPE must be returned to storage position. |  |  |  | Low (1) | Workers |
| Packing equipment away | Inadequate storage of equipment may cause damage or injury |  | Medium (3) | Ensure all power cords are unplugged and put in clean and safe area after use. Ensure work area has been thoroughly cleaned on completion of job. |  |  |  | Low (2) | Workers and Supervisor |

| Personal Protective Equipment (PPE) required: | Enclosed leather steel capped boots (As per AS/NZS 2210) Safety Glasses (As per AS/NZS 1336) Hearing protection (As per AS/NZS 1269)- where necessary |
| --- | --- |
| Training and Qualifications required by workers to complete the work: | General Construction induction certificate (White card) Induction into SWI |
| List the Permits, Certificates, SafeWork NSW Approvals, required to complete the work: |  |
| Relevant legislation: For further details refer to the Legal Requirements Register | Work in accordance with Work Health and Safety Act 2011 and Work Health and Safety Regulation 2017, workplace injury management and workers compensation act 1998, Workers Compensation Regulation 2016, AS/NZS1337.1 Personal eye protection-Eye and face protectors for occupational applications (2010), AS/NZS 1801 industrial safety helmets (1997), AS/NZS 2210.1 Safety, protective and occupational footwear-Guide to selection, care and use (2010) & AS/NZS 2210.2 to 2210.9 Occupational protective footwear. Safe Work Australia Codes of Practice - Construction Work - Work Health and Safety Consultation, Co-operation and Co-ordination - Managing the Work Environment and Facilities - First Aid in the Workplace - Hazardous Manual Tasks - Managing Noise and Preventing Hearing Loss at Work - How to manage Work Health and Safety Risks |
| Hazardous Substances |  |
| Plant/equipment that will be used on site: Maintenance checks for plant & equipment: | High Pressure Water Jet Power tools inspected and tagged monthly |
| Frequency of site/workplace inspections: | Daily site inspections to be completed by Site Supervisor |
| The control measures implemented will be reviewed in consultation with workers and or others who may be affected by the SWI. Any changes to the SWI will be communicated with workers at induction, daily pre-starts and toolbox talks. |  |

# Sign Off and Approvals

| Person responsible for site supervision of the work, inspecting and approving work areas, approval of and compliance with SWI, protective measures, plant, equipment: |  |
| --- | --- |
| Name and position: | Signature: |
| SWI Prepared by: Ben Knox – Compliance OH&S | Signature: |

| Risk Level | Description of consequence or impact | Consequence | Likelihood/Probability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Unlikely (1) | Possible (2) | Almost Certain (3) |
| High Level of harm | Actual/Potential fatality, disability or irreversible damage. Major structural failure/damage. Off-site environmental discharge/release not contained and significant long-term environmental harm. | Major (3) | Medium (3) | High (6) | High (9) |
| Medium Level of harm | Actual/Potential temporary disability, MTI or LTI. Structural failure/damage, >1-day outage. On-site environmental discharge/release contained, minor remediation, short-term environmental harm. | Moderate (2) | Low (2) | Medium (4) | High (6) |
| Low Level of harm | Incident that has the potential to cause persons to require first aid. Environmental discharge/release immediately contained, minor level clean-up with no short term environmental harm. | Minor (1) | Low (1) | Low (2) | Medium (3) |
| Level | Likelihood/Probability |  |  |  |  |
| Almost Certain | Occurs frequently; >66% chance of occurring |  |  |  |  |
| Possible | Could happen occasionally; >33% but <66% chance of occurring |  |  |  |  |
| Unlikely | May occur only in exceptional circumstances; <33% chance of occurring |  |  |  |  |
| Class/Ranking | Description/Requirements |  |  |  |  |
| High 6, 9 | Stop immediately. Implement permanent preventative controls. |  |  |  |  |
| Medium 3, 4 | Will require operational planning. Actions will be recorded on a SWI. |  |  |  |  |
| Low 1, 2 | Will require detailed pre-planning. Actions will be recorded on a SWI. Will require localised control measures. |  |  |  |  |

SWI has been developed using the hierarchy of controls to reduce the risk and likely hood of an incident/injury occurring. The use of Elimination, Substitution,

Engineering/isolation, Administrative and PPE measures will be used in the above order of preference wherever practicable.

Induction confirmation:

This SWI has been developed in consultation with the workers and has been read and signed by all workers involved in this activity. I note that I have read and understood the above safe work instruction and will ensure work is completed accordingly.

| Date: | Name: | Signature: |
| --- | --- | --- |