| Job Description: | Drop Saw |  |  | Date: |  | Risk Rating: Medium |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name & Contact Details: Robertson’s Remedial and Painting Pty Ltd # Unit 10/56 Buffalo Road Gladesville NSW 2111 Phone: (02) 9181 3519 A.B.N 16 140 746 247 |  |  |  | Site Location: |  |  |  |  |
| Name & Phone Number of Person Completing SWI: Ben Knox (0408 253 832) Name & Phone Number of the Person Supervising the Work: |  |  |  |  |  |  |  |  |
| Comments: This Safe Work Instruction (SWI) is to be used in conjunction with the Safe Work Method Statement for the task being performed. Operating instructions from the supplier of plant are to be obtained and followed. |  |  |  |  |  |  |  |  |
| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: |  |  |  | Residual Risk | Responsible Person |
| Preparing for use of drop saw | When using the drop saw the item being cut may come lose and hit operator or another workers. Manual handling risk | Medium (3) | Ensure work area is clean, tidy and well lit. The material being used must be secured to prevent movement. Ensure all individuals are moved away to a safe distance. Secure lose clothing and long hair Ensure that the saw is being used for appropriate materials. Use correct manual handling when lifting and use lifting devices where needed. |  |  |  | Low (1) | Workers |
| Complete pre start check on drop saw | The drop saw may not be working appropriately and could cause electrical shock or injury. | Medium (3) | Do not use in atmospheres where flammable materials are present. Ensure there are no defects or damage to the drop saw. Ensure the saw is property secured by Bolts/Clamps etc. Inspect blades and /or sharpness and secure in position. Position fence and/or guards at correct size and/or position. Ensure Guards and Guides are secure and operational |  |  |  | Low (1) | Workers |
| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: |  |  |  | Residual Risk | Responsible Person |
| Connecting power | Possible electrical hazard if the drop saw or power cord is damaged. | Medium (4) | Prior to use the drop saw must be checked for an electrical test tag to ensure that it is current and has been passed. Check the power lead for damage prior to use. Ensure that the power outlet is not damaged. Do not run power leads or hoses across paths or in areas where they may be damaged (e.g. run over). RCD must be used at all times. |  |  |  | Low (2) | Workers |
| Operating drop saw | Possible cuts, bruises or fractures if body parts come into contact with drop saw or out of control material Hearing may be damaged due to noise Eyes may be damaged by metal shavings | Medium (4) | Stay alert, watch what you are doing and use common sense when cutting Do not use while tired or under the influence of drugs, alcohol, or medication. Allow the drop saw to "run up" to operating speed before applying it to the job. Adopt a comfortable stance, with feet apart and well balanced, and with a clear view of the job. Never use a drop saw between the legs while sitting on the floor. Disconnect the power and place the drop saw on a bench when not in use. Ensure material to be cut is on solid workbench before commencing to cut. Do not exert undue force when sawing. Ensure drop saw stops before placing down. Note: Any faults of abnormal actions switch off machine and disconnect from power, DANGER tag and report to supervisor. All power tools and associated leads should be kept away from water. Never touch any parts that are hot. Always wear safety glasses or goggles Use correct manual handling technique and lifting devices where needed. Wear dust masks where necessary; if adequate ventilation cannot be maintained. |  |  |  | Low (2) | Workers |
| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: |  |  |  | Residual Risk | Responsible Person |
| Packing up job | Workers may trip over items not packed away correctly causing injury. | Low (2) | All tools to be checked for damage and then returned to appropriate storage position. Power leads must be rolled up and stored appropriately. All metal or other filings should be cleaned up and disposed of. All PPE must be returned to storage position. |  |  |  | Low (1) | Workers |

| Personal Protective Equipment (PPE) required: | Enclosed leather steel capped boots (As per AS/NZS 2210) Safety Glasses (As per AS/NZS 1336) Hearing protection (As per AS/NZS 1269) Padded gloves (As per AS/NZS 2161) Dust masks where necessary (As per AS/NZS 1716) |
| --- | --- |
| Training and Qualifications required by workers to complete the work: | General Construction induction certificate (White card) Induction into SWI |
| List the Permits, Certificates, SafeWork NSW Approvals, required to complete the work: |  |
| Relevant legislation: For further details refer to the Legal Requirements Register | Work Health and Safety Act 2011 and Work Health and Safety Regulation 2017, Workplace injury management and workers compensation act 1998, Workers Compensation Regulation 2016, AS/NZS1337.1 Personal eye protection-Eye and face protectors for occupational applications (2010), AS/NZS 1269.3 Occupational Noise management-Hearing protection program (2005), AS/NZS 1801 industrial safety helmets (1997), AS/NZS 2210.1 Safety, protective and occupational footwear-Guide to selection, care and use (2010) & AS/NZS 2210.2 to 2210.9 Occupational protective footwear. Safe Work Australia Codes of Practice - Construction Work - Work Health and Safety Consultation, Co-operation and Co-ordination - Managing the Work Environment and Facilities - First Aid in the Workplace - Hazardous Manual Tasks - Managing Noise and Preventing Hearing Loss at Work - How to manage Work Health and Safety Risks |
| Hazardous Substances: |  |
| Plant/equipment that will be used on site: Maintenance checks for plant & equipment: | Drop Saw Pre start check, inspection and maintenance carried out according to manufacturers specifications |
| Frequency of site/workplace inspections: | Daily site hazard analysis inspection completed by site supervisor |
| The control measures implemented will be reviewed in consultation with workers and or others who may be affected by the SWI. Any changes to the SWI will be communicated with workers at induction, daily pre-starts and toolbox talks. |  |

# Sign Off and Approvals

| Person responsible for site supervision of the work, inspecting and approving work areas, approval of and compliance with SWI, protective measures, plant, equipment: |  |
| --- | --- |
| Name and position: | Signature: |
| SWI Prepared by: Ben Knox – Compliance OH&S | Signature: |

| Risk Level | Description of consequence or impact | Consequence | Likelihood/Probability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Unlikely (1) | Possible (2) | Almost Certain (3) |
| High Level of harm | Actual/Potential fatality, disability or irreversible damage. Major structural failure/damage. Off-site environmental discharge/release not contained and significant long-term environmental harm. | Major (3) | Medium (3) | High (6) | High (9) |
| Medium Level of harm | Actual/Potential temporary disability, MTI or LTI. Structural failure/damage, >1-day outage. On-site environmental discharge/release contained, minor remediation, short-term environmental harm. | Moderate (2) | Low (2) | Medium (4) | High (6) |
| Low Level of harm | Incident that has the potential to cause persons to require first aid. Environmental discharge/release immediately contained, minor level clean-up with no short term environmental harm. | Minor (1) | Low (1) | Low (2) | Medium (3) |
| Level | Likelihood/Probability |  |  |  |  |
| Almost Certain | Occurs frequently; >66% chance of occurring |  |  |  |  |
| Possible | Could happen occasionally; >33% but <66% chance of occurring |  |  |  |  |
| Unlikely | May occur only in exceptional circumstances; <33% chance of occurring |  |  |  |  |
| Class/Ranking | Description/Requirements |  |  |  |  |
| High 6, 9 | Stop immediately. Implement permanent preventative controls. |  |  |  |  |
| Medium 3, 4 | Will require operational planning. Actions will be recorded on a SWI. |  |  |  |  |
| Low 1, 2 | Will require detailed pre-planning. Actions will be recorded on a SWI. Will require localised control measures. |  |  |  |  |

SWI has been developed using the hierarchy of controls to reduce the risk and likely hood of an incident/injury occurring. The use of Elimination, Substitution,

Engineering/isolation, Administrative and PPE measures will be used in the above order of preference wherever practicable.

Induction confirmation:

This SWI has been developed in consultation with the workers and has been read and signed by all workers involved in this activity. I note that I have read and understood the above safe work instruction and will ensure work is completed accordingly.

| Date: | Name: | Signature: |
| --- | --- | --- |