| Job Description: | Heat Gun Use | Date: | 06-02-206 | Risk Rating: Medium |
| --- | --- | --- | --- | --- |
| Name and Contact Details Robertson’s Remedial and Painting Pty Ltd # Unit 10/56 Buffalo Road Gladesville NSW 2111 Phone: (02) 9181 3519 A.B.N 16 140 746 247 |  | Site Location: 6a Francis Street, Bondi Beach |  |  |
| Name & Phone Number of Person Completing SWI: Benjamin Knox (0408 253 832) Name & Phone Number of the Person Supervising the Work: Dave Oste - 0476 265 066 |  |  |  |  |
| Comments: This Safe Work Instruction (SWI) is to be used in conjunction with the Safe Work Method Statement for the task being performed. Operating instructions from the supplier of plant are to be obtained and followed. |  |  |  |  |

| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| --- | --- | --- | --- | --- | --- |
| Setting up the job | Working near flammable or explosive materials with the potential to cause a fire or explosion | High (6) | All flammable or explosive materials are to be removed from the area prior to use. Ensure correct fire extinguishers are readily available for all hot work being carried out. Cover or remove all cables, hoses and wires that may be damaged by the work. Work in a well-ventilated area. | Low (2) | Workers |
| Complete pre start check on heat gun | Equipment failure causing injury to operators. | Medium (3) | Ensure nozzle is correctly mounted and secured and that the heat gun is not damaged or defective. Any damages or defects will be noted and the tool removed from service until it has been repaired or replaced. The heat gun should only be used to heat the specific materials and purpose for which it is designed for, according to the manufacturer’s instructions. | Low (1) | Workers |
| Connecting power | Possible electrical hazard if the machine or power cord is damaged. | High (6) | Prior to use the heat gun must be checked for an electrical test tag to ensure that it is current and has been passed. Check the power lead and the heat gun for damage prior to use. Ensure that the power outlet is not damaged. Do not run power leads or hoses across paths or in areas where they may be damaged (e.g. run over). RCD must be used at all times. | Low (2) | Workers |
| Operating heat gun | Burns to skin Fumes may be hazardous Fire from overheating materials | Medium (4) | Ensure work area is clean, tidy and well lit. Keep hands and body parts away from the heat gun when in use. Workers to wear safety glasses. Allow the tool to cool down before touching the air nozzle and carrying out maintenance work. Never point the hot air flow and the discharging plasticised material in the direction of other personnel. Stay alert and watch what you are doing. Do not use while tired or under the influence of drugs, alcohol or medication. Adopt a comfortable stance, with feet apart and well balanced, and with a clear view. Ensure that the heat gun is switched off and unplugged when performing maintenance work, adjusting the nozzle, etc to prevent inadvertent operation. Ensure adequate ventilation or where this is not possible, workers to wear a respirator/mask when using the heat gun. Keep moving the heat gun along the material, as excessive heat will generate fumes and could cause a fire. Competent person to supervise all new workers. Ensure work area is clean, tidy and well lit. Workers to have read the operators manual and are trained and competent in the use of the heat gun. Do not operate the tool near water/damp conditions. | Low (2) | Workers |
| Packing up job | Workers may trip over items not packed away correctly causing injury. | Low (2) | Heat gun to be checked for damage after use and any damages or defects noted. Heat gun to be removed from service until it has been repaired or replaced. Power leads must be rolled up and stored appropriately. All waste should be cleaned up and disposed of appropriately. | Low (1) | Workers |

| Personal Protective Equipment (PPE) required: | Enclosed leather steel capped boots (As per AS/NZS 2210) Safety Glasses (As per AS/NZS 1336) Leather gloves (As per AS/NZS 2161) Hearing protection (As per AS/NZS 1269) |
| --- | --- |
| Training and Qualifications required by workers to complete the work: | General Construction induction certificate (White card) Induction into SWI |
| List the Permits, Certificates, SafeWork NSW Approvals, required to complete the work: |  |
| Relevant legislation: For further details refer to the Legal Requirements Register | Work in accordance with Work Health and Safety Act 2011 and Work Health and Safety Regulation 2025, workplace injury management and workers compensation act 1998, Workers Compensation Regulation, AS/NZS1337.1 Personal eye protection-Eye and face protectors for occupational applications, AS/NZS 1801 industrial safety helmets, AS/NZS 2210.1 Safety, protective and occupational footwear-Guide to selection, care and use & AS/NZS 2210.2 to 2210.9 Occupational protective footwear. Safe Work Australia Codes of Practice - Construction Work - Work Health and Safety Consultation, Co-operation and Co-ordination - Managing the Work Environment and Facilities - First Aid in the Workplace - Hazardous Manual Tasks - Managing Noise and Preventing Hearing Loss at Work - How to manage Work Health and Safety Risks |
| Hazardous Substances | Fumes emitted when heating materials |
| Plant/equipment that will be used on site: Maintenance checks for plant & equipment: | Heat Gun Current electrical test tag, pre start inspections, and regular servicing as per manufacturers instructions |
| Frequency of site/workplace inspections: | Daily site inspections to be completed by Site Supervisor |
| The control measures implemented will be reviewed in consultation with workers and or others who may be affected by the SWI. Any changes to the SWI will be communicated with workers at induction, daily pre-starts and toolbox talks. |  |

# Sign Off and Approvals

| Person responsible for site supervision of the work, inspecting and approving work areas, approval of and compliance with SWI, protective measures, plant, equipment: |  |
| --- | --- |
| Name and position: | Signature: |
| SWI Prepared by: Ben Knox – Compliance OH&S | Signature: |

| Risk Level | Description of consequence or impact | Consequence | Likelihood/Probability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Unlikely (1) | Possible (2) | Almost Certain (3) |
| High Level of harm | Actual/Potential fatality, disability or irreversible damage. Major structural failure/damage. Off-site environmental discharge/release not contained and significant long-term environmental harm. | Major (3) | Medium (3) | High (6) | High (9) |
| Medium Level of harm | Actual/Potential temporary disability, MTI or LTI. Structural failure/damage, >1-day outage. On-site environmental discharge/release contained, minor remediation, short-term environmental harm. | Moderate (2) | Low (2) | Medium (4) | High (6) |
| Low Level of harm | Incident that has the potential to cause persons to require first aid. Environmental discharge/release immediately contained, minor level clean-up with no short term environmental harm. | Minor (1) | Low (1) | Low (2) | Medium (3) |
| Level | Likelihood/Probability |  |  |  |  |
| Almost Certain | Occurs frequently; >66% chance of occurring |  |  |  |  |
| Possible | Could happen occasionally; >33% but <66% chance of occurring |  |  |  |  |
| Unlikely | May occur only in exceptional circumstances; <33% chance of occurring |  |  |  |  |
| Class/Ranking | Description/Requirements |  |  |  |  |
| High 6, 9 | Stop immediately. Implement permanent preventative controls. |  |  |  |  |
| Medium 3, 4 | Will require operational planning. Actions will be recorded on a SWI. |  |  |  |  |
| Low 1, 2 | Will require detailed pre-planning. Actions will be recorded on a SWI. Will require localised control measures. |  |  |  |  |

SWI has been developed using the hierarchy of controls to reduce the risk and likelihood of an incident/injury occurring. The use of Elimination, Substitution,

Engineering/isolation, Administrative and PPE measures will be used in the above order of preference wherever practicable.

Induction confirmation:

This SWI has been developed in consultation with the workers and has been read and signed by all workers involved in this activity. I note that I have read and understood the above safe work instruction and will ensure work is completed accordingly.

| Date: | Name: | Signature: |
| --- | --- | --- |