| Job Description: | Mobile Scaffold |  |  | Date: |  | Risk Rating: Medium |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name, & Contact Details: Robertson’s Remedial and Painting Pty Ltd # Unit 10/56 Buffalo Road Gladesville NSW 2111 Phone: (02) 9181 3519 A.B.N 16 140 746 247 |  |  |  | Site Location: |  |  |  |  |
| Name & Phone Number of Person Completing SWI: Ben Knox (0408 253 832) Name & Phone Number of the Person Supervising the Work: |  |  |  |  |  |  |  |  |
| Comments: This Safe Work Instruction (SWI) is to be used in conjunction with the Safe Work Method Statement for the task being performed. Operating instructions from the supplier of plant are to be obtained and followed. |  |  |  |  |  |  |  |  |
| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: |  |  |  | Residual Risk | Responsible Person |
| Pre-start checks of mobile scaffold | Inappropriate work area may result in injury | Medium (3) | Ensure work area is clean, tidy and well lit. Ensure all individuals are moved away to a safe distance and work area is isolated. Use correct manual handling when manoeuvring the equipment. Only manoeuvre on smooth surface, ensure all penetrations are covered or secured. Remove all obstacles in the path of travel. |  |  |  | Low (1) | Workers |
|  | The equipment may not be working appropriately or may fail causing serious injury | Medium (3) | Ensure that equipment is sourced from professional companies that have an excellent safety record. Prior to operating, operator to conduct inspection to ensure safe for use. All scaffolds must have top rail middle rail and toe board. All scaffolds must have brake that is operating appropriately. Scaffold to be erected by competent person. If scaffolding is above 4m (i.e. where a worker can fall more than 4m), it is to be erected by a qualified (ticketed) scaffolder. Operators must be trained and competent in the use of equipment. Site Supervisor must induct all Workers into SWI. New Workers must be supervised by Foreman or competent person at all times. If scaffold is damaged label unsafe and remove from use. |  |  |  | Low (1) | Workers |
| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: |  |  |  | Residual Risk | Responsible Person |
| Assemble equipment | Inappropriate setup may case failure | Medium (4) | Full deck of boards, fall protection and to toe boards to be installed at each working level prior to moving to next level Only trained and competent Workers to be involved in erection. All levels must have a toe board, middle rail and top rail. Supervision to be provided to all new Workers. Hand Over Certificate to be used for scaffolds above 4m. Materials to be checked prior to erection for damage and faulty parts replaced. Ensure Scaffold is tied to building at appropriate intervals and stabilisers in place where necessary. |  |  |  | Low (1) | Workers and Supervisor |
|  | Individuals may fall form scaffold | Medium (4) | Ensure three points of contact at all times when climbing onto scaffold. Do not lean over edges of working platform. Never move scaffold with individuals on working platform. Hard hats to be worn when working under structures. Do not overload scaffold. Move slowly with caution and do not overbalance the scaffold. |  |  |  | Low (1) | Workers |
|  | Tools or material may fall from platform | Medium (4) | Operator must ensure that the work area is appropriately barricaded off to prevent unintended access. Keep all tools and objects secure in the platform. |  |  |  | Low (1) | Workers |
|  | Unauthorised individuals accessing equipment | Medium (3) | Ensure work area is secure prior to leaving site or that equipment cannot be accessed. Label all defective equipment |  |  |  | Low (1) | Workers |

| Personal Protective Equipment (PPE) required: | Enclosed leather steel capped boots (As per AS/NZS 2210) Safety Glasses (As per AS/NZS 1336) Leather gloves (As per AS/NZS 2161) |
| --- | --- |
| Training and Qualifications required by workers to complete the work: | General Construction induction certificate (White card) Induction into SWI |
| List the Permits, Certificates, SafeWork NSW Approvals, required to complete the work: |  |
| Relevant legislation: For further details refer to the Legal Requirements Register | Work in accordance with Work Health and Safety Act 2011 and Work Health and Safety Regulation 2017, workplace injury management and workers compensation act 1998, Workers Compensation Regulation 2016, AS/NZS1337.1 Personal eye protection-Eye and face protectors for occupational applications (2010), AS/NZS 1801 industrial safety helmets (1997), AS/NZS 2210.1 Safety, protective and occupational footwear-Guide to selection, care and use (2010) & AS/NZS 2210.2 to 2210.9 Occupational protective footwear. Safe Work Australia Codes of Practice - Construction Work - Work Health and Safety Consultation, Co-operation and Co-ordination - Managing the Work Environment and Facilities - First Aid in the Workplace - Hazardous Manual Tasks - Managing Noise and Preventing Hearing Loss at Work - How to manage Work Health and Safety Risks |
| Hazardous Substances |  |
| Plant/equipment that will be used on site: Maintenance checks for plant & equipment: | Mobile Scaffold Pre-start check, inspection and maintenance carried out according to manufacturer’s specifications |
| Frequency of site/workplace inspections: | Daily site inspections to be completed by Site Supervisor |
| The control measures implemented will be reviewed in consultation with workers and or others who may be affected by the SWI. Any changes to the SWI will be communicated with workers at induction, daily pre-starts and toolbox talks. |  |

# Sign Off and Approvals

| Person responsible for site supervision of the work, inspecting and approving work areas, approval of and compliance with SWI, protective measures, plant, equipment: |  |
| --- | --- |
| Name and position: | Signature: |
| SWI Prepared by: Ben Knox – Compliance OH&S | Signature: |

| Risk Level | Description of consequence or impact | Consequence | Likelihood/Probability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Unlikely (1) | Possible (2) | Almost Certain (3) |
| High Level of harm | Actual/Potential fatality, disability or irreversible damage. Major structural failure/damage. Off-site environmental discharge/release not contained and significant long-term environmental harm. | Major (3) | Medium (3) | High (6) | High (9) |
| Medium Level of harm | Actual/Potential temporary disability, MTI or LTI. Structural failure/damage, >1-day outage. On-site environmental discharge/release contained, minor remediation, short-term environmental harm. | Moderate (2) | Low (2) | Medium (4) | High (6) |
| Low Level of harm | Incident that has the potential to cause persons to require first aid. Environmental discharge/release immediately contained, minor level clean-up with no short term environmental harm. | Minor (1) | Low (1) | Low (2) | Medium (3) |
| Level | Likelihood/Probability |  |  |  |  |
| Almost Certain | Occurs frequently; >66% chance of occurring |  |  |  |  |
| Possible | Could happen occasionally; >33% but <66% chance of occurring |  |  |  |  |
| Unlikely | May occur only in exceptional circumstances; <33% chance of occurring |  |  |  |  |
| Class/Ranking | Description/Requirements |  |  |  |  |
| High 6, 9 | Stop immediately. Implement permanent preventative controls. |  |  |  |  |
| Medium 3, 4 | Will require operational planning. Actions will be recorded on a SWI. |  |  |  |  |
| Low 1, 2 | Will require detailed pre-planning. Actions will be recorded on a SWI. Will require localised control measures. |  |  |  |  |

SWI has been developed using the hierarchy of controls to reduce the risk and likely hood of an incident/injury occurring. The use of Elimination, Substitution,

Engineering/isolation, Administrative and PPE measures will be used in the above order of preference wherever practicable.

Induction confirmation:

This SWI has been developed in consultation with the workers and has been read and signed by all workers involved in this activity. I note that I have read and understood the above safe work instruction and will ensure work is completed accordingly.

| Date: | Name: | Signature: |
| --- | --- | --- |