| Job Description: | Jackhammer operation |  | Date: | 06-02-2026 |
| --- | --- | --- | --- | --- |
| Name & Contact Details: Robertson’s Remedial and Painting Pty Ltd # Unit 10/56 Buffalo Road Gladesville NSW 2111 Phone: (02) 9181 3519 A.B.N 16 140 746 247 |  | Site Location: 6a Francis Street, Bondi Beach |  |  |
| Name & Phone Number of person/s who developed SWI: Ben Knox 0408 253 832 Name & Phone Number of the person Supervising the Work: Dave Oste - 0476 265 066 |  |  |  |  |
| Comments: This Safe Work Instruction (SWI) is to be used in conjunction with the Safe Work Method Statement for the task being performed. Operating instructions from the supplier of plant are to be obtained and followed. |  |  |  |  |

| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| --- | --- | --- | --- | --- | --- |
| Preparing for use of jackhammer | Components may become loose and cause accident or break down | Medium (3) | Ensure work area is clean, tidy and well lit. Ensure personnel are adequately trained in the use of equipment. Ensure all screws and attaching mechanisms are tight and secure The material being used must be secured to prevent movement. Ensure all individuals are moved away to a safe distance. | Low (2) | Workers |
| Complete check on jackhammer prior to use | The jackhammer may not be in adequate working order and cause an accident or damage | Medium (3) | Ensure the jackhammer is switched off before connecting to power source Do not use in atmospheres where flammable materials are present. Ensure there are no defects or damage to the jackhammer. Prior to use the jackhammer must be checked for an electrical test tag to ensure that it is current and has been passed. Check the power lead and the jackhammer for damage prior to use. Ensure that the power outlet is not damaged. Do not run power leads or hoses across paths or in areas where they may be damaged (e.g. run over). RCD must be used at all times. Ensure all guards are properly in place and in good working order Ensure appropriate bit for the task is attached and secured | Low (1) | Workers |
| Connecting to power source | Potential electrical risk if jackhammer has not been switched off or power cords are damaged | Medium (3) | Ensure the power outlet is protected by an RCD and dead man switch is operating Ensure all cables, cords and power outlets are free from damage Ensure jackhammer has been electrically inspected and tagged by a competent person Never operate jack hammer with power cord near the jackhammer bit, keep well away. | Low (2) | Workers |
| Operating jackhammer | Potential injury to operator or nearby persons | Medium (4) | Stay alert and maintain concentration when using the jackhammer Do not use while tired or under the influence of drugs, alcohol, or medication. Two hands always used to operate the jackhammer Adopt a comfortable stance, with feet apart and well balanced, and with a clear view of the job. Disconnect the power when not in use. Keep hands away from moving parts. Remove the power before changing tool. Never touch any parts that are hot. Do not leave the jackhammer running when not in use Where possible, use wet methods while jackhammering. Avoid dry cutting where possible. Use dust extraction techniques Always wear the correct PPE such as safety eyewear, hard hats, hearing protection, safety boots, padded gloves and dust mask Do not tape, tie or fix the switch into the ‘on’ position. Ensure that all clothing is close fitting and that any lose clothing, jewellery or hair is secured and out of the way of the jackhammer Do not over reach when using the jackhammer Do not point directly at another person whilst in operation | Low (2) | Workers |
| Packing up after use of jackhammer | Housekeeping | Low (2) | All tools to be checked for damage and then returned to appropriate storage position out of reach of children and untrained personnel. Perform maintenance checks where required. Power leads must be rolled up and stored appropriately. Remove all bits and attachments after equipment has cooled down All work areas to be cleaned on completion of task All PPE must be returned to storage position. | Low (1) | Workers |

| Personal Protective Equipment (PPE) required: | Enclosed leather steel capped boots Safety Glasses (As per AS/NZS 1337) Protective Gloves (As per AS/NZS 2161) | Sun hats, sun cream - when working outdoors Dust Mask / Respirator Hearing Protection |
| --- | --- | --- |
| List the training required by workers to complete the work: List the qualifications of personnel doing the work: | General Construction Induction Card |  |
| List the Permits, Certificates, SafeWork NSW Approvals, required to complete the work: |  |  |
| Relevant legislation: For further details refer to the Legal Requirements Register | Work Health and Safety Act 2011 and Work Health and Safety Regulation 2025 AS/NZS1337.1 Personal eye protection-Eye and face protectors for occupational applications, AS/NZS 1269.3 Occupational Noise management-Hearing protection program, AS/NZS 1801 industrial safety helmets, AS/NZS 2210.1 Safety, protective and occupational footwear-Guide to selection, care and use & AS/NZS 2210.2 to 2210.9 Occupational protective footwear. SafeWork NSW Codes of Practice Construction Work Managing the risk of falls Work Health and Safety Consultation, Co-operation and Co-ordination Managing the Work Environment and Facilities First Aid in the Workplace Hazardous Manual Tasks How to Manage Work Health and Safety Risks |  |
| List the plant / equipment that will be used on site: | Jackhammer |  |
| List maintenance checks for plant & equipment: | Daily pre-start checks, maintenance as per manufacturers guidelines |  |
| Frequency of site/workplace inspections: Site Inspections will be completed by: | Daily site inspections & job completion inspections to be completed by Site Supervisor. |  |
| Hazardous Substances: |  |  |

Sign Off and Approvals

| Person responsible for site supervision of the work, inspecting and approving work areas, approval of and compliance with SWMS, protective measures, plant, equipment: |  |
| --- | --- |
| Name and position: | Signature: |
| SWMS Prepared by: Ben Knox – Compliance OH&S | Signature: |

| Risk Level | Description of consequence or impact | Consequence | Likelihood/Probability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Unlikely (1) | Possible (2) | Almost Certain (3) |
| High Level of harm | Actual/Potential fatality, disability or irreversible damage. Major structural failure/damage. Off-site environmental discharge/release not contained and significant long-term environmental harm. | Major (3) | Medium (3) | High (6) | High (9) |
| Medium Level of harm | Actual/Potential temporary disability, MTI or LTI. Structural failure/damage, >1-day outage. On-site environmental discharge/release contained, minor remediation, short-term environmental harm. | Moderate (2) | Low (2) | Medium (4) | High (6) |
| Low Level of harm | Incident that has the potential to cause persons to require first aid. Environmental discharge/release immediately contained, minor level clean-up with no short term environmental harm. | Minor (1) | Low (1) | Low (2) | Medium (3) |
| Level | Likelihood/Probability |  |  |  |  |
| Almost Certain | Occurs frequently; >66% chance of occurring |  |  |  |  |
| Possible | Could happen occasionally; >33% but <66% chance of occurring |  |  |  |  |
| Unlikely | May occur only in exceptional circumstances; <33% chance of occurring |  |  |  |  |
| Class/Ranking | Description/Requirements |  |  |  |  |
| High 6, 9 | Stop immediately. Implement permanent preventative controls. |  |  |  |  |
| Medium 3, 4 | Will require operational planning. Actions will be recorded on a SWI. |  |  |  |  |
| Low 1, 2 | Will require detailed pre-planning. Actions will be recorded on a SWI. Will require localised control measures. |  |  |  |  |

SWI has been developed using the hierarchy of controls to reduce the risk and likelihood of an incident/injury occurring. The use of Elimination, Substitution,

Engineering/isolation, Administrative and PPE measures will be used in the above order of preference wherever practicable.

Induction confirmation:

This SWI has been developed in consultation with the workers and has been read and signed by all workers involved in this activity. I note that I have read and understood the above safe work instruction and will ensure work is completed accordingly.

| Date: | Name: | Signature: |
| --- | --- | --- |