| Job Description: | Using a concrete screen pump |  | Date: | 06-02-2026 |
| --- | --- | --- | --- | --- |
| Name & Contact Details: Robertson’s Remedial and Painting Pty Ltd # Unit 10/56 Buffalo Road Gladesville NSW 2111 Phone: (02) 9181 3519 A.B.N 16 140 746 247 |  | Site Location: 6a Francis Street, Bondi Beach |  |  |
| Name & Phone Number of person/s who developed SWI: Ben Knox 0408 253 832 Name & Phone Number of the person Supervising the Work: Dave Oste - 0476 265 066 |  |  |  |  |
| Comments: This Safe Work Instruction (SWI) is to be used in conjunction with the Safe Work Method Statement for the task being performed. Operating instructions from the supplier of plant are to be obtained and followed. |  |  |  |  |

| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| --- | --- | --- | --- | --- | --- |
| Pre-Start Checks | Injuries to operator or public/fire hazard | Medium (3) | Before starting your concrete screed pump, carefully read and follow all of the manufacturer's instructions. Always follow the safety instructions in the manufacturer's instruction manual. Always follow local, state, and national fire and electric codes. Maintain your concrete screed pump according to the maintenance schedule for peak performance and safety. | Low (1) | Workers |
| Start concrete screed pump | Injury to operator | Medium (3) | Visually inspect the equipment before use for any damaged parts. Check equipment for fuel leaks. Remove defective equipment from service; mark or tag it as unsafe for use. Keep all body parts, lose clothing, and hair clear from engine. Generator should have an RCD installed. | Low (1) | Workers |
| Start concrete screed pump | Possible fire while refuelling | Medium (4) | Petrol and its vapours are extremely flammable, allow engine to cool at least 2 minutes before refuelling and always use fresh petrol. Make sure you have enough fuel to run a concrete screed pump for an extended period of time – during a power outage gas stations may be closed. Never refuel a concrete screed pump while it is running. Use funnel or can with spout when filling petrol. Take care with filling to ensure no spills occur. Do not over fill fuel tank. Return fuel to vehicle in a secure position. | Low (2) | Workers |
|  | Hearing damage | Medium (3) | Workers to wear hearing protection when working in close proximity to engine. | Low (1) | Workers |
|  | Objects close to engine may catch fire | Medium (3) | Ensure a clear area is maintained around the engine. Do not place any combustible materials close to the engine. | Low (2) | Workers |
|  | Injury to public or others | Medium (3) | Ensure that equipment is isolated from public to ensure that children or others do not come in contact with the engine. Use appropriate barriers, witches hats and signs as necessary. | Low (2) | Workers |
|  | Asphyxiation from fumes | High (6) | Engines emit deadly carbon monoxide gas that can kill individuals. Ensure doors are opened at all times and ventilation is adequate. Avoid working in vehicle with generator going. Ensure extraction fans are in place. | Low (2) | Workers |
|  | Hearing damage | Medium (3) | Ensure adequate ventilation when refuelling. Review Safety Data Sheet and follow safety instructions. Ensure spill kit is available and used to contain any spills. Use funnel or can with spout when filling petrol. Take care to avoid spillages and immediately clean up any spills. | Low (1) | Workers |
|  | Blowback or pipe failure causing injury to operator | Medium (3) | Ensure all hoses and equipment is well maintained Support lines with stands. Keep surging to a minimum especially in bends and elbows. Support vertical hose off solid structure. Never exceed pressure rating of machine or pipes | Low (2) | Workers |
| Start concrete screed pump | Injury to operator | Medium (4) | Visually inspect the equipment before use for any damaged parts. If damage or defects are present remove from service; mark or tag it as unsafe for use until it has been repaired or replaced. Check equipment for fuel leaks. Keep all body parts, loose clothing, and hair clear from engine. Risk of injury when pumping is started, during pumping operations, when removing a blockage and during cleaning procedures Ensure that the concrete screed pump is switched off and the power supply isolated before any servicing or repairs occurs. Ensure that hoses are tight and secured and are free from leaks/damage prior to operation. Never operate the concrete screed pump without the protective guarding in place, preventing access to exposed moving parts. Never touch any of the exposed metal parts during or immediately after operation, to prevent burns. Allow the concrete screed pump time to cool before touching/moving it. | Low (2) | Workers |
| Clean lines and lay in position | High velocity objects from concrete discharging under pressure | High (6) | Work only to be performed by trained and competent persons. Use a sponge catchment device at the discharge end. Keep discharge end clear while under pressure All Workers must wear eye protection Stop the pump and manually remove concrete follow manual handling procedures above. Ensure that lines are secured appropriately at all times | Low (2) | Workers |
| Clean site/wash down equipment | Pollution of waterways | Medium (3) | All equipment to be washed off in suitably bunded cleaning area. Never wash off in gutter | Low (1) | Workers |
|  | Eyes injury or skin damage/rash from contact with concrete | Medium (4) | Always wear safety glasses or face shield Long sleeve shirts, knee high rubber boots, long pants and gloves to be worn where necessary. Ensure adequate water is available to flush eyes where necessary. Wear barrier cream where necessary Avoid splashing concrete or coming into contact use tools etc where possible. Ensure SDS is available and followed at all times Ensure vibrator remains fully within the concrete when operating do not flick out of concrete. | Low (2) | Workers |

| Personal Protective Equipment (PPE) required: | Enclosed leather steel capped boots | Hearing Protection High Vis-Vest |
| --- | --- | --- |
| List the training required by workers to complete the work: List the qualifications of personnel doing the work: | General Construction Induction Card |  |
| List the Permits, Certificates, SafeWork NSW Approvals, required to complete the work: |  |  |
| Relevant legislation: For further details refer to the Legal Requirements Register | Work Health and Safety Act 2011 and Work Health and Safety Regulation 2025 AS/NZS1337.1 Personal eye protection-Eye and face protectors for occupational applications, AS/NZS 1269.3 Occupational Noise management-Hearing protection program, AS/NZS 1801 industrial safety helmets, AS/NZS 2210.1 Safety, protective and occupational footwear-Guide to selection, care and use & AS/NZS 2210.2 to 2210.9 Occupational protective footwear. SafeWork NSW Codes of Practice Construction Work Managing the risk of falls Work Health and Safety Consultation, Co-operation and Co-ordination Managing the Work Environment and Facilities First Aid in the Workplace Hazardous Manual Tasks How to Manage Work Health and Safety Risks |  |
| List the plant / equipment that will be used on site: | Generator |  |
| List maintenance checks for plant & equipment: | Daily pre-start checks, maintenance as per manufacturers guidelines |  |
| Frequency of site/workplace inspections: Site Inspections will be completed by: | Daily site inspections & job completion inspections to be completed by Site Supervisor. |  |
| Hazardous Substances: | Diesel and concret |  |

Sign Off and Approvals

| Person responsible for site supervision of the work, inspecting and approving work areas, approval of and compliance with SWMS, protective measures, plant, equipment: |  |
| --- | --- |
| Name and position: | Signature: |
| SWMS Prepared by: Ben Knox – Compliance OH&S | Signature: |

| Risk Level | Description of consequence or impact | Consequence | Likelihood/Probability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Unlikely (1) | Possible (2) | Almost Certain (3) |
| High Level of harm | Actual/Potential fatality, disability or irreversible damage. Major structural failure/damage. Off-site environmental discharge/release not contained and significant long-term environmental harm. | Major (3) | Medium (3) | High (6) | High (9) |
| Medium Level of harm | Actual/Potential temporary disability, MTI or LTI. Structural failure/damage, >1-day outage. On-site environmental discharge/release contained, minor remediation, short-term environmental harm. | Moderate (2) | Low (2) | Medium (4) | High (6) |
| Low Level of harm | Incident that has the potential to cause persons to require first aid. Environmental discharge/release immediately contained, minor level clean-up with no short term environmental harm. | Minor (1) | Low (1) | Low (2) | Medium (3) |
| Level | Likelihood/Probability |  |  |  |  |
| Almost Certain | Occurs frequently; >66% chance of occurring |  |  |  |  |
| Possible | Could happen occasionally; >33% but <66% chance of occurring |  |  |  |  |
| Unlikely | May occur only in exceptional circumstances; <33% chance of occurring |  |  |  |  |
| Class/Ranking | Description/Requirements |  |  |  |  |
| High 6, 9 | Stop immediately. Implement permanent preventative controls. Will require detailed pre-planning. Actions will be recorded on a SWI. |  |  |  |  |
| Medium 3, 4 | Will require operational planning. Actions will be recorded on a SWI. |  |  |  |  |
| Low 1, 2 | Will require localised control measures. |  |  |  |  |

SWI has been developed using the hierarchy of controls to reduce the risk and likelihood of an incident occurring. The use of Elimination, Substitution, Isolation, Engineering, Administration and PPE measures will be used in the above order of preference wherever practicable.

Induction confirmation:

This SWI has been developed in consultation with the workers and has been read and signed by all workers involved in this activity. I note that I have read and understood the above SWI and will ensure work is completed accordingly.

| Date: | Name: | Signature: |
| --- | --- | --- |