| NOTE: Work must be performed in accordance with this SWMS. This SWMS must be kept and be available for inspection until the high-risk construction work to which this SWMS relates is completed. If the SWMS is revised, all versions should be kept. If a notifiable incident occurs in relation to the high-risk construction work in this SWMS, the SWMS must be kept for at least 2 years from the date of the notifiable incident. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| PCBU Name & Contact Details: Robertson’s Remedial and Painting Pty Ltd # Unit 10/56 Buffalo Road Gladesville NSW 2111 Phone: (02) 9181 3519 A.B.N 16 140 746 247 |  |  |  |  | Principal Contractor (PC): | Robertson’s Remedial and Painting Pty Ltd |
| Work Manager Contact Number: |  | Dave Oste - 0476 265 066 |  |  | Date SWMS provided to PC: | 06-02-2026 |
| Work Activity: | Electrical Works |  |  |  | Workplace Location: | 6a Francis Street, Bondi Beach |
| High Risk Construction Work: | Risk of person falling more than 2 metres (note: in some jurisdictions, this is 3metres) |  |  | Work on a telecommunication tower |  | Demolition of a load-bearing structure |
|  | Likely to involve disturbing asbestos |  |  | Temporary load-bearing support for structural alterations or repairs |  | Work in or near a confined space |
|  | Work in or near a shaft or trench deeper than 1.5m or a tunnel |  |  | Work in an area that may have a contaminated or flammable atmosphere. |  | Work on or near pressurised gas mains or piping |
|  | Work on or near chemical, fuel or refrigerant lines |  |  | Work on or near energised electrical installations |  | Use of explosives |
|  | Diving Work |  |  | Work on, in or adjacent to a road, railway, shipping lane or other traffic corridor in use by traffic other than pedestrians |  | Work in an area with movement of powered mobile plant. |
|  | Work in areas with artificial extremes of temperature |  |  | Work in or near water or other liquid that involves a risk of drowning |  | Tilt-up or precast concrete elements |
| Name of workers consulted in the development of this SWMS: |  |  |  |  | Signature: |  |
| Person responsible for ensuring compliance with SWMS: |  |  |  |  | Date SWMS received: |  |
| What measures are in place to ensure compliance with the SWMS? |  |  | Toolbox meetings, SWMS sign off, job observations and supervision review. If issues with the SWMS or new hazards are identified, the supervisor must be notified. When changes are made to SWMS, it will be communicated to all workers. |  |  |  |
| Person responsible for approving and reviewing SWMS control measures: |  |  |  |  | Date SWMS received by reviewer: |  |
| How will the SWMS control measures be reviewed? |  |  | The control measures implemented will be reviewed and if necessary, revised annually or if work methods change, the control measures are not effective in controlling the risk, a new hazard/risk is identified or following an incident. The SWMS will be reviewed in consultation with workers and/or others who may be affected by the SWMS. Any changes to the SWMS will be communicated with workers at induction, daily pre-starts and toolbox talks. |  |  |  |
| Review Date: |  |  |  |  | Reviewer’s signature: |  |

| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| --- | --- | --- | --- | --- | --- |
| Planning for the set-up of job. | Untrained or unlicensed workers may access site | Medium (3) | All workers must have Construction Induction Certificate. All workers to be qualified/ licensed/ ticketed/ trained as required for the works. All workers must complete site induction and sign in / out. Where induction is not available workers to liaise with site supervisor about site hazards, emergency procedures and any other site requirements. All workers to be trained in and sign safe work method statement. Toolbox talk to be conducted prior to commencement of works All workers to be in a fit state to commence work, be zero blood alcohol concentration, not suffering from fatigue or under the influence of drugs. Competent person to supervise all new workers. | Low (1) | Workers & Supervisor |
|  | Inappropriate tool and equipment allocation | Medium (3) | Site Supervisor/Management to ensure that appropriate equipment and procedures are used for the works being completed. Site Supervisor /Management to ensure that all electrical devices have an electrical test tag prior to commencement of work. Management to ensure that adequate PPE is available to workers. Workers to complete pre-start inspection on all equipment prior to use to ensure it is appropriate for the work and is free from damages or defects. | Low (1) | Workers & Supervisor |
| Accessing and securing the site | Slips, trips and falls | Medium (3) | All workers to wear steel capped boots. Onsite inductions to be completed prior to entering the job including identification of any slip trip and fall hazards. Ensure the work area is clear and free of slip, trip and fall hazards | Low (1) | Workers |
|  | Weather conditions | Medium (3) | Workers must remain hydrated through regularly drinking water. Workers to stay out of sun where possible and implement rest breaks or job rotation where necessary. Workers must wear appropriate PPE including: sun cream, sun hats, and tinted safety glasses while working outside. | Low (1) | Workers |
|  | Environmental contamination | Medium (3) | Where possible works to be timed to avoid rain Workers to follow the project environmental requirements (where required) | Low (1) | Workers |
|  | Emergency onsite. Workers not familiar with emergency procedures | Medium (3) | All workers to ensure they are familiar with the emergency muster points of the site All workers to follow the onsite emergency procedures as outlined at induction. | Low (1) | Workers |
|  | Injury to personnel due to individuals entering work site | Medium (3) | Barricades & signage to be erected to prevent unauthorised access to work area. Where required, spotters to be used to observe personnel and re direct where needed. Alternate access to be provided where appropriate. | Low (1) | Workers & Supervisor |
| Delivery and unload materials and position in work area | Manual handling | Medium (4) | Workers to follow correct manual handling procedures: Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit. Use team lifts. Clear a path, assess the load, get a good grip, ensure a wide stance, bend the knees, keep the spine straight, use the legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Workers |
|  | Inappropriate storage areas may interfere with public | Medium (3) | Workers to discuss material storage area and work areas with Supervisor prior to commencement. Appropriate work and storage areas to be barricaded off. | Low (1) | Workers |
| Isolate all electrical, water and gas services | Electrocution or injury due to contact with services or cables | High (6) | Ensure all services have been located prior to commencement and have been isolated. Ensure workers have completed the "test before they touch" on the electrical installations. Isolate power, water and other services. Lock out and tag out all services. Do not work near live services as per AS/NZS 3000. Fibreglass ladders to be used Sign off to be completed and physically handed to the Site Supervisor from the appropriate licensed electrical and plumbing contractors before any work commences. Power boards may contain asbestos. If workers suspect the presence of asbestos, do not disturb the suspected material and contact the Site Supervisor immediately. | Low (2) | Workers & Supervisor |
| Remove existing materials/fixings etc. | Manual handling | Medium (4) | Workers to follow correct manual handling procedures: Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit. Use team lifts. Clear a path, assess the load, get a good grip, ensure a wide stance, bend the knees, keep the spine straight, use the legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Workers |
| Remove existing materials/fixings etc. | Injury due to incorrect use of tools / equipment | Medium (3) | Follow safe work instructions for grinders, sanders, drills, etc. Workers to be competent in the use of tools being used. Supervisor to monitor all new workers. | Low (1) | Workers & Supervisor |
|  | Fall from heights | High (6) | Implement work platform, physical barrier or physical restraint (In that order) for all work where a worker can fall from one level to another. Workers to follow SWI for control measure selected above such as: Mobile scaffold, scaffold, EWP, etc. Pre-start inspections must be completed on equipment daily. | Low (2) | Workers & Supervisor |
|  | Fall from ladder | High (6) | Avoid overhead electrical hazards and use fibreglass ladder Use a platform ladder where possible. Perform visual check on ladder for obvious faults and defects. Remove damaged ladders. Tag “not for use” and repair or replace. Top of the ladder should extend above the working platform/roof by no less than 1m. Never “walk” ladder to reposition it; descend ladder and relocate it. Erect ladder on clean, level surface or use self-levelling ladder. Erect extension ladder in a 4:1 ratio (4m up for every 1m from the wall) and tie off at top & bottom. Use a footer where necessary. Do not place a ladder in front of a doorway unless locked/guarded. Secure work area around the ladder. Only one person on a ladder at any time. Face the ladder at all times. Ensure legs are fully extended and latch is in place Maintain 3 points of contact on ladder Do not step above the third step from the top. Do not over-reach while on ladder. Where working from more than a couple of stories high a double lanyard fall restraint system is to be used. Fully enclosed, slip resistant footwear should be worn. Clean mud, grass etc, from boots to prevent slippage. | Low (2) | Workers |
| Remove existing materials/fixings etc. | Falling objects | Medium (3) | Use a tool pouch to carry tools. Do not leave tools and equipment on ladder rungs. Wear Hard hat and steel-capped boots | Low (1) | Workers |
| Secure conduit/ toughing/ducting | Electrocution or injury due to contact with services | High (9) | Ensure services have been located prior to commencement and are clearly marked and have been isolated, locked and tagged out. Check drawings for services and allow for inaccuracies in the drawings/plans. Isolate power, water and other services. Ensure workers have completed the "test before they touch" on electrical installations. Do not work near live services as per AS/NZS 3000. Insulated tools to be used. Fibreglass ladders to be used | Low (2) | Workers |
|  | Fall from heights | High (6) | Implement work platform, physical barrier or physical restraint (In that order) for all work where a worker can fall from one level to another. Workers to follow SWI for control measure selected above such as: Mobile scaffold, scaffold, EWP, etc. Pre-start inspections must be completed on equipment daily. | Low (2) | Workers & Supervisor |
|  | Falling objects | Medium (3) | Use a tool pouch to carry tools. Do not leave tools and equipment on ladder rungs. Wear Hard hat and steel-capped boots | Low (1) | Workers |
|  | Manual handling | Medium (4) | Workers to follow correct manual handling procedures: Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit. Use team lifts. Clear a path, assess the load, get a good grip, ensure a wide stance, bend the knees, keep the spine straight, use the legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Workers |
| Secure conduit/ toughing/ducting | Fall from ladder | High (6) | Avoid overhead electrical hazards and use fibreglass ladder Use a platform ladder where possible. Perform visual check on ladder for obvious faults and defects. Remove damaged ladders. Tag “not for use” and repair or replace. Top of the ladder should extend above the working platform/roof by no less than 1m. Never “walk” ladder to reposition it; descend ladder and relocate it. Erect ladder on clean, level surface or use self-levelling ladder. Erect extension ladder in a 4:1 ratio (4m up for every 1m from the wall) and tie off at top & bottom. Use a footer where necessary. Do not place a ladder in front of a doorway unless locked/guarded. Secure work area around the ladder. Only one person on a ladder at any time. Face the ladder at all times. Ensure legs are fully extended and latch is in place Maintain 3 points of contact on ladder Do not step above the third step from the top. Do not over-reach while on ladder. Fully enclosed, slip resistant footwear should be worn. Clean mud, grass etc, from boots to prevent slippage. Where working from more than a couple of stories high a double lanyard fall restraint system is to be used. | Low (2) | Workers |
| Run power/data/fibre for equipment | Injury due to inappropriate use of power tools | Medium (3) | Follow Safe work instruction (SWI) for power tool selected Workers to be competent in the use of the hand tools to be used. Supervisor to monitor all new workers. | Low (1) | Workers |
|  | Electrocution or injury due to contact with live services | High (9) | Isolate power at main board. Never Work Live as per AS/NZS 3000. Ensure services are located and isolated, locked out and tagged out. Run cables to the board as per the design specifications and drawings Connect the lines to the breaker. Run cables to surge diverter. Cross check all works and ensure safe and as per specifications and drawings | Low (2) | Workers |
| Terminate all conductors and Carry out testing on new/repaired electrical equipment | Electrocution or injury due to contact with services or cables | High (9) | Ensure services have been located prior to commencement and are clearly marked and have been isolated, locked and tagged out. Check drawings for services and allow for inaccuracies in the drawings/plans. Isolate power, water and other services. Ensure workers have completed the "test before they touch" on electrical installations. Do not work near live services as per AS/NZS 3000. Insulated tools to be used. Fibreglass ladders to be used | Low (2) | Workers |
| Packing up job | Housekeeping | Medium (3) | All tools to be checked for damage and then returned to appropriate storage position All waste should be cleaned up and disposed of appropriately | Low (1) | Workers |

| Personal Protective Equipment (PPE) required: | Enclosed leather steel capped boots Safety Glasses (As per AS/NZS 1337) Leather Gloves (As per AS/NZS 2161) | Hard Hats (As per AS/NZS 1801) High visibility clothing Sun hats, sun cream- when working outdoors |
| --- | --- | --- |
| List the training required by workers to complete the work: List the qualifications of personnel doing the work: | Construction induction certificate (White card) Licensed electrician Induction into SWMS |  |
| List the Permits, Certificates, SafeWork NSW Approvals, required to complete the work: | All necessary permits to be completed on site |  |
| Relevant Legislation: For further details refer to the Legal Requirements Register | Work Health & Safety Act 2011, Work Health & Safety Regulations 2025, Workplace Injury Management and Workers Compensation Act 1998 no. 86, AS/NZS1337.1 Personal eye protection-Eye and face protectors for occupational applications, AS/NZS 1801 industrial safety helmets, AS/NZS 2210.1 Safety, protective and occupational footwear-Guide to selection, care and use, AS/NZS 4602.1 High visibility safety garments-Garments for high risk applications AS/NZS 1716 Respiratory protective devices, AS1319 Safety signs for occupational environment & AS/NZS 3000 Electrical Installations. SafeWork NSW Codes of Practice: - Construction Work - Managing risks of plant - Managing the risk of falls - Work Health and Safety Consultation, Co-operation and Co-ordination - Managing the Work Environment and Facilities - First Aid in the Workplace - Hazardous Manual Tasks - How to Manage Work Health and Safety Risks - Managing Electrical Risks in the Workplace |  |
| List the plant / equipment that will be used on site: List maintenance checks for plant & equipment: | Power tools, ladders Current electrical test tag (for electrical equipment), pre-start inspection and regular servicing as per manufacturer’s instructions |  |
| Frequency of site/workplace inspections: Site Inspections will be completed by | Daily site inspections to be completed by Site Supervisor and at completion of job |  |
| Hazardous Substances: |  |  |

# Sign Off and Approvals

| Person responsible for site supervision of the work, inspecting and approving work areas, approval of and compliance with SWMS, protective measures, plant, equipment: |  |
| --- | --- |
| Name and position: | Signature: |
| SWMS Prepared by: Ben Knox – Compliance OH&S | Signature: |

| Risk Level | Description of consequence or impact | Consequence | Likelihood/Probability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Unlikely (1) | Possible (2) | Almost Certain (3) |
| High Level of harm | Actual/Potential fatality, disability or irreversible damage. Major structural failure/damage. Off-site environmental discharge/release not contained and significant long-term environmental harm. | Major (3) | Medium (3) | High (6) | High (9) |
| Medium Level of harm | Actual/Potential temporary disability, MTI or LTI. Structural failure/damage, >1-day outage. On-site environmental discharge/release contained, minor remediation, short-term environmental harm. | Moderate (2) | Low (2) | Medium (4) | High (6) |
| Low Level of harm | Incident that has the potential to cause persons to require first aid. Environmental discharge/release immediately contained, minor level clean-up with no short term environmental harm. | Minor (1) | Low (1) | Low (2) | Medium (3) |
| Level | Likelihood/Probability |  |  |  |  |
| Almost Certain | Occurs frequently; >66% chance of occurring |  |  |  |  |
| Possible | Could happen occasionally; >33% but <66% chance of occurring |  |  |  |  |
| Unlikely | May occur only in exceptional circumstances; <33% chance of occurring |  |  |  |  |
| Class/Ranking | Description/Requirements |  |  |  |  |
| High 6, 9 | Stop immediately. Implement permanent preventative controls. |  |  |  |  |
| Medium 3, 4 | Will require operational planning. Actions will be recorded on a SWMS. |  |  |  |  |
| Low 1, 2 | Will require detailed pre-planning. Actions will be recorded on a SWMS. |  |  |  |  |

SWMS has been developed using the hierarchy of controls to reduce the risk and likely hood of an incident/injury occurring. The use of Elimination, Substitution, Engineering/isolation, Administrative and PPE measures will be used in the above order of preference wherever practicable.

Induction confirmation:

This SWMS has been developed in consultation with the workers and has been read and signed by all workers involved in this activity. I note that I have read and understood the above safe work method statement and will ensure work is completed accordingly.

| Date: | Name: | Signature: |
| --- | --- | --- |
| Date: | Name: | Signature: |