| NOTE: Work must be performed in accordance with this SWMS. This SWMS must be kept and be available for inspection until the high-risk construction work to which this SWMS relates is completed. If the SWMS is revised, all versions should be kept. If a notifiable incident occurs in relation to the high-risk construction work in this SWMS, the SWMS must be kept for at least 2 years from the date of the notifiable incident. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| PCBU Name & Contact Details: Robertson’s Remedial and Painting Pty Ltd # Unit 10/56 Buffalo Road Gladesville NSW 2111 Phone: (02) 9181 3519 A.B.N 16 140 746 247 |  |  |  |  | Principal Contractor (PC): |  |
| Work Manager Contact Number: |  |  |  |  | Date SWMS provided to PC: |  |
| Work Activity: | Abrasive Blasting |  |  |  | Workplace Location: |  |
| High Risk Construction Work: | Risk of person falling more than 2 metres (note: in some jurisdictions, this is 3metres) |  |  | Work on a telecommunication tower |  | Demolition of a load-bearing structure |
|  | Likely to involve disturbing asbestos |  |  | Temporary load-bearing support for structural alterations or repairs |  | Work in or near a confined space |
|  | Work in or near a shaft or trench deeper than 1.5m or a tunnel |  |  | Work in an area that may have a contaminated or flammable atmosphere. |  | Work on or near pressurised gas mains or piping |
|  | Work on or near chemical, fuel or refrigerant lines |  |  | Work on or near energised electrical installations |  | Use of explosives |
|  | Diving Work |  |  | Work on, in or adjacent to a road, railway, shipping lane or other traffic corridor in use by traffic other than pedestrians |  | Work in an area with movement of powered mobile plant. |
|  | Work in areas with artificial extremes of temperature |  |  | Work in or near water or other liquid that involves a risk of drowning |  | Tilt-up or precast concrete elements |
| Name of workers consulted in the development of this SWMS: |  |  |  |  | Signature: |  |
| Person responsible for ensuring compliance with SWMS: |  |  |  |  | Date SWMS received: |  |
| What measures are in place to ensure compliance with the SWMS? |  |  | Toolbox meetings, SWMS sign off, job observations and supervision review. If issues with the SWMS or new hazards are identified, the supervisor must be notified. When changes are made to SWMS, it will be communicated to all workers. |  |  |  |
| Person responsible for approving and reviewing SWMS control measures: |  |  |  |  | Date SWMS received by reviewer: |  |
| How will the SWMS control measures be reviewed? |  |  | The control measures implemented will be reviewed and if necessary, revised annually or if work methods change, the control measures are not effective in controlling the risk, a new hazard/risk is identified or following an incident. The SWMS will be reviewed in consultation with workers and/or others who may be affected by the SWMS. Any changes to the SWMS will be communicated with workers at induction, daily pre-starts and toolbox talks. |  |  |  |
| Review Date: |  |  |  |  | Reviewer’s signature: |  |

| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| --- | --- | --- | --- | --- | --- |
| Planning for the setup of job. Check all Workers tickets/licences, check site, complete induction | Untrained or unlicensed workers may access site | Medium (3) | All workers must have Construction Induction Certificate. All workers to be qualified/ licensed/ ticketed/ trained as required for the works. All workers must complete site induction and sign in/out. Where induction is not available workers to liaise with site supervisor about site hazards, emergency procedures and any other site requirements. All workers to be trained in and sign safe work method statement. Toolbox talk to be conducted prior to commencement of works All workers to be in a fit state to commence work, be zero blood alcohol concentration, not suffering from fatigue or under the influence of drugs. Competent person to supervise all new workers. | Low (1) | Workers & Supervisor |
|  | Inappropriate tool and equipment allocation | Medium (3) | Site Supervisor/Management to ensure that appropriate equipment and procedures are used for the works being completed. Site Supervisor /Management to ensure that all electrical devices have an electrical test tag prior to commencement of work. Management to ensure that adequate personal protective equipment is available to workers | Low (1) | Workers & Supervisor |
| Accessing and securing the site | Slips, trips and falls | Medium (3) | All workers to wear steel capped boots. Onsite inductions to be completed prior to entering the job including identification of any slip trip and fall hazards. Ensure the work area is clear and free of slip, trip and fall hazards | Low (1) | Workers |
|  | Weather conditions I.e. heat stroke, dehydration, skin cancer, sun burn etc | Medium (3) | Workers must remain hydrated through regularly drinking water. Workers to stay out of sun where possible and implement rest breaks or job rotation where necessary. Workers must wear appropriate PPE including: sun cream, sun hats, and tinted safety glasses while working outside. | Low (1) | Workers |
|  | Environmental contamination i.e. erosion and sediment | Medium (3) | Where possible works to be timed to avoid rain Sandbags to be used to slow runoff and contain sediment Workers to follow the project environmental requirements (where required) | Low (1) | Workers & Supervisor |
| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| Accessing and securing the site | Emergency onsite i.e. fire, spill etc | Medium (3) | All workers to ensure they are familiar with the emergency muster points of the site All workers to follow the onsite emergency procedures | Low (1) | Workers |
|  | Injury to personnel due to individuals entering work site | Medium (3) | Appropriate barricades to be erected to prevent unauthorised access to work area. Appropriate signage to be erected indicating work site unauthorised access not permitted. Where appropriate, spotters to be used to observe personnel and re direct where needed. Alternate access to be provided where appropriate. | Low (1) | Workers & Supervisor |
| Delivery and unload materials and position in work area | Manual handling injury | Medium (4) | Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit Use team lifts where needed Workers to follow correct manual handling procedures: Clear a path, assess the load, get a good grip, ensure a wide stance, bend the knees, keep the spine straight, use the legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Workers |
|  | Inappropriate storage areas may interfere with public | Medium (3) | Materials must be transported in accordance with safety data sheet specifications. Secure storage areas to be allocated for products. Spill kits and fire fighting equipment to be available on site to manage emergency situations. | Low (1) | Workers |
| Setup Blaster | Exposure to hazardous substances | High (6) | Operator to select appropriate abrasive material for job. Appropriate materials include: Copper slag, aluminium oxide, zinc slag, metal shot, garnet, metal grit, plastic beads, sodium bicarbonate, and nickel slag. Inappropriate materials include: River sand, beach sand (or any white sand), pool filter material and dust from quartz rock, any recycled material that has not been treated to remove repairable dust, any substance capable of causing harm to the upper respiratory track of a person. | Low (2) | Workers |
| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| Setup Blaster | Exposure to silica dust when blasting masonry or concrete | High (6) | Avoid blasting masonry or concrete materials wherever possible. Wet blasting techniques must be used. Operators must wear supplied air respirators while operating equipment and cleaning up. Ensure work zone remains isolated as outlined above. Complete dust monitoring outside enclosure to ensure that dust does not exceed standards. Where levels are exceeded work must cease and appropriate controls implemented. | Low (2) | Workers |
|  | Injury due to equipment failure | Medium (3) | Pre-start visual inspection of equipment to be completed prior to commencement. Check machine, hoses and nozzles for fuel leaks, damage. Remove from service or repair. Store fuel in secure location. Keep all body parts, clothing and hair clear of moving machinery. | Low (1) | Workers |
| Refuelling engine | Injury to workers or public due to fire when refuelling | Medium (3) | Petrol and its vapours are extremely flammable allow engine to cool for at least two minutes before refuelling and always use fresh petrol of the correct type for the machine. Use a funnel or can with spout when filling petrol. Take care to avoid spillages and immediately clean up any spills. Do not over fill the tank. Return fuel to vehicle or secure location after filling. Never smoke when refilling and ensure all petrol and pump are a safe distance from any ignition points. | Low (1) | Workers |
| Operating blaster | Fall from heights | High (6) | Select appropriate fall protection device (Scaffold, EWP, fixed edge protection) to safely perform work at heights and follow the SWI. Ensure worker is competent in operating EWP. Ensure Scaffold is deemed safe to use. Complete Pre-start inspections daily & logbook. Provide Safe access / egress to work area. Fully enclosed, slip resistant footwear should be worn. If wearing fall arrest system, ensure emergency rescue plan in place. | Low (2) | Workers |
| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| Operating blaster | Fall from Ladder | High (6) | Use a platform fiberglass ladder where possible. Perform visual check on ladder for obvious faults and defects prior to use. Damaged ladders are to be removed from use, tag “not for use” and repair or replace ASAP. Fully enclosed, slip resistant footwear should be worn. When gaining access to area, the top of the ladder should extend above the working platform/roof by no less than 1m. Never “walk” the ladder to reposition it; descend the ladder and relocate it. Where working from more than a couple of stories high a double lanyard fall restraint system is to be used. Erect ladder on clean, level surface or use self-levelling ladder. Erect extension ladder in a 4:1 ratio (4m up for every 1m from the wall) and tie off at top and bottom. Use a footer where necessary. Do not place a ladder in front of a doorway unless locked/guarded. Secure work area around the ladder. Clean mud, grass etc, from boots to prevent slippage. Do not over-reach while on ladder. Only one person on a ladder at any one time. Ensure legs are fully extended and latch is in place Maintain three points of contact on ladder Do not step above the third step from the top. | Low (2) | Workers |
|  | Objects falling from height | Medium (4) | Use a tool pouch to carry tools. Tie tools to lanyards. Do not leave tools and equipment on ladder rungs. Workers to wear hard hat and safety boots. Implement drop zone around work area. | Low (2) | Workers |
|  | Contact with Abrasive material and blaster | High (6) | Blasting should be carried out in an enclosure where possible. Only one worker to operate blaster at a time. Where possible use guards to protect operator. Approved safety helmets with lenses designed to withstand high impact from debris. Industrial safety gloves to be worn. Safety boots to be worn at all times (waterproof where necessary). Leather coveralls to be worn. Hose never to be pointed towards individuals. | Low (2) | Workers |
| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| Operating blaster | Repetitive work and prolonged fixed postures | Medium (4) | Workers must warm up and stretch prior to commencing work. Workers to rotate tasks to avoid repetitive tasks and fixed postures. Workers to avoid excessive straining and over reaching. | Low (1) | Workers |
|  | Exposure to hazardous substances from material being blasted Dust | High (6) | Avoid sanding, grinding or drilling above head height. Select an abrasive blasting medium with qualities that will generate minimum dust levels. The worksite must be isolated from other works and workers. Implement exclusion zones. Erect signage warning of – blasting in progress, access restricted, RPE worn in exclusion zone. Workers must wear supplied air respirators. Workers must shower immediately after work. Change clothes after blasting and launder regularly. Wash face and hands prior to eating drinking or smoking. Don’t eat or drink close to the work zone. Shoulder cape, jacket or protective suit must be worn. DO NOT use on asbestos or asbestos containing materials. Cease blasting in windy conditions. Cease work if unauthorised person enters restricted zone. Wear PPE: safety glasses, airline respirator, helmet, ear plugs, protective gloves, protective footwear, protective jacket or suit. | Low (2) | Workers |
|  | Noise | Medium (3) | Wear protective hearing ear plugs or ear muffs. Isolate other workers from work area. Use silencers on equipment. | Low (1) | Workers |
|  | Objects close to engine may catch fire | Medium (4) | Keep all combustible items a safe distance from engine. Ensure a clear area is maintained around the engine. | Low (2) | Workers |
|  | Asphyxiation from pump fumes | High (6) | Engines emit deadly carbon monoxide gas, use engine outdoors only, away from open windows, vents or doors. Never use inside homes, garages, crawl spaces, or other enclosed areas. Fumes that can be fatal can build up in these areas. The use of fans, and opening doors and windows does not provide adequate fresh air. | Low (2) | Workers |
|  | Waste entering drains, contamination | Medium (4) | Waste to be confined to site. All drains to be protected. Workers to monitor waste and dispose of as required. | Low (2) | Workers |
| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| Operating blaster | Injury to public Waste paint contaminating waterways Fire | Medium (4) | Ensure site is secure from public access. Remove all waste materials from site and dispose of appropriately. Remove all tools and product or store in secure location. Clean all painting equipment in wash drums and remove all waste appropriately. Do not leave used rags in a pile as they may catch fire. Remove all rags from site and store in airtight (or approved) container to avoid combustion. Apply barrier cream to hands before commencing work. Use the hand cleaner supplied to remove paint that will not wash off with water Utilise Supervisor’s approved cleaning facilities | Low (2) | Workers |
|  | Housekeeping | Medium (3) | Remove any accumulated dust with wet cleaning methods or high efficiency filter vacuum methods. Wear approved PPE for undertaking of cleaning work. | Low (1) | Workers |

| Personal Protective Equipment (PPE) required: | Enclosed leather steel capped boots Safety Glasses or goggles Hearing Protection (ear plugs or muffs) Protective clothing – jacket or suit. | Sun hats, high visibility clothing Hard Hats or Helmet Protective Gloves Airline respirator |
| --- | --- | --- |
| List the training required by workers to complete the work: List the qualifications of personnel doing the work: | Construction Induction Card Induction into SWMS Competency in using blasting equipment |  |
| List the Permits, Certificates, SafeWork NSW Approvals, required to completed the work: | All necessary permits to be completed on site |  |
| Relevant legislation: For further details refer to the Legal Requirements Register | Work in accordance with Work Health and Safety Act 2011 and Work Health and Safety Regulation 2017, workplace injury management and workers compensation act 1998 no. 86, AS/NZS1337.1 Personal eye protection-Eye and face protectors for occupational applications (2010), AS/NZS 1269.3 Occupational Noise management-Hearing protection program (2005), AS/NZS 1801 industrial safety helmets (1997), AS/NZS 2210.1 Safety, protective and occupational footwear-Guide to selection, care and use (2010) & AS/NZS 2210.2 to 2210.9 Occupational protective footwear. Current Safety Data Sheets to be on site for the hazardous substances used SafeWork NSW Codes of Practice: - Construction Work - Managing risks of plant - Managing the risk of falls - Work Health and Safety Consultation, Co-operation and Co-ordination - Managing the Work Environment and Facilities - First Aid in the Workplace - Hazardous Manual Tasks - How to Manage Work Health and Safety Risks - Abrasive Blasting |  |
| List the plant / equipment that will be used on site: List maintenance checks for plant & equipment: | Power tools, EWP, Scaffold, Ladder, Safety Harness, Spray Gun Inspected, tested & tagged monthly |  |
| Frequency of site/workplace inspections: Site Inspections will be completed by: | Daily site inspections to be completed by Site Supervisor |  |
| Hazardous Substances: | Abrasive materials |  |

# Sign Off and Approvals

| Person responsible for site supervision of the work, inspecting and approving work areas, approval of and compliance with SWMS, protective measures, plant, equipment: |  |
| --- | --- |
| Name and position: | Signature: |
| SWMS Prepared by: Ben Knox – Compliance OH&S | Signature: |

| Risk Level | Description of consequence or impact | Consequence | Likelihood/Probability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Unlikely (1) | Possible (2) | Almost Certain (3) |
| High Level of harm | Actual/Potential fatality, disability or irreversible damage. Major structural failure/damage. Off-site environmental discharge/release not contained and significant long-term environmental harm. | Major (3) | Medium (3) | High (6) | High (9) |
| Medium Level of harm | Actual/Potential temporary disability, MTI or LTI. Structural failure/damage, >1-day outage. On-site environmental discharge/release contained, minor remediation, short-term environmental harm. | Moderate (2) | Low (2) | Medium (4) | High (6) |
| Low Level of harm | Incident that has the potential to cause persons to require first aid. Environmental discharge/release immediately contained, minor level clean-up with no short term environmental harm. | Minor (1) | Low (1) | Low (2) | Medium (3) |
| Level | Likelihood/Probability |  |  |  |  |
| Almost Certain | Occurs frequently; >66% chance of occurring |  |  |  |  |
| Possible | Could happen occasionally; >33% but <66% chance of occurring |  |  |  |  |
| Unlikely | May occur only in exceptional circumstances; <33% chance of occurring |  |  |  |  |
| Class/Ranking | Description/Requirements |  |  |  |  |
| High 6, 9 | Stop immediately. Implement permanent preventative controls. |  |  |  |  |
| Medium 3, 4 | Will require operational planning. Actions will be recorded on a SWMS. |  |  |  |  |
| Low 1, 2 | Will require detailed pre-planning. Actions will be recorded on a SWMS. |  |  |  |  |

SWMS has been developed using the hierarchy of controls to reduce the risk and likely hood of an incident/injury occurring. The use of Elimination, Substitution, Engineering/isolation, Administrative and PPE measures will be used in the above order of preference wherever practicable.

Induction confirmation:

This SWMS has been developed in consultation with the workers and has been read and signed by all workers involved in this activity. I note that I have read and understood the above safe work method statement and will ensure work is completed accordingly.

| Date: | Name: | Signature: |
| --- | --- | --- |
| Date: | Name: | Signature: |