| NOTE: Work must be performed in accordance with this SWMS. This SWMS must be kept and be available for inspection until the high-risk construction work to which this SWMS relates is completed. If the SWMS is revised, all versions should be kept. If a notifiable incident occurs in relation to the high-risk construction work in this SWMS, the SWMS must be kept for at least 2 years from the date of the notifiable incident. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| PCBU Name & Contact Details: Robertson’s Remedial and Painting Pty Ltd # Unit 10/56 Buffalo Road Gladesville NSW 2111 Phone: (02) 9181 3519 A.B.N 16 140 746 247 |  |  |  | Principal Contractor (PC): |  |
| Work Manager Contact Number: |  |  |  | Date SWMS provided to PC: |  |
| Work Activity | Sandblasting |  |  | Workplace Location: |  |
| High Risk Construction Work: | Risk of person falling more than 2 metres (note: in some jurisdictions, this is 3metres) |  | Work on a telecommunication tower |  | Demolition of a load-bearing structure |
|  | Likely to involve disturbing asbestos |  | Temporary load-bearing support for structural alterations or repairs |  | Work in or near a confined space |
|  | Work in or near a shaft or trench deeper than 1.5m or a tunnel |  | Work in an area that may have a contaminated or flammable atmosphere. |  | Work on or near pressurised gas mains or piping |
|  | Work on or near chemical, fuel or refrigerant lines |  | Work on or near energised electrical installations |  | Use of explosives |
|  | Diving Work |  | Work on, in or adjacent to a road, railway, shipping lane or other traffic corridor in use by traffic other than pedestrians |  | Work in an area with movement of powered mobile plant. |
|  | Work in areas with artificial extremes of temperature |  | Work in or near water or other liquid that involves a risk of drowning |  | Tilt-up or precast concrete elements |
| Name of workers consulted in the development of this SWMS: |  |  |  | Signature: |  |
| Person responsible for ensuring compliance with SWMS: |  |  |  | Date SWMS received: |  |
| What measures are in place to ensure compliance with the SWMS? |  | Toolbox meetings, SWMS sign off, job observations and supervision review. If issues with the SWMS or new hazards are identified, the supervisor must be notified. When changes are made to SWMS, it will be communicated to all workers. |  |  |  |
| Person responsible for approving and reviewing SWMS control measures: |  |  |  | Date SWMS received by reviewer: |  |
| How will the SWMS control measures be reviewed? |  | The control measures implemented will be reviewed and if necessary, revised annually or if work methods change, the control measures are not effective in controlling the risk, a new hazard/risk is identified or following an incident. The SWMS will be reviewed in consultation with workers and/or others who may be affected by the SWMS. Any changes to the SWMS will be communicated with workers at induction, daily pre-starts and toolbox talks. |  |  |  |
| Review Date: |  |  |  | Reviewer’s signature: |  |

| Procedure (In Steps): | Possible Hazards: | Risk Rating: | Safety Controls: | Residual Risk: | Responsible Person |
| --- | --- | --- | --- | --- | --- |
| Planning for sandblasting | Untrained or unlicensed Workers may access site causing serious injury to self or other individuals. | Medium (3) | All Workers must have Construction Induction Certificate. All Workers must complete site induction and sign in / out. Where induction is not available Workers to liaise with site supervisor about site hazards, emergency procedures and any other site requirements. All Workers to be trained in and sign safe work method statement. All Workers to be qualified/ licensed/ ticketed/ trained as required for activities performed. Competent person to supervise all new Workers. Pre-start meeting to be conducted prior to commencement of works All Workers to be in a fit state to commence work, they must be Zero blood alcohol concentration and must not be suffering from fatigue or under the influence of drugs | Low (1) | Workers & Supervisor |
|  | Inappropriate tool and equipment allocation may lead to injury | Medium (3) | Risk Assessment of site to be carried out prior to commencement of work and changes made to SWMS where necessary. Ensure that adequate workers are allocated to complete works. Management to ensure that adequate PPE is available Management to ensure that appropriate equipment and procedures are used for the works being completed. Workers to ensure electrical devices are checked prior to commencing work and a current test tag is attached to the device. Ensure adequate first aid kits and amenities are available. | Low (1) | Workers & Supervisor |
|  | Inappropriate transportation/storage of products. | Medium (3) | Materials must be transported in accordance with SDS specifications. Secure storage areas to be allocated for products. Spill kits and firefighting equipment to be available on site. | Low (1) | Painter/ Foreman |
| Accessing the site | Slips, trips and falls | Medium (3) | All Workers to wear approved rubber sole work boots. Identify slip, trip and fall hazards prior to starting work. Workers not to access areas where risk of slip trip or fall is present without appropriate protection in place. | Low (1) | Workers & Supervisor |
|  | Exposure to animals, insects, other pests, wildlife and dangerous plants. | Medium (3) | Be aware of any dangerous animal’s insects etc and avoid where ever possible. Discuss any recent sightings with the site supervisor prior to commencing work. Wear leather gloves, Long sleeve pants and shirts where there is a risk of pests such as under access hatches. | Low (1) | Workers |
| Procedure (In Steps): | Possible Hazards: | Risk Rating: | Safety Controls: | Residual Risk: | Responsible Person |
| Secure job site to ensure public do not access | Unauthorised individuals entering work site | Medium (3) | Painting to be completed during quite times where possible. Appropriate barricading & signage to be erected to prevent public or unauthorised access to work area. Spotters to be used to observe public and re direct as needed. Alternate access to be provided to public where appropriate. Implement traffic management plan where necessary. | Low (1) | Workers |
| Setup sandblasting equipment | Manual handling injury | Medium (4) | Use mechanical aids for example trolley to move sandblasting equipment into position Use team lifts if mechanical aids are unavailable Ensure there is a clear path when carrying equipment out of work area Bend at the knees ensure a straight back and brace stomach muscles when lifting equipment from the ground | Low (2) | Workers |
|  | Inappropriate storage areas Incorrect Uniform | Medium (3) | Materials must be transported in accordance with SDS specifications. Secure storage areas to be allocated for products. Spill kits and firefighting equipment to be available on site to manage emergency situations. Supervisor to ensure worker performing sandblasting has correct PPE on at all times, this includes full overalls, dust mask, goggles and mask. | Low (1) | Workers |
| Perform Sandblasting | Inhalation of sandblasted materials | High (6) | Ensure correct PPE is worn at all times: Eye protection (goggles) Ear protection (ear muffs), full breathing apparatus. Isolate work area from other work activities and workers. Implement exclusion zone with barricading & signage to prevent access. | Low (2) | Workers & Supervisor |
|  | Repetitive work Exposure of blaster on skin | High (6) | Readjust position regularly to avoid stress on muscles. Ensure no skin comes in contact with compressed air/sand (long sleeve shirt to be worn. Only competent & trained workers to perform procedure. Ensure air hose is laid out correctly and not presenting a trip hazard. | Low (2) | Workers & Supervisor |
|  | Objects falling from height | Medium (4) | Use a tool pouch to carry tools. Tie tools to lanyards. Do not leave tools and equipment on ladder rungs. Workers to wear hard hat and safety boots. Implement drop zone around work area. | Low (2) | Workers |
| Procedure (In Steps): | Possible Hazards: | Risk Rating: | Safety Controls: | Residual Risk: | Responsible Person |
| Perform Sandblasting | Fall from heights | High (6) | Select appropriate fall protection device (Scaffold, EWP, fixed edge protection) to safely perform work at heights and follow the SWI. Ensure worker is competent in operating EWP. Ensure Scaffold is deemed safe to use. Complete Pre-start inspections daily & logbook. Provide Safe access / egress to work area. Fully enclosed, slip resistant footwear should be worn. If wearing fall arrest system, ensure emergency rescue plan in place. | Low (2) | Workers |
|  | Fall from Ladder | High (6) | Use a platform fiberglass ladder where possible. Perform visual check on ladder for obvious faults and defects prior to use. Damaged ladders are to be removed from use, tag “not for use” and repair or replace ASAP. Fully enclosed, slip resistant footwear should be worn. When gaining access to area, the top of the ladder should extend above the working platform/roof by no less than 1m. Never “walk” ladder to reposition it; descend the ladder and relocate it. Erect ladder on clean, level surface or use self-levelling ladder. Erect extension ladder in a 4:1 ratio (4m up for every 1m from the wall) and tie off at top and bottom. Use a footer where necessary. Do not place a ladder in front of a doorway unless locked/guarded. Secure work area around the ladder. Clean mud, grass etc, from boots to prevent slippage. Do not over-reach while on ladder. Only one person on a ladder at any one time. Ensure legs are fully extended and latch is in place Maintain three points of contact on ladder. Where working from more than a couple of stories high a double lanyard fall restraint system is to be used. | Low (2) | Workers |
| Apply Paint to sandblasted areas | Inhalation of paint | High (6) | Ensure correct PPE is worn at all times: Eye protection (goggles) Ear protection (ear muffs), full breathing apparatus, long sleeve shirt at all times when performing spray painting procedure Ensure no public can access the area at any stage of procedure. Readjust position regularly to avoid stress on muscles Ensure no skin comes in contact with spray paint. Only Competent adequately trained workers to perform this procedure | Low (2) | Workers & Supervisor |
| Procedure (In Steps): | Possible Hazards: | Risk Rating: | Safety Controls: | Residual Risk: | Responsible Person |
| Apply Paint to sandblasted areas | Fall from heights | High (6) | Select appropriate fall protection device (Scaffold, EWP, fixed edge protection) to safely perform work at heights and follow the SWI. Ensure worker is competent in operating EWP. Ensure Scaffold is deemed safe to use. Complete Pre-start inspections daily & logbook. Provide Safe access / egress to work area. Fully enclosed, slip resistant footwear should be worn. If wearing fall arrest system, ensure emergency rescue plan in place. | Low (2) | Workers |
|  | Fall from Ladder | High (6) | Use a platform fiberglass ladder where possible. Perform visual check on ladder for obvious faults and defects prior to use. Damaged ladders are to be removed from use, tag “not for use” and repair or replace ASAP. Fully enclosed, slip resistant footwear should be worn. When gaining access to area, the top of the ladder should extend above the working platform/roof by no less than 1m. Never “walk” ladder to reposition it; descend the ladder and relocate it. Erect ladder on clean, level surface or use self-levelling ladder. Erect extension ladder in a 4:1 ratio (4m up for every 1m from the wall) and tie off at top and bottom. Use a footer where necessary. Do not place a ladder in front of a doorway unless locked/guarded. Secure work area around the ladder. Clean mud, grass etc, from boots to prevent slippage. Do not over-reach while on ladder. Ensure legs are fully extended and latch is in place Maintain three points of contact on ladder. Where working from more than a couple of stories high a double lanyard fall restraint system is to be used. | Low (2) | Workers |
|  | Objects falling from height | Medium (4) | Use a tool pouch to carry tools. Tie tools to lanyards. Do not leave tools and equipment on ladder rungs. Workers to wear hard hat and safety boots. Implement drop zone around work area. | Low (2) | Workers |
| Clean-up & pack up site | Housekeeping | Medium (3) | Ensure all left over tools are packed away & not left on site Take care when packing away materials use aids & team lifts for heavy left-over materials. Wear PPE (Gloves) if moving sharp edges. | Low (1) | Workers |

| Personal Protective Equipment (PPE) required: | Enclosed leather steel capped boots Safety Glasses (As per AS/NZS 1337) Hearing Protection Protective Gloves (As per AS/NZS 2161) | High visibility Long sleeve shirt & pants Hard Hats (As per AS/NZS 1801) Sun hats, sun cream - when working outdoors Breathing apparatus |
| --- | --- | --- |
| List the training required by workers to complete the work: List the qualifications of personnel doing the work: | Construction Induction Card Painting trade, Trained in Sandblasting, Trained in spray-painting Induction into SWMS |  |
| List the Permits, Certificates, SafeWork NSW Approvals, required to complete the work: |  |  |
| Relevant legislation: For further details refer to the Legal Requirements Register | Work in accordance with Work Health and Safety Act 2011 and Work Health and Safety Regulation 2017, workplace injury management and workers compensation act 1998 no. 86, AS/NZS1337.1 Personal eye protection-Eye and face protectors for occupational applications (2010), AS/NZS 1269.3 Occupational Noise management-Hearing protection program (2005), AS/NZS 1801 industrial safety helmets (1997), AS/NZS 2210.1 Safety, protective and occupational footwear-Guide to selection, care and use (2010) & AS/NZS 2210.2 to 2210.9 Occupational protective footwear. SafeWork NSW Codes of Practice: - Construction Work - Managing risks of plant - Managing the risk of falls - Work Health and Safety Consultation, Co-operation and Co-ordination - Managing the Work Environment and Facilities - First Aid in the Workplace - Hazardous Manual Tasks - How to Manage Work Health and Safety Risks |  |
| List the plant / equipment that will be used on site: | Sandblaster; Spray-painting equipment |  |
| List maintenance checks for plant & equipment: | Daily pre-start checks, maintenance as per manufacturers guidelines |  |
| Frequency of site/workplace inspections: Site Inspections will be completed by: | Daily site inspections & job completion inspections to be completed by Site Supervisor. |  |
| Hazardous Substances: | Paint |  |

# Sign Off and Approvals

| Person responsible for site supervision of the work, inspecting and approving work areas, approval of and compliance with SWMS, protective measures, plant, equipment: |  |
| --- | --- |
| Name and position: | Signature: |
| SWMS Prepared by: Ben Knox – Compliance OH&S | Signature: |

| Risk Level | Description of consequence or impact | Consequence | Likelihood/Probability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Unlikely (1) | Possible (2) | Almost Certain (3) |
| High Level of harm | Actual/Potential fatality, disability or irreversible damage. Major structural failure/damage. Off-site environmental discharge/release not contained and significant long-term environmental harm. | Major (3) | Medium (3) | High (6) | High (9) |
| Medium Level of harm | Actual/Potential temporary disability, MTI or LTI. Structural failure/damage, >1-day outage. On-site environmental discharge/release contained, minor remediation, short-term environmental harm. | Moderate (2) | Low (2) | Medium (4) | High (6) |
| Low Level of harm | Incident that has the potential to cause persons to require first aid. Environmental discharge/release immediately contained, minor level clean-up with no short term environmental harm. | Minor (1) | Low (1) | Low (2) | Medium (3) |
| Level | Likelihood/Probability |  |  |  |  |
| Almost Certain | Occurs frequently; >66% chance of occurring |  |  |  |  |
| Possible | Could happen occasionally; >33% but <66% chance of occurring |  |  |  |  |
| Unlikely | May occur only in exceptional circumstances; <33% chance of occurring |  |  |  |  |
| Class/Ranking | Description/Requirements |  |  |  |  |
| High 6, 9 | Stop immediately. Implement permanent preventative controls. |  |  |  |  |
| Medium 3, 4 | Will require operational planning. Actions will be recorded on a SWMS. |  |  |  |  |
| Low 1, 2 | Will require detailed pre-planning. Actions will be recorded on a SWMS. |  |  |  |  |

SWMS has been developed using the hierarchy of controls to reduce the risk and likely hood of an incident/injury occurring. The use of Elimination, Substitution, Engineering/isolation, Administrative and PPE measures will be used in the above order of preference wherever practicable.

Induction confirmation:

This SWMS has been developed in consultation with the workers and has been read and signed by all workers involved in this activity. I note that I have read and understood the above safe work method statement and will ensure work is completed accordingly.

| Date: | Name: | Signature: |
| --- | --- | --- |
| Date: | Name: | Signature: |