| NOTE: Work must be performed in accordance with this SWMS. This SWMS must be kept and be available for inspection until the high-risk construction work to which this SWMS relates is completed. If the SWMS is revised, all versions should be kept. If a notifiable incident occurs in relation to the high-risk construction work in this SWMS, the SWMS must be kept for at least 2 years from the date of the notifiable incident. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| PCBU Name & Contact Details: Robertson’s Remedial and Painting Pty Ltd # Unit 10/56 Buffalo Road Gladesville NSW 2111 Phone: (02) 9181 3519 A.B.N 16 140 746 247 |  |  |  |  | Principal Contractor (PC): |  |
| Work Manager Contact Number: |  |  |  |  | Date SWMS provided to PC: |  |
| Work Activity: | Swing Stage |  |  |  | Workplace Location: |  |
| High Risk Construction Work: | Risk of person falling more than 2 metres (note: in some jurisdictions, this is 3metres) |  |  | Work on a telecommunication tower |  | Demolition of a load-bearing structure |
|  | Likely to involve disturbing asbestos |  |  | Temporary load-bearing support for structural alterations or repairs |  | Work in or near a confined space |
|  | Work in or near a shaft or trench deeper than 1.5m or a tunnel |  |  | Work in an area that may have a contaminated or flammable atmosphere. |  | Work on or near pressurised gas mains or piping |
|  | Work on or near chemical, fuel or refrigerant lines |  |  | Work on or near energised electrical installations |  | Use of explosives |
|  | Diving Work |  |  | Work on, in or adjacent to a road, railway, shipping lane or other traffic corridor in use by traffic other than pedestrians |  | Work in an area with movement of powered mobile plant. |
|  | Work in areas with artificial extremes of temperature |  |  | Work in or near water or other liquid that involves a risk of drowning |  | Tilt-up or precast concrete elements |
| Name of workers consulted in the development of this SWMS: |  |  |  |  | Signature: |  |
| Person responsible for ensuring compliance with SWMS: |  |  |  |  | Date SWMS received: |  |
| What measures are in place to ensure compliance with the SWMS? |  |  | Toolbox meetings, SWMS sign off, job observations and supervision review. If issues with the SWMS or new hazards are identified, the supervisor must be notified. When changes are made to SWMS, it will be communicated to all workers. |  |  |  |
| Person responsible for approving and reviewing SWMS control measures: |  |  |  |  | Date SWMS received by reviewer: |  |
| How will the SWMS control measures be reviewed? |  |  | The control measures implemented will be reviewed and if necessary, revised annually or if work methods change, the control measures are not effective in controlling the risk, a new hazard/risk is identified or following an incident. The SWMS will be reviewed in consultation with workers and/or others who may be affected by the SWMS. Any changes to the SWMS will be communicated with workers at induction, daily pre-starts and toolbox talks. |  |  |  |
| Review Date: |  |  |  |  | Reviewer’s signature: |  |

| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| --- | --- | --- | --- | --- | --- |
| Planning for the setup of job. Check all Workers tickets/licences, check site, complete induction | Untrained or unlicensed workers may access site causing serious injury to self or other individuals | Medium (4) | All workers must have Construction Induction Certificate. All workers to be qualified/ licensed/ ticketed/ trained as required for the works. All workers must complete site induction and sign-in on site. Where induction is not available workers to liaise with site supervisor about site hazards, emergency procedures and any other site requirements. All workers to be trained in and sign safe work method statement. Toolbox talk to be conducted prior to commencement of works All workers to be in a fit state to commence work, be zero blood alcohol concentration, not suffering from fatigue or under the influence of drugs. Competent person to supervise all new workers. Workers to have completed confined spaces training when accessing confined spaces. | Low (2) | Workers & Supervisor |
|  | Inappropriate tool and equipment allocation may lead to injury | Medium (4) | Site Supervisor/Management to ensure that appropriate equipment and procedures are used for the works being completed. Site Supervisor /Management to ensure that all electrical devices have an electrical test tag prior to commencement of work. Ensure that adequate PPE is available to workers. | Low (2) | Workers & Supervisor |
| Accessing and securing the site | Slips, trips and falls | Medium (3) | All workers to wear steel capped boots. Onsite inductions to be completed prior to entering the job including identification of any slip trip and fall hazards. Ensure the work area is clear and free of slip, trip and fall hazards | Low (1) | Workers |
|  | Weather conditions I.e. heat stroke, dehydration, sun burn etc | Medium (3) | Workers must remain hydrated through regularly drinking water. Workers to stay out of sun where possible and implement rest breaks or job rotation where necessary. Workers must wear appropriate PPE including: sun cream, sun hats, and tinted safety glasses while working outside. | Low (1) | Workers |
|  | Emergency onsite i.e. fire, spill etc | Medium (3) | All workers to ensure they are familiar with the emergency muster points & follow the onsite emergency procedures. | Low (1) | Workers |
| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| Accessing and securing the site | Injury to personnel due to individuals entering work site | Medium (3) | Appropriate barricades & signs to be erected to prevent unauthorised access to work area. Where appropriate, spotters to be used to observe personnel and re direct where needed. Alternate access to be provided where appropriate. | Low (1) | Workers & Supervisor |
| Delivery and unload materials and position in work area | Manual handling injury | Medium (4) | Workers to follow correct manual handling procedures: Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit Use team lifts where needed Clear a path, assess the load, get a good grip, ensure a wide stance, bend the knees, keep the spine straight, use the legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Workers |
|  | Inappropriate storage areas may interfere with public | Medium (3) | Materials must be transported in accordance with safety data sheet specifications. Secure storage areas to be allocated for products. Spill kits and firefighting equipment to be available on site to manage emergency situations. | Low (1) | Workers |
| \ |  |  |  |  |  |
| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| Set up swing stage equipment | Incorrect erection procedures and inadequate design may result in equipment failure | High (6) | Ensure suitable surfaces for the safe set up of swing stage. Only trained and competent advanced scaffolder to be erect. Persons erecting to follow correct erection procedures as per AS/NZS 4576: 2020 Scaffolding. Check materials prior to erection for damage / faulty parts. Equipment to meet AS/NZS 1576 Scaffolding Appropriate footplates to be installed, top rail middle rail, toe board and mesh to be installed. Swing stage daily checklist to be completed prior to commencement Ensure scaffold is tied off to building where necessary. Ensure scaffold rating is appropriate for task and do not overload SWL to be displayed on stage. Consider wind and other environmental factors when erecting scaffold Supervision to be provided to all new workers. | Low (2) | Workers |
| Test operation of the swing stage prior to occupancy | Sudden collapse, failure of jacks, hand rail or anchor points | High (6) | Visually check for damage, leaks etc then operate the swing stage from the ground with the platform unoccupied to test the functioning and stability of the planned setup points | Low (2) | Workers |
| Operation of Swing Stage | Injury to persons accessing and working within the Cradle | Medium (4) | Cradle must be at ground level for entry via the manufacturer provided gate Approach to the stage is to be well lit. All persons are to wear appropriate PPE including appropriate harnesses Do not operate in inclement weather The working load limit will be clearly marked on the cradle of suspended scaffold and the limit will not be exceeded Materials to be evenly distributed, not concentrated in one area Ensure all exist points are secure The cradle is to be left in an inaccessible position at least 3m above the ground or adjacent deck level Ensure power supply is isolated and the cradle is secured against movement | Low (2) | Workers |
| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| Paint external areas of building Prepare surface Apply sealant Apply paint | Fall from heights | High (6) | Always wear safety harness when accessing stage (ensure secondary line is in place). Swing Stage operators must have access to either a two-way radio system or mobile phone whilst working above. Workers must review and implement safety requirements specified by suppliers. Pre-start inspections must be completed on equipment daily & logbook completed. Do not overload SWL on stage. Consider wind and other environmental factors. Lower the platform completely to ground level before exiting the swing stage. When using a swing stage, use an accredited scaffolder to suspend the swing stage from a secure needle and test hydraulics prior to handing over to painters. Repeat the operation, as above until all elevated work is complete. | Low (2) | Workers |
| Paint external areas of building Prepare surface Apply sealant Apply paint | Exposure to hazardous substances | Medium (4) | Read and have accessible the SDS for associated hazardous substance used. Wear appropriate PPE, as outlined in SDS. Have first aid supplies readily available, as per SDS. Wash hands before and after use. | Low (2) | Workers |
|  | Dust | Medium (3) | Wear eye protection at all times Wear dust mask or respirator. Cover all areas with plastic or appropriate sheeting. Use power tools with dust extraction devices. | Low (1) | Workers |
|  | Injury due to incorrect use of equipment | Medium (3) | Follow safe work instructions for grinders, sanders, drills, “Peel Away”, high-pressure hose and steam wallpaper removal. | Low (1) | Workers |
|  | Repetitive work and prolonged fixed postures | Medium (4) | Workers must warm up and stretch prior to commencing work. Workers to rotate tasks to avoid repetitive tasks and fixed postures. Workers to avoid excessive straining and over reaching. | Low (2) | Workers |
| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| Paint external areas of building Prepare surface Apply sealant Apply paint | Objects falling from height | High (6) | Appropriate fencing and hoarding to be erected to assist in protecting the site. Hard hats to be worn when overhead hazards are present. All tools equipment and materials to be secured with lanyards Stage to have mesh around it to prevent objects falling out Set up an authorised exclusion zone beneath the swing stage of 6 metres (where possible). If required use a spotter below for pedestrian traffic. Cease work immediately if person walks into the pedestrian exclusion zone. Ensure all materials and tools being used on the platform are secured to same or the painter by wrist lanyard. Be aware of wind that could carry light debris outside the established access controlled area. | Low (2) | Workers |
| Accessing Balconies for Painting | Fall from height | High (6) | Stage must be tied off to balcony prior to workers exiting. All workers must be wearing harness and attached to independent static line. Take care when stepping over balcony Independent static line to remain on while working from ladders painting balcony. Ensure line is kept at an appropriate length and that the pendulum effect is accounted for with appropriate spacing of anchor points | Low (2) | Workers |
| Clean-up site Pack up materials and tools | Injury to public Waste paint contaminating waterways Fire | Medium (3) | Ensure site is secure from public access. Remove all waste materials from site and dispose of appropriately. Remove all tools and product or store in secure location. Clean all painting equipment in wash drums and remove all waste appropriately. Do not leave used rags in a pile as they may catch fire. Remove all rags from site and store in airtight (or approved) container to avoid combustion. Apply barrier cream to hands before commencing work Use the hand cleaner supplied to remove paint that will not wash off with water Utilise Supervisor’s approved cleaning facilities | Low (1) | Workers |

| Personal Protective Equipment (PPE) required: | Enclosed leather steel capped boots (As per AS/NZS 2210) Safety Glasses (As per AS/NZS 1336) Sun hats, high visibility clothing Hard Hats |
| --- | --- |
| List the training required by workers to complete the work: List the qualifications of personnel doing the work: | Construction Induction Card Relevant trade certificate Induction into SWMS Advanced Scaffolder’s ticket |
| List the Permits, Certificates, SafeWork NSW Approvals, required to completed the work: | All necessary permits to be completed on site |
| Relevant legislation: For further details refer to the Legal Requirements Register | Work in accordance with Work Health and Safety Act 2011 and Work Health and Safety Regulation 2017, workplace injury management and workers compensation act 1998 no. 86, AS/NZS1337.1 Personal eye protection-Eye and face protectors for occupational applications (2010), AS/NZS 1269.3 Occupational Noise management-Hearing protection program (2005), AS/NZS 1801 industrial safety helmets (1997), AS/NZS 2210.1 Safety, protective and occupational footwear-Guide to selection, care and use (2010) & AS/NZS 2210.2 to 2210.9 Occupational protective footwear, AS 4576 Guidelines for scaffolding (2020) Current Safety Data Sheets to be on site for the hazardous substances used SafeWork NSW Codes of Practice: - Construction Work - Managing risks of plant - Managing the risk of falls - Work Health and Safety Consultation, Co-operation and Co-ordination - Managing the Work Environment and Facilities - First Aid in the Workplace - Hazardous Manual Tasks - How to Manage Work Health and Safety Risks |
| List the plant / equipment that will be used on site: List maintenance checks for plant & equipment: | Scaffold, Swing Stage Pre-start inspection of stage daily load test prior to commencement |
| Frequency of site/workplace inspections: Site Inspections will be completed by: | Daily site inspections to be completed by Site Supervisor |
| Hazardous Substances: |  |

# Sign Off and Approvals

| Person responsible for site supervision of the work, inspecting and approving work areas, approval of and compliance with SWMS, protective measures, plant, equipment: |  |
| --- | --- |
| Name and position: | Signature: |
| SWMS Prepared by: Ben Knox – Compliance OH&S | Signature: |

| Risk Level | Description of consequence or impact | Consequence | Likelihood/Probability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Unlikely (1) | Possible (2) | Almost Certain (3) |
| High Level of harm | Actual/Potential fatality, disability or irreversible damage. Major structural failure/damage. Off-site environmental discharge/release not contained and significant long-term environmental harm. | Major (3) | Medium (3) | High (6) | High (9) |
| Medium Level of harm | Actual/Potential temporary disability, MTI or LTI. Structural failure/damage, >1-day outage. On-site environmental discharge/release contained, minor remediation, short-term environmental harm. | Moderate (2) | Low (2) | Medium (4) | High (6) |
| Low Level of harm | Incident that has the potential to cause persons to require first aid. Environmental discharge/release immediately contained, minor level clean-up with no short term environmental harm. | Minor (1) | Low (1) | Low (2) | Medium (3) |
| Level | Likelihood/Probability |  |  |  |  |
| Almost Certain | Occurs frequently; >66% chance of occurring |  |  |  |  |
| Possible | Could happen occasionally; >33% but <66% chance of occurring |  |  |  |  |
| Unlikely | May occur only in exceptional circumstances; <33% chance of occurring |  |  |  |  |
| Class/Ranking | Description/Requirements |  |  |  |  |
| High 6, 9 | Stop immediately. Implement permanent preventative controls. |  |  |  |  |
| Medium 3, 4 | Will require operational planning. Actions will be recorded on a SWMS. |  |  |  |  |
| Low 1, 2 | Will require detailed pre-planning. Actions will be recorded on a SWMS. |  |  |  |  |

SWMS has been developed using the hierarchy of controls to reduce the risk and likely hood of an incident/injury occurring. The use of Elimination, Substitution, Engineering/isolation, Administrative and PPE measures will be used in the above order of preference wherever practicable.

Induction confirmation:

This SWMS has been developed in consultation with the workers and has been read and signed by all workers involved in this activity. I note that I have read and understood the above safe work method statement and will ensure work is completed accordingly.

| Date: | Name: | Signature: |
| --- | --- | --- |
| Date: | Name: | Signature: |