| - NOTE: Work must be performed in accordance with this SWMS. - This SWMS must be kept and be available for inspection until the high-risk construction work to which this SWMS relates is completed. If the SWMS is revised, all versions should be kept. If a notifiable incident occurs in relation to the high-risk construction work in this SWMS, the SWMS must be kept for at least 2 years from the date of the notifiable incident. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| - PCBU Name & Contact Details: - Robertson’s Remedial and Painting Pty Ltd # Unit 10/56 Buffalo Road Gladesville NSW 2111 Phone: (02) 9181 3519 A.B.N 16 140 746 247 |  |  |  |  | - Principal Contractor (PC): |  |
| - Work Manager Contact Number: |  |  |  |  | - Date SWMS provided to PC: |  |
| - Work Activity: | - Suspended Cage |  |  |  | - Workplace Location: |  |
| High Risk Construction Work: | Risk of person falling more than 2 metres (note: in some jurisdictions, this is 3metres) |  |  | Work on a telecommunication tower |  | Demolition of a load-bearing structure |
|  | Likely to involve disturbing asbestos |  |  | Temporary load-bearing support for structural alterations or repairs |  | Work in or near a confined space |
|  | Work in or near a shaft or trench deeper than 1.5m or a tunnel |  |  | Work in an area that may have a contaminated or flammable atmosphere. |  | Work on or near pressurised gas mains or piping |
|  | Work on or near chemical, fuel or refrigerant lines |  |  | Work on or near energised electrical installations |  | Use of explosives |
|  | Diving Work |  |  | Work on, in or adjacent to a road, railway, shipping lane or other traffic corridor in use by traffic other than pedestrians |  | Work in an area with movement of powered mobile plant. |
|  | Work in areas with artificial extremes of temperature |  |  | Work in or near water or other liquid that involves a risk of drowning |  | Tilt-up or precast concrete elements |
| Name of workers consulted in the development of this SWMS: |  |  |  |  | Signature: |  |
| Person responsible for ensuring compliance with SWMS: |  |  |  |  | Date SWMS received: |  |
| What measures are in place to ensure compliance with the SWMS? |  |  | Toolbox meetings, SWMS sign off, job observations and supervision review. If issues with the SWMS or new hazards are identified, the supervisor must be notified. When changes are made to SWMS, it will be communicated to all workers. |  |  |  |
| Person responsible for approving and reviewing SWMS control measures: |  |  |  |  | Date SWMS received by reviewer: |  |
| How will the SWMS control measures be reviewed? |  |  | The control measures implemented will be reviewed and if necessary, revised annually or if work methods change, the control measures are not effective in controlling the risk, a new hazard/risk is identified or following an incident. The SWMS will be reviewed in consultation with workers and/or others who may be affected by the SWMS. Any changes to the SWMS will be communicated with workers at induction, daily pre-starts and toolbox talks. |  |  |  |
| Review Date: |  |  |  |  | Reviewer’s signature: |  |

| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| --- | --- | --- | --- | --- | --- |
| Planning for the preparation and repainting building | Untrained or unlicensed Workers may access site | Medium (3) | All Workers must have Construction Induction Certificate. All Workers must complete site induction, sign in and out of site daily. Where induction is not available Workers to liaise with site controller about site hazards, emergency procedures and any other site requirements. All Workers to be trained in and sign safe work method statement. All Workers to be qualified/ licensed/ ticketed/ trained as required for activities performed. Competent person to supervise all new Workers. Toolbox talk to be conducted prior to commencement of works All Workers to be in a fit state to commence work, they must be Zero blood alcohol concentration and must not be suffering from fatigue or under the influence of drugs | Low (1) | Crane Operator & Dogger |
|  | Inappropriate tool and equipment allocation | Medium (3) | Risk Assessment of site to be carried out prior to commencement of work and changes made to SWMS where necessary. Management to ensure that adequate Workers numbers are allocated to complete works. Management to ensure that adequate personal protective equipment is available Management to ensure that appropriate equipment and procedures are used for the works being completed. Workers to ensure electrical devices are checked prior to commencing work and a current test tag is attached to the device. Ensure adequate first aid kits and amenities are available. | Low (1) | Crane Operator & Dogger |
|  | Inappropriate transportation/storage of products. | Medium (3) | Materials must be transported in accordance with SDS specifications. Secure storage areas to be allocated for products. Spill kits and fire fighting equipment to be available on site. | Low (1) | Crane Operator & Dogger |
| Accessing the site | Slips, trips and falls. | Medium (3) | All Workers to wear approved rubber sole work boots as per AS/NZS 2210. Identify slip trip and fall hazards. Workers not to access areas where risk of slip trip or fall is present without appropriate protection in place. | Low (1) | Crane Operator & Dogger |
|  | Exposure to animals, insects, other pests, wildlife and dangerous plants. | Medium (3) | Be aware of any dangerous animal’s, insects etc and avoid where ever possible. Discuss any recent sightings with the site controller prior to commencing work. Wear leather gloves where there is a risk of pests such as under access hatches, Long sleeve pants and shirts to be worn. | Low (1) | Crane Operator & Dogger |
| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| Secure job site to ensure public do not access | Injury to public or Workers due to individuals entering work site | Medium (3) | Painting to be completed during quite times where possible. Appropriate barricading & signage to be erected to prevent public and unauthorised access to work area where necessary. Spotters to be used to observe public and re-direct as needed. Alternate access to be provided to public where appropriate. Implement traffic management plan where necessary. | Low (1) | Crane Operator & Dogger |
| Unload materials and equipment from vehicle and position in work area | Manual handling injury | Medium (4) | Workers to follow correct manual handling procedures: Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit Use team lifts where needed Clear a path, assess the load, get a good grip, ensure a wide stance, bend the knees, keep the spine straight, use the legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely | Low (2) | Crane Operator & Dogger |
|  | Inappropriate storage areas may interfere with public | Medium (3) | Materials must be transported in accordance with safety data sheet specifications. Secure storage areas to be allocated for products. Spill kits and fire fighting equipment to be available on site to manage emergency situations. | Low (1) | Crane Operator & Dogger |
| Suspended Cage Safety Pre-Checks | Suspended Cage shifting, falling, striking personnel, Pedestrians and/or objects | High (6) | The use and operation of a work box/suspended cage must comply fully with the requirement set out in the AS 2550.1-1-2002.Section 6.18 Suspension of persons by Crane The Suspended Cage must comply with AS 1418.17- 1996. All personnel involved are to have a detailed copy of the task and risk assessment documented. Safety harnesses will be worn and secured as per Australian Standard 2550.1 – 2002. Section 6.18 Suspensions of persons by crane and in compliance with AS/NZS 1891.1. Site Manager/ Supervisor to ensure that agreed procedures are adhered to. The Suspended Cage must be inspected prior to use for any defects. Must obtain a current Engineers Certificate prior to using and have twelve monthly intervals of inspections of the cage. All Rigging Equipment, chains, slings, etc. must be inspected and tagged for safe use prior to use. | Low (2) | Crane Operator & Dogger |
| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| Drive Crane into position | Poor communication | Medium (4) | Workers to ensure clear communication is maintained between dogger & crane operator with signals, whistle or uninterrupted radio frequency. Toolbox talk to be completed and SWMS discussed and signed prior to any operation commencing. | Low (2) | Crane Operator & Dogger |
|  | Equipment failure due to inappropriate set up and incomplete pre-start checks. | Medium (4) | Never operate crane when lightning or wind is present. Obtain information from site controller on load ratings for surfaces. Do not drive onto surfaces that are not capable of holding the load. Identify underground services, ensure they are protected. Crane driver, dogger & rigger/s to set up crane as to AS 2550.5-2002 and in accordance with manufacturer’s and crane computer’s instructions Daily Pre-start checks to be completed and signed by crane driver. Only trained and competent workers to be involved in operating crane. Ensure crane is level & clear of excavation or embankment (outside of slip zone). Dogger to direct crane into position and ensure clear of others/structures. Ground Workers to remain clear of moving plant. Ensure flashing lights and reversing beacons are operating. | Low (2) | Crane Operator & Dogger |
| Drive Crane into position | Contact with overhead power lines, infrastructure or persons. | High (6) | Complete survey of site with site controller before entering and plan path of travel to ensure all obstacles are avoided. Maintain a safe distance from power lines, infrastructure and people: - 3m clearance up to 132,000V - 6m clearance 132,000V to 330,000V - 8m + clearance for over 330,000V Spotters and tiger tails to be used. Use insulated tag lines when operating near power lines. Use earthing point on crane when operating near overhead power lines. Obtain approved overhead power line permit before working in close proximity to overhead power lines. Permission obtained from site controller. Determine slew zone & ensure clear of obstacles, set working range limiter. Operator to set computer operated limier to ensure boom is unable to contact wires and will remain a safe distance at all times. Operator to blow out boom then slew out and then tally out boom. Spotter to be used to ensure load remains safe distance from power lines. | Low (2) | Crane Operator & Dogger |
| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| Extend outriggers onto pads using electronic controls. Pack, timbers where necessary using pigsty technique. | Outriggers striking personnel causing injury. | Medium (4) | Operator to warn surrounding persons of outriggers. Barricade work area to ensure no unauthorised access. Ensure area around outriggers is clear and that there is adequate clearance. | Low (2) | Crane Operator & Dogger |
|  | Outriggers collapsing through supporting surface. | High (6) | Checklist to be conducted at all operating sites. Ensure load bearing capacity of the ground is capable of holding the load. If there is any doubt as to the weight bearing capacity of the ground, the ground must be investigated by a competent person (engineer or geo-technician). Bottom layer of packing must be close layered. | Medium (3) | Crane Operator & Dogger |
|  | Manual handling | Medium (4) | Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit Use team lifts where needed. Workers to follow correct manual handling procedures: Clear a path, assess the load, get a good grip, ensure a wide stance, bend the knees, keep the spine straight, use the legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. Take care when releasing chains. | Low (2) | Crane Operator & Dogger |
| Unlock dog chains | Manual handling | Medium (4) | Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit Use team lifts where needed. Workers to follow correct manual handling procedures: Clear a path, assess the load, get a good grip, ensure a wide stance, bend the knees, keep the spine straight, use the legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. Take care when releasing chains. Take care when releasing chains. | Low (2) | Crane Operator & Dogger |
| Deliver weights adjacent to crane. | Traffic hazard | Medium (4) | Counterweights to be loaded behind to set up position with competent persons guiding vehicles. Workers to follow all instructions on the plan and from the traffic controllers. All Workers to wear the company’s high visibility uniform at all times. | Low (2) | Crane Operator & Dogger |
| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| Extend boom of crane and slew above weight. | Contact with overhead power lines, persons or infrastructure. | High (6) | Complete survey of site with site controller before entering and plan path of travel to ensure all obstacles are avoided. Maintain a safe distance from power lines, infrastructure and people: - 3m clearance up to 132,000V - 6m clearance 132,000V to 330,000V - 8m + clearance for over 330,000V Spotters and tiger tails to be used. Use insulated tag lines when operating near power lines. Use earthing point on crane when operating near overhead power lines. Obtain approved overhead power line permit before working in close proximity to overhead power lines. Permission obtained from site controller. Determine slew zone & ensure clear of obstacles, set working range limiter. Operator to set computer operated limier to ensure boom is unable to contact wires and will remain a safe distance at all times. Operator to blow out boom then slew out and then tally out boom. Spotter to be used to ensure load remains safe distance from power lines. | Low (2) | Crane Operator & Dogger |
| Attach and tighten chains, slings, lifters, spreader bar or other lifting equipment to weights. | Hand crush injuries | High (6) | Only trained and competent Workers to change chains. Use team lifts where necessary. Ground Workers to wear hard hats at all times. Use leather gloves where appropriate. | Low (2) | Crane Operator & Dogger |
|  | Manual handling | Medium (4) | Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit Use team lifts where needed. Workers to follow correct manual handling procedures: Clear a path, assess the load, get a good grip, ensure a wide stance, bend the knees, keep the spine straight, use the legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. Take care when releasing chains. | Low (2) | Crane Operator & Dogger |
|  | Fall from heights | High (6) | Select appropriate fall protection equipment to access heights above where a worker can fall from one level to another. Utilise handrails where possible. Do no jump from back of truck. Use steps. Remain clear of edges of crane platforms and truck tray. | Low (2) | Crane Operator & Dogger |
| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| Attach chains, slings, lifters, spreader bar or other lifting equipment to weights. | Chains not hooked properly causing load to drop from height. | High (6) | Perform visual inspection of chains and check current tag is in place. Daily pre-start inspections on slings, chains and lifting devices by competent persons to identify defective equipment to be removed from service. Use only engineer approved slings, chains and lifting devices. Only use slings, chains and lifting devices that have been designed for the purpose they are to be used for- see manufacturer’s instructions. Ensure lifting equipment is tagged displaying SWL and has been inspected for compliance. Only trained and competent dogmen to attach chains. Dogger to check chains prior to lift. Lifting gear check to be completed prior to commencement of job. All unsuitable equipment to be removed from service and repaired. | Low (2) | Crane Operator & Dogger |
|  | Equipment failure | Medium (4) | Attach lifting equipment at ground level where possible. Ensure boom is set up as per manufacturer’s instructions. Double-check all aspects of the crane each time the crane has been re-configured. Do not allow line to go slack. Slings/chains to be attached to approved lifting points capable of load. | Low (2) | Crane Operator & Dogger |
| Lift and slew weights and place onto crane | Contact with overhead power lines, persons or infrastructure | High (9) | Complete survey of site with site controller before entering and plan path of travel to ensure all obstacles are avoided. Maintain a safe distance from power lines, infrastructure and people: - 3m clearance up to 132,000V - 6m clearance 132,000V to 330,000V - 8m + clearance for over 330,000V Spotters and tiger tails to be used. Use insulated tag lines when operating near power lines. Use earthing point on crane when operating near overhead power lines. Obtain approved overhead power line permit before working in close proximity to overhead power lines. Permission obtained from site controller. Determine slew zone & ensure clear of obstacles, set working range limiter. Operator to set computer operated limier to ensure boom is unable to contact wires and will remain a safe distance at all times. Operator to blow out boom then slew out and then tally out boom. Spotter to be used to ensure load remains safe distance from power lines. | Low (2) | Crane Operator & Dogger |
| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| Lift and slew weights and place onto crane | Crane or load striking persons or infrastructure | High (6) | Ensure loads are never suspended over people and all items are secure. Only trained, competent, ticketed persons to operate crane or be dogger. Dogger to observe crane at all times and alert crane driver at slew limit and whenever else necessary. Crane crew to undertake a trial lift and crane slew to check clearances. Never stand under a load or near a crush zone. Ensure constant clear communication between dogger and crane operator: - Supply two way radios for communication. - Where possible maintain visual contact and use approved signals - Use whistles for backup. High Visibility uniform to be worn at all times. | Low (2) | Crane Operator & Dogger |
| Remove lifting gear from weights - Attach chains/lifting gear | Manual handling | Medium (4) | Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit Use team lifts where needed. Workers to follow correct manual handling procedures: Clear a path, assess the load, get a good grip, ensure a wide stance, bend the knees, keep the spine straight, use the legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Crane Operator & Dogger |
| Lift load: Operate crane to dogger instructions | Contact with overhead power lines, persons, or infrastructure. | High (6) | Complete survey of site with site controller before entering and plan path of travel to ensure all obstacles are avoided. Maintain a safe distance from power lines, infrastructure and people: - 3m clearance up to 132,000V - 6m clearance 132,000V to 330,000V - 8m + clearance for over 330,000V Spotters and tiger tails to be used. Use insulated tag lines when operating near power lines. Use earthing point on crane when operating near overhead power lines. Obtain approved overhead power line permit before working in close proximity to overhead power lines. Permission obtained from site controller. Determine slew zone & ensure clear of obstacles, set working range limiter. Operator to set computer operated limier to ensure boom is unable to contact wires and will remain a safe distance at all times. Operator to blow out boom then slew out and then tally out boom. Spotter to be used to ensure load remains safe distance from power lines. | Low (2) | Crane Operator & Dogger |
| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| Lift load: Operate crane to dogger instructions | Traffic Incident | High (6) | Dogger to communicate with traffic control and direct as needed. Spotters to be used to ensure safety of power lines and public Only dogger to unhook load | Low (2) | Crane Operator & Dogger |
|  | Crane tipping over | High (6) | Never exceed crane rated loads. Never lift a load out of designated radius. Operator to observe procedures on lift study at all times. Ensure stable foundation of crane and outriggers. Use crane cut out systems where necessary. Ensure crane remains level at all times. | Low (2) | Crane Operator & Dogger |
|  | Equipment failure | High (6) | Attach lifting equipment at ground level where possible. Ensure boom is set up as per manufacturer’s instructions, and computer instructions. Double-check all aspects of the crane each time the crane has been re-configured. Do not allow line to go slack. Slings/chains to be attached to approved lifting points capable of taking the load. | Low (2) | Crane Operator & Dogger |
|  | Unplanned rotation of load whilst slewing | High (6) | Use non-conductive tag line to assist with controlling the load. Ensure a slow slewing operation. Do not operate crane in wind speeds above 10m/s. Ensure load is free prior to lifting and is clear of catch points. | Low (2) | Crane Operator & Dogger |
| Placement of load to dogger instructions | Lowering load onto personnel causing serious injury | High (6) | All Workers to wear high visibility uniform at all times. Lower load slowly. Ensure constant clear communication between dogger and crane operator. All personnel to remain clear of load. Dogger to provide adequate warning of descending load to persons below. Do not place hand or bodily parts below the load when lowering. Use non-conductive tag line to assist with controlling the load. | Low (2) | Crane Operator & Dogger |
| Removal of slings, chains and lifting devices | Fall from heights | High (6) | Select appropriate fall protection equipment to access heights above where a worker can fall from one level to another. Utilise handrails where possible. Remain clear of edges of crane platforms and truck tray. | Low (2) | Dogger |
|  | Manual Handling | Medium (4) | Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit Use team lifts where needed. Workers to follow correct manual handling procedures: Clear a path, assess the load, get a good grip, ensure a wide stance, bend the knees, keep the spine straight, use the legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Dogger |
| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| Lifting, Lowering & manoeuvring Suspended Cage with Crane | Suspended Cage uncontrolled and/or having unexpected movements | High (6) | Prior to Suspended Cage being raised a test lift will be carried out to ensure cranes capacity is not exceeded. Travel motion shall not be used whilst personnel are suspended in the Cage The luffing and hoisting motions shall be restricted to minimum speed. Cranes with free falling capability functions shall be physically disengaged and made non operational. No more than three persons shall ride in the box at any one time, one of which shall be a certified Dogger. A copy of authorization together with the name of the certified dogger shall be displayed in operators cabin. No other loads shall be lifted or supported by the crane. The Suspended cage shall only be lifted on the main hoisting crane hook. Work from the Cage shall only be carried out when weather conditions so not present an unacceptable risk. The crane safe working load must not be less than 1.5t at the radius required when using a Suspended cage. | Low (2) | Crane Operator & Dogger |
|  | Injury to Persons or Public due to Suspended cage | High (6) | All persons to remain within confines of Suspended Cage during crane lift. Access to and from the Suspended Cage is limited to ground level or area where it can be landed onto a firm structure and secured. Crane operator to remain in the crane at the controls with the engine running at all times whilst the Cage is being suspended from crane hook. Where persons may be placed at risk due to inclement weather condition or other impending danger crane operations are to be suspended. This decision is at the discretion of the crane crew performing the work. Direct communication with the crane operator to be maintained by sight and or via two-way radio. The carrying of oxy/LPG or acetylene gas bottles in the Suspended cage is not permitted. Safety Data Sheets are to be provided for any hazardous substances that may be carried in the Suspended Cage prior to the material being brought to site. | Low (2) | Crane Operator & Dogger |
| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| Pack up Crane – Remove Chains Close outriggers (half) and remove pads close outriggers fully. | Unauthorised persons accessing crane when unattended | Medium (3) | Power supply isolated where applicable Motion brakes and locks applied appropriately Remove key from ignition Hook secured or raised clear of obstructions Motion brakes and locks applied appropriately | Low (1) | Crane Operator & Dogger |
|  | Persons being struck by moving crane | High (6) | Maintain a safe distance from infrastructure and people. Approved Uniform with high visibility to be worn at all times. Workers to monitor and direct traffic as necessary. Review and comply with Traffic Control Plan and permits where in place. Only trained and competent Workers to operate crane. Ensure clear communication between dogger and crane operator. Barricade area from public access where necessary Ground Workers to remain clear of moving plant. Operator to stay alert and maintain a safe distance from all persons or structures. Ensure flashing lights and reversing beacons are operating | Low (2) | Crane Operator & Dogger |
|  | Housekeeping | Low (2) | Centralise and store all tools and equipment at designated secured storage area | Low (1) | Crane Operator & Dogger |

| Personal Protective Equipment (PPE) required: | Enclosed leather steel capped boots (As per AS/NZS 2210) Safety Glasses (As per AS/NZS 1337) Hearing Protection Protective Gloves (As per AS/NZS 2161) Hard Hats (As per AS/NZS 1801) High visibility clothing Sun hats, sun cream- when working outdoors |
| --- | --- |
| List the training required by workers to complete the work: List the qualifications of personnel doing the work: | Construction Induction Card Relevant trade certificate Induction into SWMS |
| List the Permits, Certificates, SafeWork NSW Approvals, required to completed the work: | All necessary permits to be completed on site |
| Relevant legislation: For further details refer to the Legal Requirements Register | Work in accordance with Work Health and Safety Act 2011 and Work Health and Safety Regulation 2017, workplace injury management and workers compensation act 1998 no. 86, AS/NZS1337.1 Personal eye protection-Eye and face protectors for occupational applications (2010), AS/NZS 1269.3 Occupational Noise management-Hearing protection program (2005), AS/NZS 1801 industrial safety helmets (1997), AS/NZS 2210.1 Safety, protective and occupational footwear-Guide to selection, care and use (2010) & AS/NZS 2210.2 to 2210.9 Occupational protective footwear. SafeWork NSW Codes of Practice: - Construction Work - Managing risks of plant - Managing the risk of falls - Work Health and Safety Consultation, Co-operation and Co-ordination - Managing the Work Environment and Facilities - First Aid in the Workplace - Hazardous Manual Tasks - How to Manage Work Health and Safety Risks |
| List the plant / equipment that will be used on site: | Crane, Suspended Cage |
| List maintenance checks for plant & equipment: | Daily pre-start checks, log-book checks, service / maintenance as per manufacturers recommendations. |
| Frequency of site/workplace inspections: Site Inspections will be completed by: | Daily site inspections & job completion inspections to be completed by Site Supervisor. |
| Hazardous Substances: |  |

# Sign Off and Approvals

| Person responsible for site supervision of the work, inspecting and approving work areas, approval of and compliance with SWMS, protective measures, plant, equipment: |  |
| --- | --- |
| Name and position: | Signature: |
| SWMS Prepared by: Ben Knox – Compliance OH&S | Signature: |

| Risk Level | Description of consequence or impact | Consequence | Likelihood/Probability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Unlikely (1) | Possible (2) | Almost Certain (3) |
| High Level of harm | Actual/Potential fatality, disability or irreversible damage. Major structural failure/damage. Off-site environmental discharge/release not contained and significant long-term environmental harm. | Major (3) | Medium (3) | High (6) | High (9) |
| Medium Level of harm | Actual/Potential temporary disability, MTI or LTI. Structural failure/damage, >1-day outage. On-site environmental discharge/release contained, minor remediation, short-term environmental harm. | Moderate (2) | Low (2) | Medium (4) | High (6) |
| Low Level of harm | Incident that has the potential to cause persons to require first aid. Environmental discharge/release immediately contained, minor level clean-up with no short term environmental harm. | Minor (1) | Low (1) | Low (2) | Medium (3) |
| Level | Likelihood/Probability |  |  |  |  |
| Almost Certain | Occurs frequently; >66% chance of occurring |  |  |  |  |
| Possible | Could happen occasionally; >33% but <66% chance of occurring |  |  |  |  |
| Unlikely | May occur only in exceptional circumstances; <33% chance of occurring |  |  |  |  |
| Class/Ranking | Description/Requirements |  |  |  |  |
| High 6, 9 | Stop immediately. Implement permanent preventative controls. |  |  |  |  |
| Medium 3, 4 | Will require operational planning. Actions will be recorded on a SWMS. |  |  |  |  |
| Low 1, 2 | Will require detailed pre-planning. Actions will be recorded on a SWMS. |  |  |  |  |

SWMS has been developed using the hierarchy of controls to reduce the risk and likely hood of an incident/injury occurring. The use of Elimination, Substitution, Engineering/isolation, Administrative and PPE measures will be used in the above order of preference wherever practicable.

Induction confirmation:

This SWMS has been developed in consultation with the workers and has been read and signed by all workers involved in this activity. I note that I have read and understood the above safe work method statement and will ensure work is completed accordingly.

| Date: | Name: | Signature: |
| --- | --- | --- |
| Date: | Name: | Signature: |