| NOTE: Work must be performed in accordance with this SWMS. This SWMS must be kept and be available for inspection until the high-risk construction work to which this SWMS relates is completed. If the SWMS is revised, all versions should be kept. If a notifiable incident occurs in relation to the high-risk construction work in this SWMS, the SWMS must be kept for at least 2 years from the date of the notifiable incident. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| PCBU Name & Contact Details: Robertson’s Remedial and Painting Pty Ltd # Unit 10/56 Buffalo Road Gladesville NSW 2111 Phone: (02) 9181 3519 A.B.N 16 140 746 247 |  |  |  | Principal Contractor (PC): | Robertson’s Remedial and Painting Pty Ltd |
| Work Manager Contact Number: | Dave Oste - 0476 265 066 |  |  | Date SWMS provided to PC: | 06-02-2026 |
| Work Activity | Performing Industrial Rope Access |  |  | Workplace Location: | 6a Francis Street, Bondi Beach |
| High Risk Construction Work: | Risk of person falling more than 2 metres (note: in some jurisdictions, this is 3metres) |  | Work on a telecommunication tower |  | Demolition of a load-bearing structure |
|  | Likely to involve disturbing asbestos |  | Temporary load-bearing support for structural alterations or repairs |  | Work in or near a confined space |
|  | Work in or near a shaft or trench deeper than 1.5m or a tunnel |  | Work in an area that may have a contaminated or flammable atmosphere. |  | Work on or near pressurised gas mains or piping |
|  | Work on or near chemical, fuel or refrigerant lines |  | Work on or near energised electrical installations |  | Use of explosives |
|  | Diving Work |  | Work on, in or adjacent to a road, railway, shipping lane or other traffic corridor in use by traffic other than pedestrians |  | Work in an area with movement of powered mobile plant. |
|  | Work in areas with artificial extremes of temperature |  | Work in or near water or other liquid that involves a risk of drowning |  | Tilt-up or precast concrete elements |
| Name of workers consulted in the development of this SWMS: |  |  |  | Signature: |  |
| Person responsible for ensuring compliance with SWMS: |  |  |  | Date SWMS received: |  |
| What measures are in place to ensure compliance with the SWMS? |  | Toolbox meetings, SWMS sign off, job observations and supervision review. If issues with the SWMS or new hazards are identified, the supervisor must be notified. When changes are made to SWMS, it will be communicated to all workers. |  |  |  |
| Person responsible for approving and reviewing SWMS control measures: |  |  |  | Date SWMS received by reviewer: |  |
| How will the SWMS control measures be reviewed? |  | The control measures implemented will be reviewed and if necessary, revised annually or if work methods change, the control measures are not effective in controlling the risk, a new hazard/risk is identified or following an incident. The SWMS will be reviewed in consultation with workers and/or others who may be affected by the SWMS. Any changes to the SWMS will be communicated with workers at induction, daily pre-starts and toolbox talks. |  |  |  |
| Review Date: |  |  |  | Reviewer’s signature: |  |

| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| --- | --- | --- | --- | --- | --- |
| Planning for preparation | Untrained or unlicensed workers may access site causing serious injury to self or other individuals. | Medium (3) | All workers must have Construction Induction Training. All workers must complete site induction upon arrival to site and sign in/out. Workers must be competent and hold relevant training certificate (minimum IRATA Level 1) in Industrial Rope Access. All workers to be trained in SWMS and sign off. Competent person to supervise all new workers. Level 3 training certificate required to supervise Industrial Rope Access. Pre-start meeting to be conducted prior to commencement of works. All workers to be Fit For Work - Zero tolerance for Alcohol & Drugs, and must not be fatigued. | Low (1) | Workers & Supervisor |
|  | Inappropriate or damaged tools and equipment allocation | Medium (3) | Risk Assessment of site to be carried out prior to commencement of work. Pre-start inspections/checks completed on all tools and equipment to ensure no damage or faults. Management to ensure that adequate worker numbers are allocated to complete the works. Ensure correct, fitted and good condition PPE is available and is worn. Ensure tools and equipment are right design for activity. Workers to ensure electrical devices are checked prior to commencing work and a current passed test tag is attached to the device. Ensure adequate first aid kits and amenities are available. | Low (1) | Workers & Supervisor |
|  | Inappropriate transportation/storage of products. | Low (2) | Materials must be securely stored & transported in accordance with Safety Data Sheet (SDS) safety measures. Spill kits and firefighting equipment to be available on site to manage emergency situations. | Low (1) | Workers & Supervisor |
|  | Weather Conditions | Low (2) | Operator to assess weather conditions including wind and rain to ensure that conditions are safe for work to commence. All work to cease if weather conditions become unsafe i.e. high winds or rain or extreme heat. Workers must wear appropriate PPE including: Sun cream, sun hats, long sleeve shirts, and tinted safety glasses while working outside. Workers must remain hydrated, regularly drinking water. Workers to stay out of sun where possible and implement rest breaks and job rotation. | Low (1) | Workers |
| Drive to site | Fire or emergency in vehicle | Low (2) | Daily Pre-start check on vehicles. Vehicles must have Fire Extinguisher with current test tag. Spill Kit to be carried in vehicle. For all emergencies drivers must attempt to drive car to side of road and put hazard lights on and ensure park break is on. Take care when departing vehicle. | Low (1) | Workers |
|  | Contact with other vehicles or infrastructure. | Medium (4) | Drivers to be licensed, experienced and competent in driving. Workers to drive in a safe manner at all times and obey all road rules. Workers to comply with drug and alcohol and fatigue policies, at all times. No use of phones while operating vehicles. | Low (1) | Workers |
| Accessing the site | Slips, trips and falls | Medium (3) | Risk assess area of works for any slip, trip and falls hazards. Be cautious of pathway ahead, uneven terrains, obstructing items. Eliminate any hazards visible. All workers to wear steel cap work boots. Onsite inductions to be completed prior to entering the job. Workers not to access areas where risk of slip trip or fall is present without appropriate protection in place. | Low (1) | Workers & Supervisor |
|  | Exposure to animals, insects, other pests, wildlife and dangerous plants. | Low (2) | Be aware of any dangerous animal’s, insects etc and avoid where ever possible. Discuss any recent sightings with the site supervisor prior to commencing work. Have available emergency contacts for capturing of animals. Wear leather gloves, Long sleeve pants and shirts. | Low (1) | Workers & Supervisor |
| Secure job site to ensure public do not access | Injury to public or workers due to individuals entering work site | Medium (3) | Painting to be completed during quite times where possible. Appropriate fencing to be erected to prevent public access to work area. Appropriate signage to be erected indicating work site unauthorised access not permitted. Spotters to be used to observe public and re-direct as needed. Alternate access to be provided to public where appropriate. Implement traffic management plan and traffic control if working near live traffic. | Low (1) | Workers |
| Unload materials and equipment from vehicle | Manual handling injury | Medium (4) | Workers to follow correct manual handling procedures: - Use mechanical lifting devices and trolleys. - Only lift objects within your safe lifting limit. - Use team lifts, where needed. Clear a path, assess the load, get a good grip, ensure a wide stance, bend the knees, keep the spine straight, use the legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely | Low (2) | Workers |
|  | Inappropriate storage areas may interfere with public | Medium (3) | Materials must be securely stored & transported in accordance with SDS. Do not place hazardous substances near gutters and drains. Place in bunding or nappy’s that holds capacity of substance. Spill kits and firefighting equipment to be available on site to manage emergency situations. | Low (1) | Workers |
| Conduct activity at heights with rope access | Objects falling from height | High (6) | All tools equipment and materials to be secured with lanyards. Implement Exclusion / Drop zones around the worker at height. Appropriate barriers and signage to be erected to assist in protecting the site where necessary. Hard hats to be worn at all times. | Low (2) | Workers |
|  | Electrocution, Exposure to radio frequency | High (6) | Where possible all power to be relocated or isolated prior to contact. Operator to ensure safe distance from all live services/radio towers. Tiger tails to be placed on overhead powerlines. Use non-conductive, insulated tools only. Fibreglass poles to be used for rolling, where necessary. | Medium (3) | Workers |
| Conduct activity at heights with rope access (continued) | Fall from heights, Suffocation | High (9) | Conduct daily Pre-start Inspections and ensure all equipment is safe for use. Remove faulty or damaged equipment immediately and notify supervisor. Ensure that equipment is set up as per manufactures instructions. Competent and Licenced workers with minimum IRATA Level 1) in Industrial Rope Access. Level 2 or 3 IRATA trained person available for rescues. Ensure all anchor points are certified by structural engineer and safe for use. Check compliance plates, engineers rating specification & suitability for task. Ensure safety line is in place at all times and do not attach any items to it. Never work alone. Have licenced IRATA Level 3 Supervisor. Use rope protection when attaching slings and ropes to prevent damage. Place a knot in the end of the safety and access ropes to prevent running off. All equipment over 8kg must be attached with a separate line. Operator to wear hard hat with chin strap at all times. Descend at a slow pace, ensuring that the descender does not heat up. Measure the height you are working at, height and weight of worker. Ensure harness is set-up to take these factors into account if falls occur (fall arrest system stops before the worker hits the ground). Team members to complete check on each other to ensure equipment set up appropriately. Ensure safety harness is correct size and fit, not too tight (may cause suffocation). | Medium (3) | Workers |
| Rescue | Injury to operator, Suspension Trauma | High (9) | Workers must work in teams of two to ensure that in the event of a fall they can be rescued. Level 2 or 3 IRATA trained person available for rescues. In the event of a fall ensure suspended worker is safely removed from harness immediately. Do not leave person hanging for more than 5 minutes! Where fitted, use emergency rescue equipment to escape from harness. Where necessary, use access equipment to remove worker from heights. Individuals in fall must be reviewed by doctor and gain clearance to work. Operators must have the ability to ascend and descend at all times. All equipment subject to a fall is to be removed from service and recertified. Spotter to be in constant visual, verbal or radio communication Suspended worker to try keep moving leg muscles, keep blood circulating. If at risk of being suspended after a fall, worker to use front attachment points on fall-arrest harnesses. | Medium (3) | Workers |

| Personal Protective Equipment (PPE) required: | Enclosed leather steel capped boots Safety Glasses (As per AS/NZS 1337) Gloves (As per AS/NZS 2161) | Hard Hats with Strap (Helmets) High visibility clothing, Long Sleeve Shirts & Pants Sun hats, sun cream- when working outdoors |
| --- | --- | --- |
| List the training required by workers to complete the work: List the qualifications of personnel doing the work: | General Construction Induction Training Card (SafeWork NSW or equivalent) Induction into SWMS Minimum qualified IRATA Level 1 Technician Work Safely at Heights Training Painting Trades Certificate / Licence |  |
| List the Permits, Certificates, SafeWork NSW Approvals, required to completed the work: | All necessary permits to be completed on site |  |
| Relevant legislation: For further details refer to the Legal Requirements Register | Work in accordance with Work Health and Safety Act 2011 and Work Health and Safety Regulation 2025, workplace injury management and workers compensation act 1998 no. 86, AS/NZS1337.1 Personal eye protection-Eye and face protectors for occupational applications, AS/NZS 1269.3 Occupational Noise management-Hearing protection program, AS/NZS 1801 industrial safety helmets, AS/NZS 2210.1 Safety, protective and occupational footwear-Guide to selection, care and use & AS/NZS 2210.2 to 2210.9 Occupational protective footwear. SafeWork NSW Codes of Practice: - Construction Work - Managing risks of plant - Managing the risk of falls - Work Health and Safety Consultation, Co-operation and Co-ordination - Managing the Work Environment and Facilities - First Aid in the Workplace - Hazardous Manual Tasks - How to Manage Work Health and Safety Risks |  |
| List the plant / equipment that will be used on site: | Harness, Helmet, Ascenders/Descenders, Backup Fall Arrest Device, Pulley, Foot loops & Etrier, Karabiners, Industrial Rope, Anchors, Lanyards, |  |
| List maintenance checks for plant & equipment: | Daily visual inspections |  |
| Frequency of site/workplace inspections: Site Inspections will be completed by: | Daily site inspections & job completion inspections to be completed by Site Supervisor. |  |
| Hazardous Substances: |  |  |

# Sign Off and Approvals

| Person responsible for site supervision of the work, inspecting and approving work areas, approval of and compliance with SWMS, protective measures, plant, equipment: |  |
| --- | --- |
| Name and position: | Signature: |
| SWMS Prepared by: Ben Knox – Compliance OH&S | Signature: |

| Risk Level | Description of consequence or impact | Consequence | Likelihood/Probability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Unlikely (1) | Possible (2) | Almost Certain (3) |
| High Level of harm | Actual/Potential fatality, disability or irreversible damage. Major structural failure/damage. Off-site environmental discharge/release not contained and significant long-term environmental harm. | Major (3) | Medium (3) | High (6) | High (9) |
| Medium Level of harm | Actual/Potential temporary disability, MTI or LTI. Structural failure/damage, >1-day outage. On-site environmental discharge/release contained, minor remediation, short-term environmental harm. | Moderate (2) | Low (2) | Medium (4) | High (6) |
| Low Level of harm | Incident that has the potential to cause persons to require first aid. Environmental discharge/release immediately contained, minor level clean-up with no short term environmental harm. | Minor (1) | Low (1) | Low (2) | Medium (3) |
| Level | Likelihood/Probability |  |  |  |  |
| Almost Certain | Occurs frequently; >66% chance of occurring |  |  |  |  |
| Possible | Could happen occasionally; >33% but <66% chance of occurring |  |  |  |  |
| Unlikely | May occur only in exceptional circumstances; <33% chance of occurring |  |  |  |  |
| Class/Ranking | Description/Requirements |  |  |  |  |
| High 6, 9 | Stop immediately. Implement permanent preventative controls. |  |  |  |  |
| Medium 3, 4 | Will require operational planning. Actions will be recorded on a SWMS. |  |  |  |  |
| Low 1, 2 | Will require detailed pre-planning. Actions will be recorded on a SWMS. |  |  |  |  |

SWMS has been developed using the hierarchy of controls to reduce the risk and likely hood of an incident/injury occurring. The use of Elimination, Substitution, Engineering/isolation, Administrative and PPE measures will be used in the above order of preference wherever practicable.

Induction confirmation:

This SWMS has been developed in consultation with the workers and has been read and signed by all workers involved in this activity. I note that I have read and understood the above safe work method statement and will ensure work is completed accordingly.

| Date: | Name: | Signature: |
| --- | --- | --- |
| Date: | Name: | Signature: |