| NOTE: Work must be performed in accordance with this SWMS. This SWMS must be kept and be available for inspection until the high-risk construction work to which this SWMS relates is completed. If the SWMS is revised, all versions should be kept. If a notifiable incident occurs in relation to the high-risk construction work in this SWMS, the SWMS must be kept for at least 2 years from the date of the notifiable incident. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| PCBU Name & Contact Details: Robertson’s Remedial and Painting Pty Ltd # Unit 10/56 Buffalo Road Gladesville NSW 2111 Phone: (02) 9181 3519 A.B.N 16 140 746 247 |  |  |  |  | Principal Contractor (PC): |  |
| Work Manager Contact Number: |  |  |  |  | Date SWMS provided to PC: |  |
| Work Activity | Carpentry |  |  |  | Workplace Location: |  |
| High Risk Construction Work: | Risk of person falling more than 2 metres |  |  | Work on a telecommunication tower |  | Demolition of a load-bearing structure |
|  | Likely to involve disturbing asbestos |  |  | Temporary load-bearing support for structural alterations or repairs |  | Work in or near a confined space |
|  | Work in or near a shaft or trench deeper than 1.5m or a tunnel |  |  | Work in an area that may have a contaminated or flammable atmosphere. |  | Work on or near pressurised gas mains or piping |
|  | Work on or near chemical, fuel or refrigerant lines |  |  | Work on or near energised electrical installations |  | Use of explosives |
|  | Diving Work |  |  | Work on, in or adjacent to a road, railway, shipping lane or other traffic corridor in use by traffic other than pedestrians |  | Work in an area with movement of powered mobile plant. |
|  | Work in areas with artificial extremes of temperature |  |  | Work in or near water or other liquid that involves a risk of drowning |  | Tilt-up or precast concrete elements |
| Name of workers consulted in the development of this SWMS: |  |  |  |  | Signature: |  |
| Person responsible for ensuring compliance with SWMS: |  |  |  |  | Date SWMS received: |  |
| What measures are in place to ensure compliance with the SWMS? |  |  | Toolbox meetings, SWMS sign off, job observations and supervision review. If issues with the SWMS or new hazards are identified, the supervisor must be notified. When changes are made to SWMS, it will be communicated to all workers. |  |  |  |
| Person responsible for approving and reviewing SWMS control measures: |  |  |  |  | Date SWMS received by reviewer: |  |
| How will the SWMS control measures be reviewed? |  |  | The control measures implemented will be reviewed and if necessary, revised annually or if work methods change, the control measures are not effective in controlling the risk, a new hazard/risk is identified or following an incident. The SWMS will be reviewed in consultation with workers and/or others who may be affected by the SWMS. Any changes to the SWMS will be communicated with workers at induction, daily pre-starts and toolbox talks. |  |  |  |
| Review Date: |  |  |  |  | Reviewer’s signature: |  |

| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| --- | --- | --- | --- | --- | --- |
| Planning for the set-up of job | Untrained or unlicensed workers may access site | Medium (3) | All workers must have Construction Induction Training Card. All workers to be qualified/ ticketed/ trained as required for the works. All workers must complete site induction and sign in / out. Liaise with site management about site hazards, emergency procedures and any other site requirements. All workers to be trained in and sign SWMS. Pre-start meeting to be conducted prior to commencement of works All workers must be fit for work, zero blood alcohol concentration, not fatigued or under the influence of drugs. Competent person to supervise all new workers. | Low (1) | Workers & Supervisor |
|  | Inappropriate tool and equipment allocation | Medium (3) | Ensure appropriate equipment and procedures are used for the works being completed. Ensure all electrical equipment have a current and passed test tag prior to commencement of work. Ensure that adequate PPE is available to workers | Low (1) | Workers & Supervisor |
| Accessing and securing the site | Slips, trips and falls | Medium (3) | All workers to wear steel capped boots and hard hats. Identify any slip trip and fall hazards and appropriately manage. Ensure the work area is clear and free of slip, trip and fall hazards. | Low (1) | Workers |
|  | Weather conditions (heat, rain, wind) | Medium (3) | Workers must remain hydrated through day, regularly drinking water. Workers to stay out of sun where possible and implement rest breaks or job rotation where necessary. Consider not working in extreme heat and rainy events when planned to work outdoors. Workers must wear appropriate PPE: sun cream, sun hats, and safety glasses. | Low (1) | Workers |
|  | Unauthorised individuals entering work site | Medium (3) | Appropriate barricades & signage to be erected to prevent unauthorised access to work area. Where appropriate, spotters to be used to observe personnel and re direct where needed. Alternate access to be provided where appropriate. | Low (1) | Workers & Supervisor |
| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| Deliver and unload materials | Manual handling | Medium (4) | Workers to follow correct manual handling procedures: Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit. Use team lifts where needed. Clear a path, assess load, get a good grip, ensure a wide stance, bend the knees, keep back straight, use legs to lift, do not twist back, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Workers |
|  | Inappropriate storage areas may interfere with public | Medium (3) | Workers to discuss material storage area and work areas with site controller prior to commencement. Appropriate work and storage areas to be barricaded off. Materials delivered and handled as per SDS, where required. | Low (1) | Workers |
| Isolate all electrical, water and gas services | Contact with services or cables within building | High (6) | Ensure all services have been identified and located prior to commencing work. Refer to design plans and search wall cavities to identify. Allow for inaccuracies in plans and take caution. Do not work on or near live services. Isolate, Lock Out / Tag services in work area. Capped rubber sole boots, protective insulated gloves must be worn. Completed Isolation sign off to be physically handed to foreman from the licensed electrical or plumbing contractors before any work commences. | Low (2) | Workers |
| Remove existing materials / fixings | Manual handling | Medium (4) | Workers to follow correct manual handling procedures: Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit. Use team lifts where needed. Clear a path, assess load, get a good grip, ensure a wide stance, bend the knees, keep back straight, use legs to lift, do not twist back, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Workers |
|  | Fall from heights | High (6) | Select appropriate fall protection device (Scaffold, EWP, fixed edge protection) to safely perform work at heights and follow the SWI. Ensure worker is competent in operating EWP. Ensure Scaffold is deemed safe to use. Complete Pre-start inspections daily & logbook. Provide Safe access / egress to work area. Fully enclosed, slip resistant footwear should be worn. If wearing fall arrest system, ensure emergency rescue plan in place. | Low (2) | Workers |
| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| Remove existing materials / fixings | Fall from Ladder | High (6) | Use a platform fiberglass ladder where possible. Perform visual check on ladder for obvious faults and defects prior to use. Damaged ladders are to be removed from use, tag “not for use” and repair or replace ASAP. Fully enclosed, slip resistant footwear should be worn. When gaining access to area, the top of the ladder should extend above the working platform/roof by no less than 1m. Never “walk” the ladder to reposition it; descend the ladder and relocate it. Erect ladder on clean, level surface or use self-levelling ladder. Erect extension ladder in a 4:1 ratio (4m up for every 1m from the wall) and tie off at top and bottom. Use a footer where necessary. Do not place a ladder in front of a doorway unless locked/guarded. Secure work area around the ladder. Clean mud, grass etc, from boots to prevent slippage. Do not over-reach while on ladder. Only one person on a ladder at any one time. Ensure legs are fully extended and latch is in place Maintain three points of contact on ladder Do not step above the third step from the top. Where working from more than a couple of stories high a double lanyard fall restraint system is to be used. | Low (2) | Workers |
|  | Objects falling from height | Medium (4) | Use a tool pouch to carry tools. Tie tools to lanyards. Do not leave tools and equipment on ladder rungs. Workers to wear hard hat and safety boots. Implement drop zone around work area. | Low (2) | Workers |
|  | Incorrect use of equipment | Medium (4) | Follow manufacturers manual for equipment used on site e.g. grinders, sanders, drills, saws etc. Workers to wear required PPE: safety glasses, steel capped boots, gloves etc. Ensure that all electrical devices have an electrical test tag prior to commencement of work. Daily pre-start checks on all equipment to identify any damage and/or defects. If identified, remove from use and tag ‘Out of Service’ and get it repaired or replaced ASAP. | Low (2) | Workers |
| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| Remove existing materials / fixings | Dust generated | Medium (3) | Use power tools with dust extraction. Filters to be cleaned regularly. Ensure that there is appropriate ventilation in the work area – work outdoors, open windows and doors. Wear dust mask or respirator and safety glasses when sanding, grinding, drilling, cutting. Cover all areas with plastic or appropriate sheeting. Avoid sanding, grinding or drilling above head height. | Low (2) | Workers |
| Check materials for compliance to spec and standards | Inappropriate materials | Medium (4) | Inspect all materials to ensure that they are the correct type, quantity, labelled correctly and have are not damaged. Check all materials purchased comply with spec and that they meet relevant Australian Standards. | Low (2) | Workers |
| Cut materials to length | Inappropriate tools Rotating blades / Moving parts contact Worker complacency | Medium (4) | Adopt a comfortable stance, with feet apart and well balanced. Never use a saw between the legs while sitting on the floor. Ensure work is around waist height, not above head or shoulder height. Only use saw for materials that the blade is designed for. Ensure equipment is in good condition and safe to use. Ensure the blade is fit appropriately and not worn out / damaged. Ensure all guards are in place and in good condition. Disconnect the power when not in use, Remove the plug from the power point. Wait until the blade has stopped rotating before putting tool down. Keep hands and body parts clear of saw blade. Never cut towards a body part. Hold saw with two hands and a firm grip at all times. Ensure material being cut stays in contact with the front half of the blade. Ensure all individuals are moved away to a safe distance. Ensure that material being cut is secure prior to cutting. Wear protective gloves, eye protection. | Low (2) | Workers |
|  | Dust generated | Medium (3) | Use power tools with dust extraction. Filters to be cleaned regularly. Ensure that there is appropriate ventilation in the work area – work outdoors, open windows and doors. Wear dust mask or respirator and safety glasses when sanding, grinding, drilling, cutting (especially when cutting MDF of other hazardous substances). Cover all areas with plastic or appropriate sheeting. Avoid sanding, grinding or drilling above head height. | Low (2) | Workers |
| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| Cut materials to length | Device or power cord damaged | High (6) | Do not use electrical equipment in wet weather or where spills are - mop up any spills in the area. Prior to use, check electrical equipment for a current and passed test tag. Untested machinery is to be removed from service and tagged. Check equipment and power lead for damage or defects prior to use. Ensure that the power outlet is not damaged. Defective electrical equipment, tools or cords are tagged ‘Out of Service’. All electrical leads are kept off walkways and raised off the ground away from water/corrosives/heat/ friction and other forms of physical damage. Use lead stands. Electrical equipment to be connected to an RCD (tested & tagged) at all times. Extension leads not to exceed 30 meters. Do not ‘piggy back’ power leads. | Low (2) | Workers |
| Install materials | Fall from heights | High (6) | Select appropriate fall protection device (Scaffold, EWP, fixed edge protection) to safely perform work at heights and follow the SWI. Ensure worker is competent in operating EWP. Ensure Scaffold is deemed safe to use. Complete Pre-start inspections daily & logbook. Provide Safe access / egress to work area. Fully enclosed, slip resistant footwear should be worn. If wearing fall arrest system, ensure emergency rescue plan in place. | Low (2) | Workers |
|  | Objects falling from height | Medium (4) | Use a tool pouch to carry tools. Tie tools to lanyards. Do not leave tools and equipment on ladder rungs. Workers to wear hard hat and safety boots. Implement drop zone around work area. | Low (2) | Workers |
|  | Timbers falling | Medium (3) | Bracing and supporting timbers to be in place where necessary Timbers to be soundly secured and supported Workers to ensure that sufficient nails are placed in timbers to prevent falling | Low (1) | Workers |
|  | Nails into body from nail gun | High (6) | Complete visual check of gun prior to use Eye protection to be used when operating nail gun. Never point nail gun in direction of others Ensure that safety guard is working appropriately Place nail gun on timber prior to pressing trigger | Low (2) | Workers |
| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| Install materials | Fall from Ladder | High (6) | Use a platform fiberglass ladder where possible. Perform visual check on ladder for obvious faults and defects prior to use. Damaged ladders are to be removed from use, tag “not for use” and repair or replace ASAP. Fully enclosed, slip resistant footwear should be worn. When gaining access to area, the top of the ladder should extend above the working platform/roof by no less than 1m. Never “walk” the ladder to reposition it; descend the ladder and relocate it. Erect ladder on clean, level surface or use self-levelling ladder. Erect extension ladder in a 4:1 ratio (4m up for every 1m from the wall) and tie off at top and bottom. Use a footer where necessary. Do not place a ladder in front of a doorway unless locked/guarded. Secure work area around the ladder. Clean mud, grass etc, from boots to prevent slippage. Do not over-reach while on ladder. Only one person on a ladder at any one time. Ensure legs are fully extended and latch is in place Maintain three points of contact on ladder Do not step above the third step from the top. Where working from more than a couple of stories high a double lanyard fall restraint system is to be used. | Low (2) | Workers |
|  | Contact with working hammer | Medium (3) | Keep body parts clear of hammer. Ensure item is secure when hammering. | Low (1) | Workers |
|  | Inappropriate tools Moving parts contact Worker complacency | Medium (4) | Ensure work is around waist height, not above head or shoulder height. Ensure equipment is in good condition and safe to use. Ensure all guards are in place and in good condition. Disconnect the power when not in use, remove the plug from the power point. Keep hands and body parts clear of tools. Hold tools with a firm grip at all times. Ensure all individuals are moved away to a safe distance. Ensure that material being cut is secure prior to cutting. Wear PPE: protective gloves, eye protection. | Low (2) | Workers |
| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| Install materials | Manual handling | Medium (4) | Workers to follow correct manual handling procedures: Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit. Use team lifts where needed. Clear a path, assess load, get a good grip, ensure a wide stance, bend the knees, keep back straight, use legs to lift, do not twist back, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Workers |
| Clean-up site | Housekeeping | Medium (3) | All tools and equipment must be cleaned following work. Store away in secure area at all times. Maintain housekeeping throughout day. Clean-up off-cuts, shavings, spills etc. Reel up power leads and store appropriately. | Low (2) | Workers |
|  | Manual handling | Medium (4) | Workers to follow correct manual handling procedures: Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit. Use team lifts where needed. Clear a path, assess load, get a good grip, ensure a wide stance, bend the knees, keep back straight, use legs to lift, do not twist back, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Workers |

| Personal Protective Equipment (PPE) required: | Enclosed leather steel capped Safety Glasses Protective Gloves | Hard Hat Dusk Mask / Respirator High visibility clothing Sun hats, sun cream- when working outdoors |
| --- | --- | --- |
| List the training required by workers to complete the work: List the qualifications of personnel doing the work: | Carpenter Trade Certificate Construction induction certificate (White card) Induction into SWMS |  |
| List the Permits, Certificates, SafeWork NSW Approvals, required to complete the work: |  |  |
| Relevant Legislation: For further details refer to the Legal Requirements Register | Work Health and Safety Act 2011 and Work Health and Safety Regulation 2017, Workplace Injury Management and Workers Compensation Act 1998 no. 86, Relevant codes of practice and SafeWork NSW requirements. National Code of Practice for Manual Handling, AS/NZS1337.1 Personal eye protection-Eye and face protectors for occupational applications (2010), AS/NZS 1269.3 Occupational Noise Management-Hearing protection program (2005), AS/NZS 1801 industrial safety helmets (1997), AS/NZS 2210.1 Safety, protective and occupational footwear-Guide to selection, care and use (2010), AS/NZS 2210.2 to 2210.9 Occupational protective footwear. SafeWork NSW Codes of Practice: - Construction Work - Managing risks of plant - Managing the risk of falls - Work Health and Safety Consultation, Co-operation and Co-ordination - Managing the Work Environment and Facilities - First Aid in the Workplace - Hazardous Manual Tasks - How to Manage Work Health and Safety Risks |  |
| List the plant / equipment that will be used on site: | Portable Power Tools |  |
| List maintenance checks for plant & equipment: | Electrical equipment Test & Tag; Daily Pre-Start inspection |  |
| Frequency of site/workplace inspections: Site Inspections will be completed by: | Daily site inspections & job completion inspections to be completed by Site Supervisor. |  |
| Hazardous Substances: |  |  |

# Sign Off and Approvals

| Person responsible for site supervision of the work, inspecting and approving work areas, approval of and compliance with SWMS, protective measures, plant, equipment: |  |
| --- | --- |
| Name and position: | Signature: |
| SWMS Prepared by: Ben Knox – Compliance OH&S | Signature: |

| Risk Level | Description of consequence or impact | Consequence | Likelihood/Probability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Unlikely (1) | Possible (2) | Almost Certain (3) |
| High Level of harm | Actual/Potential fatality, disability or irreversible damage. Major structural failure/damage. Off-site environmental discharge/release not contained and significant long-term environmental harm. | Major (3) | Medium (3) | High (6) | High (9) |
| Medium Level of harm | Actual/Potential temporary disability, MTI or LTI. Structural failure/damage, >1-day outage. On-site environmental discharge/release contained, minor remediation, short-term environmental harm. | Moderate (2) | Low (2) | Medium (4) | High (6) |
| Low Level of harm | Incident that has the potential to cause persons to require first aid. Environmental discharge/release immediately contained, minor level clean-up with no short term environmental harm. | Minor (1) | Low (1) | Low (2) | Medium (3) |
| Level | Likelihood/Probability |  |  |  |  |
| Almost Certain | Occurs frequently; >66% chance of occurring |  |  |  |  |
| Possible | Could happen occasionally; >33% but <66% chance of occurring |  |  |  |  |
| Unlikely | May occur only in exceptional circumstances; <33% chance of occurring |  |  |  |  |
| Class/Ranking | Description/Requirements |  |  |  |  |
| High 6, 9 | Stop immediately. Implement permanent preventative controls. |  |  |  |  |
| Medium 3, 4 | Will require operational planning. Actions will be recorded on a SWMS. |  |  |  |  |
| Low 1, 2 | Will require detailed pre-planning. Actions will be recorded on a SWMS. |  |  |  |  |

SWMS has been developed using the hierarchy of controls to reduce the risk and likely hood of an incident/injury occurring. The use of Elimination, Substitution, Engineering/isolation, Administrative and PPE measures will be used in the above order of preference wherever practicable.

Induction confirmation:

This SWMS has been developed in consultation with the workers and has been read and signed by all workers involved in this activity. I note that I have read and understood the above safe work method statement and will ensure work is completed accordingly.

| Date: | Name: | Signature: |
| --- | --- | --- |
| Date: | Name: | Signature: |