| NOTE: Work must be performed in accordance with this SWMS. This SWMS must be kept and be available for inspection until the high-risk construction work to which this SWMS relates is completed. If the SWMS is revised, all versions should be kept. If a notifiable incident occurs in relation to the high-risk construction work in this SWMS, the SWMS must be kept for at least 2 years from the date of the notifiable incident. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| PCBU Name & Contact Details: Robertson’s Remedial and Painting Pty Ltd # Unit 10/56 Buffalo Road Gladesville NSW 2111 Phone: (02) 9181 3519 A.B.N 16 140 746 247 |  |  |  |  | Principal Contractor (PC): |  |
| Work Manager Contact Number: |  | Dave Oste - 0476 265 066 |  |  | Date SWMS provided to PC: | 06-02-2026 |
| Work Activity | Tiling of floors and Walls |  |  |  | Workplace Location: | 6a Francis Street, Bondi Beach |
| High Risk Construction Work: | Risk of person falling more than 2 metres (note: in some jurisdictions, this is 3metres) |  |  | Work on a telecommunication tower |  | Demolition of a load-bearing structure |
|  | Likely to involve disturbing asbestos |  |  | Temporary load-bearing support for structural alterations or repairs |  | Work in or near a confined space |
|  | Work in or near a shaft or trench deeper than 1.5m or a tunnel |  |  | Work in an area that may have a contaminated or flammable atmosphere. |  | Work on or near pressurised gas mains or piping |
|  | Work on or near chemical, fuel or refrigerant lines |  |  | Work on or near energised electrical installations |  | Use of explosives |
|  | Diving Work |  |  | Work on, in or adjacent to a road, railway, shipping lane or other traffic corridor in use by traffic other than pedestrians |  | Work in an area with movement of powered mobile plant. |
|  | Work in areas with artificial extremes of temperature |  |  | Work in or near water or other liquid that involves a risk of drowning |  | Tilt-up or precast concrete elements |
| Name of workers consulted in the development of this SWMS: |  |  |  |  | Signature: |  |
| Person responsible for ensuring compliance with SWMS: |  |  |  |  | Date SWMS received: |  |
| What measures are in place to ensure compliance with the SWMS? |  |  | Toolbox meetings, SWMS sign off, job observations and supervision review. If issues with the SWMS or new hazards are identified, the supervisor must be notified. When changes are made to SWMS, it will be communicated to all workers. |  |  |  |
| Person responsible for approving and reviewing SWMS control measures: |  |  |  |  | Date SWMS received by reviewer: |  |
| How will the SWMS control measures be reviewed? |  |  | The control measures implemented will be reviewed and if necessary, revised annually or if work methods change, the control measures are not effective in controlling the risk, a new hazard/risk is identified or following an incident. The SWMS will be reviewed in consultation with workers and/or others who may be affected by the SWMS. Any changes to the SWMS will be communicated with workers at induction, daily pre-starts and toolbox talks. |  |  |  |
| Review Date: |  |  |  |  | Reviewer’s signature: |  |

| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| --- | --- | --- | --- | --- | --- |
| Planning for the set-up of job. | Untrained or unlicensed workers may access site | Medium (3) | All workers must have Construction Induction Training. All workers to be qualified/ licensed/ ticketed/ trained as required for the works. All workers must complete site induction and sign in / out. Where induction is not available, liaise with site supervisor about site hazards, emergency procedures and any other site requirements. All workers to ensure they are familiar with the emergency muster points of the site and follow emergency procedures. All workers to be trained in and sign safe work method statement. Pre-start meeting to be conducted prior to commencement of works All workers to be in a fit state to commence work, be zero blood alcohol concentration, not suffering from fatigue or under the influence of drugs. Competent person to supervise all new workers. | Low (1) | Workers & Supervisor |
|  | Inappropriate tool and equipment allocation | Medium (3) | Site Supervisor/Management to ensure that appropriate equipment and procedures are used for the works being completed. Site Supervisor /Management to ensure that all electrical devices have an electrical test tag prior to commencement of work. Management to ensure that adequate PPE is available to workers. | Low (1) | Workers & Supervisor |
| Accessing and securing the site | Slips, trips and falls | Medium (3) | All workers to wear steel capped boots. Identify and ensure the work area is clear and free of slip, trip and fall hazards and obstructions. | Low (1) | Workers |
|  | Weather conditions | Medium (3) | Workers must remain hydrated through day regularly drinking water. Workers to stay out of sun where possible and implement rest breaks or job rotation where necessary. Workers must wear appropriate PPE including: sun cream, sun hats, and tinted safety glasses while working outside. | Low (1) | Workers |
|  | Unauthorised individuals entering work site | Medium (3) | Barricades & signage to be erected to prevent unauthorised access to work area. Where appropriate, spotters to be used to observe personnel and re direct where needed. Alternate access to be provided where appropriate. | Low (1) | Workers |
| Deliver materials to work area and position in work area | Manual handling | Medium (4) | Workers to follow correct manual handling procedures: Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit use team lifts. Clear a path, assess load, get a good grip, ensure a wide stance, bend knees, keep back straight, use legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Workers |
|  | Inappropriate storage areas | Medium (3) | Workers to discuss material storage area and work areas with site controller prior to commencement. Appropriate work and storage areas to be barricaded off. | Low (1) | Workers |
| Prepare surface | Housekeeping | Medium (3) | Safety glasses to be worn at all times Clean and remove any obstructions or items away from surface to ensure it is ready and no hazards are present. | Low (1) | Workers |
|  | Hazardous substances exposure | Medium (3) | Follow and read SDS prior to using hazardous substance. Ensure safety measures are followed. Ensure PPE is available, provided and worn for protection. Ensure area is well-ventilated. | Low (1) | Workers |
| Set up mixer | Electrical shock due to faulty equipment | High (6) | Workers to complete pre-start check on machine, leads plugs and power outlet for any damage. If faulty, remove from service and report to supervisor any repairs needed. Workers to check all fittings and hoses for damage prior to use. Workers to check that machine has current electrical test tag in place and is safe for use. Workers to check machine to ensure it is running safely Workers to ensure that RCD’s/ELCB’s are in use and that leads do not run across driveways pathways or near any water. Use lead stands where necessary. | Low (2) | Workers |
| Operating mixer | Moving parts | Medium (3) | Never place body parts or equipment into the moving mixer. Disconnect power when cleaning or servicing machine. Identify emergency stops before starting machine. Do not wear loose clothing or items of jewellery that can get caught. | Low (1) | Workers |
|  | Dust | Medium (3) | Wear dust mask, safety glasses, protective gloves and boots. Slowly add mix to mixer. Follow SDS for safety measures. | Low (1) | Workers |
| Operating mixer | Manual handling | Medium (4) | Workers to follow correct manual handling procedures: Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit use team lifts. Clear a path, assess load, get a good grip, ensure a wide stance, bend knees, keep back straight, use legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Workers |
| Apply base | Manual handling | Medium (4) | Workers to follow correct manual handling procedures: Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit use team lifts. Clear a path, assess load, get a good grip, ensure a wide stance, bend knees, keep back straight, use legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Workers |
|  | Repetitive work and prolonged fixed postures | Medium (4) | Workers must warm up and stretch prior to commencing work. Workers to rotate tasks to avoid repetitive tasks and fixed postures. Workers to avoid excessive straining and over reaching. | Low (2) | Workers |
|  | Hazardous substances exposure | Medium (3) | Follow and read SDS prior to using hazardous substance. Ensure safety measures are followed. Ensure PPE is available, provided and worn for protection. Ensure area is well-ventilated. | Low (1) | Workers |
| Cut tiles with Tile Cutter | Worker Complacency | Medium (3) | Keep hands clear of sharp edges. Wear leather gloves & safety glasses to be worn at all times. Remove sharp corners. | Low (1) | Workers |
| Cut tiles with Saw or Grinder | The disk may fail | Medium (3) | Ensure appropriate cutting disk is used Wheels should be used only for the specific material and purpose for which they are designed, and according to the manufacturer's recommendations. Ensure the correct flange and locking nut is in place. Ensure the guard and handles are secure. Ensure there are no defects or damage to the disc. | Low (1) | Workers |
|  | Device or power cord is damaged | Medium (3) | Prior to use, saw needs to be checked for current and passed electrical test tag. The power lead and tool should be checked for damage. Ensure that the power outlet is not damaged. Do not run power leads across paths or in areas where they may be damaged (e.g. run over) | Low (1) | Workers |
| Cut tiles with Saw or Grinder | Grinding disk or saw may come into contact with worker Kickback | Medium (4) | Hold saw or grinder a secure grip while operating. Allow saw to "run up" to operating speed before applying it to the job. Hold the saw against the work piece with minimum pressure, so it doesn't "dig in" and cause it to kick back. Adopt a comfortable stance, with feet apart and well balanced, and with a clear view of the job. Never use a saw between the legs while sitting on the floor. Disconnect the power and place the saw on a bench with the disc facing upwards | Low (2) | Workers |
|  | Dust inhalation | Medium (3) | Wear dust mask or respirator. Cover all areas with plastic or appropriate sheeting. Use power tools with dust extraction where possible. Open windows and use dust extraction devices where necessary. | Low (1) | Workers |
|  | Worker Contact | Medium (3) | Isolate work area. Ensure appropriate barriers and signage is installed to prevent unauthorised access to the work area. | Low (1) | Workers |
| Lay tiles on floor | Manual handling | Medium (4) | Workers to follow correct manual handling procedures: Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit use team lifts. Clear a path, assess load, get a good grip, ensure a wide stance, bend knees, keep back straight, use legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Workers |
|  | Repetitive work and prolonged fixed postures | Medium (4) | Workers must warm up and stretch prior to commencing work. Workers to rotate tasks to avoid repetitive tasks and fixed postures. Workers to avoid excessive straining and over reaching. | Low (2) | Workers |
| Lay tiles on walls | Fall from heights from ladder | High (6) | Use a platform ladder. Perform visual check on ladder for obvious faults and defects prior to use. Remove damaged ladders, tag “not for use” and repair ASAP. When gaining access to a working platform, the top of the ladder should extend above the working platform by no less than 1m. Never “walk” ladder to reposition it; descend ladder and relocate it. Erect ladder on clean, level surface or use self-levelling ladder. Erect extension ladder, 4meters up for every 1meter from the wall, and tie off at top and bottom. Use a footer where necessary. Do not place a ladder in front of a doorway unless locked/guarded. Clean mud, grass etc, from boots to prevent slippage. Do not over-reach while on ladder. Only one person on a ladder at any one time. Face the ladder at all times. Ensure legs are fully extended and latch is in place. Maintain three points of contact at all times. Do not step above the third step from the top. Fully enclosed, slip resistant footwear should be worn. Where working from more than a couple of stories high a double lanyard fall restraint system is to be used. | Low (2) | Workers |
|  | Manual handling | Medium (4) | Workers to follow correct manual handling procedures: Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit use team lifts. Clear a path, assess load, get a good grip, ensure a wide stance, bend knees, keep back straight, use legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Workers |
|  | Repetitive work and prolonged fixed postures | Medium (4) | Workers must warm up and stretch prior to commencing work. Workers to rotate tasks to avoid repetitive tasks and fixed postures. Workers to avoid excessive straining and over reaching. | Low (2) | Workers |
| Clean-up site | Housekeeping | Medium (3) | Remove waste and dispose of appropriately. All tools and equipment to be stored securely. Secure off work area below and above. | Low (1) | Workers |

| Personal Protective Equipment (PPE) required: | Enclosed leather steel capped boots Safety Glasses (As per AS/NZS 1337) Protective Gloves (As per AS/NZS 2161) | High visibility Long sleeve shirt & pants Hard Hats (As per AS/NZS 1801) Dust Mask / Respirator |
| --- | --- | --- |
| List the training required by workers to complete the work: List the qualifications of personnel doing the work: | Construction Induction Card Relevant trade certificate Induction into SWMS |  |
| List the Permits, Certificates, SafeWork NSW Approvals, required to complete the work: | All necessary permits to be completed on site |  |
| Relevant legislation: For further details refer to the Legal Requirements Register | Work in accordance with Work Health and Safety Act 2011 and Work Health and Safety Regulation 2025, workplace injury management and workers compensation act 1998 no. 86, AS/NZS1337.1 Personal eye protection-Eye and face protectors for occupational applications, AS/NZS 1269.3 Occupational Noise management-Hearing protection program, AS/NZS 1801 industrial safety helmets, AS/NZS 2210.1 Safety, protective and occupational footwear-Guide to selection, care and use & AS/NZS 2210.2 to 2210.9 Occupational protective footwear. SafeWork NSW Codes of Practice: - Construction Work - Managing risks of plant - Managing the risk of falls - Work Health and Safety Consultation, Co-operation and Co-ordination - Managing the Work Environment and Facilities - First Aid in the Workplace - Hazardous Manual Tasks - How to Manage Work Health and Safety Risks |  |
| List the plant / equipment that will be used on site: | Portable power tools, hand tools |  |
| List maintenance checks for plant & equipment: | Daily pre-start checks, maintenance as per manufacturers guidelines |  |
| Frequency of site/workplace inspections: Site Inspections will be completed by: | Daily site inspections & job completion inspections to be completed by Site Supervisor. |  |
| Hazardous Substances: | Grout & Silicone |  |

# Sign Off and Approvals

| Person responsible for site supervision of the work, inspecting and approving work areas, approval of and compliance with SWMS, protective measures, plant, equipment: |  |
| --- | --- |
| Name and position: | Signature: |
| SWMS Prepared by: Ben Knox – Compliance OH&S | Signature: |

| Risk Level | Description of consequence or impact | Consequence | Likelihood/Probability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Unlikely (1) | Possible (2) | Almost Certain (3) |
| High Level of harm | Actual/Potential fatality, disability or irreversible damage. Major structural failure/damage. Off-site environmental discharge/release not contained and significant long-term environmental harm. | Major (3) | Medium (3) | High (6) | High (9) |
| Medium Level of harm | Actual/Potential temporary disability, MTI or LTI. Structural failure/damage, >1-day outage. On-site environmental discharge/release contained, minor remediation, short-term environmental harm. | Moderate (2) | Low (2) | Medium (4) | High (6) |
| Low Level of harm | Incident that has the potential to cause persons to require first aid. Environmental discharge/release immediately contained, minor level clean-up with no short term environmental harm. | Minor (1) | Low (1) | Low (2) | Medium (3) |
| Level | Likelihood/Probability |  |  |  |  |
| Almost Certain | Occurs frequently; >66% chance of occurring |  |  |  |  |
| Possible | Could happen occasionally; >33% but <66% chance of occurring |  |  |  |  |
| Unlikely | May occur only in exceptional circumstances; <33% chance of occurring |  |  |  |  |
| Class/Ranking | Description/Requirements |  |  |  |  |
| High 6, 9 | Stop immediately. Implement permanent preventative controls. |  |  |  |  |
| Medium 3, 4 | Will require operational planning. Actions will be recorded on a SWMS. |  |  |  |  |
| Low 1, 2 | Will require detailed pre-planning. Actions will be recorded on a SWMS. |  |  |  |  |

SWMS has been developed using the hierarchy of controls to reduce the risk and likely hood of an incident/injury occurring. The use of Elimination, Substitution, Engineering/isolation, Administrative and PPE measures will be used in the above order of preference wherever practicable.

Induction confirmation:

This SWMS has been developed in consultation with the workers and has been read and signed by all workers involved in this activity. I note that I have read and understood the above safe work method statement and will ensure work is completed accordingly.

| Date: | Name: | Signature: |
| --- | --- | --- |
| Date: | Name: | Signature: |