| NOTE: Work must be performed in accordance with this SWMS. This SWMS must be kept and be available for inspection until the high-risk construction work to which this SWMS relates is completed. If the SWMS is revised, all versions should be kept. If a notifiable incident occurs in relation to the high-risk construction work in this SWMS, the SWMS must be kept for at least 2 years from the date of the notifiable incident. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| PCBU Name & Contact Details: Robertson’s Remedial and Painting Pty Ltd # Unit 10/56 Buffalo Road Gladesville NSW 2111 Phone: (02) 9181 3519 A.B.N 16 140 746 247 |  |  |  |  | Principal Contractor (PC): |  |
| Work Manager Contact Number: |  | Dave Oste - 0476 265 066 |  |  | Date SWMS provided to PC: | 06-02-2026 |
| Work Activity | Scaffolding erection and dismantling |  |  |  | Workplace Location: | 6a Francis Street, Bondi Beach |
| High Risk Construction Work: | Risk of person falling more than 2 metres (note: in some jurisdictions, this is 3metres) |  |  | Work on a telecommunication tower |  | Demolition of a load-bearing structure |
|  | Likely to involve disturbing asbestos |  |  | Temporary load-bearing support for structural alterations or repairs |  | Work in or near a confined space |
|  | Work in or near a shaft or trench deeper than 1.5m or a tunnel |  |  | Work in an area that may have a contaminated or flammable atmosphere. |  | Work on or near pressurised gas mains or piping |
|  | Work on or near chemical, fuel or refrigerant lines |  |  | Work on or near energised electrical installations |  | Use of explosives |
|  | Diving Work |  |  | Work on, in or adjacent to a road, railway, shipping lane or other traffic corridor in use by traffic other than pedestrians |  | Work in an area with movement of powered mobile plant. |
|  | Work in areas with artificial extremes of temperature |  |  | Work in or near water or other liquid that involves a risk of drowning |  | Tilt-up or precast concrete elements |
| Name of workers consulted in the development of this SWMS: |  |  |  |  | Signature: |  |
| Person responsible for ensuring compliance with SWMS: |  |  |  |  | Date SWMS received: |  |
| What measures are in place to ensure compliance with the SWMS? |  |  | Toolbox meetings, SWMS sign off, job observations and supervision review. If issues with the SWMS or new hazards are identified, the supervisor must be notified. When changes are made to SWMS, it will be communicated to all workers. |  |  |  |
| Person responsible for approving and reviewing SWMS control measures: |  |  |  |  | Date SWMS received by reviewer: |  |
| How will the SWMS control measures be reviewed? |  |  | The control measures implemented will be reviewed and if necessary, revised annually or if work methods change, the control measures are not effective in controlling the risk, a new hazard/risk is identified or following an incident. The SWMS will be reviewed in consultation with workers and/or others who may be affected by the SWMS. Any changes to the SWMS will be communicated with workers at induction, daily pre-starts and toolbox talks. |  |  |  |
| Review Date: |  |  |  |  | Reviewer’s signature: |  |

| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| --- | --- | --- | --- | --- | --- |
| Planning for the setup of job | Untrained or unlicensed workers may access site | Medium (3) | All workers must have Construction Induction Certificate. All workers to be licensed/trained as required for the works. All workers must complete site induction and sign in / out. Where induction is not available workers to liaise with site supervisor about site hazards, emergency procedures and any other site requirements. All workers to ensure they are familiar with the emergency muster points of the site. All workers to follow the onsite emergency procedures. All workers to be trained in and sign safe work method statement. Pre-start meeting to be conducted prior to commencement of works All workers to be in a fit state to commence work, be zero blood alcohol concentration, not suffering from fatigue or under the influence of drugs. Competent person to supervise all new workers. | Low (1) | Scaffolder/ Foreman |
|  | Inappropriate tool and equipment allocation | Medium (3) | Site Supervisor/Management to ensure that appropriate equipment and procedures are used for the works being completed. Site Supervisor /Management to ensure that all electrical devices have an electrical test tag prior to commencement of work. Management to ensure that adequate PPE is available to workers | Low (1) | Scaffolder/ Foreman |
| Accessing and securing the site | Slips, trips and falls | Medium (3) | All workers to wear steel capped boots. Onsite inductions to be completed prior to entering the job including identification of any slip trip and fall hazards. Ensure the work area is clear and free of slip, trip and fall hazards | Low (1) | Scaffolder/ Foreman |
|  | Weather conditions, UV exposure | Medium (3) | Workers must remain hydrated through regularly drinking water. Workers to stay out of sun where possible and implement rest breaks or job rotation where necessary. Workers must wear appropriate PPE including: sun cream, sun hats, and tinted safety glasses while working outside. | Low (1) | Scaffolder/ Foreman |
|  | Unauthorised individuals entering work site | Medium (3) | Appropriate barricades & signage to be erected to prevent unauthorised access to work area. Where appropriate, spotters to be used to observe personnel and re direct where needed. Alternate access to be provided where appropriate. | Low (1) | Scaffolder/ Foreman |
| Delivery and unload materials and position in work area | Manual handling injury | Medium (4) | Workers to follow correct manual handling procedures: Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit Use team lifts where needed Clear a path, assess the load, get a good grip, ensure a wide stance, bend the knees, keep the spine straight, use the legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (1) | Scaffolder/ Foreman |
|  | Inappropriate storage areas may interfere with public | Medium (3) | Materials must be transported in accordance with SDS. Secure storage areas to be allocated for products. Spill kits and firefighting equipment to be available on site to manage emergency situations. | Low (1) | Scaffolder/ Foreman |
| Inspection of scaffolding equipment prior to erection | Use of defective and damaged scaffolding equipment leading to equipment failure | Medium (4) | All scaffolding equipment must be inspected prior to use by a qualified/competent scaffolder. Any defective and damaged scaffolding must be removed, and the defective/damaged components should be identified by signs or tags to ensure workers and scaffolders are aware of the defects and damages. Workers or scaffolders should advise the supervisor of any defective or damaged scaffolding equipment. Defective and damaged scaffolding equipment will not be returned to site until it has been replaced or repaired by a qualified/competent scaffolder. Scaffolding equipment that can’t be repaired will be disposed of appropriately. | Low (2) | Scaffolder/ Foreman |
| Erect Scaffold base | Manual handling | Medium (4) | Workers to follow correct manual handling procedures: Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit Use team lifts where needed ## Clear a path, assess the load, get a good grip, ensure a wide stance, bend the knees, keep the spine straight, use the legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Scaffolder |
| Install sole boards | Risk of scaffold being out of level or collapsing after construction once loads are placed | High (6) | Place solid hardwood sole boards under every standard with a minimum dimension of 500 x 225 x 38mm Sole boards are to be placed centrally under every standard to distribute weight evenly Sole boards should have a level flat bearing with the ground Sole boards should not be levelled using any masonry product or scraps of timber | Low (2) | Scaffolder |
|  | Risk of subsidence of scaffold | Medium (4) | Larger sole boards should be used on soft or backfilled ground and spread under 2 standards where possible. Compaction by hand may be required at times prior to placement of sole boards | Low (2) | Scaffolder |
| Base out | Unsupported Standards can lead to standards falling | Medium (4) | The use of base collars allows base out to be completed by a single person without the risk of unsupported standards falling | Low (2) | Scaffolder |
|  | Scaffold distance from working face exceeds 225mm allowing potential for fall objects | High (6) | Scaffold to be erected in accordance with AS/NZ1576 and gaps not exceeding 225mm from working face | Low (2) | Scaffolder |
|  | Incorrect erection procedures and inadequate design may result in equipment failure. | Medium (4) | Only trained and competent workers to be involved in erection. Supervision to be provided to all new workers. Materials to be checked prior to erection for damage and faulty parts replaced. Workers to follow correct erection procedures as per AS/NZS 4576: 1995 Scaffolding Ensure Scaffold is tied to building at appropriate intervals | Low (2) | Scaffolder |
| Placement of standards, ledgers and transoms | Erection of lower components and failing to engage locking wedges can result in unsteady scaffold | Medium (4) | Progressively construct scaffold and engage all locking wedges in a systematic manner prior to accessing or climbing scaffold - Standards (external to working face) - Ledgers (0.5m and 1m) or brick guards - Transoms (0.5m and 1m stop ends) - Standards (adjacent to working face) - Transoms (2m above platform level) - Ledgers (2m above platform level) | Low (2) | Scaffolder |
| Placement of diagonal bracing | Inadequate bracing can result in scaffold instability | Medium (4) | Scaffold to be braced in accordance with manufacturers specifications (refer Layher instruction book) and conform to AS/NZ 1576 Should there be any doubt as to whether a brace is required, then it is required | Low (2) | Scaffolder |
| Placement of decks and toe boards | Incorrect or faulty planks can lead to an unsafe working platform | Medium (4) | Inspection of all decks for correct size and quality should be done prior to placement | Low (2) | Scaffolder |
|  | Decks which incorporate ladders have a risk of being incorrectly engaged resulting in ladder swinging down causing injury | High (6) | Ensure ladder is engaged correctly to the underside of the access deck prior to placement above Ladders which do not engage correctly should be quarantined and returned for repair | Low (2) | Scaffolder |
|  | Toe boards incorrectly fitted can lead to objects falling from working platform | Medium (4) | Install toe boards in accordance with design specifications on all working platforms | Low (2) | Scaffolder |
| Installation of access and egress | Manual handling | Medium (4) | Correct manual handling techniques shall be exercised to avoid injuries involving lifting, pulling and pushing loads are as follows. Team lifting techniques may be required | Low (2) | Scaffolder |
| Scaffold erecting Accessing platform | Falls from faulty or incorrectly installed access ladder can result in personal injury | Medium (4) | Ladder is to be industrial grade rated to 120kg Ensure ladder is secured at top and feet are bedded correctly prior to ascent Ladder to conform with AS/NZS 1892 (portable Ladders) | Low (2) | Scaffolder |
|  | Components falling from working platform during erection hitting persons below | Medium (4) | Establish an exclusion zone below platform Ensure warning signs are clearly visible | Low (2) | Scaffolder |
|  | Incomplete decks can result in falling through scaffold | Medium (4) | First platform is to be completely decked out from below prior to accessing first level | Low (2) | Scaffolder |
| Tying in Scaffold | Scaffold unstable due to inadequate tying in to frame or dwelling | Medium (4) | Scaffold to be tied in in accordance with manufacturers specifications in accordance with AS/NZ 1576.3 | Low (2) | Scaffolder |
|  | Inappropriate or insufficient scaffold ties | Medium (4) | Ensure adequate substrate for scaffold ties Install scaffold ties every 3rd standard and every 4metres in height. Installation of scaffold ties should not obstruct clear access along the full length of the working and access platforms | Low (2) | Scaffolder |
|  | Inadequate tying of adjacent scaffolds to each other may result in scaffold becoming unstable once loads are applied | High (6) | Adjacent scaffolds should be tied to each other via tube and couplers where required Additionally, these ties will be placed between 400-600mm and 900-1100mm to act as hand and mid-rails at scaffold junctions prior to advancing to next level | Low (2) | Scaffolder |
| Install Hoist Assembly | Inappropriate location for waste removal Inappropriate or insufficient ties to scaffold. | High (6) | Plan the optimum placement of the hoist for the safe removal of materials from the scaffold. Ensure that the total load of the hoist and materials is taken is accounted for. Install a hoist that has been maintained and in good condition. Ensure the hoist is secured to the scaffold. Ensure that the hoist assembly is inspected by a competent person before operation. |  | Scaffolder |
| Install Signage | Incorrect signage can lead to other users being unaware of the condition | Medium (4) | Remove all signage used during the construction of the scaffold once completed. Attach completed Scaffold Identification Tag in a prominent position (at base of access bay). Ensure erection and inspection records are completed | Low (2) | Scaffolder |
|  | Scaffold users may not be familiar with the Scaffolds load rating | Medium (4) | Ensure that ‘warning to scaffold users’ information is legible on the Scaffold Identification Tag and all information is filled out on form | Low (2) | Scaffolder/ Foreman |
| Final Inspection hand over | Failure to inspect scaffold - unstable and the possibility of causing injury | Medium (4) | Final checks that all fittings are tightened, wedges are secured and working platform complete prior to advancing to the next level Complete Handover Certificate and forward to the site supervisor and/or principal contractor’s office | Low (2) | Scaffolder |
| Operate Hoist | Falling objects | High (6) | Hoist operation must be communicated with other workers within the vicinity before use. Exclusion zones must be setup to keep people out of the drop zone. Hard hats must be worn at all times. All tools and equipment to be secure at all times. Workers to use harness where work outside the scaffold deck is necessary or where a scaffold deck cannot be placed at working levels. Only trained and competent workers to use the hoist. | Low (2) | Scaffolder/ Foreman |
| Dismantle scaffolds | Risk of scaffolder’s or objects falling | High (6) | All tools and equipment to be secure at all times. Secure off work area below and above Workers to use harness where work outside the scaffold deck is necessary or where a scaffold deck cannot be placed at working levels. Hard hats must be worn at all times. Only trained and competent workers to be used. | Low (2) | Scaffolder/ Foreman |
|  | Manual Handling hazard | Medium (4) | Workers to follow correct manual handling procedures: Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit Use team lifts where needed Clear a path, assess the load, get a good grip, ensure a wide stance, bend the knees, keep the spine straight, use the legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Scaffolder/ Foreman |
| Clean-up site Pack up materials and tools | Housekeeping Waste contamination | Medium (3) | Centralize and store gear where indicated Ensure site is secure from public access. Remove all waste materials from site and dispose of appropriately. Remove all tools and product or store in secure location. | Low (1) | Scaffolder/ Foreman |

| Personal Protective Equipment (PPE) required: | Enclosed leather steel capped boots Safety Glasses (As per AS/NZS 1336) Protective Gloves | Sun hats, high visibility clothing Hard Hats |
| --- | --- | --- |
| List the training required by workers to complete the work: List the qualifications of personnel doing the work: | Construction Induction Card Relevant trade certificate Induction into SWMS Licence to assemble and use scaffold: Basic, Intermediate, Advanced Scaffold High Risk Work Licence. |  |
| List the Permits, Certificates, SafeWork NSW Approvals, required to complete the work: |  |  |
| Relevant legislation: For further details refer to the Legal Requirements Register | Work in accordance with Work Health and Safety Act 2011 and Work Health and Safety Regulation 2025, workplace injury management and workers compensation act 1998 no. 86, AS 1269.3 Occupational Noise management, AS/NZS1337.1 Personal eye protection-Eye and face protectors for occupational applications, AS/NZS 1269.3 Occupational Noise management-Hearing protection program, AS/NZS 1801 industrial safety helmets, AS/NZS 2210.1 Safety, protective and occupational footwear-Guide to selection, care and use & AS/NZS 2210.2 to 2210.9 Occupational protective footwear & AS/NZS 4576 Guidelines for scaffolding. SafeWork NSW Codes of Practice: - Construction Work - Managing risks of plant - Managing the risk of falls - Work Health and Safety Consultation, Co-operation and Co-ordination - Managing the Work Environment and Facilities - First Aid in the Workplace - Hazardous Manual Tasks - How to Manage Work Health and Safety Risks |  |
| List the plant / equipment that will be used on site: List maintenance checks for plant & equipment: | Scaffolding Pre-start inspection and regular servicing as per manufacturer’s instructions. |  |
| Frequency of site/workplace inspections: Site Inspections will be completed by: | Suspended, Cantilevered, Spur and Hung Scaffolds and Scaffold from which a person can fall more than 4 meters inspected every 30 days. Scaffolds with less than 4 meters fall risk are inspected before use and after incident, repair, alteration or addition. |  |
| Hazardous Substances: | Glues |  |

# Sign Off and Approvals

| Person responsible for site supervision of the work, inspecting and approving work areas, approval of and compliance with SWMS, protective measures, plant, equipment: |  |
| --- | --- |
| Name and position: | Signature: |
| SWMS Prepared by: Ben Knox – Compliance OH&S | Signature: |

| Risk Level | Description of consequence or impact | Consequence | Likelihood/Probability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Unlikely (1) | Possible (2) | Almost Certain (3) |
| High Level of harm | Actual/Potential fatality, disability or irreversible damage. Major structural failure/damage. Off-site environmental discharge/release not contained and significant long-term environmental harm. | Major (3) | Medium (3) | High (6) | High (9) |
| Medium Level of harm | Actual/Potential temporary disability, MTI or LTI. Structural failure/damage, >1-day outage. On-site environmental discharge/release contained, minor remediation, short-term environmental harm. | Moderate (2) | Low (2) | Medium (4) | High (6) |
| Low Level of harm | Incident that has the potential to cause persons to require first aid. Environmental discharge/release immediately contained, minor level clean-up with no short term environmental harm. | Minor (1) | Low (1) | Low (2) | Medium (3) |
| Level | Likelihood/Probability |  |  |  |  |
| Almost Certain | Occurs frequently; >66% chance of occurring |  |  |  |  |
| Possible | Could happen occasionally; >33% but <66% chance of occurring |  |  |  |  |
| Unlikely | May occur only in exceptional circumstances; <33% chance of occurring |  |  |  |  |
| Class/Ranking | Description/Requirements |  |  |  |  |
| High 6, 9 | Stop immediately. Implement permanent preventative controls. |  |  |  |  |
| Medium 3, 4 | Will require operational planning. Actions will be recorded on a SWMS. |  |  |  |  |
| Low 1, 2 | Will require detailed pre-planning. Actions will be recorded on a SWMS. |  |  |  |  |

SWMS has been developed using the hierarchy of controls to reduce the risk and likelyhood of an incident/injury occurring. The use of Elimination, Substitution, Engineering/isolation, Administrative and PPE measures will be used in the above order of preference wherever practicable.

Induction confirmation:

This SWMS has been developed in consultation with the workers and has been read and signed by all workers involved in this activity. I note that I have read and understood the above safe work method statement and will ensure work is completed accordingly.

| Date: | Name: | Signature: |
| --- | --- | --- |
| Date: | Name: | Signature: |