| NOTE: Work must be performed in accordance with this SWMS. This SWMS must be kept and be available for inspection until the high-risk construction work to which this SWMS relates is completed. If the SWMS is revised, all versions should be kept. If a notifiable incident occurs in relation to the high-risk construction work in this SWMS, the SWMS must be kept for at least 2 years from the date of the notifiable incident. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| PCBU Name & Contact Details: Robertson’s Remedial and Painting Pty Ltd # Unit 10/56 Buffalo Road Gladesville NSW 2111 Phone: (02) 9181 3519 A.B.N 16 140 746 247 |  |  |  |  | Principal Contractor (PC): | Robertson’s Remedial and Painting Pty Ltd |
| Work Manager Contact Number: |  | Dave Oste - 0476 265 066 |  |  | Date SWMS provided to PC: | 06-02-2026 |
| Work Activity: | Demolition Works |  |  |  | Workplace Location: | 6a Francis Street, Bondi Beach |
| High Risk Construction Work: | Risk of person falling more than 2 metres (note: in some jurisdictions, this is 3metres) |  |  | Work on a telecommunication tower |  | Demolition of a load-bearing structure |
|  | Likely to involve disturbing asbestos |  |  | Temporary load-bearing support for structural alterations or repairs |  | Work in or near a confined space |
|  | Work in or near a shaft or trench deeper than 1.5m or a tunnel |  |  | Work in an area that may have a contaminated or flammable atmosphere. |  | Work on or near pressurised gas mains or piping |
|  | Work on or near chemical, fuel or refrigerant lines |  |  | Work on or near energised electrical installations |  | Use of explosives |
|  | Diving Work |  |  | Work on, in or adjacent to a road, railway, shipping lane or other traffic corridor in use by traffic other than pedestrians |  | Work in an area with movement of powered mobile plant. |
|  | Work in areas with artificial extremes of temperature |  |  | Work in or near water or other liquid that involves a risk of drowning |  | Tilt-up or precast concrete elements |
| Name of workers consulted in the development of this SWMS: |  |  |  |  | Signature: |  |
| Person responsible for ensuring compliance with SWMS: |  |  |  |  | Date SWMS received: |  |
| What measures are in place to ensure compliance with the SWMS? |  |  | Toolbox meetings, SWMS sign off, job observations and supervision review. If issues with the SWMS or new hazards are identified, the supervisor must be notified. When changes are made to SWMS, it will be communicated to all workers. |  |  |  |
| Person responsible for approving and reviewing SWMS control measures: |  |  |  |  | Date SWMS received by reviewer: |  |
| How will the SWMS control measures be reviewed? |  |  | The control measures implemented will be reviewed and if necessary, revised annually or if work methods change, the control measures are not effective in controlling the risk, a new hazard/risk is identified or following an incident. The SWMS will be reviewed in consultation with workers and/or others who may be affected by the SWMS. Any changes to the SWMS will be communicated with workers at induction, daily pre-starts and toolbox talks. |  |  |  |
| Review Date: |  |  |  |  | Reviewer’s signature: |  |

| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| --- | --- | --- | --- | --- | --- |
| Planning for the setup of job | Poor Planning of Job Activity | Medium (4) | Notify SafeWork NSW 5 days prior to undertaking demolition works involving: - The Demolition of a Structure More Than Six Metres High - Load-Shifting Machinery (e.g. An Excavator) On A Suspended Floor Check Asbestos register before beginning demolition. If not available, competent person must inspect structure for asbestos. Remove all asbestos before demolition work. Nominate competent & experienced supervisor for management of work activity. | Low (2) | Management |
|  | Untrained or unlicensed workers may access site | Medium (3) | All workers must have completed Construction Induction Training. All workers qualified/ licensed/ ticketed/ trained as required for job. All workers must complete site induction and sign in / out. All workers to be trained in and sign SWMS. Pre-start meeting to be conducted prior to commencement of works. All workers to be in a fit state to commence work, not fatigued or under the influence of drugs or alcohol. Competent person to supervise all new workers and activity. All workers to ensure they are familiar with the emergency muster points of the site and the emergency procedures. | Low (1) | Workers & Supervisor |
|  | Inappropriate tool and equipment allocation | Medium (3) | Supervisor/Management to ensure that appropriate equipment and procedures are used for the works being completed. Supervisor/Management to ensure that all electrical devices have an electrical test tag prior to commencement of work. Supervisor/Management to ensure all plant, equipment and tools are inspected for any damage or faults. Management to ensure that adequate PPE is available to workers. | Low (1) | Workers & Supervisor |
|  | Injury to personnel due to individuals entering work site | Medium (3) | Appropriate barricades & signage to be erected to prevent unauthorised access to work area. Where appropriate, spotters to be used to observe personnel and re-direct where needed. Alternate access to be provided. | Low (1) | Workers & Supervisor |
| Accessing and securing the site | Slips, trips and falls | Medium (3) | All workers to wear steel capped boots. Identify any slip trip and fall hazards and ensure the work area is clear and free of these hazards and obstructions. | Low (1) | Workers |
| Accessing and securing the site | Weather conditions, UV Exposure, Windy or Wet Conditions | Medium (3) | Workers must remain hydrated regularly drinking water. Workers to stay out of sun where possible and implement rest breaks or job rotation where necessary. Consider not working in very hit conditions. Do not work in wet weather or high wind conditions. Workers must wear appropriate PPE including: sun cream, sun hats, and tinted safety glasses while working outside. | Low (1) | Workers |
| Delivery and unload plant / equipment / tools etc. | Manual handling | Medium (4) | Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit. Use team lifts where needed. Workers to follow correct manual handling procedures: Clear a path, assess the load, get a good grip, ensure a wide stance, bend the knees, keep the spine straight, use the legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Workers |
|  | Workers and others struck by powered mobile plant including delivery vehicle. | High (6) | Prepare and implement traffic management plan on site. Implement Exclusion zones around mobile plant. Clearly identify them with signage and barricading. Implement spotter to control vehicle loading / unloading operations. | Low (2) | Workers |
|  | Workers and others being struck by vehicles in adjacent road or traffic. | High (6) | Trained and competent Traffic Controllers to direct traffic entering / exiting site and control traffic on adjacent pedestrian footpaths and roadways. Use portable traffic signals, boom gate, safety barriers, cones etc. Unload plant on-site, not on public roadway. | Low (2) | Workers |
| Demolishing building / structure | Contact with essential services | High (9) | Obtain current information on services, DBYD plans, prior to commencing work and have them available at all times. Shut off / Isolate / De-energise and cap all service lines, that are not required during the demolition process, prior to the works commencing. Discuss with service owners. Ensure all services have been located prior to commencement and have been isolated. Do not work near live services. Protect all other services retained for demolition work. Exclusion zones and safe approach distances to be established if overhead services present on site. Ensure overhead services are clearly marked and identifiable. | Medium (3) | Workers |
| Demolishing building / structure | Workers and others struck by powered mobile plant | High (6) | Prepare and implement traffic management plan for site and communicate to all workers on site. Establish exclusion zones, clearly demarcate these areas. Install overhead protective structure where work is adjacent to public. Implement spotters to guide and communicate between the operator and workers on foot. | Low (2) | Workers |
|  | Possible lead or asbestos contamination to workers or environment | Medium (4) | Prior to commencement of works, competent person to inspect the site for the presence of lead paint or asbestos. P2 respirators to be worn during demolition. If found unexpectedly, stop work immediately. Contact supervisor and owner of property. Engage a licenced asbestos removalist to safely remove asbestos. | Low (2) | Workers |
|  | Dust and flying Debris | Medium (3) | - Water Spay to be used to prevent the spread of Dusts. All PPE to be worn at all times including Safety Glasses, dust mask, and hard hats. Ensure others do not enter the work site while demolition is in progress. Implement exclusion zones with safe distances. | Low (1) | Workers |
| Remove/demolish materials as per scope | Manual handling | Medium (4) | Use mechanical lifting devices and trolleys where possible to remove demolition waste / debris. Only lift objects within your safe lifting limit. Use team lifts where needed. Workers to follow correct manual handling procedures: Clear path, assess load, get a good grip, ensure a wide stance, bend knees, keep back straight, use the legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Workers |
| Remove/demolish materials as per scope | Fall from heights | High (6) | Do not ever work from the top of a structure being demolished. Use fully decked heave duty frame trestle scaffold with bay lengths of 1.8m or less or Modular Scaffold with brick guards installed to prevent objects falling down. Ensure the scaffold is secure and has been inspected recently. Use SWI for the selected control measure. Scaffold to have safe access/egress to platform with stairs or ladder towers. Ensure ladders are secured top and bottom before accessing. Perform visual check on ladder for faults and defects prior to use. Damaged ladders are to be removed from use, tag “not for use” and repair or replace ASAP. Ensure legs are fully extended and latch is in place. Do not over-reach while on ladder. Only one person on a ladder at any one time. Face the ladder at all times. Maintain 3 points of contact. Do not step above the third step from the top. Secure all tools and materials whilst on ladders, use a tool pouch to carry tools. Do not leave tools and equipment on ladder rungs. Erect ladder on clean, level surface or use self-levelling ladder. Clean mud, grass etc. from boots to prevent slippage. Where working from more than a couple of stories high a double lanyard fall restraint system is to be used. | Low (2) | Workers |
| Demolishing building / structure | Workers injured by structural collapse | High (9) | Create Exclusion zone to prevent access to with the vicinity of potential structural collapse. Temporary braces, propping or shoring added to ensure the stability of the structure is maintained to prevent unexpected collapse of structure. Do not permit walls to stand unless effectively supported against collapse. Check if walls to be demolished are supporting other walls. Demolish structure systematically starting from the top. Prepare demolition procedure and communicate clearly to workers. Where demolition work is undertaken on suspended concrete slab: • Prepare and implement a sequential demolition plan approved by a competent structural engineer. Specifically, the plan should consider the rate of demolition and outline the load capacity limits of floors, ramps or other suspended slabs. If temporary supports are required, their specification and positioning should clearly be shown in plan. • Consider the method for moving plant from floor to floor. If using ramps, the loads on the structure should be verified. In particular, the ramp’s specifications, location and the placement of temporary supports should be documented in the demolition procedure. • Consider the means for clearing rubble from floors or other suspended slabs to prevent a build-up of rubble. • The installation of temporary supports must be according to manufacturer’s instructions and the installation of their positioning must be according to: - specifications in the demolition procedure, - instructions on when to remove rubble to prevent slabs being overloaded, - instructions on where a ramp is to be used and whether it needs temporary supports, - the prescribed distances between machines when they’re located on slabs. PPE must be worn: Hard hat, steel capped boots, dust mask / respirator, long sleeve pants and shirt. | Medium (3) | Workers |
| Demolishing building / structure | Debris striking adjacent traffic or public | Medium (3) | Install overhead protective structure over public walkway lying between public place and building being demolished: - horizontal platform of solid construction with vertical supports, - at least 250mm from edge of kerb of walkway - min overhead clearance of 2.2m to any bracings, beams or other part of overhead protective structure, and - is designed for a uniformly distributed live load of 10kPa. Prepare and implement traffic management plan for site and communicate to all workers on site. | Low (1) | Workers |
| Remove waste from site & Pack-up | Inappropriate dumping of materials | Medium (3) | Ensure a designated dumping area has been communicated to all workers on site and barricaded off. Lead or Asbestos contaminated waste to be dumped in appropriate facility – asbestos removalist will manage this. The waste from demolition must be disposed of only at a landfill site that may lawfully receive the waste. The waste must be covered during transport. Do not over load vehicle and no objects to hang out of side or top. | Low (1) | Workers |
|  | Manual handling | Medium (3) | Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit. Use team lifts where needed. Workers to follow correct manual handling procedures: Clear a path, assess the load, get a good grip, ensure a wide stance, bend the knees, keep the spine straight, use the legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (1) | Workers |
|  | Housekeeping | Medium (3) | All tools and equipment to be cleaned and stored away. Ensure work site is left in clean and tidy condition with no obstructions or protruding materials. | Low (1) | Workers |

| Personal Protective Equipment (PPE) required: | Enclosed leather steel capped boots Safety Glasses (As per AS/NZS 1337) Hearing Protection Protective Gloves (As per AS/NZS 2161) | High visibility Long sleeve shirt & pants Hard Hats (As per AS/NZS 1801) Sun hats, sun cream - when working outdoors Dust Mask / P2 Respirator |
| --- | --- | --- |
| List the training required by workers to complete the work: List the qualifications of personnel doing the work: | Construction Induction Card Induction into SWMS Supervisors Trained in: Certificate III in Demolition (CPC30413) and Certificate IV in Demolition (CPC41013) PLUS 3 years demonstrated experience, relevant to type of demolition licence. Demolition Licence or Restricted Licence (see SafeWork NSW website for which applies). |  |
| List the Permits, Certificates, SafeWork NSW Approvals, required to completed the work: | SafeWork NSW Approval 5 days prior to demolition undertaking. Demolition Permit. |  |
| Relevant legislation: For further details refer to the Legal Requirements Register | Work in accordance with Work Health and Safety Act 2011 and Work Health and Safety Regulation 2025, workplace injury management and workers compensation act 1998 no. 86, AS/NZS1337.1 Personal eye protection-Eye and face protectors for occupational applications, AS/NZS 1269.3 Occupational Noise management-Hearing protection program, AS/NZS 1801 industrial safety helmets, AS/NZS 2210.1 Safety, protective and occupational footwear-Guide to selection, care and use & AS/NZS 2210.2 to 2210.9 Occupational protective footwear. SafeWork NSW Codes of Practice: - Construction Work - Managing risks of plant - Managing the risk of falls - Work Health and Safety Consultation, Co-operation and Co-ordination - Managing the Work Environment and Facilities - First Aid in the Workplace - Hazardous Manual Tasks - How to Manage Work Health and Safety Risks |  |
| List the plant / equipment that will be used on site: |  |  |
| List maintenance checks for plant & equipment: | Daily pre-start checks, maintenance as per manufacturers guidelines |  |
| Frequency of site/workplace inspections: Site Inspections will be completed by: | Daily site inspections & job completion inspections to be completed by Site Supervisor. |  |
| Hazardous Substances: | Possible Asbestos and Lead Exposure |  |

# Sign Off and Approvals

| Person responsible for site supervision of the work, inspecting and approving work areas, approval of and compliance with SWMS, protective measures, plant, equipment: |  |
| --- | --- |
| Name and position: | Signature: |
| SWMS Prepared by: Ben Knox – Compliance OH&S | Signature: |

| Risk Level | Description of consequence or impact | Consequence | Likelihood/Probability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Unlikely (1) | Possible (2) | Almost Certain (3) |
| High Level of harm | Actual/Potential fatality, disability or irreversible damage. Major structural failure/damage. Off-site environmental discharge/release not contained and significant long-term environmental harm. | Major (3) | Medium (3) | High (6) | High (9) |
| Medium Level of harm | Actual/Potential temporary disability, MTI or LTI. Structural failure/damage, >1-day outage. On-site environmental discharge/release contained, minor remediation, short-term environmental harm. | Moderate (2) | Low (2) | Medium (4) | High (6) |
| Low Level of harm | Incident that has the potential to cause persons to require first aid. Environmental discharge/release immediately contained, minor level clean-up with no short term environmental harm. | Minor (1) | Low (1) | Low (2) | Medium (3) |
| Level | Likelihood/Probability |  |  |  |  |
| Almost Certain | Occurs frequently; >66% chance of occurring |  |  |  |  |
| Possible | Could happen occasionally; >33% but <66% chance of occurring |  |  |  |  |
| Unlikely | May occur only in exceptional circumstances; <33% chance of occurring |  |  |  |  |
| Class/Ranking | Description/Requirements |  |  |  |  |
| High 6, 9 | Stop immediately. Implement permanent preventative controls. |  |  |  |  |
| Medium 3, 4 | Will require operational planning. Actions will be recorded on a SWMS. |  |  |  |  |
| Low 1, 2 | Will require detailed pre-planning. Actions will be recorded on a SWMS. |  |  |  |  |

SWMS has been developed using the hierarchy of controls to reduce the risk and likelyhood of an incident/injury occurring. The use of Elimination, Substitution, Engineering/isolation, Administrative and PPE measures will be used in the above order of preference wherever practicable.

Induction confirmation:

This SWMS has been developed in consultation with the workers and has been read and signed by all workers involved in this activity. I note that I have read and understood the above safe work method statement and will ensure work is completed accordingly.

| Date: | Name: | Signature: |
| --- | --- | --- |
| Date: | Name: | Signature: |