| NOTE: Work must be performed in accordance with this SWMS. This SWMS must be kept and be available for inspection until the high-risk construction work to which this SWMS relates is completed. If the SWMS is revised, all versions should be kept. If a notifiable incident occurs in relation to the high-risk construction work in this SWMS, the SWMS must be kept for at least 2 years from the date of the notifiable incident. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| PCBU Name & Contact Details: Robertson’s Remedial and Painting Pty Ltd # Unit 10/56 Buffalo Road Gladesville NSW 2111 Phone: (02) 9181 3519 A.B.N 16 140 746 247 |  |  |  |  | Principal Contractor (PC): |  |
| Work Manager Contact Number: |  | Dave Oste - 0476 265 066 |  |  | Date SWMS provided to PC: | 06-02-2026 |
| Work Activity: | Plumbing Water, Gas, Sewer and Stormwater (In Ground) |  |  |  | Workplace Location: | 6a Francis Street, Bondi Beach |
| High Risk Construction Work: | Risk of person falling more than 2 metres (note: in some jurisdictions, this is 3metres) |  |  | Work on a telecommunication tower |  | Demolition of a load-bearing structure |
|  | Likely to involve disturbing asbestos |  |  | Temporary load-bearing support for structural alterations or repairs |  | Work in or near a confined space |
|  | Work in or near a shaft or trench deeper than 1.5m or a tunnel |  |  | Work in an area that may have a contaminated or flammable atmosphere. |  | Work on or near pressurised gas mains or piping |
|  | Work on or near chemical, fuel or refrigerant lines |  |  | Work on or near energised electrical installations |  | Use of explosives |
|  | Diving Work |  |  | Work on, in or adjacent to a road, railway, shipping lane or other traffic corridor in use by traffic other than pedestrians |  | Work in an area with movement of powered mobile plant. |
|  | Work in areas with artificial extremes of temperature |  |  | Work in or near water or other liquid that involves a risk of drowning |  | Tilt-up or precast concrete elements |
| Name of workers consulted in the development of this SWMS: |  |  |  |  | Signature: |  |
| Person responsible for ensuring compliance with SWMS: |  |  |  |  | Date SWMS received: |  |
| What measures are in place to ensure compliance with the SWMS? |  |  | Toolbox meetings, SWMS sign off, job observations and supervision review. If issues with the SWMS or new hazards are identified, the supervisor must be notified. When changes are made to SWMS, it will be communicated to all workers. |  |  |  |
| Person responsible for approving and reviewing SWMS control measures: |  |  |  |  | Date SWMS received by reviewer: |  |
| How will the SWMS control measures be reviewed? |  |  | The control measures implemented will be reviewed and if necessary, revised annually or if work methods change, the control measures are not effective in controlling the risk, a new hazard/risk is identified or following an incident. The SWMS will be reviewed in consultation with workers and/or others who may be affected by the SWMS. Any changes to the SWMS will be communicated with workers at induction, daily pre-starts and toolbox talks. |  |  |  |
| Review Date: |  |  |  |  | Reviewer’s signature: |  |

| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| --- | --- | --- | --- | --- | --- |
| Planning for the setup of job. Check all Workers tickets/licences, check site, complete induction | Untrained or unlicensed workers may access site causing serious injury to self or other individuals | Medium (3) | All workers must have Construction Induction Certificate. All workers to be qualified/ licensed/ ticketed/ trained as required for the works. All workers must complete site induction and sign in / out. Where induction is not available workers to liaise with site supervisor about site hazards, emergency procedures and any other site requirements. All workers to be trained in and sign safe work method statement. Toolbox talk to be conducted prior to commencement of works All workers to be in a fit state to commence work, be zero blood alcohol concentration, not suffering from fatigue or under the influence of drugs. Competent person to supervise all new workers. | Low (1) | Workers & Supervisor |
|  | Inappropriate tool and equipment allocation may lead to injury | Medium (3) | Site Supervisor/Management to ensure that appropriate equipment and procedures are used for the works being completed. Site Supervisor /Management to ensure that all electrical devices have an electrical test tag prior to commencement of work. Management to ensure that adequate personal protective equipment is available to workers | Low (1) | Workers & Supervisor |
| Accessing and securing the site | Slips, trips and falls | Medium (3) | All workers to wear steel capped boots. Onsite inductions to be completed prior to entering the job including identification of any slip trip and fall hazards. Ensure the work area is clear and free of slip, trip and fall hazards | Low (1) | Workers |
|  | Weather conditions I.e. heat stroke, dehydration, skin cancer, sun burn etc | Medium (3) | Workers must remain hydrated through regularly drinking water. Workers to stay out of sun where possible and implement rest breaks or job rotation where necessary. Workers must wear appropriate PPE including: sun cream, sun hats, and tinted safety glasses while working outside. | Low (1) | Workers |
|  | Environmental contamination i.e. erosion and sediment | Medium (3) | Where possible works to be timed to avoid rain Sandbags to be used to slow runoff and contain sediment Workers to follow the project environmental requirements (where required) | Low (1) | Workers & Supervisor |
| Accessing and securing the site | Emergency situation, Workers not familiar with emergency procedures | Medium (3) | All workers to ensure they are familiar with the emergency muster points of the site. All workers to follow the onsite emergency procedures as outlined at induction. | Low (1) | Workers |
|  | Injury to personnel due to individuals entering work site | Medium (3) | Appropriate barricades to be erected to prevent unauthorised access to work area. Appropriate signage to be erected indicating work site unauthorised access not permitted. Where appropriate, spotters to be used to observe personnel and re direct where needed. Alternate access to be provided where appropriate. | Low (1) | Workers & Supervisor |
| Delivery and unload materials and position in work area | Manual handling injury | Medium (4) | Workers to follow correct manual handling procedures: Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit Use team lifts where needed Clear a path, assess the load, get a good grip, ensure a wide stance, bend the knees, keep the spine straight, use the legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Workers |
|  | Inappropriate storage areas may interfere with public | Medium (3) | Workers to discuss material storage area and work areas with site Supervisor prior to commencement. Appropriate work and storage areas to be barricaded off. | Low (1) | Workers |
| Operation of bobcat/excavator to excavate trench | Dust generation Noise | Medium (4) | Complete pre-start checks on machine Ensure service log is up to date and kept on site. Ensure personnel are not working in the vicinity of machine & truck delivery - Ensure temporary fencing is in place to isolate work area from unessential personnel. Ensure hearing protection, hard hats, high visibility shirts, steel capped leather boots, sun protection and eye protection and dust masks are worn. Inspect land slope for safe operation of Excavators and Backhoes as per operating and codes of practice (moving mobile plant on construction sites). | Low (2) | Workers |
| Operation of bobcat/excavator to excavate trench | Contact with personnel in immediate area and other plant and equipment | High (6) | Complete pre-start checks of machine as per manufacturer’s instructions Ensure maximum lift weight is not exceeded. Ensure all personnel are well clear of the Excavators and Backhoes. Ensure all guards are fitted as per the operation manual Ensure Excavators & Backhoes does not exceed the operating perimeters as per operations manual. Never leave the machine unattended and never allow persons to stand under the equipment. Ensure safety pins are in where required. | Low (2) | Workers |
|  | Electrocution or exposure to underground services or cables | High (6) | Identify all services with DBYD, surveys and clearly mark out. Discuss with service owners to relocate, isolate / de-energise service for duration of works. Positively identify services underground with non-destructive techniques. Use non-conductive and insulated hand tools. Operator not to work near live services. Ensure machine is at least 3m clear of overhead wires. | Low (2) | Workers |
|  | Open trenches Trench collapse | High (6) | Implement exclusion zones and barricading around open excavation, when in use. Implement appropriate warning signage for open excavation. Cover trench with hard cover & secure to ground, when not in use. Ensure excavator and other plant do not enter ‘area of influence’ = 1 meter around open trench. No materials stored in area. | Low (2) |  |
| Tidy up excavations | Back strain from bending and shoveling | Medium (4) | Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit Use team lifts where needed Workers to follow correct manual handling procedures: Clear a path, assess the load, get a good grip, ensure a wide stance, bend the knees, keep the spine straight, use the legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. Use long handle shovel Leather gloves where necessary Check that tool head is securely fixed Check that other people are clear of area | Low (2) | Workers |
| Load truck | Contact with others | High (6) | Ensure all Workers wear appropriate high visibility clothing. Follow instructions for operating bob cat as above Take care to avoid others, Workers to be kept a safe distance No Workers to be in vicinity of bin while loading in place Do not over load bin or truck. | Low (2) | Workers |
| Isolate and remove existing services (pipes) | Electrocution or injury due to contact with services or cables | High (6) | Ensure all services have been located prior to commencement and have been isolated as per AS/NZS 3000. Ensure workers have completed the "test before they touch" on electrical installations. Isolate, lock out/tag out all services. Do not work near live services. Leather steel capped rubber soled boots to be worn. Fibreglass ladders to be used. | Low (2) | Workers |
|  | Manual handling | Medium (4) | Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit, Use team lifts. Workers to follow correct manual handling procedures: Clear a path, assess the load, get a good grip, ensure a wide stance, bend the knees, keep the spine straight, use the legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Workers |
|  | Inappropriate storage areas | Medium (3) | Workers to discuss material storage area and work areas with Manager prior to commencement. Appropriate work and storage areas to be fenced off and signposted. | Low (1) | Workers |
|  | Cuts and abrasions | Medium (3) | Workers to wear gloves, long sleeve pants and shirt, steel-capped safety boots, safety glasses at all times. Take care with sharp objects. | Low (1) | Workers |
|  | Asphyxiation or explosion from gas leak | High (6) | Gas to be isolated/locked out/tagged out by licensed plumber. Check all services for leaks prior to working. Clear lines of all gas prior to working. Maintain constant monitor of possible gas leaks | Low (2) | Workers |
| Remove sewage pipes | Possible hazardous contamination | Medium (4) | Avoid direct skin contact with substances. Wear impervious gloves. Any open wounds must be securely bandaged. In event of contamination, wash area thoroughly with water, then immerse in Hibitane concentrate or betadine and scrub with brush. | Low (2) | Workers |
| Install pipes Check materials for compliance to spec and standards Measure up for works. Cut materials to length with hack saw or pipe cutter | Unauthorised personnel may enter work | Medium (3) | Appropriate barriers to be erected to prevent public access to work area. Appropriate signage to be erected indicating work site unauthorised access not permitted. Spotter to be used where necessary. | Low (1) | Workers |
|  | Repetitive strain, vibration and muscular injury due to repetitive work | Medium (4) | Workers must warm up and stretch prior to commencing work. Workers to rotate tasks to avoid repetitive tasks and fixed postures. Workers to avoid excessive straining and over reaching Workers to rotate tasks and wear gloves when using grinder/chipper | Low (2) | Workers |
|  | Inappropriate materials | Medium (3) | Inspect all materials to ensure that they are the correct type, quantity, labelled correctly and have are not damaged. Check all materials purchased comply with spec and that they meet relevant Australian Standards. | Low (1) | Workers |
| Bend pipes where necessary | Cuts to fingers or body parts | Medium (3) | Keep hand and fingers clear of bending tools. The preferred technique is to use the pipe cutter instead of the saw. Use vice when cutting large pipe. Do not to touch sharp edges. Wear safety glasses, leather gloves and long sleeve shirts. All waste materials to be collected and placed into rubbish immediately not left in work area. | Low (1) | Workers |
| Connect pipes, bends and joints Check for leaks | Inappropriate use of equipment | Medium (3) | Follow SWI for Oxy. Always light with lint lighter or pilot light. Acetylene first then turn on oxy. Keep all fumes and gases from the breathing area and ensure adequate ventilation. Set flows to appropriate levels. | Low (1) | Workers |
|  | Exposure to hazardous substances | Medium (3) | SDS to be on site and all instructions followed by workers. As a minimum respirator and gloves to be worn | Low (1) | Workers |
| Back fill hole | Injury to operator | Medium (3) | Complete pre-start checks of machine as per manufacturer’s instructions Ensure maximum lift weight is not exceeded. Ensure all personnel are well clear of the Excavators and Backhoes. Ensure all guards are fitted as per the operation manual Ensure Excavators & Backhoes does not exceed the operating perimeters as per operations manual. Never leave the machine unattended and never allow persons to stand under the equipment | Low (1) | Workers |
| Clean up and remove all waste materials from site | Water leak | Medium (3) | Ensure that the system is free of all leaks. Repair any leaks immediately | Low (1) | Workers |
|  | Housekeeping | Medium (3) | Remove all waste materials from site. Follow manual handling principles above. Check for any hazards and remove where necessary | Low (1) | Workers |

| Personal Protective Equipment (PPE) required: | Enclosed leather steel capped boots Safety Glasses (As per AS/NZS 1337) Hearing Protection Protective Gloves (As per AS/NZS 2161) | High visibility Long sleeve shirt & pants Hard Hats (As per AS/NZS 1801) Sun hats, sun cream - when working outdoors |
| --- | --- | --- |
| List the training required by workers to complete the work: List the qualifications of personnel doing the work: | Construction Induction Card Relevant trade certificate Induction into SWMS |  |
| List the Permits, Certificates, SafeWork NSW Approvals, required to completed the work: | All necessary permits to be completed on site |  |
| Relevant legislation: For further details refer to the Legal Requirements Register | Work in accordance with Work Health and Safety Act 2011 and Work Health and Safety Regulation 2025, workplace injury management and workers compensation act 1998 no. 86, AS/NZS1337.1 Personal eye protection-Eye and face protectors for occupational applications, AS/NZS 1269.3 Occupational Noise management-Hearing protection program, AS/NZS 1801 industrial safety helmets, AS/NZS 2210.1 Safety, protective and occupational footwear-Guide to selection, care and use & AS/NZS 2210.2 to 2210.9 Occupational protective footwear. SafeWork NSW Codes of Practice: - Construction Work - Managing risks of plant - Managing the risk of falls - Work Health and Safety Consultation, Co-operation and Co-ordination - Managing the Work Environment and Facilities - First Aid in the Workplace - Hazardous Manual Tasks - How to Manage Work Health and Safety Risks |  |
| List the plant / equipment that will be used on site: | Excavator / Backhoe, Oxy |  |
| List maintenance checks for plant & equipment: | Daily pre-start checks, maintenance as per manufacturers guidelines |  |
| Frequency of site/workplace inspections: Site Inspections will be completed by: | Daily site inspections & job completion inspections to be completed by Site Supervisor. |  |
| Hazardous Substances: | Glues |  |

# Sign Off and Approvals

| Person responsible for site supervision of the work, inspecting and approving work areas, approval of and compliance with SWMS, protective measures, plant, equipment: |  |
| --- | --- |
| Name and position: | Signature: |
| SWMS Prepared by: Ben Knox – Compliance OH&S | Signature: |

| Risk Level | Description of consequence or impact | Consequence | Likelihood/Probability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Unlikely (1) | Possible (2) | Almost Certain (3) |
| High Level of harm | Actual/Potential fatality, disability or irreversible damage. Major structural failure/damage. Off-site environmental discharge/release not contained and significant long-term environmental harm. | Major (3) | Medium (3) | High (6) | High (9) |
| Medium Level of harm | Actual/Potential temporary disability, MTI or LTI. Structural failure/damage, >1-day outage. On-site environmental discharge/release contained, minor remediation, short-term environmental harm. | Moderate (2) | Low (2) | Medium (4) | High (6) |
| Low Level of harm | Incident that has the potential to cause persons to require first aid. Environmental discharge/release immediately contained, minor level clean-up with no short term environmental harm. | Minor (1) | Low (1) | Low (2) | Medium (3) |
| Level | Likelihood/Probability |  |  |  |  |
| Almost Certain | Occurs frequently; >66% chance of occurring |  |  |  |  |
| Possible | Could happen occasionally; >33% but <66% chance of occurring |  |  |  |  |
| Unlikely | May occur only in exceptional circumstances; <33% chance of occurring |  |  |  |  |
| Class/Ranking | Description/Requirements |  |  |  |  |
| High 6, 9 | Stop immediately. Implement permanent preventative controls. |  |  |  |  |
| Medium 3, 4 | Will require operational planning. Actions will be recorded on a SWMS. |  |  |  |  |
| Low 1, 2 | Will require detailed pre-planning. Actions will be recorded on a SWMS. |  |  |  |  |

SWMS has been developed using the hierarchy of controls to reduce the risk and likely hood of an incident/injury occurring. The use of Elimination, Substitution, Engineering/isolation, Administrative and PPE measures will be used in the above order of preference wherever practicable.

# Induction confirmation:

This SWMS has been developed in consultation with the workers and has been read and signed by all workers involved in this activity. I note that I have read and understood the above safe work method statement and will ensure work is completed accordingly.

| Date: | Name: | Signature: |
| --- | --- | --- |
| Date: | Name: | Signature: |