| NOTE: Work must be performed in accordance with this SWMS. This SWMS must be kept and be available for inspection until the high-risk construction work to which this SWMS relates is completed. If the SWMS is revised, all versions should be kept. If a notifiable incident occurs in relation to the high-risk construction work in this SWMS, the SWMS must be kept for at least 2 years from the date of the notifiable incident. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| PCBU Name & Contact Details: Robertson’s Remedial and Painting Pty Ltd # Unit 10/56 Buffalo Road Gladesville NSW 2111 Phone: (02) 9181 3519 A.B.N 16 140 746 247 |  |  |  |  | Principal Contractor (PC): | Robertson’s Remedial and Painting Pty Ltd |
| Work Manager Contact Number: |  | Dave Oste - 0476 265 066 |  |  | Date SWMS provided to PC: | 06-02-2026 |
| Work Activity | Rendering |  |  |  | Workplace Location: | 6a Francis Street, Bondi Beach |
| High Risk Construction Work: | Risk of person falling more than 2 metres |  |  | Work on a telecommunication tower |  | Demolition of a load-bearing structure |
|  | Likely to involve disturbing asbestos |  |  | Temporary load-bearing support for structural alterations or repairs |  | Work in or near a confined space |
|  | Work in or near a shaft or trench deeper than 1.5m or a tunnel |  |  | Work in an area that may have a contaminated or flammable atmosphere. |  | Work on or near pressurised gas mains or piping |
|  | Work on or near chemical, fuel or refrigerant lines |  |  | Work on or near energised electrical installations |  | Use of explosives |
|  | Diving Work |  |  | Work on, in or adjacent to a road, railway, shipping lane or other traffic corridor in use by traffic other than pedestrians |  | Work in an area with movement of powered mobile plant. |
|  | Work in areas with artificial extremes of temperature |  |  | Work in or near water or other liquid that involves a risk of drowning |  | Tilt-up or precast concrete elements |
| Name of workers consulted in the development of this SWMS: |  |  |  |  | Signature: |  |
| Person responsible for ensuring compliance with SWMS: |  |  |  |  | Date SWMS received: |  |
| What measures are in place to ensure compliance with the SWMS? |  |  | Toolbox meetings, SWMS sign off, job observations and supervision review. If issues with the SWMS or new hazards are identified, the supervisor must be notified. When changes are made to SWMS, it will be communicated to all workers. |  |  |  |
| Person responsible for approving and reviewing SWMS control measures: |  |  |  |  | Date SWMS received by reviewer: |  |
| How will the SWMS control measures be reviewed? |  |  | The control measures implemented will be reviewed and if necessary, revised annually or if work methods change, the control measures are not effective in controlling the risk, a new hazard/risk is identified or following an incident. The SWMS will be reviewed in consultation with workers and/or others who may be affected by the SWMS. Any changes to the SWMS will be communicated with workers at induction, daily pre-starts and toolbox talks. |  |  |  |
| Review Date: |  |  |  |  | Reviewer’s signature: |  |

| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| --- | --- | --- | --- | --- | --- |
| Planning for the setup of job. Check all Workers tickets/licences, check site, complete induction | Untrained or unlicensed workers may access site | Medium (3) | All workers must have Construction Induction Training. Workers qualified/ licensed/ ticketed/ trained as required for works. All workers must complete site induction and sign in / out. Where induction is not available workers to liaise with site supervisor about site hazards, emergency procedures and any other site requirements. All workers to ensure they are familiar with the emergency muster points of the site All workers to follow the onsite emergency procedures. All workers to be trained in and sign safe work method statement. Pre-start meeting to be conducted prior to commencement of works All worker must be in a fit state to commence work, not fatigued or under the influence of drugs or alcohol. Competent person to supervise all new workers. | Low (1) | Workers & Supervisor |
|  | Inappropriate tool and equipment allocation | Medium (3) | Site Supervisor/Management to ensure that appropriate equipment and procedures are used for the works being completed. Site Supervisor /Management to ensure that all electrical devices have an electrical test tag prior to commencement of work. Management to ensure that adequate personal protective equipment is available to workers | Low (1) | Workers & Supervisor |
| Accessing and securing the site | Slips, trips and falls | Medium (3) | All workers to wear steel capped boots. Onsite inductions to be completed prior to entering the job including identification of any slip trip and fall hazards. Ensure the work area is clear and free of slip, trip and fall hazards | Low (1) | Workers |
|  | Weather conditions | Medium (3) | Workers must remain hydrated through regularly drinking water. Workers to stay out of sun where possible and implement rest breaks or job rotation where necessary. Workers must wear appropriate PPE including: sun cream, sun hats, and tinted safety glasses while working outside. | Low (1) | Workers |
|  | Environmental contamination i.e. erosion and sediment | Medium (3) | Where possible works to be timed to avoid rain Sandbags to be used to slow runoff and contain sediment Workers to follow the project environmental requirements (where required) | Low (1) | Workers & Supervisor |
| Accessing and securing the site | Unauthorised individuals entering work site | Medium (3) | Barricades & signage to be erected to prevent unauthorised access to work area. Where appropriate, spotters to be used to observe personnel and re-direct. Alternate access to be provided where appropriate. | Low (1) | Workers |
| Delivery and unload materials and position in work area | Manual handling | Medium (4) | Workers to follow correct manual handling procedures: Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit. Use team lifts where needed Clear a path, assess load, get a good grip, ensure a wide stance, bend knees, keep back straight, use legs to lift, do not twist trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Workers |
|  | Inappropriate storage areas may interfere with public | Medium (3) | Materials must be transported in accordance with SDS specifications. Secure storage areas to be allocated for products. Spill kits and firefighting equipment to be available on site to manage emergency situations. | Low (1) | Workers |
| Prepare surface | Dust inhalation or in eyes | Medium (3) | Cover all areas with plastic or appropriate sheeting. Use power tools with dust extraction where possible. Open windows and use dust extraction devices where necessary. Eye protection to be worn while sanding grinding or drilling Avoid sanding above head height. Wear dust mask or respirator (As per AS/NZS 1716) when sanding, grinding, drilling | Low (1) | Workers |
|  | Incorrect use of equipment | Medium (3) | Follow operator’s manual for safe operations. Never point equipment at people. | Low (1) | Workers |
|  | Repetitive work and prolonged fixed postures | Medium (4) | Workers must warm up and stretch prior to commencing work. Workers to rotate tasks to avoid repetitive tasks and fixed postures. Workers to avoid excessive straining and over reaching. | Low (2) | Workers |
| Prepare surface | Fall from heights | High (6) | Implement the use of a work platform, physical barrier or physical restraint (In that order) for all work where a worker can fall from one level to another. Workers to follow SWI for control measure selected above such as: Mobile scaffold, scaffold, EWP, etc. Workers must review and implement safety requirements specified by suppliers. Pre-start inspections must be completed on equipment daily & logbook completed. | Low (2) | Workers |
| Prepare surface | Fall from ladder | High (6) | Avoid overhead electrical hazards. Use a fibreglass platform ladder. Perform visual check on ladder for obvious faults and defects prior to use. Damaged ladders are to be removed from use, tagged “not for use” and repaired ASAP. Fully enclosed, slip resistant footwear should be worn. The top of the ladder should extend above the working platform/roof by no less than 1m. Never “walk” ladder to reposition it; descend and relocate it. Erect ladder on clean, level surface or use self-levelling ladder. Erect extension ladder in a 4:1 ratio (4m up for every 1m from wall) and tie off at top and bottom. Use a footer where necessary. Do not place a ladder in front of a doorway unless locked/guarded Secure work area around the ladder. Clean mud, grass etc, from boots to prevent slippage. Do not over-reach while on ladder. Only one person on a ladder at any one time. Face the ladder at all times. Ensure legs are fully extended and latch is in place Maintain three points of contact on ladder Do not step above the third step from the top. Where working from more than a couple of stories high a double lanyard fall restraint system is to be used. | Low (2) | Workers |
|  | Falling objects | Medium (3) | Use a tool pouch to carry tools. Do not leave tools and equipment on ladder rungs. Wear hard hats and steel-capped boots. Implement drop-zone around area. | Low (1) | Workers |
| Set up mixer | Faulty equipment may lead to possible injury to workers or public | Medium (3) | Workers to complete pre-start check on machine for defects if faulty remove from service and report to supervisor any repairs needed. Workers to check all fittings and hoses for damage prior to use. Workers to check machine to ensure it is running safely. | Low (1) | Workers |
| Set up mixer | Electrical shock due to faulty equipment | High (6) | Workers to complete pre-start check on machine, leads plugs and power outlet for any damage if faulty remove from service and report to supervisor any repairs needed. Workers to check that machine has current electrical test tag in place and is safe for use. Workers to check machine to ensure it is running safely Workers to ensure that RCD’s/ELCB’s are in use and that leads do not run across driveways pathways or near any water. Use lead stands where necessary. | Low (2) | Workers |
|  | Injury to personnel due to fire when re-fuelling | Medium (3) | Petrol and its vapours are extremely flammable allow engine to cool for at least two minutes before refuelling and always use fresh petrol of the correct type for the machine. Use funnels or can with spout when filling petrol. Take care to avoid spillages and immediately clean up any spills. Do not over fill the tank. Return fuel to vehicle or secure location after filling. Never smoke when re-fuelling Ensure petrol, pump and combustible items are a safe distance from ignition points. Ensure a clear area is maintained around engine. | Low (1) | Workers |
| Operating mixer | Body parts or equipment caught in mixer | Medium (3) | Never place body parts or equipment into the moving mixer. Disconnect power when cleaning or servicing machine. | Low (1) | Workers |
|  | Manual handling | Medium (4) | Workers to follow correct manual handling procedures: Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit. Use team lifts where needed Clear a path, assess load, get a good grip, ensure a wide stance, bend knees, keep back straight, use legs to lift, do not twist trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Workers |
| Apply render | Fall from heights | High (6) | Implement the use of a work platform, physical barrier or physical restraint (In that order) for all work where a worker can fall from one level to another. Workers to follow SWI for control measure selected above such as: Mobile scaffold, scaffold, EWP, etc. Workers must review and implement safety requirements specified by suppliers. Pre-start inspections must be completed on equipment daily & logbook completed. | Low (2) | Workers |
|  | Fall from ladder | High (6) | Avoid overhead electrical hazards. Use a fibreglass platform ladder. Perform visual check on ladder for obvious faults and defects prior to use. Damaged ladders are to be removed from use, “tagged not for use” and repaired ASAP. Fully enclosed, slip resistant footwear should be worn. The top of the ladder should extend above the working platform/roof by no less than 1m. Never “walk” ladder to reposition it; descend and relocate it. Erect ladder on clean, level surface or use self-levelling ladder. Erect extension ladder in a 4:1 ratio (4m up for every 1m from wall) and tie off at top and bottom. Use a footer where necessary. Do not place a ladder in front of a doorway unless locked/guarded Secure work area around the ladder. Clean mud, grass etc, from boots to prevent slippage. Do not over-reach while on ladder. Only one person on a ladder at any one time. Face the ladder at all times. Ensure legs are fully extended and latch is in place Maintain three points of contact on ladder ## Do not step above the third step from the top. Where working from more than a couple of stories high a double lanyard fall restraint system is to be used. | Low (2) | Workers |
|  | Falling objects | Medium (3) | Use a tool pouch to carry tools. Do not leave tools and equipment on ladder rungs. Wear hard hats and steel-capped boots. ## Implement drop-zone around area. | Low (1) | Workers |
| Apply render | Repetitive work and prolonged fixed postures | Medium (4) | Workers must warm up and stretch prior to commencing work. Workers to rotate tasks to avoid repetitive tasks and fixed postures. Workers to avoid excessive straining and over reaching. | Low (2) | Workers |
|  | Exposure to hazardous substances | Medium (3) | Site Supervisor to ensure that SDS for all hazardous substances on site are available to all workers and that the relevant first aid requirements outlined in the SWMS are available to manage exposure. Workers to review of all hazardous substances to be used and follow safety guidelines. Renders to wear gloves when applying render | Low (1) | Workers |
| Packing up job | Housekeeping | Medium (3) | All tools to be checked for damage and then returned to appropriate storage position. All waste material is to be cleaned up and disposed of appropriately. All PPE must be returned to storage position. Only trained and competent workers to be used. | Low (1) | Workers |

| Personal Protective Equipment (PPE) required: | Enclosed leather steel capped safety boots (As per AS/NZS 2210) Safety Glasses (As per AS/NZS 1336) Hard Hats (As per AS/NZS 1801) Leather Gloves (As per AS/NZS 2261) Sun hats, sun cream, long sleeve shirts and long pants, disposable coveralls |
| --- | --- |
| List the training required by workers to complete the work: List the qualifications of personnel doing the work: | Construction Induction Card Relevant trade certificate Induction into SWMS |
| List the Permits, Certificates, SafeWork NSW Approvals, required to complete the work: |  |
| Relevant legislation: For further details refer to the Legal Requirements Register | Work in accordance with Work Health and Safety Act 2011 and Work Health and Safety Regulation 2025, workplace injury management and workers compensation act 1998 no. 86, AS 1269.3 Occupational Noise management, AS/NZS1337.1 Personal eye protection-Eye and face protectors for occupational applications, AS/NZS 1269.3 Occupational Noise management-Hearing protection program, AS/NZS 1801 industrial safety helmets, AS/NZS 2210.1 Safety, protective and occupational footwear-Guide to selection, care and use & AS/NZS 2210.2 to 2210.9 Occupational protective footwear. SafeWork NSW Codes of Practice: - Construction Work - Managing risks of plant - Managing the risk of falls - Work Health and Safety Consultation, Co-operation and Co-ordination - Managing the Work Environment and Facilities - First Aid in the Workplace - Hazardous Manual Tasks - How to Manage Work Health and Safety Risks |
| List the plant / equipment that will be used on site: List maintenance checks for plant & equipment: | Power tools and mixer Pre start inspection, current electrical test tag and regular servicing as per manufacturers recommendations |
| Frequency of site/workplace inspections: Site Inspections will be completed by: | Daily site inspections to be completed by Site Supervisor |
| Hazardous Substances: |  |

# Sign Off and Approvals

| Person responsible for site supervision of the work, inspecting and approving work areas, approval of and compliance with SWMS, protective measures, plant, equipment: |  |
| --- | --- |
| Name and position: | Signature: |
| SWMS Prepared by: Ben Knox – Compliance OH&S | Signature: |

| Risk Level | Description of consequence or impact | Consequence | Likelihood/Probability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Unlikely (1) | Possible (2) | Almost Certain (3) |
| High Level of harm | Actual/Potential fatality, disability or irreversible damage. Major structural failure/damage. Off-site environmental discharge/release not contained and significant long-term environmental harm. | Major (3) | Medium (3) | High (6) | High (9) |
| Medium Level of harm | Actual/Potential temporary disability, MTI or LTI. Structural failure/damage, >1-day outage. On-site environmental discharge/release contained, minor remediation, short-term environmental harm. | Moderate (2) | Low (2) | Medium (4) | High (6) |
| Low Level of harm | Incident that has the potential to cause persons to require first aid. Environmental discharge/release immediately contained, minor level clean-up with no short term environmental harm. | Minor (1) | Low (1) | Low (2) | Medium (3) |
| Level | Likelihood/Probability |  |  |  |  |
| Almost Certain | Occurs frequently; >66% chance of occurring |  |  |  |  |
| Possible | Could happen occasionally; >33% but <66% chance of occurring |  |  |  |  |
| Unlikely | May occur only in exceptional circumstances; <33% chance of occurring |  |  |  |  |
| Class/Ranking | Description/Requirements |  |  |  |  |
| High 6, 9 | Stop immediately. Implement permanent preventative controls. |  |  |  |  |
| Medium 3, 4 | Will require operational planning. Actions will be recorded on a SWMS. |  |  |  |  |
| Low 1, 2 | Will require detailed pre-planning. Actions will be recorded on a SWMS. |  |  |  |  |

SWMS has been developed using the hierarchy of controls to reduce the risk and likely hood of an incident/injury occurring. The use of Elimination, Substitution, Engineering/isolation, Administrative and PPE measures will be used in the above order of preference wherever practicable.

Induction confirmation:

This SWMS has been developed in consultation with the workers and has been read and signed by all workers involved in this activity. I note that I have read and understood the above safe work method statement and will ensure work is completed accordingly.

| Date: | Name: | Signature: |
| --- | --- | --- |
| Date: | Name: | Signature: |