| NOTE: Work must be performed in accordance with this SWMS. This SWMS must be kept and be available for inspection until the high-risk construction work to which this SWMS relates is completed. If the SWMS is revised, all versions should be kept. If a notifiable incident occurs in relation to the high-risk construction work in this SWMS, the SWMS must be kept for at least 2 years from the date of the notifiable incident. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| PCBU Name & Contact Details: Robertson’s Remedial and Painting Pty Ltd # Unit 10/56 Buffalo Road Gladesville NSW 2111 Phone: (02) 9181 3519 A.B.N 16 140 746 247 |  |  |  |  | Principal Contractor (PC): |  |
| Work Manager Contact Number: |  |  |  |  | Date SWMS provided to PC: |  |
| Work Activity: | Glazing |  |  |  | Workplace Location: |  |
| High Risk Construction Work: | Risk of person falling more than 2 metres (note: in some jurisdictions, this is 3metres) |  |  | Work on a telecommunication tower |  | Demolition of a load-bearing structure |
|  | Likely to involve disturbing asbestos |  |  | Temporary load-bearing support for structural alterations or repairs |  | Work in or near a confined space |
|  | Work in or near a shaft or trench deeper than 1.5m or a tunnel |  |  | Work in an area that may have a contaminated or flammable atmosphere. |  | Work on or near pressurised gas mains or piping |
|  | Work on or near chemical, fuel or refrigerant lines |  |  | Work on or near energised electrical installations |  | Use of explosives |
|  | Diving Work |  |  | Work on, in or adjacent to a road, railway, shipping lane or other traffic corridor in use by traffic other than pedestrians |  | Work in an area with movement of powered mobile plant. |
|  | Work in areas with artificial extremes of temperature |  |  | Work in or near water or other liquid that involves a risk of drowning |  | Tilt-up or precast concrete elements |
| Name of workers consulted in the development of this SWMS: |  |  |  |  | Signature: |  |
| Person responsible for ensuring compliance with SWMS: |  |  |  |  | Date SWMS received: |  |
| What measures are in place to ensure compliance with the SWMS? |  |  | Toolbox meetings, SWMS sign off, job observations and supervision review. If issues with the SWMS or new hazards are identified, the supervisor must be notified. When changes are made to SWMS, it will be communicated to all workers. |  |  |  |
| Person responsible for approving and reviewing SWMS control measures: |  |  |  |  | Date SWMS received by reviewer: |  |
| How will the SWMS control measures be reviewed? |  |  | The control measures implemented will be reviewed and if necessary, revised annually or if work methods change, the control measures are not effective in controlling the risk, a new hazard/risk is identified or following an incident. The SWMS will be reviewed in consultation with workers and/or others who may be affected by the SWMS. Any changes to the SWMS will be communicated with workers at induction, daily pre-starts and toolbox talks. |  |  |  |
| Review Date: |  |  |  |  | Reviewer’s signature: |  |

| Procedure (In Steps): | Possible Hazards: | Risk Rating: | Safety Controls: | Residual Risk: | Responsible Person: |
| --- | --- | --- | --- | --- | --- |
| Planning for the setup of job. Check all Workers tickets/licences, check site, complete induction | Untrained or unlicensed workers may access site causing serious injury to self or other individuals | Medium (3) | All workers must have Construction Induction Certificate. All workers to be qualified/ licensed/ ticketed/ trained as required for the works. All workers must complete site induction and sign in / out. Where induction is not available workers to liaise with site supervisor about site hazards, emergency procedures and any other site requirements. All workers to be trained in and sign safe work method statement. Toolbox talk to be conducted prior to commencement of works All workers to be in a fit state to commence work, be zero blood alcohol concentration, not suffering from fatigue or under the influence of drugs. Competent person to supervise all new workers. | Low (1) | Workers & Supervisor |
|  | Inappropriate tool and equipment allocation may lead to injury | Medium (3) | Site Supervisor/Management to ensure that appropriate equipment and procedures are used for the works being completed. Site Supervisor /Management to ensure that all electrical devices have an electrical test tag prior to commencement of work. Management to ensure that adequate PPE is available to workers | Low (1) | Workers & Supervisor |
| Accessing and securing the site | Slips, trips and falls | Medium (3) | All workers to wear steel capped boots. Onsite inductions to be completed prior to entering the job including identification of any slip trip and fall hazards. Ensure the work area is clear and free of slip, trip and fall hazards | Low (1) | Workers |
|  | Weather conditions I.e. heat stroke, dehydration, skin cancer, sun burn etc | Medium (3) | Workers must remain hydrated through regularly drinking water. Workers to stay out of sun where possible and implement rest breaks or job rotation where necessary. Workers must wear appropriate PPE including: sun cream, sun hats, and tinted safety glasses while working outside. | Low (1) | Workers |
|  | Environmental contamination i.e. erosion and sediment | Medium (3) | Where possible works to be timed to avoid rain Sandbags to be used to slow runoff and contain sediment Workers to follow the project environmental requirements (where required) | Low (1) | Workers |
| Procedure (In Steps): | Possible Hazards: | Risk Rating: | Safety Controls: | Residual Risk: | Responsible Person: |
| Accessing and securing the site | Emergency situation Unfamiliar with emergency procedures | Medium (3) | All workers to ensure they are familiar with the emergency muster points of the site All workers to follow the onsite emergency procedures as outlined at induction. | Low (1) | Workers |
|  | Injury to personnel due to individuals entering work site | Medium (3) | Appropriate barricades & Signage to be erected to prevent unauthorised access to work area. Where appropriate, spotters to be used to observe personnel and re direct where needed. Alternate access to be provided where appropriate. | Low (1) | Workers |
| Delivery and unload materials and position in work area | Manual handling | Medium (4) | Workers to follow correct manual handling procedures: Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit Use team lifts where needed Clear a path, assess the load, get a good grip, ensure a wide stance, bend the knees, keep the spine straight, use the legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Workers |
|  | Inappropriate storage areas may interfere with public | Medium (3) | Materials transported in accordance with SDS specifications. Secure storage areas to be allocated for products. Spill kits and firefighting equipment to be available on site to manage emergency situations. | Low (1) | Workers |
| Remove existing fittings/fixtures | Cuts to workers hands arms etc | Medium (3) | Take care when removing fixtures. Use tools where possible to handle glass rather than hands. Leather gloves and wrist guards to be worn. | Low (1) | Workers |
|  | Glass or objects in the eyes | Medium (3) | All workers to wear safety glasses at all times | Low (1) | Workers |
|  | Injury due to inappropriate use of drill | Medium (3) | Follow SWI for drilling. Two hands always used to operate drill, including small models. Allow drill to "run up" to operating speed before applying it to the job. Hold the drill against the work piece with minimum pressure, so it doesn't "dig in" and cause it to kick back. | Low (1) | Workers |
| Procedure (In Steps): | Possible Hazards: | Risk Rating: | Safety Controls: | Residual Risk: | Responsible Person: |
| Remove existing fittings/fixtures | Dust & Noise | Medium (3) | Wear dust mask or respirator, safety glasses and hearing protection. Use power tools with dust extraction where possible. Open windows and use dust extraction devices where necessary. | Low (1) | Workers |
|  | Manual handling injury | Medium (4) | Workers to follow correct manual handling procedures: Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit use team lifts where needed Clear a path, assess the load, get a good grip, ensure a wide stance, bend the knees, keep the spine straight, use the legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Workers |
| Drill holes for installation of glass | Injury due to inappropriate use of drill | Medium (3) | Follow SWI for drilling. Two hands always used to operate drill, including small models. Allow the drill to "run up" to operating speed before applying it to the job. Hold the drill against the work piece with minimum pressure, so it doesn't "dig in" and cause it to kick back. | Low (1) | Workers |
|  | Electrical hazard caused by drilling into cables | High (6) | Workers to avoid drilling into or close to cables Workers to be aware where all cables and electrical cords run in and around structures before commencing work. Isolate power prior to drilling | Low (2) | Workers |
| Install glass/secure window | Cuts and abrasions | Medium (3) | Ensure appropriate leather gloves and wrist guards, leather steel capped boots, long sleeves and long pants are worn while transporting glass. Ensure clear path has been made to site Use trolley to transport glass as close to site as possible. Safety glasses to be worn. Take extreme care when transporting and installing. | Low (1) | Workers |
|  | Injury to operator from drill use | Medium (3) | Follow SWI for drilling. Two hands always used to operate drill, including small models. Allow drill to "run up" to operating speed before applying it to the job. Hold the drill against the work piece with minimum pressure, so it doesn't "dig in" and cause it to kick back. | Low (1) | Workers |
| Procedure (In Steps): | Possible Hazards: | Risk Rating: | Safety Controls: | Residual Risk: | Responsible Person: |
| Install glass/secure window | Manual handling risk | Medium (4) | Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit or use team lifts. Workers to follow correct manual handling procedures: Clear a path, assess the load, get a good grip, ensure a wide stance, bend the knees, keep back straight, use the legs to lift, do not twist back, use a smooth action, brace stomach muscles and lower safely. | Low (2) | Workers |
|  | Exposure to chemicals | Medium (4) | Workers to wear protective gloves when working with substances. Ensure adequate ventilation in areas where chemicals are used. | Low (2) | Workers |
|  | Falling objects | High (6) | Use a tool pouch to carry tools and/or tie tools to lanyard to ensure they are secured. Do not leave tools & equipment on ladder rungs. Implement drop zones and exclusion zones. Wear PPE: steel-capped safety boots, safety glasses and hard hat. | Low (2) | Workers |
|  | Fall from ladder | High (6) | Avoid overhead electrical hazards and use fibreglass platform ladder Perform visual check on ladder for obvious faults and defects prior to use. Remove damaged ladders, tag “not for use” and repaired. When gaining access to a working platform/roof, the top of the ladder should extend above working platform by no less than 1m. Never “walk” ladder to reposition it; descend ladder and relocate it. Erect ladder on clean, level surface or use self levelling ladder. Erect extension ladder in a 4:1 ratio (4 meters up for every 1 meter from the wall) and tie off at top and bottom. Use a footer. Do not place a ladder in front of a doorway unless locked/guarded Secure work area around the ladder. Clean mud, grass etc, from boots. Slip resistant footwear worn. Do not over-reach while on ladder. 1 person on ladder at a time. Face ladder at all times. Maintain three points of contact. Do not step above the third step from the top. Ensure legs are fully extended and latch is in place. Where working from more than a couple of stories high a double lanyard fall restraint system is to be used. | Low (2) | Workers |
| Final check on installation | Installation may not be secure | Medium (3) | Complete a thorough review on work to ensure that there is no risk of the installations falling over or coming lose | Low (1) | Workers |
| Clean-up site Pack materials and tools | Housekeeping | Medium (3) | All waste to be disposed at designated waste disposal area. Sweep up and remove all glass and metal shavings and dispose of appropriately. Vacuum where necessary. | Low (1) | Workers |

| Personal Protective Equipment (PPE) required: | Enclosed leather steel capped boots Safety Glasses (As per AS/NZS 1337) Hearing Protection Protective Gloves (As per AS/NZS 2161) | High visibility Long sleeve shirt & pants Hard Hats (As per AS/NZS 1801) Sun hats, sun cream - when working outdoors |
| --- | --- | --- |
| List the training required by workers to complete the work: List the qualifications of personnel doing the work: | Construction Induction Card Relevant trade certificate Induction into SWMS |  |
| List the Permits, Certificates, SafeWork NSW Approvals, required to completed the work: | All necessary permits to be completed on site |  |
| Relevant legislation: For further details refer to the Legal Requirements Register | Work in accordance with Work Health and Safety Act 2011 and Work Health and Safety Regulation 2017, workplace injury management and workers compensation act 1998 no. 86, AS/NZS1337.1 Personal eye protection-Eye and face protectors for occupational applications (2010), AS/NZS 1269.3 Occupational Noise management-Hearing protection program (2005), AS/NZS 1801 industrial safety helmets (1997), AS/NZS 2210.1 Safety, protective and occupational footwear-Guide to selection, care and use (2010) & AS/NZS 2210.2 to 2210.9 Occupational protective footwear. SafeWork NSW Codes of Practice: - Construction Work - Managing risks of plant - Managing the risk of falls - Work Health and Safety Consultation, Co-operation and Co-ordination - Managing the Work Environment and Facilities - First Aid in the Workplace - Hazardous Manual Tasks - How to Manage Work Health and Safety Risks |  |
| List the plant / equipment that will be used on site: | Ladders and Drills |  |
| List maintenance checks for plant & equipment: | Daily pre-start checks, maintenance as per manufacturers guidelines |  |
| Frequency of site/workplace inspections: Site Inspections will be completed by: | Daily site inspections & job completion inspections to be completed by Site Supervisor. |  |
| Hazardous Substances: | Chemicals and glues used to install glass |  |

# Sign Off and Approvals

| Person responsible for site supervision of the work, inspecting and approving work areas, approval of and compliance with SWMS, protective measures, plant, equipment: |  |
| --- | --- |
| Name and position: | Signature: |
| SWMS Prepared by: Ben Knox – Compliance OH&S | Signature: |

| Risk Level | Description of consequence or impact | Consequence | Likelihood/Probability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Unlikely (1) | Possible (2) | Almost Certain (3) |
| High Level of harm | Actual/Potential fatality, disability or irreversible damage. Major structural failure/damage. Off-site environmental discharge/release not contained and significant long-term environmental harm. | Major (3) | Medium (3) | High (6) | High (9) |
| Medium Level of harm | Actual/Potential temporary disability, MTI or LTI. Structural failure/damage, >1-day outage. On-site environmental discharge/release contained, minor remediation, short-term environmental harm. | Moderate (2) | Low (2) | Medium (4) | High (6) |
| Low Level of harm | Incident that has the potential to cause persons to require first aid. Environmental discharge/release immediately contained, minor level clean-up with no short term environmental harm. | Minor (1) | Low (1) | Low (2) | Medium (3) |
| Level | Likelihood/Probability |  |  |  |  |
| Almost Certain | Occurs frequently; >66% chance of occurring |  |  |  |  |
| Possible | Could happen occasionally; >33% but <66% chance of occurring |  |  |  |  |
| Unlikely | May occur only in exceptional circumstances; <33% chance of occurring |  |  |  |  |
| Class/Ranking | Description/Requirements |  |  |  |  |
| High 6, 9 | Stop immediately. Implement permanent preventative controls. |  |  |  |  |
| Medium 3, 4 | Will require operational planning. Actions will be recorded on a SWMS. |  |  |  |  |
| Low 1, 2 | Will require detailed pre-planning. Actions will be recorded on a SWMS. |  |  |  |  |

SWMS has been developed using the hierarchy of controls to reduce the risk and likely hood of an incident/injury occurring. The use of Elimination, Substitution, Engineering/isolation, Administrative and PPE measures will be used in the above order of preference wherever practicable.

Induction confirmation:

This SWMS has been developed in consultation with the workers and has been read and signed by all workers involved in this activity. I note that I have read and understood the above safe work method statement and will ensure work is completed accordingly.

| Date: | Name: | Signature: |
| --- | --- | --- |
| Date: | Name: | Signature: |