| NOTE: Work must be performed in accordance with this SWMS. This SWMS must be kept and be available for inspection until the high-risk construction work to which this SWMS relates is completed. If the SWMS is revised, all versions should be kept. If a notifiable incident occurs in relation to the high-risk construction work in this SWMS, the SWMS must be kept for at least 2 years from the date of the notifiable incident. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| PCBU Name & Contact Details: Robertson’s Remedial and Painting Pty Ltd # Unit 10/56 Buffalo Road Gladesville NSW 2232 Phone: (02) 9181 3519 A.B.N 16 140 746 247 |  |  |  |  | Principal Contractor (PC): | Robertson’s Remedial and Painting Pty Ltd |
| Work Manager Contact Number: |  | Dave Oste - 0476 265 066 |  |  | Date SWMS provided to PC: | 06-02-2026 |
| Work Activity: | General Plumbing |  |  |  | Workplace Location: | 6a Francis Street, Bondi Beach |
| High Risk Construction Work: | Risk of person falling more than 2 metres (note: in some jurisdictions, this is 3metres) |  |  | Work on a telecommunication tower |  | Demolition of a load-bearing structure |
|  | Likely to involve disturbing asbestos |  |  | Temporary load-bearing support for structural alterations or repairs |  | Work in or near a confined space |
|  | Work in or near a shaft or trench deeper than 1.5m or a tunnel |  |  | Work in an area that may have a contaminated or flammable atmosphere. |  | Work on or near pressurised gas mains or piping |
|  | Work on or near chemical, fuel or refrigerant lines |  |  | Work on or near energised electrical installations |  | Use of explosives |
|  | Diving Work |  |  | Work on, in or adjacent to a road, railway, shipping lane or other traffic corridor in use by traffic other than pedestrians |  | Work in an area with movement of powered mobile plant. |
|  | Work in areas with artificial extremes of temperature |  |  | Work in or near water or other liquid that involves a risk of drowning |  | Tilt-up or precast concrete elements |
| Name of workers consulted in the development of this SWMS: |  |  |  |  | Signature: |  |
| Person responsible for ensuring compliance with SWMS: |  |  |  |  | Date SWMS received: |  |
| What measures are in place to ensure compliance with the SWMS? |  |  | Toolbox meetings, SWMS sign off, job observations and supervision review. If issues with the SWMS or new hazards are identified, the supervisor must be notified. When changes are made to SWMS, it will be communicated to all workers. |  |  |  |
| Person responsible for approving and reviewing SWMS control measures: |  |  |  |  | Date SWMS received by reviewer: |  |
| How will the SWMS control measures be reviewed? |  |  | The control measures implemented will be reviewed and if necessary, revised annually or if work methods change, the control measures are not effective in controlling the risk, a new hazard/risk is identified or following an incident. The SWMS will be reviewed in consultation with workers and/or others who may be affected by the SWMS. Any changes to the SWMS will be communicated with workers at induction, daily pre-starts and toolbox talks. |  |  |  |
| Review Date: |  |  |  |  | Reviewer’s signature: |  |

| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| --- | --- | --- | --- | --- | --- |
| Planning for the setup of job. | Untrained or unlicensed workers may access site causing serious injury to self or other individuals | Medium (3) | All workers must have Construction Induction Certificate. All workers to be qualified/ licensed/ ticketed/ trained as required for the works. All workers must complete site induction upon arrival to site and sign in. Where induction is not available workers to liaise with site supervisor about site hazards, emergency procedures and any other site requirements. All workers to be trained in and sign safe work method statement. Toolbox talk to be conducted prior to commencement of works All workers to be in a fit state to commence work, be zero blood alcohol concentration, not suffering from fatigue or under the influence of drugs. Competent person to supervise all new workers. | Low (1) | Workers & Supervisor |
|  | Inappropriate tool and equipment allocation may lead to injury | Medium (3) | Site Supervisor/Management to ensure that appropriate equipment and procedures are used for the works being completed. Site Supervisor /Management to ensure that all electrical devices have an electrical test tag prior to commencement of work. Management to ensure that adequate personal protective equipment is available to workers | Low (1) | Workers & Supervisor |
|  | Slips, trips and falls | Medium (3) | All workers to wear steel capped boots. Identify and ensure the work area is clear and free of slip, trip and fall hazards and obstructions. | Low (1) | Workers |
|  | Emergency onsite, Workers unfamiliar with emergency procedures | Medium (3) | All workers to ensure they are familiar with the emergency muster points of the site All workers to follow the onsite emergency procedures as outlined at induction. | Low (1) | Workers |
|  | Unauthorised individuals entering work site | Medium (3) | Barricades & signage to be erected to prevent unauthorised access to work area. Where appropriate, spotters to be used to observe personnel and re direct where needed. Alternate access to be provided where appropriate. | Low (1) | Workers |
| Deliver and unload materials to work area | Manual handling | Medium (4) | Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit. Use team lifts where needed. Workers to follow correct manual handling procedures: Clear a path, assess the load, get a good grip, ensure a wide stance, bend the knees, keep the spine straight, use the legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Workers |
|  | Inappropriate storage areas | Medium (3) | Identify and communicate a designated storage area. Ensure the area is barricaded and made safe, not protruding into pathways. | Low (1) | Workers |
| Inspect Plumbing Equipment |  |  |  |  |  |
| Isolate equipment, Lock out tag out | Electrocution or injury due to contact with services or live cables. | High (6) | Isolate, lock out/tag out services, where appropriate. Do not work on or near live services. Check services to ensure dead before working on them. Insulated tools to be used. Isolation sign off to be completed and handed to the site supervisor from the appropriate licensed plumbing contractors before any work commences. Leather steel capped rubber soled boots to be worn | Low (2) | Workers |
| Visually inspect equipment | Confined space entry- where required | High (9) | Workers must follow confined space entry procedures for entering all confined spaces Confined Space Entry Permit to be obtained from principal contractor where required Stand by person must be in place and follow all procedures Only persons with confined space entry training to enter. | Medium (3) | Workers |
|  | Cuts and abrasions | Medium (3) | Workers to wear gloves at all times. Take care with sharp objects. | Low (1) | Workers |
|  | Plumbing leaks | Medium (3) | Water to be isolated/locked out/tagged out by licensed plumber. Sign off to be completed and handed to the site supervisor from the appropriate plumber before any work commences. Check all services for leaks prior to working. Clear lines of all water prior to working. | Low (1) | Workers |
| Visually inspect equipment | Fall from heights | High (6) | Select appropriate fall protection equipment to access heights where a worker can fall from one level to another. Implement the use of a work platform, physical barrier. Follow SWI for control measure selected above such as: Mobile scaffold, Set scaffold, EWP Daily pre-start inspections must be completed on equipment & logbooks completed. Use a fibreglass platform ladder. Perform visual check on ladder for obvious faults and defects prior to use. Remove damaged ladders, tag “not for use” and repair ASAP. When gaining access to a working platform, the top of the ladder should extend above the working platform by no less than 1m. Never “walk” ladder to reposition it; descend ladder and relocate it. Erect ladder on clean, level surface or use self-levelling ladder. Erect extension ladder, 4meters up for every 1meter from the wall, and tie off at top and bottom. Use a footer where necessary. Do not place a ladder in front of a doorway unless locked/guarded. Clean mud, grass etc, from boots to prevent slippage. Do not over-reach while on ladder. Only one person on a ladder at any one time. Face the ladder at all times. Ensure legs are fully extended and latch is in place. Maintain three points of contact at all times. Do not step above the third step from the top. Fully enclosed, slip resistant footwear should be worn. Where working from more than a couple of stories high a double lanyard fall restraint system is to be used. | Low (2) | Workers |
|  | Slip, trip or fall | Medium (3) | Take care to avoid all pipes and trip hazards Do not run cables or hoses on the ground. Wear slip resistant rubber sole boots. Ensure surfaces are clean before working on. | Low (1) | Workers |
| Isolate and remove existing services | Cuts and abrasions | Medium (3) | Workers to wear gloves at all times. Take care with sharp objects. | Low (1) | Workers |
| Isolate and remove existing services (pipes) | Electrocution or injury due to contact with services or cables | High (6) | Ensure services have been located prior to commencement and are clearly marked. Check drawings for services. Sign off to be completed and physically handed to the site supervisor from the appropriate licensed electrician and plumbing contractors before any work commences. Isolate, lock out/tag out all services. Do not work near live services. Leather steel capped rubber soled boots to be worn. | Low (2) | Workers |
|  | Possible contamination (biological, hazardous or toxic materials) | Medium (4) | Avoid any personnel from making direct skin contact with substances. Employ barrier methods. Any open wounds must be securely bandaged. In the event of contamination, wash area thoroughly with water to remove any excess material, then immerse in Hibitane concentrate or betadine and scrubbed with a suitable brush. Workers to wear impervious gloves when dealing with waste. | Low (2) | Workers |
|  | Manual handling | Medium (4) | Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit Use team lifts where needed Workers to follow correct manual handling procedures: Clear a path, assess the load, get a good grip, ensure a wide stance, bend the knees, keep the spine straight, use the legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Workers |
|  | Asphyxiation or explosion from gas leak | High (6) | Gas to be isolated/locked out/tagged out by licensed plumber. Check all services for leaks & clear lines of all gas prior to working. Maintain constant monitoring for possible gas leaks | Low (2) | Workers |
| Install pipes - check material compliance to specs. Measure & Cut materials to length | Repetitive tasks and posture | Medium (4) | Workers must warm up and stretch prior to commencing work. Workers to rotate tasks to avoid repetitive tasks and fixed postures. Workers to avoid excessive straining and over reaching. | Low (2) | Workers |
|  | Inappropriate materials | Medium (3) | Inspect all materials to ensure that they are the correct type, quantity, labelled correctly and have are not damaged. Check all materials purchased comply with spec and that they meet relevant Australian Standards. | Low (1) | Workers |
| Bend pipes where necessary | Cuts to fingers or body parts | Medium (3) | Keep hand and fingers clear of bending tools. The preferred technique is to use the pipe cutter instead of the saw. Workers must wear safety glasses and leather gloves. Take care not to touch sharp edges. Use vice when cutting large pipe. | Low (1) | Workers |
| Connect pipes, bends and joints Check for leaks | Injury due to inappropriate use of oxy cutter | Medium (4) | Follow SWI for equipment used. Ensure that the metal is secure. Adopt a comfortable stance, with feet apart and well balanced. Never use between the legs while sitting on the floor. Keep all fumes and gases from the breathing area and ensure adequate ventilation. Never touch any parts that are hot. Welding jacket and protective headwear to be worn where necessary. Set flows to appropriate levels. Always light with lint lighter or pilot light. Acetylene first then turn on oxy. Heat up metal prior to cutting/welding. Ensure that off cuts or slag is secured or are falling in safe area. Avoid molten metal landing on body parts or flammable materials. | Low (2) | Workers |
|  | Exposure to hazardous substances | Medium (3) | SDS to be on site and all instructions followed by workers. As a minimum dust masks/respirator and gloves to be worn | Low (1) | Workers |
| Clean-up and remove all waste materials from site | Housekeeping Slips, trips and falls | Medium (3) | Ensure that the system is free of all leaks. Repair any leaks immediately. Clean-up spills and ensure area is clean and dry. Follow manual handling principles above. All tools to be checked for damage and then returned to appropriate storage position All waste should be cleaned up and disposed of appropriately | Low (1) | Workers |

| Personal Protective Equipment (PPE) required: | Enclosed leather steel capped boots Safety Glasses (As per AS/NZS 1337) Hearing Protection Protective Gloves (As per AS/NZS 2161) | High visibility Long sleeve shirt & pants Hard Hats (As per AS/NZS 1801) Dust Mask / Respirator |
| --- | --- | --- |
| List the training required by workers to complete the work: List the qualifications of personnel doing the work: | Construction Induction Card Relevant trade certificate Induction into SWMS |  |
| List the Permits, Certificates, SafeWork NSW Approvals, required to completed the work: | All necessary permits to be completed on site |  |
| Relevant legislation: For further details refer to the Legal Requirements Register | Work in accordance with Work Health and Safety Act 2011 and Work Health and Safety Regulation 2025, workplace injury management and workers compensation act 1998 no. 86, AS/NZS1337.1 Personal eye protection-Eye and face protectors for occupational applications, AS/NZS 1269.3 Occupational Noise management-Hearing protection program, AS/NZS 1801 industrial safety helmets (1997), AS/NZS 2210.1 Safety, protective and occupational footwear-Guide to selection, care and use & AS/NZS 2210.2 to 2210.9 Occupational protective footwear. SafeWork NSW Codes of Practice: - Construction Work - Managing risks of plant - Managing the risk of falls - Work Health and Safety Consultation, Co-operation and Co-ordination - Managing the Work Environment and Facilities - First Aid in the Workplace - Hazardous Manual Tasks - How to Manage Work Health and Safety Risks |  |
| List the plant / equipment that will be used on site: | Hand tools (e.g. pipe cutter, oxy) |  |
| List maintenance checks for plant & equipment: | Daily pre-start checks, maintenance as per manufacturers guidelines |  |
| Frequency of site/workplace inspections: Site Inspections will be completed by: | Daily site inspections & job completion inspections to be completed by Site Supervisor. |  |
| Hazardous Substances: | Glues |  |

# Sign Off and Approvals

| Person responsible for site supervision of the work, inspecting and approving work areas, approval of and compliance with SWMS, protective measures, plant, equipment: |  |
| --- | --- |
| Name and position: | Signature: |
| SWMS Prepared by: Ben Knox – Compliance OH&S | Signature: |

| Risk Level | Description of consequence or impact | Consequence | Likelihood/Probability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Unlikely (1) | Possible (2) | Almost Certain (3) |
| High Level of harm | Actual/Potential fatality, disability or irreversible damage. Major structural failure/damage. Off-site environmental discharge/release not contained and significant long-term environmental harm. | Major (3) | Medium (3) | High (6) | High (9) |
| Medium Level of harm | Actual/Potential temporary disability, MTI or LTI. Structural failure/damage, >1-day outage. On-site environmental discharge/release contained, minor remediation, short-term environmental harm. | Moderate (2) | Low (2) | Medium (4) | High (6) |
| Low Level of harm | Incident that has the potential to cause persons to require first aid. Environmental discharge/release immediately contained, minor level clean-up with no short term environmental harm. | Minor (1) | Low (1) | Low (2) | Medium (3) |
| Level | Likelihood/Probability |  |  |  |  |
| Almost Certain | Occurs frequently; >66% chance of occurring |  |  |  |  |
| Possible | Could happen occasionally; >33% but <66% chance of occurring |  |  |  |  |
| Unlikely | May occur only in exceptional circumstances; <33% chance of occurring |  |  |  |  |
| Class/Ranking | Description/Requirements |  |  |  |  |
| High 6, 9 | Stop immediately. Implement permanent preventative controls. |  |  |  |  |
| Medium 3, 4 | Will require operational planning. Actions will be recorded on a SWMS. |  |  |  |  |
| Low 1, 2 | Will require detailed pre-planning. Actions will be recorded on a SWMS. |  |  |  |  |

SWMS has been developed using the hierarchy of controls to reduce the risk and likely hood of an incident/injury occurring. The use of Elimination, Substitution, Engineering/isolation, Administrative and PPE measures will be used in the above order of preference wherever practicable.

Induction confirmation:

This SWMS has been developed in consultation with the workers and has been read and signed by all workers involved in this activity. I note that I have read and understood the above safe work method statement and will ensure work is completed accordingly.

| Date: | Name: | Signature: |
| --- | --- | --- |
| Date: | Name: | Signature: |