| NOTE: Work must be performed in accordance with this SWMS. This SWMS must be kept and be available for inspection until the high-risk construction work to which this SWMS relates is completed. If the SWMS is revised, all versions should be kept. If a notifiable incident occurs in relation to the high-risk construction work in this SWMS, the SWMS must be kept for at least 2 years from the date of the notifiable incident. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| PCBU Name & Contact Details: Robertson’s Remedial and Painting Pty Ltd # Unit 10/56 Buffalo Road Gladesville NSW 2232 Phone: (02) 9181 3519 A.B.N 16 140 746 247 |  |  |  |  | Principal Contractor (PC): | Robertson’s Remedial and Painting Pty Ltd |
| Work Manager Contact Number: |  | Dave Oste - 0476 265 066 |  |  | Date SWMS provided to PC: | 06-02-2026 |
| Work Activity: | Waterproofing |  |  |  | Workplace Location: | 6a Francis Street, Bondi Beach |
| High Risk Construction Work: | Risk of person falling more than 2 metres (note: in some jurisdictions, this is 3metres) |  |  | Work on a telecommunication tower |  | Demolition of a load-bearing structure |
|  | Likely to involve disturbing asbestos |  |  | Temporary load-bearing support for structural alterations or repairs |  | Work in or near a confined space |
|  | Work in or near a shaft or trench deeper than 1.5m or a tunnel |  |  | Work in an area that may have a contaminated or flammable atmosphere. |  | Work on or near pressurised gas mains or piping |
|  | Work on or near chemical, fuel or refrigerant lines |  |  | Work on or near energised electrical installations |  | Use of explosives |
|  | Diving Work |  |  | Work on, in or adjacent to a road, railway, shipping lane or other traffic corridor in use by traffic other than pedestrians |  | Work in an area with movement of powered mobile plant. |
|  | Work in areas with artificial extremes of temperature |  |  | Work in or near water or other liquid that involves a risk of drowning |  | Tilt-up or precast concrete elements |
| Name of workers consulted in the development of this SWMS: |  |  |  |  | Signature: |  |
| Person responsible for ensuring compliance with SWMS: |  |  |  |  | Date SWMS received: |  |
| What measures are in place to ensure compliance with the SWMS? |  |  | Toolbox meetings, SWMS sign off, job observations and supervision review. If issues with the SWMS or new hazards are identified, the supervisor must be notified. When changes are made to SWMS, it will be communicated to all workers. |  |  |  |
| Person responsible for approving and reviewing SWMS control measures: |  |  |  |  | Date SWMS received by reviewer: |  |
| How will the SWMS control measures be reviewed? |  |  | The control measures implemented will be reviewed and if necessary, revised annually or if work methods change, the control measures are not effective in controlling the risk, a new hazard/risk is identified or following an incident. The SWMS will be reviewed in consultation with workers and/or others who may be affected by the SWMS. Any changes to the SWMS will be communicated with workers at induction, daily pre-starts and toolbox talks. |  |  |  |
| Review Date: |  |  |  |  | Reviewer’s signature: |  |

| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| --- | --- | --- | --- | --- | --- |
| Planning for the setup of job. | Untrained or unlicensed workers may access site causing serious injury to self or other individuals | Medium (3) | All workers must have Construction Induction Certificate. All workers to be qualified/ licensed/ ticketed/ trained as required for the works. All workers must complete site induction upon arrival to site and sign in. Where induction is not available workers to liaise with site supervisor about site hazards, emergency procedures and any other site requirements. All workers to be trained in and sign safe work method statement. Toolbox talk to be conducted prior to commencement of works All workers to be in a fit state to commence work, be zero blood alcohol concentration, not suffering from fatigue or under the influence of drugs. Competent person to supervise all new workers. | Low (1) | Workers & Supervisor |
|  | Inappropriate tool and equipment allocation may lead to injury | Medium (3) | Site Supervisor/Management to ensure that appropriate equipment and procedures are used for the works being completed. Site Supervisor /Management to ensure that all electrical devices have an electrical test tag prior to commencement of work. Management to ensure that adequate personal protective equipment is available to workers | Low (1) | Workers & Supervisor |
|  | Slips, trips and falls | Medium (3) | All workers to wear steel capped boots. Identify and ensure the work area is clear and free of slip, trip and fall hazards and obstructions. | Low (1) | Workers |
|  | Emergency onsite, Workers unfamiliar with emergency procedures | Medium (3) | All workers to ensure they are familiar with the emergency muster points of the site All workers to follow the onsite emergency procedures as outlined at induction. | Low (1) | Workers |
|  | Unauthorised individuals entering work site | Medium (3) | Barricades & signage to be erected to prevent unauthorised access to work area. Where appropriate, spotters to be used to observe personnel and re direct where needed. Alternate access to be provided where appropriate. | Low (1) | Workers |
| Deliver and unload materials to work area | Manual handling | Medium (4) | Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit. Use team lifts where needed. Workers to follow correct manual handling procedures: Clear a path, assess the load, get a good grip, ensure a wide stance, bend the knees, keep the spine straight, use the legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Workers |
|  | Inappropriate storage areas | Medium (3) | Identify and communicate a designated storage area. Ensure the area is barricaded and made safe, not protruding into pathways. | Low (1) | Workers |
| Prepare Surface | Inappropriate application of materials | Medium (3) | Ensure all walls will be water resistant and all penetrations will be waterproofed All cuts and penetrations should be sealed All surfaces must be structurally sound, clean and dry. Also they must be free of voids, curing compounds, projections, loose materials, oils, grease etc. Check that waterproof mortars and grouts have been identified, selected, prepared and mixed to specifications. | Low (2) | Workers |
| Adequate lighting to be provided | Physical injury from inadequately lit areas | Low (2) | Work with assistance of artificial light where needed Supervisor to check and make arrangements prior to commencement of work | Low (1) | Workers |
| Application of liquid membrane | Skin and eye irritation/ inhalation of chemicals | Medium (3) | Ensure appropriate PPE is worn Heavy duty gloves to be worn Eye protection Appropriate respirator to be worn, refer to Safety Data Sheet (SDS) Area to barricaded off and ‘no entry’ signage posted Only to be used in a well ventilated area Appropriate breathing protection may be required | Low (2) | Workers |
| Application of liquid membrane | Exposure to hazardous substances | Medium (4) | Site supervisor to ensure that Safety Data Sheets (SDS) for all hazardous substances on site are available to all Workers and that the relevant fist aid requirements outlined in the SWMS are available to manage exposure. Workers to review of all hazardous substances to be used and follow safety guidelines. | Low (2) | Workers |
| Application of liquid membrane | Fall from ladder | High (6) | Avoid overhead electrical hazards and use fibreglass ladder Use a platform ladder where possible. Perform visual check on ladder for obvious faults and defects. Remove damaged ladders. Tag “not for use” and repair or replace. Top of the ladder should extend above the working platform/roof by no less than 1m. Never “walk” ladder to reposition it; descend ladder and relocate it. Erect ladder on clean, level surface or use self-levelling ladder. Erect extension ladder in a 4:1 ratio (4m up for every 1m from the wall) and tie off at top & bottom. Use a footer where necessary. Do not place a ladder in front of a doorway unless locked/guarded. Secure work area around the ladder. Only one person on a ladder at any time. Face the ladder at all times. Ensure legs are fully extended and latch is in place Maintain 3 points of contact on ladder Do not step above the third step from the top. Do not over-reach while on ladder. Fully enclosed, slip resistant footwear should be worn. Clean mud, grass etc, from boots to prevent slippage. Where working from more than a couple of stories high a double lanyard fall restraint system is to be used. | Low (2) | Workers |
| Installation of fibreglass corners | Skin and eye irritation/ inhalation of chemicals | Medium (4) | Ensure appropriate PPE is worn Heavy duty gloves to be worn Eye protection Appropriate respirator to be worn, refer to Safety Data Sheet (SDS) for single pack polyurethane fiberglass corners materials Area to barricaded off and ‘no entry’ signage posted Only to be used in a well-ventilated area Appropriate breathing protection may be required | Low (2) | Workers |
| Installation of fibreglass corners | Exposure to hazardous substances | Medium (4) | Site supervisor to ensure that Safety Data Sheets (SDS) for all hazardous substances on site are available to all Workers and that the relevant fist aid requirements outlined in the SWMS are available to manage exposure. Workers to review of all hazardous substances to be used and follow safety guidelines. | Low (2) | Workers |
| Use of Blackjack bitumen | Skin and eye irritation/ inhalation of chemicals | Medium (4) | Ensure appropriate PPE is worn including Heavy duty gloves to be worn Eye protection Appropriate respirator to be worn, Refer to Safety Data Sheet (SDS) for Blackjack Bitumen Area to barricaded off and ‘no entry’ signage posted Only to be used in a well ventilated area Appropriate breathing protection may be required | Low (2) | Workers |
|  | Exposure to hazardous substances | Medium (4) | Site supervisor to ensure that Safety Data Sheets (MSDS) for all hazardous substances on site are available to all Workers and that the relevant fist aid requirements outlined in the SWMS are available to manage exposure. Workers to review of all hazardous substances to be used and follow safety guidelines. | Low (2) | Workers |
| Place and fix aluminium angles for shower | Injury caused by inappropriate use of tools | Medium (4) | Workers to follow SWMS for drilling. Two hands always used to operate drill, including small models. Allow the drill to "run up" to operating speed before applying it to the job. Hold the drill against the work piece with minimum pressure, so it doesn't "dig in" and cause it to kick back. Ensure shelving is securely bolted to the ground. | Low (2) | Workers |
| Clean-up site | Housekeeping Slips, trips and falls | Medium (3) | All tools to be checked for damage and faults. Clean all tools and equipment used. Return all tools and equipment to appropriate storage area. All waste should be cleaned up and disposed of appropriately. Ensure area is left in safe and clean state. | Low (1) | Workers |
| Clean-up site | Manual handling | Medium (4) | Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit. Use team lifts where needed. Workers to follow correct manual handling procedures: Clear a path, assess the load, get a good grip, ensure a wide stance, bend the knees, keep the spine straight, use the legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Workers |

| Personal Protective Equipment (PPE) required: | Enclosed leather steel capped boots Safety Glasses (As per AS/NZS 1337) Hearing Protection Protective Gloves (As per AS/NZS 2161) | High visibility Long sleeve shirt & pants Hard Hats (As per AS/NZS 1801) Sun hats, sun cream - when working outdoors Dust Mask / Respirator |
| --- | --- | --- |
| List the training required by workers to complete the work: List the qualifications of personnel doing the work: | Construction Induction Card Relevant trade certificate Induction into SWMS |  |
| List the Permits, Certificates, SafeWork NSW Approvals, required to completed the work: | All necessary permits to be completed on site |  |
| Relevant legislation: For further details refer to the Legal Requirements Register | Work in accordance with Work Health and Safety Act 2011 and Work Health and Safety Regulation 2025, workplace injury management and workers compensation act 1998 no. 86, AS/NZS1337.1 Personal eye protection-Eye and face protectors for occupational applications, AS/NZS 1269.3 Occupational Noise management-Hearing protection program, AS/NZS 1801 industrial safety helmets, AS/NZS 2210.1 Safety, protective and occupational footwear-Guide to selection, care and use & AS/NZS 2210.2 to 2210.9 Occupational protective footwear. SafeWork NSW Codes of Practice: - Construction Work - Managing risks of plant - Managing the risk of falls - Work Health and Safety Consultation, Co-operation and Co-ordination - Managing the Work Environment and Facilities - First Aid in the Workplace - Hazardous Manual Tasks - How to Manage Work Health and Safety Risks |  |
| List the plant / equipment that will be used on site: | Hand Tools, Power Tools |  |
| List maintenance checks for plant & equipment: | Daily pre-start checks, maintenance as per manufacturers guidelines |  |
| Frequency of site/workplace inspections: Site Inspections will be completed by: | Daily site inspections & job completion inspections to be completed by Site Supervisor. |  |
| Hazardous Substances: | Glues, Putty, Waterproofing products |  |

# Sign Off and Approvals

| Person responsible for site supervision of the work, inspecting and approving work areas, approval of and compliance with SWMS, protective measures, plant, equipment: |  |
| --- | --- |
| Name and position: | Signature: |
| SWMS Prepared by: Ben Knox – Compliance OH&S | Signature: |

| Risk Level | Description of consequence or impact | Consequence | Likelihood/Probability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Unlikely (1) | Possible (2) | Almost Certain (3) |
| High Level of harm | Actual/Potential fatality, disability or irreversible damage. Major structural failure/damage. Off-site environmental discharge/release not contained and significant long-term environmental harm. | Major (3) | Medium (3) | High (6) | High (9) |
| Medium Level of harm | Actual/Potential temporary disability, MTI or LTI. Structural failure/damage, >1-day outage. On-site environmental discharge/release contained, minor remediation, short-term environmental harm. | Moderate (2) | Low (2) | Medium (4) | High (6) |
| Low Level of harm | Incident that has the potential to cause persons to require first aid. Environmental discharge/release immediately contained, minor level clean-up with no short term environmental harm. | Minor (1) | Low (1) | Low (2) | Medium (3) |
| Level | Likelihood/Probability |  |  |  |  |
| Almost Certain | Occurs frequently; >66% chance of occurring |  |  |  |  |
| Possible | Could happen occasionally; >33% but <66% chance of occurring |  |  |  |  |
| Unlikely | May occur only in exceptional circumstances; <33% chance of occurring |  |  |  |  |
| Class/Ranking | Description/Requirements |  |  |  |  |
| High 6, 9 | Stop immediately. Implement permanent preventative controls. |  |  |  |  |
| Medium 3, 4 | Will require operational planning. Actions will be recorded on a SWMS. |  |  |  |  |
| Low 1, 2 | Will require detailed pre-planning. Actions will be recorded on a SWMS. |  |  |  |  |

SWMS has been developed using the hierarchy of controls to reduce the risk and likely hood of an incident/injury occurring. The use of Elimination, Substitution, Engineering/isolation, Administrative and PPE measures will be used in the above order of preference wherever practicable.

# Induction confirmation:

This SWMS has been developed in consultation with the workers and has been read and signed by all workers involved in this activity. I note that I have read and understood the above safe work method statement and will ensure work is completed accordingly.

| Date: | Name: | Signature: |
| --- | --- | --- |
| Date: | Name: | Signature: |