| NOTE: Work must be performed in accordance with this SWMS. This SWMS must be kept and be available for inspection until the high-risk construction work to which this SWMS relates is completed. If the SWMS is revised, all versions should be kept. If a notifiable incident occurs in relation to the high-risk construction work in this SWMS, the SWMS must be kept for at least 2 years from the date of the notifiable incident. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| PCBU Name & Contact Details: Robertson’s Remedial and Painting Pty Ltd # Unit 10/56 Buffalo Road Gladesville NSW 2111 Phone: (02) 9181 3519 A.B.N 16 140 746 247 |  |  |  |  | Principal Contractor (PC): |  |
| Work Manager Contact Number: |  |  |  |  | Date SWMS provided to PC: |  |
| Work Activity | Traffic Control |  |  |  | Workplace Location: |  |
| High Risk Construction Work: | Risk of person falling more than 2 metres (note: in some jurisdictions, this is 3metres) |  |  | Work on a telecommunication tower |  | Demolition of a load-bearing structure |
|  | Likely to involve disturbing asbestos |  |  | Temporary load-bearing support for structural alterations or repairs |  | Work in or near a confined space |
|  | Work in or near a shaft or trench deeper than 1.5m or a tunnel |  |  | Work in an area that may have a contaminated or flammable atmosphere. |  | Work on or near pressurised gas mains or piping |
|  | Work on or near chemical, fuel or refrigerant lines |  |  | Work on or near energised electrical installations |  | Use of explosives |
|  | Diving Work |  |  | Work on, in or adjacent to a road, railway, shipping lane or other traffic corridor in use by traffic other than pedestrians |  | Work in an area with movement of powered mobile plant. |
|  | Work in areas with artificial extremes of temperature |  |  | Work in or near water or other liquid that involves a risk of drowning |  | Tilt-up or precast concrete elements |
| Name of workers consulted in the development of this SWMS: |  |  |  |  | Signature: |  |
| Person responsible for ensuring compliance with SWMS: |  |  |  |  | Date SWMS received: |  |
| What measures are in place to ensure compliance with the SWMS? |  |  | Toolbox meetings, SWMS sign off, job observations and supervision review. If issues with the SWMS or new hazards are identified, the supervisor must be notified. When changes are made to SWMS, it will be communicated to all workers. |  |  |  |
| Person responsible for approving and reviewing SWMS control measures: |  |  |  |  | Date SWMS received by reviewer: |  |
| How will the SWMS control measures be reviewed? |  |  | The control measures implemented will be reviewed and if necessary, revised annually or if work methods change, the control measures are not effective in controlling the risk, a new hazard/risk is identified or following an incident. The SWMS will be reviewed in consultation with workers and/or others who may be affected by the SWMS. Any changes to the SWMS will be communicated with workers at induction, daily pre-starts and toolbox talks. |  |  |  |
| Review Date: |  |  |  |  | Reviewer’s signature: |  |

| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| --- | --- | --- | --- | --- | --- |
| Planning for installation of signals and associated equipment | Untrained or unlicensed workers may access site | Medium (3) | All workers must have Construction Induction Certificate. All workers must complete site induction and sign the site induction form before proceeding with work. Ensure all relevant documentation is in site folder and correct before leaving depot. All workers to be trained in and sign safe work method statement. All workers to be qualified/ licensed/ ticketed/ trained as required for activities performed. Competent person to supervise all new workers. Safety Meeting to be conducted prior to commencement of works All workers to be in a fit state to commence work, zero blood alcohol concentration, not under influence of drugs and not fatigued. Traffic Control must be implemented on site before works commence. All signage to be in accordance with the traffic control plan Ensure all permits and licences are in place before works commence Ensure there is a copy of the TCAWS Manual in the vehicle at all times. Traffic Controllers performing Stop/Slow duties shall be relieved after 2 hours if continuous work and may be either rested or placed on other duties for a period of at least 15 minutes before being returned to Stop/Slow duties (RMS TCAWS Manual Section 8.1.1) Do not use mobile phones while performing traffic control duties. | Low (1) | Workers and Site Supervisor |
|  | Inappropriate tool and equipment allocation | Medium (3) | When setting up of signage use arrow board (vehicle mounted) directing traffic away from vehicle. Management to ensure that adequate workers numbers are allocated. Management to ensure that adequate PPE is available. Management to ensure that appropriate equipment and procedures are used for the works being completed. Workers to ensure electrical devices are checked prior to commencing work and a current test tag is attached to the device. Ensure adequate first aid kits and amenities are available. Ensure appropriate signage is allocated for the works Ensure all lights and arrow boards are working correctly. Workers are to have clear radio contact at all times. | Low (1) | Workers and Site Supervisor |
| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| Load Equipment into vehicles | Manual handling | Medium (4) | Always use correct manual handling techniques when lifting: Use mechanical lifting devices and trolleys where possible Only lift objects within your safe lifting limit (use team lifts where needed) Clear a path, assess the load, get a good grip, ensure a wide stance, bend the knees, keep the spine straight, use the legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Workers |
|  | Collisions causing injury or death to other drivers, pedestrians and Workers | High (6) | Workers to follow RMS road safety rules and speed limits at all times. Management to ensure 10 hour break within any period of 24hours. Drivers not to exceed 12 hours of driving for every 24 hours. All employees must have 1 × 24hours continuous rest period per week. Follow Vehicle movement plan and traffic management plan on site. | Low (2) | Workers |
| Accessing the site | Slips, trips and falls | Medium (3) | All Workers to wear approved rubber sole lace up work boots. Identify any slip trip and fall hazards. All Workers to regularly clean up waste to minimise trip hazards. Competent person to supervise all new workers. High visibility safety vests, hardhat and safety glasses must be worn. Ensure adequate lighting is in place (Day makers). All vehicles to be fitted with flashing amber light, fire extinguishers, first aids kits, SDS, spill kit | Low (1) | Workers and Supervisor |
|  | Interaction with public | Medium (3) | Workers not to approach public or intoxicated persons. Site to be secure at all times. Ensure emergency phones can be accessed if needed | Low (1) | Workers and Site Supervisor |
| Unload materials and equipment from vehicle and position in work area | Manual handling | Medium (4) | Always use correct manual handling techniques when lifting: Use mechanical lifting devices and trolleys where possible Only lift objects within your safe lifting limit (use team lifts where needed) Clear a path, assess the load, get a good grip, ensure a wide stance, bend the knees, keep the spine straight, use the legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Workers |
|  | Inappropriate storage of materials | Medium (3) | Workers to discuss material storage area and work areas with site controller prior to commencement. Appropriate work and storage areas to be barricaded off. | Low (1) | Workers and Site Supervisor |
| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| Set up Signage | Inappropriate tool and equipment allocation | Medium (3) | Traffic controllers to always have a safe escape route. Using the authorised Traffic Control Plan (TCP) set out the signage in the correct sequence in accordance with Section 3 RMS Traffic Control at Worksites Manual. 40km/h speed zones must be installed where workers are within 1.2 metres of moving traffic. Ensure all signage is the correct size and is visible to motorists and not obstructed. Ensure all existing speed signs are covered when setting up traffic control signage. Ensure the sign is weighted down at all times. Check the correct distance between signage suing the trip meter on vehicle. | Low (1) | Workers |
|  | Manual handling | Medium (4) | Always use correct manual handling techniques when lifting: Use mechanical lifting devices and trolleys where possible Only lift objects within your safe lifting limit (use team lifts where needed) Clear a path, assess the load, get a good grip, ensure a wide stance, bend the knees, keep the spine straight, use the legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Workers |
|  | Workers being struck by vehicles causing serious injury or death | High (9) | Work on closed road where possible. Traffic Control must be implemented on site before works commence. Workers to be wearing high visibility clothing including RMS approved white overalls with reflective tape at night. Spotters to be in place to observe oncoming vehicles Operate in quiet times where possible Traffic controllers to redirect vehicles where necessary Use day makers for night work. All vehicles to observe pedestrians and workers Wear gloves at all times Avoid space-limited areas where possible. Works to stay as far away from vehicles as possible Ensure all signage and equipment is secured. | Low (2) | Workers and Site supervisor |
| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| Control and Management of Site | Workers being struck by vehicles causing serious injury or death | High (9) | Traffic controllers to stand side on to traffic for 180-degree visual sight of traffic. Workers are to never turn their back on the traffic. Workers are to watch driveway movements at all times. Workers are not to cross highways. Work on closed road where possible. Traffic Control must be implemented on site before works commence. Spotters to be in place to observe oncoming vehicles Operate in quiet times where possible Traffic controllers to redirect vehicles where necessary All vehicles to observe pedestrians and workers Wear gloves at all times Avoid space-limited areas where possible. Works to stay as far away from vehicles as possible Ensure all signage and equipment is secured. Ensure a minimum of 1.5 metres distance or use a spotter with hand signals to divers to slow down. Ensure all existing speed signs are covered. In adverse weather conditions increase sign distances and or repeater signs. Ensure all workers get their required breaks when performing stop/slow duties, all breaks are to be recorded and signed off with times. | Low (2) | Workers and Site supervisor |
| Pack up and remove all waste materials and equipment from site | Inappropriate use of equipment | Medium (3) | Remove all equipment from site. Follow manual handling principles above. Check for any hazards and remove where necessary Site to be left safe Ensure spotter is observing traffic Wear gloves when handling signs Ensure all signage is secured. | Low (1) | Workers |
| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| Pack up and remove all waste materials and equipment from site | Manual handling | Medium (4) | Always use correct manual handling techniques when lifting: Use mechanical lifting devices and trolleys where possible Only lift objects within your safe lifting limit (use team lifts where needed) Clear a path, assess the load, get a good grip, ensure a wide stance, bend the knees, keep the spine straight, use the legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Workers |
|  | Inappropriate use of equipment | Medium (3) | When packing up of signage use arrow boards (vehicle mounted) directing traffic away from vehicle. Remove all equipment from site. Follow manual handling principles above. Check for any hazards and remove where necessary Site to be left safe Ensure spotter is observing traffic Wear gloves when handling signs Ensure all signage is secured. | Low (1) | Workers |

| Personal Protective Equipment (PPE) required: | Enclosed leather steel capped boots (As per AS/NZS 2210) Safety Glasses (As per AS/NZS 1336) Leather Gloves (As per AS/NZS 2161) White safety helmet, traffic controller reflective vest with identification, long sleeve shirts and long pants, Night works must wear white overalls with reflective tape. Night wand |
| --- | --- |
| List the training required by workers to complete the work: List the qualifications of personnel doing the work: | Construction induction certificate (White card), Traffic Controller (Blue Card) Select and Modify Traffic Control Plans (Red Card) Drivers Licences (NSW) Road Occupation Licence Direction to Restrict |
| List the Permits, Certificates, SafeWork NSW Approvals, required to complete the work: | Council Permits Police Permits |
| Relevant legislation: For further details refer to the Legal Requirements Register | Work in accordance with Work Health and Safety Act 2011 and Work Health and Safety Regulation 2017, workplace injury management and workers compensation act 1998 no. 86, AS/NZS1337.1 Personal eye protection-Eye and face protectors for occupational applications (2010), AS/NZS 1269.3 Occupational Noise management-Hearing protection program (2005), AS/NZS 1801 industrial safety helmets (1997), AS/NZS 2210.1 Safety, protective and occupational footwear-Guide to selection, care and use (2010) & AS/NZS 2210.2 to 2210.9 Occupational protective footwear, AS 1742.1-2003 Manual of uniform traffic control devices, AS 1742.2, AS 1742.3, AS1742.4, AS1742.7 - AS1742.14. SafeWork NSW Codes of Practice: - Construction Work - Managing risks of plant - Managing the risk of falls - Work Health and Safety Consultation, Co-operation and Co-ordination - Managing the Work Environment and Facilities - First Aid in the Workplace - Hazardous Manual Tasks - How to Manage Work Health and Safety Risks |
| List the plant / equipment that will be used on site: | Work Vehicles Associated Signs and Devices |
| List maintenance checks for plant & equipment: | Pre-start inspection and regular servicing as per manufacturers instructions |
| Frequency of site/workplace inspections: Site Inspections will be completed by: | Daily site inspections to be completed by Site Supervisor and at completion of job |
| Hazardous Substances: |  |

# Sign Off and Approvals

| Person responsible for site supervision of the work, inspecting and approving work areas, approval of and compliance with SWMS, protective measures, plant, equipment: |  |
| --- | --- |
| Name and position: | Signature: |
| SWMS Prepared by: Ben Knox – Compliance OH&S | Signature: |

| Risk Level | Description of consequence or impact | Consequence | Likelihood/Probability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Unlikely (1) | Possible (2) | Almost Certain (3) |
| High Level of harm | Actual/Potential fatality, disability or irreversible damage. Major structural failure/damage. Off-site environmental discharge/release not contained and significant long-term environmental harm. | Major (3) | Medium (3) | High (6) | High (9) |
| Medium Level of harm | Actual/Potential temporary disability, MTI or LTI. Structural failure/damage, >1-day outage. On-site environmental discharge/release contained, minor remediation, short-term environmental harm. | Moderate (2) | Low (2) | Medium (4) | High (6) |
| Low Level of harm | Incident that has the potential to cause persons to require first aid. Environmental discharge/release immediately contained, minor level clean-up with no short term environmental harm. | Minor (1) | Low (1) | Low (2) | Medium (3) |
| Level | Likelihood/Probability |  |  |  |  |
| Almost Certain | Occurs frequently; >66% chance of occurring |  |  |  |  |
| Possible | Could happen occasionally; >33% but <66% chance of occurring |  |  |  |  |
| Unlikely | May occur only in exceptional circumstances; <33% chance of occurring |  |  |  |  |
| Class/Ranking | Description/Requirements |  |  |  |  |
| High 6, 9 | Stop immediately. Implement permanent preventative controls. |  |  |  |  |
| Medium 3, 4 | Will require operational planning. Actions will be recorded on a SWMS. |  |  |  |  |
| Low 1, 2 | Will require detailed pre-planning. Actions will be recorded on a SWMS. |  |  |  |  |

SWMS has been developed using the hierarchy of controls to reduce the risk and likely hood of an incident/injury occurring. The use of Elimination, Substitution, Engineering/isolation, Administrative and PPE measures will be used in the above order of preference wherever practicable.

# Induction confirmation:

This SWMS has been developed in consultation with the workers and has been read and signed by all workers involved in this activity. I note that I have read and understood the above safe work method statement and will ensure work is completed accordingly.

| Date: | Name: | Signature: |
| --- | --- | --- |