| - NOTE: Work must be performed in accordance with this SWMS. - This SWMS must be kept and be available for inspection until the high-risk construction work to which this SWMS relates is completed. If the SWMS is revised, all versions should be kept. If a notifiable incident occurs in relation to the high-risk construction work in this SWMS, the SWMS must be kept for at least 2 years from the date of the notifiable incident. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| - PCBU Name & Contact Details: - Robertson’s Remedial and Painting Pty Ltd # Unit 10/56 Buffalo Road Gladesville NSW 2111 Phone: (02) 9181 3519 A.B.N 16 140 746 247 |  |  |  |  | - Principal Contractor (PC): |  |
| - Work Manager Contact Number: |  | - Dave Oste - 0476 265 066 |  |  | - Date SWMS provided to PC: | - 06-02-2026 |
| - Work Activity | - Install Balustrades |  |  |  | - Workplace Location: | 6a Francis Street, Bondi Beach |
| High Risk Construction Work: | Risk of person falling more than 2 metres (note: in some jurisdictions, this is 3metres) |  |  | Work on a telecommunication tower |  | Demolition of a load-bearing structure |
|  | Likely to involve disturbing asbestos |  |  | Temporary load-bearing support for structural alterations or repairs |  | Work in or near a confined space |
|  | Work in or near a shaft or trench deeper than 1.5m or a tunnel |  |  | Work in an area that may have a contaminated or flammable atmosphere. |  | Work on or near pressurised gas mains or piping |
|  | Work on or near chemical, fuel or refrigerant lines |  |  | Work on or near energised electrical installations |  | Use of explosives |
|  | Diving Work |  |  | Work on, in or adjacent to a road, railway, shipping lane or other traffic corridor in use by traffic other than pedestrians |  | Work in an area with movement of powered mobile plant. |
|  | Work in areas with artificial extremes of temperature |  |  | Work in or near water or other liquid that involves a risk of drowning |  | Tilt-up or precast concrete elements |
| Name of workers consulted in the development of this SWMS: |  |  |  |  | Signature: |  |
| Person responsible for ensuring compliance with SWMS: |  |  |  |  | Date SWMS received: |  |
| What measures are in place to ensure compliance with the SWMS? |  |  | Toolbox meetings, SWMS sign off, job observations and supervision review. If issues with the SWMS or new hazards are identified, the supervisor must be notified. When changes are made to SWMS, it will be communicated to all workers. |  |  |  |
| Person responsible for approving and reviewing SWMS control measures: |  |  |  |  | Date SWMS received by reviewer: |  |
| How will the SWMS control measures be reviewed? |  |  | The control measures implemented will be reviewed and if necessary, revised annually or if work methods change, the control measures are not effective in controlling the risk, a new hazard/risk is identified or following an incident. The SWMS will be reviewed in consultation with workers and/or others who may be affected by the SWMS. Any changes to the SWMS will be communicated with workers at induction, daily pre-starts and toolbox talks. |  |  |  |
| Review Date: |  |  |  |  | Reviewer’s signature: |  |

| Procedure | Possible Hazards: | Risk Rating | Safety Controls | Residual Risk | Responsible Person |
| --- | --- | --- | --- | --- | --- |
| Planning for Balustrade installation works | Untrained or unlicensed workers may access site | Medium (3) | All workers must have Construction Induction Certificate. All workers must complete site induction upon arrival to site and sign in. Where induction is not available workers to liaise with site controller about site hazards, emergency procedures and any other site requirements. All workers to be trained in and sign safe work method statement. All workers to be qualified/ licensed/ ticketed/ trained as required for activities performed Competent person to supervise all new workers. Pre-start meeting to be conducted prior to commencement of works All workers to be in a fit state to commence work, they must be zero blood alcohol concentration and must not be suffering from fatigue or under the influence of drugs. | Low (1) | Workers and Supervisor |
|  | Inappropriate tool and equipment allocation | Medium (3) | Risk Assessment of site to be carried out prior to commencement of work and changes made to SWMS where necessary. Management to ensure that adequate workers numbers are allocated to complete works. Management to ensure that adequate personal protective equipment is available Management to ensure that appropriate equipment and procedures are used for the works being completed. Workers to ensure electrical devices are checked prior to commencing work and a current test tag is attached to the device. Ensure adequate first aid kits and amenities are available. | Low (1) | Workers and Supervisor |
| Accessing the site | Slips, trips and falls | Medium (3) | All Workers to wear approved rubber sole lace up work boots. Onsite inductions to be completed prior to entering the job including identification of any slip trip and fall hazards. All Workers to regularly clean up waste to minimise trip hazards. Competent person to supervise all new workers. High visibility safety vests, hardhat and safety glasses must be worn. | Low (1) | Workers and Supervisor |
|  | Unauthorised individuals entering work site | Medium (3) | Appropriate barricades & signage to be erected to prevent unauthorised access to work area. Spotters to be used to observe personnel and re direct as needed. Alternate access to be provided to public where appropriate. Implement traffic management plan where necessary. | Low (1) | Workers and Supervisor |
| Unload materials and equipment from vehicle and position in work area | Manual handling | Medium (4) | Always use correct manual handling techniques when lifting: Use mechanical lifting devices and trolleys where possible Only lift objects within your safe lifting limit (use team lifts where needed) Clear a path, assess the load, get a good grip, ensure a wide stance, bend the knees, keep the spine straight, use the legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Workers |
|  | Inappropriate storage areas | Medium (3) | Workers to discuss material storage area and work areas with site controller prior to commencement. Appropriate work and storage areas to be fenced off and signposted. | Low (1) | Workers |
| Operating grinder to cut balustrades to length | The grinder or disk may fail causing injury to workers or operators. | Medium (4) | Ensure appropriate cutting disk is used Wheels should be used only for the specific material and purpose for which they are designed, and according to the manufacturer's recommendations. Ensure the correct flange and locking nut is in place. Ensure the guard and handles are secure. Ensure there are no defects or damage to the disc. | Low (2) | Workers |
|  | Power cord damaged. Electrical hazard | High (6) | Prior to use grinder needs to be checked for an electrical test tag to ensure that it is current and has been passed. The power lead and the tool should be checked for damage prior to use. Ensure that the power outlet is not damaged. Do not run power leads across paths or in areas where they may be damaged (e.g run over). Ensure all services are isolated prior to the commencement of grinding. Ensure RCD’s are used for all electrical equipment | Low (2) | Workers |
|  | Fall from heights | High (6) | Implement work platform, physical barrier or physical restraint (In that order) for all work where a worker can fall from one level to another. Workers to follow SWI for control measure selected above such as: Mobile scaffold, scaffold, EWP, etc. Pre-start inspections must be completed on equipment daily. Never work within 3m of an unguarded edge without fall protection in place (e.g. scaffolding or safety harness) Workers to wear a fall restraint harness and lanyard or inertia reel attached to an approved anchor point, Where anchorage points are to be used, they will be to be installed and inspected by a competent person prior to use. | Low (2) | Workers & Supervisor |
| Operating grinder to cut balustrades to length | Grinding disk or grinder may come into contact with worker causing serious injury (kick back). | Medium (4) | Stay alert, watch what you are doing and use common sense when grinding. Do not use while tired or under the influence of drugs, alcohol, or medication. Hold grinder with two hands while operating. Allow the grinder to "run up" to operating speed before applying it to the job. Hold grinder against work piece with minimum pressure, so it doesn't "dig in" and cause kick back. Never bump grinder on to job, or let disc hit another object while grinding. Adopt a comfortable stance, with feet apart and well balanced, and with a clear view of the job. Never use a grinder between the legs. Never put a grinder down until the disc stops rotating. Remove the plug from the power point or the air hose before changing discs. | Low (2) | Workers |
|  | Manual handling | Medium (4) | Always use correct manual handling techniques when lifting: Use mechanical lifting devices and trolleys where possible Only lift objects within your safe lifting limit (use team lifts where needed) Clear a path, assess the load, get a good grip, ensure a wide stance, bend the knees, keep the spine straight, use the legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Workers |
|  | Dust inhalation | Medium (3) | Wear dust mask or respirator (As per AS 1716) when sanding or grinding. Cover all areas with plastic or appropriate sheeting. Use power tools with dust extraction where possible. Open windows and use dust extraction devices where necessary. | Low (1) | Workers |
| Drill holes into concrete for anchor | Cuts and abrasions | Medium (3) | Ensure correct size socket or ring spanner is used when tightening bolts. Workers to wear gloves. | Low (1) | Workers |
| Anchor components to floor | Dust inhalation | Medium (3) | Wear dust mask or respirator (As per AS/NZS 1716). Use power tools with dust extraction where possible. Open windows and use dust extraction devices where necessary. | Low (1) | Workers |
| Anchor components to floor | Fall from heights | High (6) | Implement work platform, physical barrier or physical restraint (In that order) for all work where a worker can fall from one level to another. Workers to follow SWI for control measure selected above such as: Mobile scaffold, scaffold, EWP, etc. Pre-start inspections must be completed on equipment daily. Never work within 3m of an unguarded edge without fall protection in place (e.g. scaffolding or safety harness) Workers to wear a fall restraint harness and lanyard or inertia reel attached to an approved anchor point, Where anchorage points are to be used, they will be to be installed and inspected by a competent person prior to use. | Low (2) | Workers & Supervisor |
|  | Dust in eyes | Medium (3) | Wear goggles or safety glasses when sanding grinding or drilling. Avoid sanding, grinding or drilling above head height. | Low (1) | Workers |
|  | Hearing Damage | Medium (3) | Hearing protection to be worn while hammering bolts in | Low (1) | Workers |
|  | Cuts and abrasions | Medium (3) | Ensure correct size socket or ring spanner is used when tightening bolts. Workers to wear gloves. | Low (1) | Workers |
| Clean-up site | Housekeeping | Medium (3) | All tools and equipment to be secured at all times. Secure off work area below and above. Clean-up site and ensure it is left in safe, clean and tidy condition. | Low (1) | Workers |

| Personal Protective Equipment (PPE) required: | Enclosed leather steel capped boots Safety Glasses (As per AS/NZS 1337) Hearing Protection Protective Gloves (As per AS/NZS 2161) | High visibility Long sleeve shirt & pants Hard Hats (As per AS/NZS 1801) Sun hats, sun cream - when working outdoors Dust Mask / Respirator |
| --- | --- | --- |
| List the training required by workers to complete the work: List the qualifications of personnel doing the work: | Construction Induction Card Relevant trade certificate Induction into SWMS |  |
| List the Permits, Certificates, SafeWork NSW Approvals, required to completed the work: | All necessary permits to be completed on site |  |
| Relevant legislation: For further details refer to the Legal Requirements Register | Work in accordance with Work Health and Safety Act 2011 and Work Health and Safety Regulation 2025, workplace injury management and workers compensation act 1998 no. 86, AS/NZS1337.1 Personal eye protection-Eye and face protectors for occupational applications, AS/NZS 1269.3 Occupational Noise management-Hearing protection program, AS/NZS 1801 industrial safety helmets, AS/NZS 2210.1 Safety, protective and occupational footwear-Guide to selection, care and use & AS/NZS 2210.2 to 2210.9 Occupational protective footwear. SafeWork NSW Codes of Practice: - Construction Work - Managing risks of plant - Managing the risk of falls - Work Health and Safety Consultation, Co-operation and Co-ordination - Managing the Work Environment and Facilities - First Aid in the Workplace - Hazardous Manual Tasks - How to Manage Work Health and Safety Risks |  |
| List the plant / equipment that will be used on site: | Harness, EWP, Scaffold, Grinder and drill |  |
| List maintenance checks for plant & equipment: | Daily pre-start checks, maintenance as per manufacturers guidelines |  |
| Frequency of site/workplace inspections: Site Inspections will be completed by: | Daily site inspections & job completion inspections to be completed by Site Supervisor. |  |
| Hazardous Substances: | Fuel |  |

# Sign Off and Approvals

| Person responsible for site supervision of the work, inspecting and approving work areas, approval of and compliance with SWMS, protective measures, plant, equipment: |  |
| --- | --- |
| Name and position: | Signature: |
| SWMS Prepared by: Ben Knox – Compliance OH&S | Signature: |

| Risk Level | Description of consequence or impact | Consequence | Likelihood/Probability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Unlikely (1) | Possible (2) | Almost Certain (3) |
| High Level of harm | Actual/Potential fatality, disability or irreversible damage. Major structural failure/damage. Off-site environmental discharge/release not contained and significant long-term environmental harm. | Major (3) | Medium (3) | High (6) | High (9) |
| Medium Level of harm | Actual/Potential temporary disability, MTI or LTI. Structural failure/damage, >1-day outage. On-site environmental discharge/release contained, minor remediation, short-term environmental harm. | Moderate (2) | Low (2) | Medium (4) | High (6) |
| Low Level of harm | Incident that has the potential to cause persons to require first aid. Environmental discharge/release immediately contained, minor level clean-up with no short term environmental harm. | Minor (1) | Low (1) | Low (2) | Medium (3) |
| Level | Likelihood/Probability |  |  |  |  |
| Almost Certain | Occurs frequently; >66% chance of occurring |  |  |  |  |
| Possible | Could happen occasionally; >33% but <66% chance of occurring |  |  |  |  |
| Unlikely | May occur only in exceptional circumstances; <33% chance of occurring |  |  |  |  |
| Class/Ranking | Description/Requirements |  |  |  |  |
| High 6, 9 | Stop immediately. Implement permanent preventative controls. |  |  |  |  |
| Medium 3, 4 | Will require operational planning. Actions will be recorded on a SWMS. |  |  |  |  |
| Low 1, 2 | Will require detailed pre-planning. Actions will be recorded on a SWMS. |  |  |  |  |

SWMS has been developed using the hierarchy of controls to reduce the risk and likelihood of an incident/injury occurring. The use of Elimination, Substitution, Engineering/isolation, Administrative and PPE measures will be used in the above order of preference wherever practicable.

# Induction confirmation:

This SWMS has been developed in consultation with the workers and has been read and signed by all workers involved in this activity. I note that I have read and understood the above safe work method statement and will ensure work is completed accordingly.

| Date: | Name: | Signature: |
| --- | --- | --- |
| Date: | Name: | Signature: |