| - NOTE: Work must be performed in accordance with this SWMS. - This SWMS must be kept and be available for inspection until the high-risk construction work to which this SWMS relates is completed. If the SWMS is revised, all versions should be kept. If a notifiable incident occurs in relation to the high-risk construction work in this SWMS, the SWMS must be kept for at least 2 years from the date of the notifiable incident. |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| - PCBU Name & Contact Details: - Robertson’s Remedial and Painting Pty Ltd # Unit 10/56 Buffalo Road Gladesville NSW 2111 Phone: (02) 9181 3519 A.B.N 16 140 746 247 |  |  |  |  | - Principal Contractor (PC): |  |  |
| - Work Manager Contact Number: |  |  |  |  | - Date SWMS provided to PC: |  |  |
| - Work Activity | - Entry into and working within a confined space |  |  |  | - Workplace Location: |  |  |
| High Risk Construction Work: | Risk of person falling more than 2 metres (note: in some jurisdictions, this is 3metres) |  |  | Work on a telecommunication tower |  | Demolition of a load-bearing structure |  |
|  | Likely to involve disturbing asbestos |  |  | Temporary load-bearing support for structural alterations or repairs |  | Work in or near a confined space |  |
|  | Work in or near a shaft or trench deeper than 1.5m or a tunnel |  |  | Work in an area that may have a contaminated or flammable atmosphere. |  | Work on or near pressurised gas mains or piping |  |
|  | Work on or near chemical, fuel or refrigerant lines |  |  | Work on or near energised electrical installations |  | Use of explosives |  |
|  | Diving Work |  |  | Work on, in or adjacent to a road, railway, shipping lane or other traffic corridor in use by traffic other than pedestrians |  | Work in an area with movement of powered mobile plant. |  |
|  | Work in areas with artificial extremes of temperature |  |  | Work in or near water or other liquid that involves a risk of drowning |  | Tilt-up or precast concrete elements |  |
| Name of workers consulted in the development of this SWMS: |  |  |  |  | Signature: |  |  |
| Person responsible for ensuring compliance with SWMS: |  |  |  |  | Date SWMS received: |  |  |
| What measures are in place to ensure compliance with the SWMS? |  |  | Toolbox meetings, SWMS sign off, job observations and supervision review. If issues with the SWMS or new hazards are identified, the supervisor must be notified. When changes are made to SWMS, it will be communicated to all workers. |  |  |  |  |
| Person responsible for approving and reviewing SWMS control measures: |  |  |  |  | Date SWMS received by reviewer: |  |  |
| How will the SWMS control measures be reviewed? |  |  | The control measures implemented will be reviewed and if necessary, revised annually or if work methods change, the control measures are not effective in controlling the risk, a new hazard/risk is identified or following an incident. The SWMS will be reviewed in consultation with workers and/or others who may be affected by the SWMS. Any changes to the SWMS will be communicated with workers at induction, daily pre-starts and toolbox talks. |  |  |  |  |
| Review Date: |  |  |  |  | Reviewer’s signature: |  |  |

| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| --- | --- | --- | --- | --- | --- |
| Planning for confined space works | Untrained or unlicensed workers may access site/confined space | Medium (3) | All workers must have General Construction Induction Training. All workers must complete site induction and sign in / out. All workers to be trained in and sign SWMS. All workers to be qualified/ licensed/ ticketed/ trained as required for activities performed. Workers to have completed confined spaces training when working within confined spaces. Competent person to supervise all new workers and confined space activity personnel. Pre-start meeting to be conducted prior to commencement of works. All workers to be in a fit state to commence work, zero blood alcohol concentration, not under the influence of drugs or be fatigue. | Low (1) | Workers and Supervisor |
|  | Inappropriate resources for job activity e.g. people, tools and equipment | Medium (3) | Risk Assessment of site to be carried out prior to commencement of work and changes made to SWMS where necessary. Emergency & rescue procedures must be planned, workers trained and aware, and tested prior to commencing work. Confined Space Entry Permit completed and approved. Management to ensure that adequate workers are allocated to complete works (workers and stand-by person). All required PPE is available, checked and fit for purpose. Management to ensure that appropriate equipment and procedures are used for the works and environment. Pre-start inspections to be completed on all equipment and any defects or damages notified to the supervisor. Defective or damaged equipment will be removed from service until it has been repaired or replaced. Workers to ensure electrical devices are checked prior to commencing work and a current test tag is attached to the device (if electrical tools are used). Ensure adequate first aid kits, fire extinguisher and specific amenities are available. | Low (1) | Workers and Supervisor |
| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| Accessing the site | Slips, trips and falls | Medium (3) | All workers to wear approved rubber sole work lace up boots as per AS 2210 and any additional PPE as required by the SDS. Onsite inductions to be completed prior to entering the job including identification of any slip trip and fall hazards. All workers to be aware of risk management procedures to ensure they can identify, assess and control all trip hazards. Competent person to supervise all new workers. Workers not to access areas where risk of slip trip or fall is present without appropriate protection in place. | Low (1) | Workers and supervisor |
|  | UV Radiation High Temperatures | Medium (3) | Workers must wear appropriate PPE including: sun cream, sun hats, long sleeve shirts and long pants while working outside. Workers must remain hydrated through regularly drinking water. Workers to stay out of sun where possible and implement rest breaks or job rotation where necessary. Workers must not work in extreme heat. Temperature of confined space must be documented and monitored to ensure space is not overheated for individuals working within. | Low (1) | Workers |
| Secure job site | Unauthorised individuals entering work site | Medium (3) | Appropriate barricading & signage to be erected to prevent unauthorised access to the confined space. Display signs must be prominent and near the confined space entries. Spotters to be used to observe and re-direct personnel. Alternate access to be provided to public where appropriate. | Low (1) | Workers |
| Isolate Services | Stored energy release, Energised machinery or services, Inadvertent use of electrical equipment | High (6) | All potentially hazardous services must be isolated prior to any person entering the confined space. Each isolation point must be checked or supervised and not removed until all work is complete and workers have left the space. | Low (2) | Competent person |
| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| Entering the confined space and perform tasks | Confined spaces entry | High (9) | Confined spaces entry permit to be obtained prior to entry. Appoint workers and stand by personnel for duration of activity. Worker & stand by person must have confined spaces & first aid training. Before entering the confined space, ensure all PPE and rescue equipment has been checked for damage. Ensure an emergency and rescue plan has been developed, workers are trained in its use and a practice run has been trialled. Stand by person must constantly monitor and communicate with the worker from outside the confined space. Two-way radios will be used. Fixed ladders, platforms should be provided for safe access/egress. Stand by person to ensure no unauthorised personnel enter the confined space. First aid and emergency procedures to be in place and practiced ensuring that they are effective in assisting a worker in an emergency. | Medium (3) | Workers |
| Enter the confined space and perform tasks | Exposure to airborne contaminants, Low oxygen levels | High (9) | Consult with principal contractor or others to determine what contaminant was previously in the space. Cleaning, purging and ventilation will be required to be undertaken to ensure of a safe atmosphere. Air monitoring to be conducted inside confined space to determine the airborne concentration and oxygen content. Use gas detector and record results/readings before entering the confined space and monitor regularly. Respirators to be worn by workers where required. | Medium (3) | Workers |
|  | Flammable atmospheres with the potential to ignite causing explosions | High (9) | Consult with principal contractor to determine if there were any flammable materials/liquids previously used in the structure. Air monitoring to be conducted by a competent person inside the space to determine the concentration levels of any flammable gas or vapour. Ensure that the concentration levels are below 5% of the lower explosive limit of the flammable materials/liquids. Ensure that the structure is thoroughly cleaned prior to accessing. Remove all ignition sources. All work to be carried out using non-sparking tools, i.e. needle gun with copper beryllium needles, brass wire brushes. Where air-monitoring results indicate high levels of flammable gases or vapours, purging to be undertaken. Ongoing air monitoring to be undertaken during the course of work. If electric hand tools are needed for the works, a hot works permit is to be issued before any work commences. | Medium (3) | Workers |
| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| Possible Emergency situations | Untrained workers may not understand how to manage emergencies | High (6) | Workers to understand and be trained in all emergency procedures. Emergency drills to be carried out every six months. All workers must follow the site emergency procedures. Inform the supervisor immediately of any emergency and they will raise the alarm if necessary. Radio or phone contact to be made to the site office/control room for all emergency situations or for first aid assistance. | Low (2) | Workers |
| Clean-up site Pack up materials and tools | Injury to personnel | Medium (3) | Ensure site is secure from unauthorised access if works not completed. Remove all waste materials from site and dispose of appropriately. Remove all tools and product or store in secure location. | Low (1) | Workers |

| Personal Protective Equipment (PPE) required: | Enclosed leather steel capped boots Safety Glasses (As per AS/NZS 1337) Hearing Protection Protective Gloves (As per AS/NZS 2161) | High visibility Long sleeve shirt & pants Hard Hats (As per AS/NZS 1801) Sun hats, sun cream - when working outdoors Dust Mask / Respirator |
| --- | --- | --- |
| List the training required by workers to complete the work: List the qualifications of personnel doing the work: | Construction Induction Card Relevant trade certificate | Induction into SWMS Confined Space Training |
| List the Permits, Certificates, SafeWork NSW Approvals, required to complete the work: | Confined Spaces Entry Permit |  |
| Relevant legislation: For further details refer to the Legal Requirements Register | Work in accordance with Work Health and Safety Act 2011 and Work Health and Safety Regulation 2017, workplace injury management and workers compensation act 1998 no. 86, AS/NZS1337.1 Personal eye protection-Eye and face protectors for occupational applications (2010), AS/NZS 1269.3 Occupational Noise management-Hearing protection program (2005), AS/NZS 1801 industrial safety helmets (1997), AS/NZS 2210.1 Safety, protective and occupational footwear-Guide to selection, care and use (2010) & AS/NZS 2210.2 to 2210.9 Occupational protective footwear, AS 2865 Confine Spaces (2009) SafeWork NSW Codes of Practice: - Construction Work - Managing risks of plant - Managing the risk of falls - Work Health and Safety Consultation, Co-operation and Co-ordination - Managing the Work Environment and Facilities - First Aid in the Workplace - Hazardous Manual Tasks - How to Manage Work Health and Safety Risks - Confine Spaces |  |
| List the plant / equipment that will be used on site: |  |  |
| List maintenance checks for plant & equipment: | Daily pre-start checks, maintenance as per manufacturers guidelines |  |
| Frequency of site/workplace inspections: Site Inspections will be completed by: | Daily site inspections & job completion inspections to be completed by Site Supervisor. Air monitoring results every hour |  |
| Hazardous Substances: |  |  |

# Sign Off and Approvals

| Person responsible for site supervision of the work, inspecting and approving work areas, approval of and compliance with SWMS, protective measures, plant, equipment: |  |
| --- | --- |
| Name and position: | Signature: |
| SWMS Prepared by: Ben Knox – Compliance OH&S | Signature: |

| Risk Level | Description of consequence or impact | Consequence | Likelihood/Probability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Unlikely (1) | Possible (2) | Almost Certain (3) |
| High Level of harm | Actual/Potential fatality, disability or irreversible damage. Major structural failure/damage. Off-site environmental discharge/release not contained and significant long-term environmental harm. | Major (3) | Medium (3) | High (6) | High (9) |
| Medium Level of harm | Actual/Potential temporary disability, MTI or LTI. Structural failure/damage, >1-day outage. On-site environmental discharge/release contained, minor remediation, short-term environmental harm. | Moderate (2) | Low (2) | Medium (4) | High (6) |
| Low Level of harm | Incident that has the potential to cause persons to require first aid. Environmental discharge/release immediately contained, minor level clean-up with no short term environmental harm. | Minor (1) | Low (1) | Low (2) | Medium (3) |
| Level | Likelihood/Probability |  |  |  |  |
| Almost Certain | Occurs frequently; >66% chance of occurring |  |  |  |  |
| Possible | Could happen occasionally; >33% but <66% chance of occurring |  |  |  |  |
| Unlikely | May occur only in exceptional circumstances; <33% chance of occurring |  |  |  |  |
| Class/Ranking | Description/Requirements |  |  |  |  |
| High 6, 9 | Stop immediately. Implement permanent preventative controls. |  |  |  |  |
| Medium 3, 4 | Will require operational planning. Actions will be recorded on a SWMS. |  |  |  |  |
| Low 1, 2 | Will require detailed pre-planning. Actions will be recorded on a SWMS. |  |  |  |  |

SWMS has been developed using the hierarchy of controls to reduce the risk and likely hood of an incident/injury occurring. The use of Elimination, Substitution, Engineering/isolation, Administrative and PPE measures will be used in the above order of preference wherever practicable.

# Induction confirmation:

This SWMS has been developed in consultation with the workers and has been read and signed by all workers involved in this activity. I note that I have read and understood the above safe work method statement and will ensure work is completed accordingly.

| Date: | Name: | Signature: |
| --- | --- | --- |

| Date: | Name: | Signature: |
| --- | --- | --- |