| NOTE: Work must be performed in accordance with this SWMS. This SWMS must be kept and be available for inspection until the high-risk construction work to which this SWMS relates is completed. If the SWMS is revised, all versions should be kept. If a notifiable incident occurs in relation to the high-risk construction work in this SWMS, the SWMS must be kept for at least 2 years from the date of the notifiable incident. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| PCBU Name & Contact Details: Robertson’s Remedial and Painting Pty Ltd # Unit 10/56 Buffalo Road Gladesville NSW 2111 Phone: (02) 9181 3519 A.B.N 16 140 746 247 |  |  |  |  | Principal Contractor (PC): |  |
| Work Manager Contact Number: |  |  |  |  | Date SWMS provided to PC: |  |
| Work Activity | Installation of Doors |  |  |  | Workplace Location: |  |
| High Risk Construction Work: | Risk of person falling more than 2 metres (note: in some jurisdictions, this is 3metres) |  |  | Work on a telecommunication tower |  | Demolition of a load-bearing structure |
|  | Likely to involve disturbing asbestos |  |  | Temporary load-bearing support for structural alterations or repairs |  | Work in or near a confined space |
|  | Work in or near a shaft or trench deeper than 1.5m or a tunnel |  |  | Work in an area that may have a contaminated or flammable atmosphere. |  | Work on or near pressurised gas mains or piping |
|  | Work on or near chemical, fuel or refrigerant lines |  |  | Work on or near energised electrical installations |  | Use of explosives |
|  | Diving Work |  |  | Work on, in or adjacent to a road, railway, shipping lane or other traffic corridor in use by traffic other than pedestrians |  | Work in an area with movement of powered mobile plant. |
|  | Work in areas with artificial extremes of temperature |  |  | Work in or near water or other liquid that involves a risk of drowning |  | Tilt-up or precast concrete elements |
| Name of workers consulted in the development of this SWMS: |  |  |  |  | Signature: |  |
| Person responsible for ensuring compliance with SWMS: |  |  |  |  | Date SWMS received: |  |
| What measures are in place to ensure compliance with the SWMS? |  |  | Toolbox meetings, SWMS sign off, job observations and supervision review. If issues with the SWMS or new hazards are identified, the supervisor must be notified. When changes are made to SWMS, it will be communicated to all workers. |  |  |  |
| Person responsible for approving and reviewing SWMS control measures: |  |  |  |  | Date SWMS received by reviewer: |  |
| How will the SWMS control measures be reviewed? |  |  | The control measures implemented will be reviewed and if necessary, revised annually or if work methods change, the control measures are not effective in controlling the risk, a new hazard/risk is identified or following an incident. The SWMS will be reviewed in consultation with workers and/or others who may be affected by the SWMS. Any changes to the SWMS will be communicated with workers at induction, daily pre-starts and toolbox talks. |  |  |  |
| Review Date: |  |  |  |  | Reviewer’s signature: |  |

| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| --- | --- | --- | --- | --- | --- |
| Planning for the setup of job | Untrained or unlicensed workers may access site | Medium (3) | All workers must have Construction Induction Certificate. All workers to be trained as required for the works. All workers must complete site induction and sign in / out. Where induction is not available, liaise with site supervisor about site hazards, emergency procedures and any other site requirements. All workers to ensure they are familiar with the emergency muster points of the site All workers to follow the onsite emergency procedures. Pre-start meeting to be conducted prior to commencement of work. All workers to be in a fit state to commence work, be zero blood alcohol concentration, not suffering from fatigue or under the influence of drugs. Competent person to supervise all new workers. | Low (1) | Workers and Supervisor |
|  | Inappropriate tool and equipment allocation | Medium (3) | Site Supervisor/Management to ensure that appropriate equipment and procedures are used for the works being completed. Site Supervisor /Management to ensure that all electrical devices have an electrical test tag prior to commencement of work. Management to ensure that adequate PPE is available to workers | Low (1) | Workers and Supervisor |
| Accessing and securing the site | Slips, trips and falls | Medium (3) | All workers to wear steel capped boots. Onsite inductions to be completed prior to entering the job including identification of any slip trip and fall hazards. Ensure the work area is clear and free of slip, trip and fall hazards | Low (1) | Workers |
|  | Unauthorised individuals entering work site | Medium (3) | Barricades & signage to be erected to prevent unauthorised access to work area. Where appropriate, spotters to be used to observe personnel and re direct where needed. Alternate access to be provided where appropriate. | Low (1) | Workers |
| Deliver and unload materials to work area | Manual handling | Medium (4) | Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit. Use team lifts where needed. Workers to follow correct manual handling procedures: Clear a path, assess the load, get a good grip, ensure a wide stance, bend the knees, keep the spine straight, use the legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Workers |
|  | Slip, trip and falls | Medium (3) | Ensure the storage location(s) do not block any access or egress paths and protect materials from damage. Apply brakes or chock trolley wheels to ensure trolley cannot inadvertently move / roll. Barriers & signage to be erected to prevent unauthorised access to work area. | Low (1) | Workers |
| Check levels and cut and install frames | Inappropriate use of equipment | Medium (4) | Laser to be mounted just below ceiling level, persons cleared of path of beam and laser switched OFF at all times when not in use. All workers to wear the required PPE; safety glasses, gloves, hearing protection. Stay alert, watch what you are doing when cutting Do not use while tired or under the influence of drugs, alcohol, or medication. Adopt a comfortable stance, with feet apart and well balanced, and with a clear view. Never use a saw between the legs while sitting on the floor. Disconnect the power when not in use. Remove the plug from the power point before performing any services or repairs. Keep hands and body parts clear of saw blade Never cut towards a body part Hold saw with two hands and a firm grip at all times Ensure material being cut stays in contact with the front half of the blade | Low (2) | Workers |
|  | Manual Handling | Medium (4) | Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit. Use team lifts where needed. Workers to follow correct manual handling procedures: Clear a path, assess the load, get a good grip, ensure a wide stance, bend the knees, keep the spine straight, use the legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Workers |
| Installation of door components | Falling Objects | High (6) | Barriers & Signage to be erected around and below work area to prevent unauthorised access to work area. Spotters to be used to observe public and re direct as needed. All tools to be attached by lanyards when working at heights. | Low (2) | Workers |
|  | Fall from ladder | High (6) | Fibreglass ladders to be used. Use a platform ladder. Perform visual check on ladder for obvious faults and defects prior to use. Damaged ladders are to be removed from use, tagged “not for use” and repaired ASAP. Erect ladder on clean, level surface or use self-levelling ladder. Erect extension ladder in a 4:1 ratio (four meters up for every 1 meter from the wall) and tie off at top and bottom. Ensure legs are fully extended and latch is in place Do not place a ladder in front of a doorway Secure work area around the ladder. Clean mud, grass etc. from boots to prevent slippage. Avoid overhead electrical hazards Use a tool pouch to carry tools. Do not leave tools and equipment on ladder rungs. Do not over-reach while on ladder. Only one person on a ladder at any one time. Face the ladder at all times. Maintain three points of contact on ladder Do not step above the third step from the top. | Low (2) | Workers |
|  | Inappropriate use of equipment | Medium (4) | Follow SWI for drilling. All workers to wear the required PPE; safety glasses, gloves, hearing protection. Only competent workers to be used. | Low (2) | Workers |
| Install Support & Glazed Panels & aluminium track | Inappropriate use of equipment | Medium (4) | Follow SWI for drilling. All workers to wear the required PPE; safety glasses, gloves, hearing protection. Only competent workers to be used. | Low (2) | Workers |
| Install Support & Glazed Panels & aluminium track | Manual Handling | Medium (4) | Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit. Use team lifts where needed. Workers to follow correct manual handling procedures: Clear a path, assess the load, get a good grip, ensure a wide stance, bend the knees, keep the spine straight, use the legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Workers |
| Clean-up site | Housekeeping | Medium (3) | All tools to be checked for damage and then returned to appropriate storage position. Immediately remove off cuts from work area and dispose accordingly. Clean-up all mess from area and leave in safe and tidy condition. | Low (1) | Workers |

| Personal Protective Equipment (PPE) required: | Enclosed leather steel capped boots Safety Glasses (As per AS/NZS 1337) Hearing Protection (As per AS/NZS 1269) | Protective Gloves with cut resistance Hard Hats (As per AS/NZS 1801) |
| --- | --- | --- |
| List the training required by workers to complete the work: List the qualifications of personnel doing the work: | Relevant trade certificate Construction induction certificate (White card) Induction into SWMS |  |
| List the Permits, Certificates, SafeWork NSW Approvals, required to complete the work: |  |  |
| Relevant Legislation: For further details refer to the Legal Requirements Register | Work Health and Safety Act 2011 and Work Health and Safety Regulation 2017, Workplace Injury Management and Workers Compensation Act 1998 no. 86, AS/NZS1337.1 Personal eye protection-Eye and face protectors for occupational applications (2010), AS/NZS 1269.3 Occupational Noise management-Hearing protection program (2005), AS/NZS 1801 industrial safety helmets (1997), AS/NZS 2210.1 Safety, protective and occupational footwear-Guide to selection, care and use (2010), AS/NZS 2210.2 to 2210.9 Occupational protective footwear. Safe Work Australia Codes of Practice Construction Work Managing the risk of falls Work Health and Safety Consultation, Co-operation and Co-ordination Managing the Work Environment and Facilities First Aid in the Workplace, Hazardous Manual Tasks How to Manage Work Health and Safety Risks |  |
| List the plant / equipment that will be used on site: List maintenance checks for plant & equipment: | Power tools (e.g. drill, saw), ladder, hand tools Current electrical test tag (for electrical tools), pre-start inspection and regular servicing as per manufacturers recommendations |  |
| Frequency of site/workplace inspections: Site Inspections will be completed by | Daily site inspections to be completed by Site Supervisor |  |
| Hazardous Substances: | Glues |  |

# Sign Off and Approvals

| Person responsible for site supervision of the work, inspecting and approving work areas, approval of and compliance with SWMS, protective measures, plant, equipment: |  |
| --- | --- |
| Name and position: | Signature: |
| SWMS Prepared by: Ben Knox – Compliance OH&S | Signature: |

| Risk Level | Description of consequence or impact | Consequence | Likelihood/Probability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Unlikely (1) | Possible (2) | Almost Certain (3) |
| High Level of harm | Actual/Potential fatality, disability or irreversible damage. Major structural failure/damage. Off-site environmental discharge/release not contained and significant long-term environmental harm. | Major (3) | Medium (3) | High (6) | High (9) |
| Medium Level of harm | Actual/Potential temporary disability, MTI or LTI. Structural failure/damage, >1-day outage. On-site environmental discharge/release contained, minor remediation, short-term environmental harm. | Moderate (2) | Low (2) | Medium (4) | High (6) |
| Low Level of harm | Incident that has the potential to cause persons to require first aid. Environmental discharge/release immediately contained, minor level clean-up with no short term environmental harm. | Minor (1) | Low (1) | Low (2) | Medium (3) |
| Level | Likelihood/Probability |  |  |  |  |
| Almost Certain | Occurs frequently; >66% chance of occurring |  |  |  |  |
| Possible | Could happen occasionally; >33% but <66% chance of occurring |  |  |  |  |
| Unlikely | May occur only in exceptional circumstances; <33% chance of occurring |  |  |  |  |
| Class/Ranking | Description/Requirements |  |  |  |  |
| High 6, 9 | Stop immediately. Implement permanent preventative controls. |  |  |  |  |
| Medium 3, 4 | Will require operational planning. Actions will be recorded on a SWMS. |  |  |  |  |
| Low 1, 2 | Will require detailed pre-planning. Actions will be recorded on a SWMS. |  |  |  |  |

SWMS has been developed using the hierarchy of controls to reduce the risk and likely hood of an incident/injury occurring. The use of Elimination, Substitution, Engineering/isolation, Administrative and PPE measures will be used in the above order of preference wherever practicable.

# Induction confirmation:

This SWMS has been developed in consultation with the workers and has been read and signed by all workers involved in this activity. I note that I have read and understood the above safe work method statement and will ensure work is completed accordingly.

| Date: | Name: | Signature: |
| --- | --- | --- |
| Date: | Name: | Signature: |