| NOTE: Work must be performed in accordance with this SWMS. This SWMS must be kept and be available for inspection until the high-risk construction work to which this SWMS relates is completed. If the SWMS is revised, all versions should be kept. If a notifiable incident occurs in relation to the high-risk construction work in this SWMS, the SWMS must be kept for at least 2 years from the date of the notifiable incident. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| PCBU Name & Contact Details: Robertson’s Remedial and Painting Pty Ltd # Unit 10/56 Buffalo Road Gladesville NSW 2111 Phone: (02) 9181 3519 A.B.N 16 140 746 247 |  |  |  |  | Principal Contractor (PC): |  |
| Work Manager Contact Number: |  |  |  |  | Date SWMS provided to PC: |  |
| Work Activity: | Balcony Works |  |  |  | Workplace Location: |  |
| High Risk Construction Work: | Risk of person falling more than 2 metres (note: in some jurisdictions, this is 3metres) |  |  | Work on a telecommunication tower |  | Demolition of a load-bearing structure |
|  | Likely to involve disturbing asbestos |  |  | Temporary load-bearing support for structural alterations or repairs |  | Work in or near a confined space |
|  | Work in or near a shaft or trench deeper than 1.5m or a tunnel |  |  | Work in an area that may have a contaminated or flammable atmosphere. |  | Work on or near pressurised gas mains or piping |
|  | Work on or near chemical, fuel or refrigerant lines |  |  | Work on or near energised electrical installations |  | Use of explosives |
|  | Diving Work |  |  | Work on, in or adjacent to a road, railway, shipping lane or other traffic corridor in use by traffic other than pedestrians |  | Work in an area with movement of powered mobile plant. |
|  | Work in areas with artificial extremes of temperature |  |  | Work in or near water or other liquid that involves a risk of drowning |  | Tilt-up or precast concrete elements |
| Name of workers consulted in the development of this SWMS: |  |  |  |  | Signature: |  |
| Person responsible for ensuring compliance with SWMS: |  |  |  |  | Date SWMS received: |  |
| What measures are in place to ensure compliance with the SWMS? |  |  | Toolbox meetings, SWMS sign off, job observations and supervision review. If issues with the SWMS or new hazards are identified, the supervisor must be notified. When changes are made to SWMS, it will be communicated to all workers. |  |  |  |
| Person responsible for approving and reviewing SWMS control measures: |  |  |  |  | Date SWMS received by reviewer: |  |
| How will the SWMS control measures be reviewed? |  |  | The control measures implemented will be reviewed and if necessary, revised annually or if work methods change, the control measures are not effective in controlling the risk, a new hazard/risk is identified or following an incident. The SWMS will be reviewed in consultation with workers and/or others who may be affected by the SWMS. Any changes to the SWMS will be communicated with workers at induction, daily pre-starts and toolbox talks. |  |  |  |
| Review Date: |  |  |  |  | Reviewer’s signature: |  |

| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| --- | --- | --- | --- | --- | --- |
| Planning for the setup of job | Untrained or unlicensed workers may access site | Medium (3) | All workers must have completed Construction Induction Training. All workers qualified/ licensed/ ticketed/ trained as required for job. All workers must complete site induction and sign in / out. All workers to be trained in and sign SWMS. Pre-start meeting to be conducted prior to commencement of works. All workers to be in a fit state to commence work, not fatigued or under the influence of drugs or alcohol. Competent person to supervise all new workers and activity. All workers to ensure they are familiar with the emergency muster points of the site and the emergency procedures. | Low (1) | Workers & Supervisor |
|  | Inappropriate tool and equipment allocation | Medium (3) | Supervisor/Management to ensure that appropriate equipment and procedures are used for the works being completed. Supervisor/Management to ensure that all electrical devices have an electrical test tag prior to commencement of work. Supervisor/Management to ensure all plant, equipment and tools are inspected for any damage or faults. Management to ensure that adequate PPE is available to workers. | Low (1) | Workers & Supervisor |
|  | Injury to personnel due to individuals entering work site | Medium (3) | Appropriate barricades & signage to be erected to prevent unauthorised access to work area. Where appropriate, spotters to be used to observe personnel and re-direct where needed. Alternate access to be provided. | Low (1) | Workers & Supervisor |
| Accessing and securing the site | Slips, trips and falls | Medium (3) | All workers to wear steel capped boots. Identify any slip trip and fall hazards and ensure the work area is clear and free of these hazards and obstructions. | Low (1) | Workers |
|  | Weather conditions, UV Exposure, Windy or Wet Conditions | Medium (3) | Workers must remain hydrated regularly drinking water. Workers to stay out of sun where possible and implement rest breaks or job rotation where necessary. Consider not working in very hit conditions. Do not work in wet weather or high wind conditions. Workers must wear appropriate PPE including: sun cream, sun hats, and tinted safety glasses while working outside. | Low (1) | Workers |
| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| Delivery and unload plant / equipment / tools etc. | Manual handling | Medium (4) | Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit. Use team lifts where needed. Workers to follow correct manual handling procedures: Clear a path, assess the load, get a good grip, ensure a wide stance, bend the knees, keep the spine straight, use the legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Workers |
|  | Workers and others being struck by vehicles in adjacent road or traffic. | High (6) | Where required Traffic Controllers or spotters to direct traffic entering / exiting site and control traffic on adjacent pedestrian footpaths and roadways. Use portable traffic signals, safety barriers, cones etc. | Low (2) | Workers |
| Removal of tiles bedding, balustrade and brickwork for cavity flashing | Dust and flying Debris | Medium (3) | - Water Spay to be used to prevent the spread of Dusts. All PPE to be worn at all times including Safety Glasses, dust mask, and hard hats. Ensure others do not enter the work site while demolition is in progress. Implement exclusion zones with safe distances. | Low (1) | Workers |
|  | Manual handling | Medium (4) | Use mechanical lifting devices and trolleys where possible to remove demolition waste / debris. Only lift objects within your safe lifting limit. Use team lifts where needed. Workers to follow correct manual handling procedures: Clear path, assess load, get a good grip, ensure a wide stance, bend knees, keep back straight, use the legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Workers |
|  | The grinder or disk may fail causing injury to workers or operators. | Medium (4) | Ensure appropriate cutting disk is used Wheels should be used only for the specific material and purpose for which they are designed, and according to the manufacturer's recommendations. Ensure the correct flange and locking nut is in place. Ensure the guard and handles are secure. Ensure there are no defects or damage to the disc. | Low (2) | Workers |
| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| Removal of tiles bedding, balustrade and brickwork for cavity flashing | Fall from heights | High (6) | Implement work platform, physical barrier or physical restraint (In that order) for all work where a worker can fall from one level to another. Workers to follow SWI for control measure selected above such as: Mobile scaffold, scaffold, EWP, etc. Pre-start inspections must be completed on equipment daily. Never work within 3m of an unguarded edge without fall protection in place (e.g. scaffolding or safety harness) Workers to wear a fall restraint harness and lanyard or inertia reel attached to an approved anchor point, Where anchorage points are to be used, they will be to be installed and inspected by a competent person prior to use. | Low (2) | Workers & Supervisor |
|  | Grinding disk or grinder may come into contact with worker causing serious injury (kick back). | Medium (4) | Stay alert, watch what you are doing and use common sense when grinding. Do not use while tired or under the influence of drugs, alcohol, or medication. Hold grinder with two hands while operating. Allow the grinder to "run up" to operating speed before applying it to the job. Hold grinder against work piece with minimum pressure, so it doesn't "dig in" and cause kick back. Never bump grinder on to job, or let disc hit another object while grinding. Adopt a comfortable stance, with feet apart and well balanced, and with a clear view of the job. Never use a grinder between the legs. Never put a grinder down until the disc stops rotating. Remove the plug from the power point or the air hose before changing discs. | Low (2) | Workers |
| Prepare substrate | Hazardous Materials | Medium (3) | Only competent workers involved in the substrate mixing process. Wear appropriate PPE at all times during mixing and applying substrate. Ensure all Leaks of leaks around sides are cleaned immediately. Workers to follow the Safety Data Sheet for substrate. | Low (1) | Workers |
| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| Install new screed/bedding | Repetitive work and prolonged fixed postures | Medium (4) | Workers must warm up and stretch prior to commencing work. Workers to rotate tasks to avoid repetitive tasks and fixed postures. Workers to avoid excessive straining and over reaching. | Low (2) | Workers |
|  | Hazardous Materials | Medium (3) | Only competent workers involved in the concrete mixing process. Wear appropriate PPE at all times during mixing and pouring process. Ensure all Leaks of concrete around sides are cleaned immediately after monument head is attached to avoid residue burns Workers to follow the Safety Data Sheet for concrete. | Low (1) | Workers |
| Removal of door | Cuts and abrasions from using machinery and tools | Medium (3) | Leather gloves, safety glasses, hearing protection and suitable clothes must to be worn. Dust masks to be worn where necessary. | Low (1) | Workers |
|  | Manual handling | Medium (3) | Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit. Use team lifts where needed. Workers to follow correct manual handling procedures: Clear a path, assess the load, get a good grip, ensure a wide stance, bend the knees, keep the spine straight, use the legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (1) | Workers |
| Waterproofing - Application of liquid membrane | Skin and eye irritation/ inhalation of chemicals | Medium (3) | Ensure appropriate PPE is worn Heavy duty gloves to be worn Eye protection Appropriate respirator to be worn, refer to Safety Data Sheet (SDS) Area to barricaded off and ‘no entry’ signage posted Only to be used in a well ventilated area Appropriate breathing protection may be required | Low (2) | Workers |
|  | Exposure to hazardous substances | Medium (4) | Site supervisor to ensure that Safety Data Sheets (SDS) for all hazardous substances on site are available to all Workers and that the relevant fist aid requirements outlined in the SWMS are available to manage exposure. Workers to review of all hazardous substances to be used and follow safety guidelines. | Low (2) | Workers |
| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| Waterproofing - Application of liquid membrane | Fall from heights | High (6) | Implement work platform, physical barrier or physical restraint (In that order) for all work where a worker can fall from one level to another. Workers to follow SWI for control measure selected above such as: Mobile scaffold, scaffold, EWP, etc. Pre-start inspections must be completed on equipment daily. Never work within 3m of an unguarded edge without fall protection in place (e.g. scaffolding or safety harness) Workers to wear a fall restraint harness and lanyard or inertia reel attached to an approved anchor point, Where anchorage points are to be used, they will be to be installed and inspected by a competent person prior to use. | Low (2) | Workers & Supervisor |
|  | Skin and eye irritation/ inhalation of chemicals | Medium (4) | Ensure appropriate PPE is worn including Heavy duty gloves to be worn Eye protection Appropriate respirator to be worn, Refer to Safety Data Sheet (SDS) for Blackjack Bitumen Area to barricaded off and ‘no entry’ signage posted Only to be used in a well-ventilated area Appropriate breathing protection may be required | Low (2) | Workers |
|  | Exposure to hazardous substances | Medium (4) | Site supervisor to ensure that Safety Data Sheets (SDS) for all hazardous substances on site are available to all Workers and that the relevant first aid requirements outlined in the SWMS are available to manage exposure. Workers to review of all hazardous substances to be used and follow safety guidelines. | Low (2) | Workers |
|  | Repetitive work and prolonged fixed postures | Medium (4) | Workers must warm up and stretch prior to commencing work. Workers to rotate tasks to avoid repetitive tasks and fixed postures. Workers to avoid excessive straining and over reaching. | Low (2) | Workers |
| Cavity flashing | Hazardous Materials | Medium (3) | Only competent workers involved in the concrete mixing process. Wear appropriate PPE at all times during mixing and pouring process. Ensure all Leaks of concrete around sides are cleaned immediately after monument head is attached to avoid residue burns Workers to follow the Safety Data Sheet for concrete. | Low (1) | Workers |
| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| Cavity flashing | Repetitive work and prolonged fixed postures | Medium (4) | Workers must warm up and stretch prior to commencing work. Workers to rotate tasks to avoid repetitive tasks and fixed postures. Workers to avoid excessive straining and over reaching. | Low (2) | Workers |
| Operating mixer | Manual handling | Medium (4) | Workers to follow correct manual handling procedures: Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit use team lifts. Clear a path, assess load, get a good grip, ensure a wide stance, bend knees, keep back straight, use legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Workers |
| Apply base | Manual handling | Medium (4) | Workers to follow correct manual handling procedures: Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit use team lifts. Clear a path, assess load, get a good grip, ensure a wide stance, bend knees, keep back straight, use legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Workers |
|  | Repetitive work and prolonged fixed postures | Medium (4) | Workers must warm up and stretch prior to commencing work. Workers to rotate tasks to avoid repetitive tasks and fixed postures. Workers to avoid excessive straining and over reaching. | Low (2) | Workers |
|  | Hazardous substances exposure | Medium (3) | Follow and read SDS prior to using hazardous substance. Ensure safety measures are followed. Ensure PPE is available, provided and worn for protection. Ensure area is well-ventilated. | Low (1) | Workers |
| Cut tiles with Tile Cutter | Worker Complacency | Medium (3) | Keep hands clear of sharp edges. Wear leather gloves & safety glasses to be worn at all times. Remove sharp corners. | Low (1) | Workers |
| Cut tiles with Saw or Grinder | The disk may fail | Medium (3) | Ensure appropriate cutting disk is used Wheels should be used only for the specific material and purpose for which they are designed, and according to the manufacturer's recommendations. Ensure the correct flange and locking nut is in place. Ensure the guard and handles are secure. Ensure there are no defects or damage to the disc. | Low (1) | Workers |
| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| Cut tiles with Saw or Grinder | Device or power cord is damaged | Medium (3) | Prior to use, saw needs to be checked for current and passed electrical test tag. The power lead and tool should be checked for damage. Ensure that the power outlet is not damaged. Do not run power leads across paths or in areas where they may be damaged (e.g. run over) | Low (1) | Workers |
|  | Grinding disk or saw may come into contact with worker Kickback | Medium (4) | Hold saw or grinder a secure grip while operating. Allow saw to "run up" to operating speed before applying it to the job. Hold the saw against the work piece with minimum pressure, so it doesn't "dig in" and cause it to kick back. Adopt a comfortable stance, with feet apart and well balanced, and with a clear view of the job. Never use a saw between the legs while sitting on the floor. Disconnect the power and place the saw on a bench with the disc facing upwards | Low (2) | Workers |
|  | Dust inhalation | Medium (3) | Wear dust mask or respirator. Cover all areas with plastic or appropriate sheeting. Use power tools with dust extraction where possible. Open windows and use dust extraction devices where necessary. | Low (1) | Workers |
|  | Worker Contact | Medium (3) | Isolate work area. Ensure appropriate barriers and signage is installed to prevent unauthorised access to the work area. | Low (1) | Workers |
| Lay tiles on floor | Manual handling | Medium (4) | Workers to follow correct manual handling procedures: Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit use team lifts. Clear a path, assess load, get a good grip, ensure a wide stance, bend knees, keep back straight, use legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Workers |
|  | Repetitive work and prolonged fixed postures | Medium (4) | Workers must warm up and stretch prior to commencing work. Workers to rotate tasks to avoid repetitive tasks and fixed postures. Workers to avoid excessive straining and over reaching. | Low (2) | Workers |
| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| Lay tiles on floor | Fall from heights | High (6) | Implement work platform, physical barrier or physical restraint (In that order) for all work where a worker can fall from one level to another. Workers to follow SWI for control measure selected above such as: Mobile scaffold, scaffold, EWP, etc. Pre-start inspections must be completed on equipment daily. Never work within 3m of an unguarded edge without fall protection in place (e.g. scaffolding or safety harness) Workers to wear a fall restraint harness and lanyard or inertia reel attached to an approved anchor point, Where anchorage points are to be used, they will be to be installed and inspected by a competent person prior to use. | Low (2) | Workers & Supervisor |
| Reinstalment of balustrades | Fall from heights | High (6) | Implement work platform, physical barrier or physical restraint (In that order) for all work where a worker can fall from one level to another. Workers to follow SWI for control measure selected above such as: Mobile scaffold, scaffold, EWP, etc. Pre-start inspections must be completed on equipment daily. Never work within 3m of an unguarded edge without fall protection in place (e.g. scaffolding or safety harness) Workers to wear a fall restraint harness and lanyard or inertia reel attached to an approved anchor point, Where anchorage points are to be used, they will be to be installed and inspected by a competent person prior to use. | Low (2) | Workers & Supervisor |
|  | Dust in eyes | Medium (3) | Wear goggles or safety glasses when sanding grinding or drilling. Avoid sanding, grinding or drilling above head height. | Low (1) | Workers |
|  | Hearing Damage | Medium (3) | Hearing protection to be worn while hammering bolts in | Low (1) | Workers |
|  | Manual handling | Medium (4) | Workers to follow correct manual handling procedures: Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit use team lifts. Clear a path, assess load, get a good grip, ensure a wide stance, bend knees, keep back straight, use legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Workers |
| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| Reinstalment of balustrades | Cuts and abrasions from using machinery and tools | Medium (3) | Leather gloves, safety glasses, hearing protection and suitable clothes must to be worn. Dust masks to be worn where necessary. Ensure correct size socket or ring spanner is used when tightening bolts. | Low (1) | Workers |
| Remove waste from site & Pack-up | Inappropriate dumping of materials | Medium (3) | Ensure a designated dumping area has been communicated to all workers on site and barricaded off. Lead or Asbestos contaminated waste to be dumped in appropriate facility – asbestos removalist will manage this. The waste from demolition must be disposed of only at a landfill site that may lawfully receive the waste. The waste must be covered during transport. Do not over load vehicle and no objects to hang out of side or top. | Low (1) | Workers |
|  | Manual handling | Medium (3) | Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit. Use team lifts where needed. Workers to follow correct manual handling procedures: Clear a path, assess the load, get a good grip, ensure a wide stance, bend the knees, keep the spine straight, use the legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (1) | Workers |
|  | Housekeeping | Medium (3) | All tools and equipment to be cleaned and stored away. Ensure work site is left in clean and tidy condition with no obstructions or protruding materials. Secure off work area below and above. Clean-up site and ensure it is left in safe, clean and tidy condition. | Low (1) | Workers |

| Personal Protective Equipment (PPE) required: | Enclosed leather steel capped boots Safety Glasses (As per AS/NZS 1337) Hearing Protection Protective Gloves (As per AS/NZS 2161) | High visibility Long sleeve shirt & pants Hard Hats (As per AS/NZS 1801) Sun hats, sun cream - when working outdoors Dust Mask / P2 Respirator |
| --- | --- | --- |
| List the training required by workers to complete the work: List the qualifications of personnel doing the work: | Construction Induction Card Induction into SWMS Supervisors Trained in: Certificate III in Demolition (CPC30413) and Certificate IV in Demolition (CPC41013) PLUS 3 years demonstrated experience, relevant to type of demolition licence. Demolition Licence or Restricted Licence (see SafeWork NSW website for which applies). |  |
| List the Permits, Certificates, SafeWork NSW Approvals, required to completed the work: | SafeWork NSW Approval 5 days prior to demolition undertaking. Demolition Permit. |  |
| Relevant legislation: For further details refer to the Legal Requirements Register | Work in accordance with Work Health and Safety Act 2011 and Work Health and Safety Regulation 2017, workplace injury management and workers compensation act 1998 no. 86, AS/NZS1337.1 Personal eye protection-Eye and face protectors for occupational applications (2010), AS/NZS 1269.3 Occupational Noise management-Hearing protection program (2005), AS/NZS 1801 industrial safety helmets (1997), AS/NZS 2210.1 Safety, protective and occupational footwear-Guide to selection, care and use (2010) & AS/NZS 2210.2 to 2210.9 Occupational protective footwear. SafeWork NSW Codes of Practice: - Construction Work - Managing risks of plant - Managing the risk of falls - Work Health and Safety Consultation, Co-operation and Co-ordination - Managing the Work Environment and Facilities - First Aid in the Workplace - Hazardous Manual Tasks - How to Manage Work Health and Safety Risks |  |
| List the plant / equipment that will be used on site: | Grinder, drill, saw |  |
| List maintenance checks for plant & equipment: | Daily pre-start checks, maintenance as per manufacturers guidelines |  |
| Frequency of site/workplace inspections: Site Inspections will be completed by: | Daily site inspections & job completion inspections to be completed by Site Supervisor. |  |
| Hazardous Substances: | Substrate, base, waterproofing |  |

# Sign Off and Approvals

| Person responsible for site supervision of the work, inspecting and approving work areas, approval of and compliance with SWMS, protective measures, plant, equipment: |  |
| --- | --- |
| Name and position: | Signature: |
| SWMS Prepared by: Ben Knox – Compliance OH&S | Signature: |

| Risk Level | Description of consequence or impact | Consequence | Likelihood/Probability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Unlikely (1) | Possible (2) | Almost Certain (3) |
| High Level of harm | Actual/Potential fatality, disability or irreversible damage. Major structural failure/damage. Off-site environmental discharge/release not contained and significant long-term environmental harm. | Major (3) | Medium (3) | High (6) | High (9) |
| Medium Level of harm | Actual/Potential temporary disability, MTI or LTI. Structural failure/damage, >1-day outage. On-site environmental discharge/release contained, minor remediation, short-term environmental harm. | Moderate (2) | Low (2) | Medium (4) | High (6) |
| Low Level of harm | Incident that has the potential to cause persons to require first aid. Environmental discharge/release immediately contained, minor level clean-up with no short term environmental harm. | Minor (1) | Low (1) | Low (2) | Medium (3) |
| Level | Likelihood/Probability |  |  |  |  |
| Almost Certain | Occurs frequently; >66% chance of occurring |  |  |  |  |
| Possible | Could happen occasionally; >33% but <66% chance of occurring |  |  |  |  |
| Unlikely | May occur only in exceptional circumstances; <33% chance of occurring |  |  |  |  |
| Class/Ranking | Description/Requirements |  |  |  |  |
| High 6, 9 | Stop immediately. Implement permanent preventative controls. |  |  |  |  |
| Medium 3, 4 | Will require operational planning. Actions will be recorded on a SWMS. |  |  |  |  |
| Low 1, 2 | Will require detailed pre-planning. Actions will be recorded on a SWMS. |  |  |  |  |

SWMS has been developed using the hierarchy of controls to reduce the risk and likely hood of an incident/injury occurring. The use of Elimination, Substitution, Engineering/isolation, Administrative and PPE measures will be used in the above order of preference wherever practicable.

Induction confirmation:

This SWMS has been developed in consultation with the workers and has been read and signed by all workers involved in this activity. I note that I have read and understood the above safe work method statement and will ensure work is completed accordingly.

| Date: | Name: | Signature: |
| --- | --- | --- |
| Date: | Name: | Signature: |