| - NOTE: Work must be performed in accordance with this SWMS. - This SWMS must be kept and be available for inspection until the high-risk construction work to which this SWMS relates is completed. If the SWMS is revised, all versions should be kept. If a notifiable incident occurs in relation to the high-risk construction work in this SWMS, the SWMS must be kept for at least 2 years from the date of the notifiable incident |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| - PCBU Name & Contact Details: - Robertson’s Remedial and Painting Pty Ltd # Unit 10/56 Buffalo Road Gladesville NSW 2111 Phone: (02) 9181 3519 A.B.N 16 140 746 247 |  |  |  |  | - Principal Contractor (PC): |  |
| - Work Manager Contact Number: |  |  |  |  | - Date SWMS provided to PC: |  |
| - Work Activity: | - Remediation Works using Roof Jockey System |  |  |  | - Workplace Location: |  |
| High Risk Construction Work: | Risk of person falling more than 2 metres (note: in some jurisdictions, this is 3metres) |  |  | Work on a telecommunication tower |  | Demolition of a load-bearing structure |
|  | Likely to involve disturbing asbestos |  |  | Temporary load-bearing support for structural alterations or repairs |  | Work in or near a confined space |
|  | Work in or near a shaft or trench deeper than 1.5m or a tunnel |  |  | Work in an area that may have a contaminated or flammable atmosphere. |  | Work on or near pressurised gas mains or piping |
|  | Work on or near chemical, fuel or refrigerant lines |  |  | Work on or near energised electrical installations |  | Use of explosives |
|  | Diving Work |  |  | Work on, in or adjacent to a road, railway, shipping lane or other traffic corridor in use by traffic other than pedestrians |  | Work in an area with movement of powered mobile plant. |
|  | Work in areas with artificial extremes of temperature |  |  | Work in or near water or other liquid that involves a risk of drowning |  | Tilt-up or precast concrete elements |
| Name of workers consulted in the development of this SWMS: |  |  |  |  | Signature: |  |
| Person responsible for ensuring compliance with SWMS: |  |  |  |  | Date SWMS received: |  |
| What measures are in place to ensure compliance with the SWMS? |  |  | Toolbox meetings, SWMS sign off, job observations and supervision review. If issues with the SWMS or new hazards are identified, the supervisor must be notified. When changes are made to SWMS, it will be communicated to all workers. |  |  |  |
| Person responsible for approving and reviewing SWMS control measures: |  |  |  |  | Date SWMS received by reviewer: |  |
| How will the SWMS control measures be reviewed? |  |  | The control measures implemented will be reviewed and if necessary, revised annually or if work methods change, the control measures are not effective in controlling the risk, a new hazard/risk is identified or following an incident. The SWMS will be reviewed in consultation with workers and/or others who may be affected by the SWMS. Any changes to the SWMS will be communicated with workers at induction, daily pre-starts and toolbox talks. |  |  |  |
| Review Date: |  |  |  |  | Reviewer’s signature: |  |

| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| --- | --- | --- | --- | --- | --- |
| Planning for the preparation and repainting building | Untrained or unlicensed Workers may access site | Medium (3) | All Workers must have Construction Induction Certificate. All Workers must complete site induction, sign in and out of site daily. Where induction is not available Workers to liaise with site controller about site hazards, emergency procedures and any other site requirements. All Workers to be trained in and sign safe work method statement. All Workers to be qualified/ licensed/ ticketed/ trained as required for activities performed. Competent person to supervise all new Workers. Toolbox talk to be conducted prior to commencement of works. All Workers to be in a fit state to commence work, they must be Zero blood alcohol concentration and must not be suffering from fatigue or under the influence of drugs. | Low (1) | Workers & Supervisor |
|  | Inappropriate tool and equipment allocation | Medium (3) | Risk Assessment of site to be carried out prior to commencement of work and changes made to SWMS where necessary. Management to ensure that adequate Workers numbers are allocated to complete works. Management to ensure that adequate personal protective equipment is available. Management to ensure that appropriate equipment and procedures are used for the works being completed. Workers to ensure electrical devices are checked prior to commencing work and a current test tag is attached to the device. Ensure adequate first aid kits and amenities are available. | Low (1) | Workers & Supervisor |
|  | Inappropriate transportation/ storage of products. | Medium (3) | Materials must be transported in accordance with SDS specifications. Secure storage areas to be allocated for products. Spill kits and fire fighting equipment to be available on site. | Low (1) | Workers & Supervisor |
| Accessing the site | Slips, trips and falls. | Medium (3) | All Workers to wear approved rubber sole work boots as per AS/NZS 2210. Identify slip trip and fall hazards. Workers not to access areas where risk of slip trip or fall is present without appropriate protection in place. | Low (1) | Workers |
| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| Secure job site to ensure public do not access | Injury to public or Workers due to individuals entering work site | Medium (3) | Painting to be completed during quite times where possible. Appropriate barricading & signage to be erected to prevent public and unauthorised access to work area where necessary. Spotters to be used to observe public and re-direct as needed. Alternate access to be provided to public where appropriate. Implement traffic management plan where necessary. | Low (1) | Workers |
| Unload materials and equipment from vehicle and position in work area | Manual handling injury | Medium (4) | Workers to follow correct manual handling procedures: Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit. Use team lifts where needed. Clear a path, assess the load, get a good grip, ensure a wide stance, bend the knees, keep the spine straight, use the legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Workers |
|  | Inappropriate storage areas may interfere with public | Medium (3) | Materials must be transported in accordance with safety data sheet specifications. Secure storage areas to be allocated for products. Spill kits and fire fighting equipment to be available on site to manage emergency situations. | Low (1) | Workers |
| Roof Jockey System Safety Setup and Pre-Checks | Contact with overhead power lines, infrastructure or persons. | High (9) | Complete survey of site with site controller before entering and plan path of travel to ensure all obstacles are avoided. Maintain a safe distance from power lines, infrastructure and people: - 3m clearance up to 132,000V - 6m clearance 132,000V to 330,000V - 8m + clearance for over 330,000V Use insulated tag lines when operating near power lines. Obtain approved overhead power line permit before working in close proximity to overhead power lines. Spotter to be used to ensure load remains safe distance from power lines. | Medium (3) | Crane Operator & Dogger |
| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| Roof Jockey System Safety Setup and Pre-Checks | Roof jockey system failure, falling persons and/or objects | High (6) | The roof jockey must comply with manufacturer’s instructions and AS/NZS 4488.1 and AS/NZS 1891.1. All personnel involved are to have a detailed copy of the task and risk assessment documented. Safety harnesses will be worn and secured in compliance with AS/NZS 1891.1. Site Manager/ Supervisor to ensure that agreed procedures are adhered to. The roof jockey system must be inspected prior to use for any defects. Must obtain any current inspection records prior to using and have twelve monthly intervals of inspections of the cage. All rope access equipment, chains, slings, etc. must be inspected and tagged for safe use prior to use. Never use the equipment unless you have read and understood this manual and completed an approved training in the use of the power Ascender system. Ensure rope has no dirt or sand present as it will wear the Rope Grab and Rope Guide. Use a rope mat, rope bag or similar. Ensure the rope guide and rope are running smoothly through the rope mechanism. Check all equipment and components ensuring it is compatible the roof jockey and ascender system. Wear appropriate clothing and tie back any loose clothing or hair. Ensure to have an emergency plan in place. Ensure battery for ascender is charged and tested before use. Check to ensure counterweights are in good condition and weight is sufficient counterbalance the roof jockey system including the abseiler. | Low (2) | Workers |
|  | Manual handling injury | Medium (4) | Workers to follow correct manual handling procedures: Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit. Use team lifts where needed. Clear a path, assess the load, get a good grip, ensure a wide stance, bend the knees, keep the spine straight, use the legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Workers |
| Operate roof jockey system | Poor communication | Medium (4) | Workers to ensure clear communication is maintained during lifting operations. Toolbox talk to be completed and SWMS discussed and signed prior to any operation commencing. | Low (2) | Crane Operator & Dogger |
| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| Operate roof jockey system | Fall from heights | High (9) | Conduct daily Pre-start Inspections and ensure all equipment is safe for use. Remove faulty or damaged equipment immediately and notify supervisor. Ensure that equipment is set up as per manufactures instructions. Competent and Licenced workers with minimum IRATA Level 1) in Industrial Rope Access. Level 2 or 3 IRATA trained person available for rescues. Ensure all anchor points are certified by structural engineer and safe for use. Check compliance plates, engineers rating specification & suitability for task. Ensure safety line is in place at all times and do not attach any items to it. Never work alone. Have licenced IRATA Level 3 Supervisor. Use rope protection when attaching slings and ropes to prevent damage. Place a knot in the end of the safety and access ropes to prevent running off. All equipment over 8kg must be attached with a separate line. Operator to wear hard hat with chin strap at all times. Descend at a slow pace, ensuring that the descender does not heat up. Measure the height you are working at, height and weight of worker. Ensure harness is set-up to take these factors into account if falls occur (fall arrest system stops before the worker hits the ground). Team members to complete check on each other to ensure equipment set up appropriately. Ensure safety harness is correct size and fit, not too tight (may cause suffocation). Do not swing excessively while descending/ascending Ascender must be allowed to rotate freely while suspended. Operate the Ascender according to the advice contained within this user manual and pre-planned work instructions. Never operate jockey system when lightning, wind or extremes in temperatures. | Medium (3) | Workers |
|  | Objects falling from height | High (6) | All tools equipment and materials to be secured with lanyards. Implement Exclusion / Drop zones around the worker at height. Appropriate barriers and signage to be erected to assist in protecting the site where necessary. Hard hats to be worn at all times. | Low (2) | Workers |
| Procedure (In Steps): | Possible Hazards: | Risk Score | Safety Controls: | Residual Risk | Responsible Person |
| Operate roof jockey system | Chains not hooked properly causing load to drop from height. | High (9) | Perform visual inspection of chains and check current tag is in place. Daily pre-start inspections on slings, chains and lifting devices by competent persons to identify defective equipment to be removed from service. Use only engineer approved slings, chains and lifting devices. Only use slings, chains and lifting devices that have been designed for the purpose they are to be used for- see manufacturer’s instructions. Ensure lifting equipment is tagged displaying SWL and has been inspected for compliance. Only trained and competent person to attach chains. Lifting gear check to be completed prior to commencement of job. All unsuitable equipment to be removed from service and repaired. | Medium (3) | Crane Operator & Dogger |
| Rescue | Injury to operator, Suspension Trauma | High (9) | Workers must work in teams of two to ensure that in the event of a fall they can be rescued. Level 2 or 3 IRATA trained person available for rescues. In the event of a fall ensure suspended worker is safely removed from harness immediately. Do not leave person hanging for more than 5 minutes! Where fitted, use emergency rescue equipment to escape from harness. Where necessary, use access equipment to remove worker from heights. Individuals in fall must be reviewed by doctor and gain clearance to work. Operators must have the ability to ascend and descend at all times. All equipment subject to a fall is to be removed from service and recertified. Spotter to be in constant visual, verbal or radio communication Suspended worker to try keep moving leg muscles, keep blood circulating. If at risk of being suspended after a fall, worker to use front attachment points on fall-arrest harnesses. | Medium (3) | Workers |
| Pack up | Manual handling injury | Medium (4) | Workers to follow correct manual handling procedures: Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit. Use team lifts where needed. Clear a path, assess the load, get a good grip, ensure a wide stance, bend the knees, keep the spine straight, use the legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Workers |
|  | Housekeeping | Low (2) | Centralise and store all tools and equipment at designated secured storage area. | Low (1) | Crane Operator & Dogger |

| Personal Protective Equipment (PPE) required: | Enclosed leather steel capped boots (As per AS/NZS 2210) Safety Glasses (As per AS/NZS 1337) Hearing Protection Protective Gloves (As per AS/NZS 2161) Hard Hats (As per AS/NZS 1801) High visibility clothing Sun hats, sun cream- when working outdoors |
| --- | --- |
| List the training required by workers to complete the work: List the qualifications of personnel doing the work: | Construction Induction Card Relevant trade certificate Induction into SWMS Minimum qualified IRATA Level 1 Technician Work Safely at Heights Training |
| List the Permits, Certificates, SafeWork NSW Approvals, required to completed the work: | All necessary permits to be completed on site |
| Relevant legislation: For further details refer to the Legal Requirements Register | Work in accordance with Work Health and Safety Act 2011 and Work Health and Safety Regulation 2017, workplace injury management and workers compensation act 1998 no. 86, AS/NZS1337.1 Personal eye protection-Eye and face protectors for occupational applications (2010), AS/NZS 1269.3 Occupational Noise management-Hearing protection program (2005), AS/NZS 1801 industrial safety helmets (1997), AS/NZS 2210.1 Safety, protective and occupational footwear-Guide to selection, care and use (2010) & AS/NZS 2210.2 to 2210.9 Occupational protective footwear, AS/NZS 4488.1 Industrial rope access systems specifications (1997), AS/NZS 1891.1 Personal equipment for work at height manufacturing requirements for full body combination and lower body harnesses (2020). SafeWork NSW Codes of Practice: - Construction Work - Managing risks of plant - Managing the risk of falls - Work Health and Safety Consultation, Co-operation and Co-ordination - Managing the Work Environment and Facilities - First Aid in the Workplace - Hazardous Manual Tasks - How to Manage Work Health and Safety Risks |
| List the plant / equipment that will be used on site: | Harness, Helmet, Ascenders/Descenders, Backup Fall Arrest Device, Pulley, Foot loops & Etrier, Karabiners, Industrial Rope, Anchors, Lanyards, |
| List maintenance checks for plant & equipment: | Daily pre-start checks, log-book checks, service / maintenance as per manufacturers recommendations. |
| Frequency of site/workplace inspections: Site Inspections will be completed by: | Daily site inspections & job completion inspections to be completed by Site Supervisor. |
| Hazardous Substances: |  |

# Sign Off and Approvals

| Person responsible for site supervision of the work, inspecting and approving work areas, approval of and compliance with SWMS, protective measures, plant, equipment: |  |
| --- | --- |
| Name and position: | Signature: |
| SWMS Prepared by: Ben Knox – Compliance OH&S | Signature: |

| Risk Level | Description of consequence or impact | Consequence | Likelihood/Probability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Unlikely (1) | Possible (2) | Almost Certain (3) |
| High Level of harm | Actual/Potential fatality, disability or irreversible damage. Major structural failure/damage. Off-site environmental discharge/release not contained and significant long-term environmental harm. | Major (3) | Medium (3) | High (6) | High (9) |
| Medium Level of harm | Actual/Potential temporary disability, MTI or LTI. Structural failure/damage, >1-day outage. On-site environmental discharge/release contained, minor remediation, short-term environmental harm. | Moderate (2) | Low (2) | Medium (4) | High (6) |
| Low Level of harm | Incident that has the potential to cause persons to require first aid. Environmental discharge/release immediately contained, minor level clean-up with no short term environmental harm. | Minor (1) | Low (1) | Low (2) | Medium (3) |
| Level | Likelihood/Probability |  |  |  |  |
| Almost Certain | Occurs frequently; >66% chance of occurring |  |  |  |  |
| Possible | Could happen occasionally; >33% but <66% chance of occurring |  |  |  |  |
| Unlikely | May occur only in exceptional circumstances; <33% chance of occurring |  |  |  |  |
| Class/Ranking | Description/Requirements |  |  |  |  |
| High 6, 9 | Stop immediately. Implement permanent preventative controls. |  |  |  |  |
| Medium 3, 4 | Will require operational planning. Actions will be recorded on a SWMS. |  |  |  |  |
| Low 1, 2 | Will require detailed pre-planning. Actions will be recorded on a SWMS. |  |  |  |  |

SWMS has been developed using the hierarchy of controls to reduce the risk and likely hood of an incident/injury occurring. The use of Elimination, Substitution, Engineering/isolation, Administrative and PPE measures will be used in the above order of preference wherever practicable.

# Induction confirmation:

This SWMS has been developed in consultation with the workers and has been read and signed by all workers involved in this activity. I note that I have read and understood the above safe work method statement and will ensure work is completed accordingly.

| Date: | Name: | Signature: |
| --- | --- | --- |
| Date: | Name: | Signature: |