| NOTE: Work must be performed in accordance with this SWMS. This SWMS must be kept and be available for inspection until the high-risk construction work to which this SWMS relates is completed. If the SWMS is revised, all versions should be kept. If a notifiable incident occurs in relation to the high-risk construction work in this SWMS, the SWMS must be kept for at least 2 years from the date of the notifiable incident. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| PCBU Name & Contact Details: Robertson’s Remedial and Painting Pty Ltd # Unit 10/56 Buffalo Road Gladesville NSW 2111 Phone: (02) 9181 3519 A.B.N 16 140 746 247 |  |  |  |  | Principal Contractor (PC): |  |
| Work Manager Contact Number: |  | Matthew McCarthy |  |  | Date SWMS provided to PC: |  |
| Work Activity | Cladding Installation |  |  |  | Workplace Location: |  |
| High Risk Construction Work: | Risk of person falling more than 2 metres |  |  |  | Work on a telecommunication tower |  | Demolition of a load-bearing structure |
|  | Likely to involve disturbing asbestos |  |  |  | Temporary load-bearing support for structural alterations or repairs |  | Work in or near a confined space |
|  | Work in or near a shaft or trench deeper than 1.5m or a tunnel |  |  |  | Work in an area that may have a contaminated or flammable atmosphere. |  | Work on or near pressurised gas mains or piping |
|  | Work on or near chemical, fuel or refrigerant lines |  |  |  | Work on or near energised electrical installations |  | Use of explosives |
|  | Diving Work |  |  |  | Work on, in or adjacent to a road, railway, shipping lane or other traffic corridor in use by traffic other than pedestrians |  | Work in an area with movement of powered mobile plant. |
|  | Work in areas with artificial extremes of temperature |  |  |  | Work in or near water or other liquid that involves a risk of drowning |  | Tilt-up or precast concrete elements |
| Name of workers consulted in the development of this SWMS: |  |  | Paul (Killark Supervisor) |  | Signature: |  |
|  |  |  | Killark cladding installation team |  |  |  |
|  |  |  | (Killark workers sign-off at end of SWMS) |  |  |  |
| Person responsible for ensuring compliance with SWMS: |  |  | Paul (Killark Supervisor). As the on-site supervisor for the sub-contractor, he is responsible for day-to-day adherence to the safety controls. |  | Date SWMS received: |  |
| What measures are in place to ensure compliance with the SWMS? |  |  | Toolbox meetings, SWMS sign off, job observations and supervision review. If issues with the SWMS or new hazards are identified, the supervisor must be notified. When changes are made to SWMS, it will be communicated to all workers. |  |  |  |
| Person responsible for approving and reviewing SWMS control measures: |  |  | Matthew McCarthy (RPD Project Manager). As the representative for the Principal Contractor, he holds the high-level responsibility for reviewing and approving the safe systems of work. |  | Date SWMS received by reviewer: |  |
| How will the SWMS control measures be reviewed? |  |  | The control measures implemented will be reviewed and if necessary, revised annually or if work methods change, the control measures are not effective in controlling the risk, a new hazard/risk is identified or following an incident. The SWMS will be reviewed in consultation with workers and/or others who may be affected by the SWMS. Any changes to the SWMS will be communicated with workers at induction, daily pre-starts and toolbox talks. |  |  |  |
| Review Date: |  |  |  |  | Reviewer’s signature: | Matthew McCarthy |

| Procedure (In Steps): | Possible Hazards: | Risk Rating: | Safety Controls: | Residual Risk: | Responsible Person: |
| --- | --- | --- | --- | --- | --- |
| Planning for the setup of job. Check all Workers tickets/licences, check site, complete induction | Untrained or unlicensed workers may access site causing serious injury to self or other individuals | Medium (3) | All workers must have Construction Induction Certificate. All workers to be qualified/ licensed/ ticketed/ trained as required for the works. All workers must complete site induction and sign in / out. Where induction is not available workers to liaise with site supervisor about site hazards, emergency procedures and any other site requirements. All workers to be trained in and sign safe work method statement. Toolbox talk to be conducted prior to commencement of works All workers to be in a fit state to commence work, be zero blood alcohol concentration, not suffering from fatigue or under the influence of drugs. Competent person to supervise all new workers. | Low (1) | Workers and Supervisor |
|  | Inappropriate tool and equipment allocation may lead to injury | Medium (3) | Site Supervisor/Management to ensure that appropriate equipment and procedures are used for the works being completed. Site Supervisor /Management to ensure that all electrical devices have an electrical test tag prior to commencement of work. Management to ensure that adequate PPE is available to workers | Low (1) | Workers and Supervisor |
|  |  |  |  |  |  | SWMS Amendment NEXT PAGE |  |  |  |  |  |
| Accessing and securing the site |  | Public and Resident Safety (Unauthorized Access and Occupational Interface). |  | High (6) |  | SWMS Amendment: RPD will amend and re-induct the Killark sub-contractor to include a dedicated line item for balcony security. All workers must be re-inducted at the next toolbox meeting ASAP. - Access Protocol: Killark Supervisor (Paul) must receive PM (Matt) confirmation before accessing any balcony. - Daily Security: After exiting balcony work areas, Killark must reinstate the barrier mesh with the 3-point tie-wire system per span at the end of every day. - Vigilance & Repair: All workers must immediately reinstate any compromised mesh (cut ties/slashes) observed during entry or transit. - Immediate Reporting: Compromised barriers must be reported to the Killark Supervisor, who notifies the RPD PM by phone immediately. - Escalation to Hume Housing: - Significant Breaches (apparent scaffold entry): RPD will report to Hume Housing immediately. - Minimal Breaches (tampering without entry): RPD will record these in the incident register; breaches are reported to Hume Housing immediately upon recurrence. |  | Low (2) |  | Supervisors |  |
| Accessing and securing the site | Slips, trips and falls | Medium (3) | All workers to wear steel capped boots. Onsite inductions to be completed prior to entering the job including identification of any slip trip and fall hazards. Ensure the work area is clear and free of slip, trip and fall hazards | Low (1) | Workers |
|  | Weather conditions I.e. heat stroke, dehydration, skin cancer, sun burn etc | Medium (3) | Workers must remain hydrated through regularly drinking water. Workers to stay out of sun where possible and implement rest breaks or job rotation where necessary. Workers must wear appropriate PPE including: sun cream, sun hats, and tinted safety glasses while working outside. | Low (1) | Workers |
|  | Environmental contamination i.e. erosion and sediment | Medium (3) | Where possible works to be timed to avoid rain Sandbags to be used to slow runoff and contain sediment Workers to follow the project environmental requirements (where required) | Low (1) | Workers |
| Accessing and securing the site | Emergency situation Unfamiliar with emergency procedures | Medium (3) | All workers to ensure they are familiar with the emergency muster points of the site All workers to follow the onsite emergency procedures as outlined at induction. | Low (1) | Workers |
|  | Injury to personnel due to individuals entering work site | Medium (3) | Appropriate barricades & Signage to be erected to prevent unauthorised access to work area. Where appropriate, spotters to be used to observe personnel and re direct where needed. Alternate access to be provided where appropriate. | Low (1) | Workers |
| Delivery and unload materials and position in work area | Manual handling | Medium (4) | Workers to follow correct manual handling procedures: Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit Use team lifts where needed Clear a path, assess the load, get a good grip, ensure a wide stance, bend the knees, keep the spine straight, use the legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Workers |
|  | Inappropriate storage areas may interfere with public | Medium (3) | Materials transported in accordance with SDS specifications. Secure storage areas to be allocated for products. Spill kits and firefighting equipment to be available on site to manage emergency situations. | Low (1) | Workers |
| Remove existing fittings/fixtures | Fall from ladder | High (6) | Avoid overhead electrical hazards and use fibreglass platform ladder Perform visual check on ladder for obvious faults and defects prior to use. Remove damaged ladders, tag “not for use” and repaired. When gaining access to a working platform/roof, the top of the ladder should extend above working platform by no less than 1m. Never “walk” ladder to reposition it; descend ladder and relocate it. Erect ladder on clean, level surface or use self levelling ladder. Erect extension ladder in a 4:1 ratio (4 meters up for every 1 meter from the wall) and tie off at top and bottom. Use a footer. Do not place a ladder in front of a doorway unless locked/guarded Secure work area around the ladder. Clean mud, grass etc, from boots. Slip resistant footwear worn. Do not over-reach while on ladder. 1 person on ladder at a time. Face ladder at all times. Maintain three points of contact. Do not step above the third step from the top. Ensure legs are fully extended and latch is in place. | Low (2) | Workers |
|  | Fall from Heights | High (6) | Select appropriate fall protection equipment to access heights where a worker can fall from one level to another. Site supervisor must implement the use of a work platform, physical barrier or physical restraint (In that order) for all work where workers can fall from one level to another. Workers to follow Safe Work Instructions (SWI) for control measure selected above such as: Mobile scaffold, Set scaffold, Boom lift, EWP, Knuckle boom Workers must review and implement safety requirements specified by suppliers. Pre start inspections must be completed on equipment daily & logbook completed. | Low (2) | Workers |
| Remove existing fittings/fixtures | Dust & Noise | Medium (3) | Wear dust mask or respirator, safety glasses and hearing protection. Use power tools with dust extraction where possible. Open windows and use dust extraction devices where necessary. Wear dust mask or respirator (As per AS 1716) when sanding or grinding. Cover all areas with plastic or appropriate sheeting. Use power tools with dust extraction where possible. Open windows and use dust extraction devices where necessary. | Low (1) | Workers |
|  | Manual handling injury | Medium (4) | Workers to follow correct manual handling procedures: Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit use team lifts where needed Clear a path, assess the load, get a good grip, ensure a wide stance, bend the knees, keep the spine straight, use the legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Workers |
| Check materials for compliance to spec and standards | Inappropriate materials may lead to injury | Medium (3) | Inspect all materials to ensure that they are the correct type, quantity, labelled correctly and have are not damaged. Check all materials purchased comply with spec and that they meet relevant Australian Standards. | Low (1) | Workers |
| Cut materials to length | Cuts to fingers or body parts | Medium (3) | Only use saw for materials that the blade is designed for. Do not use saw in wet weather or rain and mop up any spills in the area. Ensure all individuals are moved away to a safe distance. Ensure that the work is at a good height (around waist height) Ensure that timber is secure prior to cutting. | Low (2) | Workers |
|  | Dust may be hazardous | Medium (3) | Dust mask to be worn when cutting MDF of other hazardous substances. Saws with dust extraction to be used and filters to be cleaned regularly. Ensure that there is appropriate ventilation in the area | Low (2) | Workers |
| Cut materials to length | Possible electrical hazard if the device or power cord is damaged | High (6) | Do not use saw in wet weather or rain and mop up any spills in the area. Prior to use the saw must be checked for an electrical test tag to ensure that it is current and passed. Untested machinery is to be removed from service and tagged. The power lead and the saw must be checked for damage prior to use. Ensure that the power outlet is not damaged. Do not run power leads or hoses across paths or in areas where they may be damaged (e.g. run over) RCD to be used at all times, extension lead not to exceed 30 meters Cable stands to be used where necessary. | Low (2) | Workers |
|  | Cuts and burns from saw blade | Medium (4) | Stay alert, watch what you are doing and use common sense when cutting Do not use while tired or under the influence of drugs, alcohol, or medication. Adopt a comfortable stance, with feet apart and well balanced, and with a clear view. Never use a saw between the legs while sitting on the floor. Disconnect the power when not in use. Remove the plug from the power point before performing any services or repairs. Keep hands and body parts clear of saw blade Never cut towards a body part Hold saw with two hands and a firm grip at all times Ensure material being cut stays in contact with the front half of the blade | Low (2) | Workers |
| Install cladding | Fall from ladder | High (6) | Avoid overhead electrical hazards and use fibreglass platform ladder Perform visual check on ladder for obvious faults and defects prior to use. Remove damaged ladders, tag “not for use” and repaired. When gaining access to a working platform/roof, the top of the ladder should extend above working platform by no less than 1m. Never “walk” ladder to reposition it; descend ladder and relocate it. Erect ladder on clean, level surface or use self levelling ladder. Erect extension ladder in a 4:1 ratio (4 meters up for every 1 meter from the wall) and tie off at top and bottom. Use a footer. Do not place a ladder in front of a doorway unless locked/guarded Secure work area around the ladder. Clean mud, grass etc, from boots. Slip resistant footwear worn. Do not over-reach while on ladder. 1 person on ladder at a time. Face ladder at all times. Maintain three points of contact. Do not step above the third step from the top. Ensure legs are fully extended and latch is in place. | Low (2) | Workers |
|  | Fall from Heights | High (6) | Select appropriate fall protection equipment to access heights where a worker can fall from one level to another. Site supervisor must implement the use of a work platform, physical barrier or physical restraint (In that order) for all work where workers can fall from one level to another. Workers to follow Safe Work Instructions (SWI) for control measure selected above such as: Mobile scaffold, Set scaffold, Boom lift, EWP, Knuckle boom Workers must review and implement safety requirements specified by suppliers. Prestart inspections must be completed on equipment daily & logbook completed. | Low (2) | Workers |
| Install cladding | Manual handling injury | Medium (4) | Workers to follow correct manual handling procedures: Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit use team lifts where needed Clear a path, assess the load, get a good grip, ensure a wide stance, bend the knees, keep the spine straight, use the legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Workers |
|  | Injury due to incorrect use of equipment. | Medium (3) | Follow safe work instructions for grinders, sanders, drills, etc. Workers to be competent in the use of tools being used. Supervisor to monitor all new workers. | Low (2) | Workers |
| Secure materials | Crush injury from timbers falling | Medium (3) | Bracing and supporting timbers to be in place where necessary Materials to be soundly secured and supported Workers to ensure that sufficient nails are placed in timbers to prevent falling | Low (2) | Workers |
|  | Injury from nails into body from nail gun | High (6) | Complete visual check of gun prior to use Eye protection to be used when operating nail gun. Never point nail gun in direction of others Ensure that safety guard is working appropriately Place nail gun on timber prior to pressing trigger | Low (2) | Workers |
|  | Contact with hammer | Medium (3) | Keep body parts clear of hammer | Low (2) | Workers |
|  | Injury due to incorrect use of equipment. | Medium (3) | Follow safe work instructions for grinders, sanders, drills, etc. Workers to be competent in the use of tools being used. Supervisor to monitor all new workers. | Low (2) | Workers |
| Clean-up site | Housekeeping | Medium (3) | All tools and equipment to be secured at all times. Hard hats must be worn at all times. | Low (2) | Workers |
|  | Manual handling | Medium (4) | Workers to follow correct manual handling procedures: Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit use team lifts where needed Clear a path, assess the load, get a good grip, ensure a wide stance, bend the knees, keep the spine straight, use the legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. | Low (2) | Workers |
|  |  |  |  |  |  | SWMS Amendment |  |  |  |  |  |
|  |  | Public and Resident Safety (Unauthorized Access and Occupational Interface). |  | High (6) |  | SWMS Amendment: RPD will amend and re-induct the Killark sub-contractor to include a dedicated line item for balcony security. All workers must be re-inducted at the next toolbox meeting ASAP. - Access Protocol: Killark Supervisor (Paul) must receive PM (Matt) confirmation before accessing any balcony. - Daily Security: After exiting balcony work areas, Killark must reinstate the barrier mesh with the 3-point tie-wire system per span at the end of every day. - Vigilance & Repair: All workers must immediately reinstate any compromised mesh (cut ties/slashes) observed during entry or transit. - Immediate Reporting: Compromised barriers must be reported to the Killark Supervisor, who notifies the RPD PM by phone immediately. - Escalation to Hume Housing: - Significant Breaches (apparent scaffold entry): RPD will report to Hume Housing immediately. - Minimal Breaches (tampering without entry): RPD will record these in the incident register; breaches are reported to Hume Housing immediately upon recurrence. |  | Low (2) |  | Supervisors |  |

| Personal Protective Equipment (PPE) required: | Enclosed leather steel capped boots Safety Glasses (As per AS/NZS 1337) Hearing Protection Protective Gloves (As per AS/NZS 2161) | High visibility Long sleeve shirt & pants Hard Hats (As per AS/NZS 1801) Sun hats, sun cream - when working outdoors |
| --- | --- | --- |
| List the training required by workers to complete the work: List the qualifications of personnel doing the work: | Construction Induction Card Relevant trade certificate Induction into SWMS |  |
| List the Permits, Certificates, SafeWork NSW Approvals, required to complete the work: |  |  |
| Relevant legislation: | Work Health and Safety Act 2011 and Work Health and Safety Regulation 2017, AS/NZS1337.1 Personal eye protection-Eye and face protectors for occupational applications (2010), AS/NZS 1269.3 Occupational Noise management-Hearing protection program (2005), AS/NZS 1801 industrial safety helmets (1997), AS/NZS 2210.1 Safety, protective and occupational footwear-Guide to selection, care and use (2010) & AS/NZS 2210.2 to 2210.9 Occupational protective footwear. SafeWork NSW Codes of Practice Construction Work Managing the risk of falls Work Health and Safety Consultation, Co-operation and Co-ordination Managing the Work Environment and Facilities First Aid in the Workplace Hazardous Manual Tasks How to Manage Work Health and Safety Risks |  |
| List the plant / equipment that will be used on site: | Ladders and power tools |  |
| List maintenance checks for plant & equipment: | Daily pre-start checks, maintenance as per manufacturers guidelines |  |
| Frequency of site/workplace inspections: Site Inspections will be completed by: | Daily site inspections & job completion inspections to be completed by Site Supervisor. |  |
| Hazardous Substances: | Chemicals and glues for installing cladding |  |

# Sign Off and Approvals

| Person responsible for site supervision of the work, inspecting and approving work areas, approval of and compliance with SWMS, protective measures, plant, equipment: |  |
| --- | --- |
| Name and position: Paul (Killark Supervisor) | Signature: |
| SWMS Prepared by: Ben Knox – Compliance OH&S | Signature: |

| Risk Level | Description of consequence or impact | Consequence | Likelihood/Probability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Unlikely (1) | Possible (2) | Almost Certain (3) |
| High Level of harm | Actual/Potential fatality, disability or irreversible damage. Major structural failure/damage. Off-site environmental discharge/release not contained and significant long-term environmental harm. | Major (3) | Medium (3) | High (6) | High (9) |
| Medium Level of harm | Actual/Potential temporary disability, MTI or LTI. Structural failure/damage, >1-day outage. On-site environmental discharge/release contained, minor remediation, short-term environmental harm. | Moderate (2) | Low (2) | Medium (4) | High (6) |
| Low Level of harm | Incident that has the potential to cause persons to require first aid. Environmental discharge/release immediately contained, minor level clean-up with no short term environmental harm. | Minor (1) | Low (1) | Low (2) | Medium (3) |
| Level | Likelihood/Probability |  |  |  |  |
| Almost Certain | Occurs frequently; >66% chance of occurring |  |  |  |  |
| Possible | Could happen occasionally; >33% but <66% chance of occurring |  |  |  |  |
| Unlikely | May occur only in exceptional circumstances; <33% chance of occurring |  |  |  |  |
| Class/Ranking | Description/Requirements |  |  |  |  |
| High 6, 9 | Will require localised control measures. |  |  |  |  |
| Medium 3, 4 | Will require operational planning. Actions will be recorded on SWMS. |  |  |  |  |
| Low 1, 2 | Stop immediately. Implement permanent preventative controls. Will require detailed pre-planning. Actions will be recorded on a SWMS. |  |  |  |  |

SWMS has been developed using the hierarchy of controls to reduce the risk and likelihood of an incident occurring. The use of Elimination, Substitution, Isolation, Engineering, Administration and PPE measures will be used in the above order of preference wherever practicable.

Induction confirmation:

This SWMS has been developed in consultation with the workers and has been read and signed by all workers involved in this activity. I note that I have read and understood the above safe work method statement and will ensure work is completed accordingly.

| Date: | Name: | Signature: |
| --- | --- | --- |