| NOTE: Work must be performed in accordance with this SWMS. This SWMS must be kept and be available for inspection until the high-risk construction work to which this SWMS relates is completed. If the SWMS is revised, all versions should be kept. If a notifiable incident occurs in relation to the high-risk construction work in this SWMS, the SWMS must be kept for at least 2 years from the date of the notifiable incident. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| PCBU Name & Contact Details: Robertson’s Remedial and Painting Pty Ltd # Unit 10/56 Buffalo Road Gladesville NSW 2111 Phone: (02) 9181 3519 A.B.N 16 140 746 247 |  |  |  |  | Principal Contractor (PC): |  |
| Work Manager Contact Number: |  |  |  |  | Date SWMS provided to PC: |  |
| Work Activity: | Water Testing |  |  |  | Workplace Location: |  |
| High Risk Construction Work: | Risk of person falling more than 2 metres (note: in some jurisdictions, this is 3metres) |  |  | Work on a telecommunication tower |  | Demolition of a load-bearing structure |
|  | Likely to involve disturbing asbestos |  |  | Temporary load-bearing support for structural alterations or repairs |  | Work in or near a confined space |
|  | Work in or near a shaft or trench deeper than 1.5m or a tunnel |  |  | Work in an area that may have a contaminated or flammable atmosphere. |  | Work on or near pressurised gas mains or piping |
|  | Work on or near chemical, fuel or refrigerant lines |  |  | Work on or near energised electrical installations |  | Use of explosives |
|  | Diving Work |  |  | Work on, in or adjacent to a road, railway, shipping lane or other traffic corridor in use by traffic other than pedestrians |  | Work in an area with movement of powered mobile plant. |
|  | Work in areas with artificial extremes of temperature |  |  | Work in or near water or other liquid that involves a risk of drowning |  | Tilt-up or precast concrete elements |
| Name of workers consulted in the development of this SWMS: |  |  |  |  | Signature: |  |
| Person responsible for ensuring compliance with SWMS: |  |  |  |  | Date SWMS received: |  |
| What measures are in place to ensure compliance with the SWMS? |  |  | Toolbox meetings, SWMS sign off, job observations and supervision review. If issues with the SWMS or new hazards are identified, the supervisor must be notified. When changes are made to SWMS, it will be communicated to all workers. |  |  |  |
| Person responsible for approving and reviewing SWMS control measures: |  |  |  |  | Date SWMS received by reviewer: |  |
| How will the SWMS control measures be reviewed? |  |  | The control measures implemented will be reviewed and if necessary, revised annually or if work methods change, the control measures are not effective in controlling the risk, a new hazard/risk is identified or following an incident. The SWMS will be reviewed in consultation with workers and/or others who may be affected by the SWMS. Any changes to the SWMS will be communicated with workers at induction, daily pre-starts and toolbox talks. |  |  |  |
| Review Date: |  |  |  |  | Reviewer’s signature: |  |

| Procedure (In Steps): | Possible Hazards: | Risk Rating: | Safety Controls: | Residual Risk: | Responsible Person |
| --- | --- | --- | --- | --- | --- |
| Planning for the setup of job. Check all Workers tickets/licences, check site, complete induction | Untrained or unlicensed workers may access site Unauthorise access | Medium (3) | All workers must have Construction Induction Certificate. Workers qualified/ licensed/ ticketed/ trained as required for activity. All workers must complete site induction and sign in/out. Where induction is not available workers to liaise with site supervisor about site hazards, emergency procedures and other site rules. All workers to be trained in and sign SWMS. Pre-start meeting conducted prior to commencement of works. All workers to be in a fit state to commence work, not fatigued, under the influence of drugs or alcohol. Competent person to supervise all new workers. Appropriate barricades and signage to be erected to prevent unauthorised access to work area. Spotters used to ID persons & re-direct or alternate access provided. | Low (1) | Workers & Supervisor |
|  | Inappropriate tool and equipment allocation | Medium (3) | Site Supervisor/Management to ensure that appropriate equipment and procedures are used for the works being completed. Site Supervisor /Management to ensure that all electrical devices have an electrical test tag prior to commencement of work. Management to ensure that adequate PPE is available to workers | Low (1) | Workers & Supervisor |
| Setting up work area and undertaking works from heights | Fall from heights | High (6) | Select appropriate fall protection equipment to access heights where a worker can fall from one level to another. Site supervisor must implement the use of a work platform, physical barrier or physical restraint (In that order) for all work where workers can fall from one level to another. Workers to follow SWI for control measure selected above such as: Mobile scaffold, Set scaffold, Boom lift, EWP, Knuckle boom Review and implement safety requirements specified by suppliers. Complete daily Pre-start inspections on equipment & logbook. | Low (2) | Workers |
|  | Fall from ladder | High (6) | Avoid overhead electrical hazards and use fibreglass ladder Use a platform ladder where possible. Perform visual check on ladder for obvious faults and defects prior to use. Damaged ladders are to be removed from use, “tagged not for use” and repaired ASAP. Fully enclosed, slip resistant footwear should be worn. When gaining access to a working platform/roof, the top of the ladder should extend above the working platform/roof by no less than 1m. Never “walk” ladder to reposition it; descend ladder and relocate it. Erect ladder on clean, level surface or use self-levelling ladder. Erect extension ladder in a 4:1 ratio (4 meters up for every 1 meter from the wall) and tie off at top and bottom. Use a footer where necessary. Do not place a ladder in front of a doorway unless locked/guarded Secure work area around the ladder. Clean mud, grass etc, from boots to prevent slippage. Use a tool pouch to carry tools. Do not leave tools and equipment on ladder rungs. Do not over-reach while on ladder. Only one person on a ladder at any one time. Face the ladder at all times. Ensure legs are fully extended and latch is in place Maintain three points of contact on ladder Do not step above the third step from the top. Where working from more than a couple of stories high a double lanyard fall restraint system is to be used. | Low (2) | Workers |
| Undertaking water testing | Slips, trips and falls | Medium (3) | All workers to wear steel capped boots. Onsite inductions to be completed prior to entering the job including identification of any slip trip and fall hazards. Ensure the work area is clear and free of slip, trip and fall hazards | Low (1) | # Workers |
|  | Manual handling injury | Medium (4) | Workers to follow correct manual handling process below. Use mechanical lifting devices and trolleys where possible. Only lift objects within your safe lifting limit or use team lifts. Clear a path, assess the load, get a good grip, ensure a wide stance, bend the knees, keep the spine straight, use the legs to lift, do not twist the trunk, use a smooth action, brace the stomach muscles and lower safely. Ensure clear access. | Low (2) | # Workers |
|  | Drowning | High (6) | Water to be filled only to a depth to ensure full coverage of waterproofing area. Workers to avoid working in the testing area so far as is reasonably practicable. Workers to wear appropriate PPE shoes for the task Workers to work from clean and dry areas as much as reasonably practicable. Workers to use spotters and duel person working arrangements where reasonably practicable | Low (2) |  |
|  | Electrocution or injury due to contact with services or cables | High (6) | Do not work near live services. Leather steel capped rubber soled boots to be worn. Fibreglass ladders to be used. All electrical items including extension leads and RCD’s must be inspected prior to use for any damage and must be tagged. All services to be located, marked and isolated. Services to be locked out and tagged out. Workers to keep plant and equipment clear of wires | Low (2) | # Workers |
| Clean-up site Pack up materials and tools | Injury to workers and the public | Medium (3) | Ensure site is secure from public access. Clean all equipment using approved equipment. Wrap up all waste materials and place in plastic bags and seal for appropriate disposal. Remove all tools and product or store in secure location. Only competent workers to be used. | Low (1) | Workers |

| Personal Protective Equipment (PPE) required: | Enclosed leather steel capped boots (As per AS/NZS 2210) Safety Glasses (As per AS/NZS 1337) Leather Gloves (As per AS/NZS 2161) Hard Hats (As per AS/NZS 1801) Sun hats, sun cream- when working outdoors |
| --- | --- |
| List the training required by workers to complete the work: List the qualifications of personnel doing the work: | Relevant trade certificate Construction induction certificate (White card) Induction into SWMS Experience in operation of machinery |
| List the Permits, Certificates, SafeWork NSW Approvals, required to completed the work: | All necessary permits to be completed on site |
| Relevant Legislation: For further details refer to the Legal Requirements Register | Work in accordance with Work Health and Safety Act 2011 and Work Health and Safety Regulation 2017, workplace injury management and workers compensation act 1998 no. 86, AS/NZS1337.1 Personal eye protection-Eye and face protectors for occupational applications (2010), AS/NZS 1269.3 Occupational Noise management-Hearing protection program (2005), AS/NZS 1801 industrial safety helmets (1997), AS/NZS 2210.1 Safety, protective and occupational footwear-Guide to selection, care and use (2010) & AS/NZS 2210.2 to 2210.9 Occupational protective footwear. SafeWork NSW Codes of Practice: - Construction Work - Managing risks of plant - Managing the risk of falls - Work Health and Safety Consultation, Co-operation and Co-ordination - Managing the Work Environment and Facilities - First Aid in the Workplace - Hazardous Manual Tasks - How to Manage Work Health and Safety Risks |
| List the plant / equipment that will be used on site: | Power Tools and Hand Tools |
| List maintenance checks for plant & equipment: | Daily pre-start checks, maintenance as per manufacturers guidelines |
| Frequency of site/workplace inspections: Site Inspections will be completed by: | Daily site inspections & job completion inspections to be completed by Site Supervisor. |
| Hazardous Substances: | Water dye |

# Sign Off and Approvals

| Person responsible for site supervision of the work, inspecting and approving work areas, approval of and compliance with SWMS, protective measures, plant, equipment: |  |
| --- | --- |
| Name and position: | Signature: |
| SWMS Prepared by: Andrew Johnston – Compliance OH&S | Signature: |

| Risk Level | Description of consequence or impact | Consequence | Likelihood/Probability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Unlikely (1) | Possible (2) | Almost Certain (3) |
| High Level of harm | Actual/Potential fatality, disability or irreversible damage. Major structural failure/damage. Off-site environmental discharge/release not contained and significant long-term environmental harm. | Major (3) | Medium (3) | High (6) | High (9) |
| Medium Level of harm | Actual/Potential temporary disability, MTI or LTI. Structural failure/damage, >1-day outage. On-site environmental discharge/release contained, minor remediation, short-term environmental harm. | Moderate (2) | Low (2) | Medium (4) | High (6) |
| Low Level of harm | Incident that has the potential to cause persons to require first aid. Environmental discharge/release immediately contained, minor level clean-up with no short term environmental harm. | Minor (1) | Low (1) | Low (2) | Medium (3) |
| Level | Likelihood/Probability |  |  |  |  |
| Almost Certain | Occurs frequently; >66% chance of occurring |  |  |  |  |
| Possible | Could happen occasionally; >33% but <66% chance of occurring |  |  |  |  |
| Unlikely | May occur only in exceptional circumstances; <33% chance of occurring |  |  |  |  |
| Class/Ranking | Description/Requirements |  |  |  |  |
| High 6, 9 | Stop immediately. Implement permanent preventative controls. |  |  |  |  |
| Medium 3, 4 | Will require operational planning. Actions will be recorded on a SWMS. |  |  |  |  |
| Low 1, 2 | Will require detailed pre-planning. Actions will be recorded on a SWMS. |  |  |  |  |

SWMS has been developed using the hierarchy of controls to reduce the risk and likely hood of an incident/injury occurring. The use of Elimination, Substitution, Engineering/isolation, Administrative and PPE measures will be used in the above order of preference wherever practicable.

Induction confirmation:

This SWMS has been developed in consultation with the workers and has been read and signed by all workers involved in this activity. I note that I have read and understood the above safe work method statement and will ensure work is completed accordingly.

| Date: | Name: | Signature: |
| --- | --- | --- |
| Date: | Name: | Signature: |