[Insert Work Activity Here] example Suspended Scaffold (Swing Stage) Operations

| PCBU: | [Insert PCBU Here] example Robertson’s Remedial and Painting Pty Ltd 10/56 Buffalo Road, Gladesville NSW 2111 Phone: (02) 9181 3519 ABN: 16 140 746 247 | Workplace location: | [Insert Site Address Here] |
| --- | --- | --- | --- |
| Works Manager: | ■ Project Manager Name | Date SWMS provided to PC: | [Insert Date Here] |
| Work activity: | [Insert Work Activity Here] example Suspended Scaffold (Swing Stage) Operations | Principal Contractor (PC): | [Insert Principal Contractor (PC) Here] |
| High Risk Construction Work (HRCW): ☑ Risk of a person falling more than 2 metres ☐ Likely to involve disturbing asbestos ☐ Work in or near a shaft or trench deeper than 1.5 m or a tunnel ☐ Work on or near chemical, fuel or refrigerant lines ☐ Tilt-up or precast concrete elements ☐ Work on a telecommunication tower ☐ Temporary load-bearing support for structural alterations or repairs ☐ Use of explosives ☐ Work on or near energised electrical installations or services ☐ Work on, in or adjacent to a road, railway, shipping lane or other traffic corridor in use by traffic other than pedestrians ☐ Demolition of load-bearing structure ☐ Work in or near a confined space ☐ Work on or near pressurised gas mains or piping ☐ Work in an area that may have a contaminated or flammable atmosphere ☑ Work in an area with movement of powered mobile plant ☐ Work in areas with artificial extremes of temperature ☐ Work in or near water or other liquid that involves a risk of drowning ☐ Diving work |  |  |  |
| Person responsible for ensuring compliance with SWMS: | ■ Supervisor Name | Date SWMS received: | [Insert Date Here] |
| What measures are in place to ensure compliance with the SWMS? | Toolbox meetings, SWMS sign off, job observations and supervision review. If issues with the SWMS or new hazards are identified, the supervisor must be notified. When changes are made to SWMS, it will be communicated to all workers. |  |  |
| Person responsible for reviewing SWMS control measures: | ■ Project Manager Name | Date SWMS received by reviewer: | [Insert Date Here] |
| How will the SWMS control measures be reviewed? | The control measures implemented will be reviewed and if necessary, revised annually or if work methods change, the control measures are not effective in controlling the risk, a new hazard/risk is identified or following an incident. The SWMS will be reviewed in consultation with workers and/or others who may be affected by the SWMS. Any changes to the SWMS will be communicated with workers at induction, daily pre-starts and toolbox talks. |  |  |
| Reviewer’s signature: | ■ Project Manager Name | Review date: | [Insert Date Here] |
| This SWMS must be kept and be available for inspection until the high-risk construction work to which this SWMS relates is completed. If the SWMS is revised, all versions should be kept. If a notifiable incident occurs in relation to the high-risk construction work in this SWMS, the SWMS must be kept for at least 2 years from the date of the notifiable incident. |  |  |  |

Brief description: [Insert max. 5x line description summary]

| STEP-BY-STEP SAFE WORK METHOD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Work Activity / Task | Hazard / Risk | Risk Rating (Pre) | Controls — Hierarchy of Control | Risk Rating (Post) | Responsible | Hold Point / Verification / Stop-Work Trigger |
| PHASE 1: PRE-COMMENCEMENT — SITE ESTABLISHMENT AND CERTIFICATION |  |  |  |  |  |  |  |
| 1.1 | Site Induction, Daily Sign-In and SWMS Induction | • No site awareness • SWMS controls not verified • Unauthorised site access | Low (1) | Administrative Controls: • Sign in daily — record in site register • Complete site induction on first day — record it • Read and sign this SWMS before starting — join PM WhatsApp group • Attend weekly toolbox talk — covers tasks, hazards, controls, site changes • Show your White Card — supervisor records it • Complete swing stage rescue drill before boarding the platform | Low (1) | Supervisor | STOP-WORK TRIGGER • No White Card • Not site/SWMS inducted • Rescue procedures not completed (Step 4.1) RISK CODE: SYS-L1 |
| 1.2 | Swing Stage Design and Engineering Certification | • Platform failure — inadequate design • Structure inadequate for suspension loads • Incorrect load calculations • Non-certified / modified equipment | High (9) | Engineering Controls: • Use suspension points with SWL marked — wire ropes certified, minimum 10:1 safety factor • Hoist motors rated for the load — safety rope must be independent of working rope • Test platform levelling and hoist motors before use Administrative Controls: • Keep engineering drawings and load tables on site • Confirm design against actual building — not assumed from plans • Call the design engineer before changing platform size, load, or worker numbers • Keep engineer contact details on site | Low (2) | Do: Supervisor / Verify: Engineer | HOLD POINT 1.2 1. Engineer design certified & on site 2. Structural assessment complete — anchor substrate confirmed 3. All equipment certified & tagged 4. AS/NZS 1576.4 & AS 2550.10 compliant STOP-WORK TRIGGER • Design not on site / not current • Structural capacity unconfirmed • Certification expired • Unapproved modification • Equipment damaged or non-compliant RISK CODE: WAH-H9 |
| 1.3 | Electrical Hazard Clearance | • Electrocution — energised conductors • Wire rope contact with power lines • Platform contact with building wiring/conduit • Wet conditions — increased electrical risk | High (9) | Isolation / Engineering Controls: • Survey all electrical hazards on the building face — map them before setting travel path • All energised equipment in platform path must be isolated by a licensed electrician or physically guarded • Confirm safe approach distances to overhead power lines — set travel limits to stay clear • Guard all exposed building wiring in the platform path — route wire ropes away from electrical equipment • In wet weather — all exposed electrical connections must be protected or isolated Administrative Controls: • Display electrical hazard map on platform and at ground level • Brief all workers: if electrical contact suspected — do not touch wire ropes or platform — lower from ground controls only if safe — call 000 • Engage an electrician to isolate equipment in the platform path where possible | Low (2) | Supervisor / Operator | HOLD POINT 1.3 1. Hazard survey complete & all hazards mapped 2. Platform path: live equipment isolated/protected 3. Overhead line clearances confirmed & limits set STOP-WORK TRIGGER • Hazard not mapped • Live equipment in platform path • Wire rope near power line • Wet + exposed wiring • Electrical contact → call 000 RISK CODE: ELE-H9 |
| PHASE 2: ESTABLISHMENT — ANCHOR, PLATFORM INSTALLATION AND EXCLUSION ZONES |  |  |  |  |  |  |  |
| 2.1 | Exclusion Zone and Drop Zone Management | • Falling tools/materials/debris • Wire rope/equipment failure — falling components • Platform swing — strike workers/public • Unauthorised zone access | Medium (3) | Engineering / Isolation Controls: • Barricade exclusion zone with mesh fence, water barriers, or hoarding — tape alone is not acceptable • Install catch platform or fan at building entries within the drop zone • Fit scaffold netting on platform perimeter — attach tool lanyards to all hand tools • Exclusion zone minimum 3m from platform edge plus half platform height or 6m (whichever is less) Administrative Controls: • Post "DANGER — Overhead Work — No Entry" signs at all exclusion zone access points • Notify residents and public before work starts • Station a spotter at ground level when pedestrians are near the zone • No throwing or dropping anything from the platform • Brief all other trades — no entry to exclusion zone without supervisor approval | Low (1) | Supervisor / Worker | HOLD POINT 2.1 1. Barriers in place before descent 2. Signage at all access points 3. Residents/public notified STOP-WORK TRIGGER • Zone breached • Barricading displaced/removed • Material dropped from platform • Pedestrians/residents in drop zone • Signage removed RISK CODE: SYS-M3 |
| 2.2 | Roof Anchor and Suspension Point Setup | • Anchor failure — platform drop • Roof edge fall — anchor installation • Incorrect counterweight calculation • Outrigger beam failure | High (9) | Engineering Controls: • Install anchors exactly as per engineering design — confirm substrate is adequate before attaching ropes • Counterweights: calculate minimum 3:1 safety factor against overturning — secure so they cannot shift — no loose or improvised weights • Outriggers: seat squarely, secure against uplift — do not exceed design cantilever length Administrative Controls: • Keep engineering drawings and load tables on site • No changes to anchor positions without written engineer approval • Competent person signs off every anchor point before wire ropes are attached • Photograph each anchor installation and keep on file | Low (2) | Do: Lead Technician / Verify: Supervisor | HOLD POINT 2.2 1. Anchor positions match design 2. Counterweights: weight & placement per design 3. Outrigger installation signed off 4. Hardware certified & tagged STOP-WORK TRIGGER • Anchor position off-design • Counterweights loose/improvised/underweight • Outrigger beyond cantilever limit • Substrate unconfirmed • Sign-off not obtained RISK CODE: WAH-H9 |
| 2.3 | Platform Installation and Rigging | • Platform failure — assembly • Wire rope failure • Safety device malfunction • Platform swing/spin during lowering • Rigger fall — roof edge rigging | High (9) | Engineering Controls: • Working rope and safety rope must be on separate suspension points — no shared failure point • Install rope guides to prevent snagging • Test all safety devices before loading: overspeed governor, tilt switch, limit switches, rope lock — all must work Administrative Controls: • Only workers with Advanced Scaffolding Licence (SA) may rig the platform • Inspect wire ropes before reeving — reject any with kinks, broken wires, corrosion, or diameter loss over 10% • Assemble platform per manufacturer instructions — all pins, bolts, guardrails, toe boards, and deck panels secure • Complete rigging checklist — competent person signs off — platform not used until sign-off done • Inspect rigging before each use and after any incident or weather event | Low (2) | Do: Lead Technician / Verify: Supervisor | HOLD POINT 2.3 1. SA licence verified 2. Wire ropes inspected & certified 3. Safety devices tested & functional 4. Assembly per spec; signed off STOP-WORK TRIGGER • Wire rope damaged • Safety device non-functional • Assembly incomplete • No SA licence • Safety rope not independent • Sign-off not obtained RISK CODE: WAH-H9 |
| PHASE 3: OPERATIONS — SWING STAGE WORK AND MONITORING |  |  |  |  |  |  |  |
| 3.1 | Swing Stage Pre-Start and Daily Inspection | • Undetected defect — platform failure • Wire rope deterioration • Safety device failure • Overnight weather damage | High (6) | Engineering Controls: • Run hoist motors at no-load — confirm smooth, no jerking; test brakes — platform must hold when motor stops • Check platform level; confirm guardrails, toe boards, and deck panels secure Administrative Controls: • Complete daily pre-start checklist before anyone boards — competent person signs it • Inspect wire ropes full length — no broken wires, kinks, corrosion, or loose terminations • Test all safety devices: overspeed governor, tilt switch, rope lock • Check anchors and counterweights — no movement or damage since last use • Any defect found — lock out platform, do not use until repaired • After storm, heavy rain, or high wind — inspect again before use • Competent person does full weekly inspection in addition to daily checks | Low (2) | Do: Lead Technician / Verify: Supervisor | HOLD POINT 3.1 1. Pre-start: complete, signed, defect-free STOP-WORK TRIGGER • Pre-start defect found • Safety device non-functional • Wire rope damaged • Anchor movement detected • Platform not level • Motor fault / brake not holding • Pre-start not signed off RISK CODE: WAH-H6 |
| 3.2 | Swing Stage Operation and Repositioning | • Platform failure — operation • Fall from platform • Entrapment — platform vs building • Platform swing/pendulum • Overloading | High (6) | Engineering Controls: • Each worker adjusts rope grab to fit before descent • Operate both hoists together to keep platform level • Fit building face rollers or guide rails to stop the platform spinning • All tools and materials secured on platform — nothing loose Administrative Controls: • Maximum two workers unless design allows more • Stay within arm's reach — do not lean over guardrails — no sudden movements • To reposition: raise to roof level, move along building face, then lower — no sideways movement while lowered • Confirm load is within rated capacity before every descent: workers + tools + materials • Confirm radio or visual communication with ground and roof crew before descending • Stop operations if wind exceeds 40 km/h or the platform design limit — whichever is lower | Low (2) | Operator / Worker | HOLD POINT 3.2 1. All workers: harness + rope grab on independent safety line 2. Load within rated capacity 3. Wind within operational limit 4. Comms confirmed: platform ↔ ground/roof STOP-WORK TRIGGER • Worker not on independent safety line • Platform overloaded • Wind exceeds limit • Platform not level (>150mm) • Uncontrolled swing/spin • Comms lost • Motor fault / safety device activation RISK CODE: WAH-H6 |
| 3.3 | Wind and Weather Monitoring | • Platform swing/instability — wind • Lightning strike — platform/wire ropes • Rain — slippery platform • Sudden wind change/gust • Reduced visibility — fog/rain | Medium (4) | Engineering Controls: • Anemometer on platform or at working height — reading visible to operator at all times • Rig platform tie-off points to secure to building face in high wind • Secure wire ropes and platform when not in use Administrative Controls: • Wind limits: sustained >40 km/h or gusts >50 km/h — stop work, secure platform; >60 km/h — stop all outdoor work, secure materials • Lightning: stop immediately if thunder heard or lightning seen — lower platform, all workers off roof — do not restart until 30 minutes after last strike or thunder • Rain: check platform for slip risk before continuing • Check Bureau of Meteorology forecast before every shift • Record wind readings in daily log | Low (2) | Supervisor / Operator | STOP-WORK TRIGGER • Wind >40 km/h sustained / gusts >50 km/h • Lightning within 10 km • Heavy rain / reduced visibility • Storm warning active • Wind change causing instability • Operator judges unsafe RISK CODE: ENV-M4 |
| 3.4 | Residents and Public Interface | • Fall — residents/public • Falling objects — residents/public • Overspray/dust — occupied areas • Noise/access disruption • Unauthorised zone entry | Medium (3) | Engineering Controls: • Barricade all work zones accessible to residents or public — mesh, hoarding, or water barriers — not tape alone • Barricade the full fall line below all elevated work Administrative Controls: • Give residents written notice minimum 48 hours before starting — include dates, times, and type of work • Work only within approved hours — no exceptions without written approval from council or strata • Post "CONSTRUCTION WORK IN PROGRESS" signs at building entry and work zones with contact details • Station a spotter at ground level when pedestrians are near the zone | Low (1) | Supervisor / Worker / Sub-Contractor | STOP-WORK TRIGGER • Resident/public in exclusion zone • Barricade displaced/removed • Health complaint (dust/fumes/noise) → notify PM • Work outside approved hours RISK CODE: SYS-M3 |
| 3.5 | Balcony Security (Occupied Residential Apartments) | • Resident access — balcony work area • Seal tampered — dust/silica ingress | Medium (3) | Administrative Controls: • Balcony access for workers: approved by supervisor or PM only • Dust seals must be continuous — no gaps between work area and occupied unit • If any seal is found damaged or missing — stop work in that area, reinstate seal before continuing • Report any resident access or tampered seal to PM immediately — PM records in incident register and notifies resident representative | Low (2) | Supervisor / Worker / Sub-Contractor | STOP-WORK TRIGGER • Resident in balcony work area • Seal tampered/missing → reinstate, notify PM RISK CODE: SYS-M3 |
| 3.6 | Hot and Dangerous Weather | • Heat stress/stroke/dehydration • Slip — wet surfaces • Wind — dislodged materials/instability • Lightning strike • UV exposure | Medium (3) | Engineering Controls: • Cool drinking water within 50m of all work positions at all times Administrative Controls: • Check Bureau of Meteorology forecast every morning — reschedule if extreme heat forecast • Wind: >40 km/h — stop all elevated work; >60 km/h — stop all outdoor work, secure materials • Lightning: stop immediately if thunder heard or lightning seen — lower platform, all workers off roof — wait 30 minutes after last event before resuming • Schedule shade breaks and remind workers to drink water regularly | Low (2) | Supervisor / Worker / Sub-Contractor | STOP-WORK TRIGGER • Heat stress signs observed • Wind exceeds trigger • Lightning within 10 km • Rain: surfaces unsafe RISK CODE: SYS-M3 |
| PHASE 4: EMERGENCY RESPONSE — RESCUE PROCEDURES |  |  |  |  |  |  |  |
| 4.1 | Swing Stage Rescue Procedures | • Suspension trauma (positional asphyxia) • Delayed rescue • Platform inaccessible from ground • Multiple casualty scenario | High (9) | Engineering Controls: • Platform hoist must be operable from ground level for emergency lowering • If hoist fails — alternative rescue (rope rescue, EWP, or adjacent access) available and reachable within 6 minutes • Trauma straps on all harnesses — worker or rescuer deploys them immediately on rescue Administrative Controls: • Write a site-specific rescue plan for this installation — three scenarios: (a) incapacitated worker on platform, (b) worker suspended on safety line, (c) medical emergency requiring lowering • Rescue equipment on site and accessible — second access route to platform height confirmed before work starts • All crew trained and rescue drill completed at project start — record it • Brief all workers on suspension trauma: fit trauma straps, know the symptoms, know how to self-deploy • Display rescue procedure at ground level and on the platform • Minimum two persons on site when platform is in use — one at ground level at all times • Pre-notify emergency services of swing stage work: location, height, access route • First aid qualified person on site at all times (HLTAID011) | Low (2) | Do: Supervisor / Verify: Lead Technician | HOLD POINT 4.1 1. Site-specific rescue plan on site 2. Rescue equipment on site; second access to height confirmed 3. Rescue drill completed & recorded 4. Trauma straps fitted; workers briefed 5. Minimum 2 persons on site STOP-WORK TRIGGER • Rescue plan not documented or not site-specific • Rescue drill not completed • No second means of access to working height available • Trauma straps not fitted to harnesses • Solo worker on swing stage — minimum two persons required • Emergency lowering not functional • First aid not available on site RISK CODE: WAH-H9 |

CCVS Table

| Task | Critical Control | Who Checks | How Often | What They Look For |
| --- | --- | --- | --- | --- |

| WORKER SWMS INDUCTION SIGN-OFF |  |  |  |
| --- | --- | --- | --- |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |

| WORKER SWMS AMENDMENT SIGN-OFF |  |  |  |
| --- | --- | --- | --- |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |

If work methods, site conditions, or hazards change — stop, complete this table, re-brief all workers, obtain new sign-offs before resuming. Required under WHS Regulation r.300(3).

| SWMS AMENDMENTS |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Under WHS Act s18, “reasonably practicable” requires consideration of likelihood of risk, degree of harm, what the person knows about the hazard, availability and suitability of controls, cost vs risk. If you cannot show how that decision was made, the action becomes harder to defend after an incident. |  |  |  |  |  |  |
| Date | Work Activity / Task | Hazard / Risk | Risk Rating (Pre) | Controls — Hierarchy of Control | Risk Rating (Post) | Responsible |

This SWMS is a living document. It must be reviewed if work methods change, new hazards are identified or following any incident. Supervisor/Project Manager is responsible for ensuring this document remains current. NSW WHS Regulation r.300(3).

| WORKER SWMS AMENDMENT SIGN-OFF |  |  |  |
| --- | --- | --- | --- |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |

If work methods, site conditions, or hazards change — stop, complete this table, re-brief all workers, obtain new sign-offs before resuming. Required under WHS Regulation r.300(3).

| SWMS AMENDMENTS |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Under WHS Act s18, “reasonably practicable” requires consideration of likelihood of risk, degree of harm, what the person knows about the hazard, availability and suitability of controls, cost vs risk. If you cannot show how that decision was made, the action becomes harder to defend after an incident. |  |  |  |  |  |  |
| Date | Work Activity / Task | Hazard / Risk | Risk Rating (Pre) | Controls — Hierarchy of Control | Risk Rating (Post) | Responsible |

This SWMS is a living document. It must be reviewed if work methods change, new hazards are identified or following any incident. Supervisor/Project Manager is responsible for ensuring this document remains current. NSW WHS Regulation r.300(3).

| RISK RATING MATRIX — Consequence × Likelihood |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Risk Level | Description of Consequence or Impact | Consequence | Unlikely (1) | Possible (2) | Almost Certain (3) |
| HIGH | Actual/Potential fatality, disability or irreversible damage. Major structural failure. Off-site environmental discharge not contained, significant long-term environmental harm. | Major (3) | Medium (3) | High (6) | High (9) |
| MEDIUM | Actual/Potential temporary disability, MTI or LTI. Structural failure/damage, >1-day outage. On-site environmental discharge contained, minor remediation, short-term harm. | Moderate (2) | Low (2) | Medium (4) | High (6) |
| LOW | Incident requiring first aid. Environmental discharge immediately contained, minor clean-up, no short-term environmental harm. | Minor (1) | Low (1) | Low (2) | Medium (3) |
| Level | Likelihood / Probability |  |  |  |  |
| Almost Certain (3) | Occurs frequently; >66% chance of occurring |  |  |  |  |
| Possible (2) | Could happen occasionally; >33% but <66% chance of occurring |  |  |  |  |
| Unlikely (1) | May occur only in exceptional circumstances; <33% chance of occurring |  |  |  |  |
| Class / Ranking | Description / Requirements |  |  |  |  |
| High 6, 9 | STOP immediately. Implement controls. Controls recorded on a SWMS. |  |  |  |  |
| Medium 3, 4 | Planned control. Controls recorded on a SWMS. |  |  |  |  |
| Low 1, 2 | Managed via routine procedure. |  |  |  |  |
| Note — WHS Act s.18 “reasonably practicable”: Requires consideration of the likelihood of the risk, degree of harm, knowledge of the hazard, availability and suitability of controls, and cost vs. risk. If you cannot show how that decision was made, the action becomes harder to defend after an incident. |  |  |  |  |  |

| PRE-REQUISITES — Must be in place before any phase commences |  |  |  |
| --- | --- | --- | --- |
| PPE — All Persons on Site | • Steel-capped safety boots • High-vis vest or long sleeves • Safety glasses/eye protection • Safety gloves (general handling) • Hard hat during installation and dismantling | Additional PPE — Working at Heights | • Full body harness (AS/NZS 1891.1) with rope grab on independent safety line • Trauma straps fitted to all harnesses • P2 respirator (AS/NZS 1716) — mandatory for silica, spray painting, lead and solvent-based products • Hearing protection (AS/NZS 1270) if ≥85 dBA • Sun protection SPF 50+ • Wet weather gear (non-slip footwear) for rain operations • Hard hat with chin strap during any elevated work |
| Licences / Qualifications | • White Card (all workers) • Advanced Scaffolding Licence (SA) — for swing stage rigging • EWP Licence (WP class) • Working at Heights training (current within 2 years) • IRATA/ARAA certification for rope rescue if used • Trade certificate or demonstrated competence in painting/surface preparation • First Aid HLTAID011 — minimum one person on site at all times | Permits & Approvals | • Engineer-certified swing stage design (platform loads, suspension capacity, counterweight design, AS/NZS 1576.4 compliance) • Building structural assessment confirming anchor substrate capacity • Equipment certification (all components tested and tagged) • Hot Works Permit if applicable • Confined Space entry permit if applicable • HY Strip Scaffold Work Permit where required per HY Temporary Works procedure • Electrical hazard survey and de-energisation documentation |
| Plant & Equipment | • Swing stage platform (hoist motors, wire ropes, safety devices, guardrails, toe boards) • Roof-mounted anchor points / parapet clamps • Outrigger beams and counterweight system • Anemometer (wind speed meter) • EWP (boom or scissor lift) • Pressure washer • Airless spray unit • Power tools with RCDs • Ladders • Scaffold netting • Tool lanyards | Hazardous Substances | • Chemical register on site — all paints, primers, sealers, solvents listed with current SDS (≤5 years old) • SDS available on site at all times • Flammable liquids stored in compliant cabinet per AS 1940 • P2 mask mandatory for silica, spray painting, lead, or solvent-based product tasks |
| Consultation | • Workers consulted in preparation of this SWMS per NSW WHS Regulation r.299(3) and r.300(2) • SWMS reviewed with all workers before work commences — workers sign off before starting • Rescue drill completed and recorded at project commencement • All workers members of PM’s WhatsApp group for real-time hazard communication | Legislative Basis | • Work Health and Safety Act 2011 (NSW) • WHS Regulation 2017 (NSW) rr.291–302 • AS/NZS 1576.4 Scaffolding — Suspended Scaffolding • AS/NZS 1891.1 Industrial Fall-Arrest Systems • AS 2550.10 Cranes — Suspended Personnel Platforms • AS/NZS 2311 Painting of Buildings • AS 4361.2 Guide to Lead Paint Management • AS 1940 Flammable and Combustible Liquids • SafeWork NSW Construction Work Code of Practice • Managing the Risk of Falls at Workplaces Code of Practice • Managing Risks of Hazardous Chemicals in the Workplace Code of Practice |