Project Safety Management Plan

Sulphide Junction

SIG-05

Version control

| Version | Change from previous | Date | Comment |
| --- | --- | --- | --- |
| 0.1 | First draft | 17/4/10 |  |
| 0.2 | Removals & additions | 28/4/10 |  |
| 0.3 | additions | 29/4/10 | Draft for comment |
| 0.4 | Alterations & additions | 17/5/10 | Draft for comment |
| 0.5 | Further alterations & additions | 21/5/10 | Draft for comment |
| 0.6 | Further alterations & additions | 25/5/10 | Draft for comment |
| 0.7 | Further alterations & additions | 26/5/10 | Draft for comment |
| 0.9 | Further alterations & additions | 7/6/10 |  |
| 0.10 | Update and Formatting | 1/6/10 |  |
| 0.11 | Further update | 10/6/10 | Final for Possession. Review to be undertaken post Possession (July) |
| 1.0 | Release |  |  |
| 1.1 | Updated post possession 1 | 10/7/10 | Added scope of work |
| 2.0 | Updated post review | 07/09/10 |  |
| 2.1 |  | 05/10/10 |  |

Contents

# 1	Project Summary	6

# 1.1	Project Background and History	6

# 1.2	Purpose of the Plan and Safety Objectives	6

# 1.3	Scope of the Plan	8

# 1.4	Scope of Work Summary	8

# 1.5	Main Activities and Method of Work	10

# 1.6	Methodology	11

# 1.7	Key Persons	12

# 1.7.1	Contact details	12

# 1.7.2	Project Management Structure	12

# 1.8	Timeline and Project Schedule	12

# 1.8.1	Key Activity	13

# 1.8.2	Possessions	14

# 1.9	High Risk Activities	15

# 1.10	Task Allocation	16

# 1.1	Document Interfaces	16

# 2	Management responsibility	18

# 3	Safety Risk Management	21

# 3.1	Safety Change Management	22

# 3.2 Hazard Identification and Risk Assessments	23

# 3.2	Registers	29

# 3.3	Hazard Identification and Control	32

# 3.4	Controls	33

# 3.5	Review Controls	35

# 3.6	SWMS Review	36

# 3.7	Health & Fitness	36

# 3.7.1	Health Assessments	36

# 3.7.2	Alcohol and Other Drugs	38

# 3.7.3	Fatigue	38

# 3.7.4	Management of fatigue in contractors and suppliers	40

# 4	Communication and Consultation	40

# 4.1	Consultation Arrangements and Issue Resolution	46

# 4.2	Communication requirements	48

# 4.2.1	Safety Training Requirements	48

# 4.2.2	Inductions	49

# 4.3	Safety Notices	49

# 4.4	Changes to Safety Requirements and Organisational Policies	50

# 4.5	Communication Management	50

# 5	Purchasing	51

# 5.1	Procurement and Goods Delivery	51

# 5.2	Handling, Storage, and Packaging	56

# 5.2.1	Substances/Materials Hazardous to Health	56

# 5.3	Materials/Equipment Handling	58

# 5.4	Handling Storage Packaging and Delivery	60

# 5.4.1	Delivery	60

# 5.5	Disposal	61

# 6	Contractor Management	62

# 6.2	Site Specific Assessment	63

# 7	Incident Management	65

# 8	Incident/Hazard Investigation	67

# 8.1	Serious Incident Reporting	68

# 8.2	Emergencies	69

# 8.3	Injuries and RTW	70

# 8.3.1	Injuries	70

# 8.4	Unexpected Findings of Suspected Asbestos Material	72

# 9	Training	74

# 9.1	Safety Induction	75

# 9.2	Hazard Specific Training	76

# 9.3	General Training	77

# 10	Inspection, Testing, and Servicing	77

# 10.1	Site Safety Inspection	77

# 10.2	Plant and Equipment Inspections, Testing and Services	80

# 11	Document Control	82

# 10.1 Document control	82

# 10.2 Records management	83

# 11.1	Training Register and records	85

# 12	Operational control	86

# 12.1	Design Verification	86

# 12.2	Design Hazard and Risk Analysis	86

# 12.3	Safety in Design	87

# 12.4	Design Review and Acceptance	87

# 12.5	Preliminary Design Review (PDR)	87

# 12.6	Critical Design Review (CDR)	88

# 12.7	Design Change	88

# 13	Authorities and Permits	90

# 13.1	Requesting, Receiving and Returning Work Permits.	90

# 13.2	PSA Project Safety Agreement	91

# 14	Securing and Controlling Access to Sites	91

# 14.1	Access control	91

# 14.2	Restricted areas	92

# 14.2.1	Downer EDI Areas	92

# 14.2.2	Novo Rail Worksite	92

# 14.3	Safety Signs	96

# 15	Monitoring, Reviewing and Reporting	98

# 15.1	Contractor Safety performance Evaluation	98

# 15.2	Safety Observation	98

# 15.3	Implementation Plan - Safety Concepts	98

# 15.4	Site-Specific Safety Management Plans	98

# 15.5	SWMS, SWI, and Work Plans Document Listing	100

# 15.6	Review	100

# 15.7	Review of this Document	101

# 15.8	Post-Completion Review of Safety Issues	101

# 15.9	Corrective Action	101

# 16	Reporting	102

# 17	References	102

# Appendix A	Scope of work	103

# Appendix B	Project schedule	103

# Project Schedule	103

# Appendx C Principal Contractor OHS Policy	103

# Appendix C	Hazard idetification forms and risk assessments	103

# Appendix D Safety change documentation	103

# Appendix D	Registers	103

# Appendix E	Reporting	103

# Appendix F	Communication and consultation plan	103

# Appendix G	Training plan	103

# Appendix H	Inspection schedule	103

# Appendix I	Inspection forms	103

# Appendix J	Site incident response procedure	103

# Appendix K	Project management organisation structure	103

# Appendix L	Emergency Contact list	104

# Appendix M	Review Schedule	104

# Appendix N	Document list	104

# Appendix O	Site-Specific Management Plans	104

# Appendix P	SWMS Review Tool	104

# Appendix Q	104

# Appendix R	Security Plans	104

# Appendix S	104

# Project Summary

# Project Background and History

Novo Rail is redeveloping the Downer EDI Rail Cardiff Manufacturing Centre at Sulphide Junction in preparation for the fit out of the A Set Body shells arriving from China into complete train units.

The manufacturing work on the new A Sets commenced when the pre-preparation test vehicles    arrived in August 2009.  The manufacturing work will continue through to 2013 when the last of the 78 sets will be transferred from Cardiff to Auburn for operations.  The PPP (Private Public Partnership) contract includes an option to extend the contract for a further 20 additional trains, and should this be exercised the work will continue into 2014.

The purpose of the Sulphide Junction project is to facilitate improved access to the Downer EDI Rail (DEDIR) Cardiff manufacturing Centre, by the provision of direct signalled movements between the Down Siding and the Up Main, and the full signalling of existing movements from the Down Main to the Down Siding, which currently require the operation of the ‘E’ frame mechanical points.

The RailCorp process for nomination of Principle Contractors has been used.  Due to the overall site control aspects, for the duration of the project, the Principal Contractor has been identified as being RailCorp.  Novo Rail has allocated Anthony Davis as the Principal Contractor’s Representative.  In exceptional circumstances the Principal Contractor’s Representative role will be undertaken by a competent Novo Rail person, nominated in writing by the Project Manager.  The nominated person, as a minimum will have:

- Attended RailCorp SMS Training (Safety Management System Essentials) and Construction and Maintenance Planning; Relevant competence to provide required site management, coordination and overarching Supervision.

The delineation of the rail corridor on the site is in disrepair, with a number of private dwellings land infringing unimpeded onto the site zone. Novo Rail will ensure that boundaries (with the exception of areas within the rail corridor zone – covered by specific protection plans) will be delineated and physically protected by barricading, for the duration of the project.

The maintenance and security of the physical boundaries surrounding the construction zone will be regularly reviewed as part of site inspections, audits or observations and no construction work shall be commenced until the integrity and security of the physical boundaries are confirmed.

## Purpose of the Plan and Safety Objectives

The purpose of the plan is to provide a mechanism for safety planning and risk reduction during the execution of the Sulphide Junction Project in terms of:

- Health safety and welfare (including Novo Rail representatives, contractors, service providers, visitors and members of the public).

- Track side safety (protection of employee, contractors and service providers on and around the track).

- The plan details how Novo Rail and its sub-contractors will meet their obligations for the management of safety on the project.

- The plan has been developed to ensure that those engaged on the Project under Novo Rail (including sub contractors, and labour hire persons) are aware of and comply with the relevant legislative and RailCorp requirements (as detail in the attached SWM’s).

This plan has been updated post the June 12-14 Possession to include new scope of work and the identification of further extensive asbestos contamination on site. It is the intent of the project team to review and/or change this plan when a new phase of work commences or a significant issue is discovered on site.

Prior to commencement of the project a Geotechnical Investigation was arranged by RailCorp through Jeffery and Kataiskas Pty Ltd, which had included within its scope an environmental and waste classification assessment.  EIS undertook this on the Geotechnical firm’s behalf and reported no positive identification of asbestos in the samples collected and analysed.  . Subsequent testing during construction has indentified the presence of asbestos. The project has developed a management plan to treat initial and subsequent discoveries (see appendix W).

Any work involving potential ground disturbance on the project site will be undertaken in accordance with the asbestos management plan as well as this PSMP.  Work must be approved by the Construction Manager prior to commencing on site and have an appropriate and reviewed SWMS.

The objectives of this plan are to:

- Identify the H&S performance indicators that will be used and reported on.

- Identify risk management methods and techniques by which project related hazards, risks and controls will be identified, communicated and reviewed.

- Identify the methods by which health and safety on the project shall be managed, reviewed and (where necessary) corrected.

- Identify health and safety reporting routes.

- Assist to plan and develop key safety aspects including;

  - Communication topics

  - Inspections/audits

  - Surveys

  - Emergency arrangements

  - Controls

  - Welfare provisions

  - Suitable site riles & procedures etc

- Detail emergency response, incident reporting and investigation requirements and processes.

- Detail the communication and consultation methods

- Provide an overview of project responsibility and communication routes.

- Detail safety in procurement (including contract/service provider/supplier) review and communication processes.

- Detail safety training and competency evaluation processes.

Responsibility tables have been developed to summarise an overview of the responsibility requirements in each of the Sections.  The following key provides a reference to the abbreviations and symbols used within the tables.

On this project the following roles, responsibilities and processes will be applied.

Key # - lead role * - support role.  Note not all responsibilities require lead and support roles.  Where none has been defined the responsibility sits with each party in terms of their area of control.

- Project Manager (P), Construction Manager (C), Site Supervisor (S), Health & Safety Advisor (H), General site personnel (G)

| Objectives and Targets | P | S | G | C | H |
| --- | --- | --- | --- | --- | --- |
| Set clear safety objectives and targets and provide clear monitoring and reporting mechanisms. | # |  |  |  |  |
| Identify additional performance indicators in consultation with HSEQ Team as required if required. |  |  |  |  |  |
| Monitor project health and safety performance against identified targets. |  |  |  |  |  |
| Identify trends or areas for improvement in project health and safety performance. |  |  |  |  |  |
| Formally report project performance to the HSEQ Team within stated deadlines. |  |  |  |  |  |
| Monitor project performance against the established goals and targets. |  |  |  |  |  |
| Discuss performance indicators with staff using the lead indicators to improve safe systems of work. |  |  |  |  |  |

Sulphide Junction Safety Targets have been set and are included in Project Score card

The score card has both lead and lag targets (all with weightings).  The aim is to recognise and measure the proactive activities as well as the negative with an output of driving the overall safety culture forward.  The score card will be reported on monthly.

Appendices that are updated on site in hard copy or have regular superseding versions have the sharepoint folder location shown in this plan.

## Scope of the Plan

Those that will be affected by the scope of the plan will be;

- RailCorp & rail movements (dependent on work activities)

- Other Stakeholders

  - Downer EDI Workshops facility

  - Project management

  - Third party service providers engaged on the project

  - Visitors to site

  - Novo Rail Project Management

## Scope of Work Summary

The Sulphide Junction project site is situated between the 151.500 to 153.900 kilometres. The following scope is currently being completed in line with this update version of the plan. None of the following works will be undertaken without a signed Project Safety Agreement and where applicable design approvals. The signed PSA’s for this project can be found on sharepoint .  The approved designs can be found on sharepoint .

SIG05 - Sulphide Junction > Project Records > Execution > 08 Design and System Engineering > 8.1 AFC

Note no PSA’s exist in this folder.  Please upload PSA’s into the folder and provide relevant link.

- Installation of a new crossover 153 between the Up and Down Main lines on the Sydney side of the existing 155 points.

Crossover installation shall be a combination of slide and lift in. The slide portion will be done using strong backs and rollers. The smaller portion will be lifted into the down main using a Franna crane. To enable this to take place SWMS and Lifting plans will be compiled.

Prior to the lift existing track will be lifted out and the ballast removed by excavator down to design level. SWMS, excavation plans and lifting plans will be utilised and included with site documentation.

Following the placement of the crossover the area will be flooded with ballast and welds done to join the rails. SWMS and Hot Work Permits are developed to support this work. Laing O’Rourke using Boom Logistics will undertake the work.

Work Completed: 12-14 June

- Installation of ULX’s and Main/Local Cable Route.

Under Line Crossings will be installed as per the ULX design.  Main and local cable route will be completed to cable route design, this will involved open excavation to design depth in accordance with the excavation work plan and SWMS.

Conduits will be laid in the excavation as per the ULX design; the trenches shored or benched to prevent collapse and will be compacted and back filled in accordance with design drawings. To enable this to take place potholing utilising a vacuum suck truck will take place to identify existing services. Identified services will be marked and an appropriate construction method put in place to manage hazards arising from this survey.

Due to the discovery of asbestos this work will be undertaken in line with the sites Asbestos Management Plan and the licensed asbestos contractors SWMS.

Laing O’Rourke and O’Donnell Griffin will undertake the work.

- Site Correlation of Signalling Circuits

Site correlation of existing circuits will take place to ascertain the current electrical integrity. Qualified electrical staff will hand trace wires in existing signalling locations. O’Donnell Griffin utilising contractors from JDMR Engineering and PEBE Holdings will undertake this work.

This work was completed 17 July 2010

- Installation of a new catchpoints for new 156 points at the down refuge on the Newcastle side of the existing 155 points.

Catchpoint installation shall be a lift in using a 45t all terrain cranes. To enable this to take place SWMS and Lifting plans will be compiled.

Prior to the lift existing track will be lifted out and the ballast removed by excavator and/or FEL down to design level. SWMS, excavation plans and lifting plans will be utilised and included with site documentation.

Following the placement of the catchpoint the area will be flooded with ballast and welds done to join the rails. SWMS and Hot Work Permits are developed to support this work. Laing O’Rourke using Boom Logistics will undertake the work.

All work undertaken to install 156 catchpoints must be done in accordance with the project Asbestos Management Plan and procedures contained within this document for response to the identification of asbestos and the treatment thereof.

- Cable Route and Equipment Foundation Installation

Installation of a combined services route will between Sulphide Junction Relay Room and 123 locations. Local cable route will be constructed ex existing 123C location. Signalling equipment bases shall also be installed.

A licensed ASF1 contractor using competent employees will perform installation. Novo Rail employees required to assist will do so in accordance with the asbestos management plan.

Installation of the services route will be done using and excavator, bobcat and vibrating plate compactor. The route shall be installed in accordance with supplied approved designs.

Cable shall be hauled in using both cable winching and manual methods. Cable installed shall be as per current released design and shall be certified as correct by a competent person.

O’Donnell Griffin, Laing O’Rourke and Infraworks shall undertake this work.

- Installation OHW Structures and Signalling Cantilever

Overhead wiring structures and a signalling cantilever will be lifted into place using a 45T all terrain crane. Structures will be mounted on previously casted footings and secured with appropriate fixing and chemset. This work will be done in accordance with the lift plan and SWMS. The work will be undertaken by Laing O’Rourke, Infraworks and Boom Logistics.

- Installation of Signalling Equipment

Signalling equipment such as points, frames, trainstops, posts and traction bonding shall be installed as per the approved signal and insulation plans. Each separate activity shall require it own SWMS and shall be performed by competent people. O’Donnell Griffin, Landmark Signals and Rail Corridor Protection Services shall undertake the work

Methodologies for other critical construction, test and commission activities will be developed as the requirements from finalisation of design are understood. The remaining methodologies will be summarised (as they become available) their detail include in this Project Safety Management Plan and the remaining plans content reviewed, considered, updated and implemented accordingly.

All construction, inspection, test and commissioning tasks must be performed in accordance with the methodology outlined within the SWMS/SWI and associated permits.

See Appendix A - Scope of work. There is no link in scope of works in appendix a, can not locate document in folder, the project management plan is not completed and does not contain scope.

## Main Activities and Method of Work

The purpose of the Sulphide Junction project is to facilitate improved access to the Downer EDI Rail (DEDIR) Cardiff manufacturing Centre, by the provision of direct signalled movements between the Down Siding and the Up Main, and the full signalling of existing movements from the Down Main to the Down Siding, which currently require the operation of the ‘E’ frame mechanical points.

The scope of work that Novo Rail is expected to perform on this project include:

- Design, construction, testing and commissioning of all associated infrastructure changes, including:

  - Signalling design by Aurecon, installation, test and commissioning by ODG

  - OHW by design by Aurecon, installation Laing O’Rourke

  - Track design by Aurecon, installation by Laing O’Rourke

  - Witness and hold point activities by RailCorp

Additional post-commissioning activities include;

- Removal of redundant signalling equipment and racks within Sulphide Junction relay room.

- Removals of existing “E-Frame” catch point and replacement with plain line.

The following is outside the scope of Novo rail’s responsibility:

- Removal of redundant trackside signalling infrastructure associated with the Sulphide branch line. Existing control circuitry for these functions will be removed.

- Removal of previously de-commissioned line side signalling circuits associated with the former Up Arrival sidings. The interlocking circuits will be removed by default, due to the replacement of the existing relay interlocking with a Microlok system.

- Provision of any recommendations resulting from an ongoing “Operational Noise and Vibration” assessment.

# Methodology

The project management directions, including appropriate controls, outlined in the Novo Rail Project Management Methodology (NRPMM) Guidelines, will be applied to this project. The key reference document in the AMS is 06-01-02 NR Supplement to RPMM.  A summary of the methodology against the scope of works is included in 1.3.

Individual activity methodology will be assessed as part of the SWM’s review mechanism and the HSEQ team will be asked for input as necessary.

Activities requiring additional input are listed in 1.6.3. Activities possibly requiring additional input are listed in 1.6.3, and the Project Manager will plan and arrange such input in advance as required.

# Key Persons

# Contact details

| Name | Position | Company | Contact details |
| --- | --- | --- | --- |
| David Hall | Stakeholder | RailCorp Principal Contractor | Ph: 93794205 David.Hall@railcorp.nsw.gov.com.au |
| Chris Gormlie, | Project Manager | Novo Rail | Ph mob:04066008214 CGormlie@novorail.com.au |
| Anthony Davis | Construction Manager Principal Contractor Representative | Novo Rail | Ph: 0435961831 ADavis@novorail.com.au |
| Ryan Bayden | Civil Engineer/OHW | Novo Rail | Ph: 0415 655 660 RBaden@novorail.com.au |
| Louie Anagnostopoulos, | Project Design Manager | Novo Rail | Ph: 0406 649 195 lanagnostopoulos@novorail.com.au |
| Stuart Phegan | Safety Advisor | Novo Rail | Ph: 0459 822 619 sphegan@novorail.com.au |
| Shane McEntee | Environmental Coordinator | Novo Rail | Ph: 82622232 SMcentee@novorail.com.au |
| Ben Pearson | Risk Manager | Novo Rail | Ph: 82622021 BPearson@novorail.com.au |
| Nirav Pandya | Procurement Advisor | Novo Rail | Ph: 82622176 npandya@novorail.com.au |

As necessary other contacts will be added be added to this table.

### Project Management Structure

See appendix  M –

Project management organisation structure. This link not working

## Timeline and Project Schedule

See appendix  0

Project schedule. This is not a link

# Key Activity

* Indicates on or near live track

|  | Key Activity | Estimated Dates | Parties Involved |  | Level of Risk | Additional Controls |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | *Lift & slide Crossover | 12 June 2010 - 14 June 2010 | RailCorp LOR/ODG |  | B- | Crane Lift Plan |
| 2 | *Excavations | 12 June 2010 | RailCorp LOR/ODG |  | B- | Excavation plan, PWB SWMS |
| 3 | *Drainage excavations | TBA | LOR |  | B- | PWB, SWMS, Excavation plan |
| 4 | *Cable route excavations | 16 Aug 2010 | RailCorp LOR/ODG | C+ |  | PWB, SWMS, Excavation plan |
| 5 | *Excavations and lifting of signal bases | 16 Aug 2010 | RailCorp/LOR | B- |  | PWB, SWMS, Excavation plan, lift plan |
| 6 | *Pull cables & terminate cables | 1 Sept 2010 | RailCorp/ODG | C+ |  | PWB, SWMS |
| 7 | *Construct OHW structures | 20 Aug 2010 | RailCorp/LOR | C+ |  | PWB, SWMS |
| 8 | *Installation of 156 Catchpoints including excavation & lifting works | 18 Sept 2010 | RailCorp/LOR | B- |  | PWB, SWMS, Excavation plan, lift plan |
| 9 | *Erect signalling & OHW structures | 18 Sept 2010 | RailCorp/LOR | B+ |  | PWB, SWMS |
| 10 | Installations including lifting of signal locations | 20 Sept 2010 | RailCorp/LOR | B+ |  | PWB, SWMS, lift plan |
| 11 | *Electrical testing of installations | 24 Sept 2010 | RailCorp/ODG | C+ |  | PWB, SWMS |
| 12 | Microlok installation | 24 Oct 2010 | RailCorp LOR/ODG | C+ |  | PWB, SWMS |
| 13 | *Wire & test external locations (electrical) | 1 Oct 2010 -to 3 Nov 2010 | RailCorp LOR/ODG | C+ |  | PWB, SWMS |
| 14 | Fit & paint motors (lift) | 1 Oct 2010 | RailCorp LOR/ODG | C- |  | PWB, SWMS, lift plan |
| 15 | *Commissioning | 12Feb 2011 – 13 Feb 2011 | RailCorp LOR/ODG | C+ |  | PWB, SWMS |
| 22 | De-establishment | 2011 | RailCorp LOR/ODG | C+ |  | PWB, SWMS |

Project activities, tasks and milestones, including start and end dates, are detailed in the Project Execution Phase Schedule in Appendix B.

# Possessions

Possession 1

The first possession is a scheduled possession on 11th-14th June 2010. During Possession 1 the following work will be undertaken.

- Installation of crossover for 153 points including cabling for rotary detection, lift and slide cross-over. Excavation works including;

- Installation of footings for signalling structures and some overhead structures

- Reconfiguration of 2E points rodding

- Insulation testing of existing cables

- Construct local cable routes

The work was successfully completed.

Possession 2

The second possession is a scheduled possession on 18th – 19th September 2010. During Possession 2 the following work will be undertaken.

- Stand OHW wiring structures and dress

- Install insulated rail joints

- Erect signalling structures

- Install impedance bond frames

- Install track circuit boxes

- Install and terminate local cables including cable route

- Install 156 catchpoints

- Excavation works including installation of drainage

- Excavation of cable routing.

Parties involved in undertaking the work were O’Donnell Griffin, Laing O’Rourke, Infraworks, MPK and Brefni Excavation and Earthmoving.  The work was successfully completed.

Possession 3

This possession is a possession on 30 – 31st Oct. During Possession 3 the following works will take place.

- Test operation of 153 and 156 points

- Commissioning of stage works wiring at Sulphide Junction Relay Room

- Erect overhead wiring structures and wire

- Contingency activities from possessions 1 and 2

- Commission 126 location

Parties involved in undertaking the work will be O’Donnell Griffin, Laing O’Rourke and Infraworks.

Possession 4

This possession is the commissioning possession scheduled TBA September. During Possession 4 the scope of works will be commissioned into use.

- Commissioning of new crossover and other associated signalling infrastructure

- Motorise replacement of Frame E

- Commissioning of new Microlok interlocking for the whole of Sulphide Junction

- Commissioning of the local remote Panel changes

Parties involved in undertaking the work will be O’Donnell Griffin, Laing O’Rourke and Infraworks

# High Risk Activities

The key activity table has been used to identify activities potentially of higher risk.  As such additional safety reviews, supervision, authorisations and controls will be planned and co-ordinated to mitigate risk of those highlighted as high.

The high risk activities will have scheduled reviews, inspections and audits detailed in 3.5 of this plan.

The safe work method statements of the activities detailed as high risk will be reviewed by the HSEQ team as well as project management.

Activities that have been identified as being undertaken on this project and will be considered as a high enough risk and shall generate a raised control level include:

- Demolition work

- Work with dangerous goods/materials/substances

- Lifting activities

- Welding

- Work in the rail corridor and danger zone

- Hot works

- Work with compressed air

- Working at height (including scaffolding)

- Erection and dismantling of structures including temporary structures

- Electrical work

- Work near traffic and mobile plant

- Excavations

- Confined spaces

Where high risk activities have been identified in the list above, topic specific SWMS will be produced.  The topic SWMS act as general overview documents for the activities covering the minimum RailCorp requirements for the specific hazard, this document should then be provided to the contractors/consultants/persons engaged to be involved in that work who are expected to include the controls listed in their activity specific SWMS.

# Task Allocation

Tasks and responsibilities have been allocated throughout this document in the form of responsibility tables.  These tables have been inserted into the appropriate associated sections throughout this plan.  All persons detailed in these tables will sign this plan to confirm understanding and acceptance of content. The follow cell provides details of the key used within the responsibility tables.

Key # - lead role * - support role.  Note not all responsibilities require lead and support roles.  Where none has been defined the responsibility sits with each party in terms of their area of control.

- Project Manager (P), Construction Manager (C), Site Supervisor (S), Health & Safety Advisor (H), General site personnel (G)

## Document Interfaces

| Document | Document Custodian | Relevant material | Location |
| --- | --- | --- | --- |
| Tender document |  |  | Novo Rail SharePoint |
| RailCorp SMS |  | Novo Way > RC SMS | Novo Rail SharePoint |
| Project Safety Management Plan | C Gormlie | Project Safety Management Plan The above document is this document | Project Safety Management Plan The above document is this document |
| Worksite traffic management assessment. Worksite Traffic Management Excavation & Earthworks Plan | A Davis A Davis | SMS-06-FM-0369 SMS-06-FM-0370 SMS-06-FM 0375 SMS-06-FM-0376 | Worksite TM Assessment Link Worksite Traffic Management Plan Link Excavation Planning checklist Link Excavation work plan Construction Maintenance planning blank doc |
| Construction Site Induction Record |  | SMS-06-FM-1308 SMS-11 –FM-0689 SMS-06-FM-0925 | Construction Site Induction Record Local Induction Register Local Safety Induction - Employee |
| Construction and Maintenance Planning Review Checklist |  | SMS-06-FM-0744 SMS-06-FM-0721 | Construction and Maintenance Inspection Construction and Maintenance Planning |
| Pre Work Briefing |  | SMS-06-FM-0163 | Pre Work Briefing Completed hard copies are kept on site |
| Geotechnical reports | Geotechnical firms representative. Project manager | TBA – Report from | TBA – reference report that provided initial contamination assessment result |
| Asbestos Site Investigations and risk assessment | Project manager | SMS-06-TP-0101 | Link to Waste Classification Report Link to Airborne Asbestos Monitoring Report Hazardous Materials Survey Report ADE Clearance Report |
| Safety in design review | C. Gormlie | In development |  |
| Project Hazard Log | Project Manager | SMS-06-TP-0662 | Project Hazard Log Link |
| Project safety agreement | RailCorp representative and Chris Gormlie | SMS-06-FM-1362 | Project Safety Agreement |
| Project Management Plan | Chris Gormlie | RPMM and Novorail Procedures | Project Management Plan |
| CEMP | Shane McEntee | RAILCORP PROGRAM ALLIANCE | CEMP |
| SWMS | A Davis, C Gormlie |  |  |
| Project Safety Management Plan Review | C Lloyd | Project Safety Management Plan |  |
| Site Safety Rules | A Davis | Site Safety Rules | Site Safety Rules |
| Safety Induction | A Davis | Induction register Local Induction - Visitor or contractor Local Induction - Employee Delivering a Safety Induction Guidance Document | Local Induction Register Link Local Induction – visitor or contractor link Local Induction - Employee Delivering a Safety Induction Guidance Document Induction records are currently in hard format on site |

# Management responsibility

Safety roles and responsibilities of key project members listed in 1.5.1 have been detailed in Responsibility Tables included in relevant sections throughout this document.

However, regardless of role or position, the following requirements are mandatory to all persons entering site.

On this project the following roles, responsibilities and processes will be applied.

Key # - lead role * - support role.  Note not all responsibilities require lead and support roles.  Where none has been defined the responsibility sits with each party in terms of their area of control.

- Project Manager (P), Construction Manager (C), Site Supervisor (S), Health & Safety Advisor (H), General site personnel (G)

| Managing Responsibilities | P | S | G | H |
| --- | --- | --- | --- | --- |
| Provide safety leadership to Construction Manager and Site Supervisor in line with Novo Rail’s health and safety values. | # |  |  | * |
| Allocate adequate resources to implement health and safety systems (including labour, supervision, plant, equipment materials, funding and support).. | # | * |  |  |
| Support, monitor and review the Construction Manager and Site Supervisors implementation and delivery of the requirements of the Novo Rail health and safety system. | # |  |  | * |
| Assist the Construction Manager and Site Supervisor resolve safety issues between the PC, Site Owner or other team members. | # |  |  | * |
| Have and apply full working knowledge of Novo Rail, RailCorp and PC’s Safety Management System |  |  |  |  |
| Approve the Project Safety Management Plan and related safety material including the identification of Project hazards and risk prior to commencement; and monitor its implementation. Periodically review plan and implementation. | # |  |  |  |
| Maintain acceptable project health and safety performance and communicate lead and lag performance on a monthly business to Novo Rail Senior Management and Project Staff. |  |  |  |  |
| Ensure suitable supervision on the project. Ensure suitable principal contractor representation with required skills, competence and project knowledge on site. | # |  |  |  |
| Provide suitable supervision on the project. Provide suitable principal contractor representation with required skills, competence and project knowledge on site. |  | # |  |  |
| Mentor staff (within Project Structure) in OHS and their roles and responsibilities and follow their improvements. Monitor the application of these roles and responsibilities and formally highlight where they are not being achieved. | # |  |  | * |
| Clearly understand your health and safety roles and responsibilities within the project. |  |  |  |  |
| Consider safety when conducting employee performance appraisals (within project structure). |  |  |  |  |
| Provide and maintain a safe and healthy workplace and be actively involved in ensuring a positive safety culture is instilled and maintained. |  |  |  | * |
| Initiate disciplinary process for staff with poor OHS behaviours/attitudes that prove to be recurrent or could potentially cause a significant risk/event. |  |  |  |  |
| Make and record site visits where site workers (not dependent on employer) are engaged and spoken to directly about H&S on the site and their activity. |  |  |  | * |
| Recognise and reward people who have positively impacted OHS on the project. |  |  |  |  |
| Consult with the relevant Novo Rail Managers to assemble an adequately skilled and experience Project Team. | # |  |  |  |
| Select project team and appointed sub contractors based on individual capability, fitness for selected activities as well as role competence. | # |  |  |  |
| Co-ordinate resources and activities in manner to prevent or limit risk as far as is reasonably practicable. |  | # |  |  |
| Develop and monitor the implementation of the Project Safety Management Plan and activity specific health and safety documentation within the areas of Novo Rail responsibility. |  |  |  |  |
| Coordinate, brief and manage the work activities to achieve health and safety requirements. |  | # |  |  |
| Secure and promote health and safety to ensure continuous improvement in awareness and commitment to safety. |  | # |  |  |
| Maintain an understanding of the requirements and obligations of the relevant Principal Contractor health and safety minimum standards, and rules. |  |  |  |  |
| Understand and apply detailed OHS legislative requirements relating to this project. |  |  |  | * |
| Regularly brief the Project Manager on project safety performance and forward resourcing/monitoring requirements (if higher than those identified in the Project Safety Management Plan). |  | # |  |  |
| Facilitate the resolution of safety issues between project team members |  |  |  | * |
| Liaise with the Project Safety Advisor for advice. |  |  |  |  |
| Adhere to Novo Rail safety values and seek ways to promote and improve the Health and Safety on Projects |  |  |  | * |
| Take reasonable steps to prevent risks to the health and safety of yourself or other persons whilst at work |  |  |  |  |
| Participate in the development and review of safe systems of work and complying with those requirements |  |  |  | * |
| Develop safe processes for dealing with incidents and emergencies. | # |  |  | * |
| Ensure adequate supply maintenance and training of equipment provided for the purpose of protection. |  | # |  | * |
| Follow safe working processes and procedures for dealing with accidents and emergencies |  |  |  |  |
| Wear and maintain personal protective equipment issued |  |  |  |  |
| Challenge unsafe behaviours/attitudes at any level when encountered. |  |  |  |  |
| Understand and apply general OHS legislative requirements relating to this project. |  |  |  |  |
| Provide support and assistance to the Project Manager/Site Supervisor (on request) in areas affecting health and safety including hazard/risk assessment and health and safety reviews. |  |  |  | * |
| At the request of the Project Manager or in line with the project schedule carry out health and safety audits, inspections and reviews to ensure the requirements of this Plan are successfully met |  | # |  | * |
| Manage safety compliance on the worksite |  |  |  |  |

# Safety Risk Management

Risk shall be managed in accordance with the RailCorp SMS and Novo Rail specific processes. The following section outlines the process for managing risk. The responsibility for the risk management process is summarised in the table below.  See also Safety Change Documentation

On this project the following roles, responsibilities and processes will be applied.

Key # - lead role * - support role.  Note not all responsibilities require lead and support roles.  Where none has been defined the responsibility sits with each party in terms of their area of control.

- Project Manager (P), Construction Manager (C),  Health & Safety Advisor (H), General site personnel (G)

| Risk Management | P | S | G | H |
| --- | --- | --- | --- | --- |
| Review and approve risk management outcomes. |  | # |  | * |
| Review content of submitted SWMS from all parties against review checklist. |  | # |  | * |
| Monitor the delivery of risk management strategies. | # |  |  | * |
| Produce Project Hazard Log and ensure process is in place for update and communication to all relevant parties. | # |  |  | * |
| Produce and review all safety related plans detailed in this PSMP. | # |  |  | * |
| Ensure all relevant safety checklists are completed and actions recorded and implemented. | # |  |  | * |
| Implement all safety related plans detailed in this PSMP. |  | # |  | * |
| Maintain and update project hazard log and ensure continual input and feedback. |  | # |  |  |
| Identify hazards associated with project processes, undertake risk assessment and implement control measures within Novo Rails control. Use appropriate techniques to identify potential hazards including surveillance activities, sampling, risk assessment templates such as plant & equipment etc. |  | # |  |  |
| Be proactively involved in site safety management activities. |  |  |  |  |
| Review and communicate Project Hazard Log, Activity Specific SWMS and other OHS Procedures and instructions. |  |  |  | * |
| Undertake risk assessments and implement and review appropriate risk controls |  | # |  | * |
| Produce and monitor implementation of SWM’s and SWI’s for all activities under Novo Rails control that have a potential moderate to significant risk. |  |  |  |  |
| Ensure all plant log books, pre mobilisations checks etc are maintained and undertaken and records kept of such activities. |  |  |  |  |
| Where issues are indentified these are brought to the attention of SS & CM and addressed via the corrective actions system. |  |  |  | * |
| Identify, communicate and monitor application of project zones and hazard specific management guidance. |  | # |  | * |
| Manage the implementation of the Alcohol and Other Drug Policy. Ensure competencies for testing, hygiene and equipment/calibrations are maintained. |  | # |  | * |
| Identify, develop enforce and communicate project site safety rules. |  | # |  | * |
| Implement the Novo Rail ALARP actions and reporting framework principals. |  | # |  | * |
| Ensure health is considered as part of the risk assessment process, and ensure that required medicals and surveillance are undertaken. |  | # |  |  |
| Provide advice re.: medical surveillance and sampling requirements etc |  |  |  | # |
| Report hazards and safety risks identified in the workplace. Assist in the assessment and method development process of SWMS as requested. |  |  |  |  |
| Develop and communicate emergency procedures based on project hazard and environment potential and facilitate recorded practice exercises. Ensure that there are adequate arrangements and facilities for first aid, amenities and emergency response. | # |  |  | * |
| Highlight where SWMS can be improved, are being deviated from or are unusable. |  |  |  |  |

# Safety Change Management

Safety Change Management for Sulphide Junction Project shall be undertaken in accordance with the RailCorp System Requirement for Safety Change Management. The process for achieving this is documented in the Novo Rail Alliance Program Safety Management Plan. The application of effective safety change management shall provide an assurance that changes to the railway as set out under the Alliance will be managed such that the associated safety risks are reduced to as low as reasonably practicable (ALARP).  Change in this context refers to:

- Changes to the operating railway during the lifecycle of a project (e.g. the impact of construction on the existing railway).  By their nature, these changes are temporary as they exist only for the duration of the lifecycle activity.

- Changes to the operating railway following handover of a project as a whole or project phases to RailCorp.  Residual elements of the safety risks associated with the change may need ongoing management by RailCorp so that they are maintained at ALARP.

Safety Change Management does not include OHS and Workplace Risk during the change (e.g. health and safety on construction site).  This is considered elsewhere in this document.

| Date | SCARD Title | Change No. | Brief Summary of Outcome |
| --- | --- | --- | --- |
| 23/12/08 | Safety Change Assessment and Reporting Determination | SC901001276 | The change is needed as a result of; - Additional movements for the testing and implementation of the A sets and; - The permanent 8 car configuration introduces shunting constraints that may impact on the train operations or testing. |

Safety identified through the SCARD, management of change assessments or Safety in Design review output will be managed according to Workplace Risk Management.

Risk management will be done throughout the project lifecycle during the concept, design, implementation and finalisation phases. Risk Management shall consider both OH&S and Rail Safety Aspects of the project

Project management will ensure that any changes, unless a like for like exchange (including those to equipment, materials, structure, design deviations etc) occurring on the project that could impact safety are assessed for potential impact and interface effects.

The designers are required to undertake and provide details of the safety in design review, which considers the construction, maintenance, operability and demolition risks and controls.  Hazards and risks will be indentified in relation to both OHS and Rail Safety.  The review document will be considered by Novo Rail in the creation and population of hazard logs, construction methodology and risk registers.  Designers have the responsibility to ensure that the design complies with legislative requirements and related standards.  References will be provided with the design

On this project the following roles, responsibilities and processes will be applied.

Key # - lead role * - support role.  Note not all responsibilities require lead and support roles.  Where none has been defined the responsibility sits with each party in terms of their area of control.

- Project Manager (P), Construction Manager (C), Site Supervisor (S), Health & Safety Advisor (H), General site personnel (G)

| Safety in Design and Management of Change (see also SCARD RASCIE referenced in appendix ___) SCARD RASCIE Link | P | S | G | H |
| --- | --- | --- | --- | --- |
| Monitor delivery of safety in design processes. | # |  |  |  |
| Provide guidance on the safety in design requirements to project team as required. |  |  |  | * |
| Review safety in design outcomes prior to finalisation of the design process. | # |  |  |  |
| Undertake and design safety reviews for constructability, operability and maintenance. Where design is undertaken by other parties, review safety in design outputs and build considerations into work methodology (and where applicable hazard log). | # | * |  |  |
| Implement and monitor safety in design processes into all projects involving any aspect of design which may change or create an asset. This includes monitoring procurement partners and checking that designs worked from have associated SiD assessment outputs and that they have been/are considered during creation/alteration of asset. |  |  |  |  |
| Provide all relevant parties with the opportunity to contribute to decisions impacting on the design on creation or review, where within project relevance. | # |  |  |  |
| Communicate safety in design output to all affected parties and request such information from suppliers where applicable. |  | # |  |  |
| Make contractors and suppliers aware of their safety in design responsibilities as necessary. |  | # |  |  |
| Ensure that changes to the project that have the potential to effect safety (including methods, equipment, resourcing, structure, parts etc) are assessment for impact (prior to taking place), unless change is like for like; and a written plan produced to bridge/correct identified gaps/interface requirements. | # | * |  |  |

## 3.2 Hazard Identification and Risk Assessments

Hazard identification will be undertaken in line with SMS-06-0104 (Workplace Risk Management) and Construction and Maintenance Planning (SMS-06-PR-0143). The risk management methodology of this project will include a four step approach:

- Identification and recording

- Assessment

- Methods of reduction and implementation

- Review and monitoring

They are to be recorded on a Hazard Report or, Safety Action Report, with timelines applicable and remediation specified.

Where necessary information identified in any of the steps detailed in 3.2 that could affect or influence the safety of other parties will be shared and communicated through the project hazard log or briefings.

Parties will be provided with an extract of the relevant section of the hazard log and assessments or a summary thereof, in their engagement documents.  The information shall also be covered in daily briefs as it becomes relevant.

Some aspects of planning such as monitoring and communication are detailed in the proceeding sections, however where hazards have been identified as potentially significant on a category basis (in the hazard log), additional plans shall sit under this overarching safety management plan.  The plans sitting under this PSMP are:

- Excavation management plans – produced using SMS-06-FM-0376

Sulphide – Excavation Management Plan

Excavation plans will be produced on an excavation basis. Completed excavation plans will be stored in Project folder listed below:

SIG05 - Sulphide Junction > Project Records > Execution > 09 Safety > Excavation Plans

- Worksite Traffic management plan -  produced using SMS-06-FM-0370

Sulphide -Worksite Traffic Management Plan

Worksite traffic management plans will be reviewed along with assessment on significant change (such as possession circumstance etc) and updated as required.

SIG05 - Sulphide Junction > Project Records > Execution > 09 Safety > Traffic Management

- Lifting management plans – produced using SMS-06-TP-1293

Lift plans will be produced and implemented on an individual lift basis.  The plan shall be produced by the crane operatives company and reviewed by the projects construction manager and project safety advisor against the site requirements as well as relevant AS/NZ standard including components of AS1418.

Sulphide -Mobile Crane Plan

SIG05 - Sulphide Junction > Project Records > Execution > 09 Safety > Lift Plans or Crane Plans

- Incident and Emergency response plan  - produced using SMS-15-TP-0246

Emergency Exercise SMS-15-FM-0247 see appendix ‘xxx’. The emergency response plan will be tested six monthly

- Inspection & testing plan – SMS-06-fm-0307 see appendix ‘xxx’

- Asbestos Management Plan – will be produced using the Occupational Hyginists format against criteria determined by Novo Rail

- The updated document is stored in:

SIG05 - Sulphide Junction > Project Records > Execution > 09 Safety > Hazardous Material > Asbestos

- Asbestos Removal Management Plan –  will be produced and implemented on an asbestos removal needs basis against the Railcorp SMS Template SMS-06-FM-0117.  Completed copies of the Asbestos Removal management Plans can be found in the above folder.

A hazard is something with the potential to cause harm.  A risk is the likelihood of that harm occurring.  In order to manage risk, it is important to ensure that a process is followed to identify project associated hazards.

Identifying every hazard throughout the workplace and project duration can be an extremely large and complex job; this project has therefore been broken down into 'bite-sized chunks'.

This has and will continue to be done by using the following techniques:

- Collating and using as a base information available from the site owner and their pre-existing registers and assessments, designers, service providers and previous surveillance, monitoring, and analysis activities undertaken (see diagrams below and under 3.3)

- Breaking the project down into components;

- Breaking worksites into work areas (and, if necessary, breaking down further into zones);

- Breaking each area and component down into tasks;

- Developing a list of likely hazards for the work area (project hazard log); and analysing the components of each task to identify the individual hazards present (Safe Work Method Statements).

- Considering each step of the task in terms of:

  - Activities;

  - Substances and materials used;

  - Plant, tools and equipment involved;

  - Processes;

  - Characteristics of the place where the task is carried out and environmental factors,

  - Potential influencing factors

  - Persons involved and their capabilities and requirements

The identification process has and will be undertaken by a team of project personnel who have experience of the project, site, work activities and process.  They will also be consulted in the risk rating and risk control measure part of the risk assessment process.

A summary of project related hazards is contained within the Project Hazard Log.

Continued Identification

The hazard identification process is not a once off activity.  Existing registers and assessments require review, addition and update.

Continued identification will be maintained through:

- Pre-purchase safety assessments

- Safety in design and change management reviews

- Safety audits

- Reviews on control effectiveness

- Various types of inspections

- Pre-work briefings and safety briefings

- Investigating incidents and near misses

- Consultation with employees, contractors and other interested parties

- Reviewing illness and injury records

- Surveillance activities

- Project planning

- Complaints

- Observations while doing work

- Records of lessons learnt.

All project personnel including project management, employees, contractors etc will identify hazards:

- Immediately before using premises or plant for the first time

- Before and during the installation, erection, commissioning or alteration of plant

- Before introducing changes to work practices and systems of work

- Before introducing hazardous substances and dangerous goods into the workplace

- While carrying out work

- After an incident including near misses

- When new or additional information, from an authoritative source relevant to the health or safety of workers or the public, becomes available

Unplanned Hazard Identification

Even where planned hazard identification is through, hazards can still arise or be identified on an ad hoc basis.  Unplanned hazard identification can occur while employees and contractors are doing their normal work. Where a hazard is identified in unplanned circumstances, the employees and contractors will take immediate steps to prevent others from being harmed including but not limited to:

- Removing or fixing the hazard immediately (within personal authority and competence),

- Following established procedures,

- Getting someone to warn others whilst attempts are made to control the hazard,

- Putting up temporary barriers or signs,

- Telling area supervisor who can resolve the problem,

- Filling in a Hazard Report Form and giving it to project management staff.

Hazards will be reported using Hazard Report Form SMS-06-FM-0106 , which is available from the site office and mess room.  The above process, availability and location of hazard report forms is covered in the site induction.  Completed hazard report forms will be stored in the Novo Works Folder:

SIG05 - Sulphide Junction > Project Records > Execution > 09 Safety > 9.32 Hazard Reports

##### Recording Hazard Identification Data

Once gathered or reported, the hazard identification data will be recorded so that it can be used for risk assessment activities and in determining appropriate control measures. Hazard identification and risk assessment will be undertaken in line with SMS-06-PR-0104  and SMS-06-PR-0143.

### Undertaking Risk Assessment

The risk assessment process for the project will include assessing all of the moderate to significant risks associated with each of the hazards identified during the hazard identification process. In assessing the risks, four essential steps are taken:

- Persons who may be affected are identified,

- The potential outcome of the hazard is recorded and potential consequences estimated,

- The probability or likelihood of that consequence occurring is evaluated;

- Based on these two factors, the risks are assigned priority for risk control through the use of a risk rating (see diagram or risk matrix below).

Once the initial risk rating has been established, current and required control measures against each hazard have been determined; the level of risk reduction offered by an accumulation of the controls needs to be assessed to establish the residual risk rating (see sections 3.3. and 3.4. for control).

Where the assessment team are comfortable that controls identified reduce the risk to as low as is reasonably practicable.,

•	Actions will be allocated to responsible people,

•	Time frames will be imposed on the actions;

•	A period for review of the document will be set.

Once set the assessment will be reviewed within this deadline, or on significant change, to establish the effectiveness of controls and identify if any other practicable control measures have become available.

Risk assessments will be undertaken as a team exercise involving persons with experience of the workplace, activities and plant and equipment to be used and the risk assessment process.

Due to the nature of this project (inclusive of construction elements) the workplace hazard log will be produced and maintained using SMS-06-TP-0662.  A copy of this hazard log will be provided to relevant stake holders and interested parties (see diagram in section 3.3)

For activity specific assessments the risk assessment will form part of the Safe Work Method Statement produced for the job.  Safe Work Method Statement will be recorded and produced on form SMS-06-TP0026

Conclusions reached about risks will be documented in the risk assessments and where necessary supporting information on how that decision was made included in the projects safety folder on SharePoint.

The output of all risk assessments produced for this project will be recorded and stored within the project safety folder on SharePoint and communicated to relevant stake holders including employees, contractors and other parties.

In summary, the risk management methodology of this project will include a four step approach:

•	Identification and recording

•	Assessment

•	Methods of reduction and implementation

•	Review and monitoring

They are to be recorded on a Hazard Report or, Safety Action Report, with timelines applicable and remediation specified.

## Registers

Summary of Registers

Registers have been produced for this project for core topics based on the requirements of RailCorp’s SMS.  Where possible RailCorp templates have been used as a register base.

As well as their general purpose, registers will be used as tools in inventory, maintenance, inspection and audit activities.

At the Kick-off meeting RailCorp supplied a copy of the Workplace Risk Register for the Metro North region. This information was transferred from the Workplace Risk Register onto a RailCorp SMS Project Hazard Log template and was Risk Ranked accordingly on part ‘A’ of the Project Hazard Log (PHL) register.  The PHL was then populated and the Risks remediated, to As Low As Reasonably Possible (ALARP) on Part ‘B’ of the PHL with a median score of C+ or lower.

- This information was obtained from the RailCorp website.

- The Risk Register was obtained from the stakeholder Project Manager.

- Information extracted from Hazardous substances register and MSDS

- Risks register Design Review output.

- Information contained within other safety registers.

A Review of these registers will be carried out by the project team after the June possession with the intent of updating to current project methodology – wording needs to reflect where you currently are on the project.

A list of these registers appears below;

| Name | SMS number | Owner | Where located |
| --- | --- | --- | --- |
| Construction Project Hazard Log Part A and part B | SMS-06-TP-0662 | Chris Gormlie | Project Hazard Log |
| RailCorp Safety Risk Register, |  | Chris Gormlie | Not provided to project |
| Procurement Register |  | Chris Gormlie | SIG 05 Procurement Register |
| PPE Register | SMS-06-FM-0386 | Chris Gormlie | Novo Rail To be developed Sulphide PPE register Link |
| Site Local Safety Induction Register | SMS-11-FM-0924 | Chris Gormlie | Novo Rail SharePoint/Works/SIG05/execution/safety/induction And site folder |
| Plant & Equipment Register | SMS-16-FM-0229 | Chris Gormlie | Plant & Equipment RA Template Plant & Equipment Register Link |
| Dangerous Goods and Hazardous Goods Register | SMS-06-RG-0201 | Chris Gormlie | Dangerous Goods and Hazardous Goods Register Site register to be developed |
| Testing & Tagging Register | SMS-16-FM-1401 – note this document is no longer is existence. SMS-16-FM-0229 | Chris Gormlie | Plant & Equipment Register Link Test and tag register is a hard copy document kept on site. |
| Hazardous materials Register | SMS-06-RG-0109 | CM | Hazardous Materials Register |
| Injury register | Novo Rail | FA | Injury Register no link to register |
| Plant & equipment inspection and testing register | SMS-16-FM-0085 | CM | Plant & equipment inspection and testing register |
| Fire extinguisher inspection and maintenance register | SMS-16-FM-0083 | CM | Fire extinguisher inspection and maintenance register |
| First aid kit inspection record | SMS-16-FM-0088 | CM | First aid kit inspection record |
| Scaffold inspection register | SMS-16=FM-0296 | CM | Scaffold inspection register |
| SWMS Register | SMS-06-TP-1470 | CM | Contractor SWMS Register |

# Hazard Identification and Control

Where necessary information identified in any of the steps detailed in 3.2 that could affect or influence the safety of other parties will be shared and communicated through the project hazard log or briefings.

Parties will be provided with an extract of the relevant section of the hazard log and assessments or a summary thereof, in their engagement documents.  The information shall also be covered in daily briefs as it becomes relevant.

Some aspects of planning such as monitoring and communication are detailed in the proceeding sections, however where hazards have been identified as potentially significant on a category basis (in the hazard log), additional plans shall sit under this overarching safety management plan.  The plans sitting under this PSMP are:

# Controls

The controls stated in the registers, assessments, outputs of investigations and work methods shall be included in the resulting site rules, SWMS, safe operating procedures, safety plans and procedures.  The aim of identified controls will be to reduce identified risks as far as is reasonably practicable using the hierarchy of control framework:

Controls will be allocated for each hazard identified as part of the assessment. A residual rating for each hazard identified will be made considering the level of risk reduction offered by the controls; and planned into the work method.

The control implementation will be allocated to specific persons.

The resulting residual risk rating will determine the set review frequency of assessments and the controls contained within. See also safety change management. Controls will be subject to a regular review cycle (see diagram below), frequency set depending on their safety and reliability criticality.

Controls will be built into the work methods of Novo Rail and Contractor Safe Work Method Statements.  The effectiveness and application of the controls will be reviewed periodically according to the identified residual risk through both documentation and activity/control review.  The RailCorp SWMS Checklists SMS-06-FM-1461 for Contractor SWMS and SMS-06-FM-0024 will be used to review all SWMS’s prior to their use.  The Novo Rail SWMS/JSEA Review checklist will be provided in advance to service providers to highlight the level of information expected for submission.

A summary of site rules, hazards, controls and standards shall be included in the Site Safety Induction.

- As a minimum the Site Safety Induction will include the content of the relevant RailCorp induction checklists SMS-11-FM-0925 (employee induction), SMS-11-FM- 0924 (contractor-visitor induction), and SMS-11-FM- 0396 (new employee-labour hire). Consistent topics within these checklists include: any site specific hazards and risk control measures involved in carrying out the work as identified by the risk assessment process

- Regulatory requirements or codes of practice relevant to any site specific hazards

- Site orientation including location of safe access, amenities, first aid

- Site specific safety rules or procedures including notification of changes to the work site

- Accident, emergency and evacuation procedures and associated equipment on site.

- Requirements of environmental license

All persons on site are responsible for following safety instructions and information provided to them.

The Site Supervisor and working party supervisors shall be responsible for monitoring and reviewing application and compliance to instructions, procedures, policies and plans on a day to day basis.

## Review Controls

See section Review.

| Method | Inspection | Audit | Leadership Visit | Review | Checklist | Topic of the Month | Observations (see 14.2) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Month |  |  |  |  |  |  |  |
| May 2010 | Lifting Activities – CM & PSA |  | Project Manager | SWMS – CM & PSA | Lifting - PSA | Lifting - CM & PSA | 1 per week |
| June 2010 | Excavation - CM & PSA` | Plant & Equip - PSA |  | SWMS -CM & PSA | Excavation- PSA | PPE - CM & PSA | 1 per week |
| July 2010 |  |  | AMT | SWMS - CM & PSA |  |  | 1 per week |
| August 2010 | Lifting equipment – CM & PSA | PSMP - PSM |  | PSMP – PM | Lifting- PSA | TBA | 1 per week |
| September 2010 | Electrical– CM & PSA |  | Project Manager | SWMS - CM & PSA | Electrical- PSA | TBA | 1 per week |
| October 2010 |  |  |  | SWMS - CM & PSA |  |  | 1 per week |
| November 2010 | Worksite Protection– CM & PSA | Excavations - PSA |  | PSMP – PM + PSM | Worksite Protection- PSA | TBA | 1 per week |
| December 2010 |  |  |  | SWMS - CM & PSA |  | TBA | 1 per week |
| January 2011 |  |  | Project Manager | SWMS - CM & PSA |  | TBA | 1 per week |
| February 2011 | TBA | TBA | TBA | TBA | TBA | TBA | TBA |
| March 2011 | TBA | TBA | TBA | TBA | TBA | TBA | TBA |

Corrective actions and improvement actions on this project may arise from a number of sources such as investigations, inspections, audits, suggestions, walkabouts and observations etc.  Whenever possible, the actions will be developed in accordance with the control methodology delivered earlier in this document e.g. by adopting the highest reasonable control actions (including the minimum requirements dictated by legislation, standards and codes etc).

As a minimum, all Corrective Actions or Improvement actions raised will:

•	Be recorded in the project corrective actions register  no register

•	Be allocated to a specific person

•	Be put in context

•	Have a potential consequence rating (if implementation is not undertaken) (see table below).

•	Have a due date based on the potential consequence rating.

•	Be completed by the due date or extension requested and justified.

•	Evidence of completion provided and tracked.

•	Implementation reviewed prior to close out.

•	Effectiveness reviewed post close out.

- Actions taken fed back to person(s) involved in raising the initiating event.

# SWMS Review

All SWMS that will be followed during the course of the project will be reviewed by the Project Management Team using the RailCorp SWMS Checklists SMS-06-FM-1461 for Contractor SWMS and SMS-06-FM-0024.  Further control reviews will be scheduled and take place on significant change or feedback.  . SWMS’ are kept in site folders and reviewed and approved versions are located in the following Sharepoint folder

SIG05 - Sulphide Junction > Project Records > Execution > 09 Safety > 9.8 Safe Work Method Statements

# Health & Fitness

# Health Assessments

All persons engaged on this project hold a RISI qualification and have undergone a health assessment aligned to the National Standard for Health Assessment of Rail Safety Workers as part of the RISI medical requirements.

Additional health assessments/surveillance/monitoring will be identified as part of the activity specific assessments which consider the type of work, locations, plant/equipment and substances/materials exposed to as well as the levels and periods of exposure.

Health surveillance/monitoring activities identified as potentially being required as part of this project includes:

| Health Surveillance/Monitoring/Assessment |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Substance/Hazard | Assessment/ Surveillance Type | Monitoring | Frequency | Who | Arranged by |
| Rail | RISI | Age triggered | Pre-requirement for RISI plus age triggered. | All who work on Novo Rail sites | Contract firm Training |
| Asbestos | Demography, occupational and medical history and health advice. Physical examination. Records of personal exposure. | Air monitoring of works where there is a potential for friable ACM exposure. | Surveillance/Assessment triggered pre potential exposure (based on monitoring results or incident). Frequency of repeat surveillance based on symptoms and advice from D | Persons who have been identified as having potential exposure above national exposure levels. | HR & HSEQ Team on receipt of notification. HR to maintain records. |
| Noise | Audiometric testing (part of RISI medical requirements) | Trigger for noise monitoring of work environment and equipment dependent on operations and machinery data. | Audiometric testing 12 months after initial test and then 2 years thereafter. | Triggered where exposure could occur above 8 hour time weighted average noise equivalent of 85 dB (A) or the noise peak at more than 140 dB(C). The RTA Policy 2.17 - Noise Control | Project Manager to notify HSEQ Team to arrange. HR to maintain records. |

# Alcohol and Other Drugs

This Novo Rail project will implement the RailCorp zero tolerance policy to alcohol and drugs whilst working at all locations within the Alliance.  This includes travel from home to work locations, office locations and rail corridor.

A drug and alcohol free workplace is defined as:

For Alcohol:	Less than 0.02% blood alcohol concentration, and

For Drugs:	Any level of drug less than the cut off levels stipulated by Australian Standard AS/NZS 4308.

Any person reporting to work or undertaking work on the Alliance, with drug and/or alcohol levels above these standards will be immediately removed from the Alliance, and their employing organisation advised that they will no longer be allowed to undertake work for the Alliance for a period of 12 months subject to meeting the requirements stipulated in the Novo Rail Drug and Alcohol Policy 04-04-01.

Where indication is that a positive Drug or Alcohol test has been submitted a hazard report will be produced by the Project Manager (within 12 hours of suspected result) and submitted to the HSEQ Team.

Drug testing will occur on this project in line with Novorail and RailCorp Frequencies.

Alcohol testing through the use of an appropriate accredited tester will occur on this project in line with Novorail requirements.  Tests will be undertaken by authorised persons and a record of the findings will be made on the appropriate forms.

After the initial period of 12 months, the machine will be calibrated annually by an accredited provider.  Prior to use the tester will check calibration data. Please complete these specifics not delete them then delete this comment.

# Fatigue

Due to the high risk nature of the railway work aspect, and as result of major incidents in the rail industry for which fatigue was a contributory factor, the Rail Safety Act 2008 has specific requirements for the management of fatigue. In addition, under the OHS Act and Regulation there are general requirements for all employers to manage the risk of fatigue.

RailCorp ‘s Fatigue Management Program is described in the RailCorp Managing Fatigue SMS System Requirement (SMS-20-SR-1443).  This establishes the requirements for management of fatigue for all workers, under OHS legislation, and specifically for rail safety workers under Rail Safety legislation.

As Novo Rail operates under RailCorp’s Safety Management System accreditation, the requirements of the RailCorp Fatigue Management Program apply to this Novo Rail Project

The Project Manager will consider fatigue risk factors as part of the hazard identification exercise and project planning aspects of the project.  The outputs of this consideration will be included in the project risk register and project and possession plans (insert link to plans).   The Project Manager will develop a specific roster for each possession/ shutdown.

No work period will be planned into the schedule/roster (where a site work aspect is included) that breaches any of the following parameters:

- Maximum Shift Length to be worked is - 10 Hours

- Minimum Continuous Break between shifts is – 11 Hours

- Maximum Number of shifts to be worked in a 14 day period – 13 Shifts

- Where night work is required during possessions, as a minimum persons will not be required to work longer than 3 night shifts in a row, and will be provided with at least 24 hours break before resuming day shift.

Training and awareness in fatigue and fatigue management is provided to all Novo Rail Managers and Project Managers, and will be included in each Project Safety Induction training sessions (link to induction training).

Protection Officers will confirm individuals are free from fatigue and are fit for work when conducting Pre-work Briefings and inductions into Worksite Protection Plans.

If unforeseen circumstances occur on this project where it becomes  apparent that the mandatory shift length (which includes a site work aspect) may be exceeded for previously unanticipated reasons, a fatigue risk analysis will be completed jointly by (as a minimum) the project management and safety representative.

When high risk work is being conducted (eg crane & lifting operations, work on track, operating mobile plant, etc) the site supervisor must involve all persons performing high risk tasks in the fatigue risk analysis.

The fatigue risk analysis as a minimum will take into account:

- The type and level of risk associated with the remaining activities.

- The complexity of remaining activities.

- The type of work and complexity of nature that has already occurred.

- The personal circumstances of those potentially continuing to work over the 10 hours.

- The breaks, refreshments and rest periods that will be built into the extension.

- The availability of fresh resources.

- The critical requirement to continue working.

On completing of the Fatigue Risk Analysis all parties are to be satisfied that risks have been sufficiently controlled.  Monitoring of controls and potential effects of fatigue will be undertaken by the project management team.

Completed Fatigue Risk Analyses (including that of contractors and suppliers) will be sent to the Alliance HSEQ Team at safety@novorail.com.au  by the Project Manager and will be stored in the project folder

SIG05 - Sulphide Junction > Project Records > Execution > 09 Safety > 9.39 Fatigue.

# Management of fatigue in contractors and suppliers

All contractors and suppliers of labour engaged by Novo Rail must have a safety management system that adequately addresses the management of fatigue. This will be reviewed as part of the contractor engagement process or Novo Rail policies signed up to.

The system may include having specific Fatigue Management procedures, policies, training etc. or a requirement for contractors/sub contractors to sign up to equivalent working conditions to those stipulated in the Novo Rail Fatigue Policy.

Where contractors utilise subcontractors, the processes for management of fatigue in subcontracted workers must be the equivalent or tighter than that stipulated to the primary contractor/supplier.

Audits will be carried out by the Novo Safety team to ensure compliance with the above requirements, and contractor employees will be included in the project Fatigue Risk Analysis activities if triggered on possession works.  This may be done as overall inclusion in the novo rail analysis or a prompt to the working party supervisor to conduct a separate analysis.

# Communication and Consultation

Communication and Consultation is essential to ensuring that all parties are aware of any project associated risks and the controls and the application of this Project Safety management Plan. On this project the following roles, responsibilities and processes will be applied.

Key On this project the following roles, responsibilities and processes will be applied.

Key # - lead role * - support role.  Note not all responsibilities require lead and support roles.  Where none has been defined the responsibility sits with each party in terms of their area of control.

- Project Manager (P), Construction Manager (C), Site Supervisor (S), Health & Safety Advisor (H), General site personnel (G)

| Consultation and Communication | P | S | G | H |
| --- | --- | --- | --- | --- |
| Participate in and monitor the delivery of consultation arrangements. |  |  |  | * |
| Provide project management with advice on communications and consultations process. |  |  |  | * |
| Where required, act as a communications conduit between site personnel and Project Management. |  |  |  | * |
| Participate in site communication meetings and briefings where specific safety topics are discussed and assist develop content. |  |  |  | * |
| Ensure that the outputs (including indentified hazards and controls) of assessments, surveys and hazard logs are communicated to all relevant persons (including sub contractors and visitors where applicable). |  | # |  |  |
| Communicate the requirements and obligations of the relevant PC’s Minimum Standards and rules to Novo Rail parties and contractors preferably on work allocation. Minimum standard compliance to form part of the SWM review process. |  | # |  |  |
| Ensure visibility and erection of clear and consistent signage in line with safety plans, rules, controls and site zoning requirements etc. |  | # |  |  |
| Initiate and co-ordinate OHS awareness activities and presentations. Initiate against schedule and local circumstance. |  |  |  |  |
| Work with staff to solve safety problems. |  |  |  | * |
| Acquire and communicate safety information on the work site to all relevant parties. |  | # |  |  |
| Ensure site safety rules are clearly displayed and issued at induction. |  | # |  | * |
| Minimum site training requirements, safety rules, and minimum standards to be communicated to parties during procurement process. | # |  |  |  |
| Assist the Project Manager in resolving health and safety matters with the site controller and staff. |  |  |  |  |
| Formalise consultation arrangement. Consult with project staff and were applicable sub-contractors/sub-consultants to identify and establish consultation arrangements. Facilitate process. |  | # |  |  |
| Consult on topics where significant changes to arrangements, practices or standards occur that could affect the health, safety or welfare of those on site. |  | # |  |  |
| Set up arrangements, content and ensure that site induction briefings are being conducted and are relevant and up to date. Maintain induction records. |  | # |  |  |
| Request consultation and review of control measures detailed in the project hazard log and SWMS with representative sample of relevant site personnel. Seek input into practicality, use and possible reasons for non compliance. |  | # |  | * |
| Set up site safety noticeboard and ensure documentation update. |  | # |  | * |
| Ensure appropriate handover communications exist in relation to current site/plant status, present hazards an risks, planned and ongoing works if applicable, incidents occurring etc between shifts, representatives and entities. Including where PSA states access to Rail Corp during specified hours and where no Novo Rail activities are ongoing during that period. |  |  |  |  |
| Promptly record and publicise output of consultation and communication meetings. |  |  |  | * |
| Publicise, implement and participate in the consultation arrangements for this project. | # |  |  |  |
| Acquire and communicate safety information at the worksite. |  | # |  | * |
| Raise health and safety as the first agenda item in all project meetings and conduct a Safety Share. |  |  |  |  |
| Regularly review and discuss with Project staff safety issues e.g. incidents, hazards, inspection, audit and committee meeting outcomes, health and safety performance. |  |  |  |  |
| Notify your site supervisor or health and safety representative of any health and safety issues that arise at work and report via project mechanisms as highlighted in induction. |  |  |  |  |
| Contribute to the formation and functioning of health and safety consultative mechanisms. |  |  |  |  |
| Cooperate with Novo Rails initiatives to maintain compliance with the projects consultation requirements. |  |  |  |  |

Due to the size of this project, no formal consultation committee will be set up.  Other arrangements have been made to manage communication and consultation requirements as per the OHS Act 2000 (see table below).  The outputs of the formal arrangements will be recorded and stored in the project file on site and uploaded quarterly onto the projects folder in SharePoint.

All staff and site personnel will be included in the safety communications and consultation through weekly toolbox meetings.

On initial project set up, agreement of these mechanisms will be sought from employees (project set up has taken place, did it or didn’t it? Where was the agreement recorded?).  Change this wording to reflect what took place- if you have changed the arrangements since call a meeting, brief out what you intend to do going forward and record agreement.

Discussions relating to consultation and communication arrangements will be on SMS-10-FM-0012.   These projects discussions will take place on 01/10/10 (and overall consensus to the arrangements contained in the table below was expressed.

A record of ongoing briefings and agreements/feedback to such briefings will be made on SMS 10-FM-0206 and SMS -10-FM-0207.

The following table outlines some of the communication and consultation methods that will be utilised during the project.

| What | When | BY Who? | To Who? | How |
| --- | --- | --- | --- | --- |
| Consultation arrangements | Formal consultation meetings will be held at least quarterly, or when a significant change to site safety arrangement or processes is proposed. A site OHS meeting will be conducted on a weekly basis. Quarterly OHS Meeting | Project Manager and Site Supervisor The Construction Manager and, Safety Advisor, OH&S Site Representative, Project Manager or his representative. | All persons with a direct relationship with Novo Rail in the responsibilities and communications table. All personnel on site to be present All persons with a direct relationship with Novo Rail in the responsibilities and communications table. | Information for consultation as well as the meeting time, date and location will be posted on the sites safety noticeboard at least one week prior to the meetings. SMS-10-FM-0012. Agenda minutes will be produced and erected on noticeboards on form SMS-10-FM-0017. The weekly OHS meetings will be conducted at the site Safety Advisor’s office or amenities area. - The output from any of these meetings will be fed into the safety actions register and a summary of discussions and outcomes posted on the safety noticeboard. - Quarterly site meeting be arranged with the employees Representative invited to raise any safety or other matters that may arise from time to time. |
| Topic of the month | The topic of the month (see safety review schedule) is determined from the overarching project schedule based on hazard specific risks. They will take place monthly. Guidance/summary notes on the safety topic shall be available and posted The topic of the month will take the form of safety briefings (on that particular safety topic), Accompanied by the use of checklists for use in evaluating the hazard control mechanisms. | Project Safety Advisor | All persons with a direct relationship with Novo Rail in the responsibilities and communications table. | Briefings shall be recorded. Topic checklist completion and corrective actions resulting from the checks entered into the projects corrective action register. |
| Issues Resolution | Issues can be raised as and when they are highlighted. | Anyone | Site supervisor | They should be reported through the Projects Hazard Reporting and incident reporting mechanism. Reports must be submitted to both the site controller and Novo Rail where they could have potentially involved or affected Novo Rail Project staff. |
| Project hazards, site safety rules | Communicated to parties prior to commencing work on the site. Rules communicated at entrance to site. | Site supervisor | All persons with a direct relationship with Novo Rail in the responsibilities and communications table. | Communication formally and the procurement stage and via sign on to relevant sections of the PSMP and SWMS. - Employees will be consulted and informed of all dangers &/or hazards that may affect them in the workplace and advised that they have the right to make comment or suggestions to mitigate these hazards. |
| Scheduled events (toolbox talks, pre work meetings, emergency arrangements and scenarios etc) | Toolbox talks -weekly Pre-work briefings and meetings – prior to work commencement – handovers from one shift to another and briefing of team re surrounding hazards and activity based risks and methods of control. Emergency arrangement communications – project commencement basis. Emergency scenarios –twice per annum. | Site supervisor and/or Supervisor in charge of working party | All persons with a direct relationship with Novo Rail in the responsibilities and communications table. | Toolbox talks face to face. Attendees recorded on briefing sheet. Pre-work meetings face to face. Briefing attendance recorded on briefing sheet. Handover record recorded on shift handover document. - SWMS information briefed and discussed. Emergency arrangement communications – face to face. Recorded in induction and sign on to SWMS or relevant plan Emergency scenarios – physical emergency exercise. Participation recorded |
| Safety training requirements | Determined from activity specific SWMS.\ and site requirements | Supervisor in charge of working party and site supervisor | All persons with a direct relationship with Novo Rail in the responsibilities and communications table. | SWMS record. Certificate evidence. Communicated during procurement process. |
| Inductions | General and industry specific Inductions. Site specific inductions- At least once per annum. Activity specific induction – based on SWM content – at least once per annum | Supervisor in charge of working party and site supervisor | All persons with a direct relationship with Novo Rail in the responsibilities and communications table. | WorkCover General Safety Induction Card viewed and number provided in SWMS. RISK card viewed and number recorded on SWMS. Record of communication and understanding on SWMS document. |
| Site walkabouts and observations | General walkabouts and observations on a minimum weekly basis. | Project Safety Advisor, Construction manager, employee picked from the workforce. | Whole site. | General walk around using a relevant hazard specific or general site inspection checklist. Main aim is to observe the general site standards, review activities being undertaken and the implementation of controls etc |
| Leadership visits | Bi-monthly | Project | Whole site | Undertaken against Take Safety personally Leadership Pocket Card. Output to be discussed with individuals at the time of the engagement and actions recorded in site corrective actions register. |

Information will be made known to the workforce by way of notices, one-on-one discussions, posters on OHS site noticeboards, displayed OHS site meeting agenda, and information provided in the daily pre Work Briefs (PWBs).

# Consultation Arrangements and Issue Resolution

In addition to those arrangements detailed in the paragraph and table above, the site controller will have a consultation mechanism in which Novo Rail shall be an invited member.  Information discussed in the Site controllers’ forum shall be communicated via the quarterly meetings to those in a relevant relationship to Novo Rail.

The plan requires that, at the initial formation of the site, a meeting be convened with the participating employees to confirm the consultation arrangements.    In summary of the communication and consultation mechanism is.

- All site staff will periodically have the opportunity to input into the sites safety mechanisms.

- All site staff will receive safety communications

- An employee representative will act as the main contact for consultation.

- All weekly will be documented. Quarterly OHS mechanisms will be documented, agenda advertised in advance and outputs posted on noticeboards.  Corrective actions arising from either of these mechanisms will be included in the Sites Corrective Action Register.

Information will be recorded in hard copy in on-site folders, maintained on-site and electronically on the site e/folder, including copy of the site OHS consultancy agreements between the employer and the employees.

For the Sulphide Junction the main OHS Consultancy Team will be;

| - Name | - Position |
| --- | --- |
| - Chris Gormlie | - Project Manager |
| - Anthony Davis | - Construction Manager |
| - TBA | - Safety Advisor |
| - Any other project members/parties as necessary. |  |

# Communication requirements

Communications are seen as paramount in the continuation and maintenance of safety standards on this project.  The project communications approach is based on a staged strategy, where information is raised, communicated and reiterated at different times and through relevant forums.  See diagram below:

An OHS Site meeting will be called initially every week for the first two months then two (2) weeks for the first next 4 months then, drop back to each month or, as the occasion arrives. These meetings will be advertised for inclusion of all persons on site on the site safety noticeboard and amenities rooms.  Site safety rules will be communicated as part of the contractor engagement process.

Site Evacuation Plan and Emergency procedures, including the identification of the Chief Fire Warden, the Wardens, the First Aid Officer will be publicised and posted on the safety noticeboard and persons identified at the site safety induction and every PWB daily.

Toolbox Talks will be held weekly or for any significant changes to SWMS or, any non-conformance action to be publicised and closed-out.  Also to promote further safety awareness with the production of any Safety Alerts, Safety Bulletins or any other information that may affect the safety of the employees on site.

# Safety Training Requirements

Training requirements for visitors will vary dependant on their intended site activities and access areas.

On arriving to site, they will proceed immediately to the site office, where they will be asked to provide details of their intended visit, and all relevant induction cards (e.g. RISI, WorkCover General Induction) and evidence of training and competency requirements.

Due to the proximity to the Rail Corridor, no persons will be permitted to enter into the active construction zone or Rail Corridor (whether active or inactive) without valid RISI and WorkCover General Induction Cards (or equivalent state mutually recognised qualifications as agreed by WorkCover).  Only RISI Cards issued by RailCorp will be accepted as valid.

Delivery zones will be clearly demarcated on the site map once produced or identified in the workplace traffic management plan through the worksite traffic management assessment.  Controls maintain the safety of the delivery zones are details in the projects hazard log and worksite traffic management plan.  Delivery/courier drivers will be only allowed access into the demarcated delivery zones and the site office environments (outside of the active construction zone and rail corridor – whether active or inactive) unless they hold current induction cards as highlighted above.

A justification risk assessment will be undertaken for any occurrence where it is considered necessary to waive any of the minimum training requirements above.  The assessment will include additional controls which detail how the legal requirements as well as those relating to the three stage induction are to be achieved.  Records of such waiver risk assessments will be recorded in the projects Novo Works folder (SIG05 - Sulphide Junction > Project Records > Execution > 09 Safety > RISI or OHS Induction Waiver Risk Assessments).

Training requirements for service providers will be according to the above minimum standards, disclosed requirements at the time of engagement through the procurement process, legislative and industry requirements relating to their work activities and requirements identified in their Safe Work Method Statements. All personnel/sub-contractors and their employees shall be subject to a site induction where they will be expected to provide all evidence of RISI, OHS white card and any CoCs necessary.

These requirements will be reviewed annually and updated if any significant changes occur on this site.

# Inductions

All persons entering into the active construction zone or Rail Corridor (whether active or inactive) will be site inducted by appropriate Project Management staff.   This induction is called the general site induction.

During possession works or activities deemed as high risk an additional induction specifically for the possession activity must be undertaken.

Inductions for this project will only be delivered in line with the RailCorp Delivering a Safety Induction Guidance Document and run by the following Project personnel:

- Stuart Phegan - Site Safety Advisor

- Anthony Davies– Construction Manager

- Chris Gormlie – Project Manager

See also table in communications and Consultation Section 4 for additional information. The Site Induction will be as per RailCorp SMS and the following documents used to prompt and record induction material

- Induction register

- Local Induction - Visitor or contractor

- Local Induction - Employee

A copy of this sites induction is in appendix ‘T’

### Safety Notices

All Safety notices will be recorded and raised at all Pre Work Briefs, OHS meetings and toolbox talks, with Safety Memos and Safety Alerts posted on the site Safety Noticeboard (location) and in the site meal rooms, in hard copy. Actual records will be recorded in the site folder and uploaded into the project folder on Novo Works quarterly (SIG05 - Sulphide Junction > Project Records > Execution > 09 Safety > Safety Notices  ).

### Changes to Safety Requirements and Organisational Policies

Safety plans and procedures.

Where significant changes (which are not like for like) to the project work methods, resourcing, rules, controls or processes are identified a safety change management process and management of change process will be initiated prior the change taking place in order to ensure adequate risk management.

Changes, their requirements and implications will be communicated to project members through the projects consultation and communication forum and input considered (in the final changes).

All/any changes relating to infrastructure to be reported on the SCARD system change form for action by the RailCorp/Novo Rail Safety Management personnel.

### Communication Management

For the project, site based project staff communication will be managed by the Project Manager and Construction Manager (and recorded in the form of a briefing as necessary and detailed in the previous sections); with external communication and community liaison being managed by the Alliance’s Project Communications Manager (Susan Allton).

Site signage will display project details and the Alliance project specific 1800 Project Information Line.  Separate site signs will display the name of the Principal Contractor and site personnel contact.  Safety signage is detailed later in this plan.

Communication will be undertaken in accordance with the Novo Rail Stakeholder Communications Plan found on Sharepoint in the Stakeholder and Communications folder for each project and the Novo Rail Project Management Methodology (which incorporates the RailCorp Project Management Methodology.

The Stakeholder Communications Plan provides details of:

- Key messages

- Stakeholders, likely issues and mitigations

- Communication channel  delivery mechanisms,

- Timings of communications

- Responsibilities for communications to stakeholders.

Communication and notifications of incidents, hazards and non conformances will be in line with the requirements of other parts of this plan (namely Incident Management and Corrective Actions)

# Purchasing

On this project the following roles, responsibilities and processes will be applied.

Key # - lead role * - support role.  Note not all responsibilities require lead and support roles.  Where none has been defined the responsibility sits with each party in terms of their area of control.

- Project Manager (P), Construction Manager (C), , Health & Safety Advisor (H), General site personnel (G)

| Safety in Procurement | P | C | G | H |
| --- | --- | --- | --- | --- |
| Ensure that a process is in place to assess (prior to purchase) the suitability for use, compliance and conformities with relevant standards and jurisdictional requirements and potential safety impacts of plant/equipment/structures/materials and substance purchased. Include details of process and link to appropriate tools in project Safety Management Plan. | # |  |  | * |
| Monitor the implementation of the whole Safety in Procurement process. | # |  |  | * |
| Minimum site training requirements, safety rules, and minimum standards to be communicated to parties during procurement process. | # |  |  |  |
| Apply designated safety in procurement process. | # |  |  | * |
| Ensure that procured items are entered onto relevant site registers where applicable. |  | # |  |  |
| Ensure that procured items are provided with relevant hazard and risk information and instructions. Communicate |  | # |  |  |
| Maintain update and communication of this information |  | # |  | * |
| Undertake as appropriate cross reactivity and/or accumulative impact assessments for materials introduced to existing arrangements. Communicate outcomes. |  | * |  | # |
| Include all relevant information from the above process into the sites hazard log and relevant SWM’s and SWI’s |  |  |  |  |

### Procurement and Goods Delivery

Note in terms of this plan, purchasing and procurement includes the lease of plant, equipment materials and substances as well as the engagement of parties and service providers..

In line with the review process of the safety systems and standards employed by third party contracting firms, the procurement of plant, equipment and materials with the potential to be hazardous to health also requires assessment.

A pre-purchase assessment shall be undertaken to identify potential risks associated with inputs, outputs, cross reactivity, interactions and use of equipment.  It also provides a checklist to prompt parties to obtain information pertaining to:

- Maintenance and inspection information

- Instruction manuals

- Safe use and training provisions

- Australian standards and conformity notices etc.

The assessment shall be applied to all planned procurement activities relating to equipment, plant and potentially hazardous materials and substances with a potential to pose a foreseeable significant risk in use or failure.  The following forms will be used as prompts during the pre-purchase stage:

- SMS-13-FM-1408 and/or Link to Novo Rail pre-purchase Assessment

The project procurement process will ensure that:

- The project has a demonstrable process to provide information to suppliers of minimum safety, environmental and sustainability standards, requirements, and interactions prior to engagement?

- The project has a demonstrable process to establish the competence and capabilities of contractors and suppliers in terms of safety & environmental impact of the services that they provide?

- The project has a demonstrable process of considering HSE & Sustainability performance and capabilities of contractors in their selection process of both articles and service providers?

- The project has a demonstrable process of the communication of site hazards and rules to suppliers prior to their access to site (for inclusion – where applicable in related Safe Work Methods and controls)?

- The project has a process to review:

  - HSE work methods and associated ‘shared’ generated hazards and controls associated with work activities of suppliers- in advance of their arrival on site

  - awareness of the work methods content of the individual sent to represent them on site

  - the application of disclosed methods.

- The project has a demonstrable process for assessing the safety and environmental risks associated with procured (including leased, purchased) plant, equipment, substances and materials?

- The project has a demonstrable process for capturing safety risks associated with equipment brought onto site by suppliers of services?

- The project has a demonstrable process for including the outcomes of points 6 &7 into the projects hazard log/safety risk register?

- The project has a demonstrable process for the updating of associated registers?

- The project has a demonstrable process for communicating introduced hazards to existing/relevant site staff?

- The project has a demonstrable process for obtaining and communicating relevant materials and instruction associated with the safe use and control of introduced/procured goods, materials, substances plant & equipment pre and post purchase and delivery.

- The project has a demonstrable process for controlling the use and access to procured equipment with a known medium to significant risk?

- The project has a demonstrable process for triggering assessment of associate HSE equipment/plant risks (and communication of their findings) where required due to:

  - Change in intended use or environment

  - Age

  - Condition

  - Post event

- The project has a demonstrable process for tracking the issue and whereabouts of safety or environmental related procured equipment?

- Within the scope of safety and environmental issues, that the project has a process of managing change associated with procurement?

- Key project personnel are aware of the safety in procurement requirements, understand them and implement them.

- The project has a demonstrable process for:

  - reviewing supplier performance  in terms of safety and environment throughout the project

  - communicating this information where necessary  to interested parties

  - recording a summary of safety and environmental performance and concerns for sharing within Novo rail.

Initial vetting has been undertaken on all preferred suppliers by the Novo Rail Procurement team.  For high risk or major contracts, suppliers will be approached off the RailCorp or NOP Approved Panels.

For this project RailCorp’s Pre-purchase Safety Assessment Checklist for works and services – SMS-13-FM-1409 will and has be used. Each of the Novo Rail partners also has auditable procurement and vetting processes. Registers have been created under this plan plan for:

- Substances hazardous to health – (nothing in appendix F – note the link I have inserted has a blank document that you need to populate.)

- Hazardous materials (SMS-06-RG-0109)

- Plant and equipment (SMS-16-FM-0229)

These registers shall be audited as part of the SMS audit and procurement records and assessments reviewed at least twice per annum.

Site PPE for Novo Rail staff shall be purchased from the preferred supplier and off the set PPE list.  The list has been evaluated against Australian standards and for protection against rail specific hazards.

Where items are required outside of the listings, a request shall be submitted to the Program Safety Manager accompanied by the SWMS detailing the hazard to be mitigated, who will assist in an evaluation of suitable additional controls and where required personal protective equipment additions.

Other requirements shall be specified in the activity specific SWMS and project hazard log. Articles to be procured, the schedule and the supplier, including assessment details are located on the procurement tracker

| Articles to be Procured |  |  |  |  |
| --- | --- | --- | --- | --- |
| Plant/Equipment /Material/Substance | Supplier | Quantity | Date required | Assessment required |
| Ballast | Martins Creek Quarry | 1000 tonne | June 10 | No |
| Road Base | Martins Creek Quarry | 300 tonne | June 10 | No |
| Cable | By Tender |  | July 10 | No |
| Conduits | By Tender |  | June 10 | No |
| Precast Cable Pits | Metwest | 10 | June 10 | Yes |
| Concrete | Boral | 40 Cu meters | June 10 | No |
| OHWS | By Tender | 8 Tonne | July 10 | No |
| Footings Reo | Active Steel | 2 Tonne | June 10 | No |
| Signal Bases | Metwest | 4 | June 10 | No |
| Signals | Westinghouse |  | August 10 | Yes |
| Point Motors (153 & 156) | Westinghouse | 3 | June 10 | Yes |
| Catch point | Bathurst Work Shops | 1 | June 10 | Yes |
| Impendence Bonds | Westinghouse | 6 | June 10 | No |
| Welds | Thermit Weld | 50 | June 10 | No |
| Insulated Rail Joint | Bathurst Work Shops | 12 | June 10 | No |
| Crane/Frannas | Boom Logistics | 2 | Ongoing | Yes |
| Excavator/Bobcat/FEL | Charters G&H Contracting | 2 | Ongoing | Yes |
| Super sucker | Infraworks | 1 | Ongoing | Yes |
| Tamper and Regulator | Select | 1 | Sept 10 | Yes |
| Drott | C&L Sultana | 1 | Sept 10 | Yes |

Suppliers of both goods and works/services will be provided with the relevant safety specifications and verifications as required.  Prior to engagement the following SMS Verifications/specification documents will be used:

- SMS-13-FM-1410

- SMS-13-FM-1411

Minimum standards from the NOP’s shall be called upon as required to set minimum requirements in the pre-engagement stages.

All procurement activities for the Sulphide Junction project will be carried out in accordance with the Novo Rail Strategic Procurement Management Plan. This ensures procurement of goods and/or services are based on a “best for project” approach with works tendered to sub-contractors where Novo Rail does not possess the capability nor the capacity to self perform.

Novo Rail’s bid document for the RailCorp Program Alliance defined the key capabilities and expertise of each of the NOP’s:

- Novo Rail will be responsible for electrical works (other than 1500V) aligning with their core business as an electrical contractor;

- Novo Rail will undertake civil, building and rail works (including 1500V electrical works)

- Design, testing and monitoring services will be supplied by Aurecon; and

- RailCorp will provide Track Circuit equipment already procured on behalf of the project.

- Due to Long lead times from third party suppliers, RailCorp, through its Bathurst Fabrication Centre, will be the supplier of the Crossover and Catch Point and Roding.

- RailCorp will be assisting in the provision of cable lengths less than the minimum procurement quantity from third party suppliers while Novo Rail establish its procurement strategy and arranges its own inventory management systems

- This breakdown aligns with the procurement services outlined in the Novo Rail Strategic Procurement Plan and provides a “best for project” approach for Sulphide Junction.

- Where best for project requires procurement of goods/services, each of the NOP’s will enter into legally binding contracts with suppliers/subcontractors under each NOP’s approved terms and conditions.

- The project team has identified the critical resources with long procurement lead times (i.e. exceeding 4 months) and procurement times and sought solutions to minimise those lead times to meet the project schedule.

- Novo Rail is not a legal entity which means that it cannot directly enter into contracts with subcontractors and suppliers.  Only RailCorp or a NOP will enter into contracts.  The NOP will enter into contracts in its own right, and for the purposes of that contract will not be acting as an agent for the Alliance or RailCorp.  As standard contract terms and conditions are currently under development for Novo Rail. The NOPs’ standard terms and conditions will be applied on the project.

- Labour resources will mainly be supplied by the NOP organisations and plant, equipment and materials will be procured by the NOP in accordance with the relevant discipline breakdown. Novo Rail will remain collectively responsible for all the work under the Alliance including work carried out by subcontractors and suppliers.  Subcontractors shall be vetted prior to engagement using Novo Rail pre Engagement Supplier/contractor Form, their submission shall be reviewed using answers contained within their submission and SMS-13-FM-1409. Procurement will be managed in the delivery of the project with a focus on the following key aspects:

- Obtaining comparative quotes for major items being procured;

- Construction schedule, key procurement items and critical milestones;

- Local council labour participation (as directed in the PRN);

- Liaison with RailCorp for the supply of LLTE (abbreviation) and specific rail components;

- Development of supplier relationships in relation to longer term program objectives as well as specific project objectives including incentivisation where appropriate;

- Industry capacity and resources;

- Transport, freight and storage; and

- Terms and conditions favourable to Novo Rail and covering insurances, warranties / guarantees, quality, safety and environmental aspects including flow down of RailCorp requirements.

- On arrival at site goods and work methods will be reviewed on a (representative samples basis for goods), by activity for services/works to monitor the intended/available level of compliance to disclosed requirements, standards and specifications.  Periodically during the project and dependant on risk, more in depth reviews of the safety aspects relating to goods, works and services procured will be undertaken in the form of Audits.  Suppliers may be audited directly as part of this process.

## Handling, Storage, and Packaging

# Substances/Materials Hazardous to Health

- Substances shall be procured from commercial supply routes.  Material Safety Data Sheets (MSDS) shall be obtained for each article classed as potentially hazardous to health.  Storage, handling and use shall be in line with the recommendations contained within the MSDS. Suppliers are legally obliged to provide MSDS sheets when relevant substances/materials are procured.  These will be kept with the materials as well as a copy being maintained in the site office.  Records shall be uploaded quarterly into the projects safety folder in Novo Works

- SIG05 - Sulphide Junction > Project Records > Execution > 09 Safety > 9.27 MSDS Sheets

- The Project Team will keep MSDS records up to date.  The site supervisor shall notify him/her if supplied material/substance is replaced.  The person procuring the items will ensure that MSDS’s are obtained from the supplier of the goods.

- Substances and potentially hazardous materials procured for or brought onto site shall undergo a pre-purchase assessment/evaluation (see section 5).  MSDS information and labels will be checked on arrival.  Relevant information will be included on project hazard log and in SWMS.  Substances will not be stored decanted from their original containers.

- Assessments shall be undertaken based on the classifications and categories identified in the MSDS and cross reactivity assessment undertaken by the Project Safety Advisor in terms of usage and storage.

- The CEMP for this project details potential environmental impacts and disposal requirements.  Where new substances are brought onto site or quantities increased, the Environmental Co-ordinator will be made aware of the change.

- Handling requirements, controls, surveillance/monitoring and emergency arrangement based on the content of the relevant MSDS will be detailed in the activity specific SWMS.  The following considerations will be taken into account as part of that process:

- A hazardous materials/substances register has been developed as part of this plan.

- Unless in use, substance will remain in the container that they were supplied in.  Where labels become unreadable, they will be removed from use, and disposed of as per the details in the CEMP.

- Storage zones are clearly marked on site plan and consideration has been made of their proximity to flammable sources and proximity to vehicle routes.

- No materials classed as highly flammable will be locked internally but will be kept externally in a lockable/vandal-proof steel lockable, well ventilated purpose built container, separate from the containers mentioned below.

- Signal Warning devises (detonators) will be secured in a purpose built lockable storage container kept in the Protection Officer’s vehicle.  Flammable goods will be stored in purpose built, well ventilated, lockable, vandal proof boxes externally, with applicable signage for ease of identification.

- Employees with the potential for contact/exposure to substances/materials hazardous to health will have the related hazards and required controls communicated to them.  Where health surveillance is required, this will be identified through the activity risk assessment and by reviewing the MSDS and arranged by Project Management (see health surveillance section).

- Where applicable, the pre-purchase process will ensure such substances/materials are procured and delivered through appropriately licensed providers.

## Materials/Equipment Handling

- Wherever possible, suppliers shall be requested to provide goods and materials in easily handled sizes.  Where this is not possible, efforts will be made for arrangements to be established for weight and load stability markings on packaging.

- Delivery and unloading zones marked on site plan have been identified considering reduction in handling requirements as well as pedestrian segregation. Wherever possible mechanical means of handling shall be provided.

- Where mechanical aids are not available for repetitive, unstable, uneven or potentially heavy loads, specific manual handling assessments shall be undertaken as part of the SWMS that consider:

- The controls identified will be included in the work method.  Where improvements are identified for future supply, these should be communicated back to the supplier and to the procurement division.  Load guidance limitations for manual handling as detailed below shall form the basis of such assessments.

- Lifting/moving equipment will only be used for their designated purpose.  Where accidental work outside of their scope has occurred, used items/equipment will be removed from service until inspected (where applicable tested), and deemed and tagged as fit to use prior to being put back into service.

- All site Plant and equipment will be entered onto the projects Plant & Equipment Register and a Plant and Equipment Assessment completed or obtained for them.

- Where equipment belongs to a third party, and it will be stored on site in excess of one month or has the potential to pose a high or accumulative risk, the equipment must be entered onto the projects documentation (register and assessment provided for inclusion in project files and considered in the hazard log).  Where the equipment will be on site for short duration stay, the registers and assessments may be retained and managed by the Contractors, however evidence of their maintenance needs to be periodically reviewed by Novo Rail Project Management.

- Maintenance and inspection details of materials handling plant or equipment are expected to be clearly displayed and/or available on request.  Any mechanical aid required will listed with appropriate detail in a register that will created as an appendix to this PSMP

- On the sulphide junction project, the following equipment has been included in the planned methodology:

| Mechanical aid | Fixed or mobile | Intended use | SWL | Location | Suitability |
| --- | --- | --- | --- | --- | --- |
| Hiab Truck | Mobile | Lifting | 3T | Site |  |

- The following principles must be adhered to

- All lifting or storage equipment shall be marked with SWL, and subject to regular inspection.

- All goods and stores, tools and equipment will be stored in lockable, well ventilated, storage containers on site.

- Provisions will be in place to provide signage to record contents of containers and contact details of personnel with keys to containers placed on detailed sign on the containers.

- All goods placed in containers to be placed on applicable shelving to minimise manual handling.

- Employees to use team lifting techniques as required and noted in Manual Handling SWMS.

- All/any bulky or heavy items will be unloaded by mechanical means by Franna Crane, forklift or Hiab, operated by suitably qualified and licensed personnel.

- Where front end loaders are used to lift equipment consideration will be made as to whether the requirements of AS1418 apply.

- Any manual unloading of trucks will be to the appropriate Manual Handling SWMS as found within the SWMS folder and rolled-out in the PWB prior to each day’s work/unloading.

# Handling Storage Packaging and Delivery

# Delivery

- In detailing the delivery schedule, consideration shall be made of:

- Time of day, all deliveries will be coordinated to occur during normal business hours.

- Population use in local area. The area is bounded by both domestic and industrial areas. Drivers will be cautioned to drive safely and as quietly as possible within the road rules.

- Space available in storage areas. There is ample space to receive the ballast to carry-out this installation. Ballast will be stockpiled in the regulation manner and barricaded off to prevent occupation.

- Ability and safety in delivering to area of use. All site delivery drivers will be warned and consulted with the site specific safety rules, with the NO/GO areas clearly defined and explained to all drivers.

- Site security. The site will remain under lock and key at all times for both sides of the rail corridor. When constant deliveries are to occur, the gates may be unlocked providing a competent person remains at the gateway, at all times, to maintain security and the public from entering the rail corridor.

- Driver’s knowledge of site risks (provision). Drivers to be site risk informed at the beginning of each day.

- Driver’s protection against site risks. All danger zones will be barricaded-off to prevent entry to the danger zone.

- Pedestrian and traffic segregation. There is no pedestrian traffic applicable to this site. All workers will access their workplace by authorised work vehicles that will park within the approved areas and allow passing traffic room to pass.

- Co-ordination to limit congestion and safety risk.

It has been determined that during normal operations this site does not warrant a Road Transit Authority approved traffic management plan.  This need will be reassessed for each possession and separate portion of work.  A Worksite Specific Traffic Management Plan  has been developed (based on the output of the Worksite Traffic management Assessment - SMS-06-FM-0369, and will be monitored on a regular basis under this PSMP or on significant change such as possession working.  Completed plans and assessments are located in the projects safety folder in Novo Works

SIG05 - Sulphide Junction > Project Records > Execution > 09 Safety > Traffic Management

- All deliveries are to be conveyed to site by normal freight methods utilising trucks & semi-trailers, with Hiab/Crane/Forklifts used as the means of handling receivables.

- Access to site, on the Down side of the side, will be from Montgomery St., Argenton   and will turn-around within the “Ces” on the rail corridor. All deliveries will be coordinated with a warning of delivery times and all trucks and movements within the site will be escorted by an authorised person that will accompany the delivery, unloading and decamp from site.

- These deliveries will be stored on “gluts”, above the ground to minimise contamination. Goods that must be stored under cover will be stored in the container/s.

- Ballast deliveries will access the site from the industrial area on the “UP” side, with tipper trucks delivering ballast with a turn-around provided in a safe hard standing position, as nominated and indicated by the escort or tip marshal. Drivers are to park their vehicles on hard standing areas only.

- Parking and unloading will occur on prescribed hard standing areas, suitable for the unloading to be carried out., with suitable exclusion zones temporarily erected to maintain safety on site.

- SWMS for unloading trucks, Manual Handling and Working around Plant are to be communicated to the employees at each PWB and prior to deliveries taking place.

- Loads are to be unloaded in the approved area/s and under the Supervision by a suitable person assigned

- Checks are to be made on the driver’s licenses; crane operator’s WorkCover CoCs and recorded. If remaining onsite for a considerable time, all operators must be site inducted. Those that do not have to be inducted are courier drivers, small parts delivery drivers but must report to the site office for signatures.

# Disposal

- All waste material will be disposed of in waste bins situated at relevant positions within the site. These bins will be emptied at regular intervals as required. Re-cycling to be maintained with refuse sorted into compatible waste bins as required.

- Any hazardous substances or materials (Asbestos) will be disposed of by a licensed removal contractor employing the WorkCover guidelines for disposal of friable or bonded asbestos products including an asbestos removal plan, permit and evidence of disposal at an environment / hazardous disposal dump authority supplied to project management.

- Any concrete spoil will be kept separate and conveyed by truck to a concrete re-cycling facility.

# Contractor Management

  - General

- For each direct contractor management relationship, key representatives from Novo Rail shall be identified to manage the provision of contract deliverables from both Novo Rail and the sub contractor.

- Work carried out on behalf of Novo Rail by the Sub-contractor will be in accordance with:

- The scope of work provided

- The requirements of this plan

- The standards and guidance provided pre or at the time of engagement

- NSW Rail and OHS Legislation, guidance and codes of practice

- Relevant Australian standards.

On this project the following roles, responsibilities and processes will be applied.

Key # - lead role * - support role.  Note not all responsibilities require lead and support roles.  Where none has been defined the responsibility sits with each party in terms of their area of control.

- Project Manager (P), Construction Manager (C), , Health & Safety Advisor (H), General site personnel (G)

| Contracts Management | P | C | G | H |
| --- | --- | --- | --- | --- |
| Request involvement from HSEQ Team in the review of tender documentation for Safety requirements. | # |  |  | * |
| Ensure that safety considerations have an appropriate weighting in the tender review process and that competent Novo Rail persons are consulted in the original specification review and evaluation of submitted tender documentation . | # |  |  | * |
| Undertake formal OHS Reviews of tenders | # |  |  | * |
| Monitor the safe delivery of the contract management. Monitor contractor/supplier performance against contract safety content/safety agreements. |  | # |  |  |
| Ensure that appropriate pre-purchase assessment and evaluation is undertaken in terms of the safety or product/plant and equipment procured where there is a potential of moderate-significant risk. Seek assistance from competent persons as required to assist in identification and evaluation. |  | # |  | * |
| Ensure that appropriate documentation and instruction accompanies purchases. |  | # |  |  |
| Notify the Sub-contractor throughout the project of hazards/risks that Novo Rail is aware of or may generate via communication of the hazard log and involvement in project briefings. |  | # |  |  |
| Ensure the parameters of the Project Safety Agreement are maintained. | # |  |  |  |
| Ensure contractors are provided with copies of relevant systems and site requirements. |  | # |  |  |
| Review relevant training records and licences’ etc of persons attending site. |  | # |  | * |
| Ensure contractors report incidents and hazards occurring on site to Novo Rail. |  |  |  | * |
| Specify monthly summary safety reporting requirements to major contracting firms. | # |  |  |  |
| Include contractor in consultation process. |  |  |  |  |
| Where contract is for ‘labour only’ treat persons in line with Novo Rail general employees for purposes of OHS. |  |  |  |  |
| Undertake reviews of the Sub-contractor’s health and safety documentation specific to the project during the Tender and Mobilisation Phases e.g. Project Specific Safety Management Plans and SWM’s (prepared by contractor), and workers compensation and public liability certificates etc. Method of review to include documentation content and application. | # |  |  | * |
| Review disclosed work processes for potential safety issues in integration or interface or escalation of hazards. Ensure input and communication and addition of site hazards is maintained as a two way process through the hazard log. |  | # |  | * |
| Include all Sub-contractor’s in all relevant pre-site work activities and site briefings e.g. safety induction, safety. |  |  |  |  |
| Schedule into PSMP and undertake recorded activities to verify Sub-contractor’s project health and safety performance. |  |  |  |  |
| Liaise with the contractor on matters affecting project health and safety. |  | # |  |  |
| Reiterate reporting and recording requirements to contractors. Review, investigate and encourage recording of all incident/hazard reports submitted by the sub-contractor, involve in investigation process as required and keep them informed of investigation outputs including corrective actions. | # |  |  | * |
| Report the contractor’s work practices that place Novo Rail, sub-contractor staff or other visitors at risk to their Supervisor and Novo Rail Site Supervisor or Project Safety Advisor immediately after bringing the issue to the attention of the Sub-contractor. |  |  |  |  |
| Formally follow up on reports of unsafe acts and practices of subs contractors and log realistic evaluation of safety performance in suppliers database. | # |  |  | * |
| Monitor compliance of contractor and service providers compliance with submitted safety documentation and disclosed safety management plans |  |  |  |  |

# Site Specific Assessment

- Prior to work commencing, the Site Supervisor and other appropriately skilled members of the Novo Rail Project Team (dependent on the activity nature) shall undertake reviews of the Sub-contractor health and safety documentation specific to the Job.  The RailCorp Template or Novo Rail SWMS Review Checklist shall be used to review the SWMS and documentation provided.  The Novo Rail Contractor Checklist can be provided in advance of subcontracting firm representative’s arrival on site.

- Information with regards to the generic site hazards identified in the hazard log will be provided to the subcontractor so that they can include relevant hazards and controls in their safety documentation.

- Contractors who undertake ‘high risk activities’ or engage subcontractors shall be required to provide safety management plans.

- The Site supervisor will notify the Sub-contractor of any deficiencies that require attention prior to commencement of work in the form of a comment. While reviewing such documentation, it is understood that service providers may have specific discipline expertise (for which they have been contracted) which may be outside of Novo Rails knowledge, therefore by viewing and commenting on the document, Novo Rail are neither agreeing nor authorising the documents, but reviewing them to the best of their knowledge using the resources available.

- Copies of sub-contractor party’s health and safety documentation e.g. risk assessments and SWM specific to their activities on site will be reviewed and hazards that could affect parties outside of the subcontracting firm added to the overarching project hazard log.

- Relevant sections of this Project Safety Management Plan will be communicated to third parties through the induction process.

- Sub-contractors personnel will be required to attend all relevant Pre-start and safety briefings including:

- All/any sub-contractors and their employees are to be treated as working for Novorail insofar as, they must provide before being allowed onsite;

- The Sub-Contract company must furnish Certificates of Currency of all Insurances including    Workers Compensation.

- Be site inducted

- Provide all CoCs & licensing requirements

- Wear the appropriate PPE

- Observe the site safety rules

- Attend PWB each morning.

- Comply with any Emergency Drills.

- Apply for permits as required

- Comply with relevant minimum standards as disclosed at pre engagement stage and/or activity specific briefings.

Contractor safety performance will be monitored throughout the engagement period through site inspection, SWMS (application reviews), observations and site supervisory duties.  Formal reviews of performance will be undertaken dependent on the risk of the activity being undertaken.  Recorded reviews using SMS-13-FM-0168 will be undertaken by the Construction Manager and safety Advisor and records kept in the project NovoWorks safety folder or Novo SEQ database.

# Incident Management

Incidents must be managed according to relevant Novo Rail documentation and the above flow (which is a summary of the content of the Novo Rail HSE Document Incident Management Procedure).  The reporting, escalation and investigation process will meet the general frame work requirements of the RailCorp Safety management System (SMS) elements 15 and 17.

Incident Reporting Flowchart

On this project the following roles, responsibilities and processes will be applied.

Key # - lead role * - support role.  Note not all responsibilities require lead and support roles.  Where none has been defined the responsibility sits with each party in terms of their area of control.

- Project Manager (P), Construction Manager (C), Health & Safety Advisor (H), General site personnel (G)

| Incident Reporting , Investigation and Rehabilitation | P | C | G | H |
| --- | --- | --- | --- | --- |
| Coordinate the investigation process for serious and lost time incidents | # |  |  |  |
| Monitor the timely close out of issues identified within corrective actions register. | # |  |  |  |
| Form and brief appropriate investigation team for all reported incidents and hazards. Ensure team is made up of the appropriate people with required skill sets. Ensure team are capable of identifying root causes and have authority/weighting to seek effective corrective actions.. |  |  |  |  |
| Formally present findings of investigations into potentially significant incidents to Novo Rail HSEQ Team and Senior Management | # |  |  |  |
| Ensure that an interim investigation report is produced within 1 week of the event occurring and (depending on the complexity and potential of the incident- see Novo Rail incident and hazard reporting procedure) a final in depth report identifying and specifying required corrective actions is produced within a maximum of 1 month. |  |  |  |  |
| Implement, record and monitor the effectiveness of identified corrective actions allocated to you. |  |  |  |  |
| Review and consult on the close out effectiveness of potentially significant impacting issues raised at a suitable interval after closure. Interval to be determined at time of closure. |  | # |  | * |
| Promote the reporting of incidents and near misses amongst project staff, including contractors and other site visitors. | # | * |  | * |
| Undertake incident investigations, identify route causes and identify, manage, implement and review corrective actions/ | # |  |  | * |
| Ensure prompt feedback (where permitted) to individuals initiating reports in terms of an overview of investigation outcomes and indentified corrective actions. |  | # |  | * |
| Ensure work area is made safe or situation contained upon notification by a staff member of a hazardous situation, recording the event, background and immediate actions taken in the Novo rail Incident Reporting System. |  | # |  | * |
| Manage the ongoing containment and application of event control until emergency management plan is initiated and emergency incident controller takes on the role. |  |  |  |  |
| Review status of all open incident investigations and hazards weekly. |  | # |  | * |
| Ensure that all persons entering site are made aware of the requirements to report hazards and incidents and are provided with the means and encouraged to do so. |  | # |  | * |
| Notify the Project Manager, Project Safety Advisor (see Incident reporting flow chart for escalation) of “unacceptable” or “high” risk events immediately, and where safe to do so activate containment measures for the incident. |  |  |  | * |
| Notify site controller or owner of incidents occurring on their premises (see Incident reporting flow chart) |  | # |  | * |
| Notify contractors of incidents occurring to/involving their employees |  | # |  |  |
| Promptly report unsafe work practices or unsafe work conditions (including incidents/accidents and near misses) to the Site Supervisor or Project Safety Advisor. Where individuals are unhappy with the response of the project staff email safety@novorail.com.au. |  |  |  |  |
| Make the place of work safe or if unable to do so safely (contain situation as far as possible considering your safety and authority) , notify the Site Supervisor or Project Safety Advisor immediately and report into the Sites Incident Reporting System. |  |  |  |  |
| Cooperate with the Site Supervisor, Project Manager and other staff involved in the investigation process. |  |  |  |  |
| Assist the Site Supervisor during the implementation of corrective actions identified in the investigation, including notifying the him/her where implemented corrective actions are not effective or impracticable. |  |  |  | * |
| Provide a leadership role and assist Project Manager in all investigations pertaining to serious and lost time incidents |  | # |  | * |
| Promptly report incidents/accidents to appropriate HSE Managers who will then report to the relevant authorities, such as the regulator etc (see incident reporting flow chart). |  |  |  | * |
| Review project health and safety data to identify trends in negative outcomes and act promptly to mitigate those circumstances. | # |  |  | * |
| Maintain the security and confidentiality of all incident/hazard report and personal information and documentation. Discuss the information you plan to release re incidents with the HSEQ Team. Treat involved persons with appropriate sensitivity and respect. |  |  |  |  |
| Advise Site Supervisor or Project Safety Advisor as soon as possible of any injury at work, or work related illness and report the incident into the HSEQ Reporting System. |  |  |  |  |
| Advise injured persons of their NOP’s rehabilitation Co-ordinator and prompt where required contractors line managers to do so. |  |  |  |  |
| Liaise with NOP’s Rehabilitation co-ordinator when advised to do so. Forward all medical certificates and related invoices to your NOP’s the Rehabilitation Coordinator |  |  |  |  |
| Advise the Site Supervisor or Project Safety Advisor of absence from work, the need for time off or any special needs during the course of post injury events. |  |  |  |  |
| Update incident report and investigation as new information becomes available |  |  |  |  |
| Support the injured staff member by cooperating with the rehabilitation plan provided by the NOP. |  |  |  |  |

Incidents will be investigated and reported in line with SMS17-PR-0313.  All hazards and incidents including those that do not result in damage will be reported and recorded.  Employees, contractors and visitors shall be made aware of requirements and mode at the site induction.  Project hazard report and incident report forms are available from the site office and should be given to the Novo Rail Construction Manager or the Project Safety Advisor.

A level 5 incident report and investigation form will be completed (for all damage sustaining incidents) by the project management team and forwarded to the HSEQ team and the  RailCorp Project Manager  within 24hrs of the incident occurring. Where a higher level of investigation and/or notification is required this will be advised by the HSEQ team.  Illness and the potential for work related ill health should also be recorded using the hazard and incident forms.

- All incidents/hazards reported as occurring on this project will be appropriately investigated to identify the immediate and fundamental root cause(s) of the incident/hazard. Control measures to prevent recurrence shall be identified, implemented and reviewed. The scope of the investigation will be appropriate and commensurate to the seriousness of the incident/hazard.

- At the time of incident report, every effort shall be made to contain the incident and associated hazards within the realms of safety and maintain the scene for further investigations.  Where benefit can be gained or is required for ensuring safety standards, plant, equipment, persons, area to be quarantined until corrective actions mitigating their risk and investigation into the failure (where applicable) can take place.

As far as is reasonably possible, efforts shall be made to ensure the identity of persons reporting/involved in or assisting in investigations remain confidential.

The level of investigation will be determined by the potential outcome of the incident as opposed to the realised.

# Incident/Hazard Investigation

- All incidents/hazards will be appropriately investigated to identify the immediate and fundamental root cause(s) of the incident/hazard.  Control measures to prevent recurrence should be identified, implemented and reviewed. The scope of the investigation will be appropriate and commensurate to the seriousness of the incident/hazard. The table that follows outlines the incident/hazard investigation process.

- As far as is reasonably possible, where appropriate, efforts shall be made to ensure the details of persons reporting/ providing information to assist in investigations remains confidential/protected.

- The aim of the investigation is not to attribute blame but to identify the basic and route cause(s) of the incident and raise corrective actions to prevent reoccurrence.  As part of this process, non conformances and or non compliances may be identified.  These will be managed in line with the corrective action/improvement process as well as, where necessary through the disciplinary and procurement processes.

| Step | Action |
| --- | --- |
| 1 | Program Safety Manager / Environment Manager will appoint/agree a suitable person/team to investigate (or participate in external investigations of) the incident/hazard. Project Safety/Environment Advisors/Co-ordinators may be requested to assist with investigation. The level of investigation will be determined on potential outcome as opposed to realised. |
| 2 | As a minimum a Level 5 investigation will be completed within 24hrs of the incident occurring and sent to the HSEQ Team and RailCorp Project Manager. Note: further initial investigations are to be completed within a maximum of 5 working days. |
| 3 | Investigator to collect evidence which may include: - Records of testing, inspection, maintenance and repair of plant and equipment - Risk Assessments and Safe Work Method Statements for the activity - Project/Site Specific Safety Management Plans. - Records of Project/Site Specific Safety Management Plan application. - Records of consultation meeting outputs. - Records of previous inspections, audits and reviews. - Records of health and safety training - Records of assessment and selection methods for associated competencies of key persons involved in the project/activities associated to the event. |
| 4 | Investigator to interview eyewitnesses as soon as possible after the incident occurred. Note: should be undertaken when completing the investigation of LTIs and incidents with potentially and attached to the investigation |
| 5 | Investigator to record investigation details in investigation for and the project folder and submit copy to HSEQ team (includes corrective actions and improvement plans). |
| 6 | Program Safety Manager/Environment and Sustainability Manager to review the investigation report prior to finalisation. |
| 7 | Corrective action and improvement plans to be implemented and monitored by the relevant personnel and closed out in a timely manner. Full investigations are to be completed and submitted to the HSEQ Team within seven days of incident occurring (or a written request for extension submitted). |
| 8 | AMT is to monitor the timely finalisation of corrective actions via review of the Safety Action Register. |
| 9 | Where appropriate and where significant changes to workplace arrangements/conditions/practices will result, consultation shall take place on the stated control measure. |

# Serious Incident Reporting

- The various enforcement/regulatory authorities in NSW have statutory requirements for reporting serious hazards/incidents to the regulator.   The Program Safety Manager/Environment Manager will notify the regulator of all notifiable serious incidents following discussion with the Novo Rail HSEQ Manager.  Note the Principal Contractor or RailCorp shall make necessary notifications to ATSB, ITSRR and Department of Water and Energy (outside of the process below).

- In the event a serious incident is identified, the steps outlined in the below table should be completed.

| Step | Action |
| --- | --- |
| 1 | Seek appropriate medical attention or assistance (includes emergency services, doctor etc.). |
| 2 | Take action to mitigate the risk to remaining personnel and make the immediate area safe (if safe to do so and within your authority) |
| 3 | Places or plant that are involved in serious hazard/incidents shall not be disturbed until authorisation from the regulator where it involves: - A fatality - Amputation of a limb - Placing a person on a life support system - Any other hazard/incident involving an immediate threat to life through: - Loss of consciousness of a person caused by impact of physical force, exposure to hazardous substances, electric shock or lack of oxygen - Major damage to any plant, equipment, building or structure - An uncontrolled explosion or fire - An uncontrolled escape of gas, dangerous goods or steam - Imminent risk of explosion or fire - Imminent risk of an escape of gas, dangerous goods or steam - A spill or hazard/incident resulting in exposure or potential exposure to a notifiable or prohibited carcinogen - Entrapment in a confined space - Collapse of an excavation; - Entrapment in machinery - Serious burns In the event of a hazard/incident listed above, persons are not to disturb the place or plant until direct authorised to do so by the HSEQ Manager or his/her representative. |
| 3 | Report the nature and location of the incident to Site Safety Advisor/Environment Coordinator or HSEQ Manager via the fastest possible means. Report the nature and location of the incident to (a) Principal contractor or site controller, (b) Novo Rail project Manager within one hour of notification (c) RMC 9379 444 for incidents with the potential to impact rail safety < this should be undertaken by Project Safety Advisor or Site Supervisor (d) Project Safety Advisor/Environmental Co-ordinator. Complete initial incident report notification paperwork within 12 hours of the incident occurring. Program Safety Manager will determine the level of investigation required and involve the required authorities (not already notified above). The table below will be used to determine the level of incident investigation required. Note: HSEQ Manager will notify the relevant senior management, AMT and in co-ordination with Program Level HSEQ personnel manage ongoing co-ordination with RailCorp, parent organisations and enforcement authorities. Where necessary he/she will also obtain legal advice. See 6.2 for notification timeframes |
| 4 | Record the incident in Sharepoint and email notification to Safety@novorail.com.au and sent to relevant senior management for incidents entered as lost time or medical treatment incidents |
| 5 | Cooperate in any internal or external investigation and follow up allocated actions. |

- Injured persons shall be referred to their NOP or OP Return to Work Co-ordinator for rehabilitation and return to work planning.  Return to work shall be managed and administrated by the NOP, however the Project Manager will co-operate with any reasonable suitable duty requirements in injury management or return to work plans.

- As Novo Rail are not a legal entity advice will be given to the relevant NOP/OP where an event or injury is reportable to the regulatory or enforcement authority.   This notification will be given as soon as reasonably practicable after the requirement is established.

## Emergencies

- Emergency arrangements shall be established and identified at the commencement of the site stage of this project.  The site hazard log will include potential emergency situations that could occur during the works.

- Emergency arrangements (including telephone numbers, assembly points, trained personnel etc) will be communicated and readily available in the site induction, safety noticeboards, and site specific emergency plan and in the proximity of first aid kits and key emergency alert equipment.

- Specific emergency plans shall be indentified on a task basis for potentially high risk scenarios such as:

- Confined space entries

- Working at height

- Dangerous atmospheres and release of gaseous pipeline content

- RailCorp, through their risk register have rail specific scenarios such as train derailment etc.

- Emergency evacuation (including a scenario) shall be undertaken on the site involving all available persons at least twice per year, and the outcome/activity recorded. The drill shall be recorded and any improvement actions submitted.

- Emergency personnel, first aid kits and locations on this project are documented in the project site induction and prework brief. Once site facilities and full site set up have commenced a table showing roles, responsibilities, contact numbers and locations will provided.

- The Site Emergency and Incident Management Plan has been formed under this plan based on RailCorp template based on the requirements identified during the assessment using SMS-15-TP-1424.   The plan will be reviewed in October 2010 for update to current format or on significant change.

- As part of the site set-up, further nominations for site Fire/emergency Wardens will be called for at the first OHS meeting.   The fire warden/s will be issued and wear a red hat (outside of the rail corridor) that is donned for an emergency evacuation or situation.  The first aid officer/s will take directions from the fire wardens.

- The chief fire/emergency warden will assume complete control of the workplace; all staff will act on their command and directions.

- A complete list of relevant emergency contacts including:

- HSEQ Team

- Site emergency contacts

- Fire

- Ambulance

- Local Police

- Local hospital

- Local Medical centre etc

- These details included in the Site Emergency and Incident Management Plan and will be placed on the project safety noticeboard which will be located in site shed and main office and other relevant areas as required/needed.  The location and details of these emergency contacts and procedures will be covered as part of the site safety induction.

## Injuries and RTW

## Injuries

- A First Aid Officer will be onsite at all times during working hours (check your project resourcing to ensure that this is true), the contact details are included in the table above under key contacts.

- The First Aider will record all workplace injuries and report the injury to the Construction Manager as soon as possible after treating the injured person.

- Where site conditions/activities or numbers change significantly the first aid arrangement will be re-assessed (according to clause 20 of the OHS Regulations and suitable changes in arrangements made to meet requirements.

- The Construction Manager will make arrangements for notification to the Project Manager, Project Safety Member and Novo Rail HSEQ Team.  Where the injury/treatment given is work related the Construction Manager will also notify the RailCorp Injury Hotline 1800 772779 and make note of the incident reference number (which he/she will also communicate to the above parties).   The above notifications will take place as soon as is practical and within a maximum timeframe of 12 hours.  The investigation protocol from the above sections will be followed as required.

- Return to Work coordination will be managed according to the procedures of the originating alliance partner.

- Novo Rail project management will contribute to the return to work plans at the request of the Return to Work Co-ordinator (in the form of background information to the incident and injury and the capabilities of the project to accommodate suitable duties).  They will also implement the requirements of the return to work plans by accommodating suitable duties where practical.  The process will remain under the control and management of the NOP/OP until a final certificate is issued.

- Details of the NOP RTW Coordinators are:-

- RailCorp injury hotline number 1800 772 779  24 hour hotline – SELF INSURER

- All information regarding incident / injury/ Worker’s Compensation

- Liang O’Rourke -  Kylie Gannon Landline 02 4932 3636, Mobile 0438 751 650

- O'Donnell Griffin- Peter Weston Ph 02 9947 7890, Mob 0412 174 301 or Filipa Train 02 9947 7833

- AURECON Hedy Babi – Health and Rehabilitation Manager (08) 8237 9687

- These details will also be posted on the project safety noticeboard located in both site sheds.

## Unexpected Findings of Suspected Asbestos Material

- All reasonable precautions and investigations have been undertaken during the planning stages of the project to identify potential asbestos and other hazardous contamination of the site.  The output of these investigations and associated plans can be viewed in the link below.    Known asbestos containing locations are included in the project hazardous materials Register accessible in:

- SIG05 - Sulphide Junction > Project Records > Execution > 09 Safety > Hazardous Material > Asbestos

The following steps are to be followed in the event of the accidental disturbance of suspected asbestos containing materials or identification of suspected new contamination.  Site persons will receive briefing in the unexpected findings of suspected asbestos containing materials process flow as part of the site induction.

| Step | Who | Action |
| --- | --- | --- |
| 1 | Construction Manager/Project Manager | Remove personnel from areas considered to be at risk in relation to asbestos exposure. Raise hazard report. Go to Step 2. |
| 2 | Construction Manager | Exclusion zone to be set up. Exclusion zone to be at least 1.5 meters outside of the affected area. Where safe to do so area to be covered with plastic sheeting, sand bags or damped down (from a distance). Drains downstream from the contamination to be covered as an interim measure. Occupational hygienist to provide guidance on when covers can be removed. Access to the area should be controlled and sign posted to prevent unauthorised persons entering or disturbing the area. Make a log of potentially exposed persons. Inform appropriate personnel and brief site personnel. Go to Step 3. |
| 3 | Project Manager, Occupational Hygienist & Licensed Asbestos Removalist | Contact an Occupational Hygienist to confirm the presence of asbestos containing materials, advise on potential exposure of those involved and to provide guidance on appropriate control strategies. - Occupational Hygienist for use in emergencies Ross Nefodov, AD Envirotech, 0402 389977 If occupational hygienist advises that a positive analysis result is likely continue with the following points. Following advice from the Occupational Hygienist and procurement Co-ordinator, engage a licensed AD1-B and/or AD1-F asbestos removalist to undertake asbestos clean up works. SWMS to be received from the licensed removalists including suitable respiratory protection and disposable coveralls. SWMS to detail Provision of temporary sealed asbestos waste receptacles and the removal and disposal of the asbestos containing material, cloths, vacuum bags, filters and other contaminated materials etc. Asbestos Register to be produced. Asbestos Management Plan to be produced. Asbestos Removal Work Plan to be produced. Asbestos removal work specification to be produced. If potential exposure above national levels is identified, a log of potentially exposed persons to be included in incident report and the original hazard report upgraded to be managed through the incident reporting system. HSEQ team to assist project with follow on actions from this point. Go to Step 4. |
| 4 | Occupational Hygienist | Monitoring outside the area of the asbestos contamination is to be conducted while clean-up works are being conducted to ensure that dust levels do not exceed national exposure levels. Go to Step 5. |
| 5 | Occupational Hygienist & Asset Manager or Workplace Controller | After clean-up works have been completed, airborne monitoring is to be conducted in the affected area to ensure that asbestos fibre levels are at an acceptable level. Only when the asbestos level is acceptable and the clean up works have been conducted to a satisfactory standard, will a clearance certificate be issued to allow personnel to reoccupy the affected area. |

# Training

- Where possible training and competence will be managed on this project in line with the requirements of RailCorps Managing Training and competence procedure SMS-11-PR-0747.

On this project the following roles, responsibilities and processes will be applied.

Key # - lead role * - support role.  Note not all responsibilities require lead and support roles.  Where none has been defined the responsibility sits with each party in terms of their area of control.

- Project Manager (P), Construction Manager (C), Health & Safety Advisor (H), General site personnel (G)

| Training | P | C | G | H |
| --- | --- | --- | --- | --- |
| Assist in the identification and review of project competency based training and induction needs and agree on suitable team. | # |  |  | * |
| Ensure basic task competency assessment of nominated employees is included in the team selection process of all Risk Assessments. | # |  |  |  |
| Ensure the security and confidentiality of personal data provided. |  | # |  |  |
| Ensure competent individuals and organizations are engaged to undertake training when organized specifically on behalf of the project. |  |  |  |  |
| Ensure Novo Rail project persons receive required and identified training and maintain related competencies. |  |  |  |  |
| Ensure records are kept of training provider evaluation, training content, training attendance and post course evaluation. |  | # |  | * |
| Cross check that Novo Rail staff and sub-contractors satisfy the Novo Rail health and safety induction and training requirements before they commence work on the project. View evidence and record. |  | # |  | * |
| On a representative sample, basis check that all training requirements disclosed in SWM’s are held for contractors. |  | # |  | * |
| Provide recorded OHS Instruction within authority and competence based on project documented and approved methods, processes and procedures. |  | # |  | * |
| Communicate site and task safety training requirements to relevant parties at earliest possible occasion. | # | * |  | * |
| Monitor that appropriate training has been completed and relevant certificates of competencies are held. |  | # |  | * |
| Liaise with the relevant Project Manager to arrange health and safety training and other qualifications as highlighted in site requirements and Novo Rail SWM’s prior to a person commencing work on the project. |  | # |  |  |
| Arrange for the delivery of site induction and pre-work assessments prior to the commencement and during delivery of the project. Review induction process and content on significant change to hazards or arrangements. |  |  |  | * |
| Provide safety training with the assistance of the Construction manager and project Safety Advisor | # |  |  | * |
| Assist the Project Manager to provide suitable safety training. |  |  |  |  |
| Produce safety training matrix for site. | # |  |  | * |
| Arrange and monitor the completion of training identified in training matrix, PSMP and site activity controls. | # | * |  |  |
| Plan and deliver site safety induction and toolbox talks. |  | # |  | * |
| Ensure that refresher training is triggered and undertaken for project team. | # |  |  | * |
| Maintain site training records and forward records to the Novo Rail Training Team for maintenance. |  | # |  |  |
| Periodically review all training records against minimum site/project SWM’s requirements. |  |  |  |  |

## Safety Induction

Safety Inductions will be managed in line with RailCorps SMS System Requirement for inductions SMS-11-SR-0218.

All persons attending the project either to undertake activities or to visit a project representative will complete a site safety induction.

Comply with the health and safety induction requirements of the project and will attend the site controller and where they differ Novo Rails) Project Health and Safety Induction.

A record of every person who undergoes the Project Health and Safety Induction will be maintained in The Project Induction Register by the controller of site.

The Project Safety Induction will be run by a representative from the Project Management team who is familiar with SMS-11-GD-0629 and SMS-11-GD-0639  .

The Induction refresh requirement will be on any significant change or annually.

The Project Manager will cross check a sample of those persons on site against the names on the Project Induction Register maintained by the on site project team on a quarterly basis, as part of his quarterly review.

Where conflicts arise, the Project will receive a corrective action against the implementation of   this management plan and associated RailCorp Safety Management System elements..

The responsibility of ensuring that systems are in place and implement to ensure all employees, contractor’s employees and visitors that present to site are in possession of a WorkCover NSW General Safety Induction into the Building Industry and are immediately site specifically Inducted is the overall responsibility of the Novo Rail Project Manager.

The duty of ensuring this will, on a daily basis be delegated to the site team including the Construction Manager and Project Safety Advisor.  The responsibility however shall remain with the Project Manager. Any person presenting to site will be site safety inducted before any work is allowed to begin.  At this induction the details of the site will be explained with reference to:

- The Site Induction will be as per RailCorp SMS and the following documents used to prompt and record induction material

- Induction register

- Local Induction - Visitor or contractor

- Local Induction - Employee

The Inductee will be asked to provide evidence of:

- Work Cover White (or Green) Card

- Rail Industry Safety Induction (RISI)

- Driver’s License

- Any CoCs and WorkCover Operator’s accreditation

- Emergency contact details of their next-of-kin

- If a plant operator, any manuals and service history of all plant and equipment If introducing any chemicals or fuels to the worksite, an up-to-date MSDS sheet must be presented

The Inductee is required to sign-off that information provided in the induction and been received and understood.  They will be provided with a worksite identification sticker. Details with regards to emergency contacts, date of birth, employing firm and current address will also be taken at the time of induction.  This information will be subject to data protection and stored in a secure location..

## Hazard Specific Training

The training requirements for Novo Rail Staff will be determined via a review of the Project Hazard Log and associated legislative and RailCorp requirements.  Novo Rail staff working on the Project will receive such training prior to commencing work on the Project. The training requirements will been identified in the projects Training Matrix   On all other occasions, on the job training will occur prior to undertaking activities as part of the Novo Rail, SWMS assessment and briefing process.

Training will be carried out by a competent Staff member or by an approved outside training provider and recorded in all cases. The site project staff will monitor the application of this process through cross checking a sample of those persons on site against the names on the Project Induction Register, and requesting evidence of training and qualifications listed in SWMS and the project training matrix.  The Project Manager will audit the process through his Project Safety Management Plan audit. Where conflicts arise, the Project Manager will receive a corrective action against the implementation of this management plan and associated SMS elements.  The Project manager will maintain the update population of the training matrix through the duration of the project.

The following table sumamrises mandatory training requirements.  Certificates of competency (CoC) will be reviewed in line with SMS-11-GD-0243 and SMS-11-GD-0244 .Novo Rail Job Training Requirements

| Training/Certification | Stakeholder | Special requirements | When | Responsible to check |
| --- | --- | --- | --- | --- |
| e.g. WorkCover OHS General Induction | All Project personnel under NOVO RAIL control | Card to be available at all times | Prior to arriving to project | Site Supervisor |
| RISI |  |  | When arriving at project | Safety Advisor |
| Site Induction |  |  | When arriving at project | Safety Advisor |
| Confined Spaces |  |  | When arriving at project | Safety Advisor |
| CoCs |  |  | When arriving at project | Safety Advisor |
| Asbestos Awareness |  |  |  |  |
| Asbestos Removal (friable) |  |  |  |  |
| Asbestos Removal (bonded) |  |  |  |  |
| Asbestos removal Supervior |  |  |  |  |

## General Training

The Project Manager is responsible for

- Training and the training matrix

- Identifying the training needs of management, supervisors, employees, service providers and visitors in relation to project hazards and risk

- Monitoring the identification and compliance with training requirements identified in relation to activities.

- providing safety induction training, task training and refresher training

- providing Safety Committee and Safety Representative consultative training

- specific training necessary to conform with safety requirements

- providing training in emergency procedures

- keeping appropriate records of safety training

- making sure all personnel who will be on the project work site have been inducted in accordance with

  - the requirements of the NSW Code of Practice

  - Refresher training conducted after a significant change or near miss.

The above responsibilities will remain with the project manager, but some of the duties may be delegated to the site team for application.  When this occurs the Project Manager will ensure that the process is monitored and this monitoring recorded.

# Inspection, Testing, and Servicing

The project manager will ensure that inspections are managed in accordance with the RailCorp System Requirement for Inspection SMS-16-SR-0057.

## Site Safety Inspection

The project team will carry out the site safety inspections on weekly basis.  Electronic and hard copy records of safety inspections will be made at least weekly.  Electronic copies will be scanned into the Project folder in NovoWorks or on the launch of the Novo SEQ system recorded in the CRM Database.

The follow table details the inspection requirements:

| Site Safety Inspection & Audit Checklists |  |  |  |
| --- | --- | --- | --- |
| - Who | - Form | - When | - What |
| Novo Rail Project Staff (construction manager, safety advisor, site engineer or project manager. Note PM to ensure at least one of these persons completes the task) | Construction & Maintenance Inspection SMS-06-FM-0744 Hazard related Inspections Element (16) SMS Topic specific Inspection checklists*. Periodic Contractor inspection - SMS-13-FM-0168 Safety Observation- SMS-10-FM-1356 | Monthly Weekly (the reoccurance of the self build checklist to occur at least bi monthly) Weekly Frequency may increase during higher risk period of work. Daily when work is being undertaken in either the construction zone or rail corridor. | Whole site and arrangements In line with requirements specified in 3.4 and issues on corrective actions register Contractor to be selected based on work being undertaken and level of risk Individual application of safety requirements. |
| Safety Advisor | SMS-16-FM-0070Field inspection | Monthly and daily during possessions | Whole site and arrangements |
| Project Manager | Construction and Maintenance Inspection SMS-06-FM-0744 SMS-06-FM-0303 Project Safety Management Plan Audit Observation - SMS-10-FM-1356 | Quarterly 6 monthly At least monthly | Whole site and arrangements Application of PSMP and related systems Individual application of safety requirements. |

The Project Manager, Construction Manager or a competent delegated authority will undertake regular scheduled health and safety inspections as per the above table of all works being undertaken with a direct relationship to Novo Rail activities to measure compliance with Novo Rail health and safety contract agreement implementation, legislative requirements and specific site safety rules. Assistance may be sought from the Project Safety Advisor to assist undertake some of these inspections.  At least one site safety inspection shall be undertaken weekly on this project.  Inspections will include site conditions and arrangements include security, welfare, boundaries and access and egress.

Inspection and observation frequencies above the weekly requirement shall be determined by the SWMS for an activity. As a guide daily inspections or observations will be undertaken daily for high risk activities.

Periodic site inspections (which include third party operations) will be undertaken again dependent on activity risk.

The Project Manager will be included in the circulation of all safety inspections undertaken on the project.  Corrective and improvement actions identified during the audit will be recorded on the projects corrective action register.  The register will be reviewed weekly to ensure close out within timeframes.  Closed action will be reviewed to ensure effectiveness.  The Project Manager will perform a review of the IIMS Managers Safety Inspection (I am not familiar with this document please review the statement) weekly and sign-off.

The project team will undertake at least once per week safety observation when works are being undertaken in the construction zone or rail corridor.  Observation records will be recorded on the observation form SMS-10-FM-1356 and stored in the project safety folder in Novo Works or on Novo SEQ . Actions generated will be added to the projects corrective actions register.

On this project the following roles, responsibilities and processes will be applied.

Key # - lead role * - support role.  Note not all responsibilities require lead and support roles.  Where none has been defined the responsibility sits with each party in terms of their area of control.

- Project Manager (P), Construction Manager (C),, Health & Safety Advisor (H), General site personnel (G)

| Inspection and Monitoring | P | C | G | H |
| --- | --- | --- | --- | --- |
| Develop an inspection and monitoring schedule in line with the planned project activities and potentially high risk events. Allocate key inspections and monitoring activities to specific roles and specify type of activity. |  |  |  |  |
| Review and update registers and project inspection, briefing, review and monitoring schedule in line with planned project high risk activities, information received from other parties and implement and record findings. |  |  |  |  |
| Implement and monitor all safety related schedules and plans within your remit. |  |  |  | * |
| Review project inspection schedule and monitor its delivery. |  |  |  | * |
| Conduct health and safety inspections as per schedule and to coincide with high risk activities or progress milestones. |  |  |  | * |
| Ensure that verification notes are taken during inspections, reviews and audits, and the corrective actions are allocated to appropriately responsible persons, root cause of non conformance addressed and timeframe for close out is based on associated risk. |  |  |  | * |
| Monitor and report on the timely close-out of corrective actions arising from inspection and monitoring activities. |  |  |  |  |
| Prepare and authorise the health and safety inspection and monitoring schedule . |  |  |  |  |
| Assist in inspection, review and auditing activities with and open and honest attitude on request. |  |  |  |  |
| Deliver and update the inspection and monitoring schedule. Record activities and findings. |  |  |  | * |
| Review the Corrective Actions Register and output of inspections, audits and reviews. Follow up corrective actions requirements for timely closure and effectiveness. |  |  |  | * |
| Engage where appropriate, suitably qualified persons to carry out inspection and monitoring . |  |  |  | * |
| Enter inspection outcomes Into the Corrective Actions Register and follow through completion and implementation. Identify corrective actions that address root cause of non conformance. |  |  |  | * |
| Implement corrective actions identified as a result of inspection and monitoring activity in a timely fashion and close out in register. |  |  |  |  |
| Review close out effectiveness of potentially significant impacting issues raised at a suitable interval after closure. Interval to be determined at time of closure. |  |  |  |  |
| Ensure development and implementation of project Inspection and testing plans. | # | * |  |  |
| Provide advice on corrective actions which address root cause of issues identified and assist in identifying practicable timeframes considering potential associated risk. |  |  |  |  |

Where non-conformances are observed involving repeated or high potential safety breaches, the safety breaches are to be photographed and included in the inspection report.  Where breach involves a third party the non-conformance will be fed back to procurement on processing of the non conformance for inclusion on the firms record.

## Plant and Equipment Inspections, Testing and Services

The site safety inspections will be carried out by the Construction Manager, Site Safety Advisor or other competent person, and these inspections will be carried out weekly and a summary provided to the Program Safety Manager monthly.  Part of the site safety inspections will include a review of plant and equipment standards and register review.

Service providers and suppliers are required to supply plant and equipment with appropriate inspection, test and servicing records and plans.  The documentation shall be available to the Novo Rail Project team for inspection on request.

Plant and equipment inspection and testing scheduling will be included in the Project Inspection and Testing Plan.  The project inspection and testing plan will be produced by the project manager and construction manager prior to main works commencement and included as an appendix to this plan.   The inspection and testing plan will also include the requirements to inspects construction against design during specific phases.  It will be reviewed periodically to check and trigger required maintenance inspection and testing requirements.  Plant maintenance and inspection requirements will be recorded on SMS-16-FM-0085.  The application of the requirements will be arranged and monitored by the Construction Manager.

Initially when plant arrives on site, a plant safety inspection checklist will be undertaken to ensure safety compliance with the information disclosed during pre-purchase and records of plant maintenance, inspection, testing etc.  The servicing & manufacturer’s compliance plate records will be reviewed prior to any plant being allowed to be used on Sulphide Junction site.  The following checklists have been produced for specific types of plant (above and beyond the generic checklist requirements):

- Front end loaders

- Tele Handlers

- Elevated Work Platforms

- Mobile Crane (note requirements of AS1418 must also be considered)

Where equipment is potentially of medium - high risk, or will be on site for long duration (accumulatively over once month), the equipment will be added to the sites plant and equipment register and the RailCorp Plant Hazard identification & Risk Assessment undertaken.   All plant to be used on this site will have the appropriate checklists and controls applied on arrival at site.  .  All manufacturers’ compliance plates will be sighted and the service history will be recorded in the plant service history logbook.

These records will be stored both electronically & hard copy in the site office, folder #19 & in hard copy in the site folder on form SMS-16-FM-0229.

Where plant or equipment is brought onto site by a service provider, a similar level of information will be required from the firm and a copy of the records or reference to their whereabouts on site provided to the project management team for review.

The Construction Manager has the responsibility for controlling acceptance, monitoring  and implementing the requirements with regards to site plant and equipment.  The project Safety Advisor will assist in the application of these responsibilities

The frequencies of plant inspections will be determined by the Plant & Equipment Risk Assessments and detailed on the Project Inspection and testing Plan and Plant & Equipment Registers as applicable.

A Daily Pre-Start Checklist will be carried-out by the operator & passed on to the Project Safety Advisor who will record in the site folder, in hard copy.  Where an incident or event occurs which may have detrimental effect to the plant or equipment, it is the operators responsibility to remove the item from service and bring the issue to the attention of the Project Safety Advisor.  Additional inspection and testing will be arranged as necessary, and the event and corrective/investigative actions recorded.

Electrical testing

Electrical equipment registers shall be maintained on site and regularly updated.  All portable electrical appliances on site will be inspected, tested and recorded by a competed individual monthly (unless triggered by an event prior).

The cabin/structure electrical installation supply will be inspected, tested and recorded quarterly by a competent individual and on installation.  Where the electrical installation is deemed unfit for use,  Works within the cabin/structure will cease until rectification and pass criteria is met and certified.  .

# Document Control

All documents will be stored electronically on the Novo Rail Sharepoint, with a quarterly update of hard copy registers etc. and hard copy retained in the site folders for inspection.

On the introduction of Novo SEQ, inspections, audits, reviews, incidents, hazards and corrective actions will be entered and managed through the relevant Safety Module.  All entries are auditable and traceable.

On this project the following roles, responsibilities and processes will be applied.

Key # - lead role * - support role.  Note not all responsibilities require lead and support roles.  Where none has been defined the responsibility sits with each party in terms of their area of control.

- Project Manager (P), Construction Manager (C),, Health & Safety Advisor (H), General site personnel (G)

| Documents and Records | P | C | G | H |
| --- | --- | --- | --- | --- |
| Review the identification and maintenance of project health and safety records as part of the review program and include in the project safety management plan schedule. |  |  |  |  |
| Upload all planned safety reviews, inspections, audits and briefings into the project safety schedule. |  |  |  |  |
| Establish the system for the collection, indexing, filing and storage of Health and Safety records. Ensure that the file structure replicates Rail Corps 20 SMS Elements. Maintain and manage records in accordance the elements requirements. |  |  |  |  |
| Ensure that all plant, equipment, hazardous materials, substances and dangerous goods brought onto site, have appropriate records including (as appropriate) risk assessments, instruction manuals, safe operating/use instructions, maintenance schedules and records, MSDS’s, warrantees (see also procurement requirements for new items). |  | # |  |  |
| Collate copies of such records in the projects safety filing system, and communicate and use where appropriate. |  |  |  |  |
| Provide advice on the system for collection, indexing, filing and storage of health and safety records as well as recording requirements. |  |  |  | * |
| Identify the relevant project health and safety records in accordance with Novo Rail, RailCorp and NSW Requirements. Identify Gaps in your knowledge of these prior to project commencement and include in pre project training and competence planning. | # |  |  | * |
| Ensure evidence of and maintain identified health and safety records. | # |  |  | * |
| Ensure that all site safety records are maintained on site and uploaded onto the relevant project files in the Novo Works folder at least quarterly. | # |  |  | * |
| Ensure the security and confidentiality of sensitive data provided. |  |  |  | * |

## 10.1 Document control

Documentation control is the responsibility of the Project Manager (overarching records) and the site Construction Manager (site Records).  The Project Safety Advisor will assist and provide support in meeting the documentation control requirements for Safety Documentation.  The Construction Manager and Project Manager will understand the documentation retention, issue and maintenance requirements as well as being able to locate records when required.

## 10.2 Records management

Where possible the records management will follow the principles of RailCorp SMS-05-RG-0220 and SMS-05-SR-0027 and will follow them from commencement to finalisation of the project with the timelines for records retention management adhered to (see table below).

A firm process for records management will be established after the June Possession.  For works pre and during the June possession (see table below for details of records highlighted for records management), the records will be completed and stored until the formal process is set up.  The documents will then be filed in retrospect.

Records Management will be according to the RailCorp SMS-05-RG-0220 and will follow this template until the finalisation of the project with the timelines for records retention management adhered to.

All documents will include a;.

- GA reference – General (Retention and Disposal) Authority (GA) number against which documents are sentenced for retention or disposal.

- Retention Period – The period the record must be retained after its most recent update.

- Review Interval – The interval between reviews of the record type, its template, its role in the SMS and whether or not the records produced remain useful.

- Review by – By whom the review is to be conducted.

An example of Records Management is found below.

| SMS Document/ Novo Rail Record | Record Type | Vital | Confidential | Reference | Retention period |
| --- | --- | --- | --- | --- | --- |
| Safety Score Card | Positive Safety Performance Indicator Reports | N | Y | FRDA 10.14.2 | Retain for minimum of 2 years after action Safety Performance completed. |
|  | Management Reviews of Safety Performance Reporting | N | Y | FRDA 10.16.1 | Retain for minimum of 7 years after action completed. |
|  | Corrective actions forms | N | N | FRDA 10.5.1 | Retain minimum of 5 years after action completed. |
| Workplace Risk Management | Hazard report forms | N | N | FRDA 10.16.1 | Retain minimum of 7 years after action completed. |
| Workplace Risk Management | Workplace Risk Assessment forms | N | N | FRDA 10.2.2 | Retain minimum of 30 years after action completed; then review prior to disposal. |
| Safety training records and register | Training records |  |  |  | Life of project |
| Committee Min |  |  |  |  | 7 years |
| Skills competency and licence register |  |  |  |  |  |
| Safety audit and inspection reports |  |  |  |  |  |
| Internal review |  |  |  |  |  |
| Illness/injury statistics – scorecard and investigation reports |  |  |  |  |  |
| OHS and rail safety design review reports |  |  |  |  |  |
| Site safety rules |  |  |  |  |  |
| Induction register |  |  |  |  |  |
| First Aid and fire warden lists |  |  |  |  |  |
| Emergency procedure |  |  |  |  |  |
| SWMS |  |  |  |  |  |
| Hazard ID and Risk assessment reports |  |  |  |  |  |
| Injury register and incident notifications |  |  |  |  |  |
| Records of tool box meetings/talks |  |  |  |  |  |
| PPE issue register |  |  |  |  |  |
| Register of plant and electrical equipment |  |  |  |  |  |
| Permits |  |  |  |  |  |
| MSDS |  |  |  |  |  |
| Hazardous substances register |  |  |  |  |  |
| Inspection and test reports |  |  |  |  |  |
| Plant and equipment servicing records |  |  |  |  |  |

All records will be recorded & stored in the site folder #19 & electronically on the Novo Rail Sulphide Junction SIG-05 website.

The Project Manager & the Construction Manager are responsible for the secured storage of all documentation.  The safety advisor will assist in the application of the requirement.

# Training Register and records

Records Management will be according to the RailCorp SMS-05-RG-0220 and will follow this template to finalisation of the project with the timelines for records retention management adhered to.

This includes;

- GA reference – General (Retention and Disposal) Authority (GA) number against which documents are sentenced for retention or disposal.

- Retention Period – The period the record must be retained after its most recent update.

- Review Interval – The interval between reviews of the record type, its template, its role in the SMS and whether or not the records produced remain useful.

- Review by – By whom the review is to be conducted.

- All records will be recorded & stored in the site folder #19 & electronically on the Novo Rail Sulphide Junction SIG-05 website.  Hard copy records will be uploaded into Novo Works at least quarterly.

- The Project Manager & the Construction Manager are responsible for the secured storage of all documentation.

# Operational control

# Design Verification

Each subsystem requirement must be verifiable. Baseline documentation developed during the Design Phases must establish the method of verification for each requirement. Each subsystem design is verified against the subsystem requirements developed in each phase.

Several methods may be used as part of the process of verifying design output, including:

- Technical reviews for major projects.

- Checking and approval by engineers who are properly authorised.

- Independent design checks.

- Development tests.

- Modelling and simulation.

The Design Verifier must be independent of the design production, and is nominated by the PDM or Discipline Design Manager (DDM) by means of a Verification Brief & Record (VBR) form.

The Design Verifier record verification methodology and findings in the VBR form, and documents any design deficiencies identified in a Verification/Validation Defect Log (VDL) and must not alter designs or provide the design solution.

Novo Rail Reference:

- 08-00-05 Verification Brief / Record (VBR) form

- 08-00-06 Verification Defect Log (VDL) form

RailCorp Reference:

- EPD 0011 Design Verification

# Design Hazard and Risk Analysis

The purpose of the Hazard and Risk Analysis is to:

- Identify potential hazards associated with the project

- Identify all relevant stakeholders and interfaces that could impact on safety

- Identify human factors issues

- Undertake an assessment of the risk according to the safety risk management requirements.

RailCorp Reference:

- ED 0008P Hazard and Risk Analysis,

- SMS-06-GD-0031 Hazard Identification and Safety Risk Assessment Guide

SMS-06-SR-0030 Safety Risk Management Novo Rail uses the RailCorp Design Control methodologies and includes:

- 08-04-01 System Engineering Management Guidance (for information only)

- 08-11-03 Systems Engineering Workflow (SE workflow – SW Heavy)

- 08-11-04 Systems Engineering Activity Schedule (SE-activity spreadsheet – SE Heavy)

- 06-00-01 Novo Rail project methodology (NRPMM)

- RailCorp RPMM RAE Guidelines: RPMM Base Roadmap

# Safety in Design

Safety in Design (SID) workshops should be undertaken at the preliminary and detail design phases, allowing the designers, constructors, operators and maintainers to come together and identify risks and hazards inherent in the proposed design which can either be designed out or mitigated.

An SID specialist (could be Safety Change Advisor) should facilitate SID workshops.

SED safety responsibilities are defined in 08-12-00 Project Safety Responsibility Matrix.

RailCorp SMS system requirements, procedures, guidance, forms and templates applicable to SED are defined in 08-12-01 Engineering Safety Management Plan.

Novo Rail Reference:

- Aurecon “Safety in Design Guidance Notes”, Rev1

- 08-12-00 Project Safety Responsibility Matrix

- 08-12-01 Engineering Safety Management Plan

# Design Review and Acceptance

For all drawings submitted prior to AFC (i.e. Concept, Preliminary & Detail designs), it has been agreed with RailCorp Professional Services Division (PSD) that it is not necessary to provide “wet” signatures on pre-AFC drawings produced and checked at offices that are geographically dispersed (e.g. Pitt Street, Neutral, Brisbane and Melbourne).

The relevant drawing signatories must send an email testifying that they have drawn, checked, verified or approved a drawing, and this email must accompany the Design Release Checklist (DRC).

Once the drawing is to be submitted for acceptance by RailCorp as ‘Approved for Construction’, then it must have all the relevant wet signatures.

It has also been agreed that due to the delays in obtaining wet signatures from designers and checkers in geographically distributed offices, the ‘For AFC’ drawings can be submitted without the wet signature but accompanied by an email from each signatory. Note that this is a temporary measure to expedite the acceptance of plans that have full supporting evidence of having been properly designed, checked/verified and approved for release by the Novo Rail PHs and the SED Manager, and is only in place to save the 1-2 weeks delay associated with obtaining all the wet signatures on the drawings.

The designs will be subjected to a series of Design Reviews as described below, and illustrated in the figure in Appendix D.

# Preliminary Design Review (PDR)

The Preliminary Design Review (PDR) is the formal acceptance gate to allow a project to proceed to the Detailed Design Phase. The PDR measures maturity of the Preliminary Design Baseline against pre-established exit criteria, to ensure that it meets the agreed requirements, identifies and reviews technical risk, and determines progress to the next phase.

The PDR is conducted by the authorised person, with the design review checklist (DRC) completed, and details of the PDR documented in the project action trackers and risk registers.

Following development of the preliminary design (typically the development of the preferred option, full specification, typical General Arrangement drawings, and new equipment lists), the preliminary design and supporting Target Outturn Cost (TOC) is submitted to RailCorp for review and acceptance.

Novo Rail Reference:

- 08-00-11 Preliminary Design Review (PDR) checklist

RailCorp Reference:

- EP280 Process for Acceptance of Engineering Design and

- EPD 0013 Technical Reviews

# Critical Design Review (CDR)

The Critical Design Review (CDR) takes place at, or close to completion of the Detailed Design phase. The intent is to ensure that the design is suitable in all respects to proceed to construction/fabrication of hardware, or software coding.

The CDR is conducted by an authorised person, with the CDR checklist completed and details of the CDR documented in the project action trackers and risk registers.

Following review and approval of the detailed designs (focusing on the low level site-specific design aspects, calculations, detailed drawings, and documents for construction) by the PHs, these are submitted to RailCorp for final review and acceptance as “Approved For Construction” (AFC).

Novo Rail Reference:

- 08-00-12 Critical Design Review (CDR) checklist

RailCorp Reference:

- EP280 Process for Acceptance of Engineering Design and

- EPD 0013 Technical Reviews

# Design Change

Under some circumstances it may be necessary to deviate from the approved design documentation due to unforeseen conditions encountered during construction work.

Any change that would result in a departure from the approved design documentation shall be documented, and shall be managed and approved in accordance with the following process.

Novo Rail Reference:

- 08-05-01 for the Design Change Process

- 08-05-02 Design Change Note form

RailCorp Reference:

- EPD0010 Design Approval & Acceptance

# Authorities and Permits

Permits will be issued and/or extended by the construction manager or other competent individual delegated by the Project/Construction Manager. Permits will not be issued until they have been reviewed in accordance with SMS requirements. Persons seeking permits must apply no less than 15 working days in advance of requirement.

Permits shall be kept in a control register containing the following details

- Permit Number

- Permit type

- Location

- Work package (WBS) number

- Date requested

- Date granted

- Expiry details

- Extension details

- Requestor

- Approver

- Withdrawal date

The following permits shall be employed on the project; however the need for additional permits type will be included in PSMP reviews

- Laing O’Rourke Excavation Permits used in conjunction with Excavation Plans

- Electrical Work Permits

- Hot Work Permits

# Requesting, Receiving and Returning Work Permits.

All work requiring permits will be assessed by the Construction Manager and the Project Manager prior to any work beginning.

Permits will be:

- applied for, by person in charge of the works,

- accompanied by a detailed safe work method (which include the permit as part of the required controls), and include permit required controls implemented and referenced (such as isolations, immobilisations, atmospheric tests etc)

- issued direct to working party leader prior to the work commencing.

- Communicated (records of communication) to all members of the working party.

In some cases, all members of the working party will be required to sign onto the permit; and a copy of the permit provided to the working party leader.  Where possible copies of these permits will be made available to the Possessions Officer (where one is engaged) and communicated to site staff at the PWB daily.  Where the permit is issued after the PWB a specific briefing will be arranged for relevant persons to make them aware of the permit, it’s constraints, requirements and associated controls.

Permits are non transferrable.  Where work is to be undertaken across shift so that the person in charge of the working party is to change, the permit will be withdrawn and re issued (accompanied by the appropriate briefings and retests).  Where persons working under the permit change (excluding the working party leader), work must stop and a recorded briefing be undertaken.

Where a permit is required to cater for emergencies, special conditions shall apply. The requestor will phone the Construction Manager and relay details. The Construction Manager shall seek the advice of the Novo Rail Safety and Environment team with regards to the most appropriate process for issuing permits outside of the normal process.

When either the work concludes or the permit expires (whichever occurs first), work must cease and the process for reissue of permit followed.

Permits shall be returned once work is finalised and personnel removed from the work area.  The original recipient of the work permits shall sign the permit to confirm that all persons and equipment have been removed from the area and work ceased.  Prior to the permits (and associated controls) being cancelled and removed by the Construction Manager, he will visually establish that this is the case.

Where work is undertaken under a permit, it shall be subject to a high frequency of review by Novo Rail Site management staff.

# PSA Project Safety Agreement

Works undertaken by the project will be documented in a Project Safety Agreement (PSA) and sent to Infrastructure Maintenance North for authorisation in line with RailCorp requirements. Individual PSA’s will be compiled and submitted by the construction manager for possession and non possession works. No work on site will be undertaken without the authority of a PSA

To date PSA’s have been compiled and submitted for Possession 1 and Enabling works. No work additional to that specified in the PSA will be undertaken without revision and re-approval of the PSA

The requirement for PSA’s will be reviewed during progress meetings

The Project Safety Agreement is found as appendix ‘W’ (there is currently no link and can locate an appendix folder).

# Securing and Controlling Access to Sites

# Access control

All persons entering this site must present to the site office immediately and be identified providing all required documentation before being allowed to enter this site. This will be the responsibility of the Project Manager, Construction Manager, Site Safety and all Novo Rail site personnel.

At this point, all competencies, RISI, WorkCover OHS white card details will be recorded.

All persons attending site are required to sign-in/on and sign out each day at the site office.  Where sign out is not undertaken, site project staff are within their authority to contact and request verification.  Where repetitive failure to sign out or in is observed, follow up action to be escalated to project Manager.

Security will be maintained by existing chain wire fencing along the rail corridor (with lockable gateways) . Where required the live rail lines will be delineated by water filled traffic barricades and by blue & white marking tape, placed upon wooden stakes  in areas where barricading is not suitable. The perimeter will be locked each night and remain locked during the day to stop illegal entry by the public

If the entry or egress is to be left open during the workday a spotter will be placed on duty by the gateway to direct authorised incoming traffic and impede illegal entry.

# Restricted areas

- The Rail Corridor and Danger Zone

- The Novo Rail worksite is a restricted area to the public.

- Excavations when exposed with open trenches are to be barricaded off and protected in accordance with the Workcover code of practice for excavation. along the trench or open excavations to stop falls into the trenches by workers. This affected areas will be identified and mentioned in the PWB Only authorised persons are to climb or enter any of the OHW structures

- Any exclusion zones set-up around any plant and equipment contained by bollards with safety crowd control

- Any trenches or excavations.

- Confined spaces.

- Any heights above 1.5 mtrs.

- The old asbestos containing signal hut burnt down on the site, this has been barricaded-off with warning tape and signage. The removal of this hut is not the responsibility of Novo Rail and has been highlighted on the Workplace Risk Register and reported to the Region, whose responsibility is to remediate this action.

### Downer EDI Areas

Under no circumstances will anyone be allowed access to Downer EDI offices, sheds, warehouses, storage sheds without the explicit authorisation of a Novo Rail Management person, who may authorise entry under certain circumstances and the person escorted by a Novo Rail site staff person.

Some of Downer EDI areas are condemned and contain asbestos, warning signs are displayed against this hazard.

### Novo Rail Worksite

Access to this site will be by a lockable entrance gateway at the Montgomery Rd., end of the site, secured against illegal entry and through the industrial side of the project.

If the gateway is unlocked for any reason an authorised Novo Rail person will be present at the gate to   guard against illegal entry.

Other entrance to be supplied by the downside of the project by an access lockable gate, this will be for site deliveries of ballast and other site parking.

Access to the site will be organised by visitors and others by pre-warning of attendance and by ringing the site office with the phone numbers supplied at the key contacts sign at the entrance gate/s.

Signage to this effect will be in place at the strategic positions at the either entrance to the site.

Project Site Safety Rules

Induction and safety training

Induction training and daily safety briefing must be attended by all personnel before accessing live site, and supporting documentation is submitted, approved and site access agreed.

Documented records will be maintained.  These records are to be checked to ensure that personnel are in receipt of a current induction.  If in doubt or on significant change the induction will be undertaken again.

All visitors and persons attending site must sign in and out of site register.

Site-specific and work specific safety training will be provided and undertaken by all personnel on site before starting work.

Documentation will be maintained.  Where relevant, these records will be checked prior to allowing work commencement.

All personnel on the worksite must attend appropriate refresher training and be involved in regular discussion of worksite OHS and rail safety matters.

All visitors including those that will not be involved in physical work activities who wish to access the active work site zone will be provided with the site induction and relevant extracts of the daily safety briefing.   They also must be accompanied by a person familiar with the site and current activities or all parties on site made aware of their presence.

Personal protective equipment

All personnel and visitors must wear appropriate personal protective equipment (PPE) when on the worksite.  The site minimum plus the PPE specified in the relevant safe work method statement will be the minimum accepted standard unless written assessment is undertaken to deviate.  Minimum site standard PPE for this project is:

- Hard Hat

- Safety Footwear

- Long sleeve shirt and trousers

- Safety Eyewear as required

Site access and security

All entry to, movement on, passage adjacent to, and exit from the work site of persons, vehicles and equipment will be controlled.  This will be undertaken in accordance with required RailCorp Procedures.  In Summary:

Boundaries will be physical and maintained

Gates will where practical be closed and locked

All visitors will report to site office for sign in (and out) and induction procedures

All deliveries and visits must be notified in advance of visit

Illness/injury and emergency procedures

All hazards, near misses and damage sustaining incidents must be addressed or contained if safe to do so and within authority and reported to site office on occurrence or notice.

Plant

Plant and equipment must only be operated by trained and approved operatives.

Its arrival must be notified to the construction manager and included on a plant and equipment register.

It must be accompanied by a plant risk assessment.

It must only be utilised for tasks directly in line with it’s designed and intended use as per registration and/or documentation.

Attachments which act as hoists, crane attachments, mast climbing work platforms etc must hold relevant design registration with WorkCover.

Protection of all workers and the public

Effective barricades, fencing and overhead protection will be used where applicable

The rail corridor will not be accessed unless express permission is given and under the authority of the relevant track possession, authorisation guidance and instruction.

Elevated work

All work at heights must be done in accordance with the relevant legislation, regulations, standards, codes and procedures.

Where possible work at heights will be eliminated, or fall restraint measures put in place.

Work at height requires agreement from Novo Rail site management prior to commencement.

Confined spaces

Spaces with the potential to become ‘confined’ must be assessed prior to access.  If identified as a confined space the space will be signed as such as added to the sites confined spaces register.  It will then come under strict permit and procedural control.  All persons must notify the Construction Manager of any spaces they believe could become confined or contain a dangerous atmosphere.

Electrical work, overhead wiring, installations and equipment

All electrical work, plant and equipment must comply with OHS and electrical safety legislation, regulations, standards, codes and procedures, including inspection and tagging of leads and power tools.

The presence and location of all electrical cables will be identified before starting adjacent work.

Electrical equipment brought onto site must be notified to the construction manager and included in an electrical equipment register (and be part of an inspection and testing regime).

Demolition, excavation, scaffolding, formwork and other structural frames

All demolition, excavation, scaffolding, formwork, and work with other structural frames must be done in accordance with the relevant legislation, regulations, standards, codes and procedures.

Authorisation must be provided prior to any excavation or ground penetration work.  Relevant plans, permits and searches must be in place, and represented underground infrastructure and services positively identified prior to mechanical ground breaking.   Access into and in the surrounding area of excavations is by authorisation only.   Access into unshored, benched or supported excavations of 1.5 meters or above is strictly prohibited unless direct instruction to do so is provided by a geotechnical engineer.

Scaffolding will be erected and inspected by competent persons and with consideration to site conditions and intended use.  Work on scaffolding will consider weather/ site conditions and surrounding work activities.  Any damage or contact with scaffolding must be reported immediately.

Substances and Materials Hazardous to Health

A register of hazardous substances is kept and maintained for all hazardous substances brought onto the worksite.  Any materials or substances with the potential to pose a hazard to health must be notified to the Construction Manager, included in a register, stored appropriately and accompanied by relevant MSDS Sheets.

All hazardous substances and dangerous goods must be used, handled and stored in accordance with requirements of MSDS sheets and site procedures.

Safe work practices

All requirements identified will be followed, including fire prevention and housekeeping procedures.

All construction work must be covered by an applicable SWMS. All staff must have been instructed in all of the applicable SWMS.

SWMS must be available on site.

Drugs and alcohol

RailCorp and Novo Rail have a zero tolerance policy to drugs and alcohol. Persons are not allowed at work if under the influence. You can be randomly tested for both drugs and alcohol. Readings must be returned below -0.02.

Fatigue

The Novo Rail site works under a 10hr maximum working period rule.  Work exceeding this time frame must be notified to the Project manager, Project director and HSEQ manager and risk assessment and mitigation measures put into place before work can continue.

Railway Safeworking

Always use the highest available form of protection. ‘No Authority Required’ is a work on track method that provides the lowest form of protection. Before determining the work on track method/protection consult the Hazardous Rail Corridor Locations Register.

Worksite Protection Plans must be developed in accordance with the Networks Rules. Do not go on the track unless you have been informed of the worksite protection in place, you are authorised  and there is a safe place. Do not take short cuts across live lines.

General Site Safety Rules

- No unauthorised entry to this site

- All visitors to report to site office immediately.

- All gateways to remained locked or person placed at gateway to stop illegal entry.

- PPE requirements to be observed at all times by everyone.

- All persons entering the rail corridor must have RISI training and be prepared to provide evidence when requested by relevant personnel.

- All persons must have a Work Cover white/green card and be prepared to provide evidence when requested by relevant personnel.

- All drivers’ licenses, operator’s tickets must be evident and be prepared to provide evidence when requested by relevant personnel.

- All Excavation, Hot Work, Electrical works, Confined Space and any other required permits must be applied for, authorised and be readily available at the workplace.

- All signed speed limits to be observed by all.

- Speed limit 5 kph when passing workers.

- Employees to park in employee’s car park, no parking on-site.

- All site vehicles to have flashing amber light, audible reversing alarm, first aid box, fire extinguisher, radio contact, driven by a licensed person.

- Do not remove any bollards, safety fencing, and delineation tape without the express permission of the team leader’s higher persons.

- In the event of an emergency obey all directions given by the fire wardens and first aid officers.

- Be aware of snakes and vermin present in this workplace.

- No skylarking or playing on this worksite.

- All injuries, no matter how small, must be reported to the First Aid Officer or the OHS Advisor immediately.

- Any incidents or “near misses” must be reported to the Safety Advisor immediately.

- This is a No-Smoking worksite. No smoking in any buildings, offices, amenities, ablution areas  at any time.

- All employees will observe the Emergency Evacuation Drills and participate and take instructions from the Fire Wardens who will wear red safety helmets.

# Safety Signs

This site will be signed with 600mm x 450mm following signage:-

- Project Emergency contact details of Project Key personnel at workplace entrances on both up and down sides of project, Montgomery Street and within the adjacent industrial complex.

- Contact details of Site Management personnel for after hours contact or Emergency contact details at the entrance gates.

- By specific speed signs on chain wire fencing and within site

- At dedicated parking areas on delineated positions.

- All visitors to report to Site Office signs at workplace entrances on both up and down sides of project, Montgomery Street and within the adjacent industrial complex

- Warning signs denoting Construction Site – Do Not Enter signs at workplace entrances on both up and down sides of project, Montgomery Street and within the adjacent industrial complex

- Warning Signs denoting the requirement of wearing Hard Hats, Lace-up Safety Boots, Orange Hi-Visibility Vests, Hearing and Eye protection and other PPE as required  at workplace entrances on both up and down sides of project, Montgomery Street and within the adjacent industrial complex.

- All visitors must be safety inducted signs at workplace entrances on both up and down sides of project, Montgomery Street and within the adjacent industrial complex.

- Danger Zones areas at areas of  delineated positions  delineated positions

- Confined Space signage with warning Do Not Enter

- No parking at selected no parking areas i.e., where stockpiling of ballast will occur.

- 20 KPH Speed signs along roadways as selected

- 5 KPH Speed signs along roadways as selected

- Asbestos signage

# Monitoring, Reviewing and Reporting

# Contractor Safety performance Evaluation

The Site Construction Manager will be responsible for the evaluation of all Contractors’ Safety   Performance. This issue is to be included on the site meeting agenda for comment and action as required.

An evaluation is to be recorded on SMS-13-FM-0163 by the Project Manager Chris Gormlie or the Construction Manager Anthony Davis.

# Safety Observation

Safety observations shall be recorded on the safety observation form (SMS-10-FM-1356) and records stored in the site office.  Issues raised shall have an agreed close out time recorded and agreed with the works supervisor. Actions and their closeout will be monitored by the Project Safety Advisor and reported at weekly Project Meetings.

| Position | Person | Frequency |
| --- | --- | --- |
| Project Manager | Chris Gormlie | monthly |
| Project Engineer | Ryan Baden | weekly |
| Project Construction Manager | Anthony Davis | weekly |
| Safety Advisor | Stuart Phegan | weekly |

# Implementation Plan - Safety Concepts

A Safety Concept Plan is shown on Appendix ‘S” – no link

Key

# Site-Specific Safety Management Plans

| No. | Title | Description | When required |
| --- | --- | --- | --- |
| 1 | Site Plan | Site map | 28/5/10 |
| 2 | Site Safety Rules | Looks like the SSMP itself | 20/5/10 |
| 3 | Pre-Work Brief | Brief and checklist to go w/ it | 20/5/10 |
| 4 | Worksite Protection Plan | Network Rules | 28/5/10 |
| 5 | Service Searches | If digging a hole, check for gas, electricity, etc. | TBA (Const Mgr) |
| 6 | Possession / Region Induction Sheet | Could be done w/ Pre-work brief | 20/5/10 |
| 7 | Induction Record | Also found Site Specific OHS Induction template | 20/5/10 |
| 8 | Visitors Register | Sign-on form | Ongoing |
| 9 | Emergency Procedures - Injured Person - Fire - Bomb Threat - Gas Leak - Environmental - Emergency Contact List |  | prior to start of work packages |
| 10 | Electrical Permit Low Voltage Electrical permit to work Access to substation | No tracking no. - identified by permit number when issued | prior to start of work packages |
| 11 | Electrical Inspection and Test Record |  | Ongoing |
| 12 | Possession Briefing Notes | Could be done w/ Pre-work brief | Prior to possession |
| 13 | Chemical and Hazardous Substances Register - Dangerous goods & Hazardous Substance Register - HM & HL Reg - MSDS |  | Ongoing |
| 14 | Register of risk assessments | Listing of risk assessments different to the risk register | tba |
| 15 | Environmental Management Plan or Other Environmental Docs (EAS, GIS, Maps, Studies, etc) | Environmental checklist? | tba |
| 16 | Pre-Job Plant Inspection Report | Compare w/ pre-operation inspection criteria | Actual start of project |
| 17 | Register of Plant and Equipment | How does this work with Ellipse? | Actual start of project |
| 18 | Infrastructure Booking Authority | Part of Network Rules |  |
| 19 | Hot Work Authority - Welding - Grinding - Oxy cutting - Other (detail) - Hot work - total fire ban - Hot work & fire system isolation (CBD) |  | Actual start of project |
| 20 | Fire fighting equipment |  | Actual start of project |
| 21 | Amenities - Site office - Toilet - Meal room - Clean water |  | Actual start of project |
| 22 | Traffic Management Control - Pedestrian - Network - Work vehicles - Commuter | TMP, WTMP | Actual start of project |
| 23 | Restoration Checklist | Plant |  |
| 24 | Infrastructure Certification and Handover Form | Sign-off / approval | tba |
| 25 | Safety Action Request Form |  | Actual start of project |
| 26 | Design docs |  | WIP |
| 27 | Incident report | From Workplace Injury/Incident Reporting | Actual start of project |
| 28 | Interface agreements |  | WIP |
| 29 | Incident log |  | Actual start of project |
| 30 | Hazard Report Form | From Workplace Risk Management | Actual start of project |
| 31 | Risk Register | From Workplace Risk Management | 20/5/10 |
| 32 | Risk Assessment Forms | From Workplace Risk Management | Actual start of project |
| 33 | Hazard ID forms | From Workplace Risk Management | 20/5/10 |

# SWMS, SWI, and Work Plans Document Listing

| Document title SWMS | Reference number | Publisher | Description |
| --- | --- | --- | --- |
| Site Establishment | SJ-001 | ODG | Setting up site offices, amenities & storage containers, covered walkways, parking. |
| Manual Handling | SJ-002 | ODG | Methodology of safe work in manual handling |
| Material Deliveries | SULJVN-001 | LOR | Taking delivery of materials |
| Load & unload Trucks | SULJCN-003 | LOR | Safe working in loading & unloading delivery vehicles. |

# Review

See also Sections 3.5 and 4.0.

On this project the following roles, responsibilities and processes will be applied.

Key # - lead role * - support role.  Note not all responsibilities require lead and support roles.  Where none has been defined the responsibility sits with each party in terms of their area of control.

- Project Manager (P), Construction Manager (C), Health & Safety Advisor (H), General site personnel (G)

| Internal Reviews | P | C | G | H |
| --- | --- | --- | --- | --- |
| Undertake regular scheduled project safety reviews of projects and enter results into Novo Rail HSEQ Reporting and Corrective Actions System. |  |  |  |  |
| Review the Project Safety Management Plan and associated project safety documentation (6 monthly or on a significant changed) with the aim of not only indentifying corrective actions but also areas of improvement. Take the necessary actions to address identified issues and achievable improvements. |  |  |  | * |
| Cooperate with Project Manager during the Project Safety Review process |  |  |  |  |
| Periodically undertake the Construction Maintenance Planning Checklists. Plan to undertake at key stages of project and use as a review tool during identified potentially high risk activities falling into its category. |  |  |  |  |
| Take the necessary actions to rectify issues identified during the Project Safety Management Plan Reviews. |  |  |  |  |
| Assist the Project Manager in an advisory capacity during the Project Safety Management Plan review on request. |  |  |  |  |
| Carry out a project specific OHS System Audits against Ralcorp’s 20 elements six monthly. |  |  |  |  |

# Review of this Document

The Review of this Project Safety Management Plan will be carried out by Catherine Lloyd Program Safety Manager in the first instance and the Project Manager thereafter.

A review will be conducted at the completion of this PSMP prior to sign-off by relevant persons.

When a monthly audit or application when there is a significant change in project scope, schedule, work method, key personnel or relevant statutory requirements, a review will be prompted of this plan the application of specific aspects of this plan will be monitored quarterly.  The plans content shall be reviewed at a minimum six monthly

Where deficiencies are identified within reviews and are communicated by the Novo Rail safety Manager to the Safety Manager and other key parties.

# Post-Completion Review of Safety Issues

At the completion of this project, the Safety Advisor will consult with the OHS Representative, the Construction Manager and any other stakeholders including the Region on any safety issues that may remain within this project. This may be by identification of new pits (confined spaces) and to pursue the Continual Improvement methodology required in the RailCorp SMS.

# Corrective Action

It is the responsibility of the Project Manager to ensure that corrective actions are suitable, apportioned, closed out and is effective.  The duty of the actual task may be delegated to another project team member with relevant authority; however the responsibility for ensuring the applications remains with the Project Manager

# Reporting

| Report name | Produced by | Sent to | Frequency |
| --- | --- | --- | --- |
| Monthly Progress Report | Chris Gormlie | All Stake Holders | Monthly |
| Weekly Safety Flash report | Safety Advisor | Program Safety Manager | Weekly |
| Construction & Maintenance t Checklist | Construction Manager or other Project Team Member on his request | Chris Gormlie, Catherine Lloyd | weekly |
| Corrective Actions/Improvement actions | TBA Project Team | Project Manager and allocated persons | ongoing |
| Hazard reports | Project Team | Project Manager, Program Safety Manager and | ongoing |
| Environmental report | Shane McEntee | Chris Gormlie Environment and sustainability manager | ongoing |
| Weekly Flash reports | Chris Gormlie | Dave Robotham | Weekly |
| Project Lead/lag indicator | Chris Gormlie | Stakeholders and Program Safety Manager | Monthly |
| Incident | All project parties info to be provided to Chris Gormlie/TBA | HSEQ Team | Ongoing |
| External/enforcement authority visits | Chris Gormlie | Stakeholder sand Program Safety Manager | Monthly |

# References

- RailCorp SMS – Element 6 – construction and maintenance planning

Scope of work

Project Schedule

- Appendx C        Principal Contractor OHS Policy

Hazard idetification forms and risk assessments

- Appendix D     Safety change documentation

SCARD

Registers

Dangerous Goods and Hazardous Substances Register

Fire Extinguisher Inspection and Maintenance

Plant and Equipment Inspection Testing Register

Plant and Equipment Inspection and Maintenance

Corrective Action Register

Local Induction Register

SIG05 Project Change Control Register

Reporting

Communication and consultation plan

Training plan

Inspection schedule

Inspection forms

Site incident response procedure

Project management organisation structure

Emergency Contact list

Review Schedule

Document list

SMS-05-FM-0418%20Local%20SMS%20Document%20List.xlsx

Site-Specific Management Plans

Site Specific Management Plan Checklist

SWMS Review Tool

Novo Rail SWMS Review with link

RailCorp SWMS/SWI Review Link

Security Plans

SCARD RACIE Link

Appendix V

Appendix W

Appendix X

Appendix Y

Appendix Z