TSA Dunmore Loop Extension

AMS

(General Site Housekeeping / Deliveries, Potholing, Excavation / Trenching /Haul Road,

R171– TSA

R171 TOC 3370-AMS-001

Issue No: 1.1

| Workplace Risk Assessment (WRA) Ref No: | R171-WRA-DUNMORE 2010 001 | WRA Line Item Ref No: | 1.01-2.-2.06 |
| --- | --- | --- | --- |

ACTIVITY DESCRIPTION/OVERVIEW

The content in this AMS contains information on the methodology for Site compound set up, installation of site buildings and general site set out and preparation works. General site housekeeping, Deliveries, Potholing, Excavation &Trenching and haulroad

AUTHORISATION

| Date | Title & Name | Signature |
| --- | --- | --- |
|  | Author / Site Engineer Prepared by: David Pearce |  |
|  | Author / Project Engineer Reviewed By: |  |
|  | Site Foreman Reviewed By: |  |
|  | Site Supervisor Reviewed by: |  |
|  | Project Quality, Environmental & Community Representative Reviewed By: Ray Delandro |  |
|  | Safety Advisor Reviewed by: Justin Bles |  |
|  | Safety Manager Reviewed by: Keith Breckon |  |
|  | Project Manager Reviewed by: Michael Hockin |  |

- Introduction

  - Scope

The scope of work involved in the construction of site compound, storage areas, and site set out includes the following:

- General Site Tidying and clearance activities.

- Clearance/Maintenance of Minor vegetation (Grubbing)

- Installation/Maintenance of Temporary fenced compound

- Installation/Maintenance of Site office/Meal Room

- Installation/Maintenance of Secure Container type storage huts

- Material Deliveries.

- Pot-Holing and Service Identification.

- Excavation and Trenching

  - Activity Management Structure

| Name | Position | Contact Number | Company |
| --- | --- | --- | --- |
|  | Project Manager |  | TSA |
|  | Project Engineer |  | TSA |
|  | Graduate Engineer |  | TSA |
|  | Site Supervisor |  | TSA |
|  | Site Foreman |  | TSA |
|  | OH&S Manager |  | TSA |
|  | OH&S Advisor |  | TSA |
|  | Project Quality & Environmental Representative |  | TSA |
|  | Construction Supervisor |  | TSA |
|  | Project / Commissioning Engineer |  | TSA |
|  | Project Manager |  | TSA |
|  | Signalling Construction Manager |  | TSA |
|  | Supervisor |  | TSA |

1.3 Schedule

| Start week commencing: | 23 August 2010 |
| --- | --- |
| Completion date: | TBA |

- Activity Impact’s

  - Activity Impact Details

| Impact | Details |
| --- | --- |
| Public Considerations | - Noise - Dust - Traffic – Traffic Management Plan - On site traffic |
| Environmental | - Noise - Dust & Air quality - Soil and Water - Vegetation - Spills |
| Other | - Security - RTA / Council Property |

- Activity Methodology

- General Site Tidying and Clearance activities - Install Star Pickets and Blue/White tape fencing - Clear site of redundant materials left by others - Demarcate storage areas - Clear vegetation and level area at fenced compound - Scrape off and final trim - Clearance/Maintenance of Minor Vegetation (Grubbing) - Clear weeds and general ground vegetation. - Level areas required for compound. - Level Areas required for site accommodation - Installation/Maintenance of Temporary Fenced Compound - Install fencing foot blocks - Install temporary fence panels - Install netting on roadside fence - Install JHR signage - Installation/Maintenance of Site Office/Meal Room - Set out area of Office/Meal Room - Install Office/Meal Room - Install/Maintenance Secure Container Type Storage Huts - Set out area of Containers - Install Containers - Material Deliveries - Truck access and turning - Unloading of Materials - Storage and security. - Non-Destructive Potholing - Potholing - Backfilling and protection. - Excavations and trenching - Excavation works - Installation of Conduits and pits - Backfilling - Concreting & Formwork - Placement of Formwork and steel reinforcement. - Concrete Pour - Stripping Formwork.

- Summary of Resources

  - Activity Resources Requirements

| John Holland Resources |  |  |
| --- | --- | --- |
| Personnel - Site Management - Protection Officer - JH Site Staff - Invensys Staff - ARTC staff - Fencing Contractor - Delivery Drivers - Sucker Truck driver and operator - Excavator Operator - Truck with Hi-Ab - Dog-man - Carpenter - Crane Operator - Traffic controllers - Bungalow Manufacturer Rep. - Electrician |  |  |
| Plant & Equipment - Trucks - Truck Mounted Hi-Ab - Skip bin - Site Shed - Containers - Supa Sucker Truck - 5T Excavator x 2 - 3T Dumper Truck - 15T Excavator - Concrete Pump - 130tonne crane - Cable pulling equipment - Electrical power tools - HiRail Truck - HiRail Manitou |  |  |
| Materials - Bedding sand - Re-cycled sand for backfilling - Marking Out Paint - Temporary Fencing - Back-fill materials - Roadbase/Capping - Conduits and ducts - Cable Pits and Lids - Formwork - Steel Reinforcement. - Concrete |  |  |

- Environmental

  - Site Environmental Plans (SEP’s)

Note: 	Refer to AMS Risk Assessment for further guidance when developing specific SEPs

| AMS Step No. | Company | Task Description | JH Supervisor |
| --- | --- | --- | --- |
|  | JHR | Dust control measures (water carts etc) | Andrew Mcnally |
|  | JHR | Noise monitoring | Andrew Mcnally |
|  | JHR | Silt fencing | Andrew Mcnally |
|  | JHR | Spill kits | Andrew Mcnally |
|  | JHR | Waste minimisation (providing skip bins, recycling etc) | Andrew Mcnally |

  - Permit/Approval Requirements

- RailCorp Approval prior to any Site Works

- TSA Request For Access approved prior to any works

- TSA Excavation permit prior to breaking ground

- Council approval may be required (TBC)

- Shoal Haven Council approval for traffic management (Traffic Management Plan)

  - Emergency Response & Preparedness

- OH&S Management Plan - Emergency Response

- Environmental Management Plan

  - Inductions & Minimum Required Competencies

- WorkCover General Induction for the Construction Industry

- RailCorp - Railway Infrastructure Safety Induction (RISI) Card.

- TSA General Induction

  - Other Risk Assessments

  - RailWay Safe working brief  by Qualified Protection Officer prior to any works.

  - Locate Plant Hazard Assessments (PHA’s) for each item of mobile construction plant

  - Complete JH START card prior to commencement of daily works to determine if any new hazards are present and address as they are identified

  - Workplace Risk Assessment WRA-R171-001

  - Crane Plant hazard Assessment

  - Lift Study

  - Crane crew Risk Assessment (day of lift)

- Safety Requirements

  - Task Based Risk Assessments (TRA)

Note:	Refer to AMS Risk Assessment for further guidance when developing specific TRA’s

| AMS Step No. | Company | Task Description | JH Supervisor |
| --- | --- | --- | --- |
| 1.1 | TSA | Install Star Pickets and Blue/White Tape | Andrew Mcnally |
| 1.2 | TSA | Site Establishment | Andrew Mcnally |
| 1.3 | TSA | Clear vegetation at level area at compound (grubbing) | Andrew Mcnally |
| 1.4 | TSA | De-mobilisation | Andrew Mcnally |
| 1.5 | TSA | Non-Destructive Potholing | Andrew Mcnally |
| 1.6 | TSA | Excavation/ Install Conduits & Pits/Backfill | Andrew Mcnally |
| 1.7 | TSA | Install Formwork/ Steel reinforcement/ Concrete Pour/ Stripping | Andrew Mcnally |
|  |  |  | Andrew Mcnally |
|  |  |  | Andrew Mcnally |
|  |  |  | Andrew Mcnally |
|  |  |  | Andrew Mcnally |
|  |  |  | Andrew Mcnally |

  - Permit Requirements

- TSA Excavation permit

- TSARequest for Access Permit.

- 1500v OHW Isolation Permit

  - Emergency Response & Preparedness

- SSMP (Site Specific Management Plan)

- OH&S & R Management Plan:

- TSA-OHS-ERP-001  Site Emergency Response Plan

  - Inductions & Minimum Required Competencies

- TSA  Project Induction

- RailCorp Issued RISI Card

- WorkCover Construction Induction Card

  - Other Risk Assessments

- Plant Hazard Assessments

- Pre-Start Documentation

- Daily Briefing

- Safe Working Brief

| - Attachments |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| AMS Attachments Attach: - Design/Engineering Drawings/Sketches - Site Plans - Environmental Plans - Approvals - Permits - MSDS’s - Lift Studies/Calculations - Safety Equipment Specifications - Standard Operating Procedures (SOPs) or Process Procedures) - Task Risk Assessments (when/if developed) - Emergency Plans | Attach No. | AMS Step No. | Attachment Description / Reference Details | Checked By (Name & Signature) | Date |
|  |  |  | Excavation Permit |  |  |
|  |  |  | Request for Access |  |  |
|  |  |  | Environmental Plan |  |  |
|  |  |  | Site Specific Management Plan |  |  |
|  |  |  | MSDS’s |  |  |
|  |  |  | TRA’s |  |  |
|  |  |  | Crane Plant Hazard Assessment (PHA) |  |  |
|  |  |  | Lift Study |  |  |
|  |  |  | Crane folder (insurances, registrations, competencies, DBYD, lifting equipment register, contact numbers, etc) |  |  |
|  |  |  | Traffic Management Plan |  |  |

- AMS Development Team

| Name | Position | Name | Position |
| --- | --- | --- | --- |
|  | Senior Project Engineer |  |  |
|  | Foreman / Site Supervisor |  | Site Engineer |
|  | Signals Civil Supervisor |  | Project Manager |
|  | Project Safety Manager |  | Signalling Project Engineer |
|  | Project Safety Advisor |  |  |
|  | Project Quality & Environment Engineer |  |  |
|  | Rail Safety Coordinator |  |  |

- Change Management

| Issue No | Date | Change Management Details | Actions Taken to Notify of Changes |
| --- | --- | --- | --- |
| 1.1 | 31/05/2010 | Addition of Non-Destructive Pot-Holing Task | All |
| 1.1 | 03/06/2010 | Addition of Excavation/Trenching activity and updating of site set up to include maintenance of site. | All |
| 1.1 | 18/06/2010 | Addition of Installation of Formwork, Concrete Pour and stripping formwork activities | All |

|  | RISK MATRIX. Consequences; Likelihood or Probability. People; Environment; Plant Property Assets; Program / Time; Regulatory / Cost; Reputation; A; B; C; D; E. Almost Certain (expected); Likely (will probably occur); Moderate (might occur - has happened); Unlikely (could occur - known to happen); Rare (practically impossible); Risk Ranking; Risk Description; Control Application. 1; No Incident or First Aid Injury; Negligible Impact; Low $ Loss < $1000; Less than ½ Day; Fine < $1K; Minimal Publicity; High 15; Medium 19; Low 22; Low 24; Low 25; Low 21 - 25; Tolerable. Monitor, manage and carryout activity in accordance with identified controls.; Any hazard assessed as presenting a low and/or medium risk level will be permitted to be controlled using a combination of controls as appropriate.. 2; Medical Treatment; Minor on-site impact; Medium $ Loss >$1000 - < $10K; ½ Day - 1 Day; Fine $1K - $10K; Some Local Media Coverage; High 10; High 14; Medium 18; Low 21; Low 23; Medium 17 - 20; Implement strict control measures reduce hazard to ALARP. Management must determine appropriate level of supervision required.. 3; Alternate Work or Lost Time Injury; Moderate onsite impact; High $ Loss >$10K - < $50K; >1 Day - < 1 Week; Fine $10K - $50K; Media Coverage at State Level; Extreme 6; High 9; High 13; Medium 17; Medium 20; High 9 - 16; Implement strict control measures reduce hazard to ALARP. Activity must not commence without Workplace Manager or higher approval and appropriate supervision present. Review process.; Any hazard assessed as presenting a high risk level will only be allowed to be controlled using a combination of at least one engineering control and lower level controls as appropriate.. 4; Serious or Permanent Injury; Minor offsite or Major onsite impact; Major $ Loss >$50K - < $100K; > 1 Week - < 1 Month; Fine > $50K or Legal Proceedings; Adverse Publicity; Extreme 3; Extreme 5; Extreme 8; High 12; High 16; Extreme 1 - 8; Intolerable. Activity must not commence. Eliminated hazard or introduce further controls to reduce to ALARP.; Any hazard assessed as presenting an extreme risk level will only be allowed to be controlled using elimination and engineering as the primary source of controls.. 5; Fatality; Major offsite impact; Huge $ Loss > $100K; > 1 Month; Shutdown of Project Due to Regulatory Breach; Extreme Adverse Publicity; Extreme 1; Extreme 2; Extreme 4; Extreme 7; High 11. |
| --- | --- |

| AMS STEP NO: | WORKPLACE ACTIVITY MAIN STEPS (In Sequence) | BREAKDOWN OF STEPS (In Sequence) | HAZARD/ISSUE DESCRIPTION | INHERENT (BEFORE) RISK LEVEL (TRA is required for Ratings of Medium to Extreme) | Control Action Requirements (Before Activity is Commenced) Low/Medium = Combination of Hierarchy Controls High = Engineering & Admin and/or PPE Hierarchy of Controls Extreme = Engineering Hierarchy of Controls Indicate requirement for a TRA, SEP or ITP as a control measure | ACTIVITY PLANNING, RESOURCE, EQUIPMENT AND GENERAL CONSIDERATIONS |  | RESIDUAL (AFTER) RISK LEVEL | Actions Who What When | Status Date Completed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | - No of person to do the task - Occupations of persons req’s - Licenses/Qualifications/Permits - Training requirements - Approvals, Plans & Permits - Engineering Drawings / Refs - Environmental controls | - Lift Studies / Calculations - Static Plant & Equipment - Mobile Plant & Equipment - Safety Equipment - PPE - MSDS/Hazsub’s/DG’s |  |  |  |
| 1. | Site Access | - Accessing site - Delivery of materials - Delivery of site shed - Unloading & Loading materials - Marking out of areas - De-vegetation – Grubbing | - Plant Striking OHW - Plant, equipment and vehicle traffic Collision - Manual Handling whilst unloading and loading equipment - Personnel Struck by Plant & Equipment - Dropping of loads - Crush Injuries - Abrasions from tool and equipment - Hazardous substances whilst marking out areas - Delivery Drivers not understanding Site Rules Compliance, and Competencies. - Damage to RailCorp / ARTC Infrastructure causing property damage and risk to integrity of network - Vandalism, Theft of equipment from not securing the site - Parking Arrangements – Public Interface, Council, - Project and personal fines and interference to the public - Lighting (Night Works) - Unauthorised personnel including visitors entering construction area (rail corridor) - Competency of personnel on site is not appropriate to the skills required to perform the work correctly and safely - Personnel consuming or working under the influence of alcohol and or illicit drugs causing injury to personnel, damage to property, struck by train or plant and equipment - (Env) Dust - (Env) Mud trekked onto roads | E7 | TRA for Site Access required: Control Considerations: - All personnel undertake Project Induction - Worksite protection arrangements in place and briefed - All competency cards to be carried on personnel at all times and checked by Supervisor - Site security, gates locked or when open manned. - Drug & Alcohol Policy Zero Tolerance. Note: Random Testing will be conducted on site - Fatigue Policy - Mandatory PPE, Steel cap safety boots, long pants, long shirt, Rail Safety Vest, Hard Hat, Medium Impact eye protection (Safety glasses), gloves. Other PPE as per task, U.V protection (sun) - Job rotation repetitive work manual handling – correct lifting techniques - Fresh drinking water supply is readily available, and sheltered areas away from the elements - Parking as per council requirements, - Night works – day makers not faced to residential - RailCorp Infrastructure to be protected where at risk of strike or undermining - Delivery Drivers fully escorted at all times – ensure do not exit vehicle - MSDS’s on site and brief hazardous substances used - Exclusion zones, flashing lights, plant hazard assessments, pre-start checks, slew & height restrictors and onsite traffic routes to prevent struck by plant or plant collision - 3mtr clearance at all times to OHW as per WorkCover work near OHW code of practice and safe approach distance. - Ensure all lifting gear is registered. Ensure register has been signed by qualified dogman to certify equipment has been visually inspected minimum monthly. Annual certification of lifting gear tags to be visible. - Water carts where dust will be created - Street sweepers where mud trekked onto roads | Plant, Equipment, Tools, Flat bed truck Hi-Ab Hand tools Personnel Qualifications 1 X Supervisor 1 x Leading Hand 3x Labourers 1 x Protection Officer Training: Work Cover Construction Induction RailCorp Issued RISI ticket Dogman (where slinging techniques are used ) MSDS: Marking Paint as per MSDS folder Mandatory PPE: Hard Hat, Steel Cap Ankle Height Laced Safety Boots, Railway approved Hi-Vis Vest, Long Pants, Long Shirt, Gloves, Safety Glasses Permits; John Holland Request For Access John Holland Excavation Permit Inspections Pre-start inspection plant & equipment Lifting gear inspection prior to use |  | M17 | Supervisor prior to commencing work |  |
|  |  |  | - Struck by train / rail traffic. - Injury to personnel from exposure to extreme weather conditions - Injury to personnel, muscular strains (Manual Handling) - Fatigue - UV / Skin damage / cancer - Eye damage – cataracts / cancer - Snakes, Spider, Bee Stings and Bites - Mobile Phone Usage - Slips Trips and Falls - Damage to RailCorp Infrastructure - Ground Disturbance – Damage to underground assets - Equipment Failure – Lifting gear not inspected / wear and tear - Electrocution from Overhead Wires / Underground Services - Obstruction of live tracks during non possession work - Electrocution to personnel from the use of faulty electrical equipment - Cuts. Sprains, strains, abrasions, from use of hand tools and equipment - Storage and stockpiling. Dust, run-off, spillage. - Loading and unloading equipment materials from vehicles - Lifting and Lowering Loads. - Struck, cut, crushed or stabbed by equipment on site | E8 | TRA for Site Specific Hazards: Control Considerations: - Protection Officer Briefing and plan - Mandatory PPE to be worn - Ensure that rotation of task and regular rest breaks - Ensure that water supply is readily available, and sheltered areas away from the elements. - Correct manual handling techniques are adopted. Ensure that the John Holland Manual Handling HELP poster is briefed, and where possible use mechanical aids, and team lifts. - Effective rostering and planning Adhere to 10 hour working rule Rest breaks of no less than 12 hours prior to commencing work on site Visual assessments of employees Report to supervisor if you are feeling fatigued Accommodation to be provided if required - Mandatory PPE as listed. - Trained first aider in each crew clearly identified on pre work briefing. Take care to check for any infestations, particularly during excavation and inspection of pits and underground services. - Ensure that phone calls made and received whilst in the corridor are in the designated safe place. Under no circumstances are you to be in the danger zone - A demarcation fence not less then 3 metres from the closest running rail and a suitably qualified protection officer to work within the work team and ensure that the level of work site protection is adequate for the task being performed. | Plant, Equipment, Tools, Flat bed truck Excavator Hi-Ab Hand tools Personnel Qualifications 1 X Supervisor 1 x Leading Hand 2 x Labourers 1 x Protection Officer Training: WorkCover Construction Induction RailCorp Issued RISI ticket Excavator LE license Dogman (where slinging techniques are used ) MSDS: Marking Paint as per MSDS folder Mandatory PPE: Hard Hat, Steel Cap Ankle Height Laced Safety Boots, Railway approved Hi-Vis Vest, Long Pants, Long Shirt, Gloves, Safety Glasses Permits; John Holland Request For Access John Holland Excavation Permit Inspections Pre-start inspection plant & equipment Lifting gear inspection prior to use |  | M17 |  |  |
|  |  |  | Approach distances for work performed by Ordinary Persons. Nominal phase to phase AC voltage (V); Approach distance (m). Up to and including 132,000; 3.0. Above 132,00 up to and including 330,00; 6.0. Above 330,000; 8.0. Nominal pole to earth DC voltage (V); Approach distance (m). Up to and including +/- 1500; 3.0. Approach distances for work performed by Accredited Persons with a Safety Observer Nominal phase to phase AC voltage (V); Approach distance (m). Insulated low voltage cable up to 1000, including LV ABC; 0.5. Un-insulated low voltage conductors up to 1000; 1.0. Above 1000 up to and including 33,000; 1.2. Above 33,000 up to and including 66,000; 1.4. Above 66,000 up to and including 132,000; 1.8. Above 132,000 up to and including 220,000; 2.4. 330,000; 3.7. 500,000; 4.6. Above 330,000; 8.0. Nominal pole to earth DC voltage (V); Approach distance (m). Up to and including +/- 1500; 1.0. |  | - Ground conditions and where special attention needs to be made - Ensure all housekeeping is adequate and the correct storage of equipment and rubbish - Ensure that where plant and equipment is used in close proximity to RailCorp electrical infrastructure / assets that risk controls are used such as (concrete jersey kerbs, water filled barriers). JH Supervisor must be consulted prior to operation of plant near any infrastructure - All plant must have plant hazard assessment and pre start check prior to commencing on site. All lifting gear must have a register no older than one month showing that all lifting gear has been visually inspected for wear and tear and signed by competent / qualified person i.e. dogger, rigger - Working in close proximity to overhead powerlines prior to proceeding with works adjacent to the live Overhead Power Lines, John Holland must request relcoation or isolation of the asset owner’s line before construction work proceeds. All overhead lines should be treated as live unless the operator has received access authority or other form of written documentation from the network operator. - The following mimum safe approach distances apply. Ensure that machines that have the possiblity to encroach are fitted with height restrictors, and ensure a spotter observer is in place. - NSW Code of Practice, Work near Overhead Power Lines. - Supervisors are to ensure inspections are carried out of all equipment prior to use and ensure that tags for tag testing are in date. Any equipment that is found faulty or damaged must be tagged out with out of service tag and removed from service. All equipment must be tagged and tested on a monthly basis by a licensed electrician. Ensure equipment is properly stored to prevent damage – and residual current devices to be installed on portable generators |  |  |  |  |  |
|  |  |  |  |  | - Site supervisor to brief workers into TRA’s, Excavation Permit & Prestart Work Brief. All workers to understand requirements; ask questions if unsure - Store materials in clearly marked, designated safe area. Dust, run-off and spillage to be controlled by appropriate environmental control. Eg, barriers, drainage filters / fences, covers - Ensure that the protection officer has given worksite protection briefing indicating method of protection for work on track, no personnel to enter corridor until briefed. Traffic control plans and signage if required. 10km per hour speed limit within rail corridor to avoid generation of dust. Ensure that spotters are used when any trucks or plant are reversing and that quacker alarms and flashing lights are used. When stopping or parking vehicles ensure that it is in an area that can be clearly seen by others. Ensure parking is out of the danger zone. Ensure all access gates are locked or manned at all times. - Utilise certified lifting points and correct lifting appliances i.e. Hi-Ab to be used at all times and with the correct SWL noted on the equipment. Ensure the rated capacity indicator is on at all times. Ensure personnel to remain at a safe distance during unloading, stacking unless authorized to assist. - Drivers only to conduct or manage unloading operations and risk assessments must be conducted prior to deliveries being planned to ensure all hazards from overhead services are addressed. - Competencies to operate lifting appliances are required. Dogman required for slinging, calculating loads. - Ensure that all slings, chains are inspected and tagged. Ensure register has been obtained of lifting gear, lifting register is to be no older than 1 month and must be signed by qualified persons i.e. dogger, rigger to state that gear has been visually inspected. - Drivers to undertake driver specific induction - Exclusion zones to be set up whilst unloading materials and equipment. Supervision and adequate training must be provided - Minimise ground personnel, ensure correct use of PPE and ensure hands are kept clear of any crushing or pinch points. |  |  |  |  |  |
|  |  |  |  |  | - Personnel are adequately trained in the correct use of tools. Correct PPE must be worn at all times, and if heavy lifting is involved ensure that correct manual handling techniques are adopted - All tools and equipment must be selected to minimize risk to employees, ensure correct tool for the task. Small tools to be thoroughly inspected prior to use, and ensure all |  |  |  |  |  |
| 2. | Non-Destructive Pot-Holing | - Pothole services - Identify services | - Clearing area with plant, machines or by hand. - Splashes and eye injuries from particles when using blaster / vacuum potholing systems. - Manual handling of pressure jet & vacuum hose. - Hand crush injuries, suction of limbs - Cuts, abrasions from struck by high pressure water jet - Fall’s into holes. - Failure of Equipment (Oil Spill, Hose Splits) - Crush injury from release of vacuum lid | H13 | TRA for Site Specific Hazards: Control Considerations: - When clearing large areas minimize the dust with water spray and always stockpile spoil safely, away from access roads in a well marked, designated area. - Always wear the appropriate PPE including eye and face protection, if safety glasses are being worn ensure that they have side shields to prevent small particles from entering eyes. Masks are to be worn and gloves when operating or nearby and high pressure water blasting devices. Set exclusion zones. Only authorized persons to enter blasting area. - Ensure that vacuum and jet is commenced at idle speed, and ensure that it is slowly increased to required speed once top layer has been removed. Never point at any other area other then the area required. - Ensure that only trained & certified personnel remove obstructions from pipe and this is carried out in approved areas; ensure that the vacuum electrical source has been fully isolated to prevent crush or suction of limbs. - Ensure that the jet is never directed to operator’s body parts. Ensure that the area has an implemented 3mtr exclusion zone to prevent strike by water jet or flying debris, Ensure PPE is worn as previously mentioned in mandatory PPE. - Ensure that all holes left open exposed have been covered / barricaded to prevent persons from slipping or tripping in hole, preferably backfill. - Ensure that pre start check has been completed to identify any faulty hoses. Ensure that a spill kit is located on the vacuum truck should a spill occur. - Never stand behind truck when dumping water and ensure that dumping is conducted in council approved areas only. Ensure that vacuum is operating prior to releasing latch at rear. Disconnect latch then turn off vacuum keeping well clear of rear of tank. Ensure all individuals are clear of the area where dumping is to take place “exclusion zone”, ensure that there is adequate clearance when tilting tank and that the vehicle has been parked safely. | Plant, Equipment, Tools, Sucker Truck Hand tools Personnel Qualifications 1 X Supervisor 1 x Leading Hand 2 x Labourers 1 x Protection Officer Training: Work Cover Construction Induction RailCorp Issued RISI ticket MSDS: Marking Paint as per MSDS folder Mandatory PPE: Hard Hat, Steel Cap Ankle Height Laced Safety Boots, Railway approved Hi-Vis Vest, Long Pants, Long Shirt, Gloves, Safety Glasses Permits; John Holland Request For Access John Holland Excavation Permit Inspections Pre-start inspection plant & equipment Lifting gear inspection prior to use |  | M20 | Supervisor to complete and brief staff in TRA prior to commencing work |  |
| 3. | Excavations/Install conduits and pits/Backfill. And create haul road | - Access Plant - Excavate required areas for base, pits and cable routes. - Install Conduits - Install Pits - Backfill Excavations | - Struck by falling or moving objects - Collision with mobile plant and machinery - Damage to services / electrocution - Undermining Infrastructure - Personal Injury/Back Strain - Handling injuries - Trench collapse - Falling into trench - Open pot holes | E8 | TRA for Site Specific Hazards: - John Holland Excavation Permit to be completed - Plant Hazard Assessment for all plant to be completed, signed off by supervisor, engineer, and superintendent or SSFL construction manager. - Pre-Start Checks on equipment prior to start – Ensure all plant is serviced and in good working order. Any faulty equipment to be reported and tagged out of service/ - Colour DSS and Dial B4U dig completed prior to excavating - Non destructive digging to locate services - No mechanical digging within 2 meters of a known RailCorp service must be hand dug to expose service. - Services marked out by engineer, exclusion zone set up within 5 metres of located services. - Always take care around moving plant and make eye contact with operators before approaching. Keep a safe distance clear of plant when working nearby. Use spotters in tight confines and while reversing or where plant is likely to interfere with other personnel. - 2 metre exclusion zone set up for services located and verified. 300mm distance for mechanical excavation to low voltage services (1,500 V DC 1,000 V AC) and 600 mm to high voltage. - No hand digging near high voltage or signal cables - No general excavating within 1 metre of services - Assess condition/criticality of service prior to proceeding, check that all services are accounted for before digging. - Ensure the condition of the ground is able to support infrastructure prior to commencing excavation - Ensure undermining of infrastructure does not occur in the event infrastructure is undermined the infrastructure must be made secure with sandbags and props etc. Before proceeding with excavation. - Any excavation or trench of 1.5 meters in depth must 14 have shoring/ benching as required to prevent trench collapse. - No person to work alone in trenches. | Plant, Equipment, Tools, 5T Excavator Dumper Truck Truck with Hi-Ab Hand tools Personnel Qualifications 1 x Supervisor 1 x Leading Hand 2 x Labourers 1 x Excavator Operator 1 x Protection Officer 1 x Dumper Driver 1 x Truck Driver 1 x Hi-Ab operator Training: Accredited Excavator Operator Hi-Ab operator Ticket Work Cover Construction Induction RailCorp Issued RISI ticket Truck Driver License MSDS: Marking Paint as per MSDS folder Conduit Glue Diesel Fuel Unleaded Fuel Mandatory PPE: Hard Hat, Steel Cap Ankle Height Laced Safety Boots, Railway approved Hi-Vis Vest, Long Pants, Long Shirt, Gloves, Safety Glasses Permits; John Holland Request For Access John Holland Excavation Permit Inspections Pre-start inspection plant & equipment Lifting gear inspection prior to use. Excavation Checklist Engineering Drawings/Ref SC 09 01 Construction of Cable Route SCP 15 - Installation of Trackside Equipment Marked Up ISS drawings |  | H16 | Supervisor to complete and brief staff in TRA prior to commencing work |  |
|  |  |  |  |  | Cont: - Open holes are be delineated with marker tape, covered with road plate/plywood, mesh or back filled. - Trenches not to be left unattended. - Backfill trenches before end of shift/ day - Road plates to be placed over trench if unable to backfill - If a) or b) are unable to be done then surround trench with fence or delineation tape. - Use water spray to minimise dust when moving soil, sand or other fill materials, particularly in windy conditions. Always wear the appropriate PPE including glasses / masks. Pump any water away from water courses and drains. Apply appropriate environmental controls - Take care when machinery is used near edge of trenches. Under no circumstances should persons enter trenches while backfilling is in progress. At all times the operator needs to stay aware of all persons in the vicinity of the trench. - Trenches deeper than 1.5 meters require the appropriate benching or shoring to prevent collapse. |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | Closed Open Closed Closed Closed Closed Closed Open – On the day Closed |