Workers on Foot (WoF)

Management Plan

Road Construction and Maintenance Projects

Singleton Council

12-14 Queen Street,

Singleton NSW 2330

Version 1.0 – Jan 2020

Change history (project-specific document)

Approved by Road Project Officer

| Issue | Change description | Date |
| --- | --- | --- |
| 1.0 | Initial release | Jan 2020 |

Table of Contents

Management Review	2

Endorsement of WoFMP	2

1.	Objective	4

2.	Risk Management	4

3.	Site Induction	4

4.	Prestart Toolbox Briefing	4

5.	Site communication	4

6.	Workers on foot plans	5

7.	Roles and Responsibilities	5

8.	Unplanned Events (Emergency)	6

# Objective

The objective of this plan is to identify and control project site risks associated with workers on foot working alongside plant and machinery, focusing on the separation of people and plant.

It covers all aspects of the site including compound areas, work zones and safe zones.

This report covers the following considerations:

- Separation Zones

- Plant Movements

- Plant Isolation

- Plant unloading / loading

- Parking

- Refuelling

- Site Traffic

- Vehicle Movement Plans

- Managing unplanned events

# Risk Management

WOF are addressed in the project site risk assessment (refer Project Specific Plan).

# Site Induction

All workers will have a mandatory site induction prior to commencement of work on the site.  The induction will include site communication methods, e.g. UHF, and this WOF plan with explanations and instructions of zones, roles and responsibilities.

# Prestart Toolbox Briefing

Prior to the commencement of works each day or start of shift a Toolbox briefing will be held by the Site Supervisor, or the Ganger.  During this briefing the Site Marshall and/or Spotter will be appointed.  All workers will sign the daily Toolbox briefing form.

# Site communication

All Site communication will be via UHF radio. All plant and trucks must be fitted with working UHF radios before entering site and all the workers will have hand held radio. All workers on site shall have UHF radio.

Signage will be erected at site entrances displaying the UHF channel.

All delivery trucks entering the site will call the Site Supervisor over the UHF radio prior to arrival on site and wait in the designated waiting area for instructions.

Plant operators will communicate with each other using UHF Radio.

Plant operators working with workers on foot, for example pipe culvert Installation, will be in visible contact as well as radio contact.

# Workers on foot plans

The workers on Foot plans control access into and out of zones throughout the site.  They identify high risk areas ensuring exclusion zones, no go zones and shared zones are clearly identified.  They also identify where site marshals and spotters are required.

Zones of Separation.

The following zones will be used to control pedestrian and plant movements in different areas of the site. This is a guide only as site conditions may prove implementation impractical. Where it is deemed impractical, zones shall be demarcated using appropriately coloured traffic cones as follows:

No Go Zone

- Identified in RED

- Area prohibited for workers on foot or plant at anytime

Shared Zone

- Identified in BLUE

- An area which is shared by both plant and workers on foot activities and does not have any physical barriers, when interaction cannot be eliminated.

- Area and activates coordinated by a Team Leader.

Refuelling Zone

- Identified in WHITE

- Designated location for refuelling for the work site

Parking / Compound Zone

- Identified in PURPLE

- Parking zone for both plant and heavy vehicles and light vehicles at the site

Safe Zone

- Identified in GREEN

- A safe place for workers on foot to stand away from the work area or to take a phone call.

- A safe place for the marshal to stand to manage the interaction between people and plant.

Workers on foot plans are sketched for each activity clearly identifying the different zone and pedestrian walkways. These plans will be displayed on the daily communications board.

Site Compound Area

Due to the mobile nature of routine maintenance works, a designated site compound will not be used.

For construction works the compound area will comprise of a parking zone and plant refuelling/servicing zone. The parking zone will accommodate both heavy plant and light vehicle parking. The location of these zones will be determined onsite and communicated to staff during the site induction and/or Toolbox Meeting. A separate WOF plan will not be drawn up for the site compound area.

# Roles and Responsibilities

Site Coordinator

The person who is responsible for coordinating the every activity on the site and will appoint the Site Marshal and Spotter for the day.

Site Marshal

The Site Marshal coordinates and directs the safe interaction of people and plant in a work area.

The Site supervisor (Team Leader) will be the Site Marshal for the duration of the project.

The Site Marshal is required in SHARED ZONES where there are multiple workers on foot or multiple plant movements, also RESTRICTED AREAS where there are multiple plant movements.

The Site Marshal is responsible for:

- Provides instructions to plant operators and people on foot.

- Control the number of people and plant items in the vicinity of each other using the designated zones.

- Knowing where people and plant should be at all times.

- Communicates with all parties.

It is not anticipated that a full time site Marshall will be required, however a temporary Site Marshall will be appointed to undertake the role as and when required for the duration of the task/s being undertaken.

Spotter

The spotter looks out for, and warns of potential dangers and hazards around one activity, hazard or plant item.

Generally, Council does not have the resources to appoint a full time dedicated Spotter, but will appoint a Spotter to undertake the role as and when required for the duration of the task/s being undertaken. The requirement for a Spotter is determined by the risk assessment for that activity.

Worker on Foot

Workers on Foot are any people who are walking around on the worksite.

They are responsible for

- Knowing who the Site Coordinator, Site Marshal and Spotter(s) are.

- Understand the worksite zones and the workers on foot plans.

- Follow instructions from the Site Marshal or the Spotter

- Don’t enter SHARED or RESTRICTED zones without permission from the Site Marshal. Unplanned Events (Emergency)

# Unplanned Events

In the event of an unplanned event all works will stop immediately, plant will stop and workers on foot will down tools until further instructions are given by the Site Coordinator or Site Marshal.

In the event of an unplanned or unauthorised plant or personnel movement, or where a risk is identified an emergency call will be made over the UHF radio

“STOP WORK TOOLS DOWN” will be repeated 3 times

All work will stop until the Site Marshal gives the instruction to resume works.

# Appendix A – Workers on Foot Plans

|  | Location | Activity | Date |
| --- | --- | --- | --- |
| 1 | Various | Works requiring lane closure | 2 Jan 2020 |
| 2 |  |  |  |
| 3 |  |  |  |
| 4 |  |  |  |
| 5 |  |  |  |
| 6 |  |  |  |
| 7 |  |  |  |
| 8 |  |  |  |
| 9 |  |  |  |
| 10 |  |  |  |
| 11 |  |  |  |
| 12 |  |  |  |
| 13 |  |  |  |
| 14 |  |  |  |
| 15 |  |  |  |
| 16 |  |  |  |
| 17 |  |  |  |
| 18 |  |  |  |
| 19 |  |  |  |
| 20 |  |  |  |
| 21 |  |  |  |

Workers on Foot Plan

| Date | 2 Jan 2020 | Prepared by | Traffic Manager |
| --- | --- | --- | --- |
| Location | Various | Activity | Works requiring lane closure |