Project Risk Assessment

| Project | Project Name | Proj No | Project Number | Issue No | 1.0 | Date | DD/MM/YY |
| --- | --- | --- | --- | --- | --- | --- | --- |

| Participant Name | Signature | Participant Name | Signature | Participant Name | Signature |
| --- | --- | --- | --- | --- | --- |
| 1. |  | 6. |  | 11. |  |
| 2. |  | 7. |  | 12. |  |
| 3. |  | 8. |  | 13. |  |
| 4. |  | 9. |  | 14. |  |
| 5. |  | 10. |  | 15. |  |

| Sequence of activities | Safety hazards, environmental impacts | Initial risk |  |  |  | Control actions | Residual risk |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | S or E | How severe? | How likely? | Risk rating |  | S or E | How severe? | How likely? | Risk rating |

### Director Planning and Infrastructure sign-off

|  | Name |  | Signature |  | Date |  |
| --- | --- | --- | --- | --- | --- | --- |

### Risk assessment reckoner – safety, environment and journey management

| Determine the level of risk by adding the numbers for ‘How severe’ and ‘How likely’ |  |  |  |  | How likely? |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| How severe? | Type | Health & safety | Environment | Journey mgmt | 5 Almost certain | 4 Likely |  | 3 Possible |  | 2 Unlikely | 1 Rare |
|  | 5 Catastrophic | Fatality; debilitating or multiple injuries | Persistent long-term (>10 years) impact; large group of people affected; OEH or EPA involved; widespread media attention; requires extensive remedial action | Main road or freeway closure for multiple peak periods. Unable to meet ROL conditions for >60 min. | 10 EXTREME | 9 EXTREME |  | 8 EXTREME |  | 7 HIGH | 6 HIGH |
|  | 4 Major | Severe permanently debilitating or life-threatening injury | Long-term (1–10 year) impact; complaint to OEH; likely to attract media attention; requires considerable remedial action and notification to EPA. | Main road or freeway closure for one peak period. Unable to meet ROL conditions for >30 min. | 9 EXTREME | 8 EXTREME |  | 7 HIGH |  | 6 HIGH | 5 MEDIUM |
|  | 3 Moderate | Lost time injury; non–life threatening injury with treatment off site | Medium (1 year) impact; complaint from community; remedial action required; pollutant within vicinity of site that can be managed with routine procedures | Road disrupted for >30 min outside of peak periods. Unable to meet ROL conditions >15 but < 30 min | 8 EXTREME | 7 HIGH |  | 6 HIGH |  | 5 MEDIUM | 4 LOW |
|  | 2 Minor | Medical treatment injury | Short-term <6 month impact, mild affect on community, requires some action with minor resources readily available on site | Road network delays experienced up to 30 min outside peak periods. Unable to meet ROL conditions up to 15 min | 7 HIGH | 6 HIGH |  | 5 MEDIUM |  | 4 LOW | 3 LOW |
|  | 1 Negligible | First aid treatment injury | Short-term <1 week impact, community not affected, on site incident immediately contained. | Single minor delay on a low network category road outside peak period | 6 HIGH |  | 5 MEDIUM | 4 LOW | 3 LOW |  | 2 LOW |

### Management action guide

Plan control actions based on the level of risk and the tolerance level. Risks with a higher rating are highest priority for action.

| Risk rating | Risk level | Tolerance level | Action |
| --- | --- | --- | --- |
| 8–10 | EXTREME | Intolerable | A level of risk that is so high as to require significant and urgent actions to reduce its magnitude. If these levels cannot be reduced to as low as reasonably practical (ALARP) or Tolerable level then the activity should not proceed and should be reviewed by management. |
| 6–7 | HIGH | ALARP | Immediate action required - effort must be made to reduce risk so far as is reasonably practicable |
| 5 | MEDIUM | ALARP | Action required - effort must be made to reduce risk so far as is reasonably practicable |
| 2–4 | LOW | Tolerable | A level of risk that is so low as to not require actions to reduce its magnitude further but which will be managed and monitored by the use of management systems |

### The hierarchy of controls

The hierarchy works from the most effective solution – elimination of the hazard, making for a safe workplace – to the least effective, PPE, which requires individuals to protect themselves.

Select control measures from as high up the hierarchy as is practicable, according to the severity of the risk, current knowledge about the hazard, and the availability and cost of control technology. Use a combination of control measures if a single measure does not effectively control the risk.

| Level 1 | Elimination | Remove the hazard or hazardous work practice and eliminate the risk. Design the hazard out in the planning stage before construction begins. |
| --- | --- | --- |

| Level 2 | Substitution | Use something less hazardous or a less hazardous work practice. For example, use water-based paints rather than solvent-based. Use portable traffic signals instead of traffic controllers |
| --- | --- | --- |
|  | Isolation | Isolate or separate the hazard or hazardous work practice from people. For example, isolate a mobile plant work zone from workers. Use barriers to shield or isolate the hazard. For example, guards on machines, no-go zones near operating machinery. |
|  | Engineering controls | Design and install equipment to counteract the hazard. For example, bench, batter or shore the sides of excavations, install a local exhaust ventilation system to extract fumes or dust. Using a falling object protective structure (FOPS) or a roll-over protective structure (ROPS), will minimise the risk of an operator being struck by a falling object or being crushed if the excavator rolls over |

| Level 3 | Administrative controls | Administrative controls should only be considered when other higher order control measures are not practicable, or to increase protection from the hazard. For example, induction, SWMS and training. Using permit systems to prevent unauthorised persons from entering a confined space |
| --- | --- | --- |
|  | Personal protective equipment | PPE is the lowest order control measure in the hierarchy of controls. PPE should also only be considered when other higher order control measures are not reasonably practicable or to increase protection from the hazard. PPE relies on the proper fit and use of the PPE and does nothing to change the hazard itself. example, Hi Viz clothing, hard hats, earplugs and respirators. |