TRAFFIC CONTROL AT WORK SITES SAFETY INSPECTION CHECKLIST

Date: Time: Inspector: Design & Inspect TCPs Cert No. RMS Office/Contractor: Site Supervisor: TCP Number: TCP Modified: Y/N Road/Bridge Name: Location: Type of work: Duration of work: days Time/s of work: Road Configuration:

Rate in the following manner: √ Acceptable ᵪ Not Acceptable N/A Not Applicable

Guidance Notes:

- Detailed Inspections using this checklist shall only be undertaken by personnel holding a current Design and Inspect Traffic Control Plans certificate.

- Report to the Site Office or most senior person and attend site induction or be escorted.

- Desk-top “Audit”. Review paperwork and discuss site conditions, to complete Column 1. Check items against TCP and associated documents.

- Site Inspection. Conduct site verification inspection, discussing issues with random site workers/operators, to complete Column 2 – what you see on site.

- Complete your report on site, where possible.

- If you able to make a copy of the report on site, leave a copy with the supervisors.

- Forward an additional copy to the engineer.

- For contractor sites, forward an electronic copy to Manager Contractor Safety, RMS WHS Branch.

| No. | Conditions | TCWS Section | Rating |  |
| --- | --- | --- | --- | --- |
|  |  |  | 1 | 2 |
| 1 | TCP |  |  |  |
| 1.1 | Does the work require a: A TMP? A TCP? A VMP? (See 12 below.) A PMP? | G10 |  |  |
| 1.2 | Are all required plans approved? | 4.3 |  |  |
| 1.3 | Is the approved TCP on site? | 4.4.1/2 |  |  |
| 1.4 | Have signs and devices been set out as in the TCP | 4.4.1/2 |  |  |
| 1.5 | If modifications have been made are they approved and marked on the TCP? | 4.5 |  |  |
| 1.6 | Has a TCWS Appendix D Risk Assessment (RA) been done and been attached to the TCP? | App D |  |  |
| 1.7 | Does the RA cover the risks associated with the worksite? |  |  |  |
| 1.8 | Does the RA cover the risks; including ‘out of hours’ work? |  |  |  |
| 1.9 | Is the TCP relevant for the works in progress? | 4.4.2 |  |  |
| 1.10 | Has a Road Occupancy Licence been issued and is it being complied with? | G11 |  |  |
| 1.11 | Are the requirements implemented for safe clearances to workers and pedestrians and traffic approach speeds? | 3.6 9.3 |  |  |
| 1.12 | Other |  |  |  |
| Comments Item 1 |  |  |  |  |

| 2 | Roadwork Speed Zones (RSZ) |  | Rating |  |
| --- | --- | --- | --- | --- |
|  |  |  | 1 | 2 |
| 2.1 | Has the RSZ zone been authorised? | 8.2.6 |  |  |
| 2.2 | Is a copy of the SZA form held on site? | 8.2.6 |  |  |
| 2.2 | Has the SZA form been sent to local Police? | 8.2.6(a) |  |  |
| 2.3 | Are records being kept of the times of RSZ installation? | 8.2.7 |  |  |
| 2.4 | Where a RSZ is in place, is the limit appropriate for the works being undertaken? | 8.2.3 |  |  |
| 2.5 2.5 | Is the speed limit/s operating within the approved times? | 8.2.6 |  |  |
|  | Is the length of the speed zone as per TCWS? | 8.2.4(b) |  |  |
| 2.6 | Are Advanced Speed Warning Signs used appropriately? | 8.2.5(a) |  |  |
| 2.7 | Are Speed signs duplicated at the start of the speed zone? | 8.2.5(a) |  |  |
| 2.8 | Are speed signs the correct size? | 8.2.5(b) |  |  |
| 2.9 | Are all signs installed at the correct spacing? | 8.2.5(a) |  |  |
| 2.10 | Are all the signs installed at the correct height? | 8.2.5(c) |  |  |
| 2.11 | Have conflicting speed zone signs and pavement markings been covered/removed? | 8.2.5(e) |  |  |
| 2.12 | Are repeater signs installed if required? | 8.2.5(a) |  |  |
| 2.13 | Are “ENFORCED” signs required and installed? | 8.2.5(f) |  |  |
| 2.14 | At the end of the work, has the pre-existing speed limit been reinstated? | 8.2 |  |  |
| 2.15 | Are signs covered adequately when not in use? | 3.4.1;8.2 |  |  |
| 2.16 | Other |  |  |  |
| Comments Item 2 |  |  |  |  |

| 3 | Record Keeping |  | Rating |  |
| --- | --- | --- | --- | --- |
|  |  |  | 1 | 2 |
| 3.1 | Are records being kept for roadwork speed zones? | 8.2.6 |  |  |
| 3.2 | Are records kept as required in Appendix E? | 6.1 |  |  |
| 3.2.1 | By the Works Supervisor? | 6.1.1 |  |  |
| 3.2.2 | By the Team Leader? | 6.1.2 |  |  |
| 3.3 | RA is available on site and being kept with TCP? | App D |  |  |
| 3.4 | Where PTS are used, is the form Record of Approval and Use completed and retained? | T 10.7 |  |  |
| 3.5 | Other |  |  |  |
| Comments Item 3 |  |  |  |  |
| 4 | Traffic Controllers (TCs) |  | Rating |  |
|  |  |  | 1 | 2 |
| 4.1 | Are Traffic Controllers (TCs) being used? (Night work – 4.13) | 8.1 |  |  |
| 4.2 | Are the correct number of TCs being used? | 8.1.3 |  |  |
| 4.3 | Have TC Certificates been sighted and the No’s recorded? | G10 |  |  |
| 4.4 | Is TCs high visibility clothing in good repair? | 8.1.1 (a) |  |  |
| 4.5 | Are all TCs displaying the Roads and Maritime Services logo and Authorised Traffic Controller? | 8.1.1 (c) |  |  |
| 4.6 | Is the traffic speed restricted to max of 60km/h? | 8.1.1 (d) |  |  |
| 4.7 | Is the sight distance to approaching traffic 1.5D or greater? | 8.1.1 (e) |  |  |
| 4.8 | Do TCs have a clear escape route? | 8.1.4 |  |  |
| 4.9 | Has provision been made to prevent end of queue accidents? | 8.1.1 (e) |  |  |
| 4.10 | Are TCs able to communicate with each other (line of sight, two way radios, additional TCs) | 8.1.1 (f) 3.5.7 |  |  |
| 4.11 | Are the PREPARE TO STOP (T1-18) and traffic Controller Ahead (T1-34, T1-200-2/3) signs correctly displayed? | 8.1.1(a); 8.1.4 |  |  |
| 4.12 | Are the above signs covered or removed when not required? | 8.1.4 |  |  |
| 4 | Traffic Controllers (TCs) (continued) |  | Rating |  |
|  |  |  | 1 | 2 |
| 4.13 | Are they controlling traffic in accordance with Instructions to Traffic Controllers? | 8.1.4 |  |  |
| 4.14 | It TCs are being used for night work:- - are they wearing approved clothing? - are they safely lit and visible? - do they have correct communication? - are they using lighted wands? | 8.1.5 |  |  |
| 4.15 | Other |  |  |  |
| COMMENTS ITEM 4 |  |  |  |  |
| 5 | Portable Traffic Signals (PTS) |  | Rating |  |
|  |  |  | 1 | 2 |
| 5.1 | Are PTS being used? |  |  |  |
| 5.2 | Are the PTS formally approved for use? (This may be included on the TCP approval) | 4.4.3, 10.5 |  |  |
| 5.3 | Are the PTS being used marked as complying with RMS Specification PTS/3? | 10.2 |  |  |
| 5.4 | Are they PTS correctly registered? |  |  |  |
| 5.5 | Is the approach speed of traffic reduced to 60km/h or less? | 10.7.2 |  |  |
| 5.6 | Is the minimum sight distance of 150 metres provided? | 10.7.3 |  |  |
| 5.7 | Are the PTS been correctly sighted and established? | 10.7.1 |  |  |
| 5.8 | Has a Holding Line been marked on the roadway? | TCP43 |  |  |
| 5.9 | Are procedures in place to review the end-of-queue when PTS are operating? | 3.5.7 |  |  |
| 5.10 | Have all signs associated with PTS been erected correctly? | TCP43 |  |  |
| 5.11 | Other |  |  |  |
| COMMENTS ITEM 5 |  |  |  |  |

| 6 | Flashing Arrow Sign (FAS) |  | Rating |  |
| --- | --- | --- | --- | --- |
|  |  |  | 1 | 2 |
| 6.1 | Is a FAS being used? |  |  |  |
| 6.2 | Is the FAS being used marked as complying with RMS either Specification FAS/4 or FAS/5? | 11.2 |  |  |
| 6.3 | Is it located correctly? | 11.4.4 |  |  |
| 6.4 | Is it the correct size sign? | 3.2.10; 11.4.1 |  |  |
| 6.5 | Is the correct Mode of Operation being used? | Table 11.1 |  |  |
| 6.6 | If Lane Status signs (T2-6 series) are being used in conjunction with FAS, is the message to the motorist the same? |  |  |  |
| 6.7 | Other |  |  |  |
| COMMENTS ITEM 6 |  |  |  |  |
| 7 | Variable Message Sign (VMS) |  | Rating |  |
|  |  |  | 1 | 2 |
| 7.1 | Is variable message sign being used, as specified in TCWS? | 3.2.8 |  |  |
| 7.2 | Is the message related to the road or bridge works? | 3.2.8 |  |  |
| 7.3 | Are there less than 4 words per screen and no more than 2 screens on display? | 3.2.8 |  |  |
| 7.4 | Is the sign located in a safe position? |  |  |  |
| 7.5 | Is the VMS fitted with flashing blue and red lights? If yes have them switched off/removed. | 10.7.2 |  |  |
| 7.6 | Other |  |  |  |
| COMMENTS ITEM 7 |  |  |  |  |

| 8 | Safety Barriers |  | Rating |  |
| --- | --- | --- | --- | --- |
|  |  |  | 1 | 2 |
| 8.1 | Are safety barriers installed correctly? | 9.6 |  |  |
| 8.2 | Have the correct barriers been installed? | 9.6 & 3.3.7 |  |  |
| 8.3 | Where barrier sections are used as Safety Barriers, are they in compliance with AS3845 | 9.6 |  |  |
| 8.4 | Where non rigid barrier systems are used as safety barriers, is work behind the barrier prohibited from the deflection zone? | 9.6.5 |  |  |
| 8.5 | Are water filled safety barrier elements full of water? |  |  |  |
| 8.6 | Is the safety barrier erected as designed (incorporating end protection)? | 9.6.1 |  |  |
| 8.7 | Has the approach speed of traffic been reduced to the barrier design rating? |  |  |  |
| 8.8 | Other |  |  |  |
| COMMENTS ITEM 8 |  |  |  |  |
| 9 | Signs and Devices |  | Rating |  |
|  |  |  | 1 | 2 |
| 9.1 | Are all signs and devices in good condition? | 4.4.1 |  |  |
| 9.2 | Are the signs clearly visible and not affected by other signs, plant items, vegetation, shade. Light glare etc? | 3.1.1 |  |  |
| 9.3 | Are sign faces in compliance with AS1742.3 and have Class 1 retro reflective material? | 3.2.1 |  |  |
| 9.4 | Are the correct sign sizes being used? | 3.2.2 |  |  |
| 9.5 | Are signs duplicated, where required? | 3.2.4 |  |  |
| 9.6 | Are signs erected at the correct height and position? | 3.2.8 |  |  |
| 9.7 | Are the signs erected to give the correct sight distance? | 3.2.8 |  |  |
| 9.8 | Are signs displayed on frangible mounts? | 3.2.7 |  |  |
| 9.9 | Are barrier boards sighted at right angles to the flow of traffic? | 3.3.1 |  |  |
| 9.10 | Ate there any contradictory or superfluous signs, devices or marking? | 4.3.2 |  |  |
| 9.11 | Have the needs of pedestrians been provided for? | 9.3 |  |  |
| 9 | Signs and Devices (continued) |  | Rating |  |
|  |  |  | 1 | 2 |
| 9.12 | Have the needs for cyclists been provided for? | 9.4 |  |  |
| 9.13 | Are all property accesses to the site controlled? | 9.7 |  |  |
| 9.14 | Are all the cones and bollards installed at the correct spacing? | 5.2.2 |  |  |
| 9.15 | Are the correct sized cones and bollards being used? | 3.3.3 |  |  |
| 9.16 | Where tapers are used, have they been identified as lateral shift or merge tapers and are they the correct length? | 5.2 Table 5.2 |  |  |
| 9.17 | Where there are 3 lanes or traffic or more in one direction and two lanes are closed, are the separate merge tapers of the correct length? | 5.2.9 |  |  |
| 9.18 | Are the 2 tapers separated by at least 1.5D? | 5.2.9 |  |  |
| 9.19 | Where work is beyond a crest or curve, has the taper been set up before the crest or curve? |  |  |  |
| 9.20 | Where temporary pavement marking and markers are used, do they comply with the requirements of TCWS Manual? | 3.3.6 |  |  |
| 9.21 | Other |  |  |  |
| COMMENTS ITEM 9 |  |  |  |  |

| 10 | End-of Queue |  | Rating |  |
| --- | --- | --- | --- | --- |
|  |  |  | 1 | 2 |
| 10.1 | Has the potential for end-of queue accidents been considered and appropriate action taken? | 3.5.7 (a) |  |  |
| 10.2 | Has an assessment of expected queue length been undertaken/documented? | 3.5.7 (b) |  |  |
| 10.3 | Has protection been provided where the end-of queue is likely to be within D of the first downstream PTS sign? | 3.5.7 (c) |  |  |
| 10.4 | Is a sight distance between approaching motorists and the end-of queue, being maintained at greater than 2D (open road areas_ and 1.5D (built up areas)? | 3.5.7 (c) |  |  |
| 10.5 | Where the first PTS sign is more than 4D from the control point, are repeater signs placed at intervals of not more than 4D? | 3.5.7 (c) |  |  |
| 10.6 | Is the traffic queue monitored at all times during the course of the work? | 3.5.7 (b) |  |  |
| 10.7 | Other | 3.5.7(b) |  |  |
| COMMENTS ITEM 10 |  |  |  |  |

| 11 | Workers on foot near plant |  | Rating |  |
| --- | --- | --- | --- | --- |
|  |  |  | 1 | 2 |
| 11.1 | Have workers within 3 metres of plant been trained/briefed/tool-boxed on requirements of TCWS and RMS TIP sheet? | 9.23 |  |  |
| 11.2 | Where workers are working close to revolving plant, are satisfactory risk controls in place? | 9.23 |  |  |
| 11.3 | Has a VMP been developed where the conditions listed in TCWS occur on site? | 9.23.1 |  |  |
| 11.4 | Are spotters being used near reversing plant or delivery vehicles? | 8.1.1 (a) |  |  |
| 11.5 | Other | 8.1.1 (c) |  |  |
| COMMENTS ITEM 11 |  |  |  |  |

| 12 | Work Traffic (VMPs) |  | Rating |  |
| --- | --- | --- | --- | --- |
|  |  |  | 1 | 2 |
| 12.1 | Have accelerations and deceleration lanes been provided? | 7.2 |  |  |
| 12.2 | Are U turns being undertaken safely? | 7.3 |  |  |
| 12.3 | Are reversing movements being undertaken safely? | 7.3 |  |  |
| 12.4 | Are signs provided for stock pile sites etc? | 7.7 |  |  |
| 12.5 | Are median crossovers being used correctly? | 7.8 |  |  |
| 12.6 | Has a VMP been approved and provided? Written VMP shall be prepared in 100km/h zones | 7.5;7.6 9.23.1 |  |  |
| 12.7 | Does the person authorising the VMP have traffic control qualifications? If so, what qualifications? |  |  |  |
| 12.8 | Have access and egress to the site been safely provided? | 7.2 |  |  |
| 12.9 | Are delivery vehicles required to report to a designated location/person? Is it happening on site? | 9.23 |  |  |
| 12.10 | Other |  |  |  |
| COMMENTS ITEM 12 |  |  |  |  |

| 13 | Miscellaneous |  | Rating |  |
| --- | --- | --- | --- | --- |
|  |  |  | 1 | 2 |
| 13.1 | For intermittent work are all requirements met? | 9.1.2 |  |  |
| 13.2 | Where a spotter is used, are all requirements being met? | 9.1.2 |  |  |
| 13.3 | For mobile work are all requirements being met? | 9.17 |  |  |
| 13.4 | If the work is conducted at night are all requirements being met? | 9.2 |  |  |
| 13.5 | Where travelling plant or vehicles travel slower than 20km/h below the normal road speed limit, do they comply with the requirements of TCWS? | 9.1.3, 9.1.10 |  |  |
| 13.6 | Other |  |  |  |
| COMMENTS ITEM 13 |  |  |  |  |
| ADDITIONAL COMMENTS |  |  |  |  |
| Signed (Inspector) : ………………………………………………………. |  |  |  |  |